Bridge EtherTerm/TokenTerm and BAPI Extended Communications File

XCM Name:              BRIDGE.XCM

Softerm Revision:      3.00.00 or above

Base System File:      IBMPC.SYS

The Bridge BAPI extended communications file provides access to one port via

the EtherTerm/TokenTerm interface, one port via the BAPI interface with

the Bridge XNS product, or up to 4 ports via the BAPI interface with the

Bridge TCP/IP product.  This XCM uses the Interrupt 014H interface with the

Bridge BAPI extensions.

The INSTALL program provided with Softerm is normally used to build a system

file containing this XCM module.  Use the "Customize" option to build the

system file with the INSTALL program.  However, one may be built manually using

the following DOS command:

COPY IBMPC.SYS/B+BRIDGE.XCM/B SOFTERM.SYS

If an extended text module is also desired, it may be added in the command

any place after the IBMPC.SYS entry and before the space:

COPY IBMPC.SYS/B+HERC.XTX/B+BRIDGE.XCM/B SOFTERM.SYS

or

COPY IBMPC.SYS/B+BRIDGE.XCM/B+HERC.XTX/B SOFTERM.SYS

After building the system file, it is configured using the NETCONFG program.

This program is invoked by the following command line:

NETCONFG

No parameters are required or expected.  The NETCONFG program interogates the

system file to determine which XCM module is present, and provides the

appropriate configuration options.  In the case of the BRIDGE.XCM module, the

options concern the number of network communications ports, the mapped port

number and the auto-connection establishment options.

If you are using the EtherTerm/TokenTerm interface or the XNS BAPI product,

you should set the number of network COM line to 1, and the only other option

which is valid is the COM port number to which communications for the Bridge

network will be directed.  This is defined as the "Mapped to Port #" on the

configuration menu and is normally set to 1.  However, the Bridge software

allows the use of any value from 1-4.  If you have used the MODE command to

redirect a communications port printer as an LPT printer, you should change

the value here to be other than the number of the communications port that

is attached to the printer.  This field defines the COM port number that

will be used to communicate with the Interrupt 014H software provided by

Bridge.  As far as Softerm PC is concerned, it will always know the

Bridge port as COM4.  This is the port name that you should pick when you

want to use the EtherTerm/TokenTerm connection.

Bridge Extended Communications File                                    Page 2

If you are using the TCP/IP BAPI software from Bridge, you may define up to

4 network communication ports.  For these ports, the "Mapped to Port #" is

ignored by Softerm, and the "Auto-Connect on Assign" and "Auto-Connect

Call ID" fields are valid.  The "Auto-Connect on Assign" option provides a

mechanism by which to have Softerm establish a connection as soon as a port

assigned to the Bridge asynchronous service is selected.  If you answer this

option as "Yes", Softerm will attempt to establish a connection to the "Auto-

Connect Call ID".  This call id is in the same format as the argument

you would supply to a "Connect" command.  Auto-Connection is tried only once;

if it fails, you must use the Softerm Phone Book Utilities or the File

Transfer "Dial" command to establish a connection.

Each network communications port will be available on a different Softerm

COM port, mapped in the following manner:

for 1 network COM port:

Softerm's COM4 is BAPI session 0 or

the EtherTerm/TokenTerm session

for 2 network COM ports:

Softerm's COM4 is BAPI session 1

Softerm's COM3 is BAPI session 0

for 3 network COM ports:

Softerm's COM4 is BAPI session 2

Softerm's COM3 is BAPI session 1

Softerm's COM2 is BAPI session 0

for 4 network COM ports:

Softerm's COM4 is BAPI session 3

Softerm's COM3 is BAPI session 2

Softerm's COM2 is BAPI session 1

Softerm's COM1 is BAPI session 0

Any physical COM ports present on the PC will still be available under their

usual designations, provided that the number of network ports plus the number

of physical ports does not exceed 4.  If it does, the physical ports will be

deleted, starting at the highest numbered port, until all of the network ports

can be accessed.

After running NETCONFG, if you have not used the INSTALL program, YOU MUST

EXECUTE SOFTERM WITH THE /N SWITCH:

SOFTERM/N

This forces Softerm to recognize the reconfigured communications environment.

The Softerm Phone Book Utilities and the File Transfer "Dial" command are

supported when using the Bridge BAPI interface.  To define a connection,

enter the connect name in the number field of the phone book entry.