DEC LAT Extended Communications File

XCM Name:              DECLAT.XCM

Softerm Revision:      3.00.00 or above

Base System File:      IBMPC.SYS

The DEC LAT extended communications file provides access to up to 4 ports via

the DEC Local Area Transport (LAT) interface.  This XCM uses a DEC private

interrupt.

The INSTALL program provided with Softerm is normally used to build a system

file containing this XCM module.  Use the "Customize" option to build the

system file with the INSTALL program.  However, one may be built manually using

the following DOS command:

COPY IBMPC.SYS/B+DECLAT.XCM/B SOFTERM.SYS

If an extended text module is also desired, it may be added in the command

any place after the IBMPC.SYS entry and before the space:

COPY IBMPC.SYS/B+HERC.XTX/B+DECLAT.XCM/B SOFTERM.SYS

or

COPY IBMPC.SYS/B+DECLAT.XCM/B+HERC.XTX/B SOFTERM.SYS

After building the system file, it is configured using the NETCONFG program.

This program is invoked by the following command line:

NETCONFG

No parameters are required or expected.  The NETCONFG program interogates the

system file to determine which XCM module is present, and provides the

appropriate configuration options.  In the case of the DECLAT.XCM module, the

options concern the number of network communications ports and the

auto-connection establishment options.

You may define up to 4 network communication ports.  For these ports, the

"Auto-Connect on Assign" and "Auto-Connect Call ID" fields are valid.

The "Auto-Connect on Assign" option provides a mechanism by which to have

Softerm establish a connection as soon as a port assigned to the DEC LAT

asynchronous service is selected.  If you answer this option as "Yes",

Softerm will attempt to establish a connection to the "Auto-Connect Call ID".

This call id is the service name that you would supply in the phone book

number field when using the Softerm Phone Book Utilities.  Auto-Connection is

tried only once; if it fails, you must use the Softerm Phone Book Utilities

or the File Transfer "Dial" command to establish a connection.

DEC LAT Extended Communications File                                   Page 2

Each network communications port will be available on a different Softerm

COM port, mapped in the following manner:

for 1 network COM port:

Softerm's COM4 is DEC LAT session 0

for 2 network COM ports:

Softerm's COM4 is DEC LAT session 1

Softerm's COM3 is DEC LAT session 0

for 3 network COM ports:

Softerm's COM4 is DEC LAT session 2

Softerm's COM3 is DEC LAT session 1

Softerm's COM2 is DEC LAT session 0

for 4 network COM ports:

Softerm's COM4 is DEC LAT session 3

Softerm's COM3 is DEC LAT session 2

Softerm's COM2 is DEC LAT session 1

Softerm's COM1 is DEC LAT session 0

Any physical COM ports present on the PC will still be available under their

usual designations, provided that the number of network ports plus the number

of physical ports does not exceed 4.  If it does, the physical ports will be

deleted, starting at the highest numbered port, until all of the network ports

can be accessed.

After running NETCONFG, if you have not used the INSTALL program, YOU MUST

EXECUTE SOFTERM WITH THE /N SWITCH:

SOFTERM/N

This forces Softerm to recognize the reconfigured communications environment.

The Softerm Phone Book Utilities and the File Transfer "Dial" command are

supported when using the DEC LAT interface.  To define a connection,

enter the service name in the number field of the phone book entry.

When you are in terminal emulation without a connection and you press any

key, a list of the available services will be written to the screen.  Please

note that when using a block mode terminal, you must be in character mode

for this to happen!