Honeywell VIP7813

Description

The general capabilities of the Softerm emulation of the Honeywell

VIP7813 include the full 128 character-ASCII set with 80 X 24 or

80 X 72 display and simulated 80-character status line;

asynchronous character, text, and form transmission modes; visual

and form attributes; full keyboard emulation including cursor

control, editing, device control, and function keys; auxiliary

port printer support; and support for the VAF7821 Buffered Printer

Adapter Option.  The VDF7811 72-Line Scroll Option is also

supported.

The Honeywell VIP7813 is an expanded version of the Honeywell

VIP7803 Word Processing Workstation.

Features

The display format of the Honeywell VIP7813 is 24 or 72 lines by

80 characters with the capability to display 139 characters

including the 95-character ASCII set, 11 business graphic symbols,

and 33 symbols used only in communications display mode.  Softerm

supports display of the standard ASCII character set and line

graphic characters but not the special communications symbols.

The VIP7813 also displays a 25th line called the Status Line,

which is used to indicate the current operational status of the

terminal, and as a message line separate from the data space,

providing a one line window for dialog between the operator and

the host.  Softerm provides full support for the status line

capabilities.

The displayable character set of the VIP7813 was expanded to

include special word processing symbols.  Softerm displays

equivalent symbols from the available graphics character set.  A

special help screen which defines the use of the word processing

symbols may be displayed by entering Alt?  during online operation

and then paging through the help screens.

The Honeywell VIP7813 workstation provides additional features not

provided in the VIP7801 terminal, including a simplified keyboard

with a typewriter layout, key legends designed for office use, and

an expanded command set adding terminal operations designed to

enhance word processing applications.

Softerm supports all operational modes of the VIP7813 including

Character, Text, and Form modes of operation.  The Character mode

of operation provides the capability to transmit immediately a

code or codes as they are generated by a keystroke.  In the

Character Non-echo mode, the terminal reacts to codes from the

keyboard as they are transmitted to the host.  In the Character

Echo mode, the terminal does not react to code sequences generated

by the keyboard unless these codes are echoed by the host computer

and received as normal data.

The Text mode of operation allows the operator to enter data into

the terminal where it is stored and displayed, but not sent to the

host until a transmit is initiated by the operator using the

Honeywell VIP7813            - 1 -             Terminal Emulation

EXECUTE or RETURN key if the Transmit on Return option is set.  In

Text mode, messages may be composed, visually checked for

correctness, edited if necessary, and then transmitted.  A message

can be variable in length encompassing a single character, a part

of a line, several lines, a screen, or an entire data space.

The Form mode of operation derives its name from the pre-printed

paper form on which headings, labels, and instructions on filling

in the blank spaces are not changeable by users of the form.  In

the VIP7813, these fields are considered protected.  Unlike the

paper form, the terminal provides for validation of the data as it

is entered into the blank spaces or unprotected fields.  This

validation is controlled by the Form attributes which can be

assigned in combinations as required for different fields.

Softerm supports all form attributes with the exception of Omit

Print, and all visual attributes including inverse, low intensity,

underline, hide, and blink if the video interface and display

driver used support these capabilities.  Only the unprotected data

is normally transmitted to the host in Form mode when the EXECUTE

key is entered.

Setup switch options available on the Honeywell VIP7813 and not

supported by Softerm include Test Mode, Display All Mode, and

Hold/Drop DTR in Local.  However, Softerm provides a special local

mode of operation which does not drop the DTR signal causing a

disconnect.  Softerm can be toggled between the special local and

online mode using the VIP7813 Status Display initiated with the

Alt T keyboard function.

Softerm provides support for all keyboard functions.

Softerm recognizes and supports all host-initiated VIP7813

operational commands except Firmware Version Display (will cause

Clear), and Test Results Display.

Host-initiated VAF7821 Buffered Printer Adapter commands which

Softerm recognizes but ignores include Print Multiple Copies and

Print Transmit All.  Any form following a Print Transmit All

command will be processed normally.

Honeywell VIP7813 Emulation Options

After the last standard screen in the Terminal Options mode in

Terminal Setup, the following screen will be displayed:

����� Honeywell VIP7813 Emulation Options ���Ŀ

End-of-Message Character: ETX ($03)

CR/LF Delimiters: No

Operating Mode: Character

Space Suppress: Yes

Block Transmit Mode: No

Transmit On Return: No

Echo Mode: Yes

Number Of Rows: 24

Automatic Tabbing: Delayed

Roll Mode: Yes

Honeywell VIP7813            - 2 -             Terminal Emulation

The emulation option End-of-Message Character selects the

character which terminates all transmitted messages.  This

character should be selected as either an ETX ($03), or an EOT

($04).

The emulation option CR/LF Delimiters determines if a CR/LF

(carriage return/line feed) delimiter is sent at the end of each

line in Text mode.  Yes selects CR/LF delimiters for lines

transmitted in Text mode.  Selecting No will cause no delimiter to

be sent.  This option is ignored if the Space Suppress option is

set to Yes.

The emulation option Operating Mode determines which mode Softerm

will enter when initiating the online mode from terminal setup or

when a Reset to Initial State command is received.  Text selects

the Text mode of operation, while Character selects the Character

mode of operation.

The emulation option Space Suppress determines if trailing spaces

on a line are to be suppressed on a data transmission to the host.

Selecting Yes for this option will cause trailing spaces to be

suppressed, while selecting No will cause trailing spaces to be

transmitted.

The emulation option Block Transmit Mode determines if data is

transmitted to the host as a series of blocks or as a single

transmission.  If this option is specified as Yes, the terminal

transmits messages in blocks of 256 characters.  If this option is

specified as No, the terminal transmits the entire message in one

transmission.

The emulation option Transmit on Return determines if the Enter

key functions as a transmit key or normally when operating in the

Text mode.  If this option is specified as Yes, the Enter key will

initiate a transmit operation in the Text mode.  If this option is

specified as No, the Enter key will operate normally.

The emulation option Echo Mode determines if keyboard data is

displayed or acted upon when operating in the Character mode.  If

this option is specified as Yes, keyboard data is sent to the host

and not displayed or acted upon until processed and returned from

the host.  If this option is specified as No, keyboard data is

displayed or acted upon as it is keyed.

The emulation option Number Of Rows determines the amount of

display memory available.  Selecting 24 will allocate one page of

display memory, selecting 72 will allocate three pages of display

memory.  The Next Segment, Previous Segment, Scroll Up, and Scroll

Down sequences are valid only while operating in 72 line mode.

Honeywell VIP7813            - 3 -             Terminal Emulation

The emulation option Automatic Tabbing determines the cursor

action while leaving a field in Forms Mode.  It may be set to:

Delayed - When the last position of an unprotected field is

filled the cursor will move to the first position of the next

protected field.  On the next keystroke or recieved character

the cursor will move to the first position of the next

unprotected field and display the character, or sound an

alarm if no unprotected field exists.

Immediate - When the last position of an unprotected field is

filled the cursor will move to the first position of the next

unprotected field.  An alarm will sound if no unprotected

field exists.

The emulation option Roll Mode allows or prevents rolling of data

when a LF (line feed) is received on the last line of the display.

Selecting Yes allows rolling of screen data while selecting No

prevents rolling of the screen.

Pressing Alt Enter will bring up the following menu:

������ Honeywell VIP7813 Emulation Print Options ����Ŀ

Print All: Yes

Print Control Characters:

Start:     None

End:     None

Printer Underline Code:

Set:

Reset:

Printer Type: VIP7714/16 (30 cps)

Print Key Disable on PENQ: Off

The emulation option Print All determines the extent of the data

printed when a Print Data Space buffered printer adapter command

is received.  If this option is specified as Yes, both protected

and unprotected data is printed.  If this option is specified as

No, only unprotected data is printed.

The emulation option Print Control Characters designates the

character combination to be sent to the printer at the Start and

End of printing when a Print Data Space buffered printer adapter

command is received.  The choices for Start and End control

characters are None, CR, CR/LF, CR/VT, and CR/FF.  If the

character code is specified as None, no characters are sent to the

printer.

The emulation option Printer Underline Code will accept a sequence

up to fifteen characters long.  The Set sequence should be entered

as the code necessary to set the printer used to underline mode.

The Reset sequence should be entered as the code necessary to

reset the printer used from underline mode.

Honeywell VIP7813            - 4 -             Terminal Emulation

The emulation option Printer Type is provided so that the proper

code will be returned if the host is looking to see if a specific

printer is connected.  This option may be toggled to: VIP7714/16

(30 cps), VIP7717 (30 cps), VIP7717 (120 cps), PRU7003 (30 cps),

PRU7005 (120 cps), PRU7061/66 (100 cps), and PRU7070/75 (100 cps).

The emulation option Print Key Disable on PENQ may be toggled to

On or Off.  If set to On, all Print commands generated from the

keyboard, except PRES (Printer Adapter Reset), will be blocked.

Honeywell VIP7813 Operator Status Line Display

The Honeywell VIP7813 terminal includes a Status Line,

operationally separate from the 24 lines of screen data, which may

be displayed on the 25th line by pressing the Alt T key.  Included

on the status line are the READY indicator, the modes of

operation, the error messages, and the current row and column

position of the cursor.

The status information displayed includes Insert mode, Graph mode,

Prt Active, Log Mode, Log-A Mode, Text, Form, or Char mode, Echo

mode, Tx-RET mode, Roll mode, Auto-LF mode, Local mode, and the

current Row and Col.  The status line may also display the time if

downloaded by the host or entered by the operator using the

correct escape sequence.

After entering Alt T to display the status line, the Auto-LF mode

setting can be toggled by pressing the A key.  The Local mode

setting can be toggled while the status is displayed by pressing

the L key.  Setting the terminal for Local operation will not

cause a disconnect and allows local editing to be performed in the

Char mode of operation.

The status line allows certain modes of operation to be toggled by

pressing specific keys while the status line is displayed.  Online

terminal operation is suspended and input is enabled on the status

line whenever a blinking Right Arrow appears in the first column.

To return to the online mode after displaying the status, enter

Alt T or Alt Esc.  If you exit using the Alt T key, the status

line will remain on the screen during online operation and will be

automatically updated if required.  If you exit from the status

line display using the Alt Esc key, the status line will be

cleared.

The status line is also automatically displayed when an error

condition occurs.  Status messages displayed by Softerm include

DATA OVERFLOW, INVALID COMMAND, FILL REQUIRED, ENTRY REQUIRED,

ALPHA, DIGITS, NUMERIC, ALPHA/DIGITS, ALPHA/NUMERIC, PRINTER

INVALID CMD, and READY.  If a field entry error condition, such as

attempting to enter alphabetic data in a numeric field occurs, the

alarm is sounded and the appropriate error message will be

displayed on the status line.  No additional entry is permitted

until the error condition is cleared by pressing the PgUp (RESET)

key.

To return to the online mode after displaying the status, enter

Alt T or Alt Esc.  If you exit using the Alt T key, the status

line will remain on the screen during online operation and will be

Honeywell VIP7813            - 5 -             Terminal Emulation

automatically updated if required.  If you exit from the status

line display using the Alt Esc key, the status line will be

cleared.

Honeywell VIP7813 Keyboard Functions

The following table defines the standard keyboard functions for

the Honeywell VIP7813 terminal emulation:

Keyboard		Function

Enter			CR

Alt Enter		Execute

Backspace		Backspace

Tab			Tab

Shift Tab		Back Tab

Ins			Set Insert Mode

Alt Ins		Insert Line

Ctrl Ins		Reset Insert Mode

Del			Delete

Alt Del		Shift Delete

Ctrl Break		Soft Reset

Ctrl NumLock		Stop Display

Alt A			Send Answerback

Alt B			Break

PgDn			Erase to End of Field

Alt PgDn		Next Segment

Ctrl PgDn		Erase to End of Page

PgUp			Clear (Reset)

Alt PgUp		Previous Segment

Ctrl PgUp		Shift Clear (Clear)

Alt Ctrl PgUp		Ctrl Clear (Init)

PrtSc			Print Data Space

Shift PrtSc		Shift Print

Ctrl PrtSc		Reset Print

End			Stop

Ctrl End		Shift Stop

Ctrl 0 (Zero)		Note

Ctrl -			Auto Underline

Alt Up Arrow		Scroll Up

Alt Down Arrow		Scroll Down

Honeywell VIP7813            - 6 -             Terminal Emulation

Alt Ctrl 1		Set Attribute

Alt Ctrl 2		Delete Attribute

Alt Ctrl 3		Reset Visual Attribute

Alt Ctrl 4		Text Mode

Alt Ctrl 5		Form Mode

Alt Ctrl 6		Character Mode

Alt Ctrl 7		Set Tab

Alt Ctrl 8		Clear Tab

Alt Ctrl 9		Initialize Tabs

Unshifted and shifted combinations apply to the following keys:

F1	S-Sign Off/S-Menu	Ctrl F1		Go To Page

F2	Sign Off/Menu		Ctrl F2		Search

F3	Help			Ctrl F3		Abbreviate

F4	Calculate		Ctrl F4		Copy

F5	Send Message		Ctrl F5		Move

F6	Command			Ctrl F6		Page

F7	Indent			Ctrl F7		Code

F8	Center			Ctrl F8		Replace

F9	Decimal tab

F10	Super Sub

Alt Ctrl F1	Reset

Alt Ctrl F2	Backtab

Alt Ctrl F3	Xmit

Alt Ctrl F4	Horizontal Tab

Alt Ctrl F5	Carriage Return

Alt Ctrl F6	Linefeed

Alt Ctrl F7	Keypad +

Alt Ctrl F8	Keypad -

Alt Ctrl F9	Keypad ,

Note:  The following two key combinations are provided for

keyboards that do not have 12 function keys.

Alt F1		F11 - Merge

Alt F2		F12 - Format

Use CTRL to shift the following keys:

Right Arrow	Cursor Right	Up Arrow	Cursor Up

Left Arrow	Cursor Left	Down Arrow	Cursor Down

Home		Home

VIP7813 Status Functions:

Alt T	Display VIP7813 Status Line

A	Toggle Auto Linefeed Mode

L	Toggle Local Mode

Honeywell VIP7813            - 7 -             Terminal Emulation