Datapoint Datastation 3601

Introduction

The Datapoint Datastation 3601 can be emulated using Softerm by

selecting Datapoint 3601 for the terminal emulation when

generating a configuration in terminal setup.  The Datapoint 3601

general capabilities supported by Softerm include the full 128

character set with 80 X 24 display, absolute cursor addressing,

printer on/off functions for local printer support, and roll up

capability.  The Softerm Datastation 3601 emulation is compatible

with the Datapoint 5500 or 2200/1100 via the 9462 Multi-Port.

Features

The display format of the Datastation 3601 is 24 lines by 80

characters with standard absolute cursor positioning with the home

position in the upper left of the display.  Softerm includes

support for all standard keys on the 3601 keyboard and for the

edit keys Home, Clear, Erase to End of Line, and Erase to End of

Frame.  Conversation modes in full or half duplex are supported.

Softerm supports all remote terminal commands and switch settings.

Softerm also includes support for the Local Printer Option if the

terminal configuration includes a printer.  When a Printer On

function code $1A is received from the host computer, all

subsequent data is sent to the printer until a Printer Off

function code $14 is received from the host computer.  Data

received is also displayed on the video display.

Datapoint Datastation 3601 Keyboard Functions

The following table defines the standard keyboard functions for

the Datapoint Datastation 3601 terminal emulation:

Keyboard		Function

Enter			CR, LF, or CR/LF

Shift Enter		New Line

Esc			Cancel

Alt I			INT

Backspace		BS or DEL

Shift Backspace	DEL or BS

Home			Home Up

PgUp			Clear Screen

PgDn			Erase to End of Frame

End			Erase to End of Line

Ctrl Break		Soft Reset

Ctrl NumLock		Stop Display

Datapoint 3601               - 1 -             Terminal Emulation