TRS-80 Model 16 Console

Introduction

The TRS-80 Model 16 Console can be emulated using Softerm by

selecting TRS-80 Mod16 Console for the terminal emulation when

generating a configuration in terminal setup.  This emulation can

be used when operating a personal computer system as a terminal to

a multi-terminal TRS-80 Model 16 system using the TRS Xenix

operating system.  The emulation is fully compatible with the

trs16 entry in the TERMCAPS terminal capabilities file and

programs such as Scripsit which can use this feature.

���� TRS-80 Model 16 Console Emulation Options ��Ŀ

������ Special TRS-80 Graphics Character Set ����Ŀ

Char Xlat    Char Xlat    Char Xlat    Char Xlat

_ $C9   �    g $D1   �    o $05   #   w $16

` $BB   �    h $B6   �    p $04   #   x $B0   �

a $BC   �    i $D0       q $03   #   y $B1   �

b $C8   �    j $C7   �    r $06   #   z $B2   �

c $CB   �    k $CE   �    s $BA   �    { $FE   �

d $B9   �    l $D7   �    t $B3   �    | $DD   �

e $CA   �    m $D8   �    u $CD   �    } $DB   �

f $CC   �    n $C5   �    v $C4   �    ~ $18

The TRS-80 Model 16 Console emulation includes support for the

Model II Graphics Characters in the range $00 These characters are

mapped to standard graphic characters on most personal computers.

On the Tandy Model 2000 personal computer, Softerm modifies the

standard Model 2000 graphics character set in the range $80o

emulate the Model II graphics characters.  When you exit Softerm

to DOS, or when you return to the terminal setup options menu, the

standard Model 2000 graphics characters are restored.

TRS-80 Model 16 Console Keyboard Functions

The following table defines the standard keyboard functions for

the TRS-80 Model 16 Console terminal emulation:

Keyboard		Function

Enter			CR, LF, or CR/LF

Ctrl Break		Soft Reset

Ctrl NumLock		Stop Display

Right Arrow		Cursor Right

Left Arrow		Cursor Left

Alt A			Send Answerback Message

Up Arrow		Cursor Up

Down Arrow		Cursor Down

Alt B			Break

Backspace		BS or DEL

Shift Backspace	DEL or BS

Tab			Tab

Home			Cursor Home

PgUp			Clear Screen

TRS-80 Model 16              - 1 -             Terminal Emulation

PgDn			Erase to End of Page

End			Erase to End of Line

Ins			Insert Character

Alt Ins		Insert Line

Del			Delete Character

Alt Del		Delete Line

F1	f1		F5	f5

F2	f2		F6	f6

F3	f3		F7	f7

F4	f4		F8	f8

TRS-80 Model 16              - 2 -             Terminal Emulation