Prime PST100 Video Display Terminal

Description

The Prime PST100 Video Display Terminal can be emulated using

Softerm by selecting Prime PST100 for the terminal emulation when

generating a configuration in terminal setup.  The Prime PST100

general capabilities supported by Softerm include the full 128

character ASCII set with 80 X 24 or 80 X 48 display and simulated

80-character status line; asynchronous character, and block

transmission modes; visual and form attributes; full keyboard

emulation including cursor control, device control, and function

keys; and auxiliary port printer support.

Features

The display format of the Prime PST100 is 24 or 48 lines by 80

characters with the capability to display the 95 ASCII character

set, line drawing and block graphic symbols.  The PST100 also

displays a 25th line called the Status Line, which is used to

indicate the current operational status of the terminal, and as a

message line separate from the data space, providing a one line

window for dialog between the operator and the host.  Softerm

provides full support for the status line capabilities.

Softerm supports all operational modes of the PST100 including

Character, and Block mode of operation.  The Character mode of

operation provides the capability to transmit a code or codes

immediately as they are generated by a keystroke.

The Block mode of operation allows the operator to enter data into

the terminal where it is stored and displayed, but not sent to the

host until a transmit is initiated by the operator using the Alt

RETURN or Alt Shift RETURN key.  In Block mode, messages may be

composed, visually checked for correctness, edited if necessary,

and then transmitted.  A message can be variable in length

encompassing a line, or an entire data space.

Unlike the paper form, the terminal provides for validation of the

data as it is entered into the blank spaces or unprotected fields.

This validation is controlled by the logical attributes which can

be assigned in combinations as required for different fields.

Softerm supports all visual attributes including inverse, low

intensity, underline, hide, and blink if the video interface and

display driver used support these capabilities.  The strike

through visual attribute is not supported.

Softerm supports all setup switch options available on the Prime

PST100 except: Keyboard Click, Host Baud Rate, Auxillary Baud

Rate, Rollover, and diagnostic functions.

Softerm processes all operational commands except: DSC Mode Reset,

DSC Mode Set, and Single Shift Three.

Prime PST100                 - 1 -             Terminal Emulation

Emulation Options

After the last standard screen in the Terminal Options mode in

Terminal Setup, the following screen will be displayed:

����� Prime PST100 Emulation Options ����Ŀ

Character/Block Transmission: Character

Page/Line Mode: Page

Control Representation Mode: Off

Hard/Soft Scroll: Hard

Soft Scroll Speed: 2

Screen Video: Normal

Margin Bell: Off

New Line: Off

Line Truncate: No

The emulation option Character/Block Transmission determines if

data is transmitted one character at a time when a key is pressed

or if data is transmitted in blocks.

The emulation option Page/Line Mode determines if a block mode

transmit will send the current line or the whole page.

The emulation option Control Representation Mode determines if

control characters are displayed or acted upon.  This mode is also

referred to as debug mode.

The emulation option Hard/Soft Scroll, if set to Soft, will delay

scrolling to simulate a soft scroll.

The emulation option Soft Scroll Speed will determine the amount

of delay between scrolling lines when the soft scroll option is

selected.  A value of 1 causes the slowest scroll, a value of 4

causes the fastest scroll.

The emulation option Screen Video, if set to Normal will cause

white characters on a black background; if set to Inverse, black

characters will appear on a white background.

The emulation option Margin Bell, if set to On, will cause a beep

when the cursor passes column 72.

The emulation option New Line determines if the cursor will move

to the same column on the next line or move to the first position

on the next line when a line feed character is received.  When set

to On, the cursor will move to the first position of the next

line.

The emulation option Line Truncate determines if the cursor will

stay in the last position of a line or move to the first position

of the next line when a character is entered in the last position

of a line.  When set to Yes, the cursor will remain in the last

position of a line.

Prime PST100                 - 2 -             Terminal Emulation

Operator Status Line Display

The Prime PST100 terminal includes a Status Line (25th line on the

display screen) which is operationally separate from the 24 lines

of screen data.  It can be used as a host message line, a host

conversation line, or it can be used to inform the operator of

status conditions produced automatically by the terminal.

Softerm displays the status information on row 25 whenever the Alt

T key is pressed.

The status information displayed includes ONLINE, LOCAL, CHAR,

LINE, PAGE, CNTRL, 1 PG, 2 PG, SELECTED, MODIFIED, UNPROTECTED,

ALL, LD, GR, G0, G1, MAIN SEND, AUX SEND, KEYBD LOCK, SYSTEM RDY,

SOFT LOCK, and error messages.  Online terminal operation is

suspended and input is enabled on the status line whenever a

blinking Right Arrow appears in the first column.  To return to

the Online mode after displaying the status, enter Alt T or Alt

Esc.  If you exit using the Alt T key, the status line will remain

on the screen during online operation and will be automatically

updated if required.  If you exit from the status line display

using the Alt Esc key, the status line will be cleared.

Prime PST100 Keyboard Functions

The following defines the standard keyboard functions for the

Prime PST100 emulation:

Keyboard 		Function

Enter			CR, CR/LF

Alt Enter 		Transmit

Alt Shift Enter	Page Dump

Ctrl Break		Soft Reset

Backspace		BS

Ctrl NumLock		Stop Display

Tab			Tab

Alt Tab		Back Tab

Right Arrow		Cursor Right

Up Arrow		Cursor Up

Left Arrow		Cursor Left

Down Arrow		Cursor Down

Alt T			Display PST100 Status Line

F1 - F8		Function Keys 1-8

Alt F1			HELP

Alt F2			MENU

Alt F3			AUX SND

Alt F4			STOP

Alt F5			CHAR SET

Alt PG			DN RESET

Alt END		ERASE

Alt Up Arrow		SCROLL

For the following keys press Alt F10 then the key. You may also

use Shift, Ctrl, and Shift Ctrl to shift the following keys:

F1 - F10  		PF Function Keys 1-10

Alt F1 - Alt F4	PF Function Keys 11-14

Prime PST100                 - 3 -             Terminal Emulation