Hewlett-Packard 2622A

Introduction

The Hewlett-Packard 2622A Display Terminal can be emulated using

Softerm by selecting Hewlett-Packard 2622 for the terminal

emulation when generating a configuration in terminal setup.

Softerm supports all the general capabilities of the HP 2622A.

Features

The display format of the HP 2622A display terminal is 24 lines by

80 characters and two additional lines for system soft key labels.

If Softerm has been configured for, and the emulation is set for,

Extended Text mode, 24 data line and two soft key label lines are

supported.  If Extended Text mode is not being used, the emulation

will display only 23 data lines when the soft key label lines are

displayed.  The F10 key is used to toggle the soft key label

display on or off.

Softerm supports the Line Drawing and Extended Roman character

sets.  However, some characters for which there is no equivalent

may be blanked.  Softerm supports 4 pages (7680 characters or 96

lines) of display memory and display control functions related to

multiple page operation are processed accordingly.

The HP 2622A includes a Memory Lock Mode which provides two

separate functions called overflow protect and display lock.

Softerm supports the display lock feature which allows a top

margin to be specified so that lines above the top margin become

locked and cannot be scrolled off the screen.  The HP 2622A also

includes a display functions mode which causes ASCII control codes

and escape sequences to be displayed and not executed.  Softerm

also supports this feature.  The Caps Mode and Caps Lock mode

are not supported.

The HP 2622A includes a Terminal Configuration and Datacomm

Configuration Menu which allow terminal options and parameters to

be selected and saved in non-volatile memory.  Softerm emulates

this capability through its standard Terminal Setup capability and

includes a special HP 2622A parameter screen for options not

provided as standard Softerm features.  The Local Echo option

corresponds to the Softerm communications parameter Duplex, and

the InhEolWrp(C) corresponds to the Softerm terminal emulation

parameter Auto Line Wrap.  Terminal Configuration options not

supported by Softerm include Language, Frame Rate, and Caps Lock.

Datacomm Configuration Options Baud Rate, Parity, and Chk Parity

correspond to the Softerm communications parameters Speed, Parity,

and Number of Data Bits.  RecvPace corresponds to the Softerm

communications parameter Receive Pacing and XmitPace corresponds

to Transmit Pacing.

Softerm supports all the HP 2622A keyboard functions.

Hewlett-Packard 2622A        - 1 -             Terminal Emulation

Functions provided by the AIDS, MODES, and USER KEYS, are provided

as separate keys or by the HP 2622A Status Display which allows

various modes of operation to be toggled.  Softerm provides

special keys for functions.

Block/Character Mode, Modify Mode, and Auto LF Mode may be toggled

using the HP 2622A Status Display function.

Softerm recognizes and processes all host-initiated terminal

control functions with the following exceptions: Begin User Keys

Definition Mode, End User Keys Definition Mode, and Initiate

Terminal Self Test.  If a Begin User Keys Definition Mode function

is received and subsequently followed by a transmit block request,

the current definition of the user keys will be transmitted.

All host-initiated cursor control operations and configuration

operations are recognized and supported by Softerm.  The standard

Capture Line Mode to Print capability is used for Top or Bottom

Logging.  Printer control functions for normal, expanded, and

compressed characters, and Report and Metric mode are not

supported.  Record mode allows data to be transferred from the

host computer to the printer transparently without interpretation

or display.

Softerm supports all host initiated function key and error message

operations with the exception of enabling and disabling the

function or function control keys and automatic display of

function key labels.  If an error message is received, it will

appear in the User Key Label area at the bottom of the display.

The error message will remain displayed until it is removed by the

host computer application or is replaced by the Status Line.

The Terminal ID Status function is supported by Softerm and will

cause a 2622A character string to be transmitted.

Terminal Setup Parameters

After the last standard screen in the Terminal Options mode in

Terminal Setup, the following screen will be displayed:

��� Hewlett-Packard 2622A Emulation Options �Ŀ

RETURN Def: $0D $00

Start Col: 1

XmitFnctn(A): No

SPOW(B): No

Line/Page(D): Line

InhHndShk(G): No

Inh DC2(H): No

Fld Separator: $1F

Blk Terminator: $1E

ASCII 8 Bits: No

EnqAck: Yes

Block Mode: No

Modify All Mode: No

Auto LF Mode: No

Display Memory: 2 Pages

Hewlett-Packard 2622A        - 2 -             Terminal Emulation

The emulation option RETURN Def specifies the definition of the

Enter key.  The default definition is an ASCII CR ($0D).  The

definition may consist of up to two characters.  If the second

character is a null ($00), it is ignored.

The emulation option Start Col specifies the column at which

transmission is to begin in MODIFY LINE or MODIFY ALL modes if the

line has no logical start-of-text pointer.  Its value may be from

180.

The emulation option XmitFnctn(A) specifies whether escape code

functions are both executed at the terminal and transmitted to the

host computer.  If this field is specified as Yes, the escape code

sequences generated by control keys such as PgUp (CLEAR DSPLY) and

End (CLEAR LINE) are transmitted to the host computer.  If local

echo (Half Duplex) is selected, the function is also performed

locally.  If this field is specified as No, the escape code

sequences for the major function keys are executed locally but not

transmitted to the host computer.

The emulation option SPOW(B) specifies whether or not spaces

entered through the keyboard will overwrite existing characters.

If this option is specified as No, spaces entered through the

keyboard will overwrite existing characters.  If this option is

specified as Yes, the Space Overwrite Latch is enabled.  Once

enabled, it is turned on by a carriage return and turned off by a

line feed home up, or tab.

The emulation option Line/Page(D) specifies whether or not the

terminal, when operating in block mode, will transmit data a line

at a time or a page at a time.  Line selects the line at a time

data transmission for block mode.  Page selects the page at a time

data transmission for block mode.

The emulation options InhHndShk(G) and Inh DC2(H) determine what

type of handshaking is to be used when transferring blocks of data

from the terminal to the host computer.

The emulation option Fld Separator defines the character which is

transmitted at the end of each unprotected field when the terminal

is in block page mode and a formatted display is active.  The

character may be specified as any valid ASCII character in the

range $00$7F.

The emulation option Blk Terminator defines the character which is

transmitted at the end of certain types of block data transfers

between the terminal and the host computer.  The character may be

specified as any valid ASCII character in the range $00$7F.

The emulation option ASCII 8 Bits specifies whether the ASCII SO

code for shifting to the alternate character set is effective to

the end of the line or to the next ASCII SI code even if it occurs

several lines later.  If this option is specified as Yes, SO codes

will be effective until the next SI code even if it occurs several

lines later.  If this option is specified as No, SO codes are

effective only to the end of the current line.  This option does

not affect the communications parameter Number of Data Bits.  The

emulation option ASCII 8-Bits also allows the character set used

Hewlett-Packard 2622A        - 3 -             Terminal Emulation

whenever the alternate character set is selected to be specified.

If Yes is specified, the Extended Roman becomes the default

alternate character set.  If No is specified, Line Drawing becomes

the default alternate character set.  If the ASCII 8-Bits option

is set to Yes, you should insure that the communications parameter

Number of Data Bits is set to 8, and Parity is set to None.

The emulation option EnqAck enables the use of the Hewlett-Packard

ENQ-ACK handshake as a pacing mechanism.  Selecting Yes for this

option enables the EnqAck handshake while selecting No disables

the handshake.

The emulation option Block Mode specifies whether or not the

terminal will operate in character or block mode by default.  If

this option is specified as Yes, Softerm will operate in the block

mode.  Characters entered from the keyboard in block mode are not

transmitted as they are entered.  A complete line or screen may be

entered and then transmitted using the ENTER key.  If this option

is specified as No, Softerm will operate in the character mode.

In the character mode, a character is immediately transmitted when

a keyboard key is pressed.  Softerm can be toggled between the

block and character modes using the HP 2622A Status Display

initiated with the Alt T keyboard function.

The emulation option Modify All Mode allows the Modify All Mode of

operation to be selected by default.  Yes selects the Modify All

mode of operation while No selects the Modify Off mode of

operation.  The Modify Mode can be toggled between Modify Off,

Modify Line, and Modify All using the HP 2622A Status Display

initiated by the Alt T keyboard function.

The emulation option Auto LF Mode allows the Auto Line Feed Mode

to be selected by default.  When Auto Line Feed Mode is enabled,

an ASCII line feed control code is automatically appended to each

ASCII carriage return control code generated through the keyboard.

Selecting Yes for this option enables the Auto Line Feed Mode.

Selecting No for this option disables the Auto Line Feed Mode.

The Auto Line Feed Mode can be toggled on and off using the HP

2622A Status Display initiated by the Alt T keyboard function.

The emulation option Display Memory can be set to either the 2

pages supported by the standard 2622A terminal or to 4 pages of

Display Memory.

HP 2622A Function Key Definition

When editing of the HP 2622A emulation options is complete,

pressing the Alt Enter key will cause the following screen to be

displayed:

Hewlett-Packard 2622A        - 4 -             Terminal Emulation

��� Hewlett-Packard 2622A Function Key Definitions �Ŀ

f1  Attribute: T Label:    f1

�ESC�p

f2  Attribute: T Label:    f2

�ESC�q

f3  Attribute: T Label:    f3

�ESC�r

f4  Attribute: T Label:    f4

�ESC�s

f5  Attribute: T Label:    f5

�ESC�t

f6  Attribute: T Label:    f6

�ESC�u

f7  Attribute: T Label:    f7

�ESC�v

f8  Attribute: T Label:    f8

�ESC�w

The fields displayed for each function key definition include the

Attribute, Label on the first line of each key definition, and an

80-character key definition field occupying the entire second line

of each key definition.

The Attribute field defines the function key attribute as executed

locally only L, transmitted to the host computer only T, or

treated in the same manner as the alphanumeric keys N.  If the

Attribute field is set to N, and Softerm is in the local mode, the

content of the key is executed locally.  If Softerm is in the

online full-duplex mode, the content of the key is transmitted to

the host computer.  If Softerm is in the online half-duplex mode,

the content of the key is both transmitted to the host computer

and executed locally.

The Label field defines the function key label of up to 16

characters.  The labels are displayed on rows 23 and 24 of the

screen when an F10 key is entered in the online terminal operation

mode.  The label definition is displayed as two fields of eight

characters each on two rows.  The first 8 characters of the Label

correspond to the upper portion of the label and the last 8

characters correspond to the lower portion.

The entire line (80 characters) immediately below the attribute

and label fields is available for specifying the character string

that is to be displayed, executed, and/or transmitted whenever the

function key is pressed.

Press Alt Enter to return to the terminal setup menu.

HP 2622A Status Display

The HP 2622A terminal emulation includes a status display which

includes information normally displayed on the additional screen

rows on the HP 2622A where the function key labels are displayed.

Softerm displays the status information on row 25 whenever the Alt

T key is pressed.  The status information displayed includes the

current Row and Column, Insert mode indicator, Block or Character

mode, Modify mode, Auto LF mode, and Memory Lock mode.

Hewlett-Packard 2622A        - 5 -             Terminal Emulation

The Block or Character mode setting can be toggled while the

status is displayed by pressing the B key.  Online terminal

operation is suspended and input is enabled on the status line

whenever a blinking Right Arrow appears in the first column of the

status line.  For example, to toggle from character mode to block

mode, first display the status by entering Alt T and then press

the B key to change the Character mode to Block mode.

The Modify mode setting can be toggled while the status is

displayed by pressing the M key.  Pressing the M key repeatedly

will toggle the modify mode between Modify Off, Modify All, and

Modify Line.  If Modify Line is selected, this mode remains in

effect only until the Enter or Alt Enter (Enter) key is pressed

which will return to the Modify Off mode.

The Auto LF mode setting can be toggled while the status is

displayed by pressing the A key.  Pressing the A key repeatedly

will the toggle the auto line feed mode between Auto LF Off and

Auto LF On.

The Memory Lock mode setting can be toggled while the status is

displayed by pressing the L key.  Pressing the L key repeatedly

will toggle the memory lock mode between Memory Lock Off and

Memory Lock On.  The current cursor position must be in a row

greater than 1 to activate the Memory Lock On mode.

Hewlett-Packard 2622A        - 6 -             Terminal Emulation

Hewlett-Packard 2622A Keyboard Functions

The following table defines the standard keyboard functions for

the Hewlett-Packard 2622A terminal emulation:

Keyboard		Function

Enter			RETURN

Alt Enter		ENTER

Ctrl Break		Soft Reset

Ctrl NumLock		Stop Display

Alt A			Send Answerback Message

Alt B			Break

Alt D			DEFAULT VALUES

Right Arrow		Cursor Right

Left Arrow		Cursor Left

Up Arrow		Cursor Up

Down Arrow		Cursor Down

Alt Up Arrow		ROLL UP

Alt Down Arrow		ROLL DOWN

Alt [			LEFT MARGIN

Alt ]			RIGHT MARGIN

Alt Ctrl M		CLR ALL MARGINS

Backspace		BS or DEL

Shift Backspace	DEL or BS

Alt PgDn		NEXT PAGE

Alt PgUp		PREV PAGE

Tab			TAB

Shift Tab		BACK TAB

Alt Ctrl Tab		CLR ALL TABS

Alt Right Arrow	SET TAB

Alt Left Arrow		CLEAR TAB

Alt S			SET START COL

Home			HOME UP

Ctrl Home		HOME DOWN

PgUp			CLEAR DSPLY

End			CLEAR LINE

Del			DEL CHAR

Alt Del		DEL LINE

Ins			INS CHAR

Alt Ins		INS LINE

F10			USER KEYS

Hewlett-Packard 2622A        - 7 -             Terminal Emulation

Shift PrtSc		COPY PAGE

Alt PrtSc		COPY LINE

Ctrl PrtSc		RECORD MODE

Alt T			Display HP 2622A Status

F1	f1		F5	f5

F2	f2		F6	f6

F3	f3		F7	f7

F4	f4		F8	f8

Hewlett-Packard 2622A        - 8 -             Terminal Emulation