Digital Equipment VT131

Introduction

The Digital Equipment Corporation VT131 terminal can be emulated

using Softerm PC by selecting DEC VT131 for the terminal emulation

when defining a configuration from the Setup Options menu.

Softerm supports all the general capabilities of the VT131 model

including the full 128 character set and 80 X 24 or 132 X 24

display, Smooth (Slow) Scrolling, Special Graphics Character Set,

user definable answerback message, ANSI or VT52 mode, Auto

XON/XOFF, Line/Local, New Line, Tabs, Wraparound, Auxiliary Keypad

and Cursor Mode Functions, Program function Keys, the additional

character attributes of the Advanced Video Option, and the print

and host editing features provided by the Printer Port Option.

Features

The display format of the VT131 allows for 24 lines by 80 or 132

characters.  Softerm supports the 132 column format using Extended

Text Mode or horizontal scrolling.  The VT131 has an operator

accessible Setup mode which allows terminal features to be

selected and then saved in non-volatile memory.  Softerm provides

the equivalent of this mode in the Terminal Parameters option of

the Setup Options menu and allows all VT131 features to be

specified except for: AUTO REPEAT, INTERLACE, KEYCLICK, and SCREEN

BRIGHTNESS.  In addition, Softerm allows only 1 speed to be

selected and used for both receive and transmit.

Softerm supports all available keyboard functions and provides

additional keyboard functions for Cursor Home, Erase all of the

Display, Erase to End of Line, Erase to End of Screen, Print

Screen, and Enable/Disable Auto Print.

Softerm recognizes and processes all escape and control sequences

of the VT131 in both ANSI and VT52 compatible mode.  However,

hardware limitations prevent Softerm from providing a full

implementation of some features.  Some control sequences are

completely ignored while others are processed in a limited manner.

Control Sequences which Softerm recognizes but ignores in the ANSI

mode include Request Terminal Parameters, Auto Repeat Mode, and

Invoke Confidence Test.  Softerm recognizes and processes as a

special case control sequences for Double Height Line, Double

Width Line, and Single Width Line.

If a graphics adapter card is installed and activated by enabling

the Extended Text Mode, double-high/double-wide characters are

supported fully.  If the Extended Text Mode is not enabled, for

double height lines, only the top half line is displayed.  For

double width lines, each displayable character is followed by a

space.

With the PC's Numlock key in the Off position, the VT131's Cursor

Key Mode is simulated with the cursor control keys on the PC's

numeric keypad.

Digital Equipment VT131      - 1 -             Terminal Emulation

The VT131's Keypad Application Mode is simulated in Softerm using

the Alt key in combination with a 0-9, minus, period, comma, and

Enter for the auxiliary keypad functions.  However, the Keypad

Application mode must be enabled by the host or the Keypad Mode

Emulation Option (described below) before the numeric keypad Alt-

key functions are enabled.  Softerm also enables and disables the

auxiliary keypad functions and generates the appropriate codes for

the VT52 mode commands Enter alternate keypad mode and Exit

alternate keypad mode.

In the VT52 compatibility mode, Softerm supports all control

sequences.

After the last standard screen in the Terminal Options mode in

Terminal Setup, the following screen will be displayed:

������ DEC VT131 Emulation Options ����Ŀ

Operating Mode: VT131

Print Extent: Full Screen

Print Terminator: None

Number of Columns: 80

Screen Display: Normal

Scroll Rate: Jump

Character Set: U.S. ASCII

Margin Bell: Yes

BREAK Enabled: Yes

Keypad Mode: Numeric

Local Horizontal

Scroll Increment: 1

Horizontal Scroll

Delay (1/10 Secs.): 2

Operating Mode

This option allows the initial operating mode of the terminal to

be selected, and to determine the response to the Identity or

Device Attributes control sequence.  If the VT52 operating mode is

selected, the VT131 emulation will be initialized in the VT52

mode.  If the Enter ANSI Mode command is received, the emulation

will switch to the ANSI VT131 operating mode.  However, any ANSI

mode control sequence which would cause the terminal to return to

its initial state, will cause the terminal to return to the VT52

mode.  If VT131 selected, the emulation will be initialized in the

ANSI mode and respond accordingly.

Print Extent

This option determines what portion of the screen is printed when

the Shift PrtSc keyboard function is executed or a print screen

command is received.  If this option is specified as Full Screen,

the full screen will be printed.  If this option is specified as

Scrolling Region, only the area defined in the current scrolling

region will be printed.

Print Terminator

This option specifies if a form feed character is is generated

after the screen is printed using the Shift PrtSc keyboard

function or when a print screen command is received.  If this

option is specified as Form Feed, a form feed character ($0C) is

Digital Equipment VT131      - 2 -             Terminal Emulation

appended to the data sent to the printer by the print screen

function.  If this option is specified as a None, no form feed is

included.

Number of Columns

This option allows the initial column mode of the VT131 emulation

to be selected.  If 80 is selected, the emulation will be

initialized in 80-column mode.  If 132 is selected, the emulation

will be initialized in 132-column mode.

Screen Display

This option allows the initial screen display mode to be selected

as Normal or Reverse.  If Normal is selected, the foreground and

background screen colors are displayed normally.  If Reverse is

selected, the foreground and background colors are reversed.

Scroll Rate

This option allows the scroll rate to be selected.  If Jump is

selected, data will be scrolled at a fast jumpy rate.  If 6

Lines/Second is selected, smooth scrolling is simulated by

scrolling at a slower rate.

Character Set

This option allows either the U.S.  ASCII or United Kingdom

character set to be selected.  If United Kingdom is selected, the

U.S.  # character is displayed as the British currency symbol, �.

Margin Bell

This option selects whether the audible alarm is sounded as

characters are keyed when approaching the right margin.  If this

option is set to Yes, the audible alarm is sounded when a

character is entered at column 74.  If this option is set to No,

no alarm is issued.

BREAK Enabled

This option determines if the BREAK key (Alt B) is enabled.  If

this option is set to No, the BREAK function is disabled.

Keypad Mode

This option allows the default operating mode of the numeric

keypad to be selected.  If set to Numeric and NumLock is On, the

keypad will generate numbers.  If NumLock is Off, the keypad will

generate Local Edit Functions.  If set to Application and NumLock

is On, the special keypad applications mode is enabled and

generates special code sequences.  If NumLock is Off, the keypad

will generate Local Edit Functions unless used with the Alt key,

in which case an Application Mode sequence will be generated.

Note: The host may override the setting of this option.

Local Horizontal Scroll Increment

This option selects the number of columns the screen will be

scrolled left or right when the Alt Ctrl Left Arrow or Alt Ctrl

Right Arrow local horizontal scroll keys are used.  It may be any

value in the range 1-20.  Note: These keys are disabled if a

Hercules-type graphics adapter card supporting 132 columns is

installed in the PC and you have chosen the 132-column Extended

Text Mode option.

Digital Equipment VT131      - 3 -             Terminal Emulation

Horizontal Scroll Delay (1/10 Secs.)

This option determines the amount of time in one tenth second

increments that Softerm will delay before horizontal scrolling is

initiated when no characters have been received for the amount of

time specified by the delay.  This option prevents unnecessary

shifting of the screen when a continuous stream of characters is

being received.

After the preceding options have been specified, press Alt Enter

and the following additional option screen is displayed:

��������� DEC VT131 Local Editing Set-up ������Ŀ

Edit Mode: Off

Erasure Mode: All

Edit Key Execution: Immediate

Transmit Execution Mode: Immediate

Guarded Area Transfer Mode: All

Line Transmit Mode: Off

Transfer Termination Mode: Full page

Space Compression Mode: Off

Transmit Termination Character: None

Edit Mode

This option may be toggled to Off or Enabled.  When set to Off,

the terminal is placed in an interactive mode and typed characters

are transmitted to the host immediately.  If set to Enabled, you

can edit the screen text.  When you are finished editing, the text

will be transmitted as a block to the host.

Erasure Mode

This option may be toggled to All or Unprotected, and determines

which screen characters the host can edit.  When set to

Unprotected, the host can edit only unprotected characters.  When

set to All, the host can edit all displayed characters.

Edit Key Execution

This option may be toggled to Immediate or Deferred, and

determines how the Edit key operates.  If this option is set to

Deferred, Shift Edit transmits a sequence of characters to the

host.  When these characters are echoed back, the terminal changes

between interactive and editing operation.  When this option is

set to Immediate, Shift Edit selects interactive or editing

operation immediately.

Transmit Execution Mode

This option may be toggled to Immediate or Deferred, and

determines how the Enter key operates.  If this option is set to

Deferred, the terminal transmits a character sequence to request a

block character transmission.  When the host is ready to receive

the characters, it transmits a sequence of characters to begin

transmission.  When this option is set to Immediate, the terminal

transmits characters when you press the key.

Guarded Area Transfer Mode

This option may be toggled to All or Unprotected.  When the

terminal is operating as an editing terminal, screen characters

are either protected or unprotected.  If this option is set to

Digital Equipment VT131      - 4 -             Terminal Emulation

All, all screen characters will be transmitted to the host (or

printer).  If set to Unprotected, only the unprotected characters

are transmitted.

Line Transmit Mode

This option may be toggled to Off or Enabled, and determines the

size of the block of characters that are transmitted to the host

when in edit mode.  If set to Off, the Transmit Termination

feature determines how large a block of characters to transmit.

If set to Enabled, one line of characters will be transmitted each

time you press Enter.

Transfer Termination Mode

This option may be toggled to Full page or Partial page.  When the

Line Transmit Mode is set to Off, this option determines if the

terminal transmits a block of characters to the host as a Full or

a Partial page.  If set to Full page, all characters within the

margins are transmitted.  When set to Partial page, the characters

between a partial page marker and the cursor are transmitted.  If

a partial page marker is not present, the top margin is used.  The

marker (non-displayed) is automatically positioned at the last

character transmitted.

Space Compression Mode

This option may be toggled to Off or Enabled.  If set to Off,,

unused character positions on the screen will be transmitted as

spaces.  If set to Enabled, unused character positions will be

transmitted as a sinngle control character that represents the

remaining spaces of a field.

Transmit Termination Character

This option (also called End of Block Character) may be toggled to

None, FF, ETX, EOT or CR.  It designates the control character

used to indicate the end of a transmitted character block.

After the preceding options have been specified, press Alt Enter

and the following additional option screen is displayed:

����������� DEC VT131 Emulation Options ����������Ŀ

������������������� Tab Stops ��������������������

Columns           T       T       T       T

1-40      1234567890123456789012345678901234567890

Columns   T       T       T       T       T

41-80     1234567890123456789012345678901234567890

Columns   T       T       T       T       T

81-120    1234567890123456789012345678901234567890

Columns   T       T

121-132   123456789012

Tab Stops

Softerm allows default tab stops to be defined which are set at

initialization, when Ctrl Break is pressed, or any control

sequence is received which would return the terminal to its

Digital Equipment VT131      - 5 -             Terminal Emulation

initial state.  The tab format consists of 4 string entry fields,

for columns 1-40, 41-80, 81-120, and 121-132.  Entering any non-

blank character will result in a tab stop being set at the current

location.

Operator Status Line Display

The DEC VT131 terminal emulation includes a status line which may

be displayed on the 25th line by pressing the Alt T key.  The

status information displayed includes Row and Column, Operating

Mode, 80 or 132 Column Mode, Replace or Insert mode, Printer

Status, Hold indicator, Keyboard Lock indicator, and LED

indicators L1, L2, L3, and L4.

The Operating Mode setting can be toggled while the status is

displayed by pressing the M key.  Online terminal operation is

suspended and input is enabled on the status line whenever a

blinking appears in the first column position.  For example, to

toggle from the VT131 mode to the VT52 mode, first display the

status by entering Alt T and then press the M key until VT52 is

displayed on the status line.  Pressing the C key will cause the

column mode to be toggled between 80 and 132.  Pressing the K key

will cause the LOCK (Keyboard Lock) setting to be toggled.

To return to the online mode after displaying the status, enter

Alt T or Alt Esc.  If you exit using the Alt T key, the status

line will remain on the screen during online operation and will be

automatically updated if required.  If you exit from the status

line display using the Alt Esc key, the status line will be

cleared.

DEC VT131 Keyboard Functions

The following table defines the standard keyboard functions for

the Digital Equipment VT131 terminal emulation.  Refer to the VT52

Keyboard Function Table for keyboard functions in VT52

compatibility mode.

Keyboard		Function

Enter			CR, LF, or CR/LF

Tab			Tab

Right Arrow		Cursor Forward

Alt Ctrl Right Arrow	Scroll Right

Left Arrow		Cursor Backward

Alt Ctrl Left Arrow	Scroll Left

Up Arrow		Cursor Up

Alt Ctrl Up Arrow	Home Left

Down Arrow		Cursor Down

Alt Ctrl Down Arrow	Home Right

Home			Cursor Home

Backspace		Backspace or Delete

Shift Backspace	Delete or Backspace

Digital Equipment VT131      - 6 -             Terminal Emulation

PgUp			Clear

PgDn			Erase to End of Page

End			Erase to End of Line

Ctrl Break		Soft Reset

Ctrl NumLock		Stop Display

Shift PrtSc		Print Screen

Ctrl PrtSc		Auto/Print On/Off

Alt T			Display VT131 Status

Note:  In general, the PC's F1 key functions as the "Gold Key" on

the VT131 keyboard.

F1	PF1		F3	PF3

F2	PF2		F4	PF4

The following functions are entered on the numeric keypad except

for the comma (,) and minus (-) which are entered on the main

keyboard:

Alt 0     Keypad Function 0   Alt ,	  Keypad Function ,

Alt 1     Keypad Function 1   Alt PrtSc  Keypad Function ,

Alt 2     Keypad Function 2

Alt 3     Keypad Function 3   Alt -	  Keypad Function -

Alt 4     Keypad Function 4   Alt .	  Keypad Function .

Alt 5     Keypad Function 5

Alt 6     Keypad Function 6   Alt Enter  Keypad Function ENTER

Alt 7     Keypad Function 7   Alt +	  Keypad Function ENTER

Alt 8     Keypad Function 8

Alt 9     Keypad Function 9

Digital Equipment VT131      - 7 -             Terminal Emulation