ADDS Viewpoint

Introduction

The ADDS Viewpoint display terminal can be emulated using Softerm

by selecting ADDS Viewpoint for the terminal emulation filename

when generating a configuration in terminal setup.  Softerm

supports all the general capabilities of the Viewpoint including

the full 128 character set and 80 X 24 display, conversational

half or full duplex transmission, absolute cursor addressing,

cursor control and function keys, auxiliary port operation with a

printer including print transparent, erase functions, and tagged

data for visual attributes.

Features

The display format of the ADDS Viewpoint is 24 lines by 80

characters with the ability to display all 128 ASCII character

codes including 32 control codes in the Monitor Mode.  Softerm

supports all available keyboard functions and provides additional

key sequences for Erase, Erase to End of Line, and Erase to End of

Page.

Softerm supports all remote terminal commands and switch settings

with the following exceptions:

Switch selectable settings not supported by Softerm include Video

Character Display, Audible Key Tone, and Display Parity Error.

Softerm includes support for the Auxiliary Port if the terminal

configuration includes a printer.  If a Transparent Print ON

remote command is received, all subsequent data is sent to the

printer until a Transparent Print OFF command is received.  In the

transparent mode data is sent only to the printer and not

displayed.

The Auxiliary port may also be enabled by the Ctrl PrtSc keyboard

function which emulates the standard print capability of the

Viewpoint.  Refer to the description of printer pass through in

the introduction of this chapter for additional information.

Softerm supports the Set Attribute in conjunction with the Tag Bit

Set, and Tag Bit Reset commands and all visual attributes.  If the

current visual attribute is changed by the Set Attribute command,

Softerm redisplays all tagged character positions on the screen.

ADDS Viewpoint               - 1 -             Terminal Emulation

ADDS Viewpoint Keyboard Functions

The following table defines the standard keyboard functions for

the ADDS Viewpoint terminal emulation:

Keyboard		Function

Enter			CR, LF, or CR/LF

Right Arrow		Cursor Forward

Left Arrow		Cursor Back

Up Arrow		Cursor Up

Down Arrow		Cursor Down

Alt M			Monitor Mode On/Off

Backspace		BS or DEL

Shift Backspace	DEL or BS

Home			Home Cursor

PgUp 			Erase All

PgDn 			Erase to End of Page

End			Erase to End of Line

Ctrl NumLock		Stop Display

Ctrl PrtSc		AUX

F1	F1		Shift F1	Shift F1

F2	F2		Shift F2	Shift F2

F3	F3   		Shift F3	Shift F3

ADDS Viewpoint               - 2 -             Terminal Emulation