Lear Siegler ADM-2

Introduction

The Lear Siegler ADM-2 data display terminal can be emulated using

Softerm by selecting Lear Siegler ADM-2 for the terminal emulation

when generating a configuration in terminal setup.  Softerm

supports all the general capabilities of the Lear Siegler ADM-2

including the full 128 character set and 80 X 24 display, 16

special function keys, video attributes, cursor addressing, page,

field or line editing, cursor controls, conversational full or

half duplex transmission, block mode transmission, pre-set and

user selectable tabs, program mode, ADM-1 compatibility mode, and

auxiliary port control including formatted and free-form page

print.  The ADM-2 polling option is not supported.

Features

The display format of the Lear Siegler ADM-2 is 24 lines by 80

characters with the ability to display all 128 ASCII character

codes including 32 control codes in the Program Mode.  The Lear

Siegler also displays status information on the right of the

display screen which is simulated by Softerm on the 25th line.

Softerm supports all available keyboard functions.

Functional commands recognized and processed but ignored by

Softerm include Back Page and Forward Page.

After the last standard screen in the Terminal Options mode in

Terminal Setup, the following screen will be displayed:

���� Lear Siegler ADM-2 Emulation Options ���Ŀ

Operating Mode: Character

Page Edit Mode: No

ADM-1 Mode: No

The emulation option Operating Mode selects the transmission mode

of the terminal and may be selected as either Character or Block.

In Block mode, characters entered from the keyboard are stored in

a buffer which may be corrected and edited by the operator, and

then transmitted to the host when a SEND key is pressed.

The emulation option Page Edit Mode if set to Yes will cause

character insert and delete operations to affect the entire page

of data.  If this option is set to No, character insert and delete

operations affect only the current line.

The emulation option ADM-1 Mode if set to Yes enables the ADM-1

compatibility mode.

Lear Siegler ADM-2           - 1 -             Terminal Emulation

Operator Status Line Display

The Lear Siegler ADM-2 terminal emulation includes a status line

which may be displayed on the 25th line by pressing the Alt T key.

The status information displayed includes the current Row and

Column, and indicators for Conversation Mode CONV, Page Edit Mode

EDIT, Program Mode PROG, Protect Mode PROT, Write Protect Mode

WPRT, Keyboard Lock LOCK, ADM-1 Compatibility Mode ADM1, and

Status Indicators 1through S4.  The indicated mode is on if the

field is displayed in reverse video.

The Conversation mode setting can be toggled on or off while the

status is displayed by pressing the C key.  Online terminal

operation is suspended and input is enabled on the status line

whenever a blinking Right Arrow appears in the first column

position.  For example, to toggle the CONV mode on or off, first

display the status by entering Alt T and then press the C key.

Pressing the E key will cause the EDIT indicator for the Page Edit

mode to be toggled.  Pressing the W key will cause the WPRT

indicator for Write Protect mode to be toggled on or off.

Pressing the P key will cause the PROT indicator for Protect Mode

to be toggled on or off.  Pressing the K key will cause the LOCK

indicator for Keyboard Lock to be toggled on or off.  Pressing the

U key will cause the PROG indicator for Program Mode to be toggled

on or off.

To return to the online mode after displaying the status, enter

Alt T or Alt Esc.  If you exit using the Alt T key, the status

line will remain on the screen during online operation and will be

automatically updated if required.  If you exit from the status

line display using the Alt Esc key, the status line will be

cleared.

Lear Siegler ADM-2 Keyboard Functions

The following table defines the standard keyboard functions for

the Lear Siegler ADM-2 terminal emulation:

Keyboard		Function

Enter			CR, LF, or CR/LF

Shift Enter		NEW LINE

Alt Enter		SEND PAGE UNPROTECTED

Alt Shift Enter	SEND PAGE ALL

Ctrl Enter		SEND LINE UNPROTECTED

Alt Ctrl Enter		SEND MESSAGE UNPROTECTED

Ctrl Shift Enter	SEND LINE ALL

Alt Shift Ctrl Enter	SEND MESSAGE ALL

PgDn			PAGE ERASE (Spaces)

Ctrl PgDn		SHIFT PAGE ERASE (Nulls)

Ctrl J			LINE FEED

End			LINE ERASE (Spaces)

Ctrl End		SHIFT LINE ERASE (Nulls)

Lear Siegler ADM-2           - 2 -             Terminal Emulation

Del			CHAR DELETE

Alt Del		LINE DELETE

Ins			CHAR INSERT

Alt Ins		LINE INSERT

Right Arrow		Cursor Right

Left Arrow		Cursor Left

Up Arrow		Cursor Up

Down Arrow		Cursor Down

Alt Right Arrow	PAGE FWD

Alt Left Arrow		PAGE BACK

Shift PrtSc		PRINT

Alt Shift PrtSc	SHIFT PRINT

Ctrl Break		Soft Reset

Ctrl NumLock		Stop Display

Alt A			Send Answerback Message

Alt B			Break

Alt E			EOM

Alt S			SOM

Alt T			Display ADM-2 Status Line

Backspace		BS

Shift Backspace	RUB

Tab			TAB

Alt Tab		TAB SET

Shift Tab		BACK TAB

Home			HOME

PgUp			CLEAR SPACES (Unprotected)

Alt PgUp		CLEAR NUL (Unprotected)

Ctrl PgUp		SHIFT CLEAR SPACES (All)

Alt Ctrl PgUp		SHIFT CLEAR NUL (All)

F1	F1		Shift F1	Shift F1

F2	F2		Shift F2	Shift F2

F3	F3		Shift F3	Shift F3

F4	F4		Shift F4	Shift F4

F5	F5		Shift F5	Shift F5

F6	F6		Shift F6	Shift F6

F7	F7		Shift F7	Shift F7

F8	F8		Shift F8	Shift F8

F9	F9		Shift F9	Shift F9

F10	F10		Shift F10	Shift F10

Alt F1	F11		Alt Shift F1	Shift F11

Alt F2	F12		Alt Shift F2	Shift F12

Alt F3	F13		Alt Shift F3	Shift F13

Alt F4	F14		Alt Shift F4	Shift F14

Alt F5	F15		Alt Shift F5	Shift F15

Alt F6	F16		Alt Shift F6	Shift F16

Lear Siegler ADM-2           - 3 -             Terminal Emulation