Hazeltine 2000 Display Terminal

Introduction

The Hazeltine 2000 display terminal can be emulated using Softerm

by selecting Hazeltine 2000 for the terminal emulation when

generating a configuration from the Setup Options menu.  The

Hazeltine 2000 general capabilities supported by Softerm include

the full 128-character ASCII set; 74 X 27 display; Delete and

Insert character functions; Background and Foreground functions;

Delete and Insert line functions; all Cursor Positioning

functions; and Clear Screen and Foreground functions.

Features

The Hazeltine 2000 emulation can display true 74 X 27 text size by

using Softerm PC's extended text options.  The following is a list

of how the emulator reacts to different video systems and Softerm

PC's XTX files:

Monochrome - Softerm PC is not configured for Extended Text Screen

display will be 80 X 24.  The display is updated automatically by

cursor position.  the undisplayed part of the screen may be viewed

by pressing Alt PgUp or Alt PgDn to roll the display up and down.

EGA - Softerm PC is configured for EGA Extended Text True 74 X 27

display will appear.  The Alt PgUp and Alt PgDn keys will be

disabled.  Note: Foreground and Background Intensity attributes

may be lost or displayed as different colors when running with any

XTX graphics file.  This change will not effect any functionality

of the emulator.

CGA - Softerm PC is configured for CGA Extended Text Same as EGA.

Hercules - Softerm PC is configured for Hercules Extended Text

Same as EGA.

CGA, EGA, Hercules - a graphics card installed but Softerm PC is

not configured for Extended Text Same as Monochrome.

The basic mode of operation for the Hazeltine 2000 terminal

emulation is character-by-character when set for half or full

duplex.  In this case, data which is entered at the keyboard is

sent directly to the host.  In the Batch mode of operation, the

terminal becomes a buffered editing terminal, allowing the

operator to enter data with characters transmitted to the computer

only after the SEND key is pressed.  Thus, before any data is

actually transmitted, the operator may review it for errors,

omissions and misplaced data, and take corrective action.  After

making the corrections, the operator may initiate a serial

buffered transmission of the data by pressing the SEND key.

The Auto LF-CR setup switch option corresponds to the Softerm LF

After CR option, and the Wraparound option corresponds to the

Softerm Auto Line Wrap option.

Setup switch options available on the Hazeltine 2000 and not

supported by Softerm include U/L CASE-UP and STD VIDEO-REV.

Hazeltine 2000               - 1 -             Terminal Emulation

Softerm supports only upper and lower case operation and standard

video display of light characters on a dark background.

The remote command features of the Hazeltine 2000 provide the user

with the capability to control the terminal via the host computer

software.  For the terminal to execute a remote command, the

command code must be preceded in some cases by a lead-in code

which is either a tilde ($7E) character or an ESC ($1B) character.

Softerm includes support for all remote functions.

Hazeltine 2000 Emulation Options

After the last standard screen in the Terminal Options mode in

Terminal Setup, the following screen will be displayed:

����� Hazeltine 2000 Emulation Options ���Ŀ

End of Message Character: None

Batch Mode: No

Lead-In Character: ESC

The emulation option End of Message Character may be toggled to

None, CR ($0DH), EOT ($04H), or ETX ($03H) and selects the

character which will be inserted at the end of read cursor

address, function, batch, line, and PAGE transmissions to signal

the end of message (EOM).

The emulation option Batch Mode may be toggled to No or Yes.  If

set to Yes, this option will override the half or full duplex

character mode and select the batch mode for keyboard entry and

transmission.  In batch mode, data entered on the display can be

edited before the contents of the display is transmitted.

The emulation option Lead-In Character may be toggled to Tilde

($7EH) or ESC ($1BH) and specifies the lead-in character for

remote commands.

Hazeltine 2000 Keyboard Functions

The following table defines the standard keyboard functions for

the Hazeltine 2000 terminal emulation.

Keyboard		Function

Enter			CR-LF

Alt Enter		XMIT

Ctrl Enter		XMIT Line

Shift Enter		XMIT Page

Home			Home Cursor

Ins			Insert Character

Alt Ins		Insert Line

Del			Delete Character

Alt Del		Delete Line

Shift PrtSc		PRINT

Ctrl PrtSc		Online Print On/Off

BackSpc		Backspace

Shift BackSpc		Delete

Tab			Tab

Hazeltine 2000               - 2 -             Terminal Emulation

Shift Tab		Backtab

PgUp			Clear Screen

PgDn			Clear Foreground

Right Arrow		Cursor Right

Left Arrow		Cursor Left

Up Arrow		Cursor Up

Down Arrow		Cursor Down

The following two key combinations are in effect only if Softerm

PC is configured with no Extended Text (XTX) files:

Alt PgUp		Roll Page Up

Alt PgDn		Roll Page Down

Hazeltine 2000               - 3 -             Terminal Emulation