AutoCAD® LT Release 2 "Read Me"

April 20, 1995

This document contains last-minute changes and instructions that could not be included in the printed manuals.

Table of Contents

1.	Configuring Hewlett-Packard HP-GL Plotters

2.	General Information

CalComp Plotters

Canon Bubble-Jet Printers

Hewlett-Packard Printers and Plotters

Houston Instruments DMP 60

Microsoft® Windows for Workgroups™ Printing

Dithering Output

DWG Associativity

LT Environment Variables

Boundary Hatch Editing

3.	AutoCAD LT 2 Tips and Tricks

4.	Notes for Generic CADD Users

5.	AutoCAD LT Questions and Answers

_________________________________________________________

1.	Configuring Hewlett-Packard HP-GL Plotters

AutoCAD LT supports these Hewlett-Packard plotters as HP-GL plotters:

·	The 7586B and 7596A roll-fed plotters

·	The Hewlett-Packard models 7220, 7470, 7475, 7550, 7580, 7585, 7586, ColorPro,	 DraftPro, DraftPro DXL and EXL, and DraftMaster (pen plotters)

AutoCAD LT supports Hewlett-Packard plotters that are configured for 9600 baud (2400 baud for the model 7220), 7 data bits, 1 stop bit, Xon/Xoff, and even parity, with an RS-232C serial I/O port.

The following diagram shows the recommended cabling when your plotter is configured for a serial port. Other configurations work, too, but this configuration needs only three wires and works with any Hewlett-Packard plotter. When you have a choice, attach the cable to the "Computer" jack on the plotter, rather than to the jack marked "Terminal."

2_______________________2

3_______________________3

4__________   __________4

5__________| |__________5

7_______________________7

6__________   __________6

20_________| |__________20

Note:	Strap some pins together in the connector at one end of the cable without running them to the other end.

When configured for parallel ports and plotters that support hard clip limits, make sure the Ask device for hard clip limits toggle in the HPGL Driver Setup dialog box is turned off.

2.	General Information

CalComp Plotters

Paper Size Configuration

When your system printer is configured for CalComp 907 printer in the Windows Control Panel and is selected as the system printer in AutoCAD LT, the paper-size listing in the Plot Configuration/Print Setup dialog box remains empty.

To select the correct paper size

1.	From the Program Manager, double-click the Control Panel icon.

2.	Double-click the Printers icon.

3.	From the Printers dialog box, choose Setup.

4.	Select the paper size in the Paper Size list box.

Printer Driver

When the CalComp 907 printer is configured as the system printer in the Windows Control Panel, choosing the Options button in the Print Setup dialog box fails to display the CalComp Printer dialog box.

To correct this problem, use the CalComp printer driver Version 5.2 or above.

CalComp Plotters

When your system is configured for the CalComp 907 driver, and Plot to File is selected in the Plot Configuration dialog box, a 0-byte plot file is created, and the plot file is sent to the plotter.

To correct this problem

1.	From the Program Manager, double-click on the Control Panel.

2.	Double-click on the Printers icon.

3.	From the Printers dialog box, choose Connect.

4.	From the Ports list box, select FILE. Then choose OK.

5.	Choose Close until all dialog boxes are closed.

6.	In the AutoCAD LT Plot Configuration dialog box, make sure the Plot to File option is not checked.

7.	When you choose OK to plot, enter a file name in the Windows Print to File dialog box.

Canon Bubble-Jet  Printers

To plot gray color when your system printer is configured as a Canon Bubble-Jet printer

1.	From the Program Manager, double-click the Control Panel icon.

2.	Double-click the Printers icon.

3.	From the Printers dialog box, choose Setup.

4.	Under Print Quality, select High Quality in the list box, and make sure that Enhanced Black is selected.

5.	Under Resolution, select 360 x 360 dpi.

6.	From the Setup dialog box, choose Options.

7.	Under Halftone, select Diffusion from the list box.

Hewlett-Packard Printers and Plotters

Hewlett-Packard DeskJet 550c

When your system is configured for the Hewlett-Packard DeskJet printer 550c, and Plot to File is selected in the Plot Configuration dialog box, the following error message is displayed after the file is regenerated:

"Invalid data. Aborting print job."

The file is created with 0 bytes and cannot be printed.

To correct this problem

1.	From the Program Manager, double-click on the Control Panel.

2.	Double-click on the Printers icon.

3.	From the Printers dialog box, choose Connect.

4.	From the Ports list box, select FILE. Then choose OK.

5.	Choose Close until all dialog boxes are closed.

6.	In the AutoCAD LT Plot Configuration dialog box, make sure the Plot to File option is not checked.

7.	When you choose OK to plot, enter a file name in the Windows Print to File dialog box.

Hewlett-Packard LaserJet 4

To plot dithered output correctly when the Hewlett-Packard LaserJet 4 is configured as the system printer

1.	From the Program Manager, double-click the Control Panel icon.

2.	Double-click the Printers icon.

3.	From the Printers dialog box, choose Setup.

4.	From the Setup dialog box, choose Options.

5.	In the Options dialog box Graphics area, choose HP-GL/2 from the Graphics Mode

list box.

HP-GL Pen Speed and Optimization

To control a pen's speed and motion optimization for HP-GL plotters

1.	Create a plot configuration file, as described in "Reusing a Plot

Configuration" in chapter 16, "Plotting Your Drawing," in the AutoCAD LT User's Guide.

2.	Using a text editor, edit the AutoCAD LT defaults that are stored in the plot

configuration (.pcp) file.

·	To change the pen speed, modify the value next to the variable PEN_SPEED. For valid pen speed values, check the documentation that comes with your plotter.

·	To change the pen-motion optimization, modify the value next to the variable OPTIMIZE_LEVEL according to the following list:

0	No optimization (avoid this option when printing circles)

1	Adds endpoint swap

2 	Adds endpoint swap and adds pen sorting

3	Adds endpoint swap, pen sorting, and limited-motion optimization

4	Adds endpoint swap, pen sorting, and full-motion optimization

5	Adds endpoint swap, pen sorting, full-motion optimization, and elimination of overlapping horizontal or vertical vectors

6	Adds endpoint swap, pen sorting, full-motion optimization, elimination of overlapping horizontal or vertical vectors, and elimination of diagonal vectors

3.	From the File Menu in AutoCAD LT, choose Print/Plot.

4.	In the Plot Configuration dialog box, choose Print/Plot Setup and Default 			Configuration.

5.	In the Print Setup dialog box, select Hewlett-Packard as your device. Then choose 			Get Defaults From File.

6.	In the Obtain From File dialog box, enter the name of the PCP file that has the 			modified values.

When you close the Plot Configuration dialog box to plot a file, AutoCAD LT uses the correct pen speed and motion optimization values.

Long-Axis Plots

Long-axis plotting is not supported in AutoCAD LT 2.

Houston Instruments DMP 60

When your system is configured for the Houston Instruments DMP 60 and Plot to File is selected in the Plot Configuration dialog box, the file is created with 0 bytes and cannot be printed.

To correct this problem

1.	From the Program Manager, double-click on the Control Panel.

2.	Double-click on the Printers icon.

3.	From the Printers dialog box, choose Connect.

4.	From the Ports list box, select FILE. Then choose OK.

5.	Choose Close until all dialog boxes are closed.

6.	In the AutoCAD LT Plot Configuration dialog box, make sure the Plot to File option is not checked.

7.	When you choose OK to plot, enter a file name in the Windows Print to File dialog box.

Microsoft Windows for Workgroups Printing

To plot to shared printers configured as HPGL plotters using Windows for Workgroups

1.	Follow the Windows for Workgroups instructions for sharing a printer.

2.	Follow the Windows for Workgroups instructions for connecting a network printer (for 		example, connect to LPT1).

Note:  No printer drivers need to be installed in Windows.  If you are

connecting with no printers installed, the Print Manager displays the

following message:

"There is currently no printer connected to <port>.

Do you want to install one?"

Respond with "No".

3.	Begin an AutoCAD LT session.

4.	In AutoCAD LT, open a drawing.

5.	From the File menu, choose Print/Plot.

6.	In the Plot Configuration dialog box, choose Print/Plot Setup & Default Selection.

7.	From the  Print/Plot Setup & Default Selection list box, select Hewlett-Packard (HP-				GL). Then choose Print/Plot Setup.

8.	In the HPGL Driver Setup dialog box, select a model. Then select the same port 				used in step 1 above (for example, LPT1).

9.	Choose OK to close all dialog boxes and plot the drawing.

Dithering Output

Some Windows printer drivers do not support dithering. If dithering causes some objects to disappear when printed or plotted on your device, you should disable dithering in the Plot Configuration dialog box.

DWG Associativity

With DWG associativity, you can open a DWG file by double-clicking the file icon in the Windows File Manager. During installation, you can change the association of DWG files that were previously associated with AutoCAD or AutoCAD VQ to be associated with AutoCAD LT. The new association takes effect only after Windows is restarted.

AutoCAD LT Environment Variables

Three settings have been introduced to track the last Windows system printer selected for use with AutoCAD LT. The settings appear in the [AutoCAD LT General] section of the aclt.ini file: PrevPrnName, which stores the name of the previously selected printer, PrevPort, which stores the previously selected port, and PrevPrnDRV, which stores the previously selected printer driver. Because these variables are updated automatically from the Plot Configuration dialog box when the system printer changes, you do not need to change them manually.

Boundary Hatch Editing

Editing Multiple Pick Points

Editing associative hatch patterns containing multiple pick points results in updating only the area encompassing the first pick point. To update associative hatch patterns that have multiple pick points, use the BHATCH command multiple times, once for each pick point, rather than once for multiple pick points.

Canceling an Associative Hatch Edit

Hatch associativity may be lost if you choose Cancel from the Modify Associative Hatch dialog box. To correct this problem, use the UNDO command to restore associativity to the geometry.

3.	AutoCAD LT 2 Tips and Tricks

How can I customize my icons for the Toolbox or Toolbar?

You need a Windows resource editor, such as the Borland Resource Kit or AppStudio from Microsoft Visual C++. With these editors, you can add and modify the bitmaps in the aclttb16.dll and aclttb24.dll files. (Be sure to save the original files first.)

Why does Norton Desktop for Windows and Central Point's PC Tools for Windows give the following message when files are dragged and dropped into an AutoCAD LT session?

"No association exists for file."

These applications do not support drag-and-drop for AutoCAD LT. You must use the Windows File Manager. Creating associations in these File Managers has no effect on the drag-and-drop capabilities of their drive windows.

After customizing the buttons in the Toolbar (or Toolbox), why does the following error message occur when clicking on an icon button?

"Unknown command. Press F1 key for help."

The command string associated with that button is not a valid AutoCAD LT command. To correct this problem, invoke the Toolbar (or Toolbox) Customization dialog box by right-clicking on the icon in question. Correct the command string displayed in the edit box at the bottom of the dialog.

I customized my Toolbar (or Toolbox) to add the Cancel button, and now it won't work. What can I do?

The Cancel button must have the characters "\3\3;" instead of the word "Cancel" on the command line. For information on valid command names, see the AutoCAD LT User's Guide or online Help.

4.	Notes for Generic CADD Users

A separate file conversion utility is available for Generic CADD users to convert existing GCD files into DWG format. The instructions for this routine are contained on the diskette in the file readme.cvt. This diskette is included only for Generic CADD users who have migrated to AutoCAD LT. If you need this utility, call Autodesk Customer Service.

5.	AutoCAD LT Questions and Answers

What is AutoCAD LT Release 2 for Windows?

AutoCAD LT 2.0 is the new and improved version of the world's leading, low-cost, two-dimensional professional design and drafting program. Its easy-to-use features and full upward compatibility with AutoCAD make it the best program to use whether making a transition from paper-based drafting to CAD or working with AutoCAD drawings at home or in the field.

What are some of the new features of Release 2?

Quick and Custom Setup:  Quick setup is a routine for setting drawing units, limits, and visual drawing aids. Custom Setup provides more control of the above, plus it allows users to choose from a set of predefined borders and industry-standard title blocks. Every time a new drawing is started, AutoCAD LT automatically scales system settings based on what is being drawn.

True Associative Hatching:  You need only point to an enclosed area on the screen and hatch it with the desired hatch pattern. If boundaries of the hatched area are changed, the hatching is automatically updated. You don't need to waste time selecting the lines and arcs that define the hatch boundary.

Object modification with dialog boxes: A collection of dialog boxes for modifying the properties, sizes and attributes of lines, circles, arcs, points, polylines, text, blocks, attributes and solids. These easy-to-use dialog boxes make it fast and easy to edit the properties of objects.

ToolTips:  The command name for each toolbar and toolbox button is displayed when you point to the icon. Now, with ToolTips you can move the mouse pointer over the button and the name of the command will appear. This feature works equally well on fixed or floating toolboxes.

Online Orientation:  The new online Orientation explains the differences between CAD and paper-based drafting, and is designed to help the paper drafter or first-time CAD user make a smooth transition to AutoCAD LT. The Orientation describes the common drafting tasks in CAD and its advantages over paper-based drafting.

Direct distance entry:  You need only move the mouse in the direction that you want the line to go, enter the actual distance directly from the keyboard, and the line is drawn. The popular direct distance entry feature from Generic CADD is now available in AutoCAD LT. No longer is there a need to calculate exact coordinates; you can specify the length of lines as accurately as needed.

What are the key benefits of AutoCAD LT Release 2 to a new AutoCAD LT user?

Learnability:  The time required to learn a new CAD product has often been an obstacle preventing designers and drafters from switching to CAD. AutoCAD LT Release 2 introduces new drawing setup features: ToolTips, an online Orientation, and cue cards, which are designed specifically to get new users up to speed quickly.

Ease of Use:  Modifying the properties of objects from dialog boxes, hatching updating as geometry is changed, and entering relative distances directly are just a few of the ways in which AutoCAD LT Release 2 has made it easier to design and draft.

Compatibility:  AutoCAD LT Release 2 uses the same command syntax as AutoCAD. Also, every file created in AutoCAD LT Release 2 is upwardly compatible with AutoCAD, so there is no loss in drawing investment.

Will my AutoCAD LT drawings work with AutoCAD Release 13?

Yes, AutoCAD LT drawings can be opened in AutoCAD Release 13 without any problems. No special conversions are necessary. All objects that were created in AutoCAD LT can be edited or modified in AutoCAD Release 13. AutoCAD Release 13 uses a different file format than AutoCAD LT, but it has the ability to read drawings from AutoCAD LT.

Can drawings created or modified in AutoCAD Release 13 be used in AutoCAD LT?

The new Release 13 drawing file format cannot be read by AutoCAD LT. Drawings created or modified by AutoCAD Release 13 must first be saved using the new SAVEASR12 command before being used by AutoCAD LT. This command saves the drawing in a file format that is compatible with both AutoCAD LT and AutoCAD Release 11/Release 12; there is no separate SAVEASLT command. Because some objects are transformed, be sure to assign a different drawing name when using the SAVEASR12 command to avoid overwriting the current drawing. If a drawing from Release 13 is saved with the SAVE, SAVEAS, WBLOCK, QUIT or END commands, it cannot be read by AutoCAD LT.

What if I save a drawing from AutoCAD Release 13 but later want to use it with AutoCAD LT?

If a drawing is saved in the AutoCAD Release 13 drawing file format, it can easily be converted back to the AutoCAD LT drawing file format by opening the drawing again in AutoCAD Release 13 and then using the SAVEASR12 command. This will save the drawing in a file format that is compatible with AutoCAD LT.

Will any data be lost by converting my Release 13 drawing with the SAVEASR12 command?

There are some objects from AutoCAD Release 13 that are not supported by AutoCAD LT. As part of the SAVEASR12 command, these objects are either ignored or converted to compatible objects for use in AutoCAD LT. All other objects remain as they are.

What are the key benefits of AutoCAD LT Release 2 to an existing AutoCAD LT user?

Ease of use:  The addition of new features, such as true associative hatching, divide, and measure plus the use of dialog boxes for intuitive object modification, makes this version of AutoCAD LT easier to use.

Enhanced documentation:  A new task-oriented User's Guide makes it easier to find information. Details on commands and customization options plus a quick reference card, comprehensive online Help, and cue cards give users many places to turn for information.

Windows for Workgroups printing support:  AutoCAD LT Release 2 supports networked printing using Microsoft Windows for Workgroups.

Why should a Generic CADD user switch to AutoCAD LT Release 2?

AutoCAD LT Release 2 provides Generic CADD users with the most popular features from Generic CADD, including direct distance and tracking. In addition, AutoCAD LT Release 2 provides Generic CADD users with productivity enhancements that are not available in a DOS product, such as Clipboard support and OLE.

Can users upgrade from an existing version of AutoCAD LT?

Yes, all AutoCAD LT Release 1 customers can upgrade to AutoCAD LT Release 2 directly from Autodesk. Interested customers should call 1-800-435-7771.

How can users upgrade their copy of Generic CADD or AutoSketch to AutoCAD LT Release 2?

Transfers from Generic CADD and AutoSketch are available directly from Autodesk. Interested customers should call 1-800-435-7771.

Is there a special upgrade pricing policy for moving from AutoCAD LT Release 2 to AutoCAD Release 13?

No, not at this time.

Will there be a future DOS or Macintosh version of AutoCAD LT Release 2?

No, there are currently no plans to develop a DOS or Macintosh version of AutoCAD LT Release 2.

Is AutoCAD LT Release 2 compatible with Microsoft Windows for Workgroups or Windows NT?

Yes, however, AutoCAD LT Release 2 does not support any Windows for Workgroups or Windows NT-unique functionality. Microsoft claims that all Windows 3.1 applications will run as is with Windows for Workgroups and Windows NT, and our testing has confirmed that AutoCAD LT Release 2 is compatible with these operating environments.

Will AutoCAD LT Release 2 work with Microsoft Windows 95?

AutoCAD LT Release 2 works with the preliminary beta copies of Windows 95. However, we cannot at this time guarantee that AutoCAD LT Release 2 is compatible with the final release version of Windows 95.

Is AutoCAD LT Release 2 compatible with IBM OS/2?

Yes. However, Autodesk does not support OS/2, nor has Autodesk conducted exhaustive testing of AutoCAD LT Release 2 under all versions of OS/2. It is recommended that customers use Microsoft Windows with AutoCAD LT.

Is a multiple-copy network license available for AutoCAD LT Release 2?

No, only single-use copies of AutoCAD LT Release 2 are available.

Will existing AutoLISP routines work with AutoCAD LT Release 2?

No, AutoCAD LT Release 2 does not support AutoLISP.

Does AutoCAD LT Release 2 support the use of digitizers?

No, AutoCAD LT Release 2 does not support the use of digitizers as absolute pointing devices (that is, AutoCAD LT Release 2 does not have Wintab support). A digitizer can be used as a relative pointing device, as with other Windows applications, assuming the proper drivers are installed.

Will AutoCAD LT Release 2 work with the third-party symbol libraries that were available for Release 1?

Yes. There have been no changes in AutoCAD LT Release 2 that should affect the use or operation of any third-party symbol libraries. To be certain, check with your third-party developer.