Computer Based PBX                     Courtesy of the Jolly Roger

(Originally an Apple ][ file for forgive the upper case!)

TO GET A BETTER UNDERSTANDING OF WHAT A PBX CAN DO, HERE ARE A FEW BASIC

FUNDAMENTALS.THE MODERN PBX IS A COMBINED COMPUTER,MASS STORAGE DEVICE,

AND OF COURSE A SWITCHING SYSTEM THAT CAN:

[1] PRODUCE ITEMIZED,AUTOMATED BILLING PROCEDURES,TO ALLOW THE

IDENTIFICATION AND MANAGEMENT OF TOLL CALLS. [HAHAHA]

[2] COMBINE DAYTIME VOICE GRADE COMMUNICATION CIRCUITS INTO

WIDEBAND DATA CHANNELS FOR NIGHT TIME HIGH SPEED DATA

TRANSFERS.

[3] HANDLES ELECTRONIC MAIL [ INCLUDING OFFICE MEMOS ].

[4] COMBINE VOICE CHANNELS INTO A WIDEBAND AUDIO/VISUAL

CONFERENCE CIRCUIT,WITH THE ABILITY TO XFER AND

CAPTURE SLIDES,FLIPCHARTS,PICTURES OF ANY KIND.

BOTH THE EXTERNAL AND INTERNAL CALLING CAPACITY OF THE PBX SYSTEM MUST BE

CAREFULLY CONSIDERED BECAUSE MANY BUSINESS OPERATIONS RUN A VERY HIGH RATIO

OF INTERNAL STATION TO STATION DIALING AND A LOW CAPACITY SYSTEM WILL NOT

HANDLE THE REQUESTED TRAFFIC LOAD.

A CRITICAL FACTOR IS THE NUMBER OF TRUNKS AND THE CENTRAL OFFICE FACILITIES

THAT ARE USED FOR OUTSIDE CONNECTIONS.ANOTHER IS THE NUMBER OF JUNCTIONS OR

[LINKS] THAT MAKE UP THE INTERNAL CALLING PATHS.

TO UNDERSTAND THE SERVICES AVAILABLE ON A TYPICAL COMPUTER RUN PBX IT IS

NECESSARY TO INTRODUCE THE SUBJECT OF TIME DIVISION SWITCHING.IN A TIME

DIVISION SWITCHING NETWORK ALL CONNECTIONS ARE MADE VIA A SINGLE COMMON BUS

CALLED (OF COURSE) A 'TIME-DIVISION BUS'.EVERY LINE TRUNK THAT REQUIRES A

CONNECTION WITH ANOTHER IS PROVIDED WITH A PORT CIRCUIT.ALL PORT CIRCUITS

HAVE ACCESS TO THE TIME DIVISION BUS THROUGH A TIME DIVISION SWITCH.

[WHEN TWO PORTS REQUKRE CONNECTION,THEIR TIME DIVISION SWITCHES OPERATE AT

A VERY HIGH FREQUENCY (16,000 TIMES PER SECOND).THIS TECHNIQUE,WHICH IS

CALLED 'SPEECH SAMPLING',ALLOWS MANY SIMULTANEOUS CONNECTIONS OVER THE SAME

TIME DIVKSION BUS.EACH CONNECTION IS ASSIGNED A TIME INTERVAL,THE 'TIME SLOT'

,AND THE NUMBER OF TIME SLOTS IDENTIFIES THE NUMBER OF SIMULTANEOUS CONNECT-

IONS AMONG PORTS.]

THE NEXT CRITICAL ITEM IS CIRCUIT PACKS.THE SYSTEM ELEMENTS THAT WE WILL BE

DESCRIBING IN FUTURE TUTORIALS [LINES/TRUNKS/SWITCHES,MEMORY AND CONTROL] ARE

CONTAINED ON PLUG IN CIRCUIT PACKS.EACH LINE CIRCUIT PACK CONTAINS A NUMBER

OF LINES,IN EXAMPLE,FOUR.BUT THE ASSIGNMENT OF STATION NUMBERS TO ACTUAL

PHONE LINE CIRCUITS IS FLEXIBLE.

THE SYSTEM MEMORY IS CONTAINED IN CIRCUIT PACKS WHICH PROVIDE THE CALL

PROCESSING FUNCTIONS.THE CIRCUIT PACKS ARE HELD IN SMALL FRAMES CALLED

'CARRIERS'.WITHIN EACH CARRIER,THE CIRCUIT PACKS ARE PLUGGED INTO POSITIONS:

THE 'SLOTS'.EVERY CIRCUIT CAN BE ADDRESSED BY,SAY A FIVE DIGIT NUMBER WHICH

TELLS ITS LOCATION BY CARRIER-SLOT-CIRCUIT.... [STARTING TO GET THE IDEA?]

THERE CAN BE THREE TYPES OF CARRIERS IN A MODERN PBX SYSTEM:

O LINE CARRIERS

O TRUNK CARRIERS

O CONTROL CARRIERS

THE LINE CARRIERS CONTAIN STATION LINES.IN A.T.& T.'S "DIMENSION" MODEL,FOR

EXAMPLE,A TOTAL OF 52 TO 64 LINES ARE PROVIDED.THE TRUNK CARRIERS CONTAIN

SLOTS FOR 16 TRUNK CIRCUIT PACKS.THE CONTROL CARRIER INCLUDES PROCESSOR,

MEMORY,CONTROL CIRCUITRY,DATA CHANNELS FOR ATTENDANT CONSOLE CONTROL AND

TRAFFIC MEASUREMENT OUTPUTS.

PBX SYSTEMS WILL DIRECTLY REFLECT THE TYPES OF SERVICES OFFERED AT THE C.O.

O CCSA

O CCIS

O PICTUREPHONES [SOONER THAN YOU THINK MY PHRIENDS]

COMMON CONTROL SWITCHING ARRANGEMENTS ( CCSA ) PERMIT ANY UNRESTRICTED TELE-

PHONE STATION TO CALL ANY OTHET INTERNAL OR EXTERNAL SYSTEM STATION BY USING

THE STANDARD SEVEN DIGIT NUMBER.ALTERNATE ROUTING IS A FEATURE OF CCSA SERVICE

THE INTERFACILITY,ALTERNATE ROUTED CALLING PATHS ARE ACCOMPLISHED AT THE TELE-

PHONE COMPANY CENTRAL OFFICE LEVEL,NOT AT THE PBX LEVEL.

A SYSTEM OF INTEREST TO LARGE SCALE TELEPHONE USERS IS COMMON CHANNEL INTER-

OFFICE SIGNALLING (CCIS).TYPICALLY,THIS TECHNIQUE EMPLOYS COMMON CHANNELS TO

CARRY ALL INTERFACILITY SIGNALLING INSTRUCTIONS: DIAL PULSES,ON HOOK (IDLE),

OFF HOOK (BUSY),AND SO ON,BETWEEN TWO SWITCHING CENTERS. [ GETTING WARM ].

CCIS REPLACES OLDER METHODS OF INTEROFFICE SIGNALLING SUCH AS 'IN BAND' AND

'OUT OF BAND' TECHNIQUES. BY THE WAY,REAL PHREAKS ARE SELLING THEIR BOXES TO

IDIOTS WHO STILL THINK THE'RE WORTH ALOT...THE FORMER (IN BAND) TRANSMITS

SIGNALLING DATA WITHIN THE NORMAL CONVERSATION BANDWIDTH.IT'S SHORTCOMING IS

THAT FALSE INFORMATION MAY BE TRANSMITTED DUE TO UNIQUE TONE OR NOISE

COMBINATIONS SET UP IN THE TALKING PATH. [THIS IS THE OFFICIAL REASONING].

OUT OF BAND SIGNALLING TECHNIQUES PLACED THE INTEROFFICE DATA IN SPECIAL

CHANNELS,GENERALLY ADJACENT TO AND IMMEDIATELY ABOVE THE VOICE PATH.TO PRE-

SERVE INTERCHANNEL INTEGRITY,OUT OF BAND SIGNALLING REQUIRES VERY EFFECIENT

FILTERING OR GREATER 'BAND GUARD' SEPERATION BETWEEN CHANNELS.

Brought to you in the Cookbook IV courtesy of Mondain!!!!!!!!!!!!