ADAPTEC CONFIDENTIAL

Thank you for being an ALPHA tester of Adaptec EZ-SCSI 4.0.  Please keep in mind that this is an unreleased product covered by our mutual non-disclosure agreement.  We welcome your feedback.

- The Adaptec EZ-SCSI Team

PRODUCT DESCRIPTION

Adaptec EZ-SCSI 4.0 provides complete SCSI software support for DOS, Windows 3.1/3.11, Windows 95, and Windows NT 3.51 (Windows NT 3.50 and earlier are not supported).  If you are running Windows, simply run SETUP.EXE from the root directory of the EZ-SCSI CD-ROM.  The installer will automatically determine which operating system you are running, and install the correct applications (including new 32-Bit applications).  If you are running DOS, please run DOSINST.BAT from the DISK2 subdirectory of your EZ-SCSI CD-ROM.

IMPORTANT:  If you are testing with Windows 95, please use build 450 or later.  If you are testing with Windows NT 3.51, please use build xxx or later.

SCSI TAPE BACKUP SUPPORT

Adaptec EZ-SCSI 4.0 includes a new Windows 95 32-bit tape backup application for SCSI tape drives.  This is the same software shipped with Windows 95, but SCSI support has been added.  This has been a joint effort of Colorado Memory Systems, the makers of the Windows 95 tape backup app, and Adaptec.

As part of this joint effort, Adaptec is writing the SCSI tape driver for CMS Backup.  Currently, we only support Tandberg tape drives with partial support for HP DATs.  Between ALPHA and BETA, we will be working on supporting additional drives.

The tape backup application has not yet been fully integrated into our installer.  When you install Adaptec EZ-SCSI 4.0 for the first time, the backup software will NOT work.  Before you run the CMS Backup, please run the app CMSREG.EXE which you can find in the CMSREG subdirectory of your EZ-SCSI CD-ROM.

SUBMITTING BUG REPORTS

Before submitting a bug report, please review this document to see if we are already aware of the bug.  There are three ways to submit a bug report:

1.)	If you are an Adaptec employee, please submit the bug via DCS.

2.)	If you are an external ALPHA site, please send us a detailed E-Mail describing the problem, and how to reproduce it.  You can reach us at:

ezscsi@corp.adaptec.com

From Compuserve, you would E-Mail:

INTERNET:ezscsi@corp.adaptec.com

3.)	Fax detailed bug information to:

Attn:  Adaptec EZ-SCSI Team

(408) 957-7223

OBSOLETE PRODUCTS

The following applications are no longer part of the standard Adaptec EZ-SCSI:

Arcada Backup Basics

MSCDEX.EXE

WIN1520.EXE*

W1530CFG.EXE*

VHDFMT.EXE *

*  These files are in the \OBSOLETE subdirectory of the EZ-SCSI CD-ROM.

MISSING PRODUCTS

This ALPHA build of Adaptec EZ-SCSI 4.0 is missing a few components:

- Windows 95/NT CD-R Lite software.  Allows you to copy files to your CD-R drive.

- Windows 95 Uninstaller.

- 32-Bit SCSIBench.  The 16-bit version will be recompiled for Windows 95/NT for a future build.  Will look identical to 16-Bit version.

- Windows 3.1/3.11 32-Bit Disk Access for the AIC-6360/6370.

- On-line help for many applications.

- Improved bitmap graphics.

Most of these will be made available prior to going to BETA.

SPECIAL NOTE:  CD COPIER

The “CD Copier” application (for Windows 95/NT) allows you to copy CDs from your source CD-ROM drive to your destination CD-Recorder drive.  If you are familiar with CD-Recorder technology, you already know that once you start writing a track, you cannot stop until that track has been completely written.  The write process must be uninterrupted otherwise the blank CD will be wasted.

Most users will place their CD-ROM drive and CD-Recorder on the same host adapter.  That is how we test it here at Adaptec.  In order achieve the performance that we desire, it is important that your CD-ROM drive NOT have excessive SCSI bus utilization.  Fortunately, most SCSI CD-ROM drives get on and off the bus very quickly and support SCSI disconnect.  Unfortunately, a few do not and this can cause buffer underrun conditions.

How do you test for this?  First off, make sure that you have SCSI disconnection enabled at all times.  Windows 95 allows you to enable or disable it from the Device Manager.  Please leave it enabled.  Also, many SCSI host adapters allow you to change the settings via their configuration software (e.g. SCSISelect).  These will ship with disconnection enabled by default.  If you disabled disconnection, please turn disconnect back on before trying this software.   If you have a SCSI disk drive also hooked up to the same host adapter as the CD-ROM drive and CD-Recorder drive, you’re ready to run our test.  Run Adaptec SCSIBench and do a 64K sequential read benchmark of your disk drive only.  You’ll see a number appear, typically between 1 MByte/sec and 7 Mbytes/sec.  Now, insert a data CD into your CD-ROM drive.  Add it to your benchmark by clicking on its device name.  At this point, both devices are part of the benchmark.  If you have a 3x CD-ROM drive, you will see that your CD-ROM drive is sustaining about 450 Kbytes/sec.  If you see that your disk drive’s throuput has dropped dramatically down to about the same value (e.g. 450), then you probably have a CD-ROM drive that has excessive bus utilization.  If you encounter this, you may need to set our CD Copier software for single speed copies (e.g. 1x).  We do not see this with many CD-ROM drives.  NEC SCSI drives, however, appear to have this problem.  We have also been unable to copy audio CDs from NEC CD-ROM drives.  We’re not sure if this problem is still present with their new 6x drive.  Other drives such as the Toshiba 3501 and Plextor 4x drive work  fine.

If your CD-ROM drive supports reading audio tracks across the SCSI bus, we will allow you to copy audio CDs.  We are getting a background hiss when copying from the Sony CDU-561 CD-ROM drive.  We will be working with Sony to determine if there is workaround or not.

If you have a Philips 521 CD-R drive, due to its small buffer, we recommend that you do not open run other applications while the copy is in process.

LIST OF CHANGES / KNOWN BUGS

This section documents the list of changes made since EZ-SCSI 3.12.  Also, known bugs are also listed here.

| Filename(s): | ASPI2DOS.SYS |
| --- | --- |
| Driver/App: | AIC-6260/6360/6370 ASPI manager for DOS |
| NOTES: | No changes. Still need to add 6370 Plug and Play support (e.g. AVA-1502P) |

| Filename(s): | ASPI4DOS.SYS |
| --- | --- |
| Driver/App: | AHA-1540/1542/1640 ASPI manager for DOS |
| NOTES: | No changes. |

| Filename(s): | ASPI7DOS.SYS |
| --- | --- |
| Driver/App: | AIC-7770 ASPI manager for DOS |
| NOTES: | No changes. |

| Filename(s): | ASPI8DOS.SYS |
| --- | --- |
| Driver/App: | AIC-7850/7855/7870/7880 ASPI manager for DOS |
| NOTES: | Added support for UltraSCSI host adapters (e.g. AIC-7880 based). If the BIOS is enabled, ASPI8DOS will no longer reset the SCSI bus by default. This is to preserve the soft IDs (SCAM) and termination settings assigned by the host adapter BIOS. If the BIOS is disabled, ASPI8DOS will reset the SCSI bus. Since ASPI8DOS does not have built-in support for SCAM drives, it will not perform the SCAM protocol after the reset. The user must make sure that each SCAM drive has a unique ID by setting the jumpers on the drive. Added auto-termination for the AHA-2930. ASPI8DOS will auto-configure the host adapter termination following a SCSI bus reset. The logic to scan PCI buses is now more efficient. The command line option /norst in EZ-SCSI 3.12 may cause some CD-ROM drives (e.g. NEC 3x) or tape drives (e.g. Tandberg 4100 QIC) to hang when used with an older host adapter BIOS (v1.10). This compatibility problem was caused by a bug in the drive’s firmware (the drive would report that it does not support sync I/O when it actually does). This problem has now been fixed in this version. Fixed a “Read BIOS Parameter” bug (DCS #2259). Previous versions of ASPI8DOS did not restore the stack properly after it called INT 13h, func 8 and detected an error via the carry bit. This version of ASPI8DOS will not work with the 2930 BIOS v1.01 - ALPHA A.003 due to a bug in the BIOS in its handling of termination. |

| Filename(s): | ASPIEDOS.SYS |
| --- | --- |
| Driver/App: | AHA-1740/1742/1744 enhanced mode ASPI manager for DOS |
| NOTES: | No changes. |

| Filename(s): | MA348.SYS, MA348IBM.SYS |
| --- | --- |
| Driver/App: | APA-348 (MiniSCSI Plus) ASPI manager for DOS |
| NOTES: | No changes. |

| Filename(s): | MA358.SYS, MA358IBM.SYS, MA358NEC.SYS |
| --- | --- |
| Driver/App: | APA-358/358A (MiniSCSI EPP) ASPI manager for DOS |
| NOTES: | No changes. |

| Filename(s): | MA160.SYS |
| --- | --- |
| Driver/App: | Trantor T160/T260 ASPI manager for DOS |
| NOTES: | No changes. |

| Filename(s): | MA460.SYS |
| --- | --- |
| Driver/App: | Adaptec SlimSCSI (APA-460) ASPI manager for DOS |
| NOTES: | No changes. |

| Filename(s): | ASPIDISK.SYS |
| --- | --- |
| Driver/App: | Adaptec ASPI Disk driver |
| NOTES: | If you have SMARTDRV loaded, and your removable media is write protected, and you delete files from File Manager, an error message will appear but then the files disappear from File Manager. You have to exit File Manager and restart File Manager to see the files again. This bug has been fixed. Increased wait for drive spin-up timeout to 2 minutes. Improved error message in case drive doesn’t spin-up within 2 minutes. Increased default timeout from 15 seconds to 25 seconds. |

| Filename(s): | ASPICD.SYS |
| --- | --- |
| Driver/App: | Adaptec ASPI CD-ROM driver |
| NOTES: | Fix for Kodak reported problem with raw mode reads > 32K. Fix for Toshiba CD+ problem, where doing a “dir” on a CD+ disk resulted in a “not High Sierra Format” message from DOS. ASPICD “Location of Head Command” trashes buffer. For further details, see DCS bug #2949. Still needs to be fixed. Planned multisession reader support for most CD-R drives in future builds. |

| Filename(s): | ASPIBUF.SYS |
| --- | --- |
| Driver/App: | AHA-1540/1542/1640 double-buffering device driver (for systems with > 16 MBytes of RAM). |
| NOTES: | Added INT 2Fh hook to enable/disable ASPIBUF while 32-Bit Disk Access is loaded. This allows WINASPI to load correctly and therefore allows WinASPI based applications to run (This is only when 32-Bit Disk Access is in effect). |

| Filename(s): | SJIIX.SYS |
| --- | --- |
| Driver/App: | HP ASPI scanner driver for DOS |
| NOTES: | No changes. |

| Filename(s): | FASTSCSI.386 |
| --- | --- |
| Driver/App: | 32-Bit Disk Access Transport VxD |
| NOTES: | Added support for AIC6X60.386 (6360/6370 VxD) |

| Filename(s): | AHA1540.386 |
| --- | --- |
| Driver/App: | AHA-1540/1542/1640 32-Bit Disk Access Driver |
| NOTES: | Added double-buffering capability for systems containing more than 16 Mbytes of RAM. This also adds WINASPI capabilities to Windows when in use. This fix also disables ASPIBUF while FASTSCSI and the AHA-1540 VxDs are running, supplanting its functionality. Fixed a problem where some interrupts could be lost when there are two or more cards using different interrupts, and one interrupt interrupts another. |

| Filename(s): | AHA1740.386 |
| --- | --- |
| Driver/App: | AHA-1740/1742/1744 (enhanced mode) 32-Bit Disk Access Driver |
| NOTES: | No changes. |

| Filename(s): | AIC7770.386 |
| --- | --- |
| Driver/App: | AIC-7770 32-Bit Disk Access Driver |
| NOTES: | Fixed multiple host adapter interrupt problem as described in AHA1540.386 section. Bug report is on DCS for reference. |

| Filename(s): | AIC7870.386 |
| --- | --- |
| Driver/App: | AIC-7850/7855/7870/7880 32-Bit Disk Access Driver |
| NOTES: | Fixed multiple host adapter problem as described in AHA1540.386 section. Linked with the latest HIM code to provide support for Ultra-SCSI. Due to HIM change, modified initialization code to look first for the scan type and then returning that value back to the HIM for the configuration space access. |

| Filename(s): | AFDISK.EXE |
| --- | --- |
| Driver/App: | Adaptec SCSI disk partitioning utility (for use with ASPIDISK) |
| NOTES: | No changes. |

| Filename(s): | CDPLAYER.EXE |
| --- | --- |
| Driver/App: | Adaptec DOS CD-Player |
| NOTES: | No changes. |

| Filename(s): | FASTSCSI.EXE |
| --- | --- |
| Driver/App: | 32-Bit Disk Access installer/uninstaller + write cache utility |
| NOTES: | Changed to support greater than 16 MBytes support for 1540 based systems. This change shuts down ASPIBUF while we load WINASPI so that we can use WINASPI to make calls to ASPI. |

| Filename(s): | RMVTOOL.EXE |
| --- | --- |
| Driver/App: | DOS removable media lock/unlock/eject utility |
| NOTES: | No changes. |

| Filename(s): | SCANNER.EXE |
| --- | --- |
| Driver/App: | Adaptec Scanner utility. Converts your scanner and printer into a copying machine. |
| NOTES: | Added new “About” dialog box. Thin font added to make it look more like Windows 95. Minor bug fix. See DCS #1480. Owner drawn buttons do not look right under Windows 95 if using “Marine” color scheme. |

| Filename(s): | SCSIBNCH.EXE |
| --- | --- |
| Driver/App: | Adaptec SCSI Benchmarking application |
| NOTES: | Added new “About” dialog box. SCSIBench would not work properly under Windows 95. It would incorrectly report an error that wasn’t really there. This problem would never appear under Windows 3.1x. It’s a slight difference in ASPI between the two operating systems that caused this. Modified SCSIBench to resolve this. Now works properly under Windows 3.x, Windows 95, and Windows NT 3.51. “Thin Font” Windows 95 look still to be added. |

| Filename(s): | SCSIFMT.EXE |
| --- | --- |
| Driver/App: | Adaptec SCSI low-level format utility |
| NOTES: | No changes. |

| Filename(s): | SHOWSCSI.EXE |
| --- | --- |
| Driver/App: | Adaptec SCSI Interrogator (16-bit) |
| NOTES: | Added new “About” dialog box. Epson scanners report themselves as processor device types. We added a check for Epson so that we correctly display a scanner icon. Epson has changed their Inquiry string on some of their new scanners so we’re drawing the wrong icon. Added another patch fix for Epson scanners. |

| Filename(s): | SPMU.EXE |
| --- | --- |
| Driver/App: | Adaptec SCSI Power Management (16-bit) |
| NOTES: | Added new “About” dialog box. |

| Filename(s): | WCDPLAY.EXE |
| --- | --- |
| Driver/App: | Adaptec Audio CD Player (16-bit) |
| NOTES: | Added new “About” dialog box. |

| Filename(s): | WINPART.EXE |
| --- | --- |
| Driver/App: | Adaptec Windows disk partitioning utility (16-bit) |
| NOTES: | Added new “About” dialog box. |

| Filename(s): | LANTERN.EXE (etc.) |
| --- | --- |
| Driver/App: | Magic Lantern (Photo CD Viewer) |
| NOTES: | No changes. |

| Filename(s): | CDCOPIER.EXE, CDR_VU.DLL |
| --- | --- |
| Driver/App: | Windows 95/NT CD Copier application |
| NOTES: | The following CD-Rs are supported: Plasmon RF-4100, Philips CDD-521/522, Ricoh 1060C, Sony 920S, and Yamaha CDR100. Other CD-Rs may work but have not been checked. This Sony 920S sees every recorded disk as an audio disk, causing the Contents display to incorrectly identify a recorded data disk. It does handle blank disks correctly, so this problem does not affect its ability to make copies. This is being looked into. WARNING! Simulate mode is not working yet with the Ricoh CD-R. All copies will be “real,” writing to the blank disk. This will be fixed in a future build. The following SCSI-2 CD Players have successfully copied Yellowbook CD-ROM - Toshiba 3401/3501, NEC 3x, Panasonic 504, Plextor 4Plex, and Sony 561. The remaining SCSI-2 members of their families should also work - as should other standard SCSI-2 CD players. SCSI-1 CD-ROM drives are NOT supported and will NOT be supported for the final product. Windows NT’s File Manager becomes “confused” when our application is active - since we catch check conditions that it uses to determine what action to take when Refresh is pressed. Microsoft’s CD Player causes the same problem so it is not unique to our software. Copying CD data to and from a hard disk has not yet been implemented. Saving alias information to the registry is not yet implemented. The copy warning dialog has not been implemented yet. An extra two second gap is added to the end of every track during the copy. Being looked into. Finalize time is not accurately predicted because it doesn’t take into acount the speed of the drive during the finalize process. This may result in an inaccurate estimate of the time to complete a copy. The optimum speed calculation still needs some refining. Some hardware combinations run the CD-R faster than can really be maintained, causing the transfer to fail with a buffer underrun. Error handling is not yet complete. We need to add user-friendly error reporting. Only the following types of CDs can be copied: Yellowbook CD-ROM (e.g. most software CDs), RedBook Audio, and Mixed Mode (e.g. 1st track Yellowbook CD-ROM - subsequent trakcs Redbook audio). There are problems with copying of audio CDs on some CD-ROM drives. Toshiba 3401, Panasonic 504, and the Plextor 4Plex have been successful so far. We will be looking into this further. |

| Filename(s): | DRVLIGHT.EXE |
| --- | --- |
| Driver/App: | Windows 95/NT Drive Monitoring utility |
| NOTES: | On-line help will be added. Window size and position not saved on exit. Statistics dialog - will change font of device string, will display Kbs read/written more clearly and will also show percent of reads vs. percent of writes. Device icon in tray does not flash when programs loaded from the Windows 95 shell. Main drive light window is slow to update when a device icon in the tray is under heavy activity. In minimized state, turning the menu on/off creates display problems (extra white space) when the window is restored. When devices are added to the tray, they have a white background instead of gray. |

| Filename(s): | EZREG.EXE |
| --- | --- |
| Driver/App: | Windows On-line registration utility |
| NOTES: | Bitmap missing. |

| Filename(s): | SCSIEXP.EXE |
| --- | --- |
| Driver/App: | Windows 95/NT SCSI Explorer. This is the “shell” that loads the SCSI Interrogator, Power Management, and Read/Write cache utility. |
| NOTES: | Still need to disable running of multiple instances. On-line help will be added. Tabbing from tree-view to tool dialog area to be added. Error occurs (GP fault) when no tool DLLs were successfully loaded. NOTE: The case when no tool DLL files are found is handled correctly. It is only when the tool DLL fails to load. Under Windows NT (not Win95) and 640x480 resolution, the SCSI Explorer does not display properly. Part of the tool property sheets is hidden. The window is too large to fit in the 640x480 under Windows NT because of NT’s wider system font. Currently displays the host adapter name as received from “ASPI for Win32.” This causes us to display cryptic names such as “arrow” and “sparrrow.” Still need to modify to make the host adapter name more descriptive. If you change the screen colors (e.g. use Marine color scheme), the tree-view list of devices have their icons appear in a white background instead of the default background color. |

| Filename(s): | WCDPLY32.EXE |
| --- | --- |
| Driver/App: | Windows 95/NT Audio CD-Player |
| NOTES: | 32-Bit Replacement for 16-bit WCDPLAY.EXE On-line help to be added. Cannot configure which CD-ROM drive to play (if you have multiple CD-ROM drives). Microsoft CD-Player still comes up as default CD-Player when audio CD is inserted. Need to add an option to allow user to use ours instead. Can run multiple instances. No error message when the CD is already in use. The micro CD-Player does not pop-up an error message when under Windows NT. The Mini CD-Player does not have a small title bar. The system menu options are not disabled when there is no CD. In a minimized state, turning the menu on/off on the standard CD-Player creates display problems (white space above the status bar) when the window is restored. The standard CD-Player is missing some status information (e.g. Total tracks) The jukebox CD-Player has some palette switching problems. This makes the app look real slow if you’re using 256 colors. If you select a different color scheme such as Marine (high color), the bitmap buttons are not displayed properly. Gray still appears instead of the default color. The same is true for the “Mini” CD-Player. If you select the “Lilac” (high color) scheme, the “Mini” CD-Player has a white bar along the right. Under Windows NT, the Jukebox buttons are not aligned properly. |

| Filename(s): | !ADPT001.DLL |
| --- | --- |
| Driver/App: | Windows 95/NT SCSI Power Management application |
| NOTES: | 32-Bit Replacement for 16-bit SPMU.EXE On-line help to be added. Time settings property page has a color glitch when the time input control is enabled. The entire window should be white but in parts it is gray. In the general settings property page, the Wait time default to 0 minutes. The default will be changed to 20 minutes. If you use a different background color than gray (e.g. Marine color scheme), then the graphics are not displayed properly. The “Wizard” installer and uninstaller dialog boxes do not have the 3-D look yet. |

| Filename(s): | !ADPT002.DLL |
| --- | --- |
| Driver/App: | Windows 95/NT SCSI Interrogator |
| NOTES: | 32-Bit Replacement for 16-bit SHOWSCSI.EXE TAB between the device items in the tree-view window and the fields in the tool’s property pages is not supported yet. Not all property sheets are set to non-stacked style. Under Win95, the system resource fields for SCSI host adapters (e.g. host adapter I/O address), are not yet implemented. Not all mode pages for disk drives are supported yet. Error checking will be made more robust. On-line help will be added. If you use a different background color than gray (e.g. Marine color scheme), then the graphics are not displayed properly. If you use the “Marine” color scheme, the “SCSI Features” property page has a gray background. |

| Filename(s): | !ADPT003.DLL |
| --- | --- |
| Driver/App: | Read/Write Cache utility |
| NOTES: | 32-Bit Replacement for 16-bit FASTSCSI.EXE TAB between the device items in the tree-view window and the fields in the tool’s property pages is not supported yet. On-line help will be added. If you use a different background color than gray (e.g. Marine color scheme), then the graphics are not displayed properly. |

| Filename(s): | BACKUP.EXE, CHIADI.DLL, CHIKDI.DLL, SCSIBK32.DLL |
| --- | --- |
| Driver/App: | Colorado Memory Systems Tape Backup application |
| NOTES: | Additional SCSI Tape Support under development. For now, only Tandberg tape drives are fully supported. HP DAT drives partially work. See top of this spec. for additional information. Shutting down app takes a long time. |

| Filename(s): | SETUP16.EXE |
| --- | --- |
| Driver/App: | 16-Bit installer for Windows 3.1/3.11. SETUP.EXE will launch this app if run under Windows 3.1/3.11. |
| NOTES: | Error checking needs to be added for no host adapter condition. User-specified starting CD-ROM drive letter is not yet implemented. User input of path for MSCDEX.EXE is not implemented. If MSCDEX.EXE is not in the search path, MSCDEX will not be added to AUTOEXEC.BAT. We’ll need to ask the user to insert a floppy containing MSCDEX.EXE. No warning for multi-config CONFIG.SYS file. Check box images in the host adapter selection dialog box is too close to the host adapter description text. Not all fields have keyboard shortcut support yet. If none of the DOS device drivers is selected, no warning is issued. Backup of CONFIG.SYS and AUTOEXEC.BAT not yet implemented. Emergency “patch” disk check not yet implemented. |

| Filename(s): | SETUP32.EXE |
| --- | --- |
| Driver/App: | 32-Bit installer for Windows 95/NT. SETUP.EXE will launch this app if run under Windows 95/NT. |
| NOTES: | Emergency “patch” disk check not yet implemented. |

| Filename(s): | TUTOR32.HLP |
| --- | --- |
| Driver/App: | Windows 95/NT version of Adaptec SCSITutor |
| NOTES: | Not all of the graphics images are completed yet. They will be prior to Beta. When you first run this SCSITutor, the size of the window will not be correct. This will be fixed. A 16-color title bitmap is still under development. This version of SCSITutor will look best if using 256 colors or higher. |

| Filename(s): | PRODUCTS.HLP |
| --- | --- |
| Driver/App: | Adaptec product flyers scanned into a Windows .HLP file. For Windows 95 and Windows NT only. |
| NOTES: | This file is on the Adaptec EZ-SCSI CD-ROM. Printing the flyers is not working properly at this time. This may be a quirk of Winhelp. |

| Filename(s): | EXC.EXE, XCD.DLL, XCDISM.DRV |
| --- | --- |
| Driver/App: | Incat’s Easy CD Lite Alpha build |
| NOTES: | This application currently only supports Philips CD-R drives. |