SECTION 0. INTRODUCTION

1. How to use the manual . . . . . . . . . . . . . . . . .     0 - 1

2. Overview of the TBAV utilities  . . . . . . . . . . . .     0 - 1

SECTION I. INSTALLING TBAV

1. How to install TBAV   . . . . . . . . . . . . . . . . .     I - 1

1.1 Initial installation  . . . . . . . . . . . . . .     I - 1

1.2 Menu and command syntax . . . . . . . . . . . . .     I - 2

2. Configuration . . . . . . . . . . . . . . . . . . . . .     I - 4

3. TbSetup . . . . . . . . . . . . . . . . . . . . . . . .     I - 6

3.1. The Purpose of TbSetup . . . . . . . . . . . . .     I - 6

3.2. How to use TbSetup . . . . . . . . . . . . . . .     I - 6

3.3. Command line options . . . . . . . . . . . . . .    I - 11

3.4. While executing  . . . . . . . . . . . . . . . .    I - 13

4. TbDriver  . . . . . . . . . . . . . . . . . . . . . . .    I - 16

4.1. Purpose of TbDriver  . . . . . . . . . . . . . .    I - 16

4.2. Command line options . . . . . . . . . . . . . .    I - 16

4.3. Language support . . . . . . . . . . . . . . . .    I - 19

5. System maintenance  . . . . . . . . . . . . . . . . . .    I - 20

6. Network maintenance . . . . . . . . . . . . . . . . . .    I - 21

6.1. Using DOS REPLACE  . . . . . . . . . . . . . . .    I - 21

6.2. Using PkUnZip  . . . . . . . . . . . . . . . . .    I - 21

SECTION II. ANTI-VIRUS STRATEGY

1. Protection against viruses  . . . . . . . . . . . . . .    II - 1

1.1. Introduction . . . . . . . . . . . . . . . . . .    II - 1

1.2. Basic precautions  . . . . . . . . . . . . . . .    II - 1

2. What to do when a virus strikes . . . . . . . . . . . .    II - 6

2.1. Detection of viruses . . . . . . . . . . . . . .    II - 6

2.2. Recovering from viruses  . . . . . . . . . . . .    II - 7

SECTION III. USING THE TBAV UTILITIES

1. TbScan  . . . . . . . . . . . . . . . . . . . . . . .     III - 1

1.1. The Purpose of TbScan  . . . . . . . . . . . .     III - 1

1.2. How to use Tbscan  . . . . . . . . . . . . . .     III - 2

1.3. Command line options . . . . . . . . . . . . .    III - 11

1.4. The scanning process . . . . . . . . . . . . .    III - 18

2. TbScanX . . . . . . . . . . . . . . . . . . . . . . .    III - 22

2.1. The Purpose of TbScanX . . . . . . . . . . . .    III - 22

2.2. How to use TbScanX . . . . . . . . . . . . . .    III - 22

2.3. Command line options . . . . . . . . . . . . .    III - 23

2.4. While scanning . . . . . . . . . . . . . . . .    III - 26

3. TbCheck . . . . . . . . . . . . . . . . . . . . . . .    III - 28

3.1. The Purpose of TbCheck . . . . . . . . . . . .    III - 28

3.2. How to use TbCheck . . . . . . . . . . . . . .    III - 28

3.3. Command line options . . . . . . . . . . . . .    III - 29

3.4. While checking . . . . . . . . . . . . . . . .    III - 31

3.5. Testing TbCheck  . . . . . . . . . . . . . . .    III - 31

4. TbClean . . . . . . . . . . . . . . . . . . . . . . .    III - 32

4.1. The Purpose of TbClean . . . . . . . . . . . .    III - 32

4.2. How to use TbClean . . . . . . . . . . . . . .    III - 33

4.3. Command line options . . . . . . . . . . . . .    III - 35

4.4. The cleaning process . . . . . . . . . . . . .    III - 36

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                  CONTENTS

5. Ongoing virus prevention: TbMon . . . . . . . . . . .    III - 40

5.1. TbMem  . . . . . . . . . . . . . . . . . . . .    III - 41

5.2. TbFile . . . . . . . . . . . . . . . . . . . .    III - 45

5.3. TbDisk . . . . . . . . . . . . . . . . . . . .    III - 47

6. TBAV Tools  . . . . . . . . . . . . . . . . . . . . .    III - 53

6.1. TbUtil . . . . . . . . . . . . . . . . . . . .    III - 53

6.2. TbLog  . . . . . . . . . . . . . . . . . . . .    III - 61

SECTION IV. ADVANCED USER INFORMATION

1. Memory requirements . . . . . . . . . . . . . . . . . .    IV - 1

2. TbSetup . . . . . . . . . . . . . . . . . . . . . . . .    IV - 3

2.1. Anti-Vir.Dat design considerations . . . . . . .    IV - 3

2.2. Format of TbSetup.Dat  . . . . . . . . . . . . .    IV - 3

2.3. TBAV site installation . . . . . . . . . . . . .    IV - 5

3. TbScan  . . . . . . . . . . . . . . . . . . . . . . . .    IV - 7

3.1. Heuristic scanning . . . . . . . . . . . . . . .    IV - 7

3.2. Integrity checking . . . . . . . . . . . . . . .    IV - 8

3.3. Program validation . . . . . . . . . . . . . . .    IV - 9

3.4. The algorithms . . . . . . . . . . . . . . . . .    IV - 9

3.5. The TbScan.Lng file  . . . . . . . . . . . . .     IV - 11

3.6. The TBAV.MSG file  . . . . . . . . . . . . . .     IV - 11

4. TbClean . . . . . . . . . . . . . . . . . . . . . . .     IV - 12

5. TbGensig  . . . . . . . . . . . . . . . . . . . . . .     IV - 15

5.1 The Purpose of TbGenSig . . . . . . . . . . . .     IV - 15

5.2 Defining signatures . . . . . . . . . . . . . .     IV - 15

5.3 Keywords  . . . . . . . . . . . . . . . . . . .     IV - 18

5.4 Wildcards . . . . . . . . . . . . . . . . . . .     IV - 21

Appendix A. TBAV messages

Appendix B. TbScan - Heuristic flag descriptions

Appendix C. Solving incompatibility problems

Appendix D. Batch file handling

Appendix E. Virus naming

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                 SECTION 0

SECTION 0. INTRODUCTION

1. How to use the manual

Congratulations! By purchasing the ThunderBYTE Anti-Virus utilities you

have taken the basic step in building a massive anti-viral safety wall

around your precious computer system. Setting up the appropriate defen-

se, using the TBAV utilities, is a 'personal matter'. Therefore, we

highly recommend to read this manual thoroughly, so you are well aware

of all different kinds of security measures you may take.

This manual consists of four main sections. Section I instructs you how

to install the TBAV utilities on your hard disk(s), including some

useful hints on customized initialization. Section II gives an instruc-

tion on how to prevent viruses from infecting your computer system(s)

and directions on how to handle when you actually have been struck by a

computer virus.

In section III, both purpose and functionality of all TBAV utilities are

described. For those who want to know more about the subject, some

'advanced user information' on the ThunderBYTE Anti-Virus utilities is

presented in section IV.

You may use the TBAV manual as a reference manual, via an extensive

index and appendices referring to the TBAV error messages.

=>   Note that a complete reading of the manual is indispensible in order to

become familiar with the many facets of ThunderBYTE Anti-Virus, to know

what steps can - and must - be taken to ensure adequate protection and

to be fully prepared for a complete recovery, if and when disaster

strikes.

2. Overview of the TBAV utilities

What is ThunderBYTE Anti-Virus?

ThunderBYTE Anti-Virus (TBAV) is a comprehensive toolkit designed to

protect against - and recover from - computer viruses. While TBAV

focuses heavily on numerous ways to prevent a virus infection, the

package would not be complete without various cleaner programs to purge

a system, in the unlikely event that a virus manages to slip through.

The package therefore consists of a number of programs each of which

help you to prevent viruses to do their destructive jobs. Here is a

quick overview.

Collecting software information: TbSetup

TbSetup is a program that collects information from all software found

on your system. The information will be put in files named Anti-Vir.Dat.

0 - 1

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                 SECTION 0

The information maintained in these files can be used for integrity

checking, program validation, and to clean infected files.

Enable memory resident TBAV utilities: TbDriver

~    TbDriver does not provide much protection against viruses by itself, but

must be loaded in advance to enable the memory resident ThunderBYTE

Anti-Virus utilities, such as TbScanX, TbCheck, TbMem, TbFile and TbDisk

to perform properly. It also provides basic protection against ANSI

bombs and 'stealth' viruses.

Scanning for viruses: TbScan

TbScan is both a very fast signature scanner and a so-called heuristic

scanner. Besides its blazing speed it has many configuration options. It

can detect mutants of viruses, it can bypass stealth type viruses, etc.

The signature file used by TbScan is a coded 'TbScan.Sig' file, which

can be updated by yourself in case of emergency.  TbScan is able to

disassemble and decrypt files. This makes it possible to detect suspici-

ous instruction sequences and to detect yet unknown viruses. This

generic detection, named heuristic analysis, is a technique that makes

it possible to detect about 90% of all viruses by searching for suspici-

ous instruction se-quences rather than using any signature. For that

purpose TbScan contains a disassembler, decryptor and code analyzer.

Another feature of TbScan is the integrity checking it performs when it

finds the Anti-Vir.Dat files generated by TbSetup. 'Integrity checking'

means that TbScan will check that every file being scanned matches the

information maintained in the Anti-Vir.Dat files. If a virus infects a

file, the maintained information will not match the now changed file

anymore, and TbScan will inform you about this.

TbScan performs an integrity check automatically, and it does not have

the false alarm rate other integrity checkers have. The goal is to

detect viruses and not to detect configuration changes!

Automatic scanning: TbScanX

TbScanX is the memory resident version of TbScan. This signature scanner

remains resident in memory and automatically scans those files which are

being executed, copied, de-archived, downloaded, etc. TbScanX does not

require much memory. It can swap itself into expanded, XMS, or high

memory, using only 1Kb of conventional memory.

Check while loading: TbCheck

TbCheck is a memory resident integrity checker. This program remains

resident in memory and checks automatically every file just before it is

0 - 2

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                 SECTION 0

being executed. TbCheck uses a fast integrity checking method, consuming

only 400 bytes of memory. It can be configured to reject files with

incorrect checksums, and/or to reject files that do not have a corres-

ponding Anti-Vir.Dat record.

Restoring infected boot-sector, CMOS and partition tables: TbUtil

Some viruses copy themselves into the hard disk's partition table, which

makes them far more difficult to remove than bootsector viruses. Perfor-

ming a low-level format is an effective, but rather drastic measure.

TbUtil offers a more convenient alternative by making a precautionary

back-up of uninfected partition tables and the boot sector. If an

infection occurs, the TbUtil back-up can be used as a verifying tool and

as a means to restore the original (uninfected) partition table and

bootsector without the need for a destructive disk format. The program

can also restore the CMOS configuration for you. If a back-up of your

partition table is not available, TbUtil will try to create a new

partition table anyway, again avoiding the need for a low-level format.

Another important feature of TbUtil is the option to replace the parti-

tion table code with new code offering greater resistance to viruses.

The TbUtil partition code is executed before the boot sector gains

control, enabling it to check this sector in a clean environment. The

TbUtil partition code performs a CRC calculation on the master boot

sector just before the boot sector code is activated and issues a

warning if the boot sector has been modified. The TbUtil partition code

also checks and reports changes in the RAM lay-out. These checks are

carried out whenever the computer is booted from the hard disk.

It should be noted that boot sector verification is imperative before

allowing the boot sector code to execute. A virus could easily become

resident in memory during boot-up and hide its presence. TbUtil offers

total security at this stage by being active before the boot sector is

executed. Obviously, TbUtil is far more convenient than the traditional

strategy of booting from a clean DOS diskette for an undisturbed inspec-

tion of the boot sector.

Reconstructing infected files: TbClean

TbClean is a generic file cleaning utility. It uses the Anti-Vir.Dat

files generated by TbSetup to enhance file cleaning and/or to verify the

results. TbClean can however also work without these files. It disassem-

bles and emulates the infected file and uses this analysis to recon-

struct the original file.

Resident safeguard: TbMon

TbMon is a set of memory resident anti-virus utilities, consisting of

TbMem, TbFile and TbDisk. Most other resident anti-virus products offer

0 - 3

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                 SECTION 0

you the choice to invoke them before the network is loaded and losing

the protection after the logon procedure, or to load the anti-viral

software AFTER the logon to the network, resulting in a partially

unprotected system. The ThunderBYTE Anti-Virus utilities however recog-

nize the network software and take appropriate actions to ensure their

functionality.

Controlling memory: TbMem

~    TbMem detects attempts from programs to remain resident in memory, and

ensures that no program can remain resident in memory without permis-

sion. Since most viruses remain resident in memory, this is a powerful

weapon against all such viruses, known or unknown. Permission informa-

tion is maintained in the Anti-Vir.Dat files. TbMem also protects your

CMOS memory against unwanted modifications.

Preventing infection: TbFile

TbFile detects attempts from programs to infect other programs. It also

guards read-only attributes, detects illegal time-stamps, etc. It will

make sure that no virus succeeds in infecting programs.

Protecting the disk: TbDisk

TbDisk is a disk guard program which detects attempts from programs to

write directly to disk (without using DOS), attempts to format, etc.,

and makes sure that no malicious program will succeed in destroying your

data. This utility also traps tunneling and direct calls into the BIOS

code. Permission information about the rare programs that write directly

and/or format the disk is maintained in the Anti-Vir.Dat files.

Define your own signatures (in case of an emergency): TbGensig

Since TBAV is distributed with an up-to-date, ready-to-use signature

file, you do not really need to maintain a signature file yourself. If,

however, you want to define your own virus signatures, you will need the

TbGensig utility. You can use either published signatures or define your

own ones if you are familiar with the structure of software.

Remove infected files: TbDel

The DOS 'DEL' command does not actually erase a file. It simply changes

the first filename character in the directory listing and frees up the

space by changing the disk's internal location tables. TbDel is a small

program with just one but important purpose: it replaces every single

byte in a file with zero characters before deleting it. The entire

contents are therefore obliterated and totally unrecoverable.

0 - 4

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                 SECTION I

SECTION I. INSTALLING TBAV

1. How to install TBAV

System requirements

The ThunderBYTE Anti-Virus utilities can be executed on any IBM or

compatible PC with at least 1 Mb disk space. The TBAV utilities need 256

Kb free internal memory and require DOS 3. However, DOS 5 or a later

version is recommended. The TBAV utilities are compatible with networks,

Windows, DR-DOS, etc.

1.1 Initial installation

You can install the TBAV utilities either by using the installation

procedure (which is explained below) or by a fully customized TBAV

installation (which is explained in sections  I - 3 and II).

Insert the TBAV installation diskette in the diskette drive.

Type:

A: or B:

Type:

install C:\TBAV <Enter>

+-------------------------------+

|    Express Installation     > |

|    Custom Installation      > |

|    Upgrade TBAV             > |

|    License TBAV               |

|    View TBAV.DOC file         |

|    Quit installation          |

+-------------------------------+

Since this is the first time you install the TBAV package you choose the

first option.

The installation program shows you the Licensing Agreement, and when you

are finished reading you have to press <Esc>.

The installation program now asks you to enter your name, company name

(if applicable) and to confirm the licensing agreement by typing 'YES'.

Then a new menu will pop-up which shows you the Source-path (the path

where the installation program itself resides, usually the A: drive) and

the default destination-path where the TBAV program files will be

installed, usually C:\TBAV). You can change these paths, but this is

I - 1

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                 SECTION I

normally not necessary. When the paths are configured as desired, select

'Begin Installation'.

The installation program now scans your system to make sure that it is

clean, and copies the TBAV files to the destination directory. After

that it makes a backup of the Autoexec.Bat file before making a few

modifications to it. The installation program adds the TBAV directory to

your PATH statement, and it adds a call to the TbStart.Bat file.

The TbStart.Bat file which is located in the TBAV directory contains the

following commands:

C:\TBAV\tbdriver

C:\TBAV\tbscanx

C:\TBAV\tbcheck

C:\TBAV\tbmem

C:\TBAV\tbfile

C:\TBAV\tbscan /once /alldrives

After reading the manual thoroughly, you can configure these commands to

suit your own personal needs.

In the final installation menu you are invited to make a Rescue disket-

te. You will need one empty formatted diskette. It is highly recommended

to create a Rescue diskette right now.

When you exit the installation program, TbSetup will be invoked automa-

tically to generate the reference files (Anti-Vir.Dat). When this

process has been finished it is recommended to reboot to activate the

new commands in your Autoexec.Bat file.

It is very likely that some of the TBAV utilities are going to display

messages when you reboot and continue using the computer as you normally

would. Some programs perform operations that are monitored by the TBAV

utilities, so TBAV must first 'learn' which programs need proper permis-

sion. Execute some of the programs you use regularly and at each rele-

vant query respond with 'Y' to authorize or 'N' to deny permission. TBAV

will remember the settings and not bother you again. Reboot the computer

at the end of this test run.

The TBAV utilities are now ready to monitor the system and will issue a

warning if something suspicious - or worse - is about to happen. They

will also warn you if any new file contains a possible virus - well

before it can do any harm.

1.2 Menu and command syntax

You can activate most of the TBAV utilities from within the TBAV menu,

by loading:

cd\tbav

I - 2

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                 SECTION I

tbav

In order to execute the utilities automatically, all TBAV drivers and

utilities may be executed from the DOS prompt. In a systemized setup,

however, the drivers should be installed and activated in your Con-

fig.Sys, with a device= or install= directive, or in the TbStart.Bat

file as a TSR. Similarly, most utilities can be started automatically -

in the case of TbScan restricted to once a day - in the TbStart.Bat

file. The two exceptions are TbClean and TbDel, which should be executed

only from the DOS prompt and (TbClean) from within the TBAV menu.

All commands for ThunderBYTE Anti-Virus can be used with command line

switches or options to control special features. The options may either

be written out in full, or abbreviated to their one- or two-letter

mnemonic to shorten the command line. Throughout this manual the exam-

ples are given with options in verbose, unabbreviated form for clarity.

Options must be separated by spaces. They do not need a preceding switch

character, but you may use the customary slash or hyphen switch charac-

ters if you wish.

The standard command line syntax for all ThunderBYTE Anti-Virus commands

is:

command [<path>] [<filename>] [<option>] ... [<suboption>] ...

You may review the correct syntax for any command, including a complete

option list, with the command followed by the word 'help' or a question

mark, as in:

tbcheck ?

The same on-line help is provided whenever the command is issued with an

invalid option.

The examples, presented in this manual assume that all utilities were

installed in the default \TBAV directory.

Create a recovery diskette!

It is highly recommended to make a recovery diskette. The example setups

assume you have created such a recovery diskette (see the instructions

in section II).

I - 3

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                 SECTION I

2. Configuration

The choices you made when installing the TBAV utilities may need some

tuning, for instance by appending options to the loading command. This

tuning can be done by editing the TBSTART.BAT file, which automatically

loads the memory resident utilities.

=>   If suitable, you may write the relevant commands in the Config.Sys file

instead. Do not forget to specify the .EXE extension in the Config.Sys

file!

Below, some basic hints are presented, to customize the initial settings

to suit your own needs. After initializing and rebooting your system for

the first time afterwards, TBAV will need your response in the initial

'learning' stage.

TBAV menu configuration

The TBAV menu allows some configuration, as well.

+----Main menu-----+

|  Confi+----------TBAV configuration----------+

|  TbSet|v Use colors                          |

|  TbSca|  Save configuration to TBAV.INI      |

|  TbUti|  File view utility                   |

|  TbCLe|v Wait after program execution        |

|  TBAV |  Show command line before executing  |

|  Docum|v Edit path string before scanning    |

|  Regis+--------------------------------------+

|  Quit and save   |

|  eXit (no save)  |

+------------------+

You can activate the configuration options by toggling to the relevant

choice and pressing <Enter>.

Use colors

If disabled, TBAV will be displayed in monochrome mode, which is conve-

nient for use on laptop computers.

Save configuration to TBAV.INI

All configuration values, set within the TBAV menu, are saved in the

'TBAV.INI' file, once you have selected this option. The next time you

load the TBAV utilities the configuration values in the current TBAV.INI

file will be valid. These values apply to the TBAV menu itself and the

utilities TbSetup, TbScan and TbClean. Although you may edit the TBAV.I-

NI file manually, it is recommended to let the TBAV menu shell generate

I - 4

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                 SECTION I

it. By default, the contents of the TBAV.INI file is only valid while

using the TBAV menu shell. You may, however, enable the "Use TBAV.INI

file" options (or specify the 'UseIni' switches in the TBAV.INI file

itself) for each of the utilities mentioned above. Doing so, the confi-

guration, saved in the TBAV.INI file will also be valid when TbSetup,

TbScan or TbClean are loaded from the command line. Be careful, since

options specified in the TBAV.INI file can not be undone on the command

line. TBAV will create a TBAV.INI file when enabling this option for the

first time. In this file all valid configuration switches are listed.

The disabled switches are preceded by a semicolon.

File view utility

TbSetup and TbScan generate a datafile and a logfile respectively. By

default, you can view these files from the TBAV menu using an internal

file view utility. By using this option you are able to attach your

favorite external file view utility. Enter the complete path and the

file name, including the extension.

Wait after program execution

By enabling this option, TBAV will display the message:

"Press any key to return to the TBAV utilities" after executing an

external utility.

Show command line before executing

Enabling this option will force TBAV to display the DOS command which

will load the external utility. This option comes in handy in order to

see the command(s) you specified before. After pressing <Enter> TBAV

will execute the DOS commands.

Edit path string before scanning

If enabled, you are prompted to edit or confirm the path to be scanned

after you have selected "Start scanning".

I - 5

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                 SECTION I

3. TbSetup

3.1. The Purpose of TbSetup

TbSetup is an indispensible tool, adding support to the rest of the

ThunderBYTE Anti-Virus utilities, even though it does not take an active

part in actual virus detection or cleaning itself. TbSetup organizes

control and recovery information giving extra power to the other utili-

ties. The information is gathered, mainly from program files, into a

single reference file called Anti-Vir.Dat, one each per directory. The

nature of Anti-Vir.Dat files will be explained more fully at the end of

this chapter.

Although the ThunderBYTE utilities can work perfectly well without the

Anti-Vir.Dat files it is highly recommended to have TbSetup generate

these files. The Anti-Vir.Dat files can be used for several purposes:

Integrity checking. TbScan and the memory resident TbCheck program will

perform an integrity check while scanning if it can detect the Anti-

Vir.Dat file. If a file gets infected by a virus, the information in the

Anti-Vir.Dat file will not match the actual file contents, and TbScan

and TbCheck will inform you that the file has been changed.

The TbSetup program recognizes some files that need special treatment.

An example of such a file is a disk image file of a network remote boot

disk. - Such a file that actually represents a complete disk - should be

scanned completely, and for all viruses. TbSetup will put a mark in the

Anti-Vir.Dat file to make sure that TbScan scans the complete file for

all viruses.

Once a file is infected, TbClean will reconstruct the original file. The

information in the Anti-Vir.Dat file will be of great help to TbClean.

Some infected programs can only be cured if there is information about

the program in the Anti-Vir.Dat file.

TbCheck (a tiny resident integrity checker) has no purpose if there are

no Anti-Vir.Dat files on your system.

The resident TBAV utilities need the Anti-Vir.Dat files to maintain

permission information. Without Anti-Vir.Dat files you can not get rid

of false alarms other than by disabling a complete feature.

3.2. How to use TbSetup

This is the one program where the rule applies: The less you use the

program, the better your protection against viruses! Why? Keep in mind

that an Anti-Vir.Dat file stores vital information needed to detect a

virus, as well as data for subsequent recovery and for cleaning. But

consider what would happen if you were to execute TbSetup after a virus

entered the system: the information in the Anti-Vir.Dat file would be

I - 6

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                 SECTION I

'updated' to the state of the infected file, wiping out all traces of

data needed to reconstruct the file of the original, uninfected state.

Never use TbSetup when there is the slightest evidence of a virus in

your system. Once the Anti-Vir.Dat files have been generated as part of

the initial setup, any subsequent usage of TbSetup should be confined to

directories with new or changed program files.

Please note that the Anti-Vir.Dat directory entries will have the

attribute 'hidden' and therefore do not show up when you use standard

directory commands. You can see the filenames only with the help of

special utilities.

You may load TbSetup either from the DOS command line or from the TBAV

menu. For the initial installation you can use option 'AllDrives':

TbSetup AllDrives

You may also specify which drives and paths should be processed. To

setup disks C: and D: you should enter:

TbSetup C:\ D:\

When no filename has been specified but a drive and/or path instead, the

specified path will be used as top-level path. All its sub-directories

will be processed too. When a filename has been specified only the

specified path will be processed. Sub-directories will not be processed.

Wildcards in the filename are allowed.

When executed from the DOS command line, the 'newonly' option can be

used to prevent existing information from being overwritten.

To help you remember that TbSetup needs to be executed again, the next

time you execute TbScan it will display either a small 'c' after the

file to indicate a new file or a capital 'C' if a file has simply been

changed.

Example:

You add a new file TEST.EXE to your directory C:\FOO.

TbSetup C:\FOO\TEST.EXE

Example:

You install a new product in a new directory C:\NEW.

TbSetup C:\NEW

When using the DOS command you may append a number of loading options.

These options are presented in section 3.3. of this chapter.

I - 7

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                 SECTION I

The 'TbSetup' menu

+----Main menu-----+

|  Confi+------TbSetup menu------+

|  TbSet|  Start TbSetup         |

|  TbSca|  Options menu         >|

|  TbUti|  Flags menu           >|

|  TbCLe|  Data file path/name   |

|  TBAV |  View data file        |

|  Docum+------------------------+

|  Register TBAV   |

|  Quit and save   |

|  eXit (no save)  |

+------------------+

Data file path/name

TbSetup will search for 'special' files in a file named TbSetup.Dat.

After selecting this option 'datfile' you can specify another path or

filename that contains a list of 'special' files. Option format: Datfile

[=<filename>]

Example:

TbSetup Datfile = c:\tbav\tbsetup.dat

+----Main menu-----+

|  Confi+------TbSetup menu------+

|  TbSet|  Start+-------TbSetup options--------------+

|  TbSca|  Optio|  Use TBAV.INI file                 |

|  TbUti|  Flags|  Prompt for pause                  |

|  TbCLe|  Data |  Only new files                    |

|  TBAV |  View |  Remove Anti-Vir.Dat files         |

|  Docum+-------|  Test mode (don't change anything) |

|  Register TBAV|v Hide Anti-Vir.Dat files           |

|  Quit and save|  Make executables readonly         |

|  eXit (no save|  Clear readonly attributes         |

+---------------|v Sub-Directory scan                |

+------------------------------------+

Use TBAV.INI file

By enabling this option, the TbSetup configuration values, saved in the

TBAV.INI file, will also be valid when loading TbSetup from the command

line. Be careful, since options specified in the TBAV.INI file can not

be undone on the command line. See chapter I-2 ('Configuration').

I - 8

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                 SECTION I

Prompt for pause

When you enter option 'pause' TbSetup will stop after it has processed

the contents of one window. This gives you the possibility to examine

the results.

Only new files

If you want to add new files to the Anti-Vir.Dat database, but prevent

the information of changed files from being updated use option 'newon-

ly'. Updating the information of changed files is dangerous because if

the files are infected, the information to detect and cure the virus

will be overwritten. Option 'newonly' prevents the information from

being overwritten but it still allows information of new files to be

added to the database.

Remove Anti-Vir.Dat files

If you want to stop using the ThunderBYTE utilities you do not have to

remove all the Anti-Vir.Dat files yourself. By using this option TbSetup

will neatly remove all Anti-Vir.Dat files from your system.

Test mode (Don't change anything)

If you want to see the effect of an option without the risk that somet-

hing is activated you do not want, use option 'test'. If that option is

specified the program will behave as it would normally, but it will not

change or update anything on your hard disk.

Hide Anti-Vir.Dat files

The Anti-Vir.Dat files are normally not visual in a directory listing.

If you prefer to have normal - i.e. visible - files disable this option.

=>   Note that this option only applies for new Anti-Vir.Dat files.

Make executables read-only

As TbFile guards the read-only attribute permanently it is highly

recommended to make all executable files read-only to prevent any

modifications on these files. TbSetup will do the job if you enable

option 'read-only'. Files that should not be made read-only are recogni-

zed by TbSetup.

I - 9

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                 SECTION I

Clear read-only attributes

This option can be used to reverse the operation of option 'read-only'.

If you enable this option all read-only attributes of all executable

files will be cleared.

Sub-Directory scan

By default TbSetup will search sub-directories for executable files,

unless a filename (wildcards allowed!) has been specified. If you

disable this option, TbSetup will not process sub-directories.

+----Main menu-----+

|  Confi+-----TbSetup menu------+

|  TbSet|  Start+-----TbSetup flags------+

|  TbSca|  Optio|v Use normal flags      |

|  TbUti|  Flags|  Set flags manually    |

|  TbCLe|  Data |  Reset flags manually  |

|  TBAV |  View |  Define flags         >|

|  Docum+-------+------------------------+

|  Register TBAV   |

|  Quit and save   |

|  eXit (no save)  |

+------------------+

Set flags manually

This option is for advanced users only. With this option you can manual-

ly set permission flags in the Anti-Vir.Dat record. This option requires

a hexadecimal bitmask for the flags to set. For information about the

bitmask consult the TbSetup.Dat file.

Option format: Set =<flags>

Example:

Set = 0001

Reset flags manually

This option is for advanced users only. With this option you can manual-

ly reset permission flags or prevent flags to be set in the Anti-Vir.Dat

record. This option requires a hexadecimal bitmask for the flags to

reset.  For information about the bit mask consult the TbSetup.Dat file.

Option format: Reset =<flags>

Example:

Reset = 0001

I - 10

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                 SECTION I

+----Main menu-----+

|  Confi+-----TbSetup menu------+

|  TbSet|  Start+-----TbSetup flags------+

|  TbSca|  Optio|v Use n+--Define flags to be changed--+

|  TbUti|  Flags|  Set f|  0001: Heuristic analysis    |

|  TbCLe|  Data |  Reset|  0002: Checksum changes      |

|  TBAV |  View |  Defin|  0004: Disk image File       |

|  Docum+-------+-------|  0008: Readonly sensitive    |

|  Register TBAV   |    |  0010: TSR program           |

|  Quit and save   |    |  0020: Direct disk access    |

|  eXit (no save)  |    |  0040: Attribute modifier    |

+------------------+    |  8000: Interrupt rehook      |

+------------------------------+

3.3. Command line options

TbSetup allows options to be specified on the command line.  TbSetup

recognizes option short-keys and option words. The words are easier to

memorize, and they will be used in this manual for convenience. The

syntax is as follows:

TbSetup [<path>][<filename>]... [<options>]...

option parameter      short  explanation

----------------------------------------------------------------

help                  he   help

pause                 pa   enable 'Pause' prompt

mono                  mo   force monochrome

nosub                 ns   skip sub-directories

newonly               no   do not update changed records

alldrives             ad   process all local fixed drives

allnet                an   process all network drives

remove                rm   remove Anti-Vir.Dat files

test                  te   do not create / change anything

nohidden              nh   do not make Anti-Vir.Dat files hidden

readonly              ro   set read-only attribute on executables

nordonly              nr   remove / do not set read-only attribute

set     =<flags>      se   set flags

reset   =<flags>      re   reset flags / do not set flags

datfile [=<filename>] df   data file to be used

help (he)

If you specify this option, TbSetup displays the contents of the TBSE-

TUP.HLP file (if available) in the home directory of TbSetup.  If you

specify the '?' option you will get the summarized help info as listed

above.

I - 11

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                 SECTION I

pause (pa)

When you specify 'pause' TbSetup will stop after it has processed the

contents of one window. This gives you the possibility to examine the

results.

mono (mo)

This option forces TbSetup to refrain from using colors in the screen

output. This might enhance the screen output on some LCD screens or

color-emulating monochrome systems.

nosub (ns)

By default TbSetup will search sub-directories for executable files,

unless a filename (wildcards allowed!) has been specified. If you

specify this option, TbSetup will not process sub-directories.

newonly (no)

If you want to add new files to the Anti-Vir.Dat database, but prevent

the information of changed files from being updated, use option 'newon-

ly'. Updating the information of changed files is dangerous because if

the files are infected, the information to detect and cure the virus

will be overwritten. Option 'newonly' prevents the information from

being overwritten but it still allows information of new files to be

added to the database.

alldrives (ad)

If you want TbSetup to process all local non-removable drives you can

specify this option. Except for the initial execution, it isn't a good

idea to use this option.

allnet (an)

If you want TbSetup to process all network drives you can specify this

option. Except for the initial execution, it isn't a good idea to use

this option.

remove (rm)

If you want to stop using the ThunderBYTE utilities you do not have to

remove all the Anti-Vir.Dat files yourself. By using this option TbSetup

will neatly remove all Anti-Vir.Dat files from your system.

test (te)

If you want to see the effect of an option without the risk that somet-

hing is activated you do not want, use option 'test'. If that option is

specified the program will behave as it would normally, but it will not

change or update anything on your

hard disk.

nohidden (nh)

The Anti-Vir.Dat files are normally not visual in a directory listing.

If you prefer to have normal - i.e. visible - files specify this option.

=>   Note that this option only applies for new Anti-Vir.Dat files.

readonly (ro)

I - 12

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                 SECTION I

As TbFile guards the read-only attribute permanently it is highly

recommended to make all executable files read-only to prevent any

modifications on these files. TbSetup will do the job if you enable

option 'read-only'. Files that should not be made read-only are recogni-

zed by TbSetup.

nordonly (nr)

This option can be used to reverse the operation of option 'read-only'.

If you enable this option all read-only attributes of all executable

files will be cleared.

set (se)

This option is for advanced users only. With this option you can manual-

ly set permission flags in the Anti-Vir.Dat record. This option requires

a hexadecimal bitmask for the flags to set. For information about the

bitmask consult the TbSetup.Dat file.

Option format: Set =<flags>

Example:

Set = 0001

reset (re)

This option is for advanced users only. With this option you can manual-

ly reset permission flags or prevent flags to be set in the Anti-Vir.Dat

record. This option requires a hexadecimal bitmask for the flags to

reset.  For information about the bit mask consult the TbSetup.Dat file.

Option format: Reset =<flags>

Example:

Reset = 0001

datfile (df)

After the datfile option you can specify the name of the data file to be

used.

3.4. While executing

TbSetup divides the screen into three windows: an information window

displaying data file comments across the top of the screen, a scanning

window on the left and a status window on the right.

The lower left window lists the names of the files being processed,

along with file specific information:

TEST.EXE 01234  12AB23CD   Added    * 0001

|        |      |          |        | |

|        |      |          |        | |

|        |      |          |        | 'flags' set for this file

|        |      |          |        indicates 'special' file

|        |      |          action performed

I - 13

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                 SECTION I

|        |      32-bit CRC (checksum)

|        file size in hexadecimal number

name of file in process

Do not be concerned if the information flies too fast for you to read,

or if it puzzles you. You will probably never need these details anyway.

The scanning window has an 'action performed' field indicating whether

an entry in the Anti-Vir.Dat was added, changed or updated:

Added

There was no previous entry for this file in the Anti-Vir.Dat record. A

new entry has been added.

Changed

There was an existing entry, but the file has been changed. The Anti-

Vir.Dat information has been updated.

Updated

There was an Anti-Vir.Dat record and the file was found to be unchanged.

TbSetup did, however, change some of the program's permission flags, on

account of an entry in the TbSetup.Dat file, or in compliance with a

'Set' or 'Reset' option.

The process may be aborted by pressing Ctrl-Break.

Purpose of the TbSetup.Dat file

Although the ThunderBYTE utilities perform well on almost every file

without extra help, there are some files that need particular attention.

TbSetup uses information collected in a special data file, TbSetup.Dat,

to flag such files in the Anti-Vir.Dat file. The other ThunderBYTE

utilities then use that information to determine how such a 'special'

file should be treated:

Examples of such files:

Some programs maintain configuration information inside the executable

file (EXE, COM) itself. Whenever you change the configuration of these

programs, the executable file will change as well, along with its

checksum. The new checksum will not match the one stored in the TbSe-

tup.Dat file anymore. Since some ThunderBYTE utilities use this checksum

information to verify integrity or cleanup results, they need to 'know'

when a file's checksum is not a fixed item and should be allowed to

change.

TbScan can use generic detection methods such as 'heuristic' analysis to

detect unknown viruses. Since heuristic analysis implies inevitable

I - 14

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                 SECTION I

false alarms when a file looks like a virus, TbScan may have to decide

not to do a heuristic analysis on such a program.

Some of the ThunderBYTE utilities guard the read-only attribute and make

sure that it can be removed only with the user's explicitpermission. A

few programs, however, refuse to run properly with the read-only attri-

bute set.

TbScan's default scanning method performs perfectly well with just about

any file, but there are some that need special analysis. Such a file is

the Novell NET$DOS.SYS file, not a device driver - as the filename

extension suggests - but a disk image of the bootable disk. It should be

scanned completely and for all signatures, including COM and BOOT.

The resident monitoring utilities of the TBAV package detect all sorts

of virus-specific behaviour. Some programs, even though they may act

like a virus, are still perfectly normal and should be permitted to be

executed without TBAV interference.

You need not be concerned to find that a few files will be excluded from

heuristic analysis. Those files will still be scanned the conventional

way for signatures and all the rest. Furthermore, no heuristic exclusion

will be granted unless a file matches exactly with the entry in the

TbSetup.Dat file - including its name, size and the 32-bit CRC checksum.

This eliminates security holes effectively: if a listed file is already

infected, its checksum won't match the 32-bit CRC in the TbSetup.Dat

file and the exclusion will not apply. By the same token, if a program

is infected at a later date, the result would be a change in at least

one of its characteristics; the record in the Anti-Vir.Dat file will not

match any longer and the file will be subject to full heuristic analysis

like any other.

I - 15

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                 SECTION I

4. TbDriver

4.1. Purpose of TbDriver

TbDriver does not provide much protection against viruses by itself, but

must be loaded in advance to enable the memory resident ThunderBYTE

Anti-Virus utilities, such as TbScanX, TbCheck, TbMem, TbFile and

TbDisk, to perform properly. It is the source for some of the routines

these utilities have in common, including support to generate the pop-up

window routines, driving the translation unit which enables the possibi-

lity of displaying messages in your native language, and support for

networks.

In addition, TbDriver also contains basic protection against 'Stealth'

viruses and against ANSI bombs.

How to use TbDriver

Loading TbDriver must be loaded before (one of) the other memory TbDri-

ver resident TBAV utilities. For loading instructions, please consult

the following pages.

=>   If you want protection against ANSI-bombs, you should load TbDriver

AFTER the ANSI driver.

In normal situations it is not necessary to use the 'net' option of

TbDriver.

If you install TbDriver on a machine that is booted from a boot ROM,

specify the message file with the drive and path where it can be found

AFTER the machine has booted. The default message file will not be

accessible anymore after the machine has booted.

4.2. Command line options

Tbdriver allows loading options to be specified on the command line. A

filename specification will be treated as a language file specification.

The upper three options are always available, the other options are only

available if TbDriver is not already memory resident.

I - 16

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                 SECTION I

option  parameter    short explanation

-------------------- ----- ---------------------------

help                 ?     display this helpscreen

net                  n     force LAN support

remove               r     remove TbDisk from memory

mode    =<m|c>       m     override video mode

freeze               j     freeze machine after an alert

lcd                  l     enhance output on LCD screens

noavok  =<drives>    o     assume permission when AV record is missing

quiet                q     do not display activity

secure               s     do not allow permission updates

notunnel             t     do not detect tunneling

nofilter             f     do not filter dangerous ANSI codes

nostack              ns    do not install a stack

help (?)

If you specify this option TbDriver will show you the valid command line

options as listed above.

net (n)

TbDriver cooperates well with most networks; in normal situations option

'net' will not be needed at all. It should be used only if all of the

following conditions are true: A connection to a Novell network is made,

and TbDriver.Exe is started before the logon command was used, and there

is no valid Anti-Vir.Dat record in the directory where the NET?.COM

program resides, or after the NET?.COM file has been renamed.

remove (r)

This option disables TbDriver and will try to remove the resident part

of its code from memory in an attempt to restore this memory space back

to the system. Unfortunately, this can work only if TbDriver was loaded

last. An attempt to remove a TSR after another TSR has been started will

simply leave a useless gap in memory and could disrupt the interrupt

chain. TbDriver checks whether it is safe to remove its resident code;

if not, it will simply disable itself.

mode (m)

On dual video systems TbDriver will use the currently active screen. It

may be forced to use the alternate screen with option 'mode=m' for

monochrome, or 'mode=c' for color systems.

freeze (j)

This option causes TbDriver to freeze the machine after a TbDriver

message has been displayed. This prevents users to answer TbDriver

messages without permission from the supervisor.

lcd (l)

This option improves the readability of the TbDriver messages on LCD

screens.

I - 17

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                 SECTION I

noavok (o)

This option is not recommended for normal usage. You may need it in

order to grant permission automatically for programs without an Anti-

Vir.Dat record. Option 'noavok' requires a parameter specifying the

drives to which the default permission applies. If, for example, you do

not want a message from TbMem when a TSR without Anti-Vir.Dat record is

executed from drive E: and F:, you could specify 'noavok=ef' on the

TbDriver command line. If you do want to exclude network drives you

should specify an asterisk (*). So, if you want to grant permission for

all files without Anti-Vir.Dat records on drive A:, your ramdisk F: and

your remote network drives, specify 'noavok=af*'.

quiet (q)

Some resident TBAV utilities display an activity status. TbScanX, for

instance, displays a rectangle with "*Scanning*" in the upper left

corner of your screen while scanning a file. You can disable this with

the 'quiet' option when TbDriver is loaded.

secure (s)

Some ThunderBYTE utilities are able to store permission flags in the

Anti-Vir.Dat files. If you don't want these flags to be changed, specify

this option. It has no effect on flags that are already set, so the

option 'secure' may be used after installing new programs or packages.

notunnel (t)

TbDriver normally detects tunneling attempts on the part of viruses.

'Tunneling' is a technique viruses apply to determine the location of

the DOS system code in memory, and to use that address to communicate

with DOS directly. This will inactivate all TSR programs, including

resident anti-virus software. TbDriver is able to detect 'tunneling'

attempts, and informs you about this. Some other anti-virus products

also rely on tunneling techniques to bypass resident viruses, causing

false alarms. If you are currently executing other anti-viral products,

option 'notunnel' will disable tunneling detection.

nofilter (f)

The original ANSI driver has a feature to assign text strings to keys.

Years ago, people used this feature to assign - for instance - the F10

key to the text 'DIR /W'. This reprogramming can simply be done with

embedded ANSI codes in text files. Typing such a file with the DOS

'type' command is enough to reprogram the keys. Today, almost nobody

uses this feature anymore, but it is still there. Some ill-minded people

however use this feature to make a text file which reprograms - for

instance - the Enter key to execute the text 'Del *.*', or worse...

Such a text file is called an ANSI-bomb. TbDriver protects you against

ANSI-bombs by filtering out the keyboard reprogramming codes. All other

ANSI codes will pass without interference. If you don't want this

protection, or if you want to use this obsolete ANSI feature you can

specify option 'nofilter'.

I - 18

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                 SECTION I

nostack (ns)

By default, TbDriver maintains a stack for the resident TBAV utilities.

For most systems however this isn't necessary. If you specify option

'nostack' TbDriver will use the application stack, saving a few hundred

bytes of memory. However, if the system hangs or becomes unstable, you

should discontinue use of this option.

4.3. Language support

The optional filename specification is used to determine where the

language file can be found. TbDriver retrieves pop-up window messages

from a TBDRIVER.LNG file, which it expects to find in its own home

directory. The default English language file is TBDRIVER.LNG, which may

be replaced by a file in your local language. You can order separate

language support packages at your local ThunderBYTE dealer, or download

the language file from a ThunderBYTE support BBS. Please refer to page

I-38 for further details. To load a localized language file, either

rename it to the default original, or specify the full path and filename

following the command. You may also switch to another language by

calling TbDriver again with a different message file. This will not take

up any extra memory.

I - 19

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                 SECTION I

5. System maintenance

All systems need maintenance and so do the TBAV utilities. As new

viruses emerge, TbScan's signature file must be updated to avert new

dangers. You may either get the latest signature file from your local

ThunderBYTE dealer or download the file from one of our support Bulletin

Board Systems.

Whenever you add, update or replace programs on your system, be sure to

use TbSetup to generate or update their fingerprints in the Anti-Vir.Dat

files. Sometimes you will want to create a new recovery diskette. When

you install a new version of DOS, the bootsector will be different.

Changing the configuration of your hard disks may affect the partition

tables and the CMOS setup. You should prepare a new recovery diskette

after all system modifications.

Updates

The ThunderBYTE Anti-Virus utilities are updated at frequent intervals.

Subscription to the ThunderBYTE update service (at your local dealer)

guarantees delivery of each new update. You may download new revisions

any time from any ThunderBYTE support BBS. Or check with a local bulle-

tin board regularly, as many of them offer updated versions of our

software.

The standard complete release is issued in an archive named:

TBAVxxx.ZIP, where 'xxx' stands for the three-digit version number. The

archive extension may vary on local bulletin boards using a different

archive method.

TBAV-for-Windows is distributed in an archive named TBAVWxxx.Zip. Note

that you need to download the DOS version of TBAV in addition to this

archive.

The resident ThunderBYTE Anti-Virus utilities are also available in

processor optimized formats. These processor optimized versions, named

TBAVXxxx.ZIP, are for registered users only. You can purchase these

versions via your local ThunderBYTE dealer.

Language support

The ThunderBYTE Anti-Virus utilities currently support several langua-

ges, by means of separate language files. Check your local ThunderBYTE

dealer for the availability of the TBAV support file in your language.

I - 20

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                 SECTION I

6. Network maintenance

The signature file TbScan.Sig should be replaced frequently. This can be

a lot of work if you want to update all work stations on a network

manually. Fortunately, there are several possibilities to do this job

automatically.

6.1. Using DOS REPLACE

Maintain a directory \TBAV_UPD\ on a public server drive. Any new

version of the TBAV utilities or any new signature file TbScan.Sig

should be placed it in this directory.

The work stations should execute a batch file automatically after users

log in on the network. This batch file should contain the following

lines:

rem Update the anti-virus product if a new one is available.

replace x:\tbav_upd\*.* c:\tbav /u /r

'Replace' is a standard DOS utility. It copies the files specified by

the first parameter ONLY if they are newer than the files specified in

the second parameter.

Make sure the 'replace' command is in the current path, and that the

specified paths are valid for your configuration. The 'x', used in the

above example, denotes the drive specification.

Thus, you only have to update one drive with the new signature file or

anti-virus software, and all workstations will update themselves as soon

as they log in! You can also add the /S option if you want REPLACE to

scan all directories on the workstations' drives for matching files.

Please consult the DOS manual for more details.

=>   Note: Do not forget to execute TbSetup on the new utilities in the

x:\tbav_upd directory, thus ensuring that the REPLACE command also

copies the new Anti-Vir.Dat file.

6.2. Using PkUnZip

Maintain a directory \TBAV_UPD\ on a public server drive. Any new

version of the TBAV distribution archive should be placed in this

directory.

The work stations should execute a batch file automatically after users

log in on the network. This batch file should contain the following

lines:

rem Update the anti-virus product if a new one is available.

PkUnZip -n -o x:\tbav_upd\TBAV???.ZIP c:\tbav

I - 21

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                 SECTION I

=>   Make sure the file PkUnZip.Exe is in the current path, and that the

paths specified are valid for your configuration.

Following this procedure, the 'PkUnZip' command will only come into

action when you just updated the ZIP files in the x:\tbav_upd directory.

Now you only have to update one drive with the new anti-virus software,

and all workstations will update themselves as soon as they log in!

I - 22

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                SECTION II

SECTION II. ANTI-VIRUS STRATEGY

1. Protection against viruses

1.1. Introduction

Maintaining a reliable safety system implies that you will be active in

taking measures to protect your system from virus infection, since some

viruses can hide themselves perfectly once resident in memory. At least

once every a week you should boot from a clean and write-protected

diskette and execute TbScan.

The tightness of your safety system very much depends on the amount of

time you want to invest to let the safety measures take place and the

vitality of the appropriate computer system. For use on a stand alone

computer containing low risk data, in an environment with little exchan-

ge of computer software, a daily scan will appear to be sufficient.

For company use however, in a network environment where diskettes are

exchanged frequently, where disks contain highly vulnerable information,

where a network going 'down' means the loss of an extensive amount of

money, protection must be as tight as the organisation can practically

handle.

Considering the above, a simple instruction on how to use the -highly

flexible - TBAV utilities cannot be given. It all depends on your own

demands and possibilities.

Therefore, you are advised to study this manual thoroughly so you will

be able to determine your own safety measures. To prevent viruses from

doing any harm you should at least under-take the activities as presen-

ted below.

1.2. Basic precautions

1. Install TBAV on your hard disk

You may customize the installation to suit your own needs. Make sure you

use TbSetup to maintain recovery information of all executable files of

your system! Please refer to the installation section (I) of this

manual.

In the following examples it is assumed that all utilities are copied in

the (default) directory named TBAV. For all example setups it is requi-

red that TbSetup has been executed. If your system has more hard disks

or disk partitions you should repeat the TbSetup invocation for every

drive or partition.

The example setups assume you have created a recovery diskette.

II - 1

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                SECTION II

The example setups outlined below are just intended to give you some

ideas about the use of the TBAV utilities, and these examples are not

intended as a full featured protection setup!

2. Prepare a recovery diskette

You will absolutely need a clean recovery diskette in order to be able

to get rid of a virus at all later on. Take a few minutes to prepare one

now well ahead of a possible future contamination when it would be too

late.

Take a new, empty diskette, put it in drive A:, go to your DOS directory

and execute the following commands:

Format A: /S

Copy SYS.COM A:

Now return to the TBAV directory, eg.:

CD \TBAV

Execute the MakeResc batch file:

makeresc A:

The MakeResc batch file will create a reliable recovery diskette by

creating or copying the following files.

-    A backup of the bootsector, partition sector and CMOS configu-

ration.

-    A Config.Sys file, containing:

Files=20

Buffers=20

Device=TbDriver.Exe

Device=TbCheck.Exe FullCRC

-    An Autoexec.Bat file, containing:

@echo off

echo off

PATH=A:\

TBAV

Cls

Echo Warning!!!

Echo If you suspect a virus, do NOT execute anything

Echo from the hard disk!

II - 2

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                SECTION II

-    The following files:

TBAV.EXE

TBAV.LNG

TBSCAN.EXE

TBSCAN.LNG

TBSCAN.SIG

TBDRIVER.EXE

TBDRIVER.LNG

TBCHECK.EXE

TBCLEAN.EX

TBUTIL.EXE

TBUTIL.LNG

Copy to the diskette any other utilities that could come in handy in an

emergency situation, including a simple editor to edit Config.Sys and

AutoExec.Bat files. If your hard disk needs special device drivers to

unlock added features, such as DoubleSpace or Stacker, copy the approp-

riate drivers to the recovery diskette and install them in the Con-

fig.Sys file on drive A:, taking care to avoid statements that will

access the hard disk. Be sure to check the instructions in the device

driver's manual for the correct procedures.

Make the disk write protected. Label the diskette 'Recovery' together

with a short of the PC the diskette belongs to. Store the diskette in a

safe place. Use it only in case of an emergency, so make a copy if you

need a similar diskette for general purposes.

3. Keep shady software out

Many companies do not allow employees to install or execute unauthorized

software. Or perhaps you wish to keep family members from invading your

computer with haphazard games and sundry software. TBAV provides a

watchdog function that can help to enforce this. First you will need to

add the following lines to the Config.Sys file:

Device=C:\TBAV\TbDriver.Exe

Device=C:\TBAV\TbCheck.Exe secure

If you have installed the TBAV Utilities using the TBAV installation

program, you can - instead of editing the CONFIG.SYS file - adjust the

TBSTART.BAT file, appending the 'secure' option to the TbCheck command:

C:\TBAV\TbDriver

C:\TBAV\TbCheck secure

Execute TbSetup on the system:

TbSetup C:\

II - 3

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                SECTION II

Reboot the system by pressing <Ctrl>-<Alt>-<Del>. From now on, TbCheck

will put an effective clamp on any user who tries to execute software

which has not been duly authorized first by TbSetup. Whenever someone is

trying to execute an unknown program, TBAV will display the following

message:

+---------- TBAV interception ---------+

|                WARNING!              |

| The requested program (GAME.EXE)     |

| is not authorized and can not be     |

| executed.                            |

| Execution cancelled! Press any key...|

+--------------------------------------+

4. Restrict user access

Most of the TBAV utilities are interactive. They need to communicate

with a knowledgable user in order to establish appropriate action in

ambiguous situations. Many companies, however, insist that the system

operator be the sole authority allowed to communicate with TBAV and so

avoid wrong decisions on the part of possibly inept employees. That is

why most of TBAV utilities support the option 'secure'. When this option

is specified, all user interaction with any of the TBAV utilities is

suspended. In other words, users will never be queried for permission to

allow questionable operations, avoiding erroneous decisions which may

well result in irreparable havoc. This option will also prevent the user

from disabling or unloading the TBAV utilities.

5. Never use 'strange' diskettes to boot

Only boot from your hard disk or from your original DOS diskette. NEVER

use someone else's disk to boot from. Should you have a hard disk, make

certain that you have opened the door to your floppy drive before

resetting or booting your PC.

6. Use ChkDsk frequently

Use the DOS program ChkDsk frequently (without the /F switch). ChkDsk is

able to detect some viruses, because such viruses change the disk

structure in an incorrect manner, causing disk errors in the process.

Look out for changes in the behaviour of your software or your PC. Any

change in their behaviour is suspect, unless you know its cause. Some

highly suspicious symptoms are:

-    The amount of available memory space has decreased.

-    Programs need more time to execute.

II - 4

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                SECTION II

-    Programs do not operate as they used to, or cause the system to

crash or reboot after some time.

-    Data disappears or gets damaged.

-    The size of one or more programs has increased.

-    The screen behaves strangely, or unusual information is displayed.

-    ChkDsk detects many errors.

II - 5

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                SECTION II

2. What to do when a virus strikes

2.1. Detection of viruses

The first thing to do when you become aware that your system may have

been infected is to back up all important files immediately. Use fresh

backup media and do not overwrite a previous back up set. You may need

the previous set to replace lost or contaminated files. Label the new

backup as unreliable, as some of the files may be infected.

As soon as you become aware of a virus infection it is imperative that

you boot only from a reliable, write protected recovery system diskette.

Know the symptoms

Next, execute TbScan for an indication about what is wrong, or boot from

a recovery diskette and compare its system files with those on the hard

disk to check for changes. During this test you should take care to stay

logged on to your system diskette.

TbScan will report the virus name if the virus is known, or give a

summary of file changes if the virus cannot be identified:

TbScan alldrives logname=lpt1 log

Also execute TbUtil to check the bootsector, partition code and the CMOS

configuration.

TbUtil compare

Do not execute any program on your hard disk to prevent a virus from

invading the system's memory and possibly masking the test results.

TbCheck will warn you if you accidentially try to execute an infected or

unauthorized program on your hard disk.

Please bear in mind that it is in the nature of a file virus to infect

as many programs as possible over a short period. You'll hardly find

only a few infected programs on a hard disk that is in constant use. A

TbScan virus alert flagging a mere one percent of the files on a hard-

worked system is probably just a false alarm that has nothing to do with

a real virus.

If the file compare test indicates that all of them are still the same,

you know at least that you are not dealing with a file virus.

Avoid using the same copy of the TbScan program on another system after

discovering a virus. TbScan performs a sanity check when it fires up.

Unfortunately there is no way to make software 100% virus-resistant. A

sanity check does not work if a 'stealth' type of virus is involved. A

stealth virus can hide itself completely when a self-check is being

performed. Do note that we are not dealing with a TbScan bug here. The

II - 6

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                SECTION II

failure to detect stealth viruses is common to all software performing a

sanity check. Therefore, we recommend to keep a clean version of TbScan

on a write-protected diskette. Use this diskette to check other machines

once you have found a virus in your own system.

Identify the characteristics

Viruses come in many different guises and have their own peculiarities.

It is extremely important to know at the earliest possible stage which

particular kind of virus you are dealing with. That will give you at

least some indication of the nature and the amount of the damage it may

have caused already. Some viruses infect only executable files that can

easily be reinstalled or replaced from a clean source. Others swap some

random bytes anywhere on the hard disk, which could affect data files as

well, although the results may not be noticeable for some time. Then

there are those that damage the hard disk partition table or file

allocation table, while some of the even nastier ones, the multipartite

viruses, operate in more than one area.

Whatever you do, don't panic! An inexperienced user, reacting in confu-

sion, can often create more havoc than the virus itself, such as eradi-

cating important data in no time. While an instant reformat may get rid

of the virus, it will definitely destroy all your recent work as well.

Once isolated the virus, either contact your support BBS, consult

literature on virus problems, or get in touch with a virus expert.

2.2. Recovering from viruses

While recovering from a virus infection it is particularly important to

boot only from a clean write-protected system diskette. That is the only

way to keep a virus out of the system's memory. Never execute a program

from the hard disk.

Restore the master boot sector and the DOS system files on the hard

disk, using the SYS command on the system diskette. If the bootsector or

partition code contains a virus, you may also use TbUtil to get rid of

it by restoring clean sectors:

TbUtil restore

Many modern hard disks, notably IDE or AT drives using advanced prefor-

matting methods, are low-level formatted by the supplier, ready for

partitioning and a DOS format. Do not try to low-level format these

drives yourself. It is always better to back up the partition table with

a utility such as TbUtil, which restores the partition table for you

without reformatting.

If the virus has been identified as a file virus, it will be safest to

remove the infected files (by using TbDel) and to copy or reinstall all

II - 7

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                SECTION II

executables from a clean source. A virus cleaning utility, such as

TbClean, won't always be able to fully restore the original program code

and should be used only as a last resort, such as when you don't have a

reliable backup. It may be necessary to replace data files as well if

the virus is known to cause damage in that area.

After reassuring yourself that the system is absolutely clean again, run

a careful check on all diskettes and backups to remove every single

trace of the virus. Keep in mind that it takes only one infected disket-

te to cause the same trouble all over again.

II - 8

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

SECTION III. USING THE TBAV UTILITIES

1. TbScan

1.1. The Purpose of TbScan

TbScan is a virus scanner: it has been specifically developed to detect

viruses, Trojan Horses and other such threats to your valuable data.

Most viruses consist of a unique sequence of instructions, called a

signature. Hence through checking for the appearance of such signatures

in a file we can find out whether or not a program has been infected.

Scanning all program files for the signatures of all known viruses helps

you to find out quickly whether or not your system has been infected

and, if so, by which virus.

Fast Scanning

TbScan is the fastest scanner on the market today, therefore it invites

users to invoke it from within their AUTOEXEC.BAT file every morning.

TbScan checks itself on invocation. If it detects that it has been

infected it aborts with an error. This minimizes the risk of transfer-

ring a virus by the TbScan program itself and infecting your system.

Heuristic Scanning

TbScan can detect yet unknown viruses. The built-in disassembler is able

to detect suspicious instruction sequences and abnormal program lay-

outs. This feature is called 'heuristic scanning' and it is partially

enabled by default. Heuristic scanning is performed on files and boots-

ectors.

=>   Note that virus scanners can only tell you whether or not your system

has been infected and if so, if any damage has already been done. By

that time only a non-infected backup or a recovery program such as

TbClean can properly counter a virus infection.

Scan Scheduling

Every PC owner should use a virus scanner frequently. At least one

should do to avoid damage caused by a virus. It is highly recommended to

devise your own schedule for a regular scan of your system. Creating a

special TbScan boot diskette is also recommended in this respect.

The following scan sessions (listed in order of preference) are recom-

mended:

Execute TbScan from a write-protected bootable diskette once a week.

Boot from this diskette before invoking the scanner. Booting from a

III - 1

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

diskette is the only way to make sure that no stealth virus will become

resident in memory.

Invoke a daily scan. You can invoke TbScan with the  'once' option from

within the autoexec.bat file to perform the daily scan session automati-

cally. It is not necessary to boot from the bootable TbScan diskette to

perform the daily scan.

Scan new diskettes.

1.2. How to use Tbscan

For daily use you can activate TbScan by loading the program from the

DOS commandline (eg. in the autoexec.bat file), or via the TBAV menu.

For weekly use, when scanning from the TbScan diskette, you could use

the DOS command. The TbScan DOS options are listed in section 1.3. of

this chapter.

The 'TbScan' menu

+----Main menu-----+

|  Confi+----TbScan menu-----+

|  TbSet|  Start scanning   >|

|  TbSca|  Options menu     >|

|  TbUti|  Advanced options >|

|  TbCle|  If virus found   >|

|  TBAV |  Log file menu    >|

|  Docum|  View log file     |

|  Regis+--------------------+

|  Quit and save   |

|  eXit (no save)  |

+------------------+

View log file

If one of the log file options is activated (see below) you can study

the log file using this option.

III - 2

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

The 'Path' menu

+----Main menu-----+

|  Confi+----TbScan menu-----+

|  TbSet|  Start+---------Path menu----------+

|  TbSca|  Optio|  Specified files/paths     |

|  TbUti|  Advan|  Current directory         |

|  TbCle|  If vi|  Diskette in drive A:      |

|  TBAV |  Log f|  Diskette in drive B:      |

|  Docum|  View |  All fixed Drives          |

|  Regis+-------|  All fixed Local drives    |

|  Quit and save|  All fixed Network drives  |

|  eXit (no save+----------------------------+

+------------------+

This menu may look different depending on your use of .SCN (configurable

scan) files. The lay-out above appears when you don't have any .SCN

files at all.

Specified files/paths

This option will always present you with a prompt at which you can

specify the drives, paths, or even files you want to scan. You can

specify multiple path specifications separated by spaces. The field is

initialized with the last path you scanned before you saved your confi-

guration.

Current directory

Select this option if you want to scan the directory on your system from

where you called the TBAV menu shell.

Diskette in drive A: (or B:)

If you want to scan multiple diskettes, you may wish to activate the

'repeat' option of TbScan. See the 'TbScan options' menu for more

information.

All fixed drives

This will cause all available drives (except the removable ones) to be

scanned completely. Depending on the settings in the TBAV configuration

menu you will be prompted to confirm the selected drives.

All fixed Local drives

III - 3

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

If you are connected to a network, you probably don't want to scan the

entire network always. With this option you just scan the drives which

reside in your machine. Depending on the settings in the TBAV configura-

tion menu you will be prompted to confirm the selected drives.

All fixed Network drives

With this option you can scan all network drives. Depending on the

settings in the TBAV configuration menu you will be prompted to confirm

the selected drives.

The 'TbScan options' menu

+----Main menu-----+

|  Confi+------TbScan menu------+

|  TbSet|  Start+-----TbScan options------+

|  TbSca|  Optio|  Use TBAV.INI file      |

|  TbUti|  Advan|  Prompt for pause       |

|  TbCLe|  If vi|  Quick scan             |

|  TBAV |  Log f|  Non-executable scan    |

|  Docum|  View |  Maximum Compatibility  |

|  Regis+-------|v Bootsector scan        |

|  Quit and save|v Memory scan            |

|  eXit (no save|  HMA scan forced        |

+---------------|v Upper memory scan      |

|v Windows-OS/2-virus scan|

|v Sub-Directory scan     |

|  Repeat scanning        |

|v Abort on Ctrl-Break    |

|v Fast scrolling         |

+-------------------------+

Use TBAV.INI file

TbScan searches for a file named TBAV.INI in the TbScan directory. By

enabling this option, the TbScan configuration values, saved in the

TBAV.INI file, will also be valid when loading TbScan from the command

line. Be careful, since options specified in the TBAV.INI file can not

be undone on the command line. See chapter I-2 ('Configuration').

Prompt for pause

When you activate the 'pause' option TbScan will stop after it has

checked the contents of one window. This gives you the possibility to

examine the results without having to consult a log file afterwards.

III - 4

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

Quick scan

TbScan will use the Anti-Vir.Dat files to check for file changes since

the last time. Only if a file has been changed (CRC change) or is not

yet listed in Anti-Vir.Dat it will be scanned. Normally TbScan will

always scan files.

Non-executable scan

With this option TbScan will scan non-executable files (files without

extension COM, EXE, SYS or BIN) too. If TbScan finds out that such a

file does not contain anything that can be executed by the processor the

file will be 'skipped'. Otherwise the file will be searched for COM, EXE

and SYS signatures. TbScan however will not perform heuristic analysis

on non-executable files. Since viruses normally do not infect non-

executable files it is not necessary to scan non-executable files too.

We even recommend not to use this option unless you have a good reason

to scan all files.

Once again: a virus needs to be executed to perform what it is program-

med to do, and since non-executable files will not be executed a virus

in such a file can not do anything. For this reason viruses do not even

try to infect such files. Some viruses however will write to non-execu-

table files as a result of 'incorrect' programming. If so, these non-

executable files will never harm other program or data files, but do

contain corrupted data.

Maximum compatibility

If you select this option, TbScan attempts to be more compatible with

your system. Use this option if the program does not behave as you would

expect, or even halts the system. This option will slow down the scan-

ning process. Therefore, it should only be used if necessary.

=>   Note that this option does not affect the results of a scan.

Bootsector scan

Enabling this option will force TbScan to scan the bootsector as well.

Memory scan

Enabling this option will force TbScan to scan the memory of the PC.

III - 5

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

HMA scan forced

TbScan detects the presence of an XMS-driver, and scans HMA automa-

tically. If you have an HMA-driver which is not compatible with the XMS

standard you can use the 'HMA' option to force TbScan to scan HMA.

Upper memory scan

By default TbScan identifies RAM beyond the DOS limit and scans that

too. This means that video memory and the current EMS pages are scanned

by default. You can use this option to enable the scanning of non-DOS

memory.

Windows-OS/2-virus scan

TbScan also searches for Windows and OS/2 viruses. Although there exist

only very few of these viruses, it takes TbScan relatively a lot of time

to search for them. If you don't use Windows or OS/2 you can speed up

TbScan by disabeling the detection of Windows and OS/2 virus detection.

Subdirectory scan

By default TbScan will search sub-directories for executable  files,

unless a filename (wildcards allowed!) is specified. If you disable this

option, TbScan will not scan sub-directories.

Repeat scanning

This option is very useful if you want to check a large amount of

diskettes. TbScan does not return to DOS after checking a disk, but it

prompts you to insert another disk in the drive.

Abort on Ctrl-Break

You can disable this option if you don't want to be able to abort TbScan

with Ctrl-Break.

Fast scrolling

TbScan shows the processed file in a scrolling window. There are two

methods of scrolling: fast scrolling where the files are displayed on

top of the previous ones if the window becomes filled, and the conventi-

onal slow method of scrolling where the files at the bottom 'push up'

the previous ones. By default TbScan uses the faster but less attractive

method of scrolling.

III - 6

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

The 'TbScan advanced options' menu

+----Main menu-----+

|  Confi+------TbScan menu------+

|  TbSet|  Start+------TbScan advanced options-----+

|  TbSca|  Optio|  High heuristic sensitivity      |

|  TbUti|  Advan|v Auto heuristic sensitivity      |

|  TbCLe|  If vi|  Low heuristic sensitivity       |

|  TBAV |  Log f|  Extract signatures              |

|  Docum|  View |  Configure executable extensions |

|  Regis+-------+----------------------------------+

|  Quit and save   |

|  eXit (no save)  |

+------------------+

High heuristic sensitivity

TbScan always performs a heuristic scan on the files being processed.

However, only if a file is very probably infected with a virus, TbScan

will report the file as being infected. If you use option 'heuristic',

TbScan is somewhat more sensitive. In this mode 90% of the new, unknown,

viruses will be detected without any signature, but some false alarms

may occur. Consult also section 'Heuristic scanning' of this chapter

(3.1).

Auto heuristic sensitivity

TbScan automatically adjusts the heuristic detection level after a virus

has been found. This provides you maximum detection capabilities in case

you need it, while the amount of false alarms due to heuristics remains

small in normal situations. In other words: as soon as a virus has been

found, TbScan will anticipate and proceed as if 'high heuristic sensiti-

vity' has been selected.

Low heuristic sensitivity

In this mode TbScan will almost never issue a false alarm. However still

about 50% of the new unknown viruses will be detected.

Extract signatures

This option is available to registered users only. See the chapter

'TbGensig' (IV-5) on how to use the option 'extract'.

Configure executable extensions

III - 7

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

By default, TbScan only scans file with a filename extension which

indicates that the file is a program file. Viruses which do not infect

executable code simply do not exist. Files with the extension EXE, COM,

BIN, SYS, OV? are considered to be executable.

However, there are some additional files which have an internal layout

that makes them suitable for infection by viruses. Although it is not

likely that you will ever execute most of these files, you may want to

scan them anyway.

Some filename extensions that may indicate an executable format are:

.DLL.SCR.MOD.CPL.00?.APP

The first four extensions indicate Windows executable files. They

normally display "This program requires Microsoft Windows" when you try

to execute them, so you probably won't run these files often under DOS.

Even when they are infected by a DOS virus they are not likely a threat

since you don't execute them. Therefore TbScan does not scan them by

default. To make TbScan scan these files by default, select this option

and fill out the extensions you want to have scanned. The question mark

as wildcard is allowed.

Warning! Be careful about which extensions you specify: scanning a non-

executable file causes unpredicatble results, and may result in false

alarms.

The 'If virus found' menu

In this menu, you can configure the actions TbScan should take, if

detecting a virus.

+----Main menu-----+

|  Confi+------TbScan menu------+

|  TbSet|  Start+--What if a virus is found?--+

|  TbSca|  Optio|v Present action menu        |

|  TbUti|  Advan|  Just continue (log only)   |

|  TbCLe|  If vi|  Delete infected file       |

|  TBAV |  Log f|  Kill infected file         |

|  Docum|  View |  Rename infected file       |

|  Regis+-------+-----------------------------+

|  Quit and save   |

|  eXit (no save)  |

+------------------+

Present action menu

If TbScan detects a virus, the program will display a menu containing

the possible actions to be taken: just continu, delete or rename the

infected file.

III - 8

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

Just continue (log only)

If TbScan detects an infected file it prompts the user to delete or

rename the infected file, or to continue without action. If you select

this option, TbScan will always continue. We highly recommend you to use

a log file in such situations, as a scanning operation does not make

much sense without the return messages being read (see 'Command line

options').

Delete infected file

If TbScan detects a virus in a file it prompts the user to delete or

rename the infected file, or to continue without action. If you specify

the 'delete' option, TbScan will delete the infected file automatically,

without prompting the user first. Use this option if you have determined

it is a virus infection. Make sure that you have a clean back-up, and

that you really want to get rid of all infected files at once.

Kill infected file

This option is nearly the same as the 'delete' option. However, with the

DOS 'undelete' program you can recover a deleted file, but if a file has

been deleted with the 'kill' option, recovery is not possible anymore.

Rename infected file

If TbScan detects a file virus it prompts the user to delete or rename

the infected file, or to continue without action. If you select the

'rename' option, TbScan will rename the infected file automatically,

without prompting the user first. By default, the first character of the

file extension will be replaced by the character 'V'. An .EXE file will

be renamed to .VXE, and a .COM file to .VOM. This prevents the infected

programs from being executed, spreading the infection. At the same time

they can be kept for later examination and repair.

III - 9

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

The 'TbScan LOG' menu

+----Main menu-----+

|  Confi+------TbScan menu------+

|  TbSet|  Start+-------TbScan LOG menu-------+

|  TbSca|  Optio|  Log file path/name         |

|  TbUti|  Advan|  Output to log file         |

|  TbCLe|  If vi|  Specify log-level         >|

|  TBAV |  Log f|  Append to existing log     |

|  Docum|  View |  No heuristic descriptions  |

|  Regis+-------+-----------------------------+

|  Quit and save   |

|  eXit (no save)  |

+------------------+

Log file path/name

With option logname you can specify the name of the log file to be used.

TbScan will create the file in the current directory unless you specify

a path and filename after selecting this option. If the log file already

exists, it will be overwritten. If you want to print the results, you

can specify a printer device name rather than a filename (logname=lpt1).

=>   Note: you have to combine this option with option 'log'.

Output to logfile

When you use this option, TbScan creates a log file. The log file lists

all infected program files, specifying heuristic flags (see: appendix B)

and complete pathnames.

Append to existing log

If you use this option, TbScan will not overwrite an existing log file

but append the new information to it. If you use this option often, it

is recommended to delete or truncate the log file once in a while to

avoid unlimited growth.

=>   Note: you have to combine this option with option 'log'.

No heuristic descriptions

If you enable this option TbScan will not specify the descrip-tions of

the heuristic flags in the log file. The heuristic flag descriptions are

listed in appendix B.

III - 10

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

The 'LOG level' menu

+----Main menu-----+

|  Confi+------TbScan menu------+

|  TbSet|  Start+-------TbScan LOG menu-------+

|  TbSca|  Optio|  Log f+--------Log-level menu--------+

|  TbUti|  Advan|  Outpu|  0: Log only infected files  |

|  TbCLe|  If vi|  Speci|v 1: Log summary too          |

|  TBAV |  Log f|  Appen|  2: Log suspected too        |

|  Docum|  View |  No he|  3: Log all warnings too     |

|  Regis+-------+-------|  4: Log clean files too      |

|  Quit and save   |    +------------------------------+

|  eXit (no save)  |

+------------------+

Loglevel

These levels determine what kind of file information will be stored in

the log file. The default log level is 1. You may select one of five log

levels:

0    Log only infected files. If there are no infected files do not

create or change the log file.

1    Log summary too. Put a summary and timestamp in the log file. Put

only infected files in the log file.

2    Log suspected too. Same as loglevel=1, but now also 'suspected'

files are logged. Suspected files are files that would trigger the

heuristic alarm if option 'heuristic' had been specified.

3    Log all warnings too. Same as loglevel=2, but all files that have a

warning character printed behind the filename will be logged too.

4    Log clean files too. All files being processed will be put into the

log file.

1.3. Command line options

When loaded from the DOS command line, Tbscan recognizes option short-

keys and option words. The words are easier to memorize, and they will

be used in this manual for convenience. TbScan searches for a file named

TBAV.INI in the TbScan directory.

If the keyword 'UseIni' is specified in the [TbScan] section of the

TBAV.INI file, the options will also be valid when TbScan is invoked

from the command line. Be careful, as options specified in the TBAV.INI

file can not be undone on the command line.

III - 11

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

option parameter        short  explanation

-----------------------------------------------------------------

help                    he     help

pause                   pa     enable 'Pause' prompt

mono                    mo     force monochrome

quick                   qs     quick scan (uses Anti-Vir.Dat)

allfiles                af     scan non-executable files too

alldrives               ad     scan all local non-removable drives

allnet                  an     scan all network drives

heuristic               hr     enable heuristic alerts

extract                 ex     extract signature (registered only)

once                    oo     only once a day

slowscroll              ss     enable conventional (slow) scrolling

secure                  se     user abort not allowed (reg. only)

compat                  co     maximum-compatibility mode

ignofile                in     ignore no-file-error

old                     ol     disable "this program is old" message

noboot                  nb     skip bootsector check

nomem                   nm     skip memory check

hma                     hm     force HMA scan

nohmem                  nh     skip UMB/HMA scan

nosub                   ns     skip sub-directories

noautohr                na     auto heuristic level adjust

nowin                   nw     do not scan for Windows/OS2 viruses

repeat                  rp     scan multiple diskettes

batch                   ba     batch mode. No user input

delete                  de     delete infected files

kill                    ki     kill infected files

log                     lo     output to logfile

append                  ap     log file append mode

expertlog               el     no heuristic descriptions in log

logname  =<filename>    ln     set path/name of log file

loglevel =<0..4>        ll     set log level

wait     =<0...255>     wa     amount of timerticks to wait

rename   [=<text-mask>] rn     rename infected files

exec     =.<ext-mask>   ee     specify executable extensions

You can find an explanation on most of the command line options at the

similar menu descriptions presented above.

help (he)

If you specify this option TbScan will display the help as listed above.

pause (pa)

When you activate the 'pause' option TbScan will stop after it has

checked the contents of one window. This gives you the possibility to

examine the results without having to consult a log file afterwards.

III - 12

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

mono (mo)

This option forces TbScan to refrain from using colors in the screen

output. This might enhance the screen output on some LCD screens or

color-emulating monochrome systems.

quick (qs)

TbScan will use the Anti-Vir.Dat files to check for file changes since

the last time only. Only if a file has been changed (CRC change) or is

not yet listed in Anti-Vir.Dat it will be scanned. Normally TbScan will

always scan files.

allfiles (af)

With this option TbScan will scan non-executable files (files without

extension COM, EXE, SYS or BIN) too. If TbScan finds out that such a

file does not contain anything that can be executed by the processor the

file will be 'skipped'. Otherwise the file will be searched for COM, EXE

and SYS signatures. TbScan however will not perform heuristic analysis

on non-executable files.

Since viruses normally do not infect non-executable files it is not

necessary to scan non-executable files too. We even recommend not to use

this option unless you have a good reason to scan all files. Once again:

a virus needs to be executed to perform what it is programmed to do, and

since non-executable files will not be executed a virus in such a file

can not do anything. For this reason viruses do not even try to infect

such files. Some viruses however will write to non-executable files as a

result of 'incorrect' programming. If so, these non-executable files

will never harm other program or data files, but do contain corrupted

data.

alldrives (ad)

This option causes TbScan to scan all local non-removabe disks.

allnet (an)

This option causes TbScan to scan all network drives.

heuristic (hr)

TbScan always performs a heuristic scan on the files being processed.

However, only if a file is very probably infected with a virus, TbScan

will report the file as being infected. If you use option 'heuristic',

TbScan is somewhat more sensitive. In this mode 90% of the new, unknown,

viruses will be detected without any signature, but some false alarms

may occur. Consult also section 'Heuristic scanning' of this chapter

(3.1).

extract (ex)

This option is available to registered users only. See the chapter

'TbGensig' (IV-5) on how to use the option 'extract'.

once (oo)

If you specify this option TbScan will 'remember' after its scan that is

has been executed that day, and that it should not be executed again the

III - 13

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

same day with this particular option set. This option is very useful if

you incorporate it in your AUTOEXEC.BAT file in combination with a scan

list file (.SCN):

TbScan @Everyday.Scn once rename

TbScan will now scan the list of files and/or paths specified in the

file EVERYDAY.SCN during the first boot-up of the day.

If the systems boots more often that day, TbScan will then return to DOS

immediately.  This option does not interfere with the regular use of

TbScan. If you invoke TbScan without the 'once' option it will always be

executed, regardless of a previous run with the 'once' option set.

=>   Note that if TbScan cannot write to TBSCAN.EXE because it has been

flagged 'read-only' or is located on a write-protected diskette, the

'once' option will fail and the scanner will be executed without it.

slowscroll (ss)

If you specify this option, TbScan will scroll the files in the files

window conventionally. This method is slower but looks nicer.

secure (se)

This option is available to registered users only. If this option is

specified it is no longer possible to cancel TbScan by pressing Ctrl-

Break, or to respond to a virus alert window.

compat (co)

If you select this option, TbScan attempts to be more compatible with

your system. Use this option if the program does not behave as you would

expect, or even halts the system. This option will slow down the scan-

ning process. Therefore, it should only be used if necessary.

Note that this option does not affect the results of a scan.

ignofile (in)

If this option is specified and no files can be found, TbScan will not

display the 'no files found' message, nor does it exit with errorlevel

1. This option might be useful for automatic contents scanning.

old (ol)

This option suppresses the message which appears if TbScan is 6 months

old.

noboot (nb)

If you specify this option TbScan will not scan the bootsector.

nomem (nm)

If you specify this option TbScan will not scan the memory of the PC for

viruses.

III - 14

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

hma (hm)

TbScan detects the presence of an XMS-driver, and scans HMA automa-

tically. If you have an HMA-driver which is not compatible with the XMS

standard you can use the 'HMA' option to force TbScan to scan HMA.

nohmem (nh)

By default TbScan identifies RAM beyond the DOS limit and scans that

too. This means that video memory and the current EMS pages are scanned

by default. You can use this option to disable the scanning of non-DOS

memory.

nosub (ns)

By default TbScan will search sub-directories for executable  files,

unless a filename (wildcards allowed!) is specified. If you enable this

option, TbScan will not scan sub-directories.

noautohr (na)

TbScan automatically adjusts the heuristic detection level after a virus

has been found. This provides you maximum detection capabilities in case

you need it, while the amount of false alarms due to heuristics remains

small in normal situations. In other words: as soon as a virus has been

found, TbScan will anticipate and proceed as if option 'heuristic' has

been speci-fied. If you don't want this, you can specify option 'noau-

tohr'.

nowin (nw)

TbScan also searches for Windows and OS/2 viruses. Although there exist

only very few of these viruses, it takes TbScan relatively a lot of time

to search for them. If you don't use Windows or OS/2 you can speed up

TbScan by disabeling the detection of Windows and OS/2 virus detection.

repeat (rp)

This option is very useful if you want to check a large amount of

diskettes. TbScan does not return to DOS after checking a disk, but it

prompts you to insert another disk in the drive.

batch (ba)

By enabling this option TbScan will scan without displaying any messa-

ges. Therefore, the use of a LOG file is highly

advisable.

delete (de)

If TbScan detects a virus in a file it prompts the user to delete or

rename the infected file, or to continue without action. If you specify

the 'delete' option, TbScan will delete the infected file automatically,

without prompting the user first. Use this option if you have determined

it is a virus infection. Make sure that you have a clean back-up, and

that you really want to get rid of all infected files at once.

kill (ki)

If TbScan detects a virus in a file it prompts the user to deleter or

rename the infected file, or to continue without action. If you specify

III - 15

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

the 'kill' option, TbScan will delete the infected file automatically,

without prompting the user first. However, unlike the 'delete' option,

files which have been killed can not be undeleted anymore. Be careful if

you use this option. Make sure you have a clean back-up!

log (lo)

When you use this option, TbScan creates a log file. The log file lists

all infected program files, specifying heuristic flags (see: appendix B)

and complete pathnames.

append (ap)

If you use this option, TbScan will not overwrite an existing log file

but append the new information to it. If you use this option often, it

is recommended to delete or truncate the log file once in a while to

avoid unlimited growth.

=>   Note: you have to combine this option with option 'log'.

expertlog (el)

If you enable this option TbScan will not specify the descriptions of

the heuristic flags in the log file. The heuristic flag descriptions are

listed in appendix B.

logname =<filename> (ln)

With option logname you can specify the name of the log file to be used.

TbScan will create the file in the current directory unless you specify

a path and filename after selecting this option. If the log file already

exists, it will be overwritten. If you want to print the results, you

can specify a printer device name rather than a filename (logname=lpt1).

=>   Note: you have to combine this option with option 'log'.

loglevel =<0..4> (ll)

These levels determine what kind of file information will be stored in

the log file. The default log level is 1. You may select one of five log

levels:

0    Log only infected files. If there are no infected files do not

create or change the log file.

1    Log summary too. Put a summary and timestamp in the log file. Put

only infected files in the log file.

2    Log suspected too. Same as loglevel=1, but now also 'suspected'

files are logged. Suspected files are files that would trigger the

heuristic alarm if option 'heuristic' had been specified.

3    Log all warnings too. Same as loglevel=2, but all files that have a

warning character printed behind the filename will be logged too.

4    Log clean files too. All files being processed will be put into the

log file.

=>   Note: you have to combine this option with option 'log'.

III - 16

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

wait =<0..255> (wa)

This option can be used to delay TbScan, which may be handy if you want

to scan a very busy network and you don't want to occupy the network to

havily. You have to specify the amount of timer ticks you want to insert

between every two files scanned.

rename [=<text-mask>] (rn)

If TbScan detects a file virus it prompts the user to delete or rename

the infected file, or to continue without action. If you select the

'rename' option, TbScan will rename the infected file automatically,

without prompting the user first. By default, the first character of the

file extension will be replced by the character 'V'. An .EXE file will

be renamed to .VXE, and a .COM file to .VOM. This prevents the infected

programs from being executed, spreading the infection. At the same time

they can be kept for later examination and repair.

You may also add a parameter to this option specifying the target

extension. This parameter should always contain 3 characters; question

marks are allowed. The default target extension is 'V??'.

exec =.<ext-mask> (ee)

With this option you can add filename extensions which indicate that a

file is executable. If you want to use this option, you probably want to

put it in the configuration file. Consult the explanation of the similar

menu option for more details.

Examples:

TbScan c:\ noboot

Process all executable files in the root directory and its subdirecto-

ries. Skip the bootsector scan.

TbScan \*.*

Process all files in the root directory. Don't process subdirectories.

TbScan @Local.Scn /nomem

Process all paths specified in the file Local.Scn and skip memory

scanning.

TbScan c:\ log logname=c:\test.log loglevel=2

All executable files on drive C: will be checked. A LOG file with the

name c:\test.log will be created. The log file will contain all infected

and suspected files.

TbScan \ log logname=lpt1

TbScan will scan the root directory and its subdirectories. The results

are redirected to the printer rather than to a log file.

III - 17

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

1.4. The scanning process

Choose the 'Start scanning' option in the TbScan menu or start the

TbScan program from the DOS command line. TbScan will start scanning

right away.

+-----------------------------------------------------------------+

|Thunderbyte virus detector v6.31 - (C) 1989-95, Thunderbyte B.V. |

|                                                                 |

| TBAV is upgraded every two months. Free hotline support is      |

| provided for all registered users via telephone, fax and        |

| electronic bulletin board. Read the comprehensive documentation |

| files for detailed info. BBS: +31- 59- 182 011                  |

|                                                                 |

| C:\DOS\                                                         |

| ** Unregistered evaluation version. Don't forget to register! **|

|                                                                 |

| ANSI.SYS      scanning..>        OK    signatures:        986   |

| COUNTRY.SYS   skipping..>        OK                             |

| DISKCOPY.COM  tracing...>        OK    file system:       OWN   |

| DISPLAY.SYS   scanning..>        OK                             |

| DRIVER.SYS    scanning..>        OK    directories:        01   |

| EGA.CPI       skipping..>        OK    total files:        17   |

| FASTOPEN.EXE  looking...>        OK    executables:        12   |

| FDISK.EXE     looking...>        OK    CRC verified:       10   |

| FORMAT.COM    tracing...>   E    OK    changed files:      00   |

| GRAFTABL.COM  tracing...>        OK    infected items:     00   |

| GRAPHICS.COM  tracing...>        OK                             |

| GRAPHICS.PRO  skipping..>        OK    elapsed time:    00:05   |

|                                        Kb /second:        57    |

|                                                                 |

+-----------------------------------------------------------------+

TbScan divides the screen into three windows: an information window, a

scanning window and a status window. The information window will initi-

ally display the vendor information only.

While Scanning

If TbScan detects infected files the names of the file and the virus

will be displayed in the upper window. The lower left window displays

the names of the files being processed, the algorithm in use, info and

heuristic flags, and finally an OK statement or the name of the virus

detected.

III - 18

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

Example: NLSFUNC.EXE     checking..>    FU          OK

|              |            |           |

|              |            |           result of scan

|              |            heuristic flags

|              algorithm being used to process file

name of file in process

You will see comments following each file name: 'looking', 'checking',

'tracing', 'scanning' or 'skipping'. These refer to the various algo-

rithms being used to scan files.

Other comments that TbScan can display here are the heuristic flags.

Consult the 'Heuristic flags' chapter (1.3) for more information on

these warning characters.

The lower right window is the status window. It displays the number of

files and directories encountered, the amount of viruses found. It also

displays which file system is being used: either "DOS" or "OWN". The

latter means that TbScan is able to bypass DOS. If this is the case,

TbScan reads all files directly from disk for extra security and speed.

The scanning process can be aborted by pressing Ctrl-Break.

Detecting Viruses

As soon as an infected program is found, TbScan will display the name of

the virus. If you did not specify one of the  options 'batch', 'rename'

or 'delete', TbScan will prompt you to specify the appropriate action.

If you choose to rename the file, the first character of the file

extension will be replaced by the character 'V'. This prevents the file

from being executed by accident before it has been investigated more

thoroughly.

If an infected file is detected, TbScan will display a message:

Infected by [name of virus] virus.

The file is infected by the virus mentioned.

Is Joke named [name of Joke]

There are some programs which simulate that the system is infected

by a virus. A joke is completely harmless.

Is Trojan named [name of Trojan]

The file is a Trojan Horse.Do not execute the program but delete

it.

Damaged by [name of virus]

A damaged file contains - unlike an infected file - not the virus

itself, but has been damaged by the virus.

III - 19

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

Dropper of [name of virus]

A dropper is a program that has not been infected itself, but which

does contain a bootsector virus and is able to install it in your

bootsector.

Overwritten by [name of virus]

Some viruses overwrite files. An overwritten file contains - unlike

an infected file - not the virus itself, but has been overwritten

with garbage.

It is also possible that TbScan encounters a file that seems to be

infected by a virus, although a signature could not be found. In this

case TbScan displays the prefix 'Probably' before the message.

Program Validation

If TbScan finds a file to be very suspicious and pops up with the virus

alert window, you can avoid future false alarms by pressing 'V' (Valida-

te program). Note that this only works if there is an Anti-Vir.Dat

record of the file available. Once a program is validated it will no

longer be subject to heuristic analysis, unless the program changes and

does not match the Anti-Vir.Dat record anymore. This will be the case if

such a file gets infected afterwards, so TbScan will still report

infections on these files.

=>   Note that a validated program is still subject to the conventional

signature scanning.

Heuristic Scanning

If you have specified the option 'heuristic' it is likely that TbScan

will find some files which look like a virus, and in this case TbScan

uses the prefix 'Might be' to inform you about it. So, if TbScan dis-

plays:

Probably infected by an unknown virus (level 1)

or:

Might be infected by an unknown virus (level 2)

it does not necessarily mean that the file is infected. There are a lot

of files that look like a virus but are not.

=>   The heuristic levels are explained in section IV (page 9).

False Positives

=>   Important!

False alarms are part of the nature of heuristic scanning. In default

mode it is very unlikely that TbScan issues a false alarm. However, if

III - 20

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

you have specified option 'heuristic' some false alarms might occur. How

to deal with these false alarms? If TbScan thinks it has found a virus

it tells you the reason for this suspicion. In most cases you will be

able to evaluate these reasons when you consider the purpose of the

suspected file.

=>   Note that viruses infect other programs. It is highly unlikely that you

will find only a few infected files on a hard disk used frequently. You

should ignore the result of a heuristic scan if only a few programs on

your hard disk trigger it. But, if your system behaves in a 'strange'

manner and many programs cause TbScan to issue an alarm with the same

serious flags, your system could very well be infected by a (yet un-

known) virus.

Heuristic flags

Heuristic flags consist of single characters that are printed behind the

name of the file that has been processed. There are two kinds of flags:

the informative ones are printed in lower-case characters; the more

serious flags are printed in upper-case characters.

The lower-case flags are indicative of special characteristics of the

file being processed, whereas the upper-case warnings may indicate a

virus. If the 'loglevel' is 3 or above, the important warnings will not

only appear as a warning character, but there will also be a description

printed in the log file.

How should you treat the flags? The less important lower-case flags can

be considered to be for your information only.They provide you with file

information you might find interesting. The more serious warning flags

printed in upper-case MIGHT point towards a virus. It is quite normal

that you have some files in your system which trigger an upper-case

flag.

The heuristic flag descriptions are listed in appendix B.

III - 21

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

2. TbScanX

2.1. The Purpose of TbScanX

TbScanX is the resident version of the TbScan program, checking files on

the basis of a virus signature list. Suppose you have a virus scanner

automatically executed from your autoexec.bat file. If no viruses are

found, your system is supposed to be uninfected. But, to be sure that no

virus will infect your system, you have to execute the scanner every

time after copying a file to your harddisk, after downloading a file

from a bulletin board system, or after unarchiving an archive such as a

ZIP file. Be honest, do YOU actually invoke your scanner every time you

introduce a new file into the system? If you don't, you take the risk

that within a couple of hours all files are infected by a virus...

Once loaded, TbScanX will remain resident in memory, and will automati-

cally scan all files you execute and all executable files you copy,

create, download, modify, or unarchive. The same approach is used to

protect against bootsector viruses: every time you put a diskette into a

drive the bootsector will be scanned. If the disk is contaminated with a

boot sector virus TbScanX will warn you in time!

TbScanX is fully network compatible. It does not require to reload the

scanner after logging on to the network.

2.2. How to use TbScanX

Since TbScanX is memory resident, the program can be executed and

configured from the command line or from within a batch file. It is

important to load TbScanX as early as possible after the machine has

booted. Therefore it is recommended to execute TbScanX from within the

Config.Sys file.

=>   Note that TbScanX requires TbDriver to be loaded first!

Loading TBScanX

There are three possible ways to load TbScanX:

1. From the DOS prompt or within the Autoexec.Bat file:

<path>TbScanX

2. From the Config.Sys as a TSR (Dos 4+):

Install=<path>TbScanX.Exe

The "Install=" Config.Sys command is NOT available in DOS 3.xx.

3. From the Config.Sys as a device driver:

Device=<path>TbScanX.Exe

III - 22

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

Unlike other anti-virus products, the ThunderBYTE anti-virus utlities

can be loaded before the network is started without loosing the protec-

tion afterwards.

Highload TBScanX

In addition to the three invocation possibilities users of DOS 5 and

higher versions can "highload" TbScanX in UMB (upper memory block) if it

is available:

LoadHigh <path>TbScanX.Exe

Within the Config.Sys file TbScanX can also be loaded high:

DeviceHigh=<path>TbScanX.Exe

TbScanX and MS-Windows

Windows users should load TbScanX BEFORE starting MS-Windows. If you do

that there is only one copy of TbScanX in memory, but every DOS-window

will nevertheless have a fully functional TbScanX in it. TbScanX detects

if Windows is starting up, and will switch itself in multitasking mode

if necessary. You can even disable TbScanX in one window without affec-

ting the functionality in another window.

2.3. Command line options

TbScanx can be configured from the command line. The upper four options

are always available, the other options are only available if TbScanX is

not already resident in memory.

III - 23

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

option  short  explanation

--------------------------------------------------

help      ?    display this helpscreen

off       d    disable scanning

on        e    enable scanning

remove    r    remove TbScanX from memory

noexec    n    never scan at execute

allexec   a    always scan at execute

noboot    b    do not scan bootsectors

wild      w    only viruses which appear in the wild

ems       me   use expanded memory (EMS)

xms       mx   use extended memory (XMS)

secure    s    deny access without asking

lock      l    lock PC when virus detected

api       i    load Application Program Interface

compat    c    increased compatibility

help (?)

If you specify this option TbScanX will show you the commandline options

as shown above. Once TbScanX has been loaded the help option will not

show all options anymore.

off (d)

If you specify this option TbScanX will be disabled, but it will remain

in memory.

on (e)

If you use this option TbScanX will be activated again after you dis-

abled it with the 'off' option.

remove (r)

This option can be used to remove the resident part of TbScanX from your

memory. All memory used by TbScanX will be released. Unfortunately,

removing a TSR (like TbScanX) is not always possible. TbScanX checks

whether it is safe to remove the resident part from memory, if it is not

safe it just disables TbScanX. A TSR can not be removed if another TSR

is started after it. If this happens with TbScanX it will completely

disable itself.

compat (c)

In most systems TbScanX performs troublefree. It is possible that

TbScanX conflicts with other EMS or XMS using resident software. In this

case the system will hang. Option 'compat' might solve this problem, but

due to extensive memory swapping the performance of TbScanX will slow

down.

III - 24

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

noexec (n)

TbScanX normally scans files located on removable media just before they

are executed. You can use this option to disable this feature complete-

ly.

allexec (a)

TbScanX normally scans files to be executed only if they reside on

removable media. Files on the harddisk are trusted, because these files

must have been copied or downloaded before. And by that time TbScanX has

already scanned them automatically. However if you want every file to be

scanned before executing, no matter whether on harddisk or removable

media, you should use this option.

noboot (b)

TbScanX monitors the disk system: every time the bootsector is being

read, TbScanX automatically scans the disk for bootsector viruses. If

you change a disk, the first thing DOS has to do is read the bootsector,

otherwise it does not know what kind of disk is in the drive. And as

soon as DOS reads the bootsector, TbScanX checks it for viruses. If you

don't like this feature, or if it causes problems, you can switch it off

using the 'noboot' option. This option will also save some memory

because the bootsector signatures will not be loaded.

wild (w)

There are many viruses which do not appear 'in the wild'. They only run

on ancient machines or DOS versions, or they are so buggy that they can

only survive in 'virus-laboratory' environments. If you specify option

'wild', TbScanX will only search for the viruses which appear in the

wild, saving memory and time.

secure (s)

TbScanX normally asks the user to continue or to cancel when it detects

a virus. In some business environments however this choice should not be

made by employees. By using option 'secure' it is no longer possible to

allow suspicious operations. Option 'secure' also disables option 'off'

and 'remove'.

lock (l)

System operators can use this option to instruct TbScanX to lock the

system once a virus is detected.

api (i)

This option is intended for avanced users only. It enables the Applica-

tion Program Interface of TbScanX which is needed if you want to call

TbScanX from within your application. Consult the file Addendum.Doc for

detailed programming information.

ems (me)

If you specify this option TbScanX will use expanded memory (like

provided by LIM/EMS expansion boards or 80386 memory managers) to store

the signatures and part of its program code. Since conventional memory

III - 25

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

is more valuable to your programs than expanded memory, the use of EMS

memory is recommended.

xms (mx)

If you specify this option TbScanX will use extended memory to store the

signatures and part of its program code. An XMS driver (like HIMEM.SYS)

needs to be installed to be able to use this option. XMS memory is not

directly accessable from within DOS, so every time TbScanX has to scan

data it has to copy the signatures to conventional memory.

To be able to save the original memory contents TbScanX needs a double

amount of XMS memory. Swapping to XMS is slower than swapping to EMS

memory, so if you have EMS memory available swapping to EMS is recommen-

ded. Swapping to XMS may conflict with some other software, so if you

experience problems try using TbScanX without the XMS option.

Example

Device=C:\utils\TbScanX.Exe xms noboot

2.4. While scanning

Whenever a program tries to write to an executable file (files with the

extensions .COM and .EXE), you will briefly see the text "*Scanning*" in

the upper left corner of your screen. Since TbScanX takes very little

time to scan the file, the message will only appear very briefly. The

text "*Scanning*" will also appear if you execute a program directly

from a diskette, and if DOS accesses the bootsector of a diskette drive.

Detecting Viruses

If TbScanX detects a suspicious signature that is about to be written

into a file, a window will appear with the message:

WARNING, <filename> contains <virus name>!

Abort? (Y/n)

Press "N" to continue, press any other key to abort.

If TbScanX detects a suspicious signature in a boot sector, it will

display the message:

WARNING, Disk in <drive> contains <virus name>!

Press a key...

Although a virus seems to be on the bootsector of the specified drive,

the virus cannot do anything since it has not been executed yet. Howe-

ver, if you reboot the machine with the contaminated diskette in the

drive, the virus will copy itself to your harddisk.

III - 26

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

To display the name of the virus, TbScanX needs the signature file

again. It will automatically use the signature file that was used when

you invoked the program. If the signature file is missing (because you

deleted it, or because you removed the  floppy containing it), or no

file handles are left, TbScanX will still detect viruses, but it is no

longer able to display the name of the virus. It will display [Name

unknown] instead.

III - 27

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

3. TbCheck

3.1. The Purpose of TbCheck

TbCheck is a memory-resident integrity checker, coming into action

whenever a file is about to be executed. It uses the Anti-Vir.Dat

records generated by TbSetup to detect file changes, often the first

sign of a virus infection. These records contain information, such as

file sizes and checksums, of every executable file in a directory. By

comparing this information with the actual file status it is possible to

detect any changes, including infections caused by viruses - automati-

cally.

Suppose you have a conventional integrity checker automatically invoked

in your autoexec.bat file. If no files are changed, your system is

supposed to be uninfected. But, to be sure that no virus can infect your

system, you have to execute the checker frequently.

Once loaded TbCheck will remain resident in memory, and will automa-

tically check all programs you try to execute.

TbCheck is fully network compatible. It does not require you to reload

the checker after logged on to the network.

3.2. How to use TbCheck

Since TbCheck is a memory resident program, it can be executed and

configured from the command line or from within a batch file. TbCheck

should however be started automatically and as soon as the computer

boots up, preferably during the execution of Config.Sys or Autoexec.Bat

file.

=>   Be sure TbDriver has already been loaded - TbCheck wil refuse to start

up without it.

Loading TbCheck

There are three possible ways to start TbCheck:

1. From the DOS prompt or within the Autoexec.Bat file:

<path>TbCheck

2. From the Config.Sys as a TSR (Dos 4+):

Install=<path>TbCheck.Exe

The "Install=" Config.Sys command is NOT available in DOS 3.xx.

3. To invoke TbCheck from the Config.Sys as a device driver:

Device=<path>TbCheck.Exe

III - 28

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

TbCheck should always work correctly after being started from within the

Autoexec.Bat. Unlike other anti-virus products, the Thunderbyte anti-

virus utilities can be loaded before the network is started without

losing the protection after the network is started.

Highload  TbCheck

In addition to the three invocation possibilities DOS 5 users can

"highload" TbCheck in a UMB (upper memory block) if it is available:

LoadHigh <path>TbCheck.Exe

Within the Config.Sys file TbCheck can also be loaded high:

DeviceHigh=<path>TbCheck.Exe

TbCheck and MS-Windows

Windows users should load TbCheck BEFORE starting Windows. If you do

that, there is only one copy of TbCheck in memory, but every DOS-window

will nevertheless have a fully functional TbCheck in it.  TbCheck

detects if Windows is starting up, and will switch itself into multitas-

king mode if necessary. You can even disable TbCheck in one window

without affecting the functionality in another window.

3.3. Command line options

It is possible to specify options on the command line. The upper four

options are always available, the other options are available only if

TbCheck is not memory resident.

option  parameter short explanation

------  --------- ----- ------------------

help               ?    display this helpscreen

off                d    disable checking

on                 e    enable checking

remove             r    remove TbCheck from memory

noavok [=<drives>] o    do not warn for missing AV record

fullcrc            f    calculate full CRC (slow!)

secure             s    do not execute unauthorized files

help (?)

If you specify this option TbCheck displays the above options list.

off (d)

III - 29

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

Disables TbCheck, but it will remain in memory and can be reactivated.

on (e)

TbCheck will be reactivated after having been disabled with the 'off'

option.

remove (r)

This option disables TbCheck and will try to remove the resident part

from memory. Unfortunately, this can only work if TbCheck was loaded

last. An attempt to remove a TSR after another TSR has been started will

simply leave a useless gap in memory and could disrupt the interrupt

chain. TbCheck will try to find out whether it is safe to remove its

resident code; if not, it will simply disable itself.

noavok (o)

TbCheck will look for checksum information on the file you want to be

checked in the Anti-Vir.Dat file. TbCheck will display a message if no

checksum information is found or if the specific checksum is incorrect.

This makes sure that you will receive a warning whenever a malicious

program deletes the Anti-Vir.Dat file.

Although it is recommended to maintain Anti-Vir.Dat files on all drives,

this may not always be practical with floppy disks, RAM disks or CD-ROM

disks. Option 'noavok' tells TbCheck not to look for an Anti-Vir.Dat on

specific drives. For instance, if you don't want to be alerted about the

absence of an Anti-Vir.Dat record on floppy disks (A: and B:) or on your

RAM disk (E:) you should specify:

"NoAvOk=ABE"

If you don't want a message if an Anti-Vir.Dat record is missing on

network drives, you should specify an asterisk (*) instead of a drive

letter.

If you don't specify a drive to the 'noavok' option, TbCheck will never

issue a warning if an Anti-Vir record is missing on any drive.

=>   Note that this presents a security hole for viruses: by deleting the

Anti-Vir.Dat file you will not be able to detect file changes caused by

a viral infection.

=>   Please note that the 'noavok' option does not do anything to prevent the

detection of infected programs if the Anti-Vir record is available. If a

program has been changed and the Anti-Vir record is available, you will

still get an alarm regardless of how option 'noavok' was implemented.

fullcrc (f)

By default, TbCheck only verifies that part of the file near the pro-

gram's entry point. If a virus infects the file, this area is guaranteed

to change, so this is perfectly adequate to detect all infections. Other

file changes, notably configuration variations, will not trigger the

alarm. If, however, you should ever desire a full check that detects any

III - 30

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

file changes, this option will take care of it. Note that this slows

down the system considerably - this option is not recommended for normal

(anti-virus) usage!

secure (s)

TbCheck normally asks whether the user wants to continue or cancel when

a file has been changed or when there is no checksum information availa-

ble. In a business environment it may be unwise to leave such decisions

to employees. Option 'secure' makes it impossible to execute new or

unknown programs, or programs that have been changed. Option 'secure'

also disables option 'off' and 'remove'.

3.4. While checking

Whenever a program wants to execute, TbCheck steps in to see if it

really has the authority to do so. During that time it will display

'*Checking*' in the screen's upper left hand corner. TbCheck operates at

lightning speed, therefore the message will appear only momentarily.

Since TbCheck does not take much time to check the file, you will see

the message only in a short notice of time.

Detecting File Changes

TbCheck quickly checks a program when that program is loaded. If TbCheck

detects that a file has been changed, a pop-up window will appear to

inform you. You can either choose to continue, or to abort the program

invocation.

If there is no information (Anti-Vir.Dat) about the program, TbCheck

will inform you about this too. You can either choose to continue

without checking, or to abort the program invocation.

=>   Note that you can prevent users from executing non authorized software

by combining the TbCheck command with the 'secure' option.

3.5. Testing TbCheck

Many people understandably wish to test the product they are using. In

contrast with, for instance, a word processor, it is very difficult to

test a smart integrity checker like TbCheck. You cannot change a random

25 bytes of an executable file just to find out whether or not TbCheck

will detect the file change. On the contrary, it is very likely that

TbCheck will NOT detect it because the program only checks the entry-

area of the file whereas the changed bytes might be located on another

location within the file.

III - 31

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

4. TbClean

4.1. The Purpose of TbClean

TbClean isolates viral code in an infected program and removes it. From

then on it will be safe to use the program again, as the risk of other

files being infected or damaged by it will have been securely elimina-

ted.

Generic Cleaners

TbClean works completely different compared to 'conventional cleaners.

First of all, it does not recognize any virus. Its disinfection scheme

is completely different and it works with almost any virus. Actually,

the TbClean program contains two cleaners: a 'repair' cleaner, and a

'heuristic' cleaner. The repair cleaner needs an Anti-Vir.Dat file that

is generated by the TbSetup program before the infection occurs. In this

Anti-Vir.Dat file essential information is stored, like the original

file size, the bytes at the beginning of the program, a cryptographic

checksum to verify the results, etc. This information enables TbClean to

disinfect almost every file, regardless of the virus it has been infec-

ted with, known or unknown.

No information available?

In the heuristic cleaning mode TbClean does not need any information

about viruses either, but it has the added advantage that it does not

even care about the original, uninfected state of a program. This

cleaning mode is very effective if your system is infected with an

unknown virus and yo neglected to let TbSetup generate the Anti-Vir.Dat

files in time.

In the heuristic mode, TbClean loads the infected file and starts

emulating the program code to find out which part of the file belongs to

the original program and which to the virus. The result is successful if

the functionality of the original program is restored, and the functio-

nality of the virus has been reduced to zero.

=>   Note that this does not imply that the cleaned file is 100% equal to the

original.

When TbClean uses heuristic cleaning to disinfect the program, the file

will most likely not be exactly the same as in its original state. This

is not an indication of failure of TbClean, nor does it mean the file is

still infected in some way. First of all, it is normal that the heuris-

tically cleaned file is still larger than the original. This is normal

because TbClean tries to be on the safe side and it will avoid removing

too much. The bytes left at the end of the file are 'dead' code, the

instructions will never be executed again since the 'jump' at the

III - 32

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

beginning of the program has been removed. If the cleaned file is an EXE

type file, it is likely that some bytes in front of the program - the

exeheader - are different. There are many suitable solutions to recon-

struct the exeheader, and TbClean can of course never know the original

state of the program. The functionality of the cleaned file will never-

theless be the same!

=>   Note that this only applies to heuristic cleaning: if there is a suit-

able Anti-Vir.Dat record available, the cleaned program will normally be

exactly the same as the original clean file.

It is possible that the infected file is infected with multiple viruses,

or multiple instances of the same virus! Some viruses keep on infecting

files, and in such case the infected files will keep growing. If TbClean

used its heuristic cleaning mode, it is very likely that TbClean removed

only one instance of the virus. In this case, it is necessary to repeat

the cleaning process until TbClean reports that it can not remove

anything anymore.

4.2. How to use TbClean

After tracking one or more viruses, all you should do is select the

'Start cleaning' option in the TbClean menu. After specifying the

relevant filename, TbClean will come into action. Beforehand, TbClean

allows some additional parameters. These parameters are discussed below.

The TbClean menu

You can execute TbClean in combination with some useful parameters,

which are listed in the TbClean menu. You can activate these parameters

by toggling the selection bar to the desired option and type <ENTER>. A

checkmark indicates that the specific parameter is ON.

+----Main menu-----+

|  Confi+-----TbClean menu-----+

|  TbSet|  Start cleaning      |

|  TbSca|  List-file name      |

|  TbUti|  Use TBAV.INI file   |

|  TbCLe|  Prompt for pause    |

|  TBAV |v Use Anti-Vir.Dat    |

|  Docum|v Use Heuristics      |

|  Regis|v Expanded memory     |

|  Quit |  Show program loops  |

|  eXit |  Make list file      |

+-------+----------------------+

III - 33

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

List-file name

By selecting this option you may specify a filename to be used as list-

file (see also option 'make list-file').

Use TBAV.INI file

By enabling this option, the TbClean configuration values, saved in the

TBAV.INI file, will also be valid when loading TbClean from the command

line. Be careful, since options specified in the TBAV.INI file can not

be undone on the command line. See chapter I-2 ('Configuration').

Prompt for pause

TbClean will stop disassembling information after each full screen to

let you examine the results.

Use Anti-Vir.Dat

If this option is deselected, TbClean will act as if there were no Anti-

Vir.Dat records available and will therefore perform heuristic cleaning.

Use Heuristics

If this option is deselected, TbClean will not try to apply heuristic

cleaning, even when there are no Anti-Vir.Dat records available.

Show program loops

By default TbClean keeps track of looping conditions to keep an iterati-

on that would be emulated thousands of times from being listed on your

screen. With this option TbClean 'works out' every loop.

=>   Note that TbClean will perform at a drastically reduced speed. Do not

combine this option with the 'list' option, because the list file might

grow too big.

Expanded memory

If activated, TbClean will detect the presence of expanded memory and

will use it in heuristic mode. You may disable EMS usage if it is too

slow, or if your expanded memory manager is not very stable.

III - 34

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

Make list file

TbClean will generate an output file with a chronological disassembly of

the virus being removed.

4.3. Command line options

TbClean recognizes option short-keys and option words. The words are

easier to memorize, and they will be used in this manual for convenien-

ce.

option     parameter        short explanation

---------------------------------------------------------------

help                         he   help (-? = short help)

pause                        pa   enable 'Pause' prompt

mono                         mo   force monochrome

noav                         na   do not use Anti-Vir.Dat record

noheur                       nh   do not use heuristic cleaning

noems                        ne   do not use expanded memory

showloop                     sl   show every loop iteration

list       [=<filename>]     li   create list file

Below, all command line options are explained briefly.

help (he)

If you specify this option TbClean displays the contents of the TBCLE-

AN.HLP file if it is available in the home directory of TbClean.  If you

specify the '?' option you will get the summarized help info as listed

above.

pause (pa)

TbClean will stop disassembling information after each full screen to

let you examine the results.

mono (mo)

This option forces TbClean to refrain from using colors in the screen

output. This might enhance the screen output on some LCD screens or

color-emulating monochrome systems.

noav (na)

If this option is specified, TbClean will act as if there were no Anti-

Vir.Dat records available and will therefore perform heuristic cleaning.

noheur (nh)

If this option is specified, TbClean will not try to apply heuristic

cleaning, even when there are no Anti-Vir.Dat records available.

III - 35

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

noems (ne)

If specified, TbClean will not detect the presence of expanded memory

and will use it in heuristic mode. You may disable EMS usage if it is

too slow, or if your expanded memory

manager is not very stable.

showloop (sl)

By default TbClean keeps track of looping conditions to keep an iterati-

on that would be emulated thousands of times from being listed on your

screen. With this option TbClean 'works out' every loop.

=>   Note that TbClean will perform at a drastically reduced speed. Do not

combine this option with the 'list' option, because the list file might

grow too big.

list [=<filename>] (li)

TbClean will generate an output file with a chronological disassembly of

the virus being removed.

Examples

TbClean VIRUS.EXE

TbClean will make a backup with the name VIRUS.VIR and it will disinfect

VIRUS.EXE

TbClean VIRUS.EXE TEST.EXE

TbClean will copy VIRUS.EXE to TEST.EXE and disinfect

TEST.EXE

4.4. The cleaning process

Choose 'Start cleaning' in the TBAV menu. Now you specify the name of

the file that has to be cleaned. Suppose you want an infected program

file named 'virus.exe' to be cleaned:

Enter name of program to clean. TbClean will create a backup first!

C:\VIRUS\VIRUS.EXE

The ThunderBYTE utility cleans on a file-by-file approach: clean one

file, verify the result, and proceed with the next file. This helps you

to keep track of which file is clean, which file is damaged and should

be restored from a backup, and which

file is still infected.

Enter name of cleaned file. Keep blank if infected program may be

changed.

C:\VIRUS\TEST.EXE

By specifying a different name (eg. 'test.exe') you indicate that the

cleaned file may not overwrite the original .exe file. In this example

III - 36

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

TbClean will copy VIRUS.EXE to TEST.EXE and disinfect TEST.EXE. Next,

TbClean will start the cleaning process.

If you do not specify a backup filename, TbClean will create a backup

with the '.vir' extension (eg. VIRUS.VIR) and it will disinfect the .exe

file.

+-----------------------------------------------------------------+

|  Thunderbyte clean utility v6.31 (C) 1992-95 Thunderbyte B.V.   |

+---------Infected state----------++---------Original state-------+

| Entry point (CS:IP)   34BF:0012 || Entry point (CS:IP) 34BF:0012|

| File length                     || File length         UNKNOWN! |

| Cryptographic CRC     9F90F52A  || Cryptographic CRC   UNKNOWN! |

+---------------------------------++------------------------------+

|                                                                 |

| Starting clean attempt. Analyzing infected file...              |

| Anti-Vir not found: original state unknown. Trying emulation... |

| Emulation terminated:                                           |

|                                                                 |

| C:\VIRUS\TEST.EXE                                               |

| CS:IP    Instruction     AX  BX  CX  DX  DS  SI  ES  DI  SS  SP |

| 9330:0101  mov ah,40    FFFE9330FFFFEFFFD382FFEDEFFEFFFF9520007E|

| 9330:0103  mov bx,0002  40FE9330FFFFEFFFD382FFEDEFFEFFFF9520007E|

| 9330:0106  mov cx,0016  40FE0002FFFFEFFFD382FFEDEFFEFFFF9520007E|

| 9330:0109  mov dx,cs    40FE00020016EFFFD382FFEDEFFEFFFF9520007E|

| 9330:010B  mov ds,dx    40FE000200169330D382FFEDEFFEFFFF9520007E|

| 9330:010D  mov dx,0117  40FE0002001693309330FFEDEFFEFFFF9520007E|

| 9330:0110  int 21       40FE0002001601179330FFEDEFFEFFFF9520007E|

| 9330:0112  mov ax,4CFF  40FE0002001601179330FFEDEFFEFFFF9520007E|

| 9330:0115  int 21       4CFF0002001601179330FFEDEFFEFFFF9520007E|

| 9330:0115  <End of emulation>                                   |

+-----------------------------------------------------------------+

While Cleaning

TbClean will display as much information as possible about the current

operation, as illustrated above. All the major actions will be in the

emulation window, displaying a disassembly and the register contents of

the program under scrutiny, along with a progress report. The status

windows reveal useful details of the infected file and, if TbClean can

find a suitable Anti-Vir.Dat file, its original status. You may abort

the cleaning process by pressing <Ctrl-Break>.

The job isn't done yet

A successful purge is not the end of the story! Your job is only parti-

ally completed. Some viruses damage data files. They could randomly

change bytes on your disks, swap sectors, or perform other nasty tricks.

A cleaning utility will never be able to repair your data! Check your

data files thoroughly and consult a viral expert to find out what the

III - 37

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

virus is capable of doing. If there is any doubt, restoring the data is

definitely the most reliable option.

=>   Under no circumstances should you continue to use cleaned software!

Cleaning is a temporary solution to allow you to delay a large restore

operation until the first available slack period. You should not rely on

a cleaned program for any length of time.  Please, don't take this as a

put down of antiviral cleaning agents. If your data is valuable to you,

you should care for it as much as possible, and sticking to original

software only is no more than an elementary precaution. In other words,

restore the original programs as soon as possible!

Cleaning Limitations

Although TbClean has a very high success rate and is able to clean

programs that other cleaners refuse to process, not all viruses can be

removed, and not all files can be cleaned.

Viruses that cannot be removed from an infected file:

Overwriting viruses.

This type of virus does not add itself to the end of the original

program, they just copy themselves over the original file. They do not

attempt to start the original program but they will simply return you to

DOS after they are activated or will hang the machine. Since the origi-

nal file is overwritten and damaged, no cleaner can remove the virus.

Some encrypted viruses.

TbClean is usually able to decrypt the virus. However, some viruses use

anti-debugger features that TbClean cannot cope with yet.

The way some program files are constructed makes them impossible to

clean and reinstatement will be the only option. This category includes:

EXE-programs with internal overlays.

TbScan marks these files with an 'i' flag. Any infection is bound to

cause major damage to these files. Some viruses recognize such programs

and do not infect them, but most viruses infect these programs anyway,

and corrupt the program. No cleaner can repair such damage.

Programs with sanity check routines.

Some programs - mostly anti-virus software or copy-protected programs -

perform some kind of sanity check. Heuristic cleaning of an infected

program normally results in a program that is not physically identical

to the original. Although the virus is removed from the program and the

III - 38

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

program is functionally identical to the original, the sanity check will

usually detect the slight changes and abort the program.

Multiple files cleaning

TbClean has no provisions for cleaning multiple programs in one run.

There are two reasons for this omission:

-    TbClean can not search for viruses automatically since it does not

know any virus.

-    We highly recommend to clean the system on a file-by-file approach.

Clean one file, verify the result, and proceed with the next file.

This helps you to keep track of which file is clean, which file is

damaged and should be restored from a backup, and which file is

still infected.

III - 39

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

5. Ongoing virus prevention: TbMon

The Purpose of TbMon utilities

TbMon is a set of three memory resident anti-virus utilities:

TbMem     Detects attempts of programs to remain resident in                                                                  memory, and

makes sure that no program can remain resident in memory

without permission.

TbFile    detects attempts of programs to infect other programs.

TbDisk    detects attempts of programs to write directly to the disk

(without using DOS), attempts to format, etc.

Instructions on how to use these utilities are presented below.

How to use TbMon programs

Loading TBMon programs

The TbMon programs are all loaded in the same way. For specific informa-

tion on each of the programs, such as commandline options, please refer

to the appropriate sections in this chapter.

There are three possible ways to start the TbMon programs:

From the DOS prompt or within the Autoexec.Bat file:

<path>Tbxx

From the Config.Sys as a TSR (Dos 4+):

Install=<path>Tbxxx.Exe

The "Install=" Config.Sys command is NOT available in DOS 3.xx.

To invoke a TbMon program from the Config.Sys as a device driver:

Device=<path>Tbxxx.Exe

A TbMon program should always work correctly after being started from

within the Autoexec.Bat. Unlike other anti-virus products, the Thunder-

byte anti-virus utilities can be loaded before the network is started

without losing the protection after the network is started.

Highload TBMon programs

In addition to the three invocation possibilities DOS 5 users can

"highload" TbMon programs in a UMB (upper memory block) if it is availa-

ble:

LoadHigh <path>Tbxxx.Exe

III - 40

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

Within the Config.Sys file a TbMon program can also be loaded high:

DeviceHigh=<path>Tbxxx.Exe

TBMon and MS-Windows

Windows users should load a TbMon program BEFORE starting Windows. If

you do that there is only one copy of the TbMon program in memory, but

every DOS-window will nevertheless have the fully functional TbMon

program in it. The TbMon program detects if Windows is starting up, and

will switch itself into multitasking mode if necessary. You can even

disable the program in one window without affecting the functionality in

another window.

Command line options

All TbMon utilities can be loaded in combination with several options.

You may specify the options listed below in combination with all three

utilities. The specific options are described in the relevant sections.

help (?)

If you specify this option the TbMon program will show you the brief

help as shown above.

off (d)

If you specify this option the TbMon program will be disabled, but it

will remain in memory.

on (e)

If you use this option the TbMon program will be activated again after

you disabled it with the 'off' option.

remove (r)

This option can be used to remove the resident part of the TbMon program

from your system's memory. All memory used by the TbMon program will be

released. Unfortunately, the removal of a TSR is not always possible.

The TbMon program checks whether it is safe to remove the resident part

from memory. If it is not safe it just disables the TbMon program. A TSR

can not be removed if another TSR has been started after it. If this

happens with the TbMon program, it will completely disable itself.

secure (s)

If you specify this option, the user is not allowed to give permission.

All warnings are automatically answered with 'Abort'. It is also not

possible anymore to unload the utilities with 'remove' or to disable

them with option 'off'.

5.1. TbMem

III - 41

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

The Purpose of TbMem

Most viruses remain resident in memory once they have been executed.

While resident in memory, they may have many opportunities to infect

other files in the background, interfere with the system operation, hide

themselves from virus scanners or checksummers, and/or perform other

nasty tasks.

On the other hand, because so many viruses remain resident in memory, it

is easy to detect most of them once the process of becoming resident in

memory is monitored.

TbMem monitors the system and ensures that no program will remain

resident in memory without permission. This will draw attention to any

software that attempts to remain resident, thereby reducing the likeli-

hood that a virus will be able to go unnoticed. TbMem also protects

CMOS.

What is a memory-resident program?

Most programs will be invoked by a command on the DOS command line,

perform some task, and finally terminate, placing you right back where

you started.

Some programs however continue to operate after they are terminated.

These programs load themselves into memory of your PC, remain resident

in the memory and perform some task in the background. Programs in this

category are disk caches, print spoolers, network software, etc. These

programs are often referred to as 'TSR-software', which means 'Termina-

te-and-Stay-Resident'.

Most viruses remain resident in memory too, and that is why the process

of becoming resident in memory should be controlled in some way, prefe-

rably by TbMem.

TbMem offers you the option to abort the program before it can become

resident. TbMem will guard the DOS TSR function calls, while also

monitoring important interrupts and memory structures. TbMem uses the

Anti-Vir.Dat records to determine whether a program is allowed to remain

resident in memory.

Many common TSRs will be recognized by TbSetup. However, if TbSetup

doesn't recognize a TSR, TbMem will ask your permission for the TSR to

load. Permission information will be maintained in the Anti-Vir.Dat

files, to prevent TbMem from bothering you when an approved TSR is

loading.

TbMem will also check the contents of the CMOS configuration memory

after each program termination, to make sure that programs do not change

it unnoticed. TbMem offers the possibility to restore the CMOS configu-

ration when it has been changed. Once you have 'taught' TbMem which

III - 42

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

programs are TSRs and which are not on one PC, you can use TbSetup to

set the permission flag of these files on other machines.

TbMem installs a hot key that can be used to escape from nearly all

programs.

TbMem is fully network compatible. It does not require you to reload the

checker after logging on to a network.

How to use TbMem

Since TbMem is a memory resident program, it can be executed and confi-

gured from the command line or from within a batch file. TbMem should

however be started automatically and as soon as the computer boots up,

preferably during the execution of Config.Sys or Autoexec.Bat file.

Be sure TbDriver has already been loaded - TbMem wil refuse to start up

without it.

Command line options

It is possible to specify options on the command line. The upper four

options are always available, the other options are available only if

TbMem is not memory resident.

option  parameter      short explanation

--------------------------------------------------------------

help                    ?    display this helpscreen

off                     d    disable checking

on                      e    enable checking

remove                  r    remove TbMem from memory

secure                  s    do not execute unauthorized TSRs

hotkey  <keycode>       k    specify keyboard scancode for hotkey

nocancel                n    do not install cancel hot key

nocmos                  m    do not protect CMOS

help (?)

If you specify this option TbMem will show you the brief help as shown

above.

off (d)

If you specify this option TbMem will be disabled, but it will remain in

memory.

on (e)

If you use this option TbMem will be activated again after you disabled

it with the 'off' option.

III - 43

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

remove (r)

This option can be used to remove the resident part of TbMem from your

system's memory. All memory used by TbMem will be released. Unfortunate-

ly, the removal of a TSR (like TbMem) is not always possible. TbMem

checks whether it is safe to remove the resident part from memory. If it

is not safe it just disables TbMem. A TSR can not be removed if another

TSR has been started after it. If this happens with TbMem it will

completely disable itself.

secure (s)

TbMem normally asks the user to continue or to cancel when a program

tries to remain resident in memory. In some business environments

however this choice should not be made by employees. By using option

'secure' it is no longer possible to execute new or unknown resident

software. It is also no longer possible to use option 'remove' or 'off'.

nocancel (n)

TbMem normally installs the program cancel hot key (Ctrl-Alt-Insert). If

you do not want this, specify this option. This also saves a few bytes

of memory.

hotkey (k)

TbMem offers you a reliable way to escape from any program by pressing a

special key combination. This can be used to escape from programs that

'hang', but of course also to escape from software that seems to be

malicious (although powering down and rebooting from a write-protected

system disk is recommended).

The program cancel hot key of TbMem is by default Ctrl-Alt-Insert. If

you wish, you can specify another keyboard option with option 'hotkey

=<keycode>'. The scancode is specified in a 4 digit hexadecimal number.

The far left bytes specify the shift-key mask, the far right bytes

specify the keyboard scancode. Consult your machine manual for a list of

scancodes. The default scancode is 0C52h (Ctrl-Alt-Insert). The scancode

for Ctrl-Alt-Escape is 0C01h.

nocmos (m)

TbMem normally protects the CMOS memory if available. If you do not want

TbMem to do this you can specify this option.

Examples

C:\utils\TbMem

or:

Device=C:\utils\TbMem.Exe

TbMem in process

If TbMem detects that a program tries to remain resident in memory, a

pop-up window will appear with a message. You can either choose to

continue, or to abort the program invoca-tion. If you answer 'NO' to the

question 'Remove program from memory?' the program will continue undis-

III - 44

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

turbed, and TbMem places a mark in the Anti-Vir.Dat file about this

program. Next time you invoke the same resident program, TbMem will not

disturb you again.

There are a lot of programs which normally remain resident in memory,

such as disk caches, print spoolers, etc. How does TbMem distinguish

between these programs and viruses?

TbMem uses the Anti-Vir.Dat records generated by TbSetup to keep track

of which files are normal TSRs and which are not. Most common resident

software will be marked as such by TbSetup, so you don't have to worry

about these files.

If TbMem pops up with the message that a program tries to remain resi-

dent in memory, you have to consider the purpose of the program mentio-

ned. Is the program supposed to continue to operate in the background?

The answer is obviously yes if the program mentioned is a disk cache,

print spooler, pop-up utility or system extension software.

However, if the message appears after you have finished a text proces-

sing job, or terminated a database or spreadsheet application, something

is definitely wrong! You ought to terminate the program and use a virus

scanner to check the system.

The same applies when software that operates normally without staying

resident in memory suddenly changes its behavior and tries to remain

resident in memory.

5.2. TbFile

The Purpose of TbFile

The two most perilous viral categories are the bootsector and the file

variants. File viruses all have a common purpose -they infect programs.

Infecting a program involves very unusual file manipulations that are

quite dissimilar to normal files handling procedures, so in order to

detect viral activity it is essential to keep an eye out for program

file changes involving peculiar actions.

TbFile monitors the system and detects attempts of programs to infect

other programs. Unlike other file guards, TbFile monitors the system

only for virus specific file modifications. TbFile won't generate an

alarm when a program modifies itself for configuration purposes, nor

does it bother you when you update a program or create one yourself.

Running an average system, configurations should never cause a false

alarm.

TbFile not only detects attempts to infect programs, it also offers you

the option to abort the infection process and to continue the program.

III - 45

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

TbFile detects other suspicious activities - including setting the

seconds value of time stamps to an illegal value.

TbFile has a very sophisticated infection detector and it will not give

a false alarm when you perform standard file operations. In normal

configurations you will never get a false alarm!

Files can be protected against unwanted modifications by means of the

read-only attribute. Without TbFile this standard DOS protection can be

circumvented easily. TbFile however makes sure any attempts to sabotage

the readonly attribute will not go undetected. This gives you added

security by letting you use an uncomplicated method to fully protect

your files against destruction and infection.

TbFile is fully network compatible. It does not require you to reload

the checker after logging on to a network. Other resident anti-virus

utilities force you to choose between protection before the network is

started, or protection after the network is started, but not both.

Command line options

It is possible to specify options on the command line. The upper four

options are always available, the other options are only available if

TbFile is not already memory-resident.

option     short explanation

---------- ----- -----------------------

help       ?     display this helpscreen

off        d     disable checking

on         e     enable checking

remove     r     remove TbFile from memory

secure     s     all permissions denied

allattrib  a     readonly check on all files

compat     c     allow CPM style calls

help (?)

If you specify this option TbFile will show you the brief help as shown

above.

off (d)

If you specify this option TbFile will be disabled, but it will remain

in memory.

on (e)

If you use this option TbFile will be activated again after you disabled

it with the 'off' option.

remove (r)

III - 46

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

This option can be used to remove the resident part of TbFile from your

system's memory.  All memory used by TbFile

will be released.

Unfortunately, the removal of a TSR (like TbFile) is not always possi-

ble.  TbFile checks whether it is safe to remove the resident part from

memory. If it is not safe it just disables TbFile. A TSR can not be

removed if another TSR has been started after it. If this happens with

TbFile it will completely disable itself.

secure (s)

TbFile normally asks the user to continue or to cancel when a program

tries to perform a suspicious operation. In some business environments

however this choice should not be made by employees. By using option

'secure' it is no longer possible to allow suspicious operations. It is

also no longer possible to use options 'off' and 'remove'.

allattrib (a)

TbFile normally only protects the readonly attribute of executables

(program files with the extension COM and EXE). If you want to have the

readonly check on all files add option 'allattrib'. In this case you

will always get an alarm when an attempt is made to remove the readonly

attribute of any file.

compat (c)

DOS has still some CPM - an earlier operating system - functions left.

No DOS program uses these functions anymore, but some viruses use these

functions to bypass anti-virus software. TbFile closes these backdoors

by default. Option 'compat' prevents this.

Examples

C:\utils\TbFile allattrib

or:

Device=C:\utils\TbFile.Exe allattrib

5.3. TbDisk

The Purpose of TbDisk

Many viruses try to damage the data on the disk. They accomplish this by

formatting the disk, overwriting the FAT, swapping disk sectors, etc.

Almost anything is possible.

Another category of malicious software, known as 'bootsector virus

droppers', installs a bootsector virus on the disk. The program itself

is not a virus, so detection with virus scanners and other anti-viral

software is very difficult. The only way to detect such programs is by

monitoring their behavior.

The main problem lies in the way these programs manage to avoid the

usual DOS procedures - they go directly to the BIOS, the Basic In-

III - 47

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

put/Output System. That is why you need TbDisk - to monitor the system

and to ensure that no program can write directly to disk without permis-

sion! It draws attention to any software that attempts to write directly

to disk, thereby reducing the likelihood that a virus will remain

unnoticed. TbDisk prevents viruses from damaging data on your disk and

stops bootsector virus droppers in their tracks.

TbDisk will come in handy, too, if you ever need to write protect a hard

disk. This bonus feature often helps when testing new software.

TbDisk not only informs you when a program tries to write directly to

the disk, it also offers you the option to abort the program before it

can cause any damage.

Detection of 'stealth' techniques. TbDisk is able to detect attempts to

single step through the BIOS handler, and even monitor the use of

undocumented calls that could cause disk damage.

TbDisk is able to distinguish whether DOS or an application makes direct

write attempts via Int 13h. Direct writes are perfectly legal for DOS,

but unusual for application software.

TbDisk needs little maintenance. TbDisk uses the Anti-Vir.Dat records to

determine if a program is allowed to write directly to the disk, inclu-

ding popular disk utilities, which will have been recognized by TbSetup.

In the absence of an Anti-vir.Dat record, TbDisk will ask your approval

first and, if granted, updates the record accordingly in order to avoid

repeated warnings about the same program.

TbDisk is fully network compatible. It does not require you to reload

the program after logging on to a network. Other resident anti-virus

utilities force you to choose between either protection before the

network is started, or protection after.

How to use TbDisk

Loading TBDisk

Improper installation can cause excessive amounts of false alarms! If

you want to install TbDisk in your Config.Sys or AutoExec.Bat file, it

is highly recommended to use the 'install' option of TbDisk first. If

the system continues to behave normally and TbDisk does not give false

alarms when you copy files on your hard disk, TbDisk is installed

correctly and you can remove option 'install'.

TBDisk in your Install

Failure to use option 'install' when you install TbDisk Config.Sys or

AutoExec.Bat file may cause loss of data! Option 'install' causes TbDisk

to allow all disk accesses, it will however pop-up a message like it

III - 48

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

would do in normal mode. If no false alarms occur when you copy files on

your hard disk, TbDisk is installed correctly and option 'install' can

be removed.

If TbDisk causes false alarms you should load TbDisk further ahead in

your Config.Sys or AutoExec.Bat file, until it works as it should do.

Unlike the other TBAV utilities it is recommended to load TbDisk after

other resident software!  Failure to do so can cause excessive amounts

of false alarms!

TBDisk and MS-Windows

TbDisk detects if Windows is starting up, and will switch itself into

multi tasking mode if necessary. You can even disable TbDisk in one

window without affecting the functionality in another window.  If you

configured Windows to use fast 32-bit disk access you might need TbDisk

option 'win32' if Windows displays an error-message.

Command line options

It is possible to specify options on the command line. The upper four

options are always available, the other options are available only if

TbDisk is not memory resident.

option   short explanation

------   ----- ------------------

help      ?    display this helpscreen

remove    r    remove TbDisk from memory

off       d    disable checking

on        e    enable checking

wrprot    p    make hard disk write protected

nowrprot  n    allow writes to hard disk

win32     w    allow Windows 32bit disk access

secure    s    deny access without asking

nostealth a    do not detect stealth disk access

notunnel  t    do not detect tunneling

install   i    installation test mode

help (?)

If you specify this option TbDisk will show you the brief help as shown

above. Once TbDisk has been loaded the help option will not show all

options anymore.

remove (r)

III - 49

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

This option can be used to remove the resident part of TbDisk from your

system's memory.  All memory used by TbDisk will be released.

Unfortunately, the removal of a TSR (like TbDisk) is not always possi-

ble.  TbDisk checks whether it is safe to remove the resident part from

memory. If it is not safe it just disables TbDisk. A TSR can not be

removed if another TSR has been started after it. If this happens with

TbDisk it will completely disable itself.

off (d)

If you specify this option TbDisk will be disabled, but it will remain

in memory.

on (e)

If you use this option TbDisk will be activated again after you disabled

it with the 'off' option.

wrprot (p)

Hard disks are more difficult to protect against writing than floppies,

which adds considerably to the risks involved when, for instance,

testing new software. Sometimes you might want to find out what this

software will do to your hard disk and how this could possibly affect

your valuable data. With option 'wrprot' it will be safer to do so.

Whenever a program wishes to write to a protected disk you will receive

a message such as:

"Write protect error writing drive C: A)bort, R)etry, I)gnore?"

You may then take appropriate action.

=>   Note: A software write protection solution is not absolutely reliable.

It can be bypassed, but, fortunately, viruses that are actually capable

to do so are few and far between. It can be a valuable shield against

most malicious software, despite its shortcomings.

nowrprot (n)

You can use this option to undo the option 'wrprot'.

win32 (w)

Windows 386 Enhanced Mode uses some undocumented DOS calls to retrieve

the original BIOS disk handler when 32-bit disk access has been enabled.

Since TbDisk guards these calls, 32-bit disk access will no longer be

possible, unless you specify option 'win32' when TbDisk is initialized.

=>   Note: Use this option, which reduces antiviral security to some extent,

only in Windows 386 Enhanced Mode with fast 32-bit disk access enabled!

secure (s)

TbDisk normally asks whether the user wants to continue or cancel when a

program tries to perform direct disk access. In some business environ-

ments, however, this should not be left up to employees. Option 'secure'

disables direct disk access permission to new or unknown software. It

also disables option 'off' and 'remove'.

III - 50

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

nostealth (a)

TbDisk tries to detect direct calls into the BIOS. If such an  attempt

occurs, TbDisk pops up with a message that the disk is accessed in an

unusual way. If this feature causes false alarms, you can use this

option to turn it off.

notunnel (t)

One of TbDisk's regular tasks is to detect tunneling attempts on the

part of viruses. 'Tunneling' is a technique used by viruses to determine

the location of the BIOS system code in memory, and to use that address

to communicate with the BIOS directly. This will inactivate all TSR

programs, including resident anti-virus software. TbDisk can detect

'tunneling' attempts in advance and informs you about this. Some other

antiviral products employ tunneling techniques also to bypass resident

viruses, causing a false alarm. If you make use of such other anti-virus

products, you may use the option 'notunnel' to disable tunneling-detec-

tion.

install (i)

Incorrect installation may result in a large number of false alarms. You

should use option 'install' when installing TbDisk, which will reduce

the risk of cancelling a valid disk write operation as a result of false

alarms.

TbDisk in process

What is Direct Disk Access?

Programs often access files, usually through the operating system (DOS).

Whenever a program wants to update a file, for example, it asks DOS to

write the data to disk. There are however also possibilities to write to

a disk without using DOS. This is called 'direct disk access'.

Normal programs do not write to the disk directly. However, there are

some programs that need to write to disk directly. Programs in this

category are:

-    Format utilities. A disk can only be formatted by direct disk

access.

-    Disk diagnosis utilities (such as the NORTON disk doctor, DOS

chkdsk, etc.)

-    Disk optimizers.

As many viruses are able to perform direct disk access as well, it is

essential to have some control over all this. TbDisk can distinguish

between legitimate programs and a virus with the help of the Anti-

Vir.Dat records, generated by TbSetup under your guidance.

Detecting direct disk accesses

III - 51

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

Whenever TbDisk pops up with the message that a program accesses to the

disk directly, consider its purpose carefully. While it is perfectly

acceptable for a format utility or a disk optimizer to format or edit

disk sectors, the same cannot be said about a word processor or databa-

se. When TbDisk warns you that a spreadsheet or some other 'normal'

program is about to format a sector, you can be sure that something is

wrong. Terminate the program -pronto!- and check things out with a virus

scanner before the worst can happen.

III - 52

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

6. TBAV Tools

6.1. TbUtil

The Purpose of TbUtil

TbUtil provides a defense against partition table and bootsector viru-

ses:

TbUtil copies the partition table, bootsector and CMOS data area into a

file. On a regular base you can use TbUtil to compare both the current

and the copied versions of the partition table, bootsector and CMOS data

area. After a (virus) accident you can restore the copy with the TbUtil

program.

TbUtil removes a partition table virus without having to low-level

format the hard disk, even if there is no backup of the partition table.

TbUtil removes bootsector viruses.

TbUtil creates a partition table that has some first-line virus defenses

built-in.

TbUtil replaces the infected or clean bootsector by a safe TBAV boots-

ector.

What is a partition table?

A partition is a logical drive on a hard disk. A physical hard disk can

contain multiple DOS partitions. Every DOS partition has its own drive

ID (eg. C: D: E:). The partition table contains the disk lay-out and the

start and end cylinder of every partition. The partition table also

carries information about the operating system of a partition and which

partition should be used to boot. The partition table is always located

at the first sector of the hard disk. It is called the "Master Boot

Record".

No format needed

Unlike most file viruses, partition table viruses are hard to remove.

The only solution is to low-level format the hard disk and to make a new

partition table, or to make use of undocumented DOS commands.

TbUtil makes a backup of the partition table and bootsector, and will

use this backup to compare and restore both the original partition table

and bootsector once they have been infected. You don't have to format

your disk anymore to get rid of a partition table or bootsector virus.

The program can also restore the CMOS configuration.

III - 53

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

If desired TbUtil replaces the partition table code with an immunized

partition table, containing facilities against viruses. The TbUtil

partition code will be executed before the bootsector gains control, so

it is able to check the bootsector in a clean environment. Once the

bootsector is executed it is difficult to check it, because the virus is

already resident in memory and can fool any protection. Instead of

booting from a clean DOS diskette just to inspect the bootsector, the

TbUtil partition code performs a CRC calculation on the bootsector just

before control is passed to it.

If the bootsector has been modified the TbUtil partition code will warn

you about this. The TbUtil partition code also checks the RAM layout and

informs you when it has changed. It does this every time you boot from

your hard disk.

TbUtil can replace infected and clean diskette bootsectors by a new

bootsector, which has advantages over the standard bootsector. It has

bootsector virus detection capabilities, it performs a sanity check, and

it offers you the possibility to redirect the boot process to the hard

disk without opening the diskette drive door.

How to use TbUtil

The TbUtil module contains several programs, which can be executed from

the TbUtil menu or in case of an emergency from a TbUtil recovery

diskette using the DOS command line. TbUtil allows some additional menu

options. These options are discussed below. The corresponding command

line parameters are listed in chapter 6 of this section.

The system maintenance menu

This menu contains the actual TbUtil program. The program takes care of

saving, restoring or comparing the system configuration of your PC. The

backup system configuration is stored on a diskette in a file with

either a default name or a name you can specify yourself.

=>   Warning: You can only restore a system configuration datafile on the

machine which created the datafile. If not, restoring such a file will

make your PC inaccessible!

III - 54

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

+----Main menu-----+

|  Confi+-----------TbUtil menu-----------+

|  TbSet|  Syste+-------System maintenance-------+

|  TbSca|  Immun|  Execute TbUtil                |

|  TbUti|  Immun|  Describe this machine         |

|  TbCLe|  Immun|  Save system configuration     |

|  TBAV +-------|v Compare system configuration  |

|  Documentation|  Restore system configuration  |

|  Register TBAV|v process CMOS memory           |

|  Quit and save|v process Partition code        |

|  eXit (no save|v process Bootsector            |

+---------------+--------------------------------+

Execute TbUtil

Before activating this option, you must select one of the optional

functions: save, compare or restore the system configuration. Toggle to

the desired option and type <ENTER>. A checkmark will indicate the

selected option.

Describe this machine

Enter a meaningful description of the machine. Enter something like "AT

12MHz, 4Mb, room 12, Mr. Smith". You do NOT have to remember it, TbUtil

will display it on the screen when comparing or restoring, but it helps

you to verify that the data file belongs to the machine.

Save system configuration

This option stores the partition table, bootsector and CMOS data area

into the TbUtil data file.

=>>  Attention! Since the PC is completely inaccessible to DOS if the parti-

tion table gets damaged, it is HIGHLY RECOMMENDED to store both the

TbUtil data file and the program TbUtil.Exe itself on a diskette! It is

not nice if the partition table is destroyed and the only solution to

the problem resides on the same inaccessible disk...

When loading TbUtil from the command line you must specify a filename

after the 'store' option. Using the TBAV menu, you may use the default

filename 'TBUTIL.DAT'. If you own more than one PC, it is advisable to

create one TbUtil diskette with all TbUtil data files of all your PC's

on it. Use the extension of the file for PC identification, eg.:

a:TbUtil.<number>

III - 55

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

Compare system configuration

This option enables you to check on a regular basis that everything is

still OK. If you specify this option TbUtil will compare the information

in the TbUtil data file against the partition table, bootsector and CMOS

data area. It will also  show the comment stored in the data file. And

of course, if you use this option you will also be guaranteed that the

TbUtil data file is still readable.

Restore system configuration

This option enables you to restore the partition table, bootsector, and

CMOS data area. It will ask you to confirm that the data file belongs to

the current machine. Finally it will restore the partition table,

bootsector of the partition to be used to boot, and the CMOS data area.

Process Partition code/Bootsector/CMOS memory

TbUtil will by default restore the partition code, bootsector and CMOS

if option 'restore' is specified. If you use one of the above mentioned

options in combination with the option 'restore' TbUtil will restore

just the items specified.

The TbUtil menu

Apart from the System maintenance menu, the TbUtil menu contains some

useful programs to prevent bootsector virus infection or to remove these

viruses.

+----Main menu-----+

|  Confi+-----------TbUtil menu-----------+

|  TbSet|  System maintenance menu       >|

|  TbSca|  Immunize/clean bootsector A:   |

|  TbUti|  Immunize/clean bootsector B:   |

|  TbCLe|  Immunize/clean partition code  |

|  TBAV +---------------------------------+

|  Documentation  >|

|  Register TBAV   |

|  Quit and save   |

|  eXit (no save)  |

+------------------+

Immunize/clean diskette

You can use the 'immunize' program to clean diskettes infected by a

bootsector virus or to replace the standard bootsector by a bootsector

which has advantages over the original one:

III - 56

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

It has virus detection capabilities. The bootsector will check that it

is still located on the correct place on the diskette, and that Int 13h

and/or Int 40h are still located in system ROM. This makes it possible

to detect even 'stealth' and bootsector viruses.

The TBAV bootsector is able to load the system files if they are availa-

ble on the disk, but if the DOS system files are not on the disk the

TBAV bootsector will present a small menu and offers you two possibili-

ties: retry the boot operation with another diskette, or to boot from

the harddisk. If the user selects the latter, it is not required to open

the diskette drive door.

Immunize/clean hard disk

This is a very powerful option, which can be used to clean an infected

partition table if there is no TbUtil data file. It replaces the exis-

ting partition table code by a new partition routine containing some

virus detection capabilities. The original partition code will be saved

in a file. You have to execute TbUtil from a floppy drive or you have to

specify the name of the file (the specified drive should be a diskette

drive) to store the original partition code.

If the original partition table is completely damaged and cannot be used

to build a new one, TbUtil will scan the entire disk for information

about the original disk layout. TbUtil will also search for TbUtil data

files on the hard disk. It is however recommended to store the data file

on a diskette, although it is a good idea to keep a copy of it on the

hard disk. Just in case!

If your system configuration changes, i.e. you update your DOS version,

or change the amount of memory, you need to update the information

stored in the immune partition as well. You can do this by using this

option.

In the unlikely event that the system does not boot properly, you can

restore the original partition table using the TbUtil 'restore' option

or by using the DOS 5+ 'FDISK /MBR' command (which will create a new

partition table).

If the new partition code works properly, you should make a back-up copy

of it on a diskette using the TbUtil 'store' option.

Command line options

TbUtil recognizes option-characters and option-words. The words are

easier to remember, and they will be used in this manual for convenien-

ce.

III - 57

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

option     parameter   short explanation

------     ---------   ----  --------------------

immunize   <drive>       im  Immunize/Clean boot/MBR of <drive>

getboot    <drive>       gb  Save bootsector into file

store      [<filename>]  st  Store system information

restore    [<filename>]  re  Restore system information

compare    [<filename>]  co  Compare system information

Sub-options of option 'Immunize':

norepeat                 nr  Do not ask for next diskette

nomem                    nm  Do not check for amount of RAM

batch                    ba  Do not prompt to insert a disk

Sub-options of option 'Store':

description<descr.>      de  Add description to data file

Sub-options of option 'Restore':

part                     pt  Restore partition table

boot                     bo  Restore bootsector of HD

cmos                     cm  Restore CMOS

Below, the command line options are explained briefly.

immunize diskette <drive> (im)

You can use the 'immunize' program to clean diskettes infected by a

bootsector virus or to replace the standard bootsector by a bootsector

which has advantages over the original one:

-    It has virus detection capabilities. The bootsector will check that

it is still located on the correct place on the diskette, and that

Int 13h and/or Int 40h are still located in system ROM. This makes

it possible to detect even 'stealth' and bootsector viruses.

-    The TBAV bootsector is able to load the system files if they are

available on the disk, but if the DOS system files are not on the

disk the TBAV bootsector will present a small menu and offers you

two possibilities: retry the boot operation with another diskette,

or to boot from the harddisk. If the user selects the latter, it is

not required to open the diskette drive door.

Immunize c: (im c:)

This is a very powerful option, which can be used to clean an infected

partition table if there is no TbUtil data file. It replaces the exis-

ting partition table code by a new partition routine containing some

virus detection capabilities. The original partition code will be saved

in a file. You have to execute TbUtil from a floppy drive or you have to

specify the name of the file (the specified drive should be a diskette

drive) to store the original partition code.

If the original partition table is completely damaged and cannot be used

to build a new one, TbUtil will scan the entire disk for information

about the original disk layout. TbUtil will also search for TbUtil data

III - 58

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

files on the hard disk. It is however recommended to store the data file

on a diskette, although it is a good idea to keep a copy of it on the

hard disk. Just in case!

If your system configuration changes, i.e. you update your DOS version,

or change the amount of memory, you need to update the information

stored in the immune partition as well. You can do this by using this

option. In the unlikely event that the system does not boot properly,

you can restore the original partition table using the TbUtil 'restore'

option or by using the DOS 5+ 'FDISK /MBR' command (which will create a

new partition table). If the new partition code works properly, you

should make a back-up copy of it on a diskette using the TbUtil 'store'

option.

getboot <drive> (gb)

With this option you can copy the bootsector of the specified drive into

a file.

store [<filename>] (st)

This option stores the partition table, bootsector and CMOS data area

into the TbUtil data file.

=>>  Attention! Since the PC is completely inaccessible to DOS if the parti-

tion table gets damaged, it is HIGHLY RECOMMENDED to store both the

TbUtil data file and the program TbUtil.Exe itself on a diskette! It is

not nice if the partition table is destroyed and the only solution to

the problem resides on the same inaccessible disk...

When loading TbUtil from the command line you must specify a filename

after the 'store' option. Using the TBAV menu, you may use the default

filename 'TBUTIL.DAT'. If you own more than one PC, it is advisable to

create one TbUtil diskette with all TbUtil data files of all your PC's

on it. Use the extension of the file for PC identification, eg.:

a:TbUtil.<number>

restore [<filename>] (re)

This option enables you to restore the partition table, bootsector, and

CMOS data area. It will ask you to confirm that the data file belongs to

the current machine. Finally it will restore the partition table,

bootsector of the partition to be used to boot, and the CMOS data area.

compare [<filename>] (co)

This option enables you to check on a regular basis that everything is

still OK. If you specify this option TbUtil will compare the information

in the TbUtil data file against the partition table, bootsector and CMOS

data area. It will also  show the comment stored in the data file. And

of course, if you use this option you will also be guaranteed that the

TbUtil data file is still readable.

norepeat (nr)

TbUtil will prompt you for next diskette after you immunized a diskette.

With option 'norepeat' you can disable this.

III - 59

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

nomem (nm)

If you specify this option when you are immunizing your partition code,

the partition code will skip the RAM check while booting. This is

necessary for some systems which change the memory setup during the boot

process.

batch (ba)

If you specify this option when you are immunizing diskettes, TbUtil

will not prompt you to insert a disk but assume that there is a disk

inserted.

description <descr.> (de)

Enter a meaningful description of the machine. Enter something like "AT

12MHz, 4Mb, room 12, Mr. Smith". You do NOT have to remember it, TbUtil

will display it on the screen when comparing or restoring, but it helps

you to verify that the data file belongs to the machine.

part (pt)

boot (bo)

cmos (cm)

TbUtil will by default restore the partition code, bootsector and CMOS

if option 'restore' is specified. If you use one of the above mentioned

options in combination with the option 'restore' TbUtil will restore

just the items specified.

Examples

TbUtil store

TbUtil st

TbUtil store A:TbUtil.Dat

TbUtil store A:TbUtil.Dat description = "Test machine"

TbUtil compare A:TbUtil.Dat

TbUtil restore A:TbUtil.Dat part cmos

TbUtil immunize A:

TbUtil immunize C: nomem

Type A:TbUtil.Dat

Using the anti-virus partition

If you install the Thunderbyte partition code (TbUtil immunize), you

will see the following while booting a clean system:

Thunderbyte anti-virus partition v6.31 (C) 1995 Thunderbyte BV.

Checking bootsector CRC -> OK!

Checking available RAM -> OK!

Checking INT 13h -> OK!

III - 60

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

If there is a virus in the bootsector or partition table you will see

this:

Thunderbyte anti-virus partition v6.31 (C) 1995 Thunderbyte BV.

Checking bootsector CRC -> OK!

Checking available RAM -> Failed!

System might be infected. Continue? (N/Y)

Some other messages that can be displayed are "No system.", which means

that there is no active partition on the disk, and "Disk error" of which

the meaning is obvious.

Using the TbUtil diskette

Take a new diskette, format it as a bootable diskette (eg. by using the

dos 'format /s' command). Copy the TbUtil files onto the diskette:

copy tbutil.* a:

The TbUtil files you need are:

tbutil.exe

tbutil.lng

Trouble-Shooting

In case of an emergency, eg. a damaged or infected partition table, you

should boot from the TbUtil diskette. Subsequently, you start the TbUtil

program, using the 'immunize' option:

a:\tbutil immunize c:

6.2. TbLog

The purpose of TbLog

TbLog is a TBAV log file utility. It writes a record into a log file

whenever one of the resident TBAV utilities pops up with an alert

message. Also when TbScan detects a virus a record will be written.

This utility is primarily intended for network users. If all workstati-

ons have TbLog installed and configured to maintain the same log file,

the supervisor is able to keep track of what is going on easily. When a

virus enters the network he is able to determine which machine introdu-

ced the virus, and he can take action in time.

III - 61

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

A TbLog record consists of the timestamp on which the event took place,

the name of the machine on which the event occured, and an informative

message about what happenend and which files were involved. The informa-

tion is very comprehensive and takes just one line.

How to use TbLog

Just like the other TBAV utilities TbLog can be loaded in the Config.Sys

or AutoExec.Bat file, after the TbDriver invokation.

TbLog should be installed on every workstation. If you want to use all

workstations to maintain the same log file, it is recommended to load

TbLog after the network has been started.

TbLog will by default maintain a log file with the name TbLog.Log in the

TBAV directory. If you want to use another filename or in on another

disk or directory you can specify a filename on the command line of

TbLog. In a network environment it is highly recommended to put the log

file on a server disk.

Command line options

It is possible to specify options on the command line. The upper five

options are always available, the other options are available only if

TbLog is not memory resident.

option   parameter   short explanation

----------------------------------------------------

help                  ?    display this helpscreen

remove                r    remove TbLog from memory

on                    e    enable TbLog

off                   d    disable TbLog

test                  t    log test message

machine = <machine>   m    name of your machine

secure                s    do not allow removal

help (?)

If you specify this option TbLog will show you the brief help as shown

above.

off (d)

If you specify this option TbLog will be disable, but it will remain in

memory.

on (e)

If you use this option TbLog will be activated again after you disabled

it with the 'off' option.

III - 62

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.               SECTION III

remove (r)

This option can be used to remove the resident part of TbLog from your

system's memory.  All memory used by TbLog will be released. Unfortuna-

tely, the removal of a TSR (like TbLog) is not always possible.  TbLog

checks whether it is safe to remove the resident part from memory. If it

is not safe it just disables TbLog. A TSR can not be removed if another

TSR has been started after it. If this happens with TbLog it will

completely disable itself.

test (t)

This option can be used to record a test message. If you use option

'test' at the initial invocation of TbLog, it will record the time and

machinename into the log file. If you use the 'test' option afterwards

it will just put a test message in the log file.

machine (m)

With this option you can specify the name of the machine on which TbLog

is loaded. This machine name will appear in the log file. On NetBios

compatible machines TbLog will by default use the network machine name.

On other networks - such as Novell - you have to enter the network name

on the TbLog command line.

secure (s)

If you specify this option it is not possible anymore to use options

'off' and 'remove'.

Examples:

C:\TBAV\TbLog f:\security\Tblog.log secure machine=DESK3

or:

Device=c:\TBAV\TbLog.Exe x:\logs\tblog.log machine=JOHN test

III - 63

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                SECTION IV

SECTION IV. ADVANCED USER INFORMATION

1. Memory requirements

Free memory needed:

min.                min.

to be               after

loaded:             termination:

TbScan                   200 Kb

TbScanX                  10 Kb               800 bytes

TbCheck                  4 Kb                600 bytes

TbUtil                   64 Kb

TbClean                  96 Kb

TbMem                    4 Kb                600 bytes

TbFile                   5 Kb                1 Kb

TbDisk                   4 Kb                800 bytes

TbDriver                 5 Kb                3 Kb

TbLog                    5 Kb                1 Kb

If you decide to use a log file TbScan will need an additional 16 Kb of

memory for the log file buffer. If TbScan uses its own built-in file

system it uses additional memory to keep the FAT in memory. Note that

the memory requirements are independent of the number of signatures. The

current memory requirements are adequate to manage at least 2500 signa-

tures.

The amount of memory TbScanX requires depends on the number of signatu-

res. With all features enabled TbScanX uses 30 Kb of memory when scan-

ning for 1400 family signatures. If you enable swapping TbScanX normally

uses only 1Kb of memory. You can swap to EMS and XMS memory. Of course

the remaining kilobyte of TbScanX can be loaded in upper memory.

In the heuristic cleaning mode TbClean needs much more memory, depending

on the size of the infected file. TbClean can also use expanded memory

(EMS).

Reducing memory requirements

Most PC users try to maintain as much free DOS memory as possible. The

memory resident TBAV utilities (TbScanX, TbCheck, TbMem, TbFile, TbDisk

and TbDriver) are designed to use only a little amount of DOS memory. To

decrease the memory requirements of these utilities even further do the

following:

Load the program from within the Config.Sys file. If loaded as a device

driver it has no Program Segment Prefix (PSP), which saves 256 bytes for

each TBAV utility.

IV - 1

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                SECTION IV

If you load the TBAV utilities from within the Autoexec.Bat file, load

them before establishing environment variables. DOS maintains a list of

environment variables for every resident program, so keep this list

small while installing TSRs. Once all TSRs are installed you can define

all environment variables without affecting the memory requirements of

the TSRs.

By using one of the options 'ems' or 'xms' TbScanX swaps itself to non-

DOS memory, leaving only 1 Kb of code in DOS memory. Swapping to expan-

ded memory ('ems') is preferred.

If you have DOS 5 or higher try to load the program into an upper memory

block using the "loadhigh" or "devicehigh" commands. It is recommended

to enable swapping also to limit the usage of upper memory.

Use one of the processor specific versions of the relevant TBAV utility.

They all consume less memory than the generic versions. Processor

optimized versions are available on any ThunderBYTE support BBS.

Consider the following memory saving options:

TbDriver:      NoStack

TbMem:         NoCancel

TbScanX:       NoBoot, EMX, XMS, wild

IV - 2

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                SECTION IV

2. TbSetup

2.1. Anti-Vir.Dat design considerations

ThunderBYTE Anti-Virus expects every directory on your system with

executable files to have its own Anti-Vir.Dat file. Some other anti-

virus products maintain a somewhat similar 'fingerprint' list of all

executable files, but in one large file rather than a separate file in

every directory. TBAV contains a separate file in every directory

instead of one file with all file information because of the following:

One file in every directory will ease maintenance. If you want to remove

a complete product, the accompanying Anti-Vir.Dat file can be removed as

well.

It will consume less disk space because path information need not to be

stored in the information file.

The TBAV utilities will perform faster because they do not have to

search through a huge file to locate the information of one specific

file.

Installation is easier and more reliable in network environments. On

networks it is not unusual that the same files have different drive ID's

on different workstations. In case of only one information file the

drive-ID's should be stored as well, so every workstation should main-

tain its own list. The supervisor would easily lose control in this

situation.

2.2. Format of TbSetup.Dat

Editing the TbSetup.Dat file is useful to TBAV site installation (see

IV-8). Therefore, some information on the format of this file is neces-

sary. The format of the TbSetup.Dat file is very simple. Empty lines, or

lines starting with a semi-colon (';') or percent symbol, are either

ignored or treated as comment lines. The lines with a preceding percent

symbol are also displayed in TbSetup's upper window.

Every entry in the TbSetup.Dat file has four items:

The filename. The filename must be written in capital letters and

without spaces.

The length of the file in hexadecimal notation. This field may contain a

single asterisk ('*') if an exact filelength match is not required.

The file's 32-bit CRC in hexadecimal notation. A single asterisk is

allowed if an exact checksum match is not required.

IV - 3

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                SECTION IV

The hexadecimal number representing flags that should be set when the

listed file is found on the system.

The rest of the line may be used for a brief comment.

You may use the following flags:

bit 0  (0001)  Do not perform heuristic analysis

bit 1: (0002)  Ignore CRC changes (self-modifying file)

bit 2: (0004)  Scan for all signatures (lan remote boot file)

bit 3: (0008)  Do not change read-only attribute of this file

bit 4: (0010)  The program stays resident in memory

bit 5: (0020)  The program performs direct disk access

bit 6: (0040)  Program is allowed to remove read-only attributes

bit 15:(8000)  Interrupt rehook required for TbDriver.Exe

This is what the entries in TbSetup.Dat look like:

; filename   Length 32-bit CRC  Flags  Comment

; Files that trigger the heuristic alarm of TbScan:

4DOS.COM     19FEA         *    0001   ;4Dos 4.0a

AFD.COM      0FEFE  4B351A86    0001   ;AFD debugger

ARGV0FIX.COM 001D8  431E70C0    0001   ;Argv[0]fix

EXE2COM.EXE  00BEA  49276F89    0001   ;Exe to Com conv. utility

KILL.EXE     00632  74D41811    0001   ;PcTools 6.0 utility

WATCH.COM    003E1  2353625D    0001   ;TSR monitoring utility

; Files that need to be scanned completely, for ALL viruses:

NET$DOS.SYS      *         *    0004   ;Disk image Novell boot

; Files without fixed checksum due to internal config area's:

Q.EXE            *         *    000A    ;Qedit (all versions)

TBCONFIG.COM     *         *    000A    ;all versions

Defining new entries

If you have any files that should be included in the list, please let us

know! We would like to receive a copy to enhance our products and keep

TbSetup.Dat up to date. Candidates for inclusion would be any program

that triggers the heuristic analysis of TbScan. Whenever you choose

'V)alidate program' in the TbScan message window, you will find that on

subsequent occasions TbSetup displays the value '0001' in the flags

field. If your company has many files like this installed on multiple

machines, you may want to include these files in the TbSetup.Dat file

yourself. In order to do that execute TbSetup for the file in question

and make a note of its filelength and 32-bit CRC, as displayed on the

screen. Then edit the TbSetup.Dat file entering the exact filename, the

file length and the CRC number, plus the number of any flags you wish to

IV - 4

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                SECTION IV

set for that file. If you now use TbSetup on another machine it will set

the appropriate flags automatically.

=>   Note:You may manually set or clear a flag field value when executing

TbSetup at the DOS prompt with option 'set' and 'reset' as follows:

TBSETUP TEST.EXE SET=0001

2.3. TBAV site installation

If you have to install the TBAV utilities on a lot of machines in one

company, it would be tedious to invoke for instance every single TSR and

disk utility on each machine in order to 'teach' TBAV which programs are

valid and which are not. Fortunately, this is not necessary. In the

present section, three examples are presented on how to simplify instal-

lation on several machines.

1.   If a resident utility named, for instance, TSRUTIL.EXE is used

throughout the company, you can predefine permission by using

TbSetup to determine the length and CRC of the program. Now put the

name of this program along with the other information in the file

TbSetup.Dat and assign the value '0010' to it.

Example:

TSRUTIL.EXE  01286  E387AB21  0010  ;Our TSR utility

2.   If a disk utility named, for instance, DISKUTIL.EXE is used throug-

hout the company, you can predefine permission by using TbSetup to

determine the length and CRC of the program. Now put the name of

this program along with the other information in the file TbSetup.

dat and assign the value '0020' to it.

Example:

DISKUTIL.EXE  01286  E387AB21  0020  ;Our DISK utility

If you now execute TbSetup on every machine (you have to do this anyway)

it will recognize this utility and it will set the disk access permissi-

on flag for TbMem / TbDisk automatically.

3.   If a utility named, for instance, UTIL.EXE is used throughout the

company which causes TbScan to give false positives, you can

predefine TbSetup to avoid heuristic scanning of the relevant

program. Put the name of this program along with the other informa-

tion in the file TbSetup.Dat and assign the value '0001' to it.

IV - 5

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                SECTION IV

Example:

UTIL.EXE  01286  E387AB21  0001  ;Our utility

If you now execute TbSetup on every machine (you have to do this anyway)

it will recognize this utility and TbScan will not perform heuristic

scanning.

Also consult the TbSetup.Dat file.

IV - 6

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                SECTION IV

3. TbScan

3.1. Heuristic scanning

TbScan is not just a signature scanner. It also disassembles the file

being processed, for the following purposes:

1)   By disassembling the file the scanner can restrict itself to the

area of the file where the virus might reside, reducing false

alarms and speeding up the process.

2)   It makes it possible to use the algorithmic detection method on

encrypted viruses whose signatures would otherwise remain invisible

to the scanner.

3)   And it makes it possible to detect suspicious instruction sequen-

ces.

The detection of suspicious instruction sequences is named 'heuristic

scanning'. It is a very powerful feature that enables you to detect new

or modified viruses and to verify the results of the signature scan. You

no longer have to rely on the scanner's publisher having the same virus

as you might have. In normal cases a scanner can only find a virus if

the scanner's publisher did have a sample of that virus, in order to

make a suitable signature. With heuristic scanning a signature is no

longer required, enabling the scanner to detect yet unknown viruses. You

should not underestimate the importance of heuristic scanning, since

every month at least 50 new viruses are reported. It is very unlikely

that a publisher is the first one to get these new viruses...

Heuristic level 1                       Heuristic level 2

------------------------------------------------------------

Always enabled                          Only with option 'heuris

tic' or after a virus has been

found.

Detects 50% of the unknown viruses.     Detects 90% of the viruses

Almost never causes false alarms.       Causes a few false alarms.

Displays 'Probably infected'            Displays 'Might be infec

ted'

TEST.EXE <scanning...>         OK  (no flags)

TEST.EXE <scanning...> R       OK  (nothing serious)

TEST.EXE <scanning...> FRM         might be infected by unknown virus

TEST.EXE <scanning...> FRALM#      probably infected by unknown virus

How does heuristic scanning actually work? Every program contains

instructions for the PC's processor. By looking into the file's contents

and by interpreting the instructions TbScan is able to detect the

purpose of these instructions. If the purpose seems to be to format a

disk, or to infect a file, TbScan issues a warning. There are a lot of

IV - 7

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                SECTION IV

instruction sequences which are very common for viruses, but very

uncommon for normal programs. Every suspicious instruction sequence is

assigned to a character: a heuristic flag. Every heuristic flag denotes

a score. If the total score exceeds a predefined limit, TbScan assumes

the file contains a virus.

There are actually two predefined limits: the first one is quite sensi-

tive and can be reached by some normal innocent programs. If this limit

is reached, TbScan highlights the heuristic flags that are displayed on

the screen and increases the 'suspected items' counter. TbScan does not

indicate the existence of a virus, unless you have specified the 'heu-

ristic' option. If you do have specified this option, TbScan tells you

the file 'Might be infected by an unknown virus'.

The second heuristic limit will be triggered by a lot of viruses, but

not by normal programs. If this limit is reached TbScan tells you that

the file is 'Probably infected by an unknown virus.'

=>   Note: TbScan performs heuristic analysis only nearby the entry-point of

a file. Therefore, TbScan does not detect direct writes to disk by some

disk utilities nor does TbScan detect some programs as TSR programs.

This is just the result of a specific approach to minimize false alarms.

In case of a virus, the offending instructions are always nearby the

entry-point (except when the virus is over 10Kb in size) so TbScan will

detect suspicious phenomenons in these situations anyway.

3.2. Integrity checking

TbScan will perform integrity checking while scanning. You have to use

TbSetup to generate the Anti-Vir.Dat files. Once these files exist on

your system TbScan will check that every file being scanned matches the

information maintained in the Anti-Vir.Dat files. If a virus infects a

file, the maintained information will not match anymore with the now

changed file, and TbScan will inform you about this. There are no

(command line) options to enable this feature: TbScan will perform

integrity checking automatically if it detects the Anti-Vir.Dat files.

Note that TbScan only reports file changes that could indicate a virus.

Internal configuration areas of program files may also change, but

TbScan does normally not report this. However, if a file gets infected

with any virus -known or unknown - the vital information will change and

TbScan will indeed report it to you!

It is however possible that the checked file changes itself or changes

frequently due to another cause. In this case you might want to exclude

the program from integrity checking to avoid future false alarms. TbScan

will offer you an additional menu option: 'V)alidate program'. For more

information about this menu option consult 'Program validation' (page 6

of this section).

IV - 8

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                SECTION IV

3.3. Program validation

This chapter only applies if you use TbSetup to generate the Anti-

Vir.Dat records. Without these records program validation is not an

option.

TbScan will perform as intended on most programs. There are some pro-

grams, however, that require special attention in order to avoid false

alarms. Most of these programs are recognized automatically by the

TbSetup program. Nevertheless it is certainly possible your PC contains

some program files which trigger the heuristic alarm of TbScan and/or

programs files which change frequently.

If an 'infection' has been found with the heuristic analysis or integri-

ty checking only and if there is an Anti-Vir.Dat record available,

TbScan offers an additional option in its virus-alert window:

'V)alidate program'

If you are convinced that the indicated program does not contain a

virus, you can press 'V' to set a flag in the program's record. This

makes it possible to avoid future false alarms.

There are two validation modes. If TbScan alarms you due to a file

change, the validation applies to future file changes only. If the alarm

is due to heuristic analysis, the validation only applies to heuristic

results. If the file is excluded from heuristic analysis the file will

still be checksummed.  If the file is excluded from integrity checking

TbScan will still perform heuristic analysis on the file.

=>   Note: if you replaced a file (software upgrade) and you did not use

TbSetup, TbScan will pop-up its virus alert window to inform you about

the file change. Do not select the validation option in this case,

because this would exclude the file for future integrity checking. You

should abort TbScan and execute TbSetup on the changed file(s) instead.

3.4. The algorithms

When TbScan processes a file it will display either 'Looking', 'Chec-

king', 'Tracing', 'Scanning', 'Decrypting' or 'Skipping'.

Looking

With 'Looking' TbScan indicates it has successfully located the entry

point of the program in one step. The program code has been identified

so TbScan knows where to search without the need of additional analysis.

'Looking' will be used on most known software.

IV - 9

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                SECTION IV

Checking

'Checking' indicates TbScan has successfully located the entry point of

the program, and is scanning a frame of about 2Kb around the entry

point. If the file is infected, the virus' signature will be located in

this area. 'Checking' is a very fast and reliable scan algorithm.

Checking will be used on most unknown software.

Tracing

'Tracing' means that TbScan has successfully traced a chain of jumps or

calls while locating the entry-point of the program, and is scanning a

frame of about 2Kb around this location. If the file has been infected,

the signature of the virus will be located in this area. 'Tracing' is a

fast and reliable scan algorithm. Tracing will be primarily used for

TSR-type COM files. Most viruses will force TbScan to use 'Tracing'.

Scanning

TbScan is scanning the entire file (except for the exe-header which

cannot contain any viral code). This algorithm will be used if 'Look-

ing', 'Checking' or 'Tracing' can't be used safely. This is the case

when the entry-point of the program contains other jumps and calls to

code located outside the scanning frame, or when the heuristic analyzer

found something that should be investigated more thoroughly. 'Scanning'

is a slow algorithm. Since it processes almost the entire file, inclu-

ding data areas, false alarms are more likely to occur. The 'Scanning'

algorithm will be used while scanning bootsectors, SYS and BIN files.

Decrypting

TbScan detected that the file is encrypted, and decrypts it to be able

to 'look inside'. TbScan performs signature scanning and heuristic

analysis on the decrypted code since that is very reliable and also

reveals polymorphic viruses.

Skipping

'Skipping' will occur with SYS and OVL files only. It simply means that

the file will not be scanned. As there are many SYS files containing no

code at all (like CONFIG.SYS), it makes absolutely no sense to scan

these files for viruses. The same applies to .OV? files. Many overlay

files do not deserve to be called as such as they lack an exe-header.

Such files cannot be invoked through DOS, which makes them just as

invulnerable to direct virus attacks as .TXT files are. If a virus is

reported to have infected an .OV? file, it involved one of the relative-

ly few overlay files which do contain an exe-header. In that case the

infection was the result of the virus monitoring the DOS exec-call

(function 4Bh) and infecting any program being invoked that way, inclu-

ding 'real' overlay files.

IV - 10

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                SECTION IV

3.5. The TbScan.Lng file

The TbScan.Lng file contains all texts being displayed by TbScan. You

can translate or customize the messages with any ASCII editor.

The messages are separated by the dollar sign ($). The first message

displays our address and registration info. You can edit this message as

you please, for instance adding your company logo.

You may add color codes to the TbScan.Lng file. A color code is precee-

ded by the character '|'. The following color codes are available: (all

numbers are in hex).

Color          Foreground Highlight   Background

Black          00         08          00

Blue           01         09          10

Green          02         0A          20

Cyan           03         0B          30

Red            04         0C          40

Magenta        05         0D          50

Yellow/Brown   06         0E          60

White/gray     07         0F          70

Example:  To make a highligted green character on a red background the

color code would be 0A+40=4A. To make the character blink add

80h to the result.

3.6. The TBAV.MSG file

The TBAV menu shell will display the contents of a file named TBAV.MSG

if it exists. You can use this feature to put your company logo on the

TBAV screen. You can embed color codes in this file. Consult section 3.5

('The TbScan.Lng file') for more information about color codes.

IV - 11

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                SECTION IV

4. TbClean

In order to understand the way a cleaning program operates, try to

imagine how a virus usually goes about infecting a program. The basic

principle is not difficult. A virus - just another program really - adds

itself to the end of the program it is going to infect. The additional

viral code, naturally, increases the size of the program. But simply

appending a viral program to another program is not enough, to do any

real harm - the viral code has to be executed first. So, the virus grabs

the first few bytes at the start of the program, and replaces them with

a 'jump' instruction to its own viral code. That way the virus is able

to take control as soon as the program is started. Chances are you will

never even notice the momentary delay while the extra code is executed,

doing whatever the virus has been programmed to do. The virus then

restores the original instructions and restarts the program (jump to the

start). Your program, more often than not, works as usual - and it goes

without saying that any virus worth its salt will make sure it isn't

going to draw undue attention too soon!

So, in order to purge a program, we must first restore the starting

instruction bytes, which the virus replaced with the jump to its own

code. The virus is going to need these bytes again later on, so they

will be stored somewhere in the viral code. The cleaner starts out to

find those bytes, puts them back in their old place, and truncates the

file to the original size.

Cleaner programs basically come in two types - the conventional type,

for specific types of viruses, and the far more advanced generic clea-

ner, offering a much wider scope. Let's take a closer look at both

cleaner types and find out where they differ.

Conventional cleaners

A conventional cleaner has to know which virus to remove. Suppose your

system is infected with a Jerusalem/PLO virus. When you start such a

conventional cleaner, a procedure much like the following will take

place:

IV - 12

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                SECTION IV

original program        infected program

+--------------+        +--------------+

|              |        |              |

| p            |  100:  |jump          |

| r            |        |to 2487       |

| o            |        | o            |

| g            |        | g            |

| r            |        | r            |

| a            |        | a            |

| m            |        | m            |

|              |        |              |

| c            |        | c            |

| o            |        | o            |

| d            |        | d            |

| e            |        | e            |

|              |        |              |

+--------------+        +--------------+

2487: |              |

|  VIRUS!    p |

|            r |

|jmp 100       |

+--------------+

The virus has itself linked to the program start, and after it rstored

the original bytes (the 'pr' in this example) it jumps then back to the

original program start.

The conventional cleaner: 'Hey, the signature file tells me this file is

infected with the Jerusalem/PLO virus. Ok, let's see, this virus tacks

on 1783 bytes at the end, and it overwrites the first three bytes of the

original program with a jump to itself. The original bytes are located

at offset 483 in the viral code. So, I have to take those bytes, copy

them to the beginning of the file, and I have to remove 1873 bytes of

the file. That's it!'

There are several pitfalls to worry about in a scenario like this.

Obviously, the cleaner has to be given some means to recognize the virus

it is supposed to remove. A conventional cleaner cannot cope with a

virus unless it knows what to look for. It is even more important to

establish the fact that the virus is exactly the same one that the

cleaner knows about after checking the validation data. Imagine what

whould happen if the virus used in the example had been modified and is

now 1869 bytes in size instead of 1873... The cleaner would remove too

much! This is not an exceptional case, certainly not after the unslaught

of countless so-called mutant straints. The Jerusalem/PLO family, to

name but one example, now has more than 100 mutant members!

IV - 13

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                SECTION IV

Generic cleaners

A generic cleaner works on the principle that any kind of virus -

whether or not it has made the signature 'charts' - is bad news. That's

why TbClean works with a completely different disinfection scheme that

is effective with almost all viruses - it does not even need to recogni-

ze them. Actually, TbClean represents two cleaners in one: a 'repair'

cleaner and a 'heuristic' cleaner.

Repair cleaning

Repair cleaning needs an Anti-Vir.Dat file that was generated by TbSetup

before the infection occurred. The Anti-Vir.Dat file stores vital

information about programs, including their original size, the first few

instruction codes and a cryptographic checksum. This information is

usually all it takes to disinfect a file, no matter what virus, known or

unknown, caused the infection. The cleaner will simply restore the bytes

at the beginning of the program, truncate the file to the original size,

and verify the result by way of the original checksum.

Heuristic cleaning

TbClean is the first cleaner in the world that has a heuristic cleaning

mode. This mode does not need any information about viruses either, but

it has the added advantage that it doesn't even care about the original,

uninfected state of a program. This cleaning mode is very effective if

your system is infected with an unknown virus and you neglected to let

TbSetup generate the Anti-Vir.Dat files in time.

In heuristic mode, TbClean loads the infected file and starts emulating

the program code. It uses a combination of disassembly, emulation and,

sometimes, execution to trace the flow of the viral code, pretending to

do more or less exactly what the virus would normally be doing. When the

virus gets to the original program's instructions and jumps back to the

original program code, TbClean stops the emulation process, with a

'thank you' to the virus for its cooperation in restoring the original

bytes.

The actual cleaning process involves almost the same three steps as with

repair cleaning. First the program startup code is repaired and copied

back to the file. Then the viral code, now rendered useless and ineffec-

tive, is removed and, for the sake of security, TbClean will do a final

analysis of the purged program file.

IV - 14

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                SECTION IV

5. TbGensig

5.1 The Purpose of TbGenSig

TbGenSig is a signature file compiler. Since TBAV is distributed with an

up to date, ready-to-use signature file, you do not really need the

signature file compiler.

If, however, you want to define your own virus signatures, you will need

the TbGensig utility. You can use either published signatures or define

your own ones if you are familiar with the structure of software.

One way or another, you only need to do this in case of an emergency,

like in case of the unfortunate event that your machine or even your

company is attacked by a yet unknown, thus not recognized virus. It is

recommended to send a few samples of the virus to some virus experts

anyway, in order to let scanners recognize the virus in the next upgra-

de.

Since it is not possible to explain the whole subject of virus hunting

in one manual, this document assumes you have enough experience and

knowledge to make your own signatures.

TbGenSig searches for a file named UserSig.Dat in the current directory.

This file should contain the signatures you want to add to the TBAV

signature file TbScan.Sig. TbGenSig checks the contents of the User-

Sig.Dat file and applies it to the TbScan.Sig file.

If you want to delete or modify your signatures, just edit or delete the

UserSig.Dat file and execute TbGenSig again.

TbGenSig will list all signatures in the TbScan.Sig file on screen while

being executed.

5.2 Defining signatures

Format of the  UserSig.dat text

You can create and edit the UserSig.Dat file with every DOS editor which

is able to output unformatted text. All lines starting with ';' are

comment lines. TbGenSig file ignores these lines.

Lines starting with '%' will be displayed in the upper TbGenSig window.

In the first line the name of a virus is expected. The second line

contains one or more keywords. The third line contains the signature

itself. This combination of three lines is called a 'signature record'.

IV - 15

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                SECTION IV

A signature record should look like this:

Test virus

exe com inf

abcd21436587abcd

You may use spaces in the signature for your own convenience. TbGenSig

will ignore these spaces.

Adding a published signature

If you want to add a signature that has been published, you should act

as illustrated below.

-    Edit or create the UserSig.Dat file. Convert the published signatu-

re to an acceptable format for TbGenSig.

-    Use keywords COM EXE BOOT INF

You would get:

New virus

exe com boot inf

1234abcd5678efab

-    Execute TbGenSig.

Defining a Signature with TBScan

This section is intended for advanced users who own a TBAV.KEY file.

Although the TbScan.Sig file is updated frequently, new viruses are

created every day, outpacing the regular upgrading service of this data

file. It is therefore possible that one day your system gets infected by

a recently created virus that has not yet been listed in the signature

file. TbScan will not always detect the virus in such cases, not even

with the heuristic analysis. If you are convinced that your system must

have been infected without TbScan confirming this, thischapter will

supply you with a valuable tool to detect unknown viruses with. This

section offers step-by-step assistance in creating an emergency signatu-

re that can be (temporarily) added to your copy of TbScan.Sig

-    Collect some infected files and copy them into a temporary directo-

ry.

-    Boot from a clean write-protected diskette. Do NOT execute ANY

program from the infected system, even though you expect this

program to be clean.

IV - 16

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                SECTION IV

-    Execute TbScan from your write-protected TbScan diskette with the

'extract' option set. Make sure that the temporary directory where

you stored the infected files will be TbScan's target directory.

With its 'extract' option set, TbScan will NOT scan the files but,

instead, display the first instructions that are found at the

entry-point of the infected programs.

=>   Please note that we highly recommend you to simultaneously set the 'log'

option of TbScan to generate a log file.

-    Compare the 'signatures' extracted by TbScan. You should see

something like this:

NOVIRUS1.COM    2E67BCDEAB129090909090ABCD123490CD

NOVIRUS2.COM    N/A

VIRUS1.COM      1234ABCD5678EFAB909090ABCD123478FF

VIRUS2.COM      1234ABCD5678EFAB901234ABCD123478FF

VIRUS3.COM      1234ABCD5678EFAB9A5678ABCD123478FF

If the 'signatures' are completely different, the files are either

probably not infected, or they have been infected by a polymorphic virus

that requires an algorithmic detection module to detect it.

-    There might be some differences in the 'signatures'. You can use

the question mark wildcard ('?') in this case.

A signature to detect the 'virus' in the example above could be:

1234ABCD5678EFAB ?3 ABCD123478FF

The '?3' means that there are three bytes on that position that should

be skipped.

-    Add the signature to the data file UserSig.Dat. Give the virus a

name in the first line of its entry. Specify the following key-

words: COM, EXE, INF, ATE in the second line. Enter the signature

in the third.

You would get:

New virus

exe com ate inf

1234abcd5678efab?3abcd123478ff

-    Execute TbGenSig. Make sure the resulting TbScan.Sig file is in the

TbScan directory.

IV - 17

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                SECTION IV

-    Execute TbScan again in the directory containing the infected

files. TbScan should now detect the virus.

-    Send a couple of infected files to a recommended virus expert,

preferably to the ThunderBYTE organisation.

Congratulations! You have defined a signature all by yourself! Now you

can scan all your machines in search of the new virus.

However, keep in mind that this method of extracting a signature is a

'quick-and-dirty' solution to viral problems. The extracted signature

might not detect the presence of the virus in all cases. A signature

that is guaranteed to detect all instances of the virus can be made only

after complete disassembly of the new virus. For these reasons you

should NOT distribute your home-made 'signature' to others. The signatu-

re eventually assembled by experienced anti-virus researchers will be

completely different in most cases!

5.3 Keywords

Keywords are used for several purposes. They are classified in catego-

ries. Keywords may be separated by spaces, commas or tabs. The maximum

line length is 80 characters.

At least one of the following flags should be specified:

BOOT, COM, EXE, HIGH, LOW, SYS or WIN.

Item keywords

BOOT       Signature can be found in bootsector/partition

COM        Signature can be found in COM programs.

This flag initiates the scanner to search for this signature in executa-

ble files that do not have an EXE header or device header.

=>   Note:   The file contents determines the file type, not the filename

extension!

EXE        Signature can be found in EXE programs.

This flag initiates the scanner to search for this signature in the load

module of EXE type files. EXE files are files that have an EXE header.

=>   Note: The file contents determines the file type, not the filename

extension!

HIGH      Signature can be found in HIGH memory (above program).This

flag initiates the scanner to search for this signature in

memory above the memory allocated by the scanner.

IV - 18

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                SECTION IV

This keyword is intended for resident viruses that allocate memory at

system boot, or viruses that decrease the size of the last MCB (Memory

Control Block).

=>   Note:  The flag HIGH does not mean that the signature should be searched

in UPPER memory.

LOW       Signature can be found in LOW memory. This flag initiates the

scanner to search for this signature in memory below the PSP

(Program Segment Prefix) of the scanner and in the UMB (Upper

Memory Blocks). This keyword is intended for viruses that

remain resident in memory, using the normal DOS TSR (Terminate

and Stay Resident) function calls.

SYS       Signature can be found in SYS programs.

WIN       Signature can be found in Windows programs.

Message keywords

DAM        Message prefix:  'damaged by'.

DROP       Message prefix:  'dropper of'.

FND        Message prefix:  'found the'.

INF        Message prefix:  'infected by'

Message suffix:  'virus'

JOKE       Message prefix:  'joke named'.

OVW        Message prefix:  'overwritten by'.

PROB       Mess. pre-prefix:'probably'.

TROJ       Message prefix:  'trojanized by'.

Position keywords

UATE      Signature should be found at unresolved entry-point.

Purpose:

The signature starts directly at the unresolved entry-point of the viral

code. With some polymorphic viruses, it may be possible to create a

signature from the degarbling routine, although it may either be too

short or give false positives with a global search. An initial branch

instruction  may be part of the signature.

COM type files:  top of file (IP 0100h).

EXE type files:  CS:IP as defined in the

EXE-header.

WIN type files:  Non-DOS CS:IP of the new

EXE-header.

IV - 19

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                SECTION IV

Remarks:

The keyword UATE is not allowed for BOOT, SYS, LOW, HMA or HIGH type

signatures.

ATE        Signature should be found AT ENTRY point.

Purpose:

The signature starts directly at the entry-point of the viral code. With

some polymorphic viruses, it may be possible to create a signature from

the degarbling routine, although it may either be too short or give

false positives with a global search.

Therefore the keyword ATE is used to make sure that the scanners do not

scan the entire file for the signature, but only look at the entry-point

for the signature.

The entry-point of a virus is defined by the first byte that is not

equal to either a JUMP SHORT, JUMP LONG or a CALL NEAR.

Unresolved entry point:1JUMP LONG   3

2 ...

3 JUMP SHORT   5

4 ...

5 CALL FAR     7

6 ...

7 CALL NEAR    9

8 ...

Resolved entry point:9  POP <reg>

The entry-point of the above fragment is Line 9 as this is the

first code to be executed which is not a JUMP SHORT, JUMP LONG or

CALL  NEAR or CALL FAR.

Remarks:

1)   The entry-point can be determined by a code analyzer to cope with

tricks like coding a NOP or DEC just before the branch instruction.

Therefore the results of the scanner should be tested carefully. In

case of trouble use the TbScan 'extract' option to find out what

TbScan considers to be the entry point of the program.

2)   The flag ATE is not allowed for BOOT, SYS, LOW, HMA or HIGH type

signatures.

XHD       Signature can be found at offset 2 of the EXE header.

Purpose:

This position keyword is rarely used. It should only be used to detect

the also very rare high-level language viruses; viruses written in a

language like C or Basic. These viruses normally contain standard setup

routines and library routines which are not suitable to define a signa-

IV - 20

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                SECTION IV

ture. The XHD keyword can be used as a last resort to detect such

viruses.

Remarks:

This flag may only be used for EXE or WIN type signatures.

5.4 Wildcards

In a virus signature, wildcards characters may be used to recognize so

called polymorphic (self- modifying/mutating) virus code. Below a

description is presented of the wildcard notation. All numbers are in

hexadecimal.

Position Wildcards

Position wildcards affect the position where the parts of the signature

will be matched.

Skip

?n         =  Skip n amount of bytes and continue.

?@nn       =  Skip nn amount of bytes and continue.

nn should not exceed 7F.

Variable

*n         =  Skip up to n bytes.

*@nn       =  Skip up to nn bytes and continue.

nn should not exceed 1F.

Opcode wildcards

The 'opcode' wildcards are shaped to detect instruction ranges:

Low opcode

nL         =  One of the values in the range n0-n7.

High opcode

nH         =  One of the values in the range n8-nF.

Intended use of the opcode wildcards:

Suppose a polymorphic virus puts a value in a word register (using a MOV

WREG,VALUE instruction), and increments a register (using an INC WREG

instruction, and pops a word register from the stack (using a POP

instruction). Both the registers and the value are variable.

You could code it like this:

bh4l5h

IV - 21

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                SECTION IV

B8-BF are the opcodes for 'MOV WREG,VALUE', 40-47 are the opcodes for

'INC WREG', and 58-5F are the opcodes for 'POP REG'.

Example

To show the power of the use of the appropriate keywords and wildcards

here is the signature of the Haifa.Mozkin virus. This virus is highly

polymorphic and encrypted. It contains a small variable decryptor to

decrypt the virus.

There are two problems here: most bytes are encrypted or variable, thus

not suitable to be part of a signature, and the remainder is short and

would cause dozens of false alarms.

However, using the appropriate keywords and wildcards, it is possible to

define a reliable signature. The signature below is used by TbScan to

detect the Haifa.Mozkin virus.

Haifa.Mozkin

com exe ate inf

bh?2bh?109?2*22e80?24l4h75fl

Let's analyze it.

The first line describes the name of the virus. The second line tells

the scanner to search for this signature in COM and EXE type files. It

also tells the scanner that it should report the file as infected if the

signature can be matched. The keyword ATE instructs the scanner to match

this signature only at the resolved entry-point of the file. The virus

starts of course with decrypting itself, so it is guaranteed that the

scanner will scan this location. The ATE instruction limits the scope of

this signature to just one position in a file, so this will reduce the

chances of false alarms significantly.

The third line is the signature definition. Let's reverse engineer it:

bh?2      This means: a byte in the range B8-BF followed by two variable

bytes. B8-BF is a 'MOV WREG,VALUE' instruction. From the

register we only know it is a word register, the value is

unknown as well.

bh?109    This means: another 'MOV WREG,VALUE' instruction. The register

is a word register, and from the value we know that it is in

the range 0900 to 09FF.

?2*2      This means: skip two to four bytes. This instruction is inser-

ted by the virus to make it harder to define a signature.

IV - 22

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                SECTION IV

2e80?2    This means: the virus performs an arithmethic byte sized

operation with an immediate value (decrypts one byte) with a

CS: segment override. The exact operation, the memory location

and the value are unknown.

4l        This means: a byte in the range 40-47. This is an 'INC WREG'

instruction. The virus increments the counter to the next byte

to be decrypted.

4h        This means: a byte in the range 48-4F. This is a 'DEC WREG'

instruction. The virus decrements the iteration count.

75fl      Opcode 75 is a JNZ instruction. If the decremented register

did not reach zero, the virus jumps back and repeats the

operation. How much does it jump? That tells the 'fl' part:

somewhere between -16 (F0h) to -8 (F7h) bytes.

Although the signature language of TbGenSig is very powerful, there are

viruses which are so highly polymorphic that they require even more

sophisticated wildcards, keywords or even special detection algorithms.

The explanation however of these wildcards, keywords or algorithmic

detection definitions is so complicated that it is not suitable to be

presented in a user manual.

IV - 23

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                APPENDIX A

Appendix A. TBAV messages

The TBAV utilities may display various messages when executed. Most

messages will be clear enough, but here is some additional information

followed by the name of the appropriate TBAV utility.

TbClean

Anti-Vir.Dat record          The Anti-Vir.Dat record has been found, but

found: information mat-      the information matches the current state

ches the current state       of the file. The Anti-Vir.Dat record was

of the file. Anti-           created after the file got infected, or the

Vir.Dat file was created     file is not changed at all. TbClean is

after the infection.         going to emulate the file to clean it heu-

Trying emulation...          ristically.

Anti-Vir.Dat record          The Anti-Vir.Dat record that belongs to the

found: reconstructing        infected file has been found. The informa-

original state...            tion will be used to reconstruct the file.

Anti-Vir.Dat record not      The Anti-Vir.Dat file did not exist or did

found: original state        not contain information of the infected

unknown. Trying emulati-     program, so the original state of the in-

on...                        fected program is unknown to TbClean. Tb-

Clean will switch to its heuristic mode to

determine the state of the original file.

Note: to prevent a situation like this,

make sure to use the TbSetup program to

generate the Anti-Vir.Dat records.

These records are of great help to TbClean.

When the file is already infected it is too

late to generate the Anti-Vir.Dat records.

Emulation terminate:         The emulation process has been terminated

<reason>                     for the reason specified. TbClan will now

<reason> can be one of       consult the collected information to see if

the following:               it can disinfect the file.

Jump to BIOS code.           The virus tried to perform a call or jump

directly into BIOS code. This process can

not be emulated so it will be aborted. The

program can probably not be disinfected.

Approached stack crash.      The emulated program is approaching a

crash. Something went wrong while emulating

the program so it will be aborted. The

program can probably not be disinfected.

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                APPENDIX A

Attempt to violate li-       TbClean will not disassemble this program

cense agreements.            for obvious reasons.

Encountered keyboard         The emulated program tries to read the

input request.               keyboard. This is very unusual for viruses,

so the file is probably not infected at

all.

Encountered an invalid       The emulator encountered an unknown in-

instruction.                 struction. For some reason the emulation

failed. The program can probably not be

disinfected.

DOS program-terminate        The emulated program requests DOS to stop

request.                     execution. The program is not infected at

all, or infected by an overwriting virus

that does not pass control to its host

program. The program can not be disinfec-

ted.

Jumped to original pro-      The program jumped back to the start po-

gram entry point.            sition. It is very likely it is infected.

The program can probably be disinfected.

Undocumented DOS call        This is very common for viruses that add

with pointers to reloca-     themselves in front of the COM type pro-

ted code.                    gram. The program can probably be disin-

fected.

Encountered an endless       TbClean encountered a situation in which

loop.                        the program is executing the same in-

struction sequences over and over again for

hundreds of thousands of times. It is unli-

kely that the program will ever escape from

this loop, so the emulation will be abor-

ted.

Ctrl-break pressed.          The user pressed <Ctrl><Break> so the clean

attempt is aborted.

Emulation aborted for        If this message is shown, please send a

unknown reason.              copy of the file being emulated to ESaSS BV

or one of the support BBSs.

Sorry, the collected         The heuristic cleaning mode of TbClean is

information is not suf-      aborted and has not been successful. The

ficient to clean file...     only option left is to restore the file

from a backup or to re-install the program.

Collected enough infor-      The emulation of the virus provided TbClean

mation to attempt a re-      with all information to disinfect the file.

liable clean opera-

tion...

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                APPENDIX A

Some DOS error occured.      Some DOS error occured while trying to

TbClean aborted!             clean the file. Check that no files are

read-only or located on a write protected

disk, and make sure there is a reasonable

amount of free disk space.

The clean attempt seems      It seems that TbClean removed the virus

to be successful. Test       from the file. No doubt about the virus: it

the file carefully!          is gone. However, take care and test the

file carefully to see if it works as expec-

ted.

Reconstruction failed.       TbClean tried to reconstruct the original

Program might be over-       file with the help of the Anti-Vir.Dat

written. Trying emulati-     record. However, the attempt failed. Tb-

on...                        Clean is going to emulate the file to try

to clean it heuristically.

Reconstruction succes-       The file has been reconstructed to its

sfully completed.            original state with help of the information

of the Anti-Vir.Dat record. The CRC (check-

sum) of the original file and the cleaned

file are completely equal, so the cleaned

file is almost certain equal to the origi-

nal file.

Staring clean attempt.       TbClean is analyzing the infected file and

Analyzing infected fi-       tries to locate the Anti-Vir.Dat record.

le...

TbDriver

Another version of Tb-       You started a TbDriver.Exe with another

Driver is already resi-      version number or processor type than the

dent!                        TbDriver already in memory.

Cannot remove TbDriver.      You tried to remove TbDriver from memory,

Unload other TSRs first!     but other resident software as loaded after

TbDriver. Resident software can only be

removed from memory by unloading them in

reversed order.

LAN support was  already     You tried to use the option 'net' for a

installed.                   second time, or TbDriver already enabled

network support automatically.

TbDriver not active.         The resident TBAV utilities need TbDriver,

Load TbDriver first!         so you have to load TbDriver first.

TbDriver is not <versi-      The version of TbDriver found in memory

on>.                         does not match the version number of this

resident TBAV utility. Make sure you do not

mix version numbers!

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                APPENDIX A

This version of TbDriver     You are using a processor optimized version

requires a <typeID> pro-     of TbDriver which can not be executed by

cessor.                      the current processor.

TbScan

Cannot create logfile.       The specified log file path is illegal, the

disk is full or write protected, or the

file already exists and cannot be overwrit-

ten.

[Cannot read datafile]       TbScan needs access to its data file to be

able to tell you the name of the virus. If

it cannot access the data file it displays

this message instead of the virus.

Command line error.          An invalid or illegal commandline option

has been specified.

No matching executable       The specified path does not exist, is emp-

files found.                 ty, or is not an executable file.

Sanity check failed!         TbScan detected that its internal checksum

does not match anymore. TbScan is possibly

contaminated by a virus. Obtain a clean

copy of TbScan, copy the program on a write

protected diskette, boot from that diskette

and try again!

TbScanX

Data file not found.         TbScanX has not been able to locate the

data file.

Not enough memory.           There is not enough free memory to process

the data file. Try to enable swapping, or

if you are already doing so, try another

swapping mode. See also section IV, chapter

"Memory requirements".

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                APPENDIX B

Appendix B. TbScan - Heuristic flag descriptions

# - Decryptor code found

The file possibly contains a self-decryption routine. Some copy-protec-

ted software is encrypted so this warning may appear for some of your

files. If, however, this warning appears in combination with, for

example, the 'T' warning, there could be a virus involved and TbScan

assumes the file is contaminated! Many viruses encrypt themselves and

cause this warning to be displayed.

! - Invalid program.

Invalid opcode or out-of-range branch. The program has either an entry

point that has been located outside the body of the file, or reveals a

chain of 'jumps' that can be traced to a location outside the program

file. Another possibility is that the program contains invalid processor

instructions. The program being checked is probably damaged, and cannot

be executed in most cases. Anyway, TbScan does not take any risk and

uses the 'scan' method to scan the file.

1 - 80186+ instructions

The file contains instructions which can not be executed by 8088 proces-

sors, and require a 80186 or better processor.

@ - Strange instructions

The file contains instructions which are not likely to be generated by

an assembler, but by some code generator like a polymorphic virus

instead.

? - Inconsistent header.

The program being processed has an exe-header that does not reflect the

actual program lay-out. Many viruses do not update the exe-header of an

EXE file correctly after they have infected the file, so if this warning

pops up frequently, it seems you have a problem.

c - No integrity check

This warning indicates that no checksum/recovery information has been

found about the indicated file. It is highly recommended to use TbSetup

in this case to store information of the mentioned file. This info can

be used later on for integrity checking and to recover from virus

infections.

h - Hidden or System file.

The file has the 'Hidden' or the 'System' file attribute set. This means

that the file is not visible in a DOS directory display but TbScan will

scan it anyway. If you don't know the origin and/or purpose of this

file, you might be dealing with a 'Trojan Horse' or a 'joke' virus

program. Copy such a file onto a diskette; then remove it from it's

program environment and check if the program concerned is missing the

file. If a program does not miss it, you will have freed some disk

space, and maybe you have saved your system from a future disaster in

the process.

i - Internal overlay.

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                APPENDIX B

The program being processed has additional data or code behind the load-

module as specified in the exe-header of the file. The program might

have internal overlay(s), or configuration or debug information appended

behind the load-module of the EXE file.

p - Packed or compressed file.

The program has been packed or compressed. There are some utilities that

are able to compress a program file, like EXEPACK or PKLITE. If the file

was infected after the file had been compressed, TbScan will be able to

detect the virus. However, if the file had already been infected before

it was compressed, the virus has also been compressed in the process,

and a virus scanner might not be able to recognize the virus anymore.

Fortunately, this does not happen very often, but you should beware! A

new program might look clean, but can turn out to be the carrier of a

compressed virus. Other files in your system will then be infected too,

and it is these infections that will be clearly visible to virus scan-

ners.

w - Windows or OS/2 header.

The program can be or is intended to be used in a Windows (or OS/2)

environment. TbScan will also scan this file for Windows and OS/2

viruses too.

A - Suspicious Memory Allocation

The program uses a non-standard way to search for, and/or to allocate

memory. A lot of viruses try to hide themselves in memory so they use a

non-standard way to allocate this memory. Some programs (high-loaders or

diagnostic software) also use non-standard ways to search or allocate

memory.

B - Back to entry.

The program seems to execute some code, and after that it jumps back to

the entry-point of the program. Normally this would result in an endless

loop, except when the program has also modified some of its instructi-

ons. This is quite common behaviour for computer viruses. In combination

with any other flag TbScan will report a virus.

C - File has been changed

This warning can only appear if you used TbSetup to generate the Anti-

Vir.Dat files. If this warning appears this means that the file has been

changed. If you did not upgrade the software it is very likely that a

virus infected the file! Note that TbScan does not display this warning

if only some internal configuration area of the file changes. This

warning means that code at the program entry point, the entry-point

itself and/or the file size have been changed.

D - Direct disk access

This flag is displayed if the program being processed has instructions

near the entry-point to write to a disk directly. It is quite normal

that some disk-related utilities cause this flag to be displayed. As

usual, if many of your files (which have no business writing directly to

the disk) cause this flag to be displayed, your system might be infected

by an unknown virus.

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                APPENDIX B

=>   Note that a program that accesses the disk directly does not always have

to be marked by the 'D' flag. Only when the direct disk instructions are

near the program entry point it will be reported by TbScan. If a virus

is involved the harmful instructions are always near the entry point,

which is the place where TbScan looks for them.

E - Flexible Entry-point

The program starts with a routine that determines its own location

within the program file. This is rather suspicious because sound pro-

grams have a fixed entry-point so they do not have to determine this

location. For viruses however this is quite common: about 50% of the

known viruses cause this flag to be displayed.

F - Suspicious file access

TbScan has found instruction sequences common to infection schemes used

by viruses. This flag will appear with those programs that are able to

create or modify existing files.

G - Garbage instructions.

The program contains code that seems to have no purpose other than

encryption or avoiding recognition by virus scanners. In most cases

there will not be any other flags since the file is encrypted and the

instructions are hidden. In a few cases this flag will appear for

'normal' files. These files however are badly designed, which is the

reason the 'garbage' flag appears.

J - Suspicious jump construct.

The program did not start at the program entry point. The code has

jumped at least two times before reaching the final start-up code, or

the program jumped using an indirect operand. Sound programs should not

display this kind of strange behaviour. If many files cause this warning

to be displayed, you should investigate your system thoroughly.

K - Unusual stack.

The EXE file being processed has an odd (instead of even) stack offset

or a suspicious stack segment. Many viruses are quite 'buggy' by setting

up an illegal stack value.

L - program Load trap

The program might trap the execution of other software. If the file also

causes flag M (memory resident code) to be displayed, it is very likely

that the file is a resident program that determines when another program

is executed. A lot of viruses trap the program load and use it to infect

the program. Some anti-virus utilities also trap the program load.

M - Memory resident code.

TbScan has found instruction sequences which could cause the program to

hook into important interrupts. A lot of TSR (Terminate and Stay Resi-

dent) programs will trigger this flag, because hooking into interrupts

is part of their usual behaviour. If, however, a lot of non-TSR programs

cause this warning flag to appear, you should be suspicious. It is

likely that your files have been infected by a virus that remains

resident in memory. Note that this warning does not appear with all true

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                APPENDIX B

TSR programs. Nor can TSR detection in non-TSR programs always be relied

upon.

N - Wrong name extension

Name conflict. The program carries the extension .EXE but appears to be

an ordinary .COM file, or it has the extension .COM but the internal

layout of an .EXE file. A wrong name extension might in some cases

indicate a virus, but in most cases it does not.

O - code Overwrite.

This flag will be displayed if TbScan detects that the program overwri-

tes some of its own instructions. However, it does not seem to have a

complete (de)cryptor routine.

R - Suspicious relocator

Flag 'R' refers to a suspicious relocator. A relocator is a sequence of

instructions that changes the proportion of CS:IP. It is often used by

viruses. Those viruses have to relocate the CS:IP proportion because

they have been compiled for a specific location in the executable file;

a virus that infects another program can hardly ever use its original

location in the file as it is appended to this file. Sound pro-

grams'know' their location in the executable file, so they don't have to

relocate themselves. On systems that operate normally only a small

percentage of the programs should therefore cause this flag to be

displayed.

S - Search for executables

The program searches for *.COM or *.EXE files. This by itself does not

indicate a virus, but it is an ingredient of most viruses anyway (they

have to search for suitable files to spread themselves).  If accompanied

by other flags, TbScan will assume the file is infected by a virus.

T - Invalid timestamp.

The timestamp of the program is invalid: e.g. the number of seconds in

the timestamp is illegal, or the date is illegal or later than the year

2000. This is suspicious because many viruses set the timestamp to an

illegal value (like 62 seconds) to mark that they already infected the

file, preventing themselves from infecting a file for a second time

around. It is possible that the program being checked is contaminated

with a virus that is still unknown, especially if many files on your

system have an invalid timestamp. If only a very few programs have an

invalid timestamp you'd better correct it and scan frequently to check

that the timestamp of the files remains valid.

U - Undocumented system call.

The program uses unknown DOS calls or interrupts. These unknown calls

can be issued to invoke undocumented DOS features, or to communicate

with an unknown driver in memory. Since a lot of viruses use undocumen-

ted DOS features, or communicate with memory resident parts of a previ-

ously loaded instance of the virus, it is suspicious if a program

performs unknown or undocumented communications. Nevertheless, it does

not necessarily indicate a virus, since some 'tricky' programs use

undocumented features also.

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                APPENDIX B

V - Validated program

The program has been validated to avoid false alarms.

-    The design of this program would normally cause a false alarm by

the heuristic scan mode of TbScan, or:

-    This program might change frequently, and the file is excluded from

integrity checking.

These exclusions are stored in the Anti-Vir.Dat file by either TbSetup

(automatically) or by TbScan (manually).

Y - Invalid bootsector.

The bootsector is not completely in accordance with the IBM defined

bootsector format. It is possible that the bootsector contains a virus

or has been corrupted.

Z - EXE/COM determinator.

The program seems to check whether a file is a COM or EXE type program.

Infecting a COM file is a process that is not similar to infecting an

EXE file, which implies that viruses able to infect both program types

should also be able to distinguish between them. There are of course

also innocent programs that need to find out whether a file is a COM or

EXE file. Executable file compressors, EXE2COM converters, debuggers,

and high-loaders are examples of programs that may contain a routine to

distinguish between EXE and COM files.

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                APPENDIX C

Appendix C. Solving incompatibility problems

Although TBAV utilities have been designed to cooperate with other

resident software, other software may not, causing system errors or

worse.

Problem:  If any TBAV utility tries to display a message, the text

'message file <filename> could not be opened' appears.

Solution: Specify the FULL path and filename of the file you will use as

message file after the TbDriver loading command. The default

file name is "TbDriver.Lng".

Problem:  One of your utilities is loading a TSR into memory without an

'executable' filename extension, such as .EXE or .COM. Since

TbSetup will only create Anti-Vir.Dat records for files with

an 'executable' extension, there is no Anti-Vir.Dat, so TbMem

is not able to record the TSR permission information.

Solution: Run TbSetup and specify the exact filename of the TSR. TbSetup

will create an Anti-Vir.Dat record, regardless of the filename

extension, so TbMem is now able to record its information.

Although the Anti-Vir.Dat record exists, TbScan will not use

it to check the CRC to avoid false alarms.

Problem:  You are running a network.

TbScanX is installed succesfully, but it does not display the

"*scanning*" message while accessing files. It does not detect

viruses either.

TbCheck is installed succesfully, but it does not display the

"*checking*" message while accessing files. It does not detect

viruses either.

TbFile is installed succesfully, but it does not detect anyt-

hing anymore.

TbMem is installed succesfully, but it does not detect TSRs

anymore.

Solution: Use the command 'TbDriver net' after the network has been

loaded.

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                APPENDIX C

Problem:  The system sometimes hangs when the message "*scanning*" is on

the screen.

Solution: Try TbScanX without option 'EMS' or 'XMS'. If TbScanX now

works without any problems, add option 'EMS' or 'XMS' again

along with option 'compat'. On some systems the TbScanX 'XMS'

option cannot be used at all, because these systems do not

allow the use of extended memory by resident software.

If the problem is related to option 'XMS' and can not be

solved with option 'compat', you can use option 'XMSseg =

<value>' to change the XMS swap segment address. The value

should be between 2000 and 8000. The default value is 4000.

Problem:  After you have given permission for a program to remain resi-

dent in memory, TbMem asks the same question the next time.

Solution: 1)   The 'secure' option of TbDriver is specified. Remove this

option, reboot and try again.

2)   The program mentioned does not appear in the Anti-Vir.Dat

file and therefore TbMem cannot permanently store the

permission flag. Use TbSetup to generate the Anti-Vir.Dat

record of this program!

Problem:  The system sometimes hangs when you answer 'YES' (abort pro-

gram) to a TbMem message.

Solution: None. Some resident programs deeply interfere with the system,

and once they are rejected from memory the state of the system

is not stable anymore.

Problem:  When you load TbDisk from the DOS command prompt everything

works OK. However, when you install TbDisk from within the

Config.Sys or AutoExec.Bat file it keeps on warning that

programs write to disk directly.

Solution: Load TbDisk at the end of your AutoExec.Bat file.

Problem:  You formatted the hard disk using DOS FORMAT.COM, but TbDisk

did not come up with a message until the process was almost

finished.

Solution: This is not a problem. A high level format program like DOS

FORMAT.COM does actually not format the disk, but it reads all

tracks to locate possible bad spots, and finally it clears the

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                APPENDIX C

FAT and directory structure. Only this last step implies a

disk write, so only this last step is detected by TbDisk.

Problem:  After you have given permission for a program to perform

direct disk access, TbDisk asks the same question next time.

Solution: 1)   The 'secure' option of TbDriver is specified.  Remove

this option, reboot and try again.

2)   The program mentioned does not appear in the Anti-Vir.Dat

file and therefore TbDisk can not permanently store the

permission flag. Use TbSetup to generate the Anti-Vir.Dat

record ofthis program!

Problem:  If you try to use Windows fast 32 bit disk access, Windows

comes up with an error message.

Solution: Use option 'win32' on the TbDisk command line.

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                APPENDIX D

Appendix D. Batch file handling

All TBAV utilities return to DOS with an error code which you can use by

the DOS 'errorlevel' command. Consult your DOS manual for information

how to use this command.

TbScan terminates with one of the following exit codes:

Errorlevel      0        No viruses found / No error occurred

1        No files found

2        Error occurred

3        Files have been changed

4        Virus found by heuristic analysis

5        Virus found by signature scanning

255        Sanity check failed

TbUtil terminates with one of the following exit codes:

Errorlevel      0        No error occurred

1        When option 'compare' fails or an error occurs

All other ultilities exit with one of the following exit codes:

Errorlevel      0        No error occurred

1        Error occurred

To detect within a batch file whether a resident TBAV utility has been

loaded you can check for the device names. All TBAV utilities install a

device name, whether they are loaded in the Config.Sys or AutoExec.Bat.

You can use the DOS 'if exist' construction to check for the device

names:

@echo off

if not exist SCANX echo TbScanX has not been loaded!

Or you can branch to a label by using the goto command:

If not exist SCANX goto noscanx

bla bla

:noscanx

bla bla

The TBAV device names are:

TbScanX:            SCANX

TbCheck:            TBCHKXXX

TbMem:              TBMEMXXX

TbFile:             TBFILXXX

TbDisk:             TBDSKXXX

TbLog:              TBLOGXXX

TBAV user manual (C) Copyright 1995 Thunderbyte B.V.                APPENDIX E

Appendix E. Virus naming

How many viruses does TbScan detect?

Most of the TbScan signatures are family signatures: one signature

detects a whole set of viruses. All these viruses are related to each

other. The Jerusalem signature for instance covers more than 100 viru-

ses. For this reason one cannot tell how many viruses TbScan detects.

Some competitive products treat each virus mutant as a separate virus,

thus claiming to detect over 2000 viruses. TbScan, however can detect

viruses using 'only' 1000 signatures. If you want to compare virus

scanners, you have to rely on the tests frequently published in magazi-

nes.

The virus naming convention

TbScan follows the CARO virus naming recommendations. CARO is an organi-

sation in which leading anti-virus researchers participate. Viruses are

grouped in a hierarchical tree, which indicates to which family viruses

belong. TbScan shows the complete CARO name where possible. Many other

anti-virus products however just show the family name or the member

name. For instance, the 'Leprosy.Seneca.493' virus might be indicated by

the familiy name 'Leprosy' or member name 'Seneca', or even by the

variant name '493'.

Anti-virus products developed by non CARO members might even use a

completely different name. TbScan however tries to display as much of

the name as possible. If TbScan is not able to distinguish between the

'Leprosy.Seneca.493' and 'Leprosy.Seneca.517' viruses, both viruses are

indicated by the name 'Leprosy.Seneca'.

Some viruses mutate themselves frequently. To detect all instances of

such a virus it is sometimes necessary to use multiple signatures.

Although these signatures cover exactly the same virus, they do have a

slightly different indication. Behind the name of the virus you will see

a number between anglebrackets. This number however has nothing to do

with the name of the virus, but is there just for maintenance reasons.