CGALIB -- A graphics library for CGA.

Yes, CGA.  You can stop laughing now.

Written by trixter@oldskool.org.

Last updated 20050626

(Note:  This library is UNFINISHED and some routines are still UNIMPLEMENTED.

Skip to the end of this document to read the "To-Do" list to see what is still

unfinished.)

Intro:

This library is unfinished, so the documentation is also unfinished.  Here is

the quick skinny:  CGALIB is a graphics and hardware library for IBM CGA

graphics cards.

The library directly interfaces with the Motorola 6845 character generator

implemented on all true IBM CGA cards to get some things done, like

initializing a screen mode without automatically clearing screen RAM, or

altering registers to make new video modes, etc.  The m6845 had clone variants

that found their way into Hercules, AT&T, Amstrad, etc. with some extended

modes; these extended features are currently unsupported (but might be added

in the future).  While the m6845 is also present on the MDA, the MDA is also

not supported.

CGALIB provides low-level pixel and line primitives, and also a high-level

"wrapper" interface that makes the library easier to use. The wrapper formally

defines some text-based and graphics-based video modes, and wraps common calls

like pixels, lines, boxes, etc. to the appropriate low-level routines.

However, the wrapper interface adds a tiny bit of time to each call due to the

redundant passing of parameters, so if you want the full raw speed (or you're

trying to implement something the wrapper can't do yet), then by all means

feel free to call the low-level primitives directly.

CGALIB also provides a Motorola 6845 character generator interface unit, which

allows you to exactly specify things like vertical total scanlines displayed,

horizontal sync offsets, and other fun registers so that you can experiment

and create new modes.  (Some of these modes, such as 160x100x16, already have

support in CGALIB; others, like a 90x30 text mode or 256x200x4 graphics mode,

have not yet been implemented.)

Specs:

Graphics modes supported include text tweaks (40x25, 40x50, 40x100, 80x25,

80x50, 80x100, 160x100), common graphics (320x200x4 and 640x200x2), and the

99.98% forgotten true 16-color CGA composite 160x200 mode.  All the tools

exist for you to create your own modes (if you know what you're doing!), as

well as completely control existing modes (for example, you have access to the

"third" CGA red-cyan-white palette, can change the 4th black color to any

color you want, etc.).

Mode specifics:

320x200x4 and 640x200x2 are handled normally, no tricks.

Text modes are not handled normally.  :-)  With the exception of 160x100x16,

all of the text modes are implemented with the extended ASCII 177 ("")

character and treat colors as being from 0 to 255.  (This means you can

consider the text modes as having "256 colors", like VGA, except that almost

half of the colors are duplicates of each other, and the palette is fixed.)

This was done on purpose because, on a real CGA monitor, the alternating

pixels blend nicely together.  This allows you to "plot" a pixel by laying

down a single byte, no bit shifting needed.  (In reality, there are duplicate

color combinations, so the actual number of unique "colors" is factoral(16) or

16+15+14...+1 = 136 unique combinations.)  The organization of text modes in

this manner is so easy to manipulate that the UNoptimized line routines for

textmodes are, for most modes, faster than the highly-optimized assembler line

routines for graphics modes.

Library specifics:

Low-level primitives include:

- setting the video mode (properly)

- setpixel

- line (including optimized special cases for horizontal and vertical lines)

- clearing the graphics buffer

High-level routines in the "wrapper" library include:

- Setting modes and filling out a description record with mode details

- high-level drawing primitives:

- rectangle (outline)

- box (filled)

It is worth noting that, as of this writing in the year 2005, there are things

this library does that can't even BEGIN to be emulated by MESS, DOSBox, etc.

I encourage the creators of all PC emulators to use this library to write and

test programs on a real PC/CGA, then use the results to improve the graphics

portion of their emulators.

Using the library:

Sorry, this is the "unfinished" portion of our story.  But it's easy to figure

out; the code is very well documented, so look at testline.pas for an example

of how things work.  Here's some of the files included and an explanation of

what they are for:

cgalib.pas       8499   6-12-105   0:50 CGALIB main "wrapper" library

This is the main wrapper library that initializes video modes, sets mode

variables, etc.

cgalib_g.pas     1661   6-12-105   0:52 CGALIB Global Variables and definitions

This is global variables used by all of the units in CGALIB *except* the

low-level routines.  This is intentional in case you want to use the low-level

routines yourself without all of the wrapper baggage.

crtctwkg.pas     4106   5-23-105   0:15 Tweaks CRTC regs. to make new graf mode

crtctwkt.pas     3974   6-09-105  23:51 Tweaks CRTC regs. to make new text mode

These are interactive programs that let you fuck around with video modes.

Check the source code so you can see what the various keys do.  (256x200.dmp

was produced by the graphics tweaker, for example.)  To avoid losing sync on

your monitor, remember the golden rule:

If (VerticalTotal+1)*(MaximumScanLine+1)+VerticalTotalAdjust isn't 262,

or at least very close to it, the monitor won't be able to sync.

m320_640.pas     3759   6-09-105  21:49 CGALIB low-level routines for graphics

m_lowres.pas    10766   6-10-105   0:00 CGALIB low-level code for 160x100x16

m_textm.pas      8209   6-09-105  23:59 CGALIB low-level code for text modes

Low-level primitives for various video modes.

m6845ctl.pas     9905   6-09-105   0:35 CGALIB Motorola 6845 char. gen. code

The Motorola MC6845 character generator control unit.  Even if you don't use

the functions it provides, it is still useful in that it has all of the common

data values listed as constants.  Check out the colors section :-)

testline.pas     4234   6-12-105  18:01 Visually shows line drawing performance

testpix.pas       654   6-26-105  16:27 Tests the setpixel routine

Check these out for some good examples of how to use the library.

History:

I'm a freak, what can I say?  When I was a teenager, with my clone 8086/CGA, I

played lots of games and marveled at how some coders were able to achieve

quite fast performance on such a limited machine.  When I was older, I got

into the demoscene.  Now that I'm 34 (ACK!) I find myself trying to merge the

two.  Demos... for 8088/CGA... running demos...  A few feverish nights and

many weeks of neglecting responsibilities later, voila:  We have CGALIB.  Yes,

I am crazy.  Someday I hope to create (maybe with other people's help) the

first true 8088/CGA demo that is a *decent* demo, not just some cracker

screens linked into a megademo.  There are lots of things to consider in doing

so, like what to do about music (pc speaker sux, adlib takes up too much CPU

time, digitized sound is too large, etc.)... but I digress.

It may interest you to know that all of the code was written on a real 4.77MHz

8088 with CGA.  In fact, it gets more hard-core than that:  The exact dev

platform -- which I'm writing this text file on right now -- is an IBM Model

5150, the first PC.  By "first", I'm talking about the 83-key keyboard, 5153

color CGA monitor, 5-slot-Rev.B motherboard True Blue IBM original.  The only

thing I added was a 20MB MFM hard drive so that I could more easily develop

the code and make backups.

To-Do:

Tons of stuff still aren't implemented.  On the list to do:

- text-based 60Hz Pageflip tweakmodes for 136 colors

- hidden/offscreen buffering of primitives (both raw and linear)

- more useful wrapper routines like clipping

- more low-level primitives (most notably bitmap handling)

- more high-level primitives like polygons (including filled), scaled bitmaps

using fixed-point math (hint: ADD AX,STEPSIZE gives you AH.AL 8.8 fixed point)

- optimize the textmode lines.  They are currently a generic bresnham (sp?)

that calls setpixel; a better solution would be to do what the graphics line

routines do and increment the address/bitmask as part of the line

calculations.  But I'm lazy.

Anyway, as fun as this has been, I have to get back to reality for a while,

which includes finishing all of the video production for Mindcandy 2, fixing

up the house, etc., etc.  Real Life(tm) stuff.  So, CGALIB, thank you for

diverting my attention for a few weeks so that I could slowly climb out of the

pit of depression that I was living in.  Remember, kids:  Nothing beats

depression like a few obsessive weeks of programming something useless!