LOWRES Unit Documentation

=========================

The LOWRES unit provides access to a 160 by 100 pixel 16 color

undocumented graphics mode.  It is mentioned briefly in the IBM PC

Technical Reference Manual, but no specifics are given as to how this

mode is actually achieved.  The mode is possible on CGA, EGA, MCGA, and

VGA video boards.  Though the resolution is very low, the 160x100 mode

is useful for applications that must run on CGA boards but need more

colors than the 4 available in the 320x200 CGA mode.

The 160x100 "graphics" mode is actually a text mode.  The video

registers are changed so that normal 80x25 color text mode squeezes four

times as many rows of text characters onto the screen.  This produces

80x100 characters.  Then, the screen is filled with character 221, "�".

Each character on the screen is used for a pair of two horizontally

adjacent pixels.  Of the pair, the left pixel's color is controlled by

the foreground color of the "�" character at that location and the right

pixel's color is controlled by the background color.  Finally, the

"blink" option on the video board must be turned off so that pixels do

not blink when the right-half (odd column) pixels (which use the text

background color) have a color value is greater than 7.  Text characters

normally blink if the background color is in the 8...15 range.

100 rows of characters are squeezed onto the screen by telling the video

board that each character becomes one quarter as tall as before.  The

values of several CGA board registers must be modified to change the

text character height and to increase number of displayed rows to 100.

On CGA cards, the character height is changed from 8 to 2 pixels.  MCGA

and some CGA-compatible cards use more than 8 pixels for text characters

by internally doubling the text character height register.  Setting the

character height register to 2 still works correctly on these cards.

On VGA boards, the text character height is normally 16, so it is

changed to 4.  EGA boards, however, normally use a text height of 14

pixels.  There is no way to change the text height to exactly one

quarter of 14 pixels.  A text height of 3 is used, which is slightly too

small: you actually get 116 rows of text instead of the desired 100.

Blink is suppressed to get 16 background colors, so that the odd columns

of pixels do not cause blinking pixels.  This is done on CGA boards by

setting bit 5 of port 3d8 hex to zero.  On EGA and VGA cards, blink is

suppressed by using subfunction 3 hex, function 10 hex of interrupt 10

hex and setting BL to 0.

The 160x100 mode uses 16000 bytes at b800:0000 hex to store 16000 4-bit

pixels, which is twice the amount of memory that would be required in a

normal graphics mode.  Each even byte of video memory is wasted to store

the character 221 ("�").  Moving blocks of video memory requires that

only the odd bytes (storing the text foreground and background colors)

be copied.

It is possible to use similar methods to those discussed above to get a

160x100 monochrome mode that will run on IBM monochrome boards and

Hercules monochrome boards.  It would not, however, be possible to get

three-intensity pixels (on, bright, and off) because a bright background

may not be combined with a normal foreground (and visa-versa) on these

boards.

LOWRES Unit Interface

---------------------

procedure enter_lowres_mode;

This routine enters the 160x100 mode by choosing normal 80x25 color

text mode and then tweaking the video text character height

register to obtain 80 by 100 "characters".  Each "character"

handles two pixels through the foreground and background colors.

16 background colors are obtained by turning the blink option off.

Pixels may not be plotted until "prepare_low()" is called.

procedure prepare_low(x1, y1, x2, y2: integer; fill, c1, c2: byte);

This routine prepares the video memory by filling it with a given

character using given foreground and background colors.  The usual

calling parameters are fill=221, c1=0, and c2=0.  This will fill

the screen with character 221 (the IBM special block character

whose left half is set: "�") with the color being black.  c1 and c2

may be set to other colors (usually equal to each other) to fill

the screen with a desired color.  Other fill characters may be used

to produce different effects.  For instance, characters 176 "�",

177 "�", or 178 "�" may be used to mix colors producing an 80x100

mode with 256 or more colors.  Filling the screen with character

177 with a foreground color of blue and a background color of

magenta will produce what looks like a new color between blue and

magenta.  This effect, however, does not work well on VGA boards

because the 9th column of pixels is left blank in the text font.

When two "�" characters are placed side-by-side ("��"), vertical

bars are visible.

procedure exit_lowres_mode;

Restores the screen to normal text mode.

function video_card_detected: integer;

This routine is used by enter_lowres_mode() to determine which

video card is present.  It uses the Turbo Pascal graphics unit to

perform this test.  You may want to use this routine to test for

unsupported video cards such as the Monochrome or Hercules boards.

procedure lplot(x, y: integer; c: byte);

Plot a pixel at a given x, y coordinate with a given color.  The

color should be in the 0...15 range.  Remember, this routine will

produce undetermined results if prepare_low() is not called

beforehand.  (Fill the screen with character 221, "�".)

function lgetdotcolor(x, y: integer): byte;

Determine what color pixel is at a given x, y screen position.

procedure lline(x1, y1, x2, y2: integer; c: byte);

Draw a line from (x1, y1) to (x2, y2) with color c.  Uses the

Bresenham line drawing algorithm.

procedure lbox(x1, y1, x2, y2: integer; c: byte);

Draw an unfilled box (rectangle) with upper-left corner at (x1, y1)

and lower right corner at (x2, y2) using color c.

procedure lfill_box(x1, y1, x2, y2: integer; c: byte);

Draw a filled box (rectangle) with upper-left corner at (x1, y1)

and lower right corner at (x2, y2) using color c.

procedure lwrite(st: string);

Write text onto the screen using the built-in 5x6 pixel

proportional font using the current foreground and background

colors at the current cursor position.  The cursor position is

updated after writing text so later calls will place text directly

afterwards on the screen.  Use ltextcolor() and ltextbackground()

to set the text color, and use lgotoxy() to set the text position

(if desired) before calling.

procedure lwrite_chr(ch: char);

Write a single character using current position and color.  Updates

text position.

procedure lwrite_num(x: integer);

Write an integer value onto the screen.

procedure lgotoxy(x, y: integer);

Change the text position.  (x, y) refers to the lower left corner

of the next character to be written.

procedure ltextcolor(c: byte);

Change the current text foreground color.

procedure ltextbackground(c: byte);

Change the current text background color.