© 1995 Dana Peters

INTRODUCTION

Overview

Legalities

History

Background

Files

CONSTRUCTION

Skills

Parts

Schematic

Chip Pinouts

Theory of Operation

Construction Notes

CoCo Keyboard Connection

PC Keyboard Connection

Power Connection

Reset Connection

Serial Connection

Firmware Download

Testing

OPERATION

Choosing a Keyboard

Keymaps

Num Lock

Caps Lock

Macro Keys

Keyboard Reset

EPILOGUE

About the Author

Acknowledgements

# Introduction

## Overview

Want a “real” keyboard for your CoCo? Here’s plans for a project to construct an interface to let you use an IBM “AT” keyboard with your CoCo. For those who have re-packaged their hardware inside of a PC case, this is the perfect way of making your machine more useable and professional looking.

The interface connects to the CoCo via the standard keyboard connector. Because it emulates a real CoCo keyboard at the hardware level, no special software drivers are required. This also ensures the AT keyboard will work with all software, even those programs which access the hardware directly (many do).

Besides providing ordinary keyboard facilities, the interface also sports a few extra features. The extra AT keyboard keys can be used as programmable macro keys. The macros can be re-defined at any time using the keyboard, and are stored indefinitely. The ability to reset the computer via the Ctrl Alt Delete key sequence is also provided.

This interface should be usable by all models of CoCo (I, II, and III). Any IBM-compatible “AT” keyboard should work. Please note that the older 84-key XT keyboards are not supported.

## Legalities

This document and associated files are freeware. They may be freely distributed provided they remain intact and unchanged. You may create derived works including commercial product from these plans, however I ask that you inform me of your intentions and acknowledge me in your documentation.

These plans are provided “as is” with no guarantee of correctness or fitness for purpose either explicit or implied. I disclaim any responsibility for losses incurred through the use of these plans. If you totally fry your system building this thing, I will be sympathetic, but not legally responsible in any way. By using these plans, you are deemed to have accepted these conditions of use.

## History

This document has undergone the following changes:

| Revision | Release Date | Comment |
| --- | --- | --- |
| 0.8 | Dec 9/94 | Pre-release to reviewers |
| 1.0 | Jan 5/95 | First general release |

## Background

There have been many attempts at improving the CoCo keyboard over the years. The original CoCo shipped with what was dubbed a “chicklet” keyboard from the resemblance of the keys in size and texture to the chewing gum by the same name. Universally hated, several companies such as Spectrum Projects, KeyTronic, Micronix, Mark Data, and HJL supplied replacement keyboards which provided a much more professional feel. Tandy’s supplied much improved keyboards with the CoCo II and III, but was still molded into the case rather than being detachable. Many CoCo owners (particularly ones who fancied putting their CoCo into a PC case) made their keyboards detachable by grafting a cable between the keyboard and motherboard. There where also attempts at hooking PC keyboards to the CoCo. OWL-Ware produced an AT keyboard interface for the whopping price of $119 US, keyboard not included! (from Rainbow magazine, January 1989, page 69). There is also plans for an interface by Bob Puppo which I have not personally seen. From what I’ve heard, it works only with XT style keyboards, and is relatively complex.

## Future

If there is sufficient demand, myself or another interested party may produce parts kits, printed circuit boards, or completed units for sale. There may also be new firmware revisions to fix bugs or add new features. Please email me and enquire about the latest developments (see section “About the Author” for my address).

## Files

The following files are included with these plans:

| Filename | Description |
| --- | --- |
| AT2COCO.PS | Postscript project plans (this document) |
| AT2COCO.BAS | Basic program (saved in ASCII) to download HC11 firmware |
| AT2COCO.SR | S-record version of HC11 firmware |

# Construction

## Skills

Please do not attempt to construct this project if you are unsure of whether these instructions are sufficient for you. Some prior experience with constructing circuits involving IC’s is a must. I am assuming that most people will want to construct this project using wire-wrap technique as this is cheap, easy, and yields a compact and robust end product.

## Parts

The AT keyboard interface requires one of each of the following chips:

| Number | Manufacturer | Package | Description |
| --- | --- | --- | --- |
| MC68HC811E2P | Motorola | 48 pin DIP | 8 bit microcontroller |
| MT8808AE | Mitel | 28 pin DIP | 8x8 analog switch |
| MC34064 | Motorola | TO-92 | Low voltage inhibit |

The interface also needs the following discrete components:

| Qty | Description | Qty | Description |
| --- | --- | --- | --- |
| 1 | 48 pin DIP wire-wrap socket | 1 | 10 µF capacitor |
| 1 | 28 pin DIP wire-wrap socket | 1 | single-throw switch |
| 1 | wire-wrap proto board | 1 | normally-open push button |
| 1 | 8.0 MHz crystal | 1 | 1K resistor |
| 1 | 10M resistor | 1 | LED |
| 2 | 18 pF capacitors | 24 | 10K resistors |
| 1 | 5 pin DIN connector |  |  |

You will also require some connectors and cable to hook the interface to the CoCo’s keyboard connector. See the section “CoCo Keyboard Connection” for more details.

## Schematic

VDD

PC0

PC1

PC2

PC3

PC4

PC5

PC6

PC7

PD0

PD1

PD2

PD3

PD4

PD5

PB0

PB1

PB2

PB3

PB4

PB5

PB6

PB7

PA0

PA1

PA2

PA3

PA4

PA5

PA6

PA7

E

MC68HC811E2P

VDD

*Reset

In

*Reset

Gnd

MC34064

VDD

8.0 MHz

Extal

Xtal

VDD

VDD

10M

STRB

VDD

+5V

Clock

Data

Grd

VDD

+5V

Grd

MODA

VRL

VSS

*IRQ

VRH

*XIRQ

STRA

MODB

Reset

Run

VDD

1K

AX0

AX1

AX2

AY0

AY1

AY2

Data

VSS

VDD

Reset

Strobe

VEE

X0

X1

X2

X3

X4

X5

X6

X7

Y0

Y1

Y2

Y3

Y4

Y5

Y6

Y7

MT8808AE

R1

R2

R3

R4

R5

R6

R7

C1

C2

C3

C4

C5

C6

C7

C8

See section “CoCo Keyboard Connection”

See section “PC Keyboard Connection”

Download

Reset

See section “Reset Connection”

See section “Power Connection”

10 µF

NC

NC

NC

NC

NC

NC

Pin 3

MC6821

See section “Serial Connection”

All resistors 10K unless otherwise marked.

NC = No Contact

18 pF

18 pF

+

PE0

PE1

PE2

PE3

NC

NC

NC

NC

## Chip Pinouts

Here are the pinouts of the various components. Please note that these are shown from the bottom of the chips (for ease of wire-wrapping).

VDD

PD5

PD4

PD3

PD2

PD1

PD0

*IRQ

*XIRQ

*Reset

PC7

PC6

PC5

PC4

PC3

PC2

PC1

PC0

Xtal

Extal

STRB

E

STRA

MODA

48

47

46

45

44

43

42

41

40

39

38

37

36

35

34

33

32

31

30

29

28

27

26

25

PA7

PA6

PA5

PA4

PA3

PA2

PA1

PA0

PB7

PB6

PB5

PB4

PB3

PB2

PB1

PB0

PE0

PE1

PE2

PE3

VRL

VRH

Vss

MODA

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

MC68HC811E2P

AY1

AY0

AX2

AX1

AX0

X1

X3

X5

X7

VDD

Y0

Y1

Y2

Y3

28

27

26

25

24

23

22

21

20

19

18

17

16

15

AY2

Strobe

VEE

Data

VSS

X0

X2

X4

X6

Reset

Y7

Y6

Y5

Y4

1

2

3

4

5

6

7

8

9

10

11

12

13

14

MT8808AE

*Reset

In

Grd

MC34064

## Theory of Operation

The central part in this design is the Motorola 68HC11 microcontroller. It essentially is an 8-bit computer on a chip, with an instruction and register set similar to the 6809, although not quite as powerful. There are many versions of the 68HC11, but the 68HC811E2 has the following goodies:

- 256 bytes internal RAM

- 2K internal EEPROM

- asynchronous and synchronous serial controllers

- 4 channel 8-bit A/D converters

- 2 general purpose 8-bit I/O ports

- timer system controlling an 8-bit I/O port

The HC11 has many options and abilities I won’t go into, but suffice it to say that it is a very capable chip that can “do it all” letting you design circuits with very low chip count. It’s a shame this project can’t take advantage of all the HC11 has to offer, but with so many diverse options, using them all would be difficult!

To emulate the CoCo keyboard matrix, the Mitel 8808 has been employed. This chip is intended to be used to switch analog telephone calls, but its 8x8 switching matrix is perfect for this application. This could be done with a number of digital chips instead, but the single chip solution allows a much more easily constructed project. My apologies if you really like wire-wrapping!

## Construction Notes

Wire-wrap the circuit according to the schematic diagram shown above. I recommend using a proto board with copper pads (not strips) on the bottom. This permits the discrete components to be soldered in place for greater reliability. I would also recommend using resistor packs to provide the many 10K resistors used in the project. I used three 9-pin SIP packages, each containing eight resistors with one common pin. It is even more convenient if you plug the resistor packs into wire-wrap sockets to allow the connections to the resistors to be wire-wrapped instead of soldered.

Be careful how you wire the crystal part of the circuit. It is important to keep the leads as short as practically possible. The components should be laid out in the manner suggested by the schematic diagram.

The switch, button, and LED in the schematic are only needed when downloading the firmware to the HC11. Because they are not used when the circuit is in normal operation, you needn’t bother in making these accessible. Instead of the switch and/or button, you can use PCB staking pins and an option jumper instead, if you wish.

## CoCo Keyboard Connection

In this section I assume that you are connecting to an “F” board CoCo I, CoCo II or CoCo III, which all have the mylar film connector for the keyboard. Earlier CoCos used an in-line pin connection which will still work with this project, although I won’t bother to describe the appropriate connections.

To connect to interface to the computer’s keyboard port, I recommend you use a ribbon cable. I used a 34 pin cable with crimp-on header connectors at both ends. Since the keyboard port only has 16 pins, one of which is unused, using a 34 pin cable seems like overkill. But this allows standard connectors and ribbon cable to be used with a minimum of fuss. If it bothers you that less than half the wires are being used, then go ahead and deal with each wire separately instead of using a crimp-on connector. There’s several ways of actually plugging into the keyboard port. You can leave the existing mylar film connector in place, and cram a strip of staking pins into the connector into which the ribbon cable can plug in. This will work, although it isn’t too sturdy. It has the advantage that it doesn’t require changes to your motherboard and a CoCo keyboard can be easily plugged in if necessary. Alternatively, you can unsolder the mylar connector and solder in a 34 pin header instead. You will have to cut off 18 of the pins to allow the remaining 16 to be soldered in. This is the approach I’ve taken for my machine. The mylar connector can be put onto the interface board so that a CoCo keyboard can be connected if needed. In fact, you will need a CoCo keyboard prior to downloading the firmware to the HC11, so it may be best if the approach of using a strip of staking pins is used at first. Then, when the interface is working to your satisfaction, you can switch to the header method.

Front

Back

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

| Here is the CoCo’s keyboard connector looking down from the top of the board: Pin | Description |
| --- | --- |
| 1 | R1 |
| 2 | R2 |
| 3 | NC |
| 4 | R3 |
| 5 | R4 |
| 6 | R5 |
| 7 | R6 |
| 8 | R7 |
| 9 | C1 |
| 10 | C2 |
| 11 | C3 |
| 12 | C4 |
| 13 | C5 |
| 14 | C6 |
| 15 | C7 |
| 16 | C8 |

## PC Keyboard Connection

| The PC keyboard connects a 5-pin DIN connector. Radio Shack supplies a cabinet mount connector, or you can buy an inexpensive PC keyboard extender and cut off one end if you want an in-line connector. Here is the pinout looking at the back of the cabinet mount connector for ease of wiring: Pin | Description |
| --- | --- |
| 1 | Clock |
| 2 | Data |
| 3 | NC |
| 4 | Ground |
| 5 | +5V |

5

4

3

2

1

## Power Connection

All that the interface and AT keyboard need is +5V; where you get it is up to you. If you have your CoCo inside of a PC case, I suggest taking power from an extra floppy power connector you’re likely to have. If not, then steal the power from any convenient place on the CoCo motherboard. Use medium gauge wire from the power supply to the interface board and the interface board to the PC keyboard to avoid a voltage drop. The 10 µF capacitor should bridge the supply lines where they enter the interface board. I suggest you verify with a voltage meter that what you think you’ve connected to +5V is actually +5V.

## Reset Connection

To allow the CoCo to be reset from the keyboard, a wire must be run to the non-grounded side of the CoCo’s reset button. Conveniently, this is available on the top side of the board on most CoCos, right on the reset switch itself. You can run a separate wire from the interface board for this, or use an otherwise unused wire in the keyboard ribbon cable.

## Serial Connection

In order to download the HC11 firmware, we need to send data to the HC11’s onboard serial controller. The interface utilizes the CoCo’s “bit banger” serial port for this purpose. It order to avoid having to add RS-232 buffer circuitry to the interface, we need to connect to the bit banger transmit signal before it has been level-shifted, right at the PIA where it originates. Connect to pin 3 of the MC6821 which handles the serial I/O port. On the CoCo III and American CoCo II’s, this is a no-brainer, as there is only one 6821. On other CoCo’s there are two 6821, so you need to be careful which one you use. On Korean CoCo II’s, use IC4 (not IC5). On CoCo I’s (D&E board), use U4 (not U8). This connection is only used for HC11 firmware download, not during normal operation, but it will not impair normal use of the bit-banger port, so you might as well leave it always connected. You can run a separate wire from the interface board for this, or use an otherwise unused wire in the keyboard ribbon cable.

## Firmware Download

Once you’ve constructed the keyboard interface circuit, you must program the EEPROM of the HC11 with the firmware for this application. We are able to do this without special equipment using a remarkable feature of the HC11 called “bootstrap mode”. In bootstrap mode, the processor reads 256 bytes from its on-board serial controller into RAM, and jumps to it. The downloaded program then reads more data from the serial controller and programs the 2K of on-board EEPROM.

The Basic program “AT2COCO.BAS” is supplied to perform the firmware download function. It has both the bootstrap code and the firmware code in S-records embedded in Basic “DATA” statements. The program reads the data and sends it to the HC11 at 1200 baud using “PRINT #-2”. Be sure that your machine is running at stock speed so that the baud rate is correct (or supply the correct constant in the POKE statement which adjusts the baud rate to 1200).

Note that in order to run the download program, you need a working keyboard! This is a chicken and egg problem if you’re unable to plug in your CoCo keyboard because you’ve replaced the Mylar connector with a header. I simply plugged the unsoldered Mylar connector into the connector attached to the keyboard ribbon cable. It doesn’t make a solid connection, but it’s good enough to type the commands needed to get the interface firmware loaded.

If for some reason you want to use your own method of loading the firmware, I’ve supplied the raw S-record file called “AT2COCO.SR”. It contains the same data as the Basic program minus the DATA statements, bootloader code, and other Basic commands.

Down to business. To load the HC11 firmware, follow these steps:

- Ensure the serial connection from the interface to the CoCo is in place (the keyboard connection need not exist, although having it won’t impair the download as long as a CoCo keyboard can still be used).

- Switch the interface board to “download”.

- Power up the CoCo and interface board.

- Load and run “AT2COCO.BAS”.

- Reset the interface board. At this point, the LED should be off.

- Press Enter to begin the download of the bootstrap loader code (after a 10 second time delay to allow firmware upgrades to AT keyboard only systems). As soon as this completes, the LED should turn on.

- After a 10 second time delay to let you check the state of the LED, the download of EEPROM load will begin. At this point, the LED should turn off.

- If the download is completes successfully, the LED should turn on solid. If an error is detected, the LED will go into a flash cycle to indicate a problem.

- Switch the interface to “Run”.

- Plug in an AT keyboard and reset the interface.

If the LED did not behave as expected, the download did not succeed and you need to check your work over. Not a fun task, so be very careful the first time when you are wiring it up, it will pay off.

## Testing

If you’re successfully wired up the interface circuit, downloaded the HC11 firmware, and connected all the interfaces, you should now be able to power up your system and type characters on your AT keyboard which appear on your CoCo’s screen. If not, it’s time to verify that everything is indeed hooked up properly. A voltmeter and logic probe may be necessary to track down the problem.

When the interface is powered up or reset, it will “handshake” with the AT keyboard during initialization. The LED is turned on while handshaking, and turned off when initialization completes. If the LED turns on and stays on, initialization has failed (as will happen if no AT keyboard is plugged in). If the LED doesn’t turn on at all (or flashes wildly) then most likely the processor is insane. Most AT keyboards will also flash their LEDs while initializing. If the LEDs on the keyboard or interface do not behave as expected, you have a problem.

# Operation

## Choosing a Keyboard

As mentioned in the section “Overview”, this interface is designed to work only with AT keyboards. XT keyboards use a different (much simpler) protocol. Since AT keyboards are plentiful now, and XT keyboard are getting rare, the extra firmware needed to handle XT keyboards was not developed. Electrically, the same circuitry would suffice to interface to XT keyboards, so only the firmware would need to be changed to support them, however.

In general, AT keyboards have 101 keys, while XT keyboards have only 84. There are exceptions, however; I have a 84 key AT keyboard, which works fine (although it lacks F11 and F12 keys). There are no 101 key XT keyboards, so you can be certain that these will work. Some keyboards have an AT/XT switch on the bottom; ensure it is set appropriately.

As many 286 systems are being retired from service, the (hopefully) perfectly good keyboards of these machines are also being scrapped. If you look around you should be able to pick up a second hand AT keyboard for next to nothing.

## Keymaps

If you take a close look at both a CoCo and AT keyboard, you will soon realize that there is a distinct difference in the layout of non-alphabetic keys. Because of this, the interface supports two different modes of operation: PC keymap and CoCo keymap. The two modes can be toggled between using the Scroll lock key. When the Scroll lock LED is off, the interface is in PC keymap; when it is on, CoCo keymap is in effect.

In PC keymap, the interface maps the keys based what is printed on the AT keyboard’s keycaps. For example, pressing Shift 2 will generate the @ character. When in CoCo keymap, pressing the same keys will generate the “ character. The CoCo keymap maintains a one-to-one correspondence of PC keys with CoCo keys based partly on their physical layout. This is useful for games which depend on the CoCo’s layout of keys, or for users who can’t quite teach their fingers a different key layout.

## Num Lock

The NumLock key toggles the state of the corresponding keyboard LED, and changes the behavior of the keypad keys. When the NumLock LED is on, the keypad keys behave the same as the keys “0”-”9” and “.”. When NumLock is off, the keypad keys behave the same as the arrow keys and macro keys to the keypad’s immediate left (on a 101-key keyboard). This is an important feature for 84-key AT keyboards which lack this group of keys.

## Caps Lock

This is a potentially confusing feature since both the CoCo and the keyboard have a their own concept of caps lock. In addition, the caps lock state of the keyboard is only effectual when PC keymap is in use. Add to this the fact that Basic on the CoCo defaults to caps lock on, and OS-9 defaults to caps lock off, and the potential for confusion is even greater! But don’t worry, as long as you’re clear what is going on in the CoCo and in the keyboard, you’ll avoid confusion.

In PC keymap mode, the way it works is this: pressing the CapsLock key toggles the state of the corresponding LED and the keyboard’s caps lock state. While the keyboard’s caps lock is on, the interface will automatically press shift whenever an alphabetic character is typed. By pressing Shift CapsLock the interface sends a Shift 0 to the CoCo, thereby toggling its caps lock state. Shift 0 is already assigned to the ) character on AT keyboards, so an alternate key sequence needed to be used.

In CoCo keymap mode, the keyboard’s caps lock state is irrelevant as the shift key is controlled exclusively by the AT keyboard’s shift key. Also, pressing Shift 0 will toggle the CoCo’s cap lock state.

So if there are two caps locks, which one should you use? It’s up to you, but the keyboard based caps lock has the advantage that its state is obvious from the LED. However, when using Basic, you will need to turn the CoCo’s caps lock off, and the keyboard’s caps lock on. Since OS-9 defaults to CoCo caps lock off, it’s easier to use the keyboard’s caps lock.

## Macro Keys

One of the best features of this interface is it’s ability to use the extra keys on the AT keyboard as macro keys. A pre-recorded sequence of keystrokes can be associated with each macro key. The sequence is re-played whenever the key is hit. This is especially useful for commonly used commands, and for defining application specific key combinations for the named keys such as Insert and Page Up. The macro sequences are re-definable at any time through the keyboard, and are saved indefinitely in EEPROM. Each of the 16 available macro keys can have up to 32 keystrokes stored. The sequence is stored exactly as typed, including overlapping keystrokes (one key held down while another is pressed). The timing of keystrokes is not stored, so the keys are replayed at a fixed rate regardless of the delay between keystrokes when recorded.

The macro keys are F1 through F10, and the keys Insert, Delete, Home, End, Page Up, and Page Down. To define a macro key, perform the following steps:

- Press Shift MacroKey.

- Type the sequence of characters you want recorded.

- Press Shift MacroKey again.

The macro keys pressed in the first and third steps must match for the sequence to be stored (as a guard against accidentally re-defining a macro). You cannot use macro key while recording another macro. An attempt to do so will stop the recording. The recording will also stop if the sequence exceeds 32 bytes in length. To erase a macro, simply press Shift MacroKey twice to record a null macro.

To play a macro, press MacroKey. The keys are played back at a pre-determined rate which cannot be adjusted. The playback is not affected by the state of the keyboard caps lock when invoked; the state of the keyboard caps lock is significant only when the macro is recorded. In contrast, the state of the CoCo caps lock does not affect the recording, but it is significant when the macro is played back.

Note that while recording a macro, it is best to avoid key roll-over in order to reduce the number of bytes needed to store the macro. Roll-over is when while typing quickly you press a second key before releasing the first one. Because of how the keys are stored when a macro is recorded, a two key sequence without roll-over takes 2 bytes, where as with roll-over, the same sequence takes 4 bytes. The 32 byte limit is exceeded much more easily if overlapping key sequence information is unnecessarily stored due to roll-over. As a rule of thumb, type slowly when recording macros.

## Keyboard Reset

Another desirable feature of the interface is the ability to reset the CoCo via the “three finger salute,” the “Vulcan nerve pinch,” or Ctrl Alt Delete key combination. Pressing these three keys will cause the CoCo’s reset line to be asserted for a fixed period of time, regardless of how long the keys are held down.

Most CoCo III owners know that there is an “Easter egg” hidden in the CoCo III’s Basic ROM which is triggered if you hold down Ctrl Alt while resetting. Surprisingly, this nifty trick turned out to have practical value as well. Some programs (particularly games) liked to usurp the warm restart vector of the CoCo so that pressing reset would cause you to re-start the program instead of re-entering Basic. This was done both to provide a degree of copy protection, as well as permitting the user to reset until (by random chance) the red/blue colour aliasing which many games employed would have the correct phase to give the proper effect. The problem was that you needed to power off the CoCo to regain control of the machine. But by pressing Ctrl Alt and resetting, then resetting a second time, you could perform a cold restart without powering off your machine!

In order to preserve this useful workaround, you can still reset in this fashion by pressing Shift Ctrl Alt Delete, release all the keys, and press Ctrl Alt Delete again.

# Epilogue

## About the Author

I hope that many people in the CoCo community have as much fun building this interface as I’ve had designing it. I encourage you to send your questions, comments, and greetings to me via email at the address Dana_Peters@Mitel.COM (use ai745@Freenet.Carleton.CA if that fails for some reason). You can also contact me via post at:

Dana Peters

#7-49 Sherway Drive

Nepean, Ontario

Canada K2J 2K3

The CoCo I was my first computer which I received at the age of 15. Since then I’ve owned a CoCo II, TRS-80 Model IV, CoCo III, Amiga 500, Sun 3/60, Amiga 1200, Sparcstation 1, and 486DX/66. But I’ve always had a special fondness for the CoCo, and although I sold my original CoCo systems long ago, I felt a nostalgic need to re-assemble a working CoCo. I was surprised to find a CoCo community still active through listserv@pucc.princeton.edu. I had recently completed an HC11 based AT keyboard interface for my Amiga, and thought that it wouldn’t be much work to adapt it for use on the CoCo. Enthusiastic feedback from the network lead me to complete the project and produce this document. If you’re reading this, you’ve probably come up with a hardware hack or two on your machine. I’d like to encourage you to document your work (no matter how small), and make it available to the community.

## Acknowledgments

Thanks to Alan Dekok for his suggestions and letting me test the interface on his machine, and to David Linsley for providing information regarding the Dragon’s keyboard. A few historical tidbits and information regarding the various models of CoCo came from Tandy’s Little Wonder, The Color Computer 1980-1991, a book by F.G. Swygert.

Thanks to Stan Burton, for creating a high quality development system for the HC11 which he was kind enough to release as freeware for the Amiga. ADEV11 is available from Aminet sites.

And thank you to my special friend Aysa September for putting up with my geeky projects.