PUN - UTILITY

- # -

>x

PUN

===

The PUN Utility is a Diskette Un-Repair utility.  It allows the user to easily reverse the effects of a "repair (alien)" command that was used to convert the data address marks used by TRSDOS to that used by LDOS.  This will allow TRSDOS I to access a directory written to by LDOS.  This program will only work in a Model I (the Mod III cannot reproduce the desired address mark), and will only operate on a single density, single sided, 5" floppy disk.  The syntax of the PUN command is:

=====================================================

!  PUN,:a                                           !

!                                                   !

!          :a  = drive number to un-repair          !

!                                                   !

=====================================================

When the appropriate parameter has been specified, the directory will be read into memory, and immediately written back with the Mod I read-protect data address marks.  This will allow TRSDOS I to access the directory again.

In order to gain compatibility between the models I and III, LDOS has chosen to write the single density directories on a Mod I using the User Defined Address Mark.  This will cause TRSDOS to report a read error on the directory, or at least cause a lot of thrashing while the system does a re-seek 10 times for each sector.  One pass through PUN, and no more problems.