MODEM PROTOCOL DOCUMENTATION

By Ward Christensen     1/1/82

I will maintain a master copy of this.  Please pass on changes or suggestions

via CBBS/Chicago at (312) 545-8086, CBBS/CPMUG (312) 849-1132 or by voice at

(312) 849-6279.

Last Revision: 6/18/85  By Henry C. Schmitt.

State Table Appendix.

Previous Revisions: 1/13/85 By John Byrns.

CRC Option Addendum.

8/9/82  By Ward Christensen.

Change ACK to 06H (from 05H).

| This version of the document was downloaded from the CBBS/CPMUG on 6/13/85

| and the addition of the revision marks (`|') and minor editorial changes were

| made by Henry C. Schmitt.

Many people ask me for documentation on my modem protocol, i.e. the one used

in  the various modem programs in CPMUG, on volumes 6, 25, 40, 47... so here

it is.  At the request of Rick Mallinak on behalf of the guys at Standard Oil

with IBM P.C.s, as well as several previous requests, I finally decided to

put my modem protocol into writing.  It had been previously formally

published only in the AMRAD newsletter.

Table of Contents

1.  DEFINITIONS

2.  TRANSMISSION MEDIUM LEVEL PROTOCOL

3.  MESSAGE BLOCK LEVEL PROTOCOL

4.  FILE LEVEL PROTOCOL

5.  DATA FLOW EXAMPLE INCLUDING ERROR RECOVERY

6.  PROGRAMMING TIPS.

|  7.  OVERVIEW OF CRC OPTION

|  8.  MESSAGE BLOCK LEVEL PROTOCOL, CRC MODE

|  9.  CRC CALCULATION

| 10.  FILE LEVEL PROTOCOL, CHANGES FOR COMPATIBILITY

| 11.  DATA FLOW EXAMPLES WITH CRC OPTION

| Appendix 1.  MODEM PROTOCOL STATE TABLE

-------- 1. DEFINITIONS.

<soh>     01H

<eot>     04H

|  <ack>     06H

<nak>     15H

<can>     18H

|  <C>       43H

-------- 2. TRANSMISSION MEDIUM LEVEL PROTOCOL

Asynchronous, 8 data bits, no parity, one stop bit.

The protocol imposes no restrictions on the contents of the data being

transmitted.  No control characters are looked for in the 128-byte data

messages.  Absolutely any kind of data may be sent - binary, ASCII, etc.  The

protocol has not formally been adopted to a 7-bit environment for the

transmission of ASCII-only (or unpacked-hex) data , although it could be

simply by having both ends agree to AND the protocol-dependent data with

7F hex before validating it.  I specifically am referring to the checksum,

and the block numbers and their ones-complement.

Those wishing to maintain compatibility of the CP/M file structure, i.e.

to allow modemming ASCII files to or from CP/M systems should follow this

data format:

* ASCII tabs used (09H); tabs set every 8.

* Lines terminated by CR/LF (0DH 0AH)

* End-of-file indicated by ^Z, 1AH.  (one or more)

* Data is variable length, i.e. should be considered a continuous stream

of data bytes, broken into 128-byte chunks purely for the purpose of

transmission.

* A CP/M "peculiarity": If the data ends exactly on a 128-byte boundary,

i.e. CR in 127, and LF in 128, a subsequent sector containing the

^Z EOF character(s) is optional, but is preferred.  Some utilities or

user programs still do not handle EOF without ^Zs.

* The last block sent is no different from others, i.e. there is no

"short block".

-------- 3. MESSAGE BLOCK LEVEL PROTOCOL

Each block of the transfer looks like:

<SOH><blk #><255-blk #><--128 data bytes--><cksum>

in which:

<SOH>       = 01 hex

<blk #>     = binary number, starts at 01 increments by 1, and wraps 0FFH to

00H (not to 01)

<255-blk #> = blk # after going thru 8080 "CMA" instr, i.e. each bit

complemented in the 8-bit block number.  Formally, this is the

"ones complement".

<cksum>     = the sum of the data bytes only.  Toss any carry.

-------- 4. FILE LEVEL PROTOCOL

---- 4A. COMMON TO BOTH SENDER AND RECEIVER:

All errors are retried 10 times.  For versions running with an operator

(i.e. NOT with XMODEM), a message is typed after 10 errors asking the

operator whether to "retry or quit".

Some versions of the protocol use <can>, ASCII ^X, to cancel

transmission.  This was never adopted as a standard, as having a single

"abort" character makes the transmission susceptible to false termination due

to an <ack> <nak> or <soh> being corrupted into a <can> and cancelling

transmission.

The protocol may be considered "receiver driven", that is, the sender

need not automatically re-transmit, although it does in the current

implementations.

---- 4B. RECEIVE PROGRAM CONSIDERATIONS:

The receiver has a 10-second timeout.  It sends a <nak> every time it

times out.  The receiver's first timeout, which sends a <nak>, signals the

transmitter to start.  Optionally, the receiver could send a <nak>

immediately, in case the sender was ready.  This would save the initial

10 second timeout.  However, the receiver MUST continue to timeout every

10 seconds in case the sender wasn't ready.

Once into a receiving a block, the receiver goes into a one-second

timeout for each character and the checksum.  If the receiver wishes to <nak>

a block for any reason (invalid header, timeout receiving data), it must wait

for the line to clear.  See "programming tips" for ideas.

Synchronizing:  If a valid block number is received, it will be:

1) the expected one, in which case everything is fine; or

2) a repeat of the previously received block.  This should be considered

OK, and only indicates that the receiver's <ack> got glitched, and

the sender re-transmitted;

3) any other block number indicates a fatal loss of synchronization,

such as the rare case of the sender getting a line-glitch that looked

like an <ack>.  Abort the transmission, sending a <can>

---- 4C. SENDING PROGRAM CONSIDERATIONS.

While waiting for transmission to begin, the sender has only a single

very long timeout, say one minute.  In the current protocol, the sender has a

10 second timeout before retrying.  I suggest NOT doing this, and letting the

protocol be completely receiver-driven.  This will be compatible with

existing programs.

When the sender has no more data, it sends an <eot>, and awaits an

<ack>, resending the <eot> if it doesn't get one.  Again, the protocol could

be receiver-driven, with the sender only having the high-level 1-minute

timeout to abort.

-------- 5. DATA FLOW EXAMPLE INCLUDING ERROR RECOVERY

Here is a sample of the data flow, sendin' a 3-block message.  It

includes the two most common line hits - a garbaged block, and an <ack> reply

getting garbaged.  <xx> represents the checksum byte.

SENDER                                       RECEIVER

times out after 10 seconds,

<---            <nak>

<soh> 01 FE -data- <xx> --->

<---            <ack>

<soh> 02 FD -data- <xx> --->    (data gets line hit)

<---            <nak>

<soh> 02 FD -data- <xx> --->

<---            <ack>

<soh> 03 FC -data- xx   --->

(ack gets garbaged)  <---            <ack>

<soh> 03 FC -data- xx   --->

<---            <ack>

<eot>                   --->

<---            <ack>

-------- 6. PROGRAMMING TIPS.

* The character-receive subroutine should be called with a parameter

specifying the number of seconds to wait.  The receiver should first call it

with a time of 10, then <nak> and try again, 10 times.

After receiving the <soh>, the receiver should call the character

receive subroutine with a 1-second timeout, for the remainder of the message

and the <cksum>.  Since they are sent as a continuous stream, timing out of

this implies a serious like glitch that caused, say, 127 characters to be

seen instead of 128.

* When the receiver wishes to <nak>, it should call a "PURGE" subroutine,

to wait for the line to clear.  Recall the sender tosses any characters in

its UART buffer immediately upon completing sending a block, to ensure no

glitches were misinterpreted.

The most common technique is for "PURGE" to call the character receive

subroutine, specifying a 1-second timeout, and looping back to PURGE until a

timeout occurs.  The <nak> is then sent, ensuring the other end will see it.

* You may wish to add code recommended by John Mahr to your character

receive routine - to set an error flag if the UART shows framing error, or

overrun.  This will help catch a few more glitches - the most common of which

is a hit in the high bits of the byte in two consecutive bytes.  The <cksum>

comes out OK since counting in 1-byte produces the same result of adding

80H + 80H as with adding 00H + 00H.

| -------- 7. OVERVIEW OF CRC OPTION

|      The CRC used in the Modem Protocol is an alternate form of block check

| which provides more robust error detection than the original checksum.

| Andrew S. Tanenbaum says in his book, Computer Networks, that the CRC-CCITT

| used by the Modem Protocol will detect all single and double bit errors, all

| errors with an odd number of bits, all burst errors of length 16 or less,

| 99.997% of 17-bit error bursts, and 99.998% of 18-bit and longer bursts.

|      The changes to the Modem Protocol to replace the checksum with the CRC

| are straight forward.  If that were all that we did we would not be able to

| communicate between a program using the old checksum protocol and one using

| the new CRC protocol.  An initial handshake was added to solve this problem.

| The handshake allows a receiving program with CRC capability to determine

| whether the sending program supports the CRC option, and to switch it to CRC

| mode if it does.  This handshake is designed so that it will work properly

| with programs which implement only the original protocol.  A description of

| this handshake is presented in section 10.

| -------- 8. MESSAGE BLOCK LEVEL PROTOCOL, CRC MODE

|       Each block of the transfer in CRC mode looks like:

| <SOH><blk #><255-blk #><--128 data bytes--><CRC hi><CRC lo>

|       in which:

| <SOH>       = 01 hex

| <blk #>     = binary number, starts at 01 increments by 1, and wraps 0FFH to

|               00H (not to 01)

| <255-blk #> = ones complement of blk #.

| <CRC hi>    = byte containing the 8 hi order coefficients of the CRC.

| <CRC lo>    = byte containing the 8 lo order coefficients of the CRC.

| -------- 9. CRC CALCULATION

| ---- 9A. FORMAL DEFINITION OF THE CRC CALCULATION

|      To calculate the 16 bit CRC the message bits are considered to be the

| coefficients of a polynomial.  This message polynomial is first multiplied by

| X^16 and then divided by the generator polynomial (X^16 + X^12 + X^5 + 1)

| using modulo two arithemetic.  The remainder left after the division is the

| desired CRC.  Since a message block in the Modem Protocol is 128 bytes or

| 1024 bits, the message polynomial will be of order X^1023.  The hi order bit

| of the first byte of the message block is the coefficient of X^1023 in the

| message polynomial.  The lo order bit of the last byte of the message block

| is the coefficient of X^0 in the message polynomial.

| ---- 9B. EXAMPLE OF CRC CALCULATION WRITTEN IN C

| /*

|      This function calculates the CRC used by the "Modem Protocol".  The

|   first argument is a pointer to the message block.  The second argument is

|   the number of bytes in the message block.  The message block used by the

|   Modem Protocol contains 128 bytes.

|      The function return value is an integer which contains the CRC.  The

|   lo order 16 bits of this integer are the coefficients of the CRC.  The

|   lo order bit is the lo order coefficient of the CRC.

| */

| int calcrc(ptr, count) char *ptr; int count; {

|     int crc, i;

|     crc = 0;

|     while(--count >= 0) {

|         crc = crc ^ (int)*ptr++ << 8;

|         for(i = 0; i < 8; ++i)

|             if(crc & 0x8000)

|                 crc = crc << 1 ^ 0x1021;

|             else

|                 crc = crc << 1;

|         }

|     return (crc & 0xFFFF);

|     }

| -------- 10. FILE LEVEL PROTOCOL, CHANGES FOR COMPATIBILITY

| ---- 10A. COMMON TO BOTH SENDER AND RECEIVER:

|      The only change to the File Level Protocol for the CRC option is the

| initial handshake which is used to determine if both the sending and the

| receiving programs support the CRC mode.  All Modem Programs should support

| the checksum mode for compatibility with older versions.

|      A receiving program that wishes to receive in CRC mode implements the

| mode setting handshake by sending a <C> in place of the initial <nak>.  If

| the sending program supports CRC mode it will recognize the <C> and will set

| itself into CRC mode, and respond by sending the first block as if a <nak>

| had been received.  If the sending program does not support CRC mode it will

| not respond to the <C> at all.

|      After the receiver has sent the <C> it will wait up to 3 seconds for the

| <soh> that starts the first block.  If it receives a <soh> within 3 seconds

| it will assume the sender supports CRC mode and will proceed with the file

| exchange in CRC mode.  If no <soh> is received within 3 seconds the receiver

| will switch to checksum mode, send a <nak>, and proceed in checksum mode.

|      If the receiver wishes to use checksum mode it should send an initial

| <nak> and the sending program should respond to the <nak> as defined in the

| original Modem Protocol.

|       After the mode has been set by the initial <C> or <nak> the protocol

| follows the original Modem Protocol and is identical whether the checksum or

| CRC is being used.

| ---- 10B. RECEIVE PROGRAM CONSIDERATIONS:

|      There are at least 4 things that can go wrong with the mode setting

| handshake:

|      1. the initial <C> can be garbled or lost.

|      2. the initial <soh> can be garbled.

|      3. the initial <C> can be changed to a <nak>.

|      4. the initial <nak> from a receiver which wants to receive in checksum

|         can be changed to a <C>.

|      The first problem can be solved if the receiver sends a second <C> after

| it times out the first time.  This process can be repeated several times.  It

| must not be repeated a too many times before sending a <nak> and switching to

| checksum mode or a sending program without CRC support may time out and

| abort.

|      Repeating the <C> will also fix the second problem if the sending

| program cooperates by responding as if a <nak> were received instead of

| ignoring the extra <C>.

|      It is possible to fix problems 3 and 4 but probably not worth the

| trouble since they will occur very infrequently.  They could be fixed by

| switching modes in either the sending or the receiving program after a large

| number of successive <nak>s.  This solution would risk other problems

| however.

| ---- 10C. SENDING PROGRAM CONSIDERATIONS.

|      The sending program should start in the checksum mode.  This will insure

| compatibility with checksum only receiving programs.  Anytime a <C> is

| received before the first <nak> or <ack> the sending program should set

| itself into CRC mode and respond as if a <nak> were received.

|      The sender should respond to additional <C>s as if they were <nak>s

| until the first <ack> is received.  This will assist the receiving program in

| determining the correct mode when the <soh> is lost or garbled.  After the

| first <ack> is received the sending program should ignore <C>s.

| -------- 11. DATA FLOW EXAMPLES WITH CRC OPTION

| ---- 11A. RECEIVER HAS CRC OPTION, SENDER DOESN'T

|      Here is a data flow example for the case where the receiver requests

| transmission in the CRC mode but the sender does not support the CRC option.

| This example also includes various transmission errors.  <xx> represents the

| checksum byte.

| SENDER                                      RECEIVER

|                              <---            <C>

|                                      times out after 3 seconds

|                              <---            <nak>

| <soh> 01 FE -data- <xx> --->

|                              <---            <ack>

| <soh> 02 FD -data- <xx> --->    (data gets line hit)

|                              <---            <nak>

| <soh> 02 FD -data- <xx> --->

|                              <---            <ack>

| <soh> 03 FC -data- <xx> --->

|         (ack gets garbaged)  <---            <ack>

|                                      times out after 10 seconds

|                              <---            <nak>

| <soh> 03 FC -data- <xx> --->

|                              <---            <ack>

| <eot>                   --->

|                              <---            <ack>

| ---- 11B. RECEIVER AND SENDER BOTH HAVE CRC OPTION

|      Here is a data flow example for the case where the receiver requests

| transmission in the CRC mode and the sender supports the CRC option.  This

| example also includes various transmission errors.  <xxxx> represents the

| 2 CRC bytes.

| SENDER                                        RECEIVER

|                                <---            <C>

| <soh> 01 FE -data- <xxxx> --->

|                                <---            <ack>

| <soh> 02 FD -data- <xxxx> --->    (data gets line hit)

|                                <---            <nak>

| <soh> 02 FD -data- <xxxx> --->

|                                <---            <ack>

| <soh> 03 FC -data- <xxxx> --->

|         (ack gets garbaged)    <---            <ack>

|                                        times out after 10 seconds

|                                <---            <nak>

| <soh> 03 FC -data- <xxxx> --->

|                                <---            <ack>

| <eot>                     --->

|                                <---            <ack>

| -------- Apendix 1. MODEM PROTOCOL STATE TABLE

| ---- A1A. CONSIDERATIONS

|      The Modem Protocol can be considered a group of states and transitions.

| States represent certain actions taken by the program and certain expected

| results for those actions.  The transitions are actions taken in reponse to a

| particular result, actions which can result in another state.

|      The state table shows the complete set of states for a program with the

| CRC option.  Programs without this option should ignore the <C> result in the

| Send-Init state and also ignore the Rec-Init-CRC state.

|      There is a minor difference between the Data Flow Examples given by Ward

| Christensen and John Byrns.  This difference is the reaction of the sender

| when the <ACK> to a block is garbled (not lost).  In Ward's example the

| sender reacts by retransmitting the current block.  In John's example the

| garbled <ACK> is ignored and nothing happens until the reciever has a timeout

| and sends a <NAK>.  The state table uses the first method of reacting to a

| garbled <NAK>.  This is the recommended method as the retransmission of a

| data block, even at the lowest baud rates, takes considerably less time than

| waiting for a timeout from the receiver.

|      In the State Table, n is the current block number (therefore n-1 is, of

| course, the previous block number); r is the retry counter and c is the CRC

| handshake retry counter.  The actions n+, r+ and c+ are incrementing the

| appropriate counter.  It should be noted that the action n+ will always cause

| r = 0 or, to put it another way, whenever a block is successfully sent and

| recieved the retry counter is reset.  When a r+ action causes r to reach the

| threshold, an error is generated and the program is aborted.

|      A Result in angle brackets (i.e. < >) is the reciept of that character.

| A Result of "Block..." is the reciept of a complete, valid data block.

| Results of Other and Timeout are the reciept of any unlisted input (invalid

| or incomplete blocks included) and the occurance of a timeout in the

| character recieve routine, respectively.

|      A specific check is made for <EOT> when expecting the first data block.

| This is because some installations (e.g. CompuServe) will send an <EOT> to

| signal that the processor is too busy to successfully transfer a file.

| ---- A1B. STATE TABLE

| State

|    Action on entry

|       Result      Action on result                              Next State

| Send-Init

|    Set checksum mode, n = 0

|       <NAK>       Get data for first block, n+                  Send-Data

|       <C>         Set CRC mode, get data for first block, n+    Send-Data

|       Other       r+                                            Send-Init

|       Timeout     Error                                         Abort

| Send-Data

|    Send Block n

|       <ACK>       Get data for next block, n+                   Send-Data, or

|                                                              Send-EOT, if EOF

|       <NAK> or

|       Other       r+                                            Send-Data

|       Timeout     Error                                         Abort

| Send-EOT

|    Send <EOT>

|       <ACK>       --                                            Exit

|       Other       r+                                            Send-EOT

|       Timeout     Error                                         Abort

| Rec-Init-CRC

|    Set CRC mode, Send <C>, n = 1

|       Block n     Store data, send <ACK>, n+                    Rec-Data

|       <EOT>       Error                                         Abort

|       Other       r+                                            Rec-Init-CRC

|       Timeout     c+                                            Rec-Init-CRC

|       c+ threshold

|                   Set checksum mode, r = 0                      Rec-Init-Cksm

| Rec-Init-Cksm

|    Send <NAK>

|       All         --                                            Rec-Data

| Rec-Data

|    --

|       Block n     Store data, send <ACK>, n+                    Rec-Data

|       Block n-1   Send <ACK>, r+                                Rec-Data

|       <EOT>       If n = 1, Error                               Abort

|                   Else  Send <ACK>                              Exit

|       Other or

|       Timeout    Send <NAK>, r+                                 Rec-Data

| Abort

|    Display error, clean up, abort program

| Exit

|    Clean up, exit program