------------------------------------------------------------

DGTerm EMULATOR PROGRAM REFERENCE MANUAL 1.2

------------------------------------------------------------

**********************************************************************

*                                                                    *

*  DGTerm is copyrighted by Stephen Tomback 1984/85/86.  It is made  *

*  available to the public domain and may be freely distributed.     *

*  It is not to be sold nor should the copyright statement be        *

*  altered or removed.                                               *

*                                                                    *

*  If you find this software useful, a $25 donation or whatever      *

*  you think is fair would be appreciated.                           *

*                                                                    *

*                    5 Troy Lane   Bedford, NY 10506                 *

*                                                                    *

**********************************************************************

--->  OVERVIEW

The Dasher D210 Emulator Program (DGTerm) allows your TRS-80 Model

4/4P/4D to function as a Data General Corporation Dasher D210

terminal.  DGTerm is compatible and may be used with any application

which uses a Data General 6052, 6053, DASHER D100, or DASHER D200.

All the models listed are upward compatible with the functions and

features provided by the D210 display terminal.

--->  FEATURES

The DGTerm emulator can generate all 95 displayable ASCII characters,

the delete character, 30 of the 32 control characters and numerous

special character sequences.  The keyboard cannot generate two control

characters: 1E and 1F which are used as header codes for function

sequences.  The emulation for the characters and functions that are

not visible on the Model 4/4P/4D keyboard are shown in Figure 1.

A feature supported by DGTerm is the host controlled Enable Printer

Pass Through and Disable Printer Pass Through commands compatible with

Data General D400 and D450 terminals.  All characters transmitted by

the host computer are sent to the printer without being displayed on

the video display.

Upon receipt of the command sequence 1E 46 60, all subsequent

characters from the host computer will be sent to the printer without

affecting the display.  Receipt of 1E 46 61 will cause character flow

to return to the display.

DGTerm responds to the READ MODEL ID command with the 6-byte sequence

1E 6F 23 28 58 59 if the  printer is ready and 1E 6F 23 28 50 59 if

the printer is not ready.

--->  CONNECTING THE COMPUTER

The Model 4/4P/4D connects through a modem or directly to a host

computer through the RS-232C port located at the rear of the computer.

--->  ESTABLISHING PROTOCOL

DGTerm uses the COM/DVR program of the TRSDOS Operating System to

configure the RS-232C port to the terminal characteristics for the

specific application with the host computer.  These characteristics

include baud rate, word length, stop bit, parity, output control line

status, and input control line status.

The commands SET, SETCOM, and SYSGEN are used to configure the RS-232C

port and save the configuration.  If you are not familiar with these

commands, refer to the TRSDOS Operating System Manual.

As an example, let's assume we are going to connect a host computer

with the following communication characteristics:

Baud Rate           1200

Word Length            8

Stop Bit               1

Parity                NO

Type the following commands:

SET *CL TO COM/DVR <ENTER>

SETCOM (BAUD=1200,WORD=8,STOP=1,PARITY=NO) <ENTER>

SYSGEN <ENTER>

The above configuration is saved and will be established each time you

press the RESET button.

To display the current configuration of the RS-232C port type SETCOM

<ENTER>

Use the SETCOM and SYSGEN commands to adjust and save the RS-232C

configuration.

--->  USING THE EMULATOR

To use the Emulator press the RESET button so that the RS-232C

configuration will be established.

To start the emulator program type DGTERM <ENTER>.

A  help  screen will be displayed.

Because the Model 4/4P/4D keyboard does not contain the same number of

keys as a Data General keyboard, several keys are emulated as a

combination of key strokes.  The Help screen (Figure 1), provides a

complete list of these combinations.

To enter the terminal mode form the Help screen press <BREAK>.

At anytime you are in the terminal mode and wish to view the Help

screen press <CTRL-O>.

To exit from DGTerm and return to TRSDOS, press <CTRL-E> from the Help

screen.

--->  ERROR CONDITIONS

If you call up the DGTerm program without setting the Communications

Line (*CL) to the COM/DVR program, an error message will be displayed

and you will be returned to TRSDOS.

If a Print Pass Through ON command is received and the printer is not

available, the screen will be cleared and an error message will be

displayed.  Correcting the printer problem will initiate the printing

process.  A CTRL-A will cause the output to the printer to be directed

to the screen.  In either case, when the Print Pass Through OFF

command is received, the screen will be restored to its original

display before the fault occurred.

--->  LIMITATIONS

Due to the hardware constraints of the Model 4/4P/4D, not all of the

features found in the Data General D210 terminal can be emulated.  The

following are features either not supported or are only partially

supported in the DGTerm program.

Features NOT supported

- ANSI mode

- Blink

- Dim

- OFF-LINE mode

- Underscore

Features PARTIALLY supported

- Baud Rate (9600 Max)

--------------------------------

HELP SCREEN

--------------------------------

DATA GENERAL D210 TERMINAL EMULATOR  1.2  2/86

----------- Commands ------------------------------- Characters --------------

CARRIAGE RETURN...<DOWN ARROW>                {.....<CLEAR><SHIFT><COMMA>

CTRL keys 0,A-Z...<CLEAR><0,letter>           }.....<CLEAR><SHIFT><PERIOD>

CURSOR LEFT.......<SHIFT><LEFT-ARROW>         |.....<CLEAR><SHIFT><SLANT>

CURSOR RIGHT......<SHIFT><RIGHT-ARROW>        ~.....<CLEAR><SHIFT><SEMICOLON>

CURSOR DOWN.......<SHIFT><DOWN-ARROW>         \.....<CLEAR><SLANT>

CURSOR UP.........<SHIFT><UP-ARROW>           ^.....<CLEAR><SEMICOLON>

CURSOR HOME.......<UP-ARROW>                  `.....<SHIFT><@>

DELETE............<LEFT-ARROW>                [.....<CLEAR><COMMA>

ERASE EOL.........<CTRL-L>                    ].....<CLEAR><PERIOD>

ERASE PAGE........<SHIFT><CLEAR>              _.....<CLEAR><ENTER>

ESCAPE............<BREAK>

NEWLINE...........<ENTER>

TAB...............<RIGHT-ARROW>           Copyright 1984/85/86  S. Tomback

----------- Functions --------------------------------------------------------

C1-C4...<F3><n><ENTER>         | Return to TRSDOS

SHIFT C1-C4...<SHIFT><F3><n><ENTER>  | from this menu with:

FUNCTION keys 1-15...<F1><n><ENTER>         |       <CTRL-E>

CTRL FUNCTION keys 1-15...<F2><n><ENTER>         |

SHIFT FUNCTION keys 1-15...<SHIFT><F1><n><ENTER>  | Return to this menu

SHIFT CTRL FUNCTION keys 1-15...<SHIFT><F2><n><ENTER>  | from emulator with:

CMD-BREAK ESCAPE...<CTRL-@>               |       <CTRL-O>

SHIFT ARROW keys...<CLEAR><arrow>         | <BREAK> to continue..

(Figure 1)

040386