MEMTEST ver 1.3                          By Shane Dawalt [71076,511]                         For Model 4 with TRSDOS 6.x.x     Those who  are familar  with  the Model  III  know that  a memory  testerprogram was provided  under TRSDOS  1.3.  It was  a nice  tool to have  aroundwhen a bug  crept into  the system.   Unfortunately, the  Model 4 didn't  comewith a  test  program.   The  Model 4P,  and  possibly  the  4D, have  startupdiagnostics which leaves the  'old' Model 4 owners  in the dark.  MEMTEST  waswritten to shead some light in that dark corner of the Model 4.     MEMTEST is a  program which  tests all  available memory  in the  system.MEMTEST automatically tests for the memory.  MEMTEST will check and  configurefor:          o  Main core memory (20k)          o  Up to 32 memory banks (max = 32k per bank)          o  Video memory (2k)          o  Graphics memory (32k)                               Program Execution                               =================     MEMTEST can  be forced  to output  data  to the  printer as  well as  thevideo.  Simply  entering  "MEMTEST<enter>"  on  the  command  line will  causeoutput to flow to  the video.  Entering  "MEMTEST P<enter>" will cause  outputto flow to the video and printer.                               User Interaction                               ================     The user need  only read the  screen as  the testing continues.   <BREAK>may be pressed at any  time to escape from a  memory test.  Upon exiting by  a<BREAK>, or  completion  of  testing,  the  system  will  be  replaced to  itsoriginal condition  prior to  the  execution of  MEMTEST  thus preventing  thefamilar reboot which was a tradition with the Model III tester.                                 Screen Output                                 =============     At initialization,  the screen  will  be cleared  and banner  informationwill be printed.  During testing, the  memory area name will be displayed  anda series of  numbers will be  displayed directly below  the memory area  name.The numbers are the most significant byte of the address being tested.   Thesenumbers are displayed  in 2K  intervals regardless  of the  memory area  beingchecked.  Note that an address relative to  0 is generated for the testing  ofthe graphics board.  This is generated by multiplying the y coordinate by  128then adding the BYTE value of the x coordinate.     If a byte error occurs during testing, an error line is displayed at  thebottom of  the  screen  along  with  the  total  number  of  errors  for  thatparticular memory area.  (After testing of each memory area is completed,  theerror counter  is zeroed.)   The error  line  contains the  address where  theerror occured, the test byte used and a bit map of all errored bits  returned.The bit  map  is  a series  of  numbers  from  7 to  0  where  7  is the  mostsignificant bit and 0  is the least siginficant  bit.  Only the numbers  whichare displayed are  the actual  errored bits.   The errored  bits are the  bitswhich are  logical  1  after  testing.   Those  which  are  not displayed  arecorrect.  To determine what  the original byte  returned from the memory  was,Exclusive OR (XOR) the bit map with the test byte.A typical error line output to the screen would look like this:Error at address 7F08x.  Test byte: 55H  Errored bits:    6 5     2 1 0Error count =      2     This means  that the  byte used  to test  location 7F08x  was 55H.   Thatreturned from  the  memory  location at  7F08x  must  have  been 01010101  XOR01100111 = 00110010 or, for those of  you who like hexidecimal, 55h XOR 67h  =32h.  This means that bits  6,5,2,1 and 0 are  errored (a rather nasty  memoryproblem!).  The error  count simply  means that  a previous  memory error  hasoccured.  The current total of memory errors is 2.                                Hardcopy Output                                ===============     A hardcopy of errors  can be obtained  if a printer  is connected to  thesystem.  Refer to  "Program Execution" on  how to  declare a hardcopy  output.The ouput will be essentially the same as the screen.  The only difference  iswhen testing  is completed  for  a particular  memory  area and  if no  errorsoccured, "No errors occured"  will be printed on  the page.  Also, the  "Errorcount" message will be displayed at the end of the normal message line on  theprinter.                                 Other things                                 ============     MEMTEST does not  destroy memory  contents nor  does MEMTEST destroy  thebank usage flags  of TRSDOS.   Because of  this, applications  may use, or  belinked to, upper banks during testing without  fear of a system crash or  lostdata and the conclusion  of testing.  Memory  will be destroyed between  2600hand 3000h.  This is reserved for program loading/usage.        *** WARNING *** WARNING *** WARNING *** WARNING *** WARNING ***     MEMTEST shuts down  the RTC  interrupt during  the testing process.   Thereal time clock  will not  be accurate  after MEMTEST  has completed  testing.Since the RTC  interrupt is  extinguished, the  cursor will  not flash.  If  aprinter error  occurs,  the  computer will  seem  to  freeze  up.  MEMTEST  iswaiting until DOS  returns control.  During  this time,  no keyboard input  ispossible, i.e. the  <BREAK> key  will not  exit the  program.  After about  10seconds, MEMTEST will resume normal operations.        *** WARNING *** WARNING *** WARNING *** WARNING *** WARNING ***                                    Finally                                    =======     MEMTEST was developed and  tested on a  Model 4 with  128K RAM and an  RSgraphics board installed.  MEMTEST  tested the entire  153.6K of RAM space  injust over 30 seconds  (without hardcopy output).   This timing check will  nothold if the  harcopy option  is selected.   For the  skeptics, MEMTEST  checkseach memory byte with 4 different masks before continuing.A SPECIAL NOTE:          When testing the  graphics memory, be  aware that a  fault          in the  memory  DOES  NOT  necessarily  imply  a  bad  RAM          location.  The  Tandy graphics  board is  build using  the          idea that  4,  64K RAMs  can  be used  like  8, 32K  RAMs.          (This is somewhat comparable to  the bank memory where  8,          64K RAMs are  made to  look like  16, 32K  RAMs).  A  gate          array and  various PAL  chips are  implemented to  convert          the 4 bit  data from  the 4, 64K  RAMs to  8 bit data  and          visa-versa.  Along with doing this bazaar conversion,  the          gate array  is in  charge of  the storing  of the  options          register data, storing  of both  x and  y coordinates  and          taking care of the data register.  As can be seen, a  flaw          in the  gate  array  could  render  any  I/O  port to  the          graphics board and/or any RAM location faulty, whether  it          is or  not.   MEMTEST  does  not  try  to interrogate  the          graphics board to decide where the error is if one  occurs          (this is a job for the tech.).     Comments or questions  may be  sent to my  EMAIL box  or messages may  beleft on the  LDOS forum.   For forum  messages, please  mark them  with my  IDnumber listed in the heading of this document.     Shane Dawalt