DOCUMENTATION FOR THE DIGITAL SMITH CHART PROGRAM `SMITH' BY AA4LSmith calculates resistance and reactance at any point along a transmission line. It also calculates reflection coefficient `rho'and vswr.Resistance and reactance as measured at some point on the transmissionline are entered in response to prompts. The program then calculatesR & X for each n degree point for the next half wave length TOWARD THEGENERATOR. `n' is user selectable. Values for points toward the loadmay be derived indirectly as follows:    Assume that a transmission line is 230 deg long, and that R & X have been measured at the trans-mitter end. It is desired to find the R & X at the antenna. Since all values are the same for each half wave length of a (lossless) line,thisline is equivalent to a line which is 230 -180 = 50 deg long. Hencethe R & X at the antenna is equal to the R & X at a point 180 - 50 or130 deg toward the hypothetical generator from the point at which themeasurements were taken. Think about it!The equivalent of a Smith chart gamma swr circle may be created byasking for a listing for a line which has a known point of R=swr*Z0or R=swr/Z0 and j0.Conductance `G' and susceptance `B' are also calculated for each ndegree point. This data is very useful for the calculation ofparallel matching stubs.To avoid divide by zero errors, enter R=0.000001 or such for a shortedline and R=999999 or such for an open line. Values near 90 deg are somewhat invalid under some conditions. (The tangent of 90 deg is infinite.)The following formula is used by SMITH to calculate physical line lengths: theta = length in ft * 0.367 * freq in mhz / velocity factor.73 es gl de AA4L Bob Johnson 11305 Rums Hill Raleigh NC 27614