NETWORK.BAS V1.0                  AC ELECTRONIC CIRCUIT ANALYSIS PROGRAM                  Configured for the Model 4         Bruce A. Trolli                                                                                                                  2/24/85                                        1        1.0) GENERAL                NETWORK/BAS  outputs  the ac frequency reponse for a user             described  electronic  circuit.  The circuit  description  is             prepared  as  an  input  text  file  containing  a  two  line             specification   for  each  element  in   the   circuit.   The             specification  of  each element requires the user  to  assign             node  numbers  to each node in the circuit.  This  method  is             similar  to  many  commercially  available  circuit  analysis             programs.                  When  NETWORK/BAS is run,  it builds a matrix from  the             user's circuit description file and evaluates the output node             voltage at each frequency under the assumption that the input             node has a 1 volt signal with zero phase angle applied to it.                  At run time the user may specify the  frequencies to be             calculated  as  either a linearly or  logarithmically  spaced             interval.  Output  is  sent to the screen by default  but  an             optional output datafile may be specified at runtime.                  Models   are   provided  for   Resistors,   Capacitors,             Inductors,  Bipolar  transistors (current controlled  current             sources),  Fets  (voltage  controlled  current  sources,  and             Opamps (voltage controlled voltage sources).                                        2            2.0) ACKNOWLEDGEMENTS                  Network/BAS  is  an adaptation of an  existing  circuit             analysis  program  to  run on  the  Model 4.   The   original             article:  "Verify Network Frequency Response With This Simple             Basic Program",  Werner A. Schnider, EDN magazine, October 5,             1977,   was  written  for  a Hewlett  Packard  9830A  desktop             calculator.                   The  program  was  modified to run on an Apple  II  and             reappeared  in  the  magazine  as:  "Basic  Program  Performs             Circuit   Analysis",   Richard  Steincross,   EDN   magazine,             September 1,  1982. This version expanded the capabilities of             the  program  by  adding inductors to  the  network  elements             supported.  It  also  allowed the user to specify  input  and             output nodes at run time.  After testing this program, I have             removed  this  feature because it did  not  produce  reliable             output for a number of simple circuits.                                        3            3.0) REQUIREMENTS                  NETWORK/BAS  runs  under   TRS-80  or  Microsoft BASIC.             It requires a single disk drive Model 4 micro. If your system             has  enough  memory to let BASIC max out its 64k program  and             data space limitation, then NETWORK will be able to handle 40             nodes. If your system has less memory it will be necessary to             adjust the number of nodes ( Y in line 1200 of the program).                  Because the program output has been limited to text, it             can  run  with either the monochrome or  color  adapter.  The             input  and output formats have purposely been kept simple  to             allow  most  systems to run the  program.  (Besides,  I  have             enough  problems making programs print the correct answer let             alone print it on a jazzed up screen.)                                         4            4.0) RUNNING THE PROGRAM            4.1) Number the nodes                       To analyze your circuit, it will be necessary to number             the nodes on your schematic diagram.  These node numbers will             be  used to prepare a precise description of the  circuit  so             that NETWORK can construct the circuits network equations.                  The  input  node must be node number 1 and  the  output             node must be the highest node in the circuit.                  AC  ground is node 0.  The 1 volt input that is applied             to node number 1 is referenced to node 0.            4.2) Prepare Input File                  Prepare   a  text  file  that  contains  a   two   line             description   for  each  circuit  element  according  to  the             definitions below. Note that comments may be added after the            element  type so that the origional reference designations of             your circuit's schematic may be retained as part of the input             file.  This should make it easy to go back and forth  between             your schematic and the input file.                  Also  comments may be added anywhere in the input  file             by placing a ";" at the beginning of the line.             Resistors:                  R                                from node, to node, value (in ohms)                       Example:  A 1000 ohm resistor between nodes 1 and 2.                       R                       1,2,1000                                              Capacitors:                  C                  from  node,  to node,value(in micro farads)                       Example: A 10 uf capacitor between nodes 3 and 6.                       C                       3,6,10                                        5             Inductors:                  L                  from node, to node, value (in henries)             Fets:                   F                  gate node,  source node,  drain node,  transconductance                                                           (amps/volt)                                                   Example: A common source fet with 10 mmhos transconductance.                                                   F                                  2,0,4,.01             Bipolar Transistors:                  B                  base node, emitter node, collector node, beta, b-e resistance                                                                  (ohms)                  Example: Emitter follower with beta=100                                         B                       3,4,0,100,.001             Op Amps:                  O                  + in, - in, + out, - out, gain, output resistance (ohms)                  Example: Voltage follower.                       O                       1,2,2,0,1000000,100                                        6        4.3) RUN THE NETWORK/BAS PROGRAM                      The  program will prompt for an input filename.  It             assumes that your circuit desctription is contained in a file             with a .NET extension.  If you can't remeber the name of  the             circuit  desctription  file,   hit  the  ENTER  key  and  all             filenames  with  extensions .NET on the default disk will  be             listed on the screen.                      The  program  will  tell you the  number  of  nodes             actually  found in the circuit description file and tell  you             which nodes are being used as input and output nodes. At this             time verify these with your origional schematic to catch  any             errors   that  you  may  have  had  in  your  input   circuit             description.                      The  program  will  now  ask for the  name  of  the             optional output file that will be used to store the data from             the program. You can avoid creating the output file by mearly             hitting the ENTER key.                  When  the  program asks for the frequency range  to  be             analyzed,  either a linear increment (a positive number) or a             number  of logarithmically space points (a  negative  number)             may be specified.  If the logarithmic spacing is chosen,  the             number entered is the total number of points to be calculated             over the entire frequency interval.                  At the end of the analysis, the program will prompt for             a  new  frequency range.  If you decline the  opportunity  to             recaclulate  over  a new range of  frequencies,  the  program             terminates.                 4.4) Cautions                 Be  aware that there are some common pitfalls  that  can             occurr  with  circuit simulation programs of this  type.  One             that  frequently occurs is that where a dynamically  unstable             circuit is modeled (your amplifier is really an  oscillator).             Also  be  sure that there are some resistive elements  in  LC             circuits   to  prevent  resonances  from  peaking  into   the             ionosphere. These are only common sense.                 Always   specify   some   resistance  in   the   bipolar             transisitor and opamp models to prevent them from blowing the             program away.                                        7        5.0) SAMPLE CIRCUITS                 Sample  circuit description files have been included  to             help you get the hang of setting up circuits. These are:             Sample1.net - A low pass RC filter circuit

(1 hz cuttoff)             Sample1.net - A high pass RL filter circuit (1 hz cuttoff)             Sample3.net - A  bandpass LC filter circuit  (1 hz center)             Edntest.net - The test circuit from the origional EDN article                                        8        6.0) COMMENTS AND FEEDBACK                 I  would appreciate some feedback from anyone  who  uses             the  program.  I  am planning to improve the program  in  the             future in the following areas:             - Provide a more general purpose analysis that lets               you pick inputs (multiple) and outputs (multiple)               independant of node numbering.             - Allow the output of one analysis to be fed into the input               of the next to get around the memory limitations.             - Add a plot routine (lin, log and semi log) to the program.               If anyone has a general purpose routine like this, I'd               like to hear from you. Most of the public domain stuff is               junk unfortunately.             - Run  it  through a compiler.  Right now the  circuits  are                solved using Cramer's rule which stinks when it comes to               circuits above about 6 or 7 nodes. In general try to get               it to run faster.                                                Bruce  Trolli                                                Cleveland RBBS                                                (216-3310510)                                                       9