SET512/DOC     This is a handy utility that is used to turn off the auto-answer mode onthe TeleTrends TT512P internal modem for the Tandy TRS-80 Model 4P.   I havenot been able to find any references to the name of the original author, butI would like to take this opportunity to thank him/her for this needed utility.     The command is invoked by typing "SET512" at the TRSDOS Ready prompt.There are no parameters.     There are two versions of SET512/CMD and /SRC included in this /ARC file.SET512/CMD and /SRC are original program (with one minor enhancement) whileSET512V2/CMD and /SRC are much enhanced.     The original SET512/CMD that I downloaded from the 80 Micro BBS would abortany JCL file that it was included in.  I disassembled the /CMD file and noticedthat when SET512 called the @EXIT supervisor call it failed to zero the HLregister pair.  DOS interpreted this as an error condition and caused the JCLfile to abort.  I added one line to the source file and re-assembled it.  Thesource code looked like this before added the line:       ...       LD      A,@EXIT         ; Return to DOS       RST     28H             ;       ...     I added the following line between these two:       LD      HL,0000H        ;       without an errorThis allowed the JCL file to continue with any other commands with out the"Job aborted" message calling a halt.     There was one more "flaw" that I felt existed with the original program.SET512 had to load COM/DVR before it could talk to the modem.  If COM/DVR hadalready been installed, then SET512 would abort without sending "ATS0=0" to themodem.  I saw this as unacceptable, so I added code to the original source thatchecks to see if the communication line device *CL has been installed.  If *CLis present then SET512V2 branches to the routine that sends "ATS0=0" to themodem.  If COM/DVR is NOT installed then SET512V2 installs it and continueson.  See the two /SRC files for a comparison.     If you happen to know who the original author is I would appreciate a noteletting me know.  Also if you have any suggestions or make any changes pleaselet me know.  I can be reached at the following addresses:       CompuServe   73720,1574       GEnie        C.CASSIDY