SmartWatch Background Task Instructions The three programs SMART1/CMD, SMART3/CMD and SMART4/CMD are copyrighted by Duane M. Saylor, 854 Eads Court, Newport News, VA.  As such, they may not be sold as a separate item or included with another software package or hardware device.  The programs are being distributed as "freeware".  That is, they may be copied and distributed to others freely.  If a user of these utilities finds them useful, he is ask to make a contribution to the author of $10.00 or more as his conscience dictates. Since I do not have unlimited funds for Compuserve access, I suggest you direct your questions to me on my BBS system known as the VolksBoard at 804-220 0003.  I will answer some questions on Compuserve but if it begins to require too much online time my support will be limited to questions on the VolksBoard. SMART is a background task that can be installed in memory and SYSGENed. It requires the installation of the Dallas Semiconducter SmartWatch DS 1216E as described in the CLOCK/DOC file. The Model I version takes 267 bytes of memory, the Model III version takes 266 bytes of memory and the Model 4,4P version takes 306 bytes of memory. Once installed, any time lost by the system clock during disk I/O will automatically be corrected within the time interval you have specified. SMART1 (TIME=,SLOT=,YANK=)  -  MODEL I under LDOS SMART3 (TIME=,SLOT=,YANK=)  -  MODEL III,4,4P under LDOS SMART4 (TIME=,SLOT=,YANK=)  -  MODEL 4,4P under TRSDOS 6 or LS-DOS 6 TIME= is the time interval for the task to update the system clock. SLOT= is the slot in the task vector table. YANK= permits removal of the task module or disabling of the task. The slot and the time update interval is selected when you install.  Slots 0 to 7 are suggested and time intervals 1 to 63 seconds.  The default slot is 0 and the default interval is 60 seconds.  The interval is not exactly the number of seconds entered but is within 7%.  Thus if you enter T=60 you will actually update every 64.25 seconds.  More accuracy would require mutiplying the number input by 3.74 instead of 4.  Multiplying by 4 in assembler is two rotate instructions.  Multiplying by 3.74 gets a bit more complicated! Once installed the time update interval can be changed by using the T parameter with a different number.  If you enter a number larger than 64 the resulting time interval used is modulo 64.  Thus if you enter a Time of 65 you will get 1 second.  If you enter a time of 64 you will get 63.75 (approx). If the SMART module is the first one in memory above HIGH$, it can be removed by using the Yank parameter.  Again the Yank parameter can be shortened to Y if desired. If the SMART module is not the first one in memory above HIGH$, the background task will be disabled but the SMART module will remain in memory. If the task has been disaabled, it can be re-enabled by giving the SMART command with no parameters (in which case the default interval of 60 seconds will be used) and the task will be installed in the same slot it was removed from.  You can also specify a selected time interval using the T parameter.                                    Page 1 Model 4 Installation SMART4 (SLOT=1,TIME=20) NOTE:Both Slot and Time can be shortened to the letter S and T. Will install the task in the next available high memory location on a Model 4 using task slot 1 and a time interval of 20 seconds. The command SYSGEN will save the task into the CONFIG/SYS file for automatic installation on the next boot. If you are operating with a hard drive you should use the command SYSGEN (DRIVE=n) where n is the drive number of your boot drive while operating from the hard disk. Model III installation SMART3 (SLOT=1,TIME=20) Will install the task in the next available high memory location on a Model III using task slot 1 and a time interval of 20 seconds. The command SYSTEM (SYSGEN) will save the task into the CONFIG/SYS file for automatic installation on the next boot.  If you are operating with a hard drive you should use the command SYSTEM (SYSGEN,DRIVE=n) where n is the drive number of your boot drive while operating from the hard disk. Model I installation SMART1 (SLOT=1,TIME=20) Will install the task in the next available high memory location on a Model III using task slot 1 and a time interval of 20 seconds. The command SYSTEM (SYSGEN) will save the task into the CONFIG/SYS file for automatic installation on the next boot.  If you are operating with a hard drive you should use the command SYSTEM (SYSGEN) followed by COPY CONFIG/SYS.CCC :n where n is the drive number of your boot drive while operating from the hard disk.                                      Page 2##