ICON CONVERSION    I started using Dave Millers  PostMaster program and noticed  that I had seen  some of the icons  previously in one of the  public domain PrintMaster  or PrintShop  icon files.   I had  several left over from when I was using P2DOTx to convert to  DOTWRITER fonts.   I dug  out my  trusty zapper  and noticed  that the data  that makes  up the  icon is  indeed the same.  The method that PostMaster, PrintMaster and PrintShop use to  format the Data for processing, however is quite different.   PrintShop  seems to be the  easiest to understand  so I will  start with it.  PrintShop icon sets consist of two files,  a  name file and  a data  file.   The name  file usually starts  with GR and has a /NAM extention. eg.  GRxxxx/NAM.  The data  file usually  starts  with GR  and has  a /DAT extension.  I  have seen different  file ids and  extensions but you should  be aware that ICONV requires /NAM and /DAT extenders be used  on PrintShop files.   Each Name in  the /NAM file is allowed  16 bytes.   The name,  if  shorter than 16  bytes is usually  padded with zero bytes,  I have  seen 20H (space) bytes used  for padding  though.   Each name  in the  /NAM file  has 572  bytes associated with  it in the  /DAT file and  in the same  order.  An icon consists of  bits  88 horizontal  by 52 ver-  tical.  The bits can be either on "1" or off "0".  88 x 52 =  4576 bits,  or divided by 8 result  in 572 bytes of data for  each icon.   PrintMaster files are a little different.   The name file is  the same  format but  is identified  by the  /SDR extention.  The /SDR extention is required to work correctly with ICONV.  The Data file is  identified by the  /SHP extention and adds  some bytes to the file.  Each icon in the data file is again  represented by the same  amount  of  bytes  (572)  but has 5  additional  bytes  per  icon.    Each  icon  data  field  in  PrintMaster consists of the 0B 34 58 00 bytes,  572 bytes of  icon data and one more byte of 00,  for  577 total bytes per  icon.  I don't really know what the 0B 34 58 series of bytes  are used for  but have observed  that the 0BH  is 11 decimal  and is A Vertical  Tab printer code.  the  34H and 58H bytes  are 52 and  88 decimal  and that  is the  dot field size for  each icon.   PrintShop files usually  are limited  to 120  icons each and  PrintMaster to 140.  I have seen larger though and allow for  it in ICONV.   PostMaster Icon sets  are limited to  30 each and  are a bit  different than either PrintShop or PrintMaster.   Instead of  using two files  as in  the case  of the others,  PostMaster  combines the names and  data in one  file.  The names are in  the beginning of the  file and consist of  22 bytes each and  use 20H for padding. I think Dave does this for spacing con-  sideration on his  Library print  outs but  I am  not really  sure.   If not a  full (30  name) file  the remainder of the  name portion is padded out by one  FFH byte and 21 20H bytes  to keep the  name field 22  bytes long.  for a  total of 660  bytes for all 30 names.   The  next byte,  the 661st or byte  660,  depending on whether you start  with 0 or 1 starts the  0B 34 58 00 as in PrintMaster.  The next 572 bytes represent  the icon data  and the  00 byte  used in  PrintMaster is not  used in PostMaster.   You will notice that 30 times 16 bytes  for the names, plus 30 times 576 for the icon data Make each  PostMaster Icon File 17632 bytes long.   Having explained the  above  you  can  see  the difficulty I  encountered when trying to  get an even  LRL (logical record  length) to work  the  PrintMaster  file.  :-)  The number of  files is derived by  dividing LOF of the  Name file by 2 and  hoping the LOF  was not  botched  during file transfer.  You  may want to ZAP the file to check on this.   Once the number  of files  are known in either  the PrintMaster  or PrintShop  files,  it simply becomes a matter of pure drudge to convert  the files.   If  using basic,  you  can enter  basic without  specifying  files,  as only 3 are needed.  You open the name  and data file  for input  and the  PostMaster icon  file for  output.   The Source files are read in until you reach 30 or  end of  file.   They are  padded with  20 bytes  to reach 22  bytes length then written to the PostMaster file.  after the  30  name  files   are   added   processing   continues.  The  PostMaster files are moved byte  for byte but the 577th byte  is stripped off.  Processing  continues 30  icons at  a time  until all  names in  the  source  file  have been processed.  PrintMaster files are handled  in a similar  manner the main  difference being the adding of 0B  34 58 00 to the beginning  of the icon data bits.   Steve Jerkins came over to the TrashBin while I was convert-  ing some of  the  files  and  commenced  with  the "Gee Wizz  that's neat but....".   Steve  is responsible  for the bells  and whistles  in the  program,  and I  might add  slowing an  already slow  process  half  again.   I  asked myself,  Do I  really want to rewrite  this in  assembly,  or do  I want to  play with PostMaster?  It's obvious the latter urge won out.  I grabbed the trusty ol' BASCOM and you see the result.  Use  the ICONV/CMD if you really have a bunch of files to convert  as the ICONV basic program is sloooow.   An ascii version of  the basic program has  been provided for  you to pick apart.  The only difference in the programs other than speed, is the  adjustment done to  the first loop for timing purposes.  You  didn't really think I would allow  my name on the screen for  a mere  few  miliseconds  did  you?   By  the  way make sure  ICONV/OVR is available  or the program  will bomb.   You are  free of course to take it out  if you like because Steve and  I hereby release it to the world.  We make no claim that the  program is bullet  proof  but  I  have converted PrintMaster  files containing over 600 icons.  A  new  thought....  Speaking  of  the  "Gee  Wizz  Buts"...  Wouldn't it be nice to convert PrintMaster,  PostMaster,  or  PrintShop icons to /BLK files for PRODRAW or vice versa.  In  the interest of  science (that's  my excuse  to the wife for  the time I spend  on the machine),  I  Lifted 2 Blocks using  PRODRAW and figure 1.  and 2.  are the result of zapping the  Blocks.  Figure 1 is the 88 by 52 bit solid block. figure 2.  is a 40 by 20 block.   The first two bytes in each file rep-  resent the length of  the file,  not counting  the first two  bytes.   The next seven bytes is the file name sans the /BLK  extention.   The next two bytes are the x or row bit count -  1 the next byte  is the y or  column count -1.   How come we  need two  bytes to  represent the  x or  row count  you ask?  Cause we cam have an x count  of up to 640 I guess.   I hope  to have this Idea working by the next issue.    The basic program, ICONV/ASC is commented well enough to let  you understand  how it works.  If you examine the two sample  files provided you should quickly grasp the logic used.  The  files provided are  each, short  PrintMaster  and PostMaster  name and data files.  Have fun.                             Figure  1.      0000:00=46 02 42 49 47 47 45 52 20 57 00 33 FF FF FF FF     0000:10=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF     0000:20=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF     0000:30=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF     0000:40=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF     0000:50=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF     0000:60=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF     0000:70=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF     0000:80=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF     0000:90=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF     0000:A0=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF     0000:B0=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF     0000:C0=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF     0000:D0=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF     0000:E0=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF     0000:F0=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF                 Zap listing of second sector left out.      0002:00=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF     0002:10=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF     0002:20=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF     0002:30=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF     0002:40=FF FF FF FF FF FF FF FF 00 00 00 00 00 00 00 00     0002:50=00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00                           Figure  2.      0000:00=6E 00 73 62 20 20 20 20 20 27 00 13 FF FF FF FF     0000:10=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF     0000:20=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF     0000:30=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF     0000:40=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF     0000:50=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF     0000:60=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF     0000:70=00 00 FF FF FF FF FF FF FF FF FF FF FF FF FF FF     0000:80=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF     0000:90=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF     0000:A0=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF     0000:B0=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF     0000:C0=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF     0000:D0=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF     0000:E0=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF     0000:F0=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF        Steve Jerkins and Tom Perry  can be reached for comment     or discussion on The TrashBin BBS,  8N1, 300/1200/2400,     24 hour operation.  (804)588-3013.

Located in Norfolk,     Virginia. Tom can also be reached at 2054 Parkview Ave.     Norfolk Virginia  23503.  Voice phone is (804)588-6564.     if calling voice please remember, Tom lives on the east     coast.