V O I C E 4   -   InstructionsTo use VOICE4/CMD, you must have a TRS-80 Model 4 or 4D, the cassetteinterface cable from Radio Shack and a cassette recorder.  It runs with 64Kof memory, using TRSDOS 6.2 or 6.3.Commands-R>ecord:  This allows you to record speech.  To record, attach the blackcassette input plug to the EAR socket on the tape recorder, and attach thegrey cassette output plug to the amplifier.  Then put the recorder intoRECORD mode.  Press <R> and <ENTER>, and talk into the tape recorder'smicrophone for up to 60 seconds.  You may be able to record slightly moreor less, depending on if there are high-memory drivers and filters.  Talkingfor too long will crash the system.S>ave:  After you record speech, you will want to save it on a disk.  You cansave to a disk file following TRSDOS 6 conventions.  Make sure to type thefilename in UPPERCASE letters (TRSDOS will not accept lowercase).L>oad:  This command lets you load in previously saved voice files.  You canthen play them back using the <P>lay command.P>lay:  Before you use this command, make sure that the grey output plug ishooked up to an audio amplifier.  Once this is set, hit <P>lay and listen tothe computer play it back.  The quality of the recording will be poor (as inthe talking games for the Model 3), but understandable.Q>uit:  Exits to TRSDOS.  Any VOICE files in memory cannot be recovered onceyou exit to DOS.Technical information:VOICE4 is the TRSVOICE/CMD program ("Speech Box From the Junkyard") modifiedfor operation under TRSDOS 6.2.  It utilizes all of the memory from 3200H toFFFFH, allowing 8-9K more space than the Model 3 version.You can modify VOICE4 to play back through the internal speaker in the Model4 using the command:PATCH VOICE4/CMD (X'30AD'=90)Note that playback quality through the internal speaker is considerably poorerthan through a separate audio amplifier.Model 4P users can use the PLAYBACK function of this program withoutmodification, but can't record due to the lack of a cassette port on theirmachines.