Supplied Software Description          File Name     Description          WD/DCT        Hard disk driver     WDFORMAT/CMD  Hard disk formatter     WDVERLOK/CMD  Hard disk verifier     WD/SVC        TRSDOS 6 Hard disk driver     WDFORMAT/SVC  TRSDOS 6 Hard disk formatter     WDVERLOK/SVC  TRSDOS 6 Hard disk verifier              1.  Boot up the computer using your system diskette. This should be a BACKUP, not a MASTER.          2.  Copy the hard disk driver from the supplied diskette onto your system diskette.          3.  Install the driver in memory by typing the command: SYSTEM(DRIVE=d,DISABLE,DRIVER="WD") {"WD/SVC" on TRSDOS 6}, where d is the desired logical drive number(1-7).  Answer the questions according to the physical specifics of your hard drive and your desired logical drive.  See the enclosed description for further information.  Optionally, a JCL can be used for this step.          4.  You will now format the hard disk using the WDFORMAT utility. The syntax is WDFORMAT :d (or WDFORMAT/SVC :d on TRSDOS 6).  The drive will be physically formatted and thestandard FORMAT utility will be entered automatically.  Give thename & password you desire. You will be informed that the destination disk is a hard disk.          5.  You may now optionally verify the hard disk using theWDVERLOK utility.  The syntax is WDVERLOK :d (or WDVERLOK/SVC:d on TRSDOS 6).  Any errors on the disk will be assigned sothat the operating system does not attempt to use the space.    6.  Repeat steps 3, 4 & 5 for any other logical drives you wish to setup.          7.  Type the command: SYSTEM(SYSGEN) (SYSGEN on TRSDOS 6).          8. Type the command: BACKUP :0 :d, where d is your desired new system drive.          9.  Up to this point, the hard drive has NOT been the system drive(:0).  In order to make this change, type the command: SYSTEM(SYSTEM=d).  Your floppy will then be known as :d and the hard drive as :0. You can make this an AUTO command on your floppy boot disk.                    Description of WD/DCT (or WD/SVC)          The main driver is a little under 256 bytes long and will only load once.  Each addtional drive adds about 25 bytes to this number.  Pressing <BREAK> at any time will abort the process.          The first question asked is the drive select address.  The default is 1.  After answering this question, the program will search all active drives to see if they are using the main driver. If so, it will use that driver. This means that if you are reinstalling the SAME logical drive number, you should RESET.          Now, enter the starting head number.  The default is 1. Used heads will not be locked out, so that you may use the cylinder offset feature.  Note that you should only enter a number which actually exists on your drive.          The next question is the number of heads.  The default is 1.  The program will trap you from entering a value which will exceed eight heads.          Next, enter the number of sectors per gran. The default is 16.  Other values can be used, but there are certain limitations.  The program will not allow you to enter a bad value.          Next, enter the starting cylinder number.  The default is 0.  Large values are allowed in order to access large drives.          Next, enter the number of cylinders for this logical drive.  The default is 153. Values greater than 203 must be even.          Next, enter the step rate.  The default is 0 , but 6 and 3 are other common values.          Finally, answer Y if this is a removable disk. The default is N.               If everything went well, a message will now be printed and the operation is done!