X10024.cmd and X42100.cmd are assembly language programs written for the

model 4 (TRSDOS 6.2) that facilitate the movement of ascii files from one

computer to the other.  It is no longer necessary to SETCOM or SET *CL from

the dos prompt, as the two programs take care of all those boring

details.  The transfer is at 4800 baud, and file opening and closing is

automatic for both computers.

.

Both programs initialize the RS-232 line for proper baud rate, parity,

etc.  The programs will also install COM/DVR if it is not already installed,

assuming you have COM/DVR available on a disk in your system.  The programs

will abort if they are unable to locate *CL and cannot install COM/DVR.

.

Both programs utilize X-on/X-off protocall to insure that neither computer

gets ahead of the other.

.

Instructions can be displayed by typing the program name without a filename

(X10024<enter>).

To transfer a file from the model 100 to the model 4, you type 'X10024

filename' at the dos prompt.  On the model 100 you can upload from either TEXT

(F3 - save to "com:77n1e") or BASIC (save "com:77n1e").  Basic automatically

saves to COM: in ascii format.  X10024 then copies the incoming RS-232 line

into the file specified on the command line.  It strips out any unnecessary

line feeds during the transfer.  If the buffer fills on the model 4 (a most

un-likely event, considering the memory size of the model 100), a control-S is

sent to the Model 100 to stop transmission, and the buffer is written to disk.

X10024 then issues a control-Q to the M-100 and the transfer continues. The

program terminates upon receipt of a ^Z (which is the last character in every

Model 100 ascii file) or when the Model 4 break key is pressed.  X10024 then

closes the file, resets the wakeup vector in *CL and returns to dos.

.

X42100 just goes the other way.  The file is sent from the model 4 by typing

'X42100 filename', and is loaded into the model 100 through TEXT (F2 - load

from "com:77n1e") or basic (load "com:77n1e").  If the model 100 gets over-run

it will send a control-S to the model 4, which will halt transmission of the

text until receipt of a control-Q or 3 seconds has passed, whichever comes

first.

.

Current versions of both programs are 1.00.01. (Aug. 31, 1986)

The hardware necessary is simply an RS-232 cable/null modem.  Parts are

available from Radio Shack.  Or you can make your own: Purchase 2- DB-25 male

connectors (R/S# 276-1547) with hoods and a piece of telephone or microphone

(2 conductor shielded) cable.  Wire Pin 2 to 3, 3 to 2, and 7 to 7 (with the

shield if using mic cable).  That's a null modem!

.

Source code and notification of updates are available to registered

users.  Contact Paul Jaeger, 61 Jane St., New York, NY 10014, 212-691-3039,

CIS: 76010,2324, MCImail: 237-3942