YAPPXFER.DOCYAPP(tm) protocol for Packet Radio binary file transfer(c) 1986, Jeff Jacobsen - WA7MBLRevision 1.1   06/23/86----------------------------------------------------------YAPP PACKET TYPESSize of packet data ranges from 2-258 bytes.len = length of following bytes (not including len)RR Rcv_Rdy      ACK  01RF Rcv_File     ACK  02AF Ack_EOF      ACK  03   [1.1]AT Ack_EOT      ACK  04   [1.1]SI Send_Init    ENQ  01HD Send_Hdr     SOH  len  (Filename)  NUL  (File Size in ASCII)  NUL  (Opt)DT Send_Data    STX  len  (Data)    {if len=0 then data length = 256}EF Send_EOF     ETX  01ET Send_EOT     EOT  01NR Not_Rdy      NAK  len  (Optional Reason in ASCII)CN Cancel       CAN  len  (Optional Reason in ASCII)CA Can_Ack      ACK  05{Following are for use in Server mode}RI Rcv_Init     ENQ   02   len   (Filespec requested - wildcard allowed)TX Text         DLE   len  (ASCII text for display)  {to send text from server}{Unimplement - Reserved for Server Commands}UU Comands      ENQ   03                ENQ   04                ENQ   05                ...                     ENQ   FF     STATE TABLES for SENDState         Send Pkt        Upon                  Next StateS (Send)      SI              RR                    SH                              RF                    SD                              NR                    Start/Done                              First few timeouts    S                              RI                    S                              Other/Timeout         AbortSH (Hdr)      HD              RF                    SD                              NR                    Start/Done                              Other/Timeout         AbortSD (Data)     DT              (Not EOF)             SD                              (EOF)                 SESE (EOF)      EF              AF (More files)       SH                              AF (No More Files)    ST                              Other/Timeout         AbortST (EOT)      ET              AT                    Start/Done                              Other/Timeout         Start/DoneSTATE TABLES for RECEIVEState      On    If           Do          Send Pkt       Next State  R (Rec)    SI (ready)                       RR              RH           SI (opt)*       open file        RF              RD           SI (not ready)                   NR              Start/Done           NR                                               Start/Done           Other/Timeout                                    Abort(opt)*  If header info not desired it is ok to        open file and indiate ready for fileRH (Hdr)   HD              open file        RF              RD           HD (no room)                     NR              Start/Done           SI                                               RH           ET                               AT              Start/Done           Other/Timeout                                    AbortRD (Data)  DT              store data                       RD           EF              close file       AF              RH           Other/Timeout                                    AbortOTHER STATEAny State          On CN         Send CA                   Start/Doneexcept CWCW (Can_Wait)      On CA                                   Start/Done                   On CN         Send CA                   CW                   Timeout       Display Err Msg           Start/DoneAbort                            Send CN                   CW--------Starting StatesFor 2 users not using Remote Server    Sender goes to State S, Receiver to State R after manually    exchanging file names.  Since File Name is known, Recevier    may optionally send RF from state R and bypass file header.    Sender should give Receiver ample time to get into Receive    State.For Remote Server Mode  USER (Start)                                       Next State  User wishing to Send                               S  User wishing to Receive          Send RI           R  Other Server Request             Send COMMAND      Start  SERVER (Start)  Gets SI                                            R  Gets RI             (file fnd)                     S                      (no file)    Send NR           Start  Gets COMMAND                    (Do Command)       Start---------------------------------------------------------All states except Send Data should timeout if nothing is heardfor a period of time Tc (Crash timer).  Send Data isn't expectingany input and if it should receive some it must be a Cancel, ora TX (text) packet, otherwise Abort.--------Revision 1.1 - Added AF & AT to acknowledge End of File andEnd of Transfer.  This allows an automated sender to know thata file was received successfully.  It also helps get past aproblem in the TNC-2 code by requiring the sender to wait for theAck-EOT before switching from transparent mode back to conversemode......................................                                 ..    Downloaded in 1990 from:     ..                                 ..      ********************       ..      *     8/N/1 #1     *       ..      *   904/377-1200   *       ..      * Gainesville, FL  *       ..      * Guy Omer - SYSOP *       ..      ********************       ..                                 .. Software support for the TRS-80 ....................................