Notes on installing and using ZCLOCK63.ZCLOCK63 is designed to be used with TRSDOS 6.x, and the Kenmore TechnologiesZTIME1 real time clock board.  ZCLOCK63 will obtain the correct date and timefrom your ZTIME1 and plug that data into TRSDOS.  You no longer need to foolaround with the TIME and DATE prompts on bootup  Another utility, ZTIME, isavailable for setting the clock and performing other functions on it.  All ofthese utilities are designed for use with TRSDOS 6.x.  All utilities whichcome with the board are only for CP/M usage.To install ZCLOCK63, you must first copy ZCLOCK63/CMD to your system disk.Then, you must disable the DATE and TIME system prompts with the following:SYSTEM (DATE = NO)SYSTEM (TIME = NO)Now, you must SYSGEN the system so that these prompts will not appear the nexttime you boot.  This is done with the command:SYSGENThe last thing to do is create an AUTO file that the system will execute onbootup.  You do this by the command:AUTO ZCLOCK63Now reboot your system and you will have the correct DATE and TIME automa-tically posted for you by ZTIME.....every time you bootup.You must have SYS8/SYS on your system disk in order to use this utility!!!!That library contains the DOS TIME and DATE commands which are used by thisutility.If you set your ZTIME1 to some port address other than 020H, which is thecorrect one for TRS80 machines, you must adjust ZCLOCK63 accordingly.  Youmust change two bytes in order for the utility to run properly. This isaccomplished as follows:PATCH ZCLOCK63/CMD USING ZCLK/FIX. ZCLK/FIX. For changing the CBASE address in ZCLOCK63/CMD. Replace XX below with your CBASE PORT + 9H. Replace YY below with your CBASE PORT + 7HD01,7F=XX:F01,7E=0E 29 EDD01,84=YY:F01,83=0E 27 ED.EOPNOTE:  The ZTIME1 will NOT work on Non-Gate array Model 4/4P/4D machines!  ItWILL work with the XL8er board and/or the Hi Res boards installed.