(c) CompuServe Information Service                VIDTEX Executive (VIDTEX/CMD)                          INTRODUCTION The CompuService Information Service Executive  is  a  machinelanguage  program  which can be run on any 16K TRS-80 Level IIcomputer which has communications capabilities. It is a  smartterminal  program  emulator which allows your computer to com-municate with the CompuServe Information Service and any othercomputer system which has  telephone  answering  capabilities.VIDTEX  is  a  smart  terminal emulator in that it responds tocertain control codes to perform  special  funcctions.  VIDTEXdiffers  from  CompuServe's MicroNET Executive in that it doesnot include error free file  transfer  capabilities.  However,this will be included in a future version.                       HARDWARE REQUIRED In order to run this version of VIDTEX you must have a  TRS-80buss decoding version of the MICROCONNECTION by The MicroPeri-pheral Corporation.                          USING VIDTEX VIDTEX resides in memory between locations  7000H  and  775AH.The  entry point is also 7000H. It is supplied as a /CMD file.To run the program, type VIDTEX while in DOS. VIDTEX is designed to run on all configurations of the  TRS-80Level  II  computer. In order to do this, it must first deter-mine the configuration of the machine on which it  is  runningand then make modifications to itself which allow the featuresavailable to be used. It first determines if a  lower  case  modification  has  beeninstalled.  If one has, the video driver is modified to permitlower case output.  If  one  has  not,  the  video  driver  ismodified to convert all lower case output to upper case. It then checks to see if a printer is connected to the  expan-sion  interface.  If  one is, be sure that it is turned on andready before VIDTEX is run.                         REDEFINED KEYS VIDTEX has its own keyboard decoder in it. This permits it  toredefine  the keys and allows it to do its own debouncing. Thekeys which have been redefined are described below. The down arrow key functions as a control key. Thus, to type acontrol-C, you would press the down arrow and C  keys  at  thesame  time.  The up arrow is an ASCII up arrow, the left arrowis a backspace and the right arrow is a  horizontal  tab.  TheBREAK key is a control-C and the CLEAR key is an ESCAPE key. Some characters are simulated on the keyboard by  combinationsof  other  keys.  A  left  square  bracket  is produced with acontrol 8, a right square bracket by a control  9,  an  under-score  by  a  control  dash,  a vertical bar by a control 1, a      VIDTEX FOR THE MICROCONNECTION-  page 2 tilde by a control 5, a back quote by  a  control  7,  a  leftbrace by a control <, a right brace by a control > and a back-slash  by  a control slash. Note that some of these characterscannot be displayed on the TRS-80. Some keys perform functions locally to the TRS-80 and  do  notsend  any character. A control R will start copying all outputto the printer provided one is attached and ready. A control Twill stop the copying to the printer. A shift  0  (zero)  willtoggle  the  shift  lock  function of the keyboard. When shiftlock is in effect, you can only type in upper case. When shiftlock is not in effect, you can type in upper  or  lower  case.Note that you do not need the lower case modification in orderto  send  lower  case characters. Any lower case letters beingdisplayed will still be translated to upper case. A control  Xwill exit VIDTEX and reboot your TRS-80 to DOS. The following summarized the keys which have been redefined.           Labeled Key        Hex      Function          ===========        ===      ========           Down arrow                 Control key          Up arrow           5E      ASCII up arrow          Left arrow         08      Backspace          Shift lft. arrow   15      Control U          Right arrow        09      Horizontal tab          BREAK              03      Control C          Shift BREAK                Break Sequence          CLEAR              1B      Escape          Shift CLEAR        7F      Delete          Control 1          7C      Vertical bar          Control 5          7E      Tilde          Control 7          60      Back apostrophe          Control 8          5B      Left sq. bracket          Control 9          5D      Right sq. bracket          Control <          7B      Left brace          Control dash       5F      Underscore          Control >          7D      Right brace          Control /          5C      Backslash          Control X                  Exit and reboot          Control R                  Turn on printer          Control T                  Turn off printer          Shift Zero                 Toggle shift lock Any key held down for 2 seconds or more will repeat.                         VIDEO SUPPORT VIDTEX also has its own video driver which responds to most ofthe ASCII control characters. The control characters it recog-nizes and how it reponds to them is  as  follows.  A  carriagereturn  will  return  the  cursor to the beginning of the sameline the cursor is currently on. A line  feed  will  move  thecursor  down one line without changing its relative horizontalposition. A horizontal tab will move the cursor  to  the  nexttab position. A form feed will clear the screen. These charac-ters  must  be  sent  to the TRS-80 over the line and will notfunction locally.     VIDTEX FOR THE MICROCONNECTION-  page 3                     TECHNICAL INFORMATION VIDTEX also responds to certain ESCAPE  sequences  for  cursorcontrol.  The  sequences must be generated by programs runningon the computer with which the TRS-80  is  communicating.  Thefollowing  is a list of the cursor control codes and the func-tions they perform. Note  the  difference  between  lower  andupper case. This is very IMPORTANT!!           <ESC> m                   Enter wide character mode          <ESC> l                   Enter normal mode          <ESC> A                   Move up one line          <ESC> B or LF             Move down one line          <ESC> C or Space          Move right one space          <ESC> D or Bk Space       Move left one space          <ESC> H                   Home cursor          <ESC> K                   Clear to end of line          <ESC> J                   Clear to end of page          <ESC> j or FF             Clear page          <ESC> Y line column       Absolute positioning The cursor can be moved directly to any position on the screenby absolute positioning. The sequence to do this is...                       <ESC> Y lin colWhere "lin" is the ASCII character whose  value  is  the  linenumber +31 and "col" is the ASCII character whose value is thecolumn  number  +31.  If  the line specified is too large, thecursor will be positioned at the bottom  line.  Similarly,  ifthe column specified is too large, the cursor wil be position-ed  at  the  last  column. For example, the following sequencewould position the  cursor  at  the  4th  line  and  the  10thcolumn......                         <ESC> Y # )Note that the character '#' has the ASCII value of 35 which is31+4 and the character ')' has the ASCII value of 41 which  is31+10.