L1000/CTL & L1000P/CTL                        SuperScripsit & SuperScripsit PRO                                 Printer Drivers                                       for                        Epson L-1000 24 pin Action Printer                                                                         Copyright (C) 1989                                        by                              Kachina Software Group                                                                                                                     INTRODUCTION                                   ============          The SuperScripsit and SuperScripsit  PRO versions of L1000/CTL  are  the     copyrighted property of the  Kachina Software  Group. You  are granted a     limited license to use L1000/CTL and to copy and further distribute  it,     provided the following conditions are met:          1) No more than $5.00 may be charged for such copying and distribution.          2) Distribution  of  L1000/CTL  may only occur  if the  two versions are     distributed in their  original  archive and  only  in  their un-modified     state.          This software is being  offered as "shareware". Shareware relies on  the     inherent  honesty of  the users of a  product to voluntarily  contribute     money to offset the cost of producing the software if,  and only if, the     user finds the product useful.          This  user  support concept allows  the end-user to  receive  a  quality     product at very low cost after being able to  put it to practical use in     the end-user's own computer environment. The  author  benefits  by being     able  to enter the  commercial  software market  without  needing  large     amounts of venture capital.          Shareware - it only works with your support!          Any voluntary contributions for the use of these printer drivers will be     appreciated and should be sent to:                                   Kachina Software Group                                9112 Dempsey Dr NE                              Albuquerque, NM 87109          The author request  a $10.00 registration fee for the  use  of either of     the printer drivers supplied.          The L1000  printer drivers support Proportional Printing and spacing, as     well as the following additional pitches:                              4 Characters per inch (CPI)                         6 Characters per inch                         8 Characters per inch                         10 Characters per inch                         12 Characters per inch                         15 Characters per inch                         17 Characters per inch                         20 Characters per inch                Dcoumentation               L1000 & L1000P              July 1989                                      Page 2          In addition, the System Print Codes defined in your SuperScripsit manual     will now perform the following features:                          CLEAR and -     Toggles underlining on/off                     CLEAR and =     Toggles Draft vs LQ mode                     CLEAR and +     Toggles Bold (doublestrike)                     CLEAR and /     Toggles Roman vs Sans Serif                     CLEAR and .     Sets subscripting mode                     CLEAR and *     Sets superscripting mode                     CLEAR and >     Ignored                     CLEAR and ?     Pauses Printout                                             INSTALLATION                                   ============          After extracting the appropriate driver from  the archive (L1000/CTL for     Model  4 SuperScripsit Version 01.01.xx or  L1000P/CTL for SuperScripsit     PRO), it should be copied to your SuperScripsit disk.          It  is   suggested  that,   at  this  point,  the   default  options  in     SuperScripsit be adjusted to reflect the use of this new printer driver.     At the main menu of SuperScripsit, select "S" for System set-up options.     At  the next displayed  menu,  select  "O"  for Open  Document  options.     Position cursor  to the fifth line using the arrow  keys  and then press     SHIFT-CLEAR on the Printer Type  line. This will clear  out any previous     entry. Now type L1000 (do not type  in the /CTL extension). If you wish,     you may make any other default changes here like your pitch  setting. It     is  suggested that you use the P default  Proportional pitch  because of     its very pleasing aspect on this printer but  any of the available pitch     selections  may  be used as the  default. Press ENTER  to return to  the     Set-up  menu. Now,  select "P"  for Printer  options  and  determine the     method  of  justification  you wish to  use.  If  P is used as the pitch     setting,  P used  for justification would also  be appropriate.  If P is     used for justification for any other pitch, it will be equivalent to the     M -  Monospacing selection. Now,  press  ENTER  to return to the  Set-up     menu, and BREAK to return to the main menu.                                               DEFAULTS                                     ========          When the printer driver is  enabled to print  your document (Control P),     certain default actions are  taken with  the printer. First, the printer     is  initialized to its  default DIP switch settings. Second, the printer     is  placed in the LQ (Letter  Quality) mode, and finally, the  character     type style selected is Roman. This should be apparent by the left orange     light on the SelecType panel becoming lit.                                     TOGGLE PRINT CODES                                ====== ===== =====          The codes described as toggles in the introduction (CLEAR plus '-', '=',     '+', and '/' turn on the feature the first time they are encountered and     turn off the feature the next time the printer driver sees the code.                 Dcoumentation               L1000 & L1000P              July 1989                                      Page 3          Underlining (CLEAR and -)     -----------          Even  though  the  Epson  L1000  documentation indicates  that  enabling     underlining will  underline  all  characters except horizontal tabs, the     L1000/CTL  printer drivers will not allow  underlining  of  spaces. This     gives a much more pleasing aspect to underlining in a document.               Roman vs Sans Serif (CLEAR and =)     -------------------          The default condition of the  printer is Roman  character  typestyle. If     you  wanted your  entire  document  in  Sans Serif type style, you could     start the document  with  the CLEAR =  code. In other circumstances, you     can switch back and forth between the two type styles.               Bold (CLEAR and +)     ----          Using  the Bold toggle to highlight certain words is common  practice in     word  processing.  In this case, the  code  for doublestrike is actually     sent to the  printer. Emphasized printing  may be set-up in a user print     code (discussed later in this document).               LQ vs Draft (CLEAR and /)     -----------          Preceeding a document with  the CLEAR / will  force the printer into the     draft mode. Besides the faster printing that will ensue, there will also     be less wear and tear on the ribbons and  print head while your document     is still in its rough stages. This command has no effect in Proportional     Print mode, however.                                        NON-TOGGLE PRINT CODES                              ========== ===== =====          Subscripting (CLEAR and .)     ------------          For both subscripting and  superscripting,  when the codes are utilized,     they must be followed by the opposite code to return to normal print. In     other  words,  when subscripting a word or character, it is preceeded by     CLEAR. and followed by CLEAR*.               Superscripting (CLEAR and *)     --------------          Just the opposite  of what  was described  in  the  above  paragraph  is     necessary  for superscripting. When superscripting  a word or character,     it is preceeded by CLEAR* and followed by CLEAR.                     Dcoumentation               L1000 & L1000P              July 1989                                      Page 4          Top the form (CLEAR and >)     ------------          Ignored by the printer driver.               Pause Printout (CLEAR and ?)     --------------          Most useful when dealing with Daisy wheel printers and needing to change     the printwheel, nonetheless, this code is enabled to pause the printout.                                       USER DEFINED PRINT CODES                             ==== ======= ===== =====          The hard coded System Print Codes are limited to the 8  described  above     and two  of those are unavailable to the print driver and thus cannot be     modified. Fortunately, the author of SuperScripsit provided for  20 user     defined print codes. Entries have been devised for all 20 positions that     may  be useful  for the user  of an L-1000  Epson printer. The codes  to     place  within the System Set-up menu  under Printer  Codes  are provided     below along with a description of what they will provide:          Code    Units   Sequence                        Comments     0       0       27 113 0                        Norm Style     1       0       27 113 1                        Outline     2       0       27 113 2                        Shadow     3       0       27 113 3                        Out+Shadow     4       0       27 52                           Italics

On     5       0       27 53                           Cxl Italic     6       0       27 80                           10 Pitch     7       0       27 77                           12 Pitch     8       0       27 103                          15 Pitch     9       0       27 15                           Condensed     !       0       18                              Cxl Cndsd     "       0       27 119 1                        Dbl High     #       0       27 119 0                        Cxl Dbl Hi     $       0       27 116 1 171 27 116 0           1/2 Symbol     %       0       27 116 1 172 27 116 0           1/4 Symbol     &       0       27 116 1 155 27 116 0           Cents Symb     '       0       27 116 1 248 27 116 0           Degree Sym     (       0       27 116 1 249 27 116 0           Dot Symbol     )       0       27 116 1 241 27 116 0           Plus Minus     @       0       27 116 1 246 27 116 0           Divide Sym          Of course, you can define your  own symbols - perhaps ones that you find     that you  utilize more  often.  Some additional  comments about the ones     displayed above:          CLEAR0 is used to return to normal printing after using CLEAR1 or CLEAR2     or CLEAR3.          The   pitch  control   codes  provided   may   cause   problems  because     SuperScripsit  does  not know  that the pitch has  been changed (margins     would be different, etc.). They are otherwise perfectly usable.                Dcoumentation               L1000 & L1000P              July 1989                                      Page 5          CLEAR" and CLEAR# should  not  be  used  when the tractor is pulling the     paper according to the printer manual. (For single sheets only).          While a  Units entry may be made for the  symbols  described above (they     are all 15 units wide and would be entered as  015) they  will only work     correctly when in Proportional Print. In any other pitch,  SuperScripsit     would  assume that each symbol was  the width of 15 characters and would     thereby  cause both display and printing  problems. The effect of having     each character listed with 0 units is negligible in Proportional Print.               Note  that  the   L1000P  driver  has  not  been  actually  tested  with     SuperScripsit PRO. However, the only differences are the ORG and certain     SuperScripsit  entry points. The driver should  work just as well as the     regular SuperScripsit driver.          Note  that  the  L-1000 has been  described as an upgraded LQ-500. Since     EPSON recommends the use of an LQ-500 printer driver if an L-1000 driver     is not defined,  it  is entirely  possible that this driver  would  work     effectively for an LQ-500 printer.          SuperScripsit is a registered trademark of Tandy Radio Shack.     Action Printer is a registered trademark of EPSON America, Inc.