Dallas Semiconducter SmartWatch DS121ESourcing in UK:Joseph Electronics, 2 The Square,Broad Street,Birmingham B15 1APTel 021-643-2011Information obtained from 1987-1988 Product Data Book  FEATURES *SmartWatch keeps track of hundredths of seconds, seconds, minutes, hours, days, date of month, months and years *Adds timekeeping to any 28-pin JEDEC Byte Wide memory location *Embedded lithium energy cell maintains calendar time for more than 10 years in the absence of power *Timekeeping function is transparent to memory operation *Month and year determine the number of days in each month *Proven gas-tight socket contacts *Full 10% Vcc operating range *Operating temperature range 0 deg C to 70 deg C *Accurate to within 1 min./month @ 25 deg C PIN NAMES PIN 1 - A14  - Address Bit 14 (*RESET)PIN 8 - A2   - Address Bit 2  (READ/*WRITE)PIN 10  A0   - Address Bit 0  (Data Input)PIN 11  DQ0  - I/O0 (Data Output)PIN 14  GND  - GroundPIN 20  *CE  - Conditioned Chip EnablePIN 22  *OE  - Output EnablePin 28  Vcc  - + 5 VDC to the Socket All pins pass through to the Socket except 20.  DESCRIPTIONThe DS1216E  is  a  28-pin,  600-mil-wide  DIP  socket  with  a  built-in  CMOStimekeeper function and an embedded lithium  energy source to maintain time anddate.   It accepts any 28-pin bytewide ROM  or volatile RAM.   A key feature ofthe SmartWatch is  that  the  timekeeper  function  remains  transparent to thememory device  placed  above.   The  SmartWatch  monitors  Vcc  for  an out-of-tolerance condition.   When such a condition occurs, an internal lithium energysource is automatically switched on to prevent loss of watch data. Using the SmartWatch saves  PC board space since  the combination of SmartWatchand the mated memory device  take up no more  area than the memory alone.   TheSmartWatch uses pins 1,8,10,11,20 and 22 for timekeeper control.  All pins passthrough to the socket  receptacle except  for pin  20 (*CE)  which is inhibitedduring the transfer of time information. The  SmartWatch  provides  timekeeping   information  including  hundredths  ofseconds, seconds, minutes, hours, days, date, month, and year information.  Thedate at the end  of the  month is  automatically adjusted  for months with lessthan 31 days,  including correction for leap years.  The SmartWatch operates ineither 24-hour or 12-hour format with an AM/PM indicator. OPERATION A highly structured sequence  of  64  cycles  is  used  to  gain access to timeinformation and temporarily disconnect  the mated  memory from  the system bus.Information transfer into  and  out  of  the  SmartWatch  is  achieved by usingaddress bits A0 and  A2,  control signals *OE and  *CE,  and Data I/O line DQ0.All SmartWatch data transfers are accomplished  by executing read cycles to themated memory address space.   Write  and read  functions are  determined by thelevel of address bit A2.   When address bit A2 is low, a write cycle is enabledand data must be input on address bit A0.  When address bit A2 is high,  a readcycle is  enabled and  data is  output on  data I/O  line DQ0.   Either controlsignal (*OE or *CE) must transition low to  begin and high to end memory cycleswhich are directed to the SmartWatch.  However, both control signals must be inan active state during  a memory cycle.   Communication  with the SmartWatch isestablished by pattern recognition of a serial bit stream of 64 bits which mustbe matched by executing 64 consecutive write cycles, placing address bit A2 lowwith the proper data on address bit  A0.   The 64 write cycles are used only togain access to the  SmartWatch.   Prior  to  executing  the  first  of 64 writecycles, a read cycle would be executed by holding A2 high.  The read cycle willreset the  comparison  register  pointer  within  the  SmartWatch  insuring thepattern recognition starts with the first bit of the sequence.   When the firstwrite cycle  is executed,  it  is compared  to bit  1 of  the 64-bit comparisonregister.  If a match is found,  the pointer increments to the next location ofthe comparison register and awaits the next write cycle.   If a match is found,the pointer does not advance and all subsequent write cycles are ignored.  If aread cycle occurs at any time during pattern recognition,  the present sequenceis aborted and the comparison  register pointer is reset.   Pattern recognitioncontinues for a total of 64 write cycles as described above,  until all bits inthe comparison register are matched  (C5 3A A3 5C  C5 3A A3 5C).   With correctmatch of 64 bits,  the SmartWatch  is enabled and data  transfer to or from thetimekeeping  registers  may  proceed.   The  next  64  cycles  will  cause  theSmartWatch to either receive data on Data  In (A0) or transmit data on Data Out(DQ0),  depending on the level  of A2.   Cycles to  other locations outside thememory  block  can  be   interleaved  with  (*CE)   and  (*OE)  cycles  withoutinterrupting the pattern recognition sequence or  data transfer sequence to theSmartWatch. And unconditional reset to the SmartWatch occurs by either bringing A14 low, ifenabled,  or on power up.   This RESET  can occur during pattern recognition orwhile accessing  the SmartWatch  register.   RESET causes  access to  abort andforces  the  comparison  register  pointer  back  to  Bit  0  without  changingregisters. NONVOLATILE CONTROLLER OPERATION The  DS1216E  SmartWatch  performs  circuit  functions  required  to  make  thetimekeeping function nonvolatile.  First,  a switch is provided to direct powerfrom the batter  or Vcc  supply,  depending on  which voltage is greater.   Thesecond function provides power fail detection.   Power fail detection occurs attypically  4.25  volts.    Finally  the  nonvolatile  controller  protects  theSmartWatch register contents by ignoring any  inputs after power fail detectionhas occurred.   Power fail detection also has  the same effect on data transferas the *RESET input. SMARTWATCH REGISTER INFORMATION The SmartWatch information is contained in 8 registers of 8 bits each which aresequentially accessed one  bit at a  time after the  64-bit pattern recognitionsequence has been completed.  When updating the SmartWatch registers, each mustbe handled in groups  of 8 bits.  Writing and  reading individual bits within aregister could produce  erroneous results.   Data  contained in  the SmartWatchregisters are in binary  coded decimal format  (BCD).   Reading and writing theregisters is always accomplished by stepping through all 8 registers,  startingwith bit 0 of register 0 and ending with bit 7 of register 7. The registers aredefined as follows: Register    7     6     5     4      3     2     1     0       Range    0    |      0.1 sec           |       0.01 sec       |    00-99    1    |  0  |     10 sec       |       seconds        |    00-59    2    |  0  |     10 min       |       minutes        |    00-59    3    |12/24|  0  |10/ap|  hr  |       hour           |    00-12 or 00-24    4    |  0  |  0  |*osc |*rst  |  0  |     day        |    01-07    5    |  0  |  0  |  10 date   |       date           |    01-31    6    |  0  |  0  |  0  |10 mon|       month          |    01-12    7    |     10 year            |       year           |    00-99 AM-PM/12-24 MODE Bit 7 of the hours register is defined as the 12-24 hour mode select bit.  Whenhigh,  the 12-hour mode is selected.   In 12-hour mode,  bit 5 is the AM/PM bitwith logic high being PM.  In the 24 hour mode, bit 5 is the second 10-hour bit(20-23 hours). OSCILLATOR AND RESET BITS Bits 4 and 5 of the day register  are used to control the *RESET and oscillatorfunctions.   Bit 4 controls the  *RESET (pin1).  When the *RESET  bit is set tological 1,  the *RESET  input pin is$ ignored.   When the *RESET  bit is set tological 0,  a low input  on the *RESET  pin will cause  the SmartWatch to abortdata transfer without changing data in the watch registers.  Bit 5 controls theoscillator.   This bit is shipped  from Dallas Semiconductor  set to logical 1,which turns the oscillator off. When set to logical 0,  the oscillator turns onand the watch becomes operational. ZERO BITS Registers 1, 2, 3,  4, 5, and 6 contain one or more bits which will always readlogical  0.   When  writing  these  locations,  either  a  logical  1  or  0 isacceptable. ############################################################################################################