/*  * LU4/CMD - LU (Library Utility) format library file maintainer  *      by David A. Huelsmann  *      Version 1.01 12/04/85 *      For R/S Model IV/IVP/IVD under TRSDOS 6.x *  * DESCRIPTION  *    LU was a program to manipulate CP/M LU format libraries.  *    The original CP/M library program LU is the product  *    of Gary P. Novosielski. The original LU program was set up *    by Stephan C. Hemminger on Version 7 UNIX. * *    The primary uses of LU4 is to combine several files together for *    upload/download to a personal computer and to archive files (preferably *    after sQueezing).  * * Usage: lu4 cmd library [files] ...  *  * Cmd functions are:  * u - Update, add files to library     (also creates new libraries)  * t - Table of contents  * e - Extract files from library  * p - Print files in library to CRT * d - Delete files in library  * r - Reorganize library  *  * EXAMPLES:  * lu4 t foo/lbr:1              list all files in FOO/LBR found on Drive 1 * * lu4 e foo/lbr 1/c 2/c:1      extract files 1/c, 2/c from FOO/LBR  *                              write 1/c to default drive, 2/c to drive 1 * * lu4 p foo/lbr 1/c            display 1/c from FOO/LBR to CRT  *                              (Not recommended for binary files) * * lu4 u foo/lbr 1/c:0 2/c 3/c:1 *                              add or replace files in FOO/LBR  *                              (Note that you can specify where the files *                              can be found on your drives.) *  * lu4 p foo/lbr 1/c >*pr       redirect output of listing to attached printer * * lu4 t foo/lbr >*pr           redirect table of contents to printer * * When creating a new library, you will be prompted for the maximum  * number of entries it can contain. Assuming NEW/LBR doen't exist ...  * lu4 u new/lbr                create an empty library  * lu4 u new/lbr a/c b/c d/c:1  create NEW/LBR, add files.  *  * The Reorganize option causes <lbrfile>/tmp to be created, and  * the contents of the old library to be copied into it. * The primary purpose of the Reorganize function is to recompact the * library file after files have been deleted. The LU4/CMD program always * adds new files to the end of the library and does not attempt to reclaim * space. The old library will be removed and the new library will be * renamed to the original library name. *  * This program is public domain software, no warranty intended or  * implied.  *  *  * PORTABILITY  *      The original version by Stephan C. Hemminger was set up for   *      Version 7 UNIX, and was not useable as is. It was hacked  *      to fit the Lattice C compiler for MSDOS and CP/M-86 by T. Jennings. * *      There were subtantial revisions made to the Jenning's version *      (to the point of rewriting major portions of the source *      code over several months) in order to get this program running *      under ALCOR C for the R/S Model 1 computer. Some of the major  *      limitations experienced were a lack of tell() or lseek()  functions, *      and as a result a necessity to use random file kludges to accomplish *      the same end result. One additional feature added was the creation date *      of the file within the library. The Model 4 version was adapted *      from the Model I version maintaining compatibility between the *      libraries i.e., each can read the library created by the other. *      Additional functions added in the Model 4 version were expansion *      of the displays to take advantage of the 80 column screen; a little *      utilization of the reverse video capabilities; a check to *      ensure that there is enough memory available from the heap to *      accomodate all functions; a listing of the size of each file *      in bytes as well as sectors; and a beep sound generation whenever *      an error is detected. *  *      Also, I have made random changes to the source merely to reflect  *      my programming taste; the original code was quite good. I have also  *      changed the wording of some errors, etc., more in line with the *      current flavor of messages in the micro environment.  *  *      There is no problem with text or binary files; they are treated *      identically. NULL or 0 characters are added to the  end of all files  *      to round it up to a multiple of 256 bytes.  *      This should not present a problem with any files, data or CMD. *  *      Note that all files are kept as multiples of 256 bytes, and this *      is not compatible with the CP/M utility LU. In addition, the internal *      directory structure is not the same as in the CP/M utility LU. Please *      do not use the R/S version to attempt to unlibrary CP/M libraries. *  * TECHNICAL NOTES ON VERSION 1.01 *      This version is essentially a beta test version. Please report all  *      "bugs" to the author on the TRS-80 conference area of the TBC BBS. * * Undocumented Features: * *      This version has a restore command. This command takes the form of: *      lu x foo.lbr 1/c *      where 1/c is a previously deleted *      file and x is the command to restore. This command should be carefully *      used because you may set up situations where two files reside in your *      library with the same name! This makes extraction difficult (to say the *      least). To resolve such a situation would require a ZAP-like program to *      rename one of the files directly in the library's directory. * *      To determine which version you are currently using, you may just type *      lu4 which will present the help screen as well as the version  *      or you may type lu4 v foo.lbr and the version will be displayed. * *  Directory Structure: * *      The libraries' internal directory structure is the same in all *      versions. It is from 1 to 7 sectors long (256 bytes/sector). *      Each directory entry is 32 bytes long (a maximum of 8 entries *      per sector is possible). Directory entry 0 contains the *      following information: Byte 0 is 0 (indicating ACTIVE entry, byte *      14-22 contains the creation date of the directory, byte 25 contains *      the number of sectors in the directory.                                  * *      Each file entry is similar to the above as follows: Byte 0 is 0 *      to indicate ACTIVE file or FF to indicate unused directory slot, *      or FE to indicate a deleted directory slot. Bytes 2-9 is the file *      name. Bytes 10-13 is the extension. Bytes 14-21 is the date the file *      was added to the directory. Bytes 23-24 is the sector offset in which *      the file starts in the library and bytes 25-26 is the length of the *      file in sectors. * * Using Upper or Lower Case: * *      LU4/CMD is very understanding of case. Both upper and lower case *      are accepted. * * Naming conventions: * *      Libraries are normally provided with the extension LBR to make them *      recognizable to all users. * * Source Code: * *      The source code is not provided. * * *  UNIX is a trademark of AT&T. * ** CP/M is a trademark of Digital Research.  * *** ALCOR is a trademark of ALCOR Systems. * **** AZTEC is a trademark of Manx Software Systems, Inc. * *                        David Huelsmann *                        TRS-80 Conference *                        Chairman *                        TBC BBS (505) 821-7379 */###############################################################################################################################################################################################################