WINDOWS 3.1 DRIVERS FOR TANDY LASER PRINTERS

VERSION 01.00.03   03/15/93

PLEASE READ

-------------------------------------------------

By using the following Windows 3.1 Driver Library

(the "Software"), you are hereby agreeing to the

included license agreement.  Please review the

file LICENSE.TXT.

INSTALLING A PRINTER DRIVER

---------------------------------

1.  In the Main group, choose the Control Panel icon.

2.  In Control Panel, choose the Printers icon.

The Printers dialog box appears. If you have never

installed a printer, the dialog box expands to

display a list of printers available in the Windows

version 3.1 package, and the Add button is

unavailable. Go to step 4.

3.  Choose the Add button.

The dialog box expands to display the List Of

Printers.

4.  In the List Of Printers box, select Install

Unlisted or Updated Printer.

5.  Choose the Install button.

A dialog box appears instructing you to insert the

disk that has the printer-driver file on it.

6.  Insert the Tandy Printer Drivers for Windows

diskette, or type the full path name of the driver.

Then choose the OK button.

Another dialog box appears, listing several

printers.

7.  In the List Of Printers box, select the printer you

want.

8.  Choose the OK button.

The Printers dialog box appears. Your printer is

now listed in the Installed Printers list.

9.  Choose the Close button.

10. For instructions on how to configure your

printer, see Chapter 5, "Control Panel," in the

"Microsoft Windows User's Guide."

USING A PRINTER DRIVER

-----------------------------

The following printers are supported:

Catalog

Number    Printer                    Notes

--------------------------------------------------

26-2866   Tandy DMP  135 (Epson)     1,3

26-2868   Tandy DMP  136 (Epson)     1,2,3

26-2887   Tandy DMP  137 (Epson)     1,2

26-2864   Tandy DMP  202 (IBM)       1

26-2880   Tandy DMP  203 (IBM)       1

26-2883   Tandy DMP  204 (Epson)     1,2

26-2904   Tandy DMP  205 (Epson)     1,2

26-2906   Tandy DMP  206 (Epson)     1

26-2839   Tandy DMP  240 (Epson)     1,2,3

26-2886   Tandy DMP  250 (Epson)     1,2

26-2818   Tandy DMP  300 (IBM)       1,4

26-2849   Tandy DMP  302 (Epson)     1,3,4

26-2876   Tandy DMP  310 (IBM)

26-2817   Tandy DMP 2102 (IBM)       1

26-2850   Tandy DMP 2103 (Epson)     1,3,4

26-2865   Tandy DMP 2104 (Epson)     1,3

26-2845   Tandy DMP 2130 (Epson)     1,3

26-2873   Tandy JP  250 (Epson)      3,4

26-2888   Tandy LP 400 (HP)          5

26-2889   Tandy LP 410 (HP)          5

26-2889   Tandy LP 410 (PostScript)  5,6,8

26-2890   Tandy LP 800 (HP)          5

26-2890   Tandy LP 800 (PostScript)  5,7,8

26-2838   Tandy LP 950 (HP)          5

26-2879   Tandy LP 990 (HP)          5

26-2804   Tandy LP-1000

Notes:

1.  At least three paper sources are provided: Tractor,

Manual, and Lower Tray.

Use Tractor with fanfold paper. The paper loading

feature of the printer creates a top margin.

Use Manual when you are using the single sheet

paper loading method. The paper loading feature of

the printer creates a top and a bottom margin.

Lower Tray does not allocate a top or bottom margin

and can be used in two ways:

1)when you are manually adjusting the top of form

so no top margin is needed,

2)when you are using the pull tractor method (not

available on all printers). See your printer's

user's guide for instructions on using the pull

tractor method.

2.  The printer supports color printing. If you are

using the standard black ribbon, run Windows

Control Panel Printer Setup Options to deselect the

color option.

3.  The printer driver supports a different emulation

than the factory default emulation. The emulation

you must use is shown in brackets next to the

printer's name. See your printer's user's guide for

instructions on changing the printer's emulation.

4.  The printer supports a cut sheet feeder to

automatically load single sheets of paper. See your

printer's user's guide and cut sheet feeder's

user's guide for installation instructions.

To enable the printer driver to use the sheet

feeder, run Windows Control Panel Printer Setup to

select the Auto Sheet Feeder paper source.

5.  Three paper sources are provided: Manual, Upper

Tray, and Lower Tray. The sheet feeder provided

with the printer can be used with the Manual or

Upper Tray paper sources. An optional second sheet

feeder is available  to use as the Lower Tray paper

source.

6.  The printer supports the PostScript command set

with the optional PDL Cartridge and 1 MB of

additional memory. See your printer's user's guide

for details.

7.  The printer supports the PostScript command set

with the optional PDL Cartridge and 2 MB of

additional memory. See your printer's user's guide

for details.

8.  The printer's front panel controls the selection of

the paper source and overrides the printer driver's

selection. The printer driver selection has no

effect.