HOW TO SET THE SWITCHES ON YOUR MEMORY BOARDS:  (VERY IMPORTANT)

There are only a limited number of possibile combinations, so rather than

go into the theory, I will simply enumerate each of them and tell you EXACTLY

how to set the switches.

Sometimes reading the switches is confusing, and occasionally a user has

set ALL the switches backwards.  This is guaranteed not to work.  Here's a

hint:  Your OLD board had switch 2 ONLY set on.  We normally ship new boards

set the same way.  If you read the switches as 2 off, all others on, simply

turn your eyes upside down and proceed.

One 2 meg board ONLY (not recommended - gives you only 1 meg swap)

Set the board for switch 2 only.

One 1 meg board, one 2 meg board:

Set the 2 meg board for switch 2 only.

Set the 1 meg board for switches 2 & 8.

Two 2 meg boards:

Set one of the boards 2 only.

Set the other board 2 & 8.

Two 2 meg boards plus one 1 meg board:

Set one of the 2 meg boards for 2 only.

Set the other 2 meg board for 2 & 8.

Set the 1 meg board for switches 2 & 6.

Three 2 meg boards:

Set one of the boards 2 only.

Set another board 2 & 8.

Set the third board 2 & 6.

Three 2 meg boards plus one 1 meg board:

Set one of the 2 meg boards 2 only.

Set another 2 meg board 2 & 8.

Set the third 2 meg board 2 & 6.

Set the 1 meg board 2, 6, & 8.