P C   F l t   P l a n   1.1

Computer Flight Planning for DOS

Copyright 1990-1992 Geir A. Granviken

D O C U M E N T A T I O N

------------------------------------------------------------------------

C O N T E N T S

Part I - General Information

1.   GENERAL

1.1  Disclaimer

1.2  Registration

1.3  Distribution

2.   INTRODUCTION

2.1  About the program

2.2  Documentation

2.3  Hardware requirements

Part II - Quick Start

3.   QUICK START GUIDE

3.1  Installing

3.2  Starting the program

3.3  Configuring and Preparing the program

3.4  Making a flight plan

3.5  Ending and Restarting PC Flt Plan

3.6  Nav Computer

Part III - Detailed Guide

4.   INSTALLING

5.   STARTING PC FLT PLAN

6.   PREPARING THE PROGRAM FOR USE

6.1  Configuring

6.2  Aircraft Data

6.3  Nav-Database

7.   HOW TO MAKE A FLIGHT PLAN

7.1  Data

7.2  Create/Edit a Route

8.   ROUTE EDITOR

8.1  Legs

8.2  To / From Fields

8.3  Automatic or Manual Calculation of

course/dist:

8.4  Flight Data

8.5  Map View

9.   PRINT/VIEW A FLIGHT PLAN

9.1  Flight Plan Data

9.2  Print or Show the flight plan

10.  ABOUT THE FLIGHT PLAN

10.1 Leg Times

10.2 Fuel

10.3 Corrections

10.4 How to use the ETO/RTO/ATO fields

11.  NAV COMPUTER

12.  TERMS & ABBREVIATIONS USED IN THE PROGRAM

------------------------------------------------------------------------

Part I - General Information

1.   GENERAL

1.1  Disclaimer

THIS PROGRAM MAY ONLY BE USED IF THE USER AGREES TO THE FOLLOWING

TERMS:

There is no warranty of any kind. In no event shall the author be

liable for any damages, including any lost profits or other

incidental or consequential damages arising out of the use of or

inability to use this program, even if the author has been advised

of the possibility of such damages. The author shall not be liable

for any possible wrong calculations made by the program.

1.2  Registration

IMPORTANT: The demo program package (NOT the registered disk

version) may freely be distributed in it's unmodified form and

tested, like shareware. BUT in it's unregistered state, saving a

route to disk is disabled.

If you find the program useful, it may be registered for USD 25,-.

You will receive a registration number as soon as possible to be

inserted under the configuration option. A valid reg. number will

enable saving routes to disk. Registration will also be valid for

the next version.

This program must NOT be used by COMMERCIAL-, MILITARY- or

GOVERNMENTAL operators without a special licence agreement with the

author.

Also for general use in a flying club or similar, a special agree-

ment is required.

For registration, comments or questions, contact:

-------------------------------------------------------------------

Geir A. Granviken                        Tel: (Intl+47) 3-285565

Godheimvn. 85                 After 3 Jun 93: (Intl+47) 31 28 55 65

N-3440 Roeyken

Norway

-------------------------------------------------------------------

1.3  Distribution

The demo program package (NOT the registered disk version) can be

freely distributed, but all files must be included and they must

not be altered in any way. The configuration file FPL.CFG (created

by the program) or the file FPL.REG (if you have that), shall NOT

be included. No fees must be charged, except for diskette costs.

The demo/sw package consists of the following files:

- README       - General info

- MAILER       - Registration form

- FPL11.DOC    - Documentation (This file)

- FPL.EXE      - Main program file

- FPLSETUP.EXE - Program file (normally called from main program)

- FPL.DTA      - Data file (will be made by program if not present)

- FPL2.DTA     - Data file ( "   "   "   "     "    "   "     "   )

- FPLFLY.DTA   - Data file, aircraft data

- FPLFAC.DTA   - Data file, nav database

- FPLFAC.IDX   - Data file, nav database

- FPLFAC.TXT   - Nav database in ASCII text format

- FPLFIL.DIR   - Route file remarks (used by the program)

- ********.RT* - Examples, saved routes.

2.   INTRODUCTION

2.1  About the program

The program is used to make operational flight plans for VFR or IFR

flights, - fixed wings or helicopters. (VFR = Visual Flight Rules,

IFR = Instrument Flight Rules.)

A flight can be defined to the destination and to one or two

alternates. In addition, a list of other diversion fields may be

created.

The program has a nav-database with lat/long coordinates, magnetic

variation and frequencies for airfields, navigational aids, inter-

sections or other frequently used waypoints. The facilities

included are examples (they are correct though), but the user can

enter his own waypoints into the database as he creates or edits a

route, and in that way build up a database.

The program also provides a small register where some key data for

the different aircraft used may be stored.

The routes created may be saved on disk.

There are 4 different flight plan printouts to be selected from.

One of these is a knee-board size flight plan.

2.2  Documentation

This file makes up the documentation. If you find that some topics

are covered to lightly (or it's just pure lousy all together...),

the program should hopefully be fairly easy to use, and if you take

a close look at the sample routes, it should be possible to make

some sense of it. And there are no hidden features not shown in the

menus or input options.

Chapter 3 gives a quick introduction to get started, while the

following chapters goes more in detail.

2.3  Hardware requirements

- IBM PC/XT/AT/PS2 or compatible.

- Minimum 512 KB RAM.

- Any display type, but to use the map view function, a VGA display

is required.

- Hard-disk not required, but the program will work much faster. On

a floppy disk system, the drive should be at least 720 KB.

- A disk cache program is recommended but not required.

SMARTDRV.SYS (or .EXE) which is included with DOS 5.0 and Windows

is such a cache program, and works very well.

- To be able to print out all four flight plan variants, the

printer should be able to print IBM's graphic characters (line

drawing characters).

Part II - Quick Start

3.   QUICK START GUIDE

This part is for those who want to get started right away, and

prefer to just go ahead and try a program instead of reading a pile

of documentation. If you want details on a topic, it may be looked

up in part III.

3.1  Installing

Note: The disk version is supplied with an install program. If this

is used, the instructions below may be disregarded.

Make a directory on your hard-disk, name it what you like (sugges-

tion: FPL), and put all necessary files in it. To be able to look

at the sample routes, all files except README, MAILER and FPL11.DOC

are required.

3.2  Starting the program

Go to the directory you have made, type FPL <Return>. The first

time, before you have configured the program, you may enter a

switch for display type: M for mono display, B for black & white

display connected to a colour adapter, and C for colour display.

3.3  Configuring and Preparing the program

The first time you use the program, you should select SET-UP from

the Main menu, and then CONFIGURE. Enter the data asked for, or

just press <Enter> to leave the item unchanged. After you have gone

through this, a file called FPL.CFG is created.

The next thing you should do, is to select AIRCRAFT from the Set-up

menu, and define your aircraft.

If the nav database is not prepared in your copy of the program

(you will see this by the files FPLFAC.DTA and FPLFAC.IDX missing,

and that the program warns you about it), a file called FPLFAC.TXT

will be included. In that case you will need to build the nav-

database from FPLFAC.TXT, - unless you want to make your own

database. This is done by selecting IMPORT in the Nav-Database

editor.

IMPORTANT: The program will find an existing record in the nav-

database dramatically faster when the database is sorted in

alphabetical order. If you append new records, or change any of the

idents, you should select SORT/COMPRESS DATABASE in the Nav-Data-

base menu afterwards.

3.4  Making a flight plan

From the Main menu, select FLIGHT PLAN. At the Flight Plan menu,

you may see an example route already there. A route in the work

area is retained until you delete it or load another one.

CREATING LEGS IN THE ROUTE EDITOR:

When you want to make a new route from scratch, select NEW ROUTE,

answer YES to erase the present route, and enter the ROUTE EDITOR.

Select ADD LEG, and start entering data. Airfields and other nav-

points are entered by it's identifier. If it is not found in the

nav-database, you may enter the data manually, and, if desired,

save it in the database. Leg #0 contains the departure field and

nothing else. For the other legs, you will also be asked for true

airspeed, wind etc.

AUTOMATIC/MANUAL COURSE/DISTANCE:

Usually you will enter "A", and let the program calculate the

course & distance. But if you are planning a VFR flight for

example, and pick out landmarks on the map, you may want to measure

course & distance with a ruler instead of finding the coordinates.

If so, select "M", and enter these data manually. (In that case,

the Lat/Long and magnetic variation fields may be left open.)

SID'S, APPROACHES (Mostly IFR-related):

Since a normal leg is calculated as a straight line between the two

waypoints, it will be incorrect if a SID or approach involves extra

turns and manoeuvring (which they usually do). There is a feature

which allows you to reserve a certain distance or time for such

cases.

If you want to start out on a Standard Instrument Departure (SID),

you put the departure field in leg #0 as usual. In leg #1 you enter

*SID in the FROM field, and in the TO field you enter where the SID

ends. After typing in TAS etc., you will be asked for GS, DIST and

TIME for the SID. You need only enter two of these items. What you

enter here will not be affected by the TAS and Wind given for the

leg. But the program uses these data, if given, to tell you time

and GS if the leg was flown direct. You can then adjust the GS

and/or time figures as appropriate.

Similarly, for an instrument approach, you enter *APP in the FROM

field (or *ILS or what you like, just make sure it starts with the

"*") and in the TO field enter where the approach terminates

(usually the airfield...). The leg before this one, should end

where the approach starts (well, where else?).

Note: If a SID or approach is just a straight leg (for example a

straight in approach from a beacon), you may enter the data as for

any other normal leg, without using the procedure described above.

ALTERNATES:

When you have reached your destination, you may add legs to an

alternate (or two), if needed. First you create a leg that only

contains *ALT (or *ALT1 or *ALTN etc., just make sure it starts

with "*ALT") in the FROM field. In the following legs, the route to

the alternate is described. If you need two alternates, you enter

*ALT2 in the FROM field after you have reached the first alternate,

and then go on to the second.

DIVERSION FIELDS (IFR and VFR):

In the real world, you sometimes have to change alternate right

before departure, and you do not have the time to make a new flight

plan. But this may be prepared for by having a list giving the time

and fuel burn from the destination to various diversion fields on

your flight plan. Even if you are flying VFR, such a list may

become handy. It is created by entering *DIV in the FROM field in

the leg after the last one of the main (and alternate) route. Each

of the following legs contains only your DESTINATION in the FROM

field, and a DIVERSION AIRFIELD in the TO field. You enter fuel

flow etc. as normal, but note that these values should be an

average for the entire flight from destination to the diversion

field. But there is an additional item asked for, which is extra

time in minutes for approach etc. This item may also be used for

adding some extra minutes if the flight cannot be expected to be a

straight line.

When this feature is used, the list will appear in the flight plan,

with time and fuel burn from the destination field to each of the

optional diversion fields. Also, it will show the total fuel

required if the airfield is used as the alternate instead of the

planned alternate(s).

MAP VIEW:

A VGA display is required for this feature. It may be selected from

the Flight Plan menu or from the Route Editor. It is not very

sophisticated, but it will show you the route, and optionally all

other points from the nav-database in the area covered. If you want

a printout, you will need to use GRAPHICS.COM and the <Prt Scrn>

key. But at least you may select black & white colours (will remove

the menu also) to avoid a totally cluttered printout.

PRINT/SHOW FLIGHT PLAN:

Exit the Route Editor, and select Print/Show flight plan from the

Flight Plan Menu. You will first be taken to a screen where you

enter type of aircraft, what reserves you want and some weight and

fuel data. When completed, a screen with a bunch of data about the

flight will be shown. From here, you may view the flight plan on

screen, print it out or write it to a text file.

3.5  Ending and Restarting PC Flt Plan

The program may be exited from the Main Menu, or directly from the

info screen where you select printing. In both cases, the route and

the other flight plan data will be retained.

3.6  Nav Computer

Selected from the Main menu. This does nothing to your flight plan,

but is used if you want to look up quickly how far it is between

two points, and optionally how long it will take with various

airspeeds and winds. It also gives some more details than the route

editor, like rhumb line distance versus great circle distance, and

initial and final courses for great circles.

Part III - Detailed Guide

4.   INSTALLING

Note: The disk version is supplied with an install program. If this

is used, the instructions below may be disregarded.

If you have a hard-disk, make a directory and copy the required

files into this. The program may also be installed on a floppy

disk. Even if you have a hard-disk, it might be convenient to also

have the program on a floppy which you can bring with you.

The files required to run the program are:

- FPL.EXE              - FPLFAC.DTA

- FPLSETUP.EXE         - FPLFAC.IDX

- FPLFLY.DTA

The following files should also be copied into the directory if you

want to see an example of data making up a flight plan:

- FPL.DTA              - FPL2.DTA

If these files are not present, they will be made by PC Flt Plan

during program run. They contain the route and flight plan data

from the last run.

If you want to be able to load the sample route files, you should

also copy the following files:

- Files ending with .RTx, where x is a number

- FPLFIL.DIR

The two EXE-files work fine if compressed with PKLite(tm), a

shareware program that makes EXE- and COM files significantly

smaller.

5.   STARTING PC FLT PLAN

Make the disk and directory where the program was installed the

current directory (i.e. CD C:\FPL), then type FPL <Return>. The

first time, before you have gone through the configure option, you

may enter a switch for display type. (If you don't, you will be

asked for it anyway.) The switches are:

- M = Mono display connected to a mono adapter (MDA, Hercules)

- B = Black & White display connected to a colour adapter

- C = Colour display and colour adapter.

To use a switch, type FPL, a space, the switch and then <Return>.

Example, which starts PC Flt Plan with mono colour settings:

fpl m<Return>

FPLSETUP.EXE may be run directly, but is normally called from the

PC Flt Plan Main menu. If you go through the CONFIGURE option in

the Set-up menu, a configuration file, FPL.CFG, will be created. Do

not delete this. After you have configured the program, you no

longer need to use any of the switches, nor will you be asked what

display type you have on start-up.

6.   PREPARING THE PROGRAM FOR USE

6.1  Configuring

The first time the program is run, you should go through the

CONFIGURE option in the Set-up menu. When you press <Return>, you

store the data in the input field and proceed to the next item. If

you press <Esc>, configuring is terminated and all items are

restored to their original settings. The various items are

explained as you work your way through, but if you find any of them

hard to understand, just leave the item unchanged and press

<Enter>. None of the items are required, except the colour

settings, and maybe the printer codes, if you have problems getting

the flight plan printed out.

6.2  Aircraft Data

The next thing you should do, is to enter data for the aircraft you

will use. This is done with the option AIRCRAFT DATA in the Set-up

menu. You may also specify aircraft data when making a flight plan,

but it is much more convenient to have it in this little database.

The aircraft data that can be stored are:

- AIRCRAFT:  You may do this two ways: You can store the AIRCRAFT

TYPE, i.e. Cessna 172. Or, if you will operate several different

aircraft of the same type, but with slightly different Dry

Operating Weights for example, you may have one entry for each

aircraft and instead enter the AIRCRAFT REGISTRATION here.

- REMARKS:  The data here are not used in the flight plans so far,

but the entry will be shown when you select an aircraft when

making a flight plan. If you have stored the aircraft by

registration, you may put the type and configuration here.

- ICAO FLIGHT PLAN NAME:  Not used by the program so far, but the

entry is reserved for the aircraft code used in ICAO flight

plans. Example: Cessna 172 is "C172".

- WEIGHT UNIT:  K=Kg/L=Lbs. All weight data for an aircraft must be

in the unit you enter here.

- DRY OPERATING WT:  Aircraft weight, with full oil tanks,

equipment, catering and crew. Includes everything except Payload

and Fuel.

- MAX ZERO FUEL WT:  If applicable. If not, enter 0.

- MAX TAKE-OFF WT:  Yes, just that.

- MAX LANDING WT:  If applicable. If not, enter 0.

- START/TAXI FUEL:  Fuel normally used for start-up and taxi before

take-off.

- RESERVE FUEL FLOW:  Fuel consumption rate normally used when

calculating the reserve required by the flight rules you operate

under.

- HOLDING FUEL FLOW:  Fuel consumption rate normally used when

calculating extra holding, extra reserves, company fuel etc.

- MAX FUEL CAPACITY:  Max tank capacity. Not mandatory.

- UNUSABLE FUEL:  On some aircraft, a certain amount of the fuel in

the tanks are considered not usable, and can therefore not be

counted as available fuel when planning a flight.

- ACTUAL FUEL LOAD:  Typical Fuel Load in the same unit as the

fuel-data above . If Fuel Load varies, you may enter 0, and enter

the number when making the flight plan.

- ACTUAL FUEL WT:  The WEIGHT of ACTUAL FUEL LOAD above. Must be in

the same unit as the other weight data. If ACTUAL FUEL LOAD and

ACTUAL FUEL WT are in the same unit, i.e. lbs, you may enter 0.

If FUEL FLOW-data and WEIGHT-data are in different units, i.e.

gallons and pounds, PC Flt Plan must know the relationship between

these to be able to calculate landing weights etc. This relation is

found when the program is given ACTUAL FUEL LOAD and ACTUAL FUEL

WEIGHT in their respective units.

6.3  Nav-Database

GENERAL:

The Nav-Database is accessed from the Set-up menu. This is where

airfields, navigational aids, intersections and other facilities or

waypoints are stored. Any frequently used point may be stored here,

like special check-points, landmarks, oil-rigs - you name it...

When preparing the program for use, it is a good idea to enter the

facilities in your local area. You can later add points as you

create routes, since the route editor allows you to enter data for

a facility not found in the nav-database, and save it.

When in the route editor, searches are made on the facility's

identifier. The program will find an existing facility in the nav-

database dramatically faster when the database is sorted in

alphabetical order. The reason is that the program first executes a

fast alphabetical search, and, if the facility is not found, a

sequential search from top to bottom is initiated. The database is

not maintained in a sorted order automatically however. It is

therefore recommended to use the SORT/COMPRESS function (explained

below) after having added new facilities, or changed any of the

identifiers. This function also removes empty records, which will

appear when facilities are deleted.

DATABASE STRUCTURE:

The database consists of the files FPLFAC.DTA and FPLFAC.IDX. It is

strongly recommended to maintain backup copies of these files on a

separate disk!

Each facility makes up a record. Each record consists of the

following fields:

- Ident (For example airport code)

- Name  (Name of the facility)

- Latitude (North/South geographical coordinates)

- Longitude (East/West geographical coordinates)

- Magnetic Variation

- Frequency

IMPORTANT: Commas should not be used in any of the fields, as these

are used for separating the fields when exporting to- or importing

from an ASCII file!

Note that in most countries, the three letter and four letter

airfield codes are totally different. The 3 letter codes are those

used by airlines and their international SITA network, while in

most countries the 4 letter ICAO codes are used in ATC flight

plans, notams, weather forecasts etc. USA is a bit special though,

if you take a 3 letter US airfield code and put a "K" in front of

it, you have the 4 letter ICAO code. In PC Flt Plan, it is

recommended to use the four letter codes for airfields, to

distinguish them from the airport VOR's (In the US, the idents are

often the same). Also, the Map View function will then be able to

distinguish between airfields and other facilities (nav-aids and

fixes never have a 4 letter code).

Examples:

3 letter code    ICAO code

London/Heathrow        LHR            EGLL

New York/Kennedy       JFK            KJFK

For those who want some more technical info: The file (fplfac.dta)

is a random access file. Searches are done in a separate, parallel

file (fplfac.idx), which contains only idents, to speed up the

sequential search somewhat. As mentioned, the alphabetical search

is very fast, even on large databases, but the sequential search is

not that bad either; - on a 386sx/25 with a cache program

installed, PC Flt Plan uses a little over 2 seconds to search

through a database with 3000 records.

CREATING/EDITING FACILITIES:

From the Main menu, select SET-UP, then select NAV-DATABASE. Here,

facilities may be viewed, edited, added, listed or written to a

text file in ASCII format. Note that if you delete a facility, a

blank record will appear, as the record itself is not removed. When

you add a new facility, the data will be stored in the first empty

record found, or appended to the end. The SORT/COMPRESS function

will remove any blank records and sort the database in alphabetical

order.

An alternate way to edit the database, is to use the EXPORT

function (in the Nav-Database menu) to export it to an ASCII file.

The file is comma delimited, which means that it may be imported

into most database- or spreadsheet systems, or you may edit it in

your favourite text editor. After external editing, use the IMPORT

function to rebuild the database from the ASCII file. If you have

specified a text editor in the configuration, you may call this

from the function EXPORT/IMPORT/EDITOR. Otherwise, you have to

leave PC FLT Plan, run your editor, then restart PC Flt Plan. The

format of the ASCII file, necessary to import records correctly, is

explained in detail in the program.

FUNCTIONS IN THE NAV-DATABASE EDITOR:

- NEW RECORD:  To add a new facility. The program first asks for

the identifier, and checks for no others records with the same

ident.

If the database covers several countries, you may run into

duplicate identifiers. A way to handle that is to add the two

letter ICAO country code. An example: Midhurst VOR in England has

the identifier "MID". The country code for Great Britain is "EG",

so the ident may be entered as "MID EG". This will never be

necessary for airfields however, as long as their 4 letter ICAO

codes are used.

- FIND RECORD:  Searches for a facility with a given identifier.

- SEARCH ON NAME:  Searches for all records containing the name, or

part of a name, you specify. For example "okman" will find

"Brookmans Park". Note that the search is made on the NAME fields

only, not the idents.

- EDIT RECORD:  Change any item in a record. If you change the

ident, a search for duplicates will be performed.

- DELETE RECORD:  Deletes the facility presently shown, and

replaces it with a blank record.

- LIST RECORDS:  Lists out facilities on screen. If anything is

entered in the input field which appears, only records with

identifiers starting with the letters you type will be listed.

- SORT & COMPRESS:  Sorts the database in alphabetical order, and

removes any blank/empty records. It first writes the records to a

temporary text file. This file is sorted using the DOS sort, or

any other sort program specified in CONFIGURE. If the ASCII file

becomes larger than 64 Kilobytes, the DOS sort will not handle it

however, requiring an external sort program. Many good and small

shareware programs that sort text files are available for this

purpose. After this process, the database is rebuilt from the

resulting sorted file.

- EXPORT/IMP./EDITOR:  From here, you may export the database to a

comma delimited ASCII file, or import from the same. When

importing, the records may be appended to the existing database,

or replace the existing database. If an external editor is

specified in CONFIGURE, this is also accessed from here

- HELP:  A brief help screen.

- QUIT:  Returns to the Set-up menu.

7.   HOW TO MAKE A FLIGHT PLAN

7.1  Data

The flight plan is made from two sets of information:

- ROUTE DESCRIPTION:  The route to be flown from point of depar-

ture, through several waypoints to the destination, and if

required, to one or two alternates and/or a list of diversion

fields. Altitude/flight level, winds, true airspeed and fuel flow

are entered for each leg. For IFR flights, special legs like

SID's, approaches etc. may be entered.

This is the information that is stored when you save a route.

When you load a route; winds, TAS etc. may be changed quickly for

the entire flight without having to edit each leg.

- FLIGHT PLAN DATA: Aircraft (and it's data), reserves, fuel load

and payload. These data are not saved together with the route,

but are contained between program sessions.

Normally, you will first create the route to be flown, or load a

previously saved route and make the necessary changes, - and then

enter the flight plan data. The program takes you through an input

screen for these data when you select PRINT/VIEW FLIGHT PLAN from

the Flight Plan menu.

7.2  Create/Edit a Route

Select FLIGHT PLAN from the Main menu. To clear the route in the

work area, if any, and start from scratch; select NEW ROUTE and

answer Yes to erase.

If you want to use a route saved on disk, select LOAD ROUTE, and

select from the routes shown. Any route present in the work area

will be deleted when loading a new one. You may edit the loaded

route in the Route Editor, or you may change FL/Alt, TAS, Wind or

Fuel Flow globally for all or several legs by selecting GLOBAL

CHANGE from the Flight Plan menu.

8.   ROUTE EDITOR

8.1  Legs

Enter the Route Editor by selecting FLIGHT PLAN from the Main menu,

then ROUTE EDITOR from the Flight Plan Menu.

Select ADD LEG to create new legs.

When some legs have been created, the <Left> and <Right> keys can

be used to move between the legs. <PgDn> or <PgUp> will advance/go

back 10 legs respectively. Select EDIT LEG if you want to make

changes. The <Up> and <Down> keys are used for moving between menu

items.

If you delete or insert a leg, be aware that the TO and FROM fields

will not match between the legs before and after, you must there-

fore edit these as necessary after such an operation!

Leg #0 holds the point of departure only. No further data are

entered for this leg.

8.2  To / From Fields

In the TO and FROM fields, you enter the facility's identifier. The

program then searches the nav-database. If the facility is not

found, you may enter lat/long coordinates, name, magnetic variation

and frequency manually. You will then also be asked if you want to

save the waypoint in the nav-database.

You do not have to enter lat/long coordinates if you want to enter

course and distance manually instead of letting the program

calculate these (See 8.3). This may be useful if you plan a trip

where you just pick landmarks on the map as waypoints and measure

course/ distance with a ruler.

A star (*) as the first character in the FROM-field denotes a

special leg, which is used for SID's, STAR's, approaches etc. They

are also used together with codes to separate the route to

destination, to the alternate(s), and the diversion list:

*xxxx - A "*" and up to 4 characters is used when you want to enter

a SID, STAR, or approach instead of a straight leg. "xxxx"

can be anything (i.e. "SID", "ILS", "APPR" etc.) except for

the codes below. In the TO field enter the point where the

leg ends. For these legs you will be asked for estimated

average ground speed, time or distance. You only have to

enter 2 of the values. For example if you want to reserve

10 minutes of fuel for an approach at the destination, you

can enter 10 minutes and the average GS for the approach.

The program will then calculate the distance this

represents and add it to the total trip distance.

*ALTx - Means that the route after this leg describes flight to the

ALTERNATE(s). No further data are entered in a leg with

this mark, it is used as a separation line. "x" may be

anything, for example "1" or "2". This code may be used

twice in a route description. If used, the route will be

divided into the following sections:

- Before any *ALTx  :  From DEP.FIELD to DESTINATION

- After first *ALTx :  From DESTINATION to ALTERNATE 1

- After second *ALTx:  From ALTERNATE 1 to ALTERNATE 2

(The reason you can have two alternates is that under some

regulations, you can file a flight plan to an airfield

which is below weather minimums, - provided that you have

two alternates. The first alternate must then have landing

minimums, the second must be above alternate minimums.)

*DIV -  Means that the following legs make up a DIVERSION list.

These legs are shown on the flight plan as a list of

optional alternates. They show time and fuel used for

direct flight from DESTINATION to the various DIVERSION

fields, plus extra time for approach or extra route

distance, which may be entered for these legs.

8.3  Automatic or Manual Calculation of course/dist:

For straight legs you enter "A" or "M" for Auto or Manual course &

distance. If you select "A", course & dist will be calculated from

the lat/long coordinates and magnetic variation for the waypoints.

If for example you find that the course differs a degree or so from

what is found on your IFR-chart, and you want it to be the same, -

you may select "M" and enter the magnetic course manually. When

using the "M" option, no lat/long coordinates or magnetic variation

are required.

8.4  Flight Data

When FROM and TO for a leg are defined, you enter the following

data:

- FL/ALT:  Flight Level or Altitude. These data are not used by any

calculations, but are shown as information about planned- or

minimum altitudes on the flight plan. It is recommended to enter

the values as in ATC flight plans; for example "F090" for Flight

Level 90, "A050" for 5000 feet. Or you may use "CLMB", "DESC",

"VFR" or whatever you like.

- TAS:  True AirSpeed in knots.

- WIND:  Wind Direction (what direction the wind comes FROM), and

Wind Speed in knots.

- F.FLOW:  Fuel consumption per hour.

8.5  Map View

(Selected from the Flight Plan menu or in the Route editor)

This function only works with a VGA display or better. When you

select MAP VIEW, you will see a graphical representation of the

route and waypoints. From the Route Editor, the map will be centred

around the current waypoint, while from the Flight Plan menu, the

map will fit the entire route.

The program does not have graphical printing capability, but if you

load the DOS program GRAPHICS.COM (which is a resident program)

before starting PC Flt Plan, you can print out the display by

pressing <Prt Scrn> (if your printer supports graphics that is!).

Before you press <Prt Scrn>, press P to change all the colours to

black & white. This also removes the menu from the display. To get

the colours and menu back, press P again, or simply press <ESC> to

quit the map display. This is somewhat crude of course, but it does

work.

9.   PRINT/VIEW A FLIGHT PLAN

9.1  Flight Plan Data

When the route has been defined, you select PRINT/SHOW FLIGHT PLAN

from the Flight Plan menu. You will then be taken to an input

screen, where the Flight Plan Data are entered. The following data

may be entered or changed here:

- DATE:  Date of flight.

- FLTNO/CS:  Flight Number or Call-sign.

- AIRCRAFT:  The aircraft picked from the aircraft database, or one

that you specify here.

- SKED DEP:  Scheduled time of departure (block).

- SKED ARR:  Scheduled time of arrival.

- FINAL RESERVE:  Reserve required by the flight regulations you

operate under. Enter minutes and fuel flow.

- ROUTE RESERVE:  Additional reserve in PERCENT. If used, the

amount will be calculated as the specified percent of fuel

required to destination and to the alternate(s), if any.

- EXTRA HOLDING:  May be used for expected holding, company fuel or

similar. Enter minutes and fuel flow.

- START/TAXI:  Fuel used for start-up and taxi, before take-off.

The minutes are estimated time from off block to airborne. Will

be counted when ETA is calculated.

- UNUSABLE FUEL:  On some aircraft, the last rest of fuel in the

tanks is considered not usable, and can therefore not be counted

as available fuel.

- ACTUAL RAMP FUEL:  Planned fuel load. AMOUNT must be in the same

unit as the fuel flow figures above. WEIGHT must be in the same

unit as the other weight data. If Fuel LOAD and WEIGHT are in the

same unit, just enter 0 in the weight field.

- DRY OPERATING WT:  Aircraft weight, with full oil tanks,

equipment, catering and crew. Includes everything except payload

and fuel.

- ACTUAL PAYLOAD:  Planned payload.

9.2  Print or Show the flight plan

When you have entered the Flight Plan Data, a screen with a summary

of the flight will be shown. From here you may print out the flight

plan or view it on the screen. Explanation of some of the items

showed:

- TIME:  Airborne time (hr:mins) to destination.

- BURN:  Start/Taxi-fuel + trip-fuel to destination.

- DIST:  Distance (NM) to destination. Includes approaches.

- AVG.WC:  Average wind component to destination (plus=tail-wind,

minus=head-wind).

- DEP:  Scheduled Time of Departure.

- ETA:  Estimated Time of Arrival at destination. Taxi time at the

departure field is counted in.

- NO WIND:  Burn/Time to destination if there had been no winds.

- REQUIRED FUEL:  Total fuel required, considering flight to

destination, alternates(s) (if any), winds, reserves and

start/taxi.

- EXTRA FUEL:  The difference between Required fuel and Actual fuel

load. The time shown is the approximate flight time this represe-

nts, calculated from the average consumption to destination and

alternate(s).

If you have trouble printing out the flight plan, try to find the

proper printer codes and enter them under the configuration option

in the Set-up menu.

If this doesn't work, you may try another solution: Select PRINT

FLIGHT PLAN TO FILE. The file written is in plain ASCII, and may be

imported into a word processor and printed from here. For the

"knee-board size", select a font with a pitch of around 17

characters/inch and normal line height.

The two A4-variants send only plain ASCII-text to the printer, and

should therefore be usable on most printers.

10.  ABOUT THE FLIGHT PLAN

10.1 Leg Times

In the flight plan produced, the time for each leg is rounded up or

down to the nearest whole minute. But the small difference between

the accurate leg time and the whole minute is added or subtracted

to the next leg time. This is the same method used in professional

flight planning systems used by airlines. The reason for this is

that on a trip with many legs, the total time may become several

minutes wrong if for example most of the legs happen to be rounded

down.

10.2 Fuel

The fuel amount shown in the last column of the flight plan is the

remaining fuel you would have over the checkpoint if you departed

with the TOTAL FUEL REQUIRED. This way you can see if you have all

your specified reserves intact along the flight. If you have less

fuel, you have started to consume your reserve, route reserve or

extra holding fuel.

10.3 Corrections

The correction factors shown on the flight plan are fuel and time

to destination, and total fuel required if the conditions were:

- NO WIND:  No winds at all.

- +20KT:  20 knots more TAIL-WIND than the average wind component

you get from the winds you have planned for.

- -20KT:  20 knots more HEAD-WIND than the average wind component

you get from the winds you have planned for.

Example: If you get an average wind component of +7 knots (tail-

wind), the "+20" will be the figures for 27 knots tail-wind, while

the "-20" will be the figures for 13 knots head-wind. The "no wind"

will be for no winds, no matter what the planned winds are.

10.4 How to use the ETO/RTO/ATO fields

After take-off, you start filling out all the Estimated Time Over-

head (ETO) fields all the way down to destination, using the leg

times given. Usually, you will only put down the minutes, except

when starting on a new hour.

When you pass a waypoint, you write down the time in the Actual

Time Overhead (ATO) column, then calculate the new estimated time

for the next waypoint, and put this in the Revised Time Overhead

(RTO) column for that point. This way, you will have a fresh

estimate for the next reporting point for ATC (and yourself).

As you now pass the waypoints, it will be very easy to see if you

are ahead or behind the flight plan by comparing ATO and the

original ETO (the latter was calculated in relation to the take-off

time).

11.  NAV COMPUTER

This function is entered from the Main menu. It does nothing to

your flight plan, but is used if you want to look up quickly how

far it is between two points, and optionally how long it will take

with various airspeeds and winds. It also gives some more details

than the route editor. The following data are calculated:

GREAT CIRCLE: - Initial course, in true & magnetic

- Final course,    "   "  "    "

- Average course,  "   "  "    "

- Distance.

RHUMB LINE:   - Course, in true & magnetic

- Distance.

(Great circles and rhumb lines are explained in chapter 12.)

12. TERMS & ABBREVIATIONS USED IN THE PROGRAM

Acc    - Accumulated time

AMT    - Average Magnetic Track

ATC    - Air Traffic Control

ATO    - Actual Time Overhead (when passing a waypoint)

ATT    - Average True Track

Alt    - Altitude

Arr    - Arrival

Burn   - Fuel Burn

CS     - Call Sign

Dist   - Distance in nautical miles

DOW    - Dry Operating Weight. Weight of everything except payload

(cargo/passengers) and fuel

Dep    - Departure

Dest   - Destination

ETA    - Estimated Time of Arrival

ETD    - Estimated Time of Departure

ETO    - Estimated Time Overhead

FF     - Fuel Flow (Fuel consumption per hour)

FL     - Flight Level

Flt No - Flight Number

GS     - Ground Speed. Speed in relation to the ground after wind

effect is taken into account

Kts    - Knots ( = speed in nautical miles per hour)

LWT    - Landing Weight

MH     - Magnetic Heading

MLWT   - Max Landing Weight

MT     - Magnetic Track

MTOW   - Max Take-Off Weight

MZFW   - Max Zero Fuel Weight

NM     - Nautical Miles (1 NM = 1.852 meters)

RTO    - Revised estimated Time Overhead. (You may also encounter

RTO used as an abbreviation for Rejected Take-Off.)

Rte Res- Route Reserve. In percent of fuel used to destination and

alternate(s)

TAS    - True AirSpeed

TOW    - Take-Off Weight

WC     - Wind Component (Headwind or tailwind)

ZFW    - Zero (wing) Fuel Weight. Weight of everything except fuel.

Larger aircraft usually have a max limit on this, because if

the aircraft body is loaded up while the wings remain light

(little fuel in the wing tanks), the bending loads on the wings

will be excessive.

Great Circle - The shortest line between two points on the earth. Over a

large distance, or at high latitudes, the course may

change considerably trough the flight.

Rhumb Line   - (Also called loxodrome track) A line between two points

on the earth which crosses all meridians at the same

angle. This means that the course will remain the same

all along the route. Over a large distance, or at high

latitudes, a rhumb line will be significantly longer than

the great circle.

Note: In the route editor, all legs are calculated as great

circles. The course is the average course.