Convention

The following conventions are used throughout this document.

'Enter'    means the Enter key or the Return key.

Type 'key' means press the specified key without 'Enter'.

Press F?   means press the specified function key without 'Enter'.

Enter      means type in the specified text and followed by 'Enter'

key.

Select/Use an option:

use cursor keys to move the highlighted option and press

'Enter', or type the upper-cased character on the left.

GAF is not case sensitive, use either upper or lower cased key.

Using The Menu

The GAF system menu, at the top of the screen, is the primary user interface

to the system.  System menu provides sub-menu function selection (function

menus): Info, Run, Display, Control, Adapt, Method, Segment, Feedback,

Eval, Other, and Help.  Use the first character of each selection or use

cursor keys (right/left arrows) to select the sub-menu.  When a function

menu is selected a pull down menu appears to provide function selection.  To

select a function either by the character on the left of that function or move

the cursor with up/down arrow keys and then "Enter".  When a function is

selected, further menu selection or data entry menu may be provided by that

function.  Press ESC in function menu will return to system menu.  Press

ESC under system menu will bring run function menu, except when the

system menu was ESCaped from any of the running screen (i.e. from

adapting or simulation).  Please refer GAF.DOC for details.  The following

steps are typical for function selection under simulation or adapting.

press "ESC" to escape back to system menu

select desired function menu (see above)

select desired function (see above)

press "ESC" again under system menu to go back

Simulation

This simple example demonstrates the application of the fuzzy logic to

control a motor based on its angular position feedback.  Lets assume that this

is a robot arm, and its arm extension limit (in angular position) is from 0

degree to 70 degree.  The inputs to the control loop are the position, position

error, and the speed, the output is the current to the motor.  The example

also demonstrates that using fuzzy logic to simulate the feedback loop, e.g.

the position feedback and the angular speed.  The file "robotarm.gaf" defines

the control loop, and the file "robotarm.fbk" defines the feedback loop.

Run Simulation

After start the demo, you'll find the system menu is at the top of the

screen.  Select run menu by typing 'R', then select "simulation mode"

by either type 'S' or move high lighted cursor to "simulation mode" and

press 'Enter'.  After simulation mode is selected, GAF displays the

control segment on the screen and waits for user input.  Please refer to

GAF.DOC for details of the display windows.  Type 'F' to start the

simulation. You should see the trend plots of inputs and outputs on the

right and the rules and firing status on the left of the screen.

Reset the system

Press function key F7 to reset the system.  The system reset is defined

by the INITIALIZATION declaration in all segments of the system.

Type 'F' again to run the simulation.

View the feedback loop

Now switch to the feedback loop by pressing F3, the screen displays the

feedback segment.  Press F7 to reset the system, and then type 'F'.

Now watch the plot of the inputs and the outputs for the feedback loop.

Stepping through

Let's switch back to the control loop by pressing F3 twice, and reset it

by F7.  Now press 'Enter' to see the demo program goes one step.  The

program allows you to do one step by 'Enter' or multiple steps by typing

2 to 9, or 10 steps by typing '0'.  Type '0' and watch the program goes

through 10 steps and note different rules are fired.  At any time type 'F'

will bring you to the end of the plot frame.

Continuous mode

Press 'C' to run the simulation continuously.  You can stop continuous

mode by pressing 'C' again or ESCape (press ESC) back to system

menu.  While in continuous mode press F7 to reset the system.

Change variable value

Stop continuous mode by pressing 'C'.  Now let's change the simulation

from raising the arm to lowering the arm.  Press F7 and then ESC to

escape back to system menu.  Press 'S' to select segment menu.  The

segment menu allows you to switch segment and change characteristics

of the segment.  Switch to feedback segment by selecting "segment

number" under segment menu, and enter '1' for feedback segment.

Select segment menu again, and select "set input value".  GAF pops all

inputs in a menu beneath the segment menu, select "REFERENCE"

input and enter its new value to 0.  Select segment menu one more time

to change the input variable "POSITION" value to 70 (from "set input

value").  Now get back to the simulation by pressing ESC, press F3 to

switch back to control segment, and start simulation by stepping through

it.   Note that you can also change the "POSITION" value from the

control segment (segment 0).

View the plot

From system menu type 'D' to select display menu.  The display menu

allows you to rearrange the display windows.  Let's select "select plot

only mode" by typing '5', and then escape back to simulation.  GAF

should redraw the screen with trend plot takes most of the screen.  Run

simulation by hiting F7 and then 'F'.

View the combined result

From Display menu select "select whole display mode".  The right

portion of the screen splits into two parts, the upper half shows the plot,

and the lower half displays the combined result for the output.  The

combined result shows the trapezoid result of all fired rules (rules

generate non-zero output).  Rerun the simulation by pressing F7, and

then 'F'.  Now, the combined result tells you about the firing results of

all rules and the defuzzified crisp output (the current of the motor) from

all fired rules.

View details about the fuzzy inference

The above exercise only allows you to view the combined result of the

output.  GAF also allows you to examine the firing details of the rules.

This feature along with the stepping mode is the best way to tuneup or

debug the control segment.  From the segment menu, select "display rule

firing", GAF highlights the first rule by using the left column of the text

window.  Use down arrow key to move to rule 5 and press 'Enter' to

select it.  Go back to the simulation, you'll find the selected rule's fuzzy

sets are displayed to the left of the combined result.  Note that each

fuzzy set has the same color as the text window.  Try previous exercises

to view the changes of the fuzzy rule firing status.

Disable rules

GAF allows user to disable individual rule for testing fuzzy segment.

Let's disable rule 10, this will effectively break the control system.

Since there is no rule to respond when the POS_ERROR is

VERY_LARGE, so after system reset (press F7) there will be no output

from the control segment.  From segment menu select "disable rule",

GAF highlights the rule ID 0 on the left of the screen.  Press 'End' to

move the highlight cursor to rule 10, and then type 'Enter'.  Now restart

the simulation by pressing F7 then 'F'.  You should see the rule 10 is

marked with 'X' to indicate it's been disabled, and also that there is no

response to the position of the robot arm.

Enable rules

There are two options under segment menu allow you to enable rule(s).

The "enable all rules" option enables all rules in this segment.  The

"enable rule" enables the selected rule.  So, type 'A' under segment

menu to enable rule 10.

Change sampling rate

Another important feature in GAF is that you can tune your system by

trying different sampling rate (or cycle time defined in each segment).

From control menu select "sim schedule rate", GAF prompts for new

rate.  Enter 0.2 (in second) and restart the simulation.

Increment/decrement sampling rate

An easy way to change the simulated sampling rate is to use the '+' and

'-' keys.  Press '+' each time will cause the sample rate to increase

0.005 second, and press '-' will decrease the sample rate by 0.005

second.  The amount of increment/decrement (default is 0.005 second)

can be changed through the control menu's "Increment value" option.

Now put the simulation in continuous mode by 'C', reset it by F7 and

then use '+' and '-' keys to adjust the sample rate to find out what's the

max stable rate of the control loop.

Change output gain

You can change the gain of the output, from segment menu's "set output

gain" option.  The default gain is 1.0.

Check individual segment

This feature allows you to examine one segment at a time.  Let's

examine the feedback loop's response at a constant motor current.  Press

F3 to switch to the feedback segment.  From run menu select "check

segment".  Then from segment menu change the value of input variable

"CURRENT" to 1.0.  Run the simulation by typing 'F' to see the

response of constant current input.

Getting online help

Press F1 to get online help.

Display the system limits

Press F2 to see the system limits.

View segment information

Press F4 to see the variable definition, initialization and normal

functions, and other information for the selected display segment.

View symbol definition

Press F5 to see the fuzzy set symbols' definition for this segment.

View preset definition

Press F6 to view the preset definition for this segment.

Set new plot time frame

The default plot frame period is five seconds (except in sim clock mode),

this can be changed through display menu's "plot duration" option.

View local variables

The default display mode is to display IN, OUT, and INOUT variables,

the LOCAL variables are not displayed.  Use "display local variable"

option in display menu to enable/disable LOCAL variables.

Quit the program

Press 'Alt-Q' to quit the program under simulation or select "quit"

option from run menu.

Adapting the Fuzzy Segment

The genetic adapt mode is an offline adaptation mode.  While fuzzy logic

mimics the human's imprecise reasoning, the genetic algorithm mimics the

evolution of the nature.  The genetic algorithm uses randomization techniques

to reach local optimum without exhausting searching through the solution

state space.  With randomization, genetic algorithm is also capable of

escaping from local optimum and hopping into a new and better state.  Apply

genetic algorithm in fuzzy control eliminates problems associated with fuzzy

system.  GAF uses genetic algorithm to derive proper rules and fuzzy sets

from the initial rules.  By changing, adding, deleting rules and fuzzy

membership sets of the fuzzy system, the genetic algorithm automatically

adapts and optimizes the fuzzy control system.

The adapting example is built on top of the simulation example, by adding

a third evaluation segment.  The file "robotarm.evl" defines the evaluation

segment.

Start Adapting

From system menu, type 'R' to bring up run menu.  Select "adapt

mode" by pressing 'A'.  GAF starts adapting the control loop and

displays the control segment along with adapt status on the top of the

screen.

Pause every iteration

Press Alt-P to halt the adapting after each iteration.  Press 'C' to

continue next iteration.  Press Alt-C to switch to continuous mode.

Pause every cycle

Press Alt-P to stop the adapting after each cycle.  Press 'C' to continue

next cycle.  Press Alt-C to switch to continuous mode.

Change size of gene pool

Press 'Esc' while in continuous adapting, GAF brings up the system

menu.  Select adapt menu by type 'A'.  From adapt menu select 'max

best items', and enter a new pool size.  This will change the number of

genes will be saved (and evaluated) by GAF.

Change minimum score

Again, from adapt menu, select "minimum score", GAF prompts for

new minimum value.  Enter 0.5, this tells GAF to accept all evaluation

results ,which are greater or equal than 0.5.  The default of the

minimum score is 0.6.

Change method

The method menu provides user to control the algorithm of genetic

adaptation.  Select "adding rule" from method menu, enter 0 for its new

weight.  With zero weight, GAF will not generate (adding) new rule into

the control loop.

Print the result

Select 'Other' menu from the system menu, and type 'A' to generate

report for all genes in the gene pool.  The result will be saved in the

adapt.rpt file of the current directory.

Generate statistic report

Again type 'S' (generate statistic log) from 'Other' menu, GAF produces

a statistic report file with the file name specified by 'statistic to' at the

bottom of the menu.  The log includes statistics for all methods.  Based

on the statistic you may want to adjust the weight of the methods.