The Beepster Junior

Warren Associates

3076 Hidden Forest Drive

Snellville, GA 30278

(404) 985-4620 voice

(404) 985-6522 fax

(404) 985-6537 bbs

What is the Beepster Jr? It is a rather small program which can send

alpha and digital messages to personal pagers from a standard PC

compatible computer with a 1200 baud modem.

In the shareware (unregistered) form, it will handle up to 15 users and

their pagers. The registered version is able to handle up to 30 users.

Once the software is registered, the right side of the display is used

for the second row of 15 users.

The command line is used to configure the com port.

BEEPJR COM0             find an active modem

BEEPJR COM1             for com1

BEEPJR COM2             for com2

BEEPJR COM3             for com3

BEEPJR COM4             for com4

BEEPJR DETECT           show a list of active com ports.

Once the system has been configured, it will not need to be specified

the next time you run the program. If you change your modem port address

you will need to change the configuration as well.

You should contact your paging service provider and find out their

protocol, baud rate, and telephone number. Once you have this

information you are ready to configure the System Setup.

If you have to dial the telephone area code to call the number, you also

need to enter the area code as well. On all calls which do not require

you to enter the area code to dial, do NO enter the area code! This is

how the system decides how to dial the number!

The Dial Command string may need to be changed if your telephone system

needs a prefix digit to get an outside line. If there is nothing in the

Dial command string, it will default to "ATDT" for a standard tone dial.

Once you have the pager information collected, you can lock the database

to prevent anyone from modifying the database file. This is done by

setting a DOS environment variable on the computer running the program.

add the following line to your AUTOEXEC.BAT file:

SET BJLOCK=YES

To unlock the database, perform this command at the DOS prompt and then

start up the Beepster Jr. program:

SET BJLOCK=