Program STSORBIT PLUS

Space Shuttle and Satellite Orbit Simulation

With Multi-Satellite Tracking

(Enhanced Version for 286/386/486 Computers)

(C) Copyright David H. Ransom, Jr., 1989-1993

All rights reserved.

Version 9338

September 15, 1993

by David H. Ransom, Jr.

Rancho Palos Verdes, California, USA

Bulletin Board Systems

----------------------

RPV ASTRONOMY BBS

(310) 541-7299 @ 14400/9600/2400 Baud

RPV HOTLINE BBS

(310) 544-8977 @ 2400/1200 Baud

Program STSORBIT PLUS Satellite Orbit Simulation                  Page ii

TABLE OF CONTENTS

INTRODUCTION ......................................................1

HARDWARE AND SOFTWARE REQUIREMENTS ................................6

PROGRAM DESCRIPTION ...............................................7

STSORBIT PLUS FILES ...............................................9

STSPLUS MAP PROJECTIONS AND DATABASES .............................12

PROGRAM SETUP AND USAGE NOTES .....................................14

DOS 5.0 CONFIG.SYS Setup ........................................14

Using a RAM Disk ................................................15

Copying Files for STSORBIT PLUS .................................16

Slow Computers and 80x87 Math Coprocessor Chips .................17

Starting Program STSORBIT PLUS ..................................19

Predicting Visible Satellite Passes .............................20

Predicting Satellite Passes with STSPLUS ......................21

Predicting Satellite Passes with TRAKSTAR .....................23

Printing Graphics Screens .......................................26

Known STSPLUS Problems and Bugs .................................26

Preparing 2-Line Elements using VEC2TLE by Ken Ernandes ...........29

PROGRAM OPERATION .................................................31

STSORBIT PLUS SATELLITE TRACKING FEATURES .........................33

Orthographic Projection Maps ....................................33

Rectangular Projection World Maps ...............................34

Rectangular Projection Quadrant Maps ............................34

Rectangular Projection Zoom Maps ................................35

Location Maps with Isocontours ..................................36

Tracking Station Maps with Isocontours ..........................36

Location and Features Labels ....................................37

Big Clock Options ...............................................39

Satellite Motion Maps ...........................................39

Satellite Position and Orbit Projections ........................40

Satellite Visibility ............................................41

User's Circle of Visibility .....................................41

Spacecraft Circle of Visibility .................................42

SUN and Solar Features ..........................................43

TDRS and Real Time Satellite Features ...........................45

Ground Tracking Stations and .TRK Files..........................47

Event Timers and Audible Alarms .................................49

Pausing the Ground Track Display (F6 Key) .......................52

Switching between MET and T+Epoch ...............................52

Using FAST Time (F4 Key) ........................................53

On-line Help (F1 Key) ...........................................53

ACTIVE KEYS DURING GROUND TRACK DISPLAY ...........................55

STSORBIT PLUS MAIN MENU ...........................................58

F1  Convert Keplerian Data to 2-Line Format .....................59

Example Data Input and Output ...............................61

Received Keplerian Orbital Data Form ........................63

F2  Read/Update NASA/NORAD 2-Line Elements ......................64

Update Current TDRS and Real Time Satellites ................64

F3  Data Output and Pass Prediction Selections ..................68

Setting up Position and State Vector Data Output ............71

Setting up Tabular Pass Predictions .........................74

Data Mode 1: Azimuth/Elevation Data Format ..................78

Data Mode 2: Latitude/Longitude Data Format .................79

Data Mode 3: Topocentric RA/DEC Data Format .................80

Program STSORBIT PLUS Satellite Orbit Simulation                  Page ii

Data Mode 4: Ascending Node X-Y-Z State Vector ..............81

Data Mode 5: X-Y-Z Cartesian State Vector, 2 Data Lines .....83

Data Mode 6: X-Y-Z Cartesian State Vector, Comma Delim ......85

Data Mode 7: X-Y-Z Cartesian State Vector, Labeled Data .....87

Data Mode 9: Pass Predictions ...............................89

F4  Calculate Satellite Positions with TRAKSTAR .................91

F5  Set Launch Time and Date ....................................91

Using File STSPLUS.LTD for Launch Date & Time ...............92

F6  Set/Read/Save TDRS and Real Time Satellites .................93

F1  Display/Modify Satellite Assignments ....................93

F2  Save SCF Satellite Configuration File ...................96

F3  Read SCF Satellite Configuration File ...................97

F4  Clear Static and Real Time Satellites ...................97

F7  Set FILENAMES and PATHS .....................................97

F8  Set Program TIME and DATE....................................98

F1 Restore System Date and Time .............................100

F2 Set DOS System Clock .....................................100

F3 Set Simulated Date and Time using Calendar Method ........101

F4 Set Simulated Date and Time using MET ....................101

F9 Display Current RIGHTIME Corrections .....................102

F10 Set UTC OFFSET and DAYLIGHT Flag ........................102

F9  DOS Shell ...................................................103

F10 Set STSORBIT PLUS Program Options and Features ..............103

ENTER  Resume Mission ...........................................103

ESC    Quit STSORBIT PLUS and Save Current Mission ..............103

PROGRAM OPTIONS AND FEATURES MENU .................................105

F1  Program STSORBIT PLUS Information ...........................105

F2  Set New Local Coordinates ...................................105

F3  Set Display Features ........................................107

F4  Select Satellite Coordinates ................................107

F5  Show Ascending & Descending Node Data .......................108

F6  Set Map Projection and Size .................................108

F7  Enable/Disable EVENT TIMERS .................................108

F8  Enable/Disable Audible ALARMS ...............................109

F9  Set UTC Time Offset and Daylight Flag .......................109

F10 Enable/Disable Printer Logging ..............................110

SET DISPLAY FEATURES ..............................................112

F1  Display LOCAL Circles of Visibility .........................112

F2  Display TDRS and Real Time Satellites .......................112

F3  Display Additional Map Grid Lines ...........................113

F4  Display Tracking Stations ...................................113

F5  Display Ground Track: DOTS/LINE .............................113

F6  Display Spacecraft Circle of Visibility .....................114

F7  Display South Atlantic Anomaly Zone .........................114

F8  Display Terminator, Sun and Spacecraft Lighting .............114

F9  Display Map Locations and Features ..........................115

F10 Display Lakes and Rivers ....................................115

STSORBIT PLUS's Orbital Model .....................................116

Accurate Time and the Personal Computer ...........................118

Methods for Setting DOS Time ....................................119

Maintaining Accurate DOS Time ...................................120

Programs TIMESET and RIGHTIME ...................................122

Computer Bulletin Board Systems ...................................126

STSORBIT PLUS Revision History ....................................128

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 1

INTRODUCTION

------------

Program STSORBIT PLUS is an enhanced version of STSORBIT, my original

orbital tracking and display program. As a general rule, a 286 or better

computer (AT-class IBM compatible) is required. A math coprocessor chip is

STRONGLY RECOMMENDED and will dramatically improve performance; the math

coprocessor chip is REQUIRED for acceptable performance when using the

orthographic projection. Some users report "usable" performance on an XT-

class machine WITH a math coprocessor. See the section HARDWARE

REQUIREMENTS for additional information and discussion. The program is

intended for use during Space Shuttle missions and for general satellite

tracking using NASA/NORAD 2-Line Orbital Elements. One primary satellite

and up to 32 additional "static" or "real time" satellites may be

simultaneously tracked in real time on most computers. Both orthographic

and rectangular map projections are available, displaying the Earth as a

globe or the more traditional "flat" map. Tabular line-of-sight satellite

pass predictions may be made from within STSORBIT PLUS and passes of

interest may be easily displayed.

STSORBIT PLUS is very accurate when used with current (and accurate)

2-line elements. The program has been qualified by the USAF and meets the

requirement of placing a satellite within 0.5 km over a 24-hour time span

from the epoch time of the 2-line elements. It is used daily at the USAF

Central Computer Complex at Cape Canaveral, by the DOD C-Band Radar Network

at their worldwide radar sites, and at half a dozen NASA and aerosapce

control centers around the country. The program is made available to school

teachers through the NASA Spacelink BBS and the NASA Teacher Resource

Centers. Numerous schools have made contact with the space shuttle via

SAREX amateur radio while using STSORBIT PLUS.

STSORBIT PLUS is copyrighted software; you are hereby granted a non-

exclusive license for non-commercial or educational use only. Agencies of

the U.S. Government are also hereby granted a non-exclusive license for

internal use. Use STSORBIT PLUS if you like it, discard it if you don't.

There are no warranties of any kind. If you wish to use STSORBIT PLUS

commercially, write for license information. The only request I make of

users is that they take the time to complete and return the confidential

questionnaire in file README. The questionnaire gives users a chance to

offer comments and suggestions, and lets me know that people use and

appreciate STSORBIT PLUS. Registration of STSORBIT PLUS is inexpensive and

optional -- but will be appreciated and will encourage me to continue

supporting and enhancing the program.

Program STSORBIT PLUS (which I will usually refer to as STSPLUS from

here on) is intended to display the position and ground track of an

orbiting satellite on a selection of maps ranging from a full map of the

world to zoom maps showing considerable detail. The program has special

features implemented at the request of NASA astronauts and others for use

during a NASA Space Shuttle mission. With the appropriate 2-line elements,

STSPLUS displays the position and ground track of a variety of satellites

such as the Space Shuttle, the Hubble Space Telescope, the Gamma Ray

Observatory, or the Russian MIR Space Station. Accurate TDRS coverage,

including times for acquisition and loss of signal, is calculated for

satellites which use that satellite network for communications. Special

Location and Tracking Station displays show concentric isocontours, circles

of equal satellite altitude; these special maps can be especially valuable

for visual or amateur radio sightings.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 2

**********************

*  IMPORTANT NOTICE  *

**********************

After almost four years, I have discontinued support for the

simple orbital model first used in my original STSORBIT program.

The accuracy of that model is marginal at best and timely 2-line

orbital elements for space shuttle missions and other satellites

are now widely available. If you wish to use the simple orbital

model, use STSORBIT or a version of STSORBIT PLUS prior to 9240.

Users who wish to convert space shuttle or satellite state

vectors to 2-line format should see the section "Preparing 2-Line

Elements using VEC2TLE" for a description of a program by Kenneth

Ernandes written specifically for this purpose. VEC2TLE has been

validated during several space shuttle missions and is highly

recommended.

The initial premise for STSORBIT was to attempt to duplicate the wall

map in NASA's Mission Control Center in Houston, Texas. Before I started

this project, I had seen several other programs which tracked satellites

but each fell short of my map and display objectives for one reason or

another. I therefore set out to do the job myself. STSORBIT and now

STSORBIT PLUS have been the result. Since then other programs have appeared

which produce similar information, most notably Paul Traufler's excellent

TRAKSAT (which was inspired by STSORBIT). It may be, of course, that others

will judge this effort lacking for some tasks, but no one program can do

everything. One problem is that of screen size and resolution: the wall map

at Mission Control Center is some twenty feet wide with an impressive pixel

resolution, very different from the typical personal computer monitor. The

NASA wall map shows essentially the entire globe in a cylindrical

projection; STSORBIT also used a cylindrical projection and restricted the

vertical display to latitudes from +85 degrees to -85 degrees in order to

achieve reasonable proportions and vertical resolution while at the same

time showing recognizable land features. STSORBIT PLUS now presents the

Earth as a globe using an orthographic projection with zoom while still

retaining the original cylindrical (rectangular) projection. STSPLUS adds

many additional features and improved accuracy over the original STSORBIT.

Initially, and as a consequence of a lack of accurate orbital data for

Space Shuttle missions while they were in progress, I did not try to be

especially precise with respect to the orbital mathematics. Additionally,

mathematical complexity had to be held to a reasonable minimum if older

computers not equipped with a math coprocessor were to be able to maintain

the presentation in real time. My somewhat casual attitude toward

mathematical precision changed with the launch of the Hubble Space

Telescope (HST) and the regular availability of US Space Command 2-Line

Elements via modem from TS Kelso's Celestial BBS. Until HST, I had been

content to manually adjust the orbital data occasionally during the course

of a typical five day mission and live with the errors inherent in my

original simple orbital model. The accuracy of that model degrades rapidly

after five or ten orbits and, although it could be adjusted from time to

time during a mission, more accurate data are now readily available prior

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 3

to a launch and during a mission. The NASA SpaceLink BBS in Huntsville,

Alabama began posting 2-line orbital elements for the Space Shuttle in

early 1991 due in part to my persistent and continuing requests; Bill

Anderson, Jeff Ehmen, and Flint Wild, sysops of the SpaceLink BBS, are

continually upgrading the services available.

Beginning in mid-1990, therefore, STSORBIT was extensively modified to

read orbital data from these USSPACECOM 2-line elements and thereby

maintain significantly improved accuracy over long periods of time. As an

incidental benefit, the ground tracks of other satellites (such as the

Russian space station MIR) could also be displayed. At present, the orbital

model (SGP4) used with 2-line elements is accurate only for low Earth

orbits. Deep space orbits, defined as orbits having an orbital period

greater than or equal to 225 minutes, require a more complex orbital model

(SDP4) which takes into account solar and lunar perturbations for best

accuracy. STSPLUS calculates data and displays a ground track for deep

space objects but the accuracy of these data has not been validated; it is

believed to be "reasonably" accurate. I plan to add the SDP4 orbital model

to STSPLUS in a future release when time permits.

At about the same time, STSORBIT also found its way to the NASA

Johnson Space Center in Houston, Texas. Quite a few individuals from JSC

sent me comments and suggestions for further improving the program, among

them Ron Parise of the STS-35/ASTRO-1 crew. Ron suggested that I make

modifications to allow the display of Mission Elapsed Time (MET) for

shuttle missions while using the USSPACECOM 2-line elements. This would allow

both the higher accuracy of the USSPACECOM 2-line orbital data and permit

following the mission timeline using MET. Since launch time and date are

not included in the 2-line elements but are required to compute MET, these

data must be entered independently. Another suggestion from Ron and others

was to include the Sun, solar terminator (calculated at Mean Sea Level),

and spacecraft lighting conditions to determine if the spacecraft is

visible.

Not satisfied with the somewhat rough map used with STSORBIT (a

digitized EGA world map), I upgraded the maps to use a modified version of

the World Data Base II. This had the desired effect, to the point where

rivers and other landmarks could easily be recognized on the monitor and on

downlinked orbiter television. As a side effect, however, the processor

overhead increased dramatically -- some slower computers not equipped with

a math coprocessor were unable to keep up. I therefore essentially "froze"

the original STSORBIT program (except for minor updates) and created this

new program, STSORBIT PLUS, intended for the faster, more capable

processors. Since mid-1991, STSPLUS has also spread throughout the various

NASA Centers and around the world.

In addition to NASA and individual users all over the world, STSPLUS

and STSORBIT are also being used in an educational setting. As many as 1100

high schools participated in the Inspire Project, a VLF propagation test

flown on STS-45 and for which STSPLUS was one of the recommended tools. At

a middle school in Kansas, the program is projected in the school

auditorium from time to time during a mission to show the children

graphically what is happening and to give them a sense of "real time"

participation in our space program. At an Air Force training facility,

STSORBIT is one of many tools used to prepare Air Force officers for their

duties in the Air Force Space Command. The program was widely distributed

at a recent National Association of Science Teachers convention and by

radio amateurs at regional "ham fests". It is also available to educators

through the NASA Teacher Resource Centers and the NASA Spacelink BBS.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 4

In perhaps its most prestigious installtion, STSPLUS is the software

used by the NASA/JPL Multimission Computer Control Center in Pasadena,

California, to display the ground track of Earth-orbiting satellites. The

Canadian Space Agency used STSPLUS as part of their briefing government

officials during the STS-52 mission in October, 1992. Intelsat used STSPLUS

operationally in May of 1992 at their Launch Control Center in Washington,

DC, and at five tracking stations around the world during the exciting STS-

49 mission, the maiden flight of Endeavour and the rescue/reboost of the

INTELSAT-VI satellite. Intelsat was kind enough to send me a letter saying

that STSORBIT PLUS was "critical to mission success"! Numerous other

official and semi-official installations use STSPLUS as the primary

satellite tracking software or to supplement other software.

A brief biographical note: I am a retired physicist and engineer who

spent all of his professional life in the world of electronics, data

communications and, more recently, computers. As a young man I was actively

involved in the early American space program including projects such as

Ranger, Mariner, Mercury, Gemini, and Apollo. Exciting times indeed! I

spent considerable time at the Jet Propulsion Laboragory in the early

1960's as a contractor on Ranger and Mariner; my respect and admiration for

JPL and its people has, if possible, increased over the intervening

decades. My interest in space has continued to this day. The desire to

"keep in touch" with our Space Shuttle missions was one of the incentives

in the development of this software. I continue to be astonished that a

relatively inexpensive personal computer is sufficient to perform

calculations that pushed the limits of our best mainframe computers only a

decade or so ago. If STSORBIT PLUS also serves to help spark the interest

of young people in science and technology or can be a learning tool at any

level, I will have more than achieved my goals.

No discussion of satellite tracking would be complete without thanks

to Major T. S. Kelso, USAF, who almost single handedly brought satellite

tracking within the reach of "ordinary folks". TS's Celestial BBS has been

providing unclassified 2-line orbital elements direct from US Space Command

(formerly NORAD, the North American Air Defense Command) at Cheyenne

Mountain, Colorado, since 1986 or so. For many years, Celestial BBS was the

only electronic source for orbital elements in the world. The Celestial BBS

may be reached at (513) 427-0674 and is located near Dayton, Ohio. TS has

also written a variety of satellite tracking software and his most recent

program, TRAKSTAR, may be used directly from within STSPLUS to generate

tabular data on upcoming satellite passes.

Special thanks to Paul Traufler for his friendship and encouragement.

Our regular telephone conversations have generated many a new idea and the

synergism has been beneficial to us both. Our two programs, STSORBIT and

TRAKSAT, have engaged us in a friendly rivalry which has, I think, improved

both programs many fold. I may have provided the initial spur to Paul to

write TRAKSAT (in order to improve on my "sloppy orbital math", as Paul

described it) but TRAKSAT has in turn kept my nose to the grindstone and is

recognized by many as the standard against which other satellite tracking

programs are judged. The emphasis of the two programs is slightly

different, with STSORBIT concentrating on the graphical display and TRAKSAT

on high precision analytical and predictive techniques. I highly recommend

TRAKSAT for the serious satellite tracker. My thanks as well for Paul's

help in upgrading STSORBIT to use the USSPACECOM 2-Line Elements and other

technical assistance.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 5

Thanks also to Rob Matson and Joel Runes. Rob for offering comments

and code to help me implement several of STSPLUS's more exotic features;

Rob coined the phrase "isocontours" to describe the circle of equal

satellite altitude around a location and his fine SKYMAP program generates

high accuracy printed star maps with or without satellite tracks. And Joel

for keeping us all up to date with current elements for space shuttle

missions and his patience in testing innumerable beta versions of STSPLUS,

thereby helping to track down some of the more subtle bugs.

Finally, my thanks to all those individuals who have taken the time to

write or leave a message on my BBS with comments and suggestions. While I

haven't implemented every suggestion, many are now included and the

feedback is most welcome.

For individuals interested in our space program and who have access

to a modem, I recommend NASA's SpaceLink Bulletin Board System in

Huntsville, Alabama, (205) 895-0028, available twenty four hours per day,

300 to 2400 baud. NASA SpaceLink, located at the NASA Marshall Space Flight

Center and with 8 lines, provides a wealth of information on NASA and its

projects. 2-line orbital elements for a Space Shuttle mission are usually

available while the mission is in progress. In addition to educational

materials and software (including my programs STSORBIT PLUS, STSORBIT and

JPLCLOCK), general information on NASA programs and plans, news releases,

and images from prior spacecraft missions such as Voyager, SpaceLink also

devotes a complete section to current news and information on the Space

Shuttle. I particularly appreciate the STS Mission Press Kit, posted about

two weeks before each mission, which provides a great deal of information

on the upcoming mission, payload and crew as well as broadcast schedules on

NASA Select Television, Satellite F2-R, Transponder 13. Mission status

reports are generated daily during the course of a mission. I regularly

call SpaceLink and post files of interest on my own RPV ASTRONOMY BBS.

For current Space Shuttle orbital information (if a mission is in

progress), 2-line elements for more than 1,000 satellites, and the most

recent versions of STSORBIT PLUS, STSORBIT, TRAKSTAR, TRAKSAT, and SKYMAP,

call RPV ASTRONOMY BBS or RPV HOTLINE BBS (see title page for numbers and

data rates available). The main system has well over 1,500 more or less

regular users and is often busy, so please be patient. If you wish to

receive STSORBIT PLUS (or any of my other programs) on disk, see file

README for information.

David H. Ransom, Jr.

7130 Avenida Altisima

Rancho Palos Verdes, CA 90274

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 6

HARDWARE AND SOFTWARE REQUIREMENTS

----------------------------------

An AT-class computer equipped with a 286 processor (running at 8 MHz)

and a 287 math coprocessor chip is the minimum system used for all program

testing and development. While other systems may give acceptable

performance, this minimum configuration assures that most features will

execute as described and in real time. Performance with 386/387 and 486DX

systems will be considerably superior to 286 systems. Note that NO TESTING

is performed on systems not equipped with a math coprocessor chip. The

following minimum hardware is recommended:

286/386/486 IBM-compatible computer

287/387/487 math coprocessor chip

VGA color display

Hard disk with up to 3MB available

RAM disk with at least 500K space

The 287/387 (and 487 for 486SX processors) math coprocessor chip is

STRONGLY RECOMMENDED and is REQUIRED for acceptable performance. The

calculations relating to orbital mechanics are very complex and STSPLUS

will use the coprocessor if one is equipped; performance is improved by

about an order of magnitude. Other "fast" processor and coprocessor

combinations may yield acceptable performance. Math coprocessor chips are

now reasonably inexpensive and the performance improvement is impressive

and well worth the modest cost. As an example, my vintage Zenith laptop,

equipped with an 80C88 processor and an 8087 math coprocessor, is just able

to keep up in real time (rectangular modes ONLY!) when running at a clock

speed of 8 MHz but the map drawing times are very slow. However, an 8 MHz

286 computer without a math coprocessor does NOT provide reasonable

performance; map drawing times are painfully slow.

STSPLUS is intended to be used with an EGA or VGA video adapter and a

color monitor; with these systems, the display is in color. Because of its

improved vertical resolution, the VGA is recommended over the EGA. A

monochrome display with shades of gray may also be used with the program

(with the "/M" command line option). Because of hardware limitations, CGA

and HGC systems can only present graphics in monochrome; although those

display adapters are supported to some degree in current versions of

STSPLUS, that support may NOT continue in future versions. The original

STSORBIT will continue to support CGA and HGC monitors.

A hard disk is required for performance reasons and for storage of the

program, map databases and orbital elements files. A RAM disk with

sufficient space to hold the various data files is also recommended for

improved performace and to reduce wear and tear on the hard disk during

periods of extended use.

Although the program may execute properly on other software operating

systems, STSPLUS has been designed and tested using standard configurations

of Microsoft DOS 3.3 and 5.0. No optional Terminate and Stay Resident

programs (TSR's) or "shell" programs have been tested except for Tom

Becker's RIGHTIME. Third party memory management programs and Digital

Research DRDOS may experience problems although some users report that the

latest release of DRDOS 6.0 works correctly.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 7

PROGRAM DESCRIPTION

-------------------

A typical Space Shuttle orbit is nearly, but almost never exactly,

circular with an altitude of approximately 160 nautical miles to a maximum

of approximately 300 nautical miles and an inclination of about 28 degrees

through about 57 degrees. Occasional missions, especially military

missions, fly at higher altitudes and/or inclinations and often use more

elliptical orbits. Prior to 1990, little of this information was known to

very good accuracy by the casual listener. Initially, therefore, the

interested would-be mission observer may have only the time and date of

launch and intended orbital altitude and inclination to initialize a

tracking program. Given the geographical coordinates of the Kennedy Space

Center and assuming a circular orbit, the data is sufficient to calculate

at least a rough idea of the Shuttle's position for the first several

orbits. After that, additional information was required if the position was

going to be very close. This was the method used in my original STSORBIT

program when 2-line orbital elements were not available.

Estimated 2-line elements are usually available prior to a space

shuttle launch and I usually post "adjusted" 2-line elements within two

hours of a launch. "Real" 2-line elements from NASA or US Space Command are

usually available 8 to 12 hours after launch. 2-line elements yield a more

accurate position over longer time periods (provided no orbital maneuvers

are performed). Using 2-line elements for any satellite is quite simple; no

adjustment of orbital parameters is necessary. An abbreviated version of

the 2-line element file available at the time of this release of STSPLUS is

included in the distribution files; this abbreviated file contains

approximately 150 satellites while the "full" file as posted on my BBS

typically has more than 700 satellites. The data for each satellite is

referenced to a specific date and time, the "Epoch" of the data. As a

general rule, orbital calculations will be relatively accurate for 10 to 20

days after the Epoch date; the lower the orbit, the greater the effect of

factors such as atmospheric drag and the less accurate the calculations

will be as time passes.

STSPLUS displays a portion of the Earth using either an orthographic

porjection (the Earth seen as a globe) or cylindrical projection (similar

to the Mercator projection commonly used). The maps show most of the

Earth's land boundaries and continental areas. Major oceans, seas, and

rivers are easily recognizable. Considerable detail is shown at higher zoom

factors. Automatic map generation ensures that the satellite is always

displayed. The display shows the selected satellite as a small symbol or

icon, the projected orbital ground track for the next three hours and the

the past one and a half hours, and many other features including circles of

visibility, TDRS coverage, and the solar terminator.

Data is displayed which gives the current ground track position of the

satellite, known as the "sub-satellite point", antenna or viewing angles,

spacecraft lighting, TDRS communications coverage (when applicable), and a

variety of other information. A selection of map modes and display features

allow users to configure the program to meet their requirements.

STSPLUS has been validated by the USAF and is approved for use on the

Eastern and Western Ranges. It is sufficiently accurate (given current and

accurate 2-line orbital elements, of course!) that the program is being

used operationally by NASA, USAF, and Intelsat. Among the current users

are:

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 8

NASA/JPL Multimission Computer Control Center, Pasadena, CA

NASA/Lewis User Operations Facility, Cleveland, OH

Rockwell Mission Control & Support, Downey, CA

Aerojet Emergency Control Center, Sacramento, CA

USAF Central Computer Complex, Cape Canaveral, FL

DOD C-Band Radar Network, worldwide radar sites

In addition to these official or semi-official users, thousands of amateur

radio operators and "just plain folks" all over the world use STSPLUS to

track the space shuttle and other satellites.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 9

STSORBIT PLUS FILES

-------------------

STSORBIT PLUS is normally distributed via bulletin board systems in

archived form using the ZIP format by PKWare. Note that all files (except

map databases) for STSORBIT PLUS are called "STSPLUS" in order to conform

to DOS filename requirements and to avoid confusion with the similarly

named files for the original STSORBIT. The following files are usually

included (files marked with "*" are available separately):

STSPLUS.EXE        Main STSPLUS Program     (required)

STSPLUS.DOC        Documentation            (not required)

STSPLUS.ICO        Icon for WINDOWS 3       (optional)

STSPLUS.KEY        STSPLUS Active Keys      (optional)

STSPLUS.LOC        Map Locations & Features (optional)

STSPLUS.LTD        Satellite Launch Dates   (optional)

STSPLUS.SCF        Satellite Config File    (optional)

STSPLUS.TRK        NASA Tracking Stations   (optional)

STSPLUS.CTY        City Coordinates         (optional)

STSPLUS.INI        Initialization data      (see below)

EARTH4.MCX         Level 4 Map Index        (required)

EARTH4.MCP         Level 4 Rect Map Data    (required)

EARTH4.XYZ         Level 4 Ortho Map Data   (required)

EARTH3.MCX         Level 3 Map Index        (optional) *

EARTH3.MCP         Level 3 Rect Map Data    (optional) *

EARTH3.XYZ         Level 3 Ortho Map Data   (optional) *

EARTH2.MCX         Level 2 Map Index        (optional) *

EARTH2.MCP         Level 2 Rect Map Data    (optional) *

EARTH2.XYZ         Level 2 Ortho Map Data   (optional) *

EARTH1.MCX         Level 1 Map Index        (optional) *

EARTH1.MCP         Level 1 Rect Map Data    (optional) *

EARTH1.XYZ         Level 1 Ortho Map Data   (optional) *

MSHERC.COM         Hercules driver          (required for HGC)

TLEnnn.TXT         2-Line Elements          (optional)

NASA.TRK           NASA Tracking Stations   (not required)

CIS.TRK            Russian Tracking Stations(not required)

INTELSAT.TRK       INTELSAT Tracking Stns   (not required)

SPACENTR.TRK       Other Tracking Stations  (not required)

STSLNDG.TRK        Orbiter Landing Sites    (not required)

README             STSPLUS Confidential Questionnaire

QUICK.DOC          Quick Start Instructions

SOP9311.ICO        Alternate STSPLUS Icon   (not required)

Files noted as "(required)" must be in the current default directory (or in

a specified directory in some cases) for program operation. Files noted as

"(optional)" are not required when STSPLUS is operated but provide

additional features or information if present. In order to minimize the

disk space required, all.EXE files been compressed with PKWare's PKLITE

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 10

Professional; these files require additional time to begin execution since

they are decompressed "on the fly" at load time.

***  IMPORTANT NOTE ***

File STSPLUS.INI contains initialization data from previous runs

of the program. If file STSPLUS.INI is not present it will be

created. Note that if STSPLUS.INI was written by a version prior

to 9332, all data will be ignored and the program must be

initialized as if being run for the first time.

STSPLUS can use map databases with different degrees of map detail.

Level 4, required for operation, contains the minimum detail and Level 1

contains the maximum detail. As noted in the list above, three files are

used for each level of map detail: MCX files contain an index of the map

data; MCP files contain map coordinates for rectangular projection; and

XYZ files contain map coordinates for orthographic projection. STSPLUS

checks for the levels that are present and uses the level appropriate for

the zoom factor in effect or, if that level is not present, the maximum

level that is present. Level 1 is checked first, then Level 2, etc. Level 4

files MUST be present or an error message is displayed and the program

aborts.

***  IMPORTANT NOTE ***

STSPLUS assumes that if a particular level of map database is

found, ALL lower levels of map database are present. Missing

levels of map database will cause a program error.

File STSPLUS.KEY is a quick reference list of the keys that are

active while the map is displayed and includes a brief description of the

function of each key. It has been extracted from this documentation.

File STSPLUS.LTD contains the launch date and time for selected

satellites. The file may be updated when new satellite launch date and

time data is entered via the program.

Files with filetype .TRK are tracking station locations or other

locations of interest which may be plotted on the map. These files may be

created or edited with a standard ASCII editor. If you wish to use a

different TRK file, use Function Key F7 from the Main Menu to select

the desired file.

File STSPLUS.SCF is a sample multiple satellite configuration file

which may be used to display multiple TDRS and real time satellites. Up to

16 additional satellites may be tracked using this feature.

File TLEnnn.TXT (where "nnn" will be a number such as "153") is a set

of USSPACECOM 2-line elements as of the date of the file. Note that the 2-

line elements should only be used for ten to twenty days after the epoch

date for each satellite if reasonable accuracy is to be maintained. Current

orbital elements are regularly posted on my bulletin board system and

elsewhere. Other files with 2-line elements are also available; they

typically have names like GSFCnnn.TXT or N2L-nnn.TXT for general

satellites, and STSmmNnn.TXT for Space Shuttle missions. Space Shuttle

orbital elements are usually posted at least daily during missions; because

of orbital maneuvers, Space Shuttle elements more than 24 hours old may

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 11

yield inaccurate positions.

File STSPLUS.LOC contains geographic coordinates and labels for

selected locations and major oceans and seas. These labels may optionally

be displayed on the maps. The file may be edited with a standard ASCII

editor to add or delete locations and features. See the section "Location

and Features Labels" for additional information.

Other files, such as 2-line elements for an upcoming Space Shuttle

mission or a mission in progress, may be included from time to time. Files

with 2-line orbital elements normally have filetype ".TXT" or ".TLE". Some

common satellite name abbreviations are:

STS       Space Shuttle missions

HST       Hubble Space Telescope

GRO       Compton Gamma Ray Observatory

UARS      Upper Atmosphere Research Satellite

TOPEX     Topex/Poseidon Earth Resources Satellite

ROSAT     Roentgen Satellite Observatory

MIR       Russian Space Station

There are many other satellites for which data is available. US Space

Command (formerly NORAD) currently tracks some 7000+ objects, of which data

for more than 700 is usually included in the full TLEnnn.TXT files posted

on my bulletin board system.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 12

STSPLUS MAP PROJECTIONS AND DATABASES

-------------------------------------

Cartographers and navigators have long wrestled with the problem of

map projections, the process of transferring location information from a

sphere to a flat surface or map. The U.S. Geological Survey publishes "An

Album of Map Projections", Professional Paper 1453, that contains some 90

basic projections in over 130 different modifications and aspects. Each

projection or modification was developed to serve some specific need or to

optimize certain parameters. The primary concern with any map projection is

distortion. For STSPLUS, this means the accuracy with which the selected

portion of the Earth's surface is displayed. Naturally, the viewer desires

accuracy in area, shape, and distance; unfortunately, you can't have all

three simultaneously with a single map projection.

The cylindrical or rectangular projection, used here and with the

original STSORBIT program, is a good compromise where drawing speed is to

be minimized. The map coordinate data translate exactly to screen pixels

with a minimum of computer processing. However, this projection is unable

to diplay the polar regions well; the distortion increases non-linearly as

the latitude increases. I have chosen to limit the latitude to plus and

minus 85 degrees to avoid some of the severe problems which occur very near

the poles. As the magnification is increased, the distortion inherent in

this projection is reduced for the area shown but different latitudes are

displayed at different scales for a given magnification. A satellite

appears to move more rapidly in high latitudes than at mid or equatorial

latitudes.

The orthographic projection views the world as a sphere and thus only

a single hemisphere can be seen at any given time. However, since the map

center may be placed at any desired point, the "hemisphere" may include a

pole. The projection is calculated as if the viewer were at a great

distance from the Earth and therefore can see a complete hemisphere. This

makes the projection ideal for viewing high inclination satellite orbits.

Perspective is not included in the projection calculations for simplicity.

The orthographic projection has the advantage that ANY area of the Earth

may be viewed, including the poles, and the scale remains the same for any

given magnification and is independent of the area viewed. At the center of

the map, circles of visibility appear as true circles; however, area

distortion increases toward the edge of the screen and is especially

noticeable when the full hemisphere is displayed.

The map databases used for STSPLUS are an adaptation of the Micro

World Database II ("WDB"), generously placed in the public domain by Peter

Pospeschil and Antonio Riveria, and were produced in about 1986. The

original data were from the U. S. Central Intelligence Agency (CIA) as

distributed by the National Technical Information Service (NTIS). There are

several known errors in the database: one island in the South Pacific is

missing its northwestern portion, and several islands near the North Pole

are classified as "lakes" instead of "islands" and are therefore rendered

in the wrong map color. There are also numerous gaps in coastlines which

make color fill very difficult over the range of magnifications used here.

The original map data are identified by "level of detail" with Level 1

having the greatest detail and Level 5 having the least detail. I have

elected to use Levels 1 through 4 for STSPLUS; Level 5 is so coarse as to

be almost useless. I have also written several utility programs to extract

the map coordinate data from the original WDB files by level of detail and

to build an index file for each level to speed access to the data. For the

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 13

orthographic projection, the data are also converted from latitude and

longitude to geocentric Cartesian coordinates to minimize subsequent

processing time. Although the resulting map database files are

substantially smaller than the original data files, they are still quite

large for the higher levels of detail; for example, the Level 1 files

require approximately 1.5MB.

STSPLUS attempts to select the map database files appropriate to a

given magnification and map projection. If the selected level is not

present, the program tries the next lower level until the selection process

reaches Level 4, the lowest level of detail. The Level 4 map database files

are required for operation of the program and are included in the standard

program distribution. The Level 3 map database files are included with

program registration and are available separately on my BBS. The Level 2

and Level 1 map database files are available separately to registered

users. See file README for registration information.

Because of the size of the map database files, and because certain

third party memory managers have had problems with the dynamic memory

allocation (as implemented by Microsoft BASIC) used in STSPLUS prior to

Version 9240, map data are ALWAYS read directly from disk. This means that

map drawing times, even for the rectangular projection, are somewhat slower

than with previous versions of STSPLUS. It also means that the program uses

the disk files every time the map is redrawn. Users who plan on running

STSPLUS for long periods of time may wish to place the map database files

on a RAM disk to minimize wear and tear on their disk drive. Use Function

Keys F7+F3 from the Main Menu to set up the correct map database path.

The structure of the map database files is the same for all levels of

detail and the index file (filetype .MCX) is the same size in each case.

STSPLUS cannot distinguish between levels except by file names. Thus, if

you are using a high speed computer such as a 486DX, you may rename the

files to force STSPLUS to use a higher level of detail for a given zoom

or magnification factor. All three files (.MCX, .MCP, and .XYZ) for a

particular level must be kept together with the same filename or chaos will

result! Naturally, drawing times will be increased as the price of the

improved detail.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 14

PROGRAM SETUP AND USAGE NOTES

-----------------------------

The following notes may prove helpful in setting up STSPLUS to operate

most efficiently on your system or to provide hints in ways that some of

the system and program features may be used to advantage.

DOS 5.0 CONFIG.SYS Setup

------------------------

Users with DOS 5.0, especially those who take advantage of the high

memory management capabilities and those who use add-on memory managers,

should include the following line in their CONFIG.SYS file:

STACKS=9,256

This command causes DOS to allocate more memory for the internal stacks

used by DOS and some applications programs. DOS 5.0 seems more sensitive to

the amount of allocated stack space than were prior DOS versions and

programs which executed with no problems on DOS 3.3 may fail on DOS 5.0. A

common symptom of insufficient internal stack space is that STSPLUS

"freezes" the computer and a reboot is required. Other unpredictable errors

can also result from stack errors. "9,256" creates 9 stacks of 256 bytes

each; the "256" may be replaced by "384" or "512", and the "9" may be

replaced by "15" if the basic STACKS command improves but does not cure the

problems. See your DOS manual for additional information.

The use of memory managers such as EMM386, QEMM386, and 386MAX with

386 and 486 systems will cause the processor to operate in the Virtual 8086

Mode, a "feature" which is not well documented by Microsoft. Users should

be aware that this may cause considerable additional processor overhead,

especially with graphics. For example, my 486DX/33 typically draws the

orthographic map in almost half the time when a memory manager is NOT

present but the improvement is less significant with a 386SX/20:

486DX/33 Drawing Time WITHOUT 386MAX:       3.68 seconds

486DX/33 Drawing Time WITH 386MAX:          6.70 seconds

386SX/20 Drawing Time WITHOUT QEMM386:     17.91 seconds

386SX/20 Drawing Time WITH QEMM386:        21.97 seconds

As the example times illustrate, the time saving is proportionally higher

with a faster computer. Note, however, that a memory manager may be

required for the use of extended or expanded memory and for certain

applications. The use of different CONFIG.SYS files, selected by a batch

file or other methods, can optimize performance for various applications.

Another method is to boot the computer from a floppy disk (suitably

formatted and configured) when the "simple" CONFIG.SYS is to be used.

You can still use HIMEM.SYS and load DOS in high memory when EMM386 or

another memory manager is not used by just including the following lines in

CONFIG.SYS:

DEVICE=C:\DOS\HIMEM.SYS

DOS=HIGH

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 15

Using a RAM Disk

----------------

A RAM disk is a simulated disk drive, created in the extended memory

of your computer. Under most circumstances, it is much faster to read and

write data to a RAM disk than to an ordinary disk drive. Remember that all

data on a RAM disk is lost when the computer is shut down or power is lost.

However, disk caching such as that provided by the Microsoft SMARTDRV

driver or, better yet, hardware caching if it is included in your system,

can perform as well as or even better than a RAM disk. Performance of disk

caching varies as a function of both hardware and software, so testing may

be required to determine the optimum configuration for a given computer.

Because of program memory requirements, a RAM disk should be used only

if your computer is equipped with expanded or extended memory. Using a RAM

disk in conventional memory (the memory up to 640K) will use memory that

STSPLUS (as well as most other programs) may need to operate correctly. The

actual size RAM disk you can provide will depend upon how much memory is

equipped in your computer and what memory may be required for other uses.

The following line may be added to your CONFIG.SYS file and will

create a 3000Kb RAM disk using the RAMDRIVE software provided with

Microsoft DOS 5.0:

DEVICE=G:\WINDOWS\RAMDRIVE.SYS 3000 /E

where the file "RAMDRIVE.SYS" is located in "G:\WINDOWS\" in this example.

The "3000" specifies the size of the RAM disk in Kb, and the "/E" instructs

the program to use EXTENDED memory. See your DOS manual for additional

information.

Many of the files used by STSPLUS are read only once when the program

is first started. Little gain in performance will be achieved by putting

these files on a RAM disk. The map database files (EARTH*.*) and the

locations and features file (STSPLUS.LOC or whatever alternate filename(s)

will be used) are read each time the map is drawn. If you frequently change

satellites, moving the 2-line elements file(s) to a RAM disk may also

improve performance. Users with fast hard drives and/or effective disk

caching software will probably notice little or no difference in

performance when using a RAM disk; however, if the program is being run for

extended periods of time, using a RAM disk will eliminate hard disk use

(and wear and tear) during program operation.

All map database files MUST be in the MAP DATABASE path displayed when

F7 is pressed from the Main Menu. The size of the RAM disk must be large

enough to contain all map database files used for normal operation. Once

the maximum map database level has been chosen, ALL lower level map

database files must also be present in the directory. For the lower zoom

factors, Levels 3 and 4 are sufficient; even when using the higher zoom

factors, most users will be satisfied with Levels 2, 3, and 4. Map database

file sizes are shown in the following table:

EARTH1.MCP    613800 08-20-92   6:44a

EARTH1.MCX     11914 08-20-92   6:44a

EARTH1.XYZ    920700 08-20-92   6:44a

3 files 1546414 bytes

EARTH2.MCP    373948 08-06-92   1:32a

EARTH2.MCX     11914 08-06-92   1:33a

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 16

EARTH2.XYZ    560922 08-06-92   1:32a

3 files  946784 bytes

EARTH3.MCP     88144 08-06-92   1:04a

EARTH3.MCX     11914 08-06-92   1:04a

EARTH3.XYZ    132216 08-06-92   1:04a

3 files  232274 bytes

EARTH4.MCP     44804 08-06-92   1:07a

EARTH4.MCX     11914 08-06-92   1:07a

EARTH4.XYZ     67206 08-06-92   1:07a

3 files  123924 bytes

I use a batch file called "S.BAT", located in the same directory as

STSPLUS.EXE, which takes care of copying the map database files to my RAM

disk (drive J: in the example below) the first time the batch file is run

and then starts STSPLUS:

@echo off

if exist j:\earth4.mcp goto Run

copy /b \sop\earth4.* j:

copy /b \sop\earth3.* j:

copy /b \sop\earth2.* j:

copy /b \sop\stsplus*.loc j:

:Run

stsplus /r %1

The files copied to the RAM disk in the example require somewhat less than

1.5Mb. The line which begins "if exist ..." tests to see if the files have

already been copied to the RAM disk and jumps to the label ":Run" if so.

The "%1" allows me to enter an additional command line option (such as

"/M" or "/EGA" for testing. Depending upon where the map database files are

located, the drive and/or path will have to be changed in the lines which

copy the files. I also set the various paths and filenames in STSPLUS using

F7 from the Main Menu to those required for use with the RAM disk.

Copying Files for STSORBIT PLUS

-------------------------------

STSPLUS is intended to be used on systems with a hard disk. If all

files are present, up to 3MB may be required. While it is possible to

operate the program from a high density floppy disk (with some files

omitted), map drawing times are painfully and unacceptably slow.

I recommend that a separate directory called STSPLUS be created for

all of the required and optional files. If you received STSPLUS on disk

with unpacked files, simply copy all files from the floppy disk(s) to your

hard disk using the command:

COPY /B A:*.* C:\STSPLUS

where the floppy drive is assumed to be A:, the hard disk is assumed to be

C:, and the subdirectory STSPLUS already exists. Use the commands

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 17

C:

CD \

MD STSPLUS

to create the subdirectory STSPLUS if it does not already exist.

If you received the program in compressed, self-extracting .EXE format

(with a filename like SOP9338A.EXE and/or SOP9338B.EXE), create the STSPLUS

directory as above then log into the STSPLUS directory and unpack the files

with the commands:

C:

CD \STSPLUS

A:SOP9338A

and repeat the last command for each .EXE file received, changing the

command to reflect the correct filename.

If you received the program in compressed format (.ZIP), copy the .ZIP

file(s) to the STSPLUS directory and then enter the command:

PKUNZIP <filename>

where <filename" is the name of the .ZIP file to unpack. After all files

have been unpacked, you may delete the .ZIP files (but keep a backup copy

just in case!). You MUST use PKUNZIP Version 1.1 or later to successfully

unpack .ZIP files received from my BBS or from NASA SpaceLink BBS!

********************

*  IMPORTANT NOTE  *

********************

If you are upgrading from a prior version of STSPLUS and

experience problems, delete the file STSPLUS.INI to force the

program to recreate its initialization parameters! This will

resolve most path and filename problems. Use Function Keys F7 and

F10+F3 from the Main Menu to set all program paths, filenames,

and options.

Slow Computers and 80x87 Math Coprocessor Chips

-----------------------------------------------

STSPLUS has been designed for 80286/80287 or better computers equipped

with an EGA or VGA color display. While the program can be executed on some

older 8088 (XT-class) computers equipped with the 8087 math coprocessor,

performance is seriously degraded. But it would seem that warnings and

suggestions can NEVER convince people that their old clunker is past its

prime or that the calculations required for orbital mechanics and graphics

are very complex and tax even a powerful computer. The best mainframe

computers we had a decade or more ago had trouble doing what I now take for

granted on a personal computer!

Not all personal computers are created equal. Further, the 80x87 math

coprocessor chip can do many of the calculations ten or twenty times faster

than the main processor. For 8088 (XT-class) and 80286 (AT-class)

computers, this makes a tremendous difference AND for a very modest cost,

often well under $100. Some users report problems with coprocessor chips

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 18

manufactured by IIT and USLI; while the problems may be related to the

computer rather than the coprocessor chip, I recommend avoiding coprocessor

chips by those manufacturers. Finally, I really don't want to hear from

users how slow this program runs on older machines; I recommend the

original STSORBIT if you want the best performance from any computer not

equipped with a math coprocessor chip. Having said that, I DO run STSPLUS

on my old 8MHz 8088/8087 LCD laptop with most features enabled.

STSPLUS always runs as fast as the processor will permit and, since

most features are available for all computer configurations, it is the

responsibility of the user to select program features and options

consistent with the desired performance. For example, the solar terminator

requires considerable time to perform the required calculations and to

update the display and, if this feature is not required, performace will be

enhanced if it is disabled.

As features have been added to STSPLUS, it has become increasingly

difficult for very slow computers or computers not equipped with a math

coprocessor to perform the necessary calculations in a reasonable time.

STSPLUS tests to determine whether or not a math coprocessor chip is

present and will use it if so. The following table illustrates the

difference the processor and a math coprocessor chip make:

Processor   Speed   Coprocessor   Time (secs)

---------------------------------------------

486DX       33 MHz      YES               6.2

386DX       20 MHz      YES              12.8

386SX       20 MHZ      YES              16.8

386SX       20 MHz       NO              66.0

286          8 MHz      YES              30.0

286         12 MHz       NO              86.6

8088         8 MHz      YES              65.0

8088         8 MHz       NO             426.4

The tests were performed with STSPLUS by measuring the time required from

the Main Menu until the satellite appeared on the world map display using

the rectangular map projection. All data were resident in memory (no disk

operations required). All tests were made using the same display options

(most were enabled). Slightly better performance can be achieved from the

slower computers if some options, such as Sun terminator, are disabled.

Clearly, the 8088 without the math coprocessor chip is not acceptable, and

NONE of the computers without the math coprocessor chip performs very well.

When STSPLUS begins, it checks to determine the type of processor and

math coprocessor. If no math coprocessor chip is detected, the following

caution message is displayed. Press ENTER (or any key) to continue.

** CAUTION **

STSORBIT PLUS has detected NO MATH COPROCESSOR CHIP!

CPU Type = 80386DX or 80387SX

NDP Type = (not installed)

STSPLUS requires a Math Coprocessor Chip for acceptable performance.

Calculation and map drawing times will be VERY SLOW. There is no

remedy except to add a Math Coprocessor Chip to your computer.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 19

Users without a coprocessor should consider using

STSORBIT

which is my original, simplified tracking program.

If, in spite of everything, you insist on using STSPLUS on your old

clunker, here are a few cautions and reminders.

1.   Especially at startup and when drawing the maps, long time delays can

be expected with no math coprocessor -- on the order of minutes in

some cases. Status messages are presented on the screen during some

(but not all) of these delays. Note also that STSPLUS.EXE is

compressed to save disk space and is decompressed at load time; this

may cause a noticeable delay on some systems.

2.   Avoid using the orthographic projection with slow computers; the map

drawing times are much longer than the rectangular projections. Also

avoid high zoom factors since it is possible that the satellite may

move during the drawing process to the point where as soon as the

display is completed, it's time to redraw the map again. This process

will repeat endlessly and a reboot may be required. Restart the

program without the "/R" command line option and use F10+F6 from the

Main Menu to select WORLD or QUAD maps.

3.   If you have a monochrome monitor, experiment with the "/M" command

line option to force monochrome operation. On some monochrome systems

the program may otherwise fail with or without an error message. On

other monochrome systems, certain colors may not be visible when

simulated using shades of gray.

4.   The original CGA display, even when equipped with a color monitor, can

display reasonable graphics (640x320) ONLY in monochrome. Your color

CGA monitor buys you nothing for graphics that are quite inferior to

the EGA and VGA.

Starting Program STSORBIT PLUS

------------------------------

STSPLUS uses file STSPLUS.INI to save various information required for

operation. Since the format of that file may change from time to time,

STSPLUS ignores the file unless it was written by a recent program version.

If the file does not exist, STSPLUS will automatically create it.

To start program STSPLUS, use a batch file similar to "S.BAT" in the

preceeding section or enter one of the following commands:

STSPLUS            Automatic monitor, CGA/HGC/EGA/VGA

STSPLUS /EGA       Force EGA (or lower) monitor

STSPLUS /CLK       Use 43 or 60 lines for graphics display of

data and large clock characters. NOTE: This

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 20

feature available with EGA and VGA displays

ONLY! It is ignored for CGA/HGC monitors.

STSPLUS /CGA       Force CGA monitor

STSPLUS /M         Force monochrome operation, EGA/VGA

STSPLUS /R         Resume last mission automatically

Only one display option (/EGA or /CGA) may be used. Other options may

be combined and entered in any order. For example, using the following

command will resume the prior mission and force EGA mode:

STSPLUS /R/EGA

If you are using a Hercules Graphics Card, run the program MSHERC

prior to running STSPLUS. This Microsoft program works with compiled BASIC

programs to enable use of the Hercules Graphics Card. Orthographic

projections are NOT supported for Hercules Graphics Cards. One user

reported that setting the HGC to FULL and selecting Page Zero (using

software supplied with the HGC) was sufficient for proper operation. At

least one HGC "clone" user reported that STSPLUS could not be run under any

circumstances.

If you have already run STSPLUS and simply wish to resume the prior

mission, use the /R (resume) command line option:

STSPLUS /R

STSPLUS will sense the monitor type, enable color for EGA and VGA systems,

then proceed directly to plotting the mission. The data from the last run,

as saved in file STSPLUS.INI, is used to initialize the program. If 2-line

elements were used, that file must also be present. Once started in this

manner, pressing the ENTER key after the map is displayed will return to

the Main Menu.

Predicting Visible Satellite Passes

-----------------------------------

One of the most popular uses for a satellite tracking program is to

show when a satellite of interest will be visible from a specified

location. Using STSPLUS, my wife and I have spotted four different Space

Shuttle missions, the Hubble Space Telescope, the Russian MIR Space

Station, and many others with the naked eye. The trick, of course, is

knowing when and where to look for the satellite.

During normal operation, STSPLUS displays two timers in the form of

countdown clocks (Minutes:Seconds), "AOS" and "LOS", for the user's

location (as well as for a number of other events).  AOS, Acquisition of

Signal, is the time remaining until the satellite is next within the circle

of visibility. LOS, Loss of Signal, is the time remaining until the

satellite next passes outside the circle of visibility. STSPLUS looks ahead

four hours (240 minutes) from the time the map is drawn to determine the

next AOS and LOS event; the clocks are blank if the time is greater than

240 minutes. A quick inspection of these clocks can therefore determine if

a potentially visible pass is upcoming within four hours.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 21

Using its internal pass prediction feature, STSPLUS can look ahead

in 48 hour blocks and list the passes when the selected satellite will

be within the local circle of visibility (line of sight, no lighting

constraints applied).

For more extensive tabular data on upcoming visible passes, external

software must be used. There are a number of satellite tracking programs,

most notably Paul Traufler's TRAKSAT and TS Kelso's TRAKSTAR, which can

generate tabular data for a given satellite or group of satellites listing

when the satellite(s) will be visible subject to various visibility

constraints.

Predicting Satellite Passes with STSPLUS

----------------------------------------

STSPLUS has a pass prediction capability (using F3 from the Main Menu)

in addition to its graphics capabilities to make Line-of-Sight pass

predictions for the current satellite quick and easy. Once a satellite has

been selected (with F2 from the Main Menu), pressing F3 from the Main Menu,

and selecting Data Format 9 (Pass Predictions ) will quickly produce a

listing of the Line-of-Sight passes for that satellite over the next 48

hour period. See the section "Pass Predictions and Data Output" for

additional information. Here are some comments and suggestions for

satellite viewing and tracking with STSPLUS.

1.   The pass predictions are listed in blocks of 48 hours beginning with

the current real or simulated time, and are given an arbitrary pass

number from 1 to 99. The data include the "AOS" or Acquisition of

Signal when the satellite rises above the local horizon, "MAX

VISIBILITY" or the time at which the satellite reaches the highest

altitude above the local horizon, "LOS" or Loss of Signal when the

satellite sets below the local horizon, and "Duration" or the total

time the satellite is above the local horizon. Note that "local

horizon" means the true horizon rather than local terrain. For either

visual or electronic tracking, the altitude above the local horizon

when the satellite may actually be tracked is usually at least five

degrees even under optimum conditions.

CAUTION: Users with slow computers, especially those without a math

coprocessor, will find that pass predictions may require considerable

time -- up to tens of minutes using a slow 286 without a 287!

2.   Be sure to use current orbital elements for the satellite. My RPV

Astronomy BBS always has current 2-line orbital elements for some 650+

satellites and these data are also available from other electronic

sources. For Low Earth Orbit satellites like the Space Shuttle or MIR,

orbital elements should be no more than about ten days old; for higher

orbit satellites, orbital elements remain accurate for longer periods,

up to about 30 days. Satellite maneuvers can radically change the

orbital elements. Pass predictions can be no more accurate than the

orbital elements used!

3.   Examine the "Alt" (maximum altitude) given under the "MAX VISIBILITY"

columns for each pass and select a suitable pass. As a general rule,

the higher the maximum altitude, the better the visibility. If you

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 22

wish to see a particular pass as a ground track display, enter the

pass number and STSPLUS will set simulated time to about 30 seconds

prior to the time of maximum visibility and prepare the display. Press

"L" while the ground track is displayed to use the Location Map

centered on your location with the "bulls-eye" concentric circles of

equal altitude. You may stop/pause the display by pressing Function

Key F6, then move the satellite forward or backward in time using the

"+" and "-" keys and adjust the time step (1, 10, or 60 seconds) with

Function Key F4. Press ENTER to resume normal (simulated time)

operation.

4.   STSPLUS lists Line-of-Sight passes, those passes where the satellite

rises above the local true horizon, and all dates and times are given

in Coordinated Universal Time (UTC). Remember that under most

circumstances a "visible pass" means that the satellite is in full

sunlight and the viewer is in darkness. Although there are exceptions

in unusual situations, this generally restricts the times for visible

passes to the several hours prior to dawn and the several hours after

sunset. (Note, however, that "visible" to a ham radio operator or

radar tracking station simply means above the horizon!) In most cases,

the Space Shuttle and satellites such as MIR Space Station and Hubble

Space Telescope are visible with the naked eye given favorable

lighting and weather conditions. Satellites in higher altitude orbits

will be visible sooner before dawn and longer after sunset. Satellites

in very high orbits, no matter how large the satellite, are seldom

visible without high power binoculars or a telescope.

5.   The geometry of the pass and the attitude and geometry of the

spacecraft are also important. The relative angles between the Sun,

the satellite, and the viewer determine how light is reflected from

the surfaces of the spacecraft to you, the viewer. A spacecraft

passing between you and the Sun may not reflect much light to you and

may therefore not be visible even at higher altitudes. On the other

hand, a spacecraft nearer the horizon but on the other side of you

from the Sun may appear brilliantly lighted. The kinds of surfaces on

the spacecraft are important too; solar panels and flat surfaces can

reflect enough light to appear the most brilliant objects in the sky

while larger but rounded spacecraft may seem all but invisible.

6.   Given otherwise good conditions and favorable weather, the single most

important factor is spacecraft apparent altitude ("Alt") during a

pass. This is the spacecraft's apparent elevation above your local

horizion. Depending upon local conditions, an altitude of at least 5

to 10 degrees will generally be necessary before a spacecraft can be

seen even under the best lighting conditions. In the Los Angeles area,

at least 20 or 30 degrees is a better number to use because of smog,

haze and light polution (except when looking out over the Pacific

Ocean).

7.   After altitude, the azimuth ("Azm") is the number which describes the

direction from the viewer to the spacecraft at any moment. This is

given in the sense NESW, North to East to South to West, in degrees.

For a good pass after sunset, for example, an azimuth ranging from 60

to 150 degrees would indicate a pass moving generally from the

Northeast to the Southeast, ideal lighting conditions with the Sun in

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 23

the West.

8.   Remember that STSPLUS automatically sets SIMULATED TIME when using the

pass prediction feature to display passes. If simulated time is

already in effect, pass predictions start from the current simulated

time and a new simulated time is automatically set for a selected

pass. Once set, simulated time remains in effect until changed by

selecting another pass or until reset with F8 from the Main Menu. To

return to "real time", press F8+F1 from the Main Menu. The Main Menu

displays the current time with the time mode in effect labeled as

"Current Time" or "Simulated Time".

Predicting Satellite Passes with TRAKSTAR

-----------------------------------------

In order to generate detailed tabular predictions for satellite

passes, an external program is required. I recommend two programs for this

purpose: Paul Traufler's TRAKSAT and TS Kelso's TRAKSTAR. Each programs is

copyrighted by the respective author and is readily available. They are

both fine programs and set a standard against which other satellite

tracking programs are measured for performance and accuracy.

I have selected TRAKSTAR as the default external program used with

STSPLUS for two reasons: first, the program quickly produces very accurate

tabular data without graphics; and second, TRAKSTAR requires minimum memory

and is easily configured for seamless operation with STSPLUS.

TRAKSAT is a very large program with many features, and most computers

will not have sufficient memory to execute TRAKSAT when STSPLUS "shells" to

DOS. In order to run TRAKSAT, users must first exit STSPLUS (press "ESC"

from the Main Menu).

For additional information on these programs, contact the authors:

TRAKSTAR:      Dr. T. S. Kelso

2340 Raider Drive

Fairborn, OH  45324-2001  USA

BBS: Celestial BBS

(513) 427-0674

300/1200/2400/4800/9600 baud

TRAKSAT:       Paul Traufler

111 Emerald Drive

Harvest, AL  35749  USA

BBS: RPV Astronomy BBS

(310) 541-7299

2400/9600/14400 baud

RPV Hotline BBS

(310) 544-8977

1200/2400 baud

As a courtesy to the author, I suggest enclosing a stamped, self-addressed

envelope if you write and request a reply. The current version of each

program is always posted on the indicated BBS and messages may be left

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 24

there for the author.

There is no "standard" filetype used for 2-line elements files;

typical filetypes in regular use are ".TXT", ".TLE", ".N2L", and ".ELE";

some files also include comment lines, multi-line commentary, or additional

data, some or all of which must be removed prior to use with most satellite

tracking programs. As released by Dr. Kelso, TRAKSTAR expects a filetype of

".TLE" and cannot accept any other filetype without being re-compiled.

STSPLUS defaults to both filetype ".TXT" and ".TLE". TRAKSAT defaults to

filetype ".TXT" only. However, 2-line elements files, even on Dr. Kelso's

Celestial BBS, may use either ".TLE" or ".TXT", depending upon the file.

For the past four or five years, Paul Traufler and I have been jointly

releasing file TLEnnn.TXT (usually as file TLEnnn.ZIP, where "nnn" is a

number like "143"). Until February, 1993, the file was named NASAnnn.TXT.

This file is a sorted concantenation of files TLE.TXT and GROUP000.TLE from

Celestial BBS and currently includes 2-line elements for some 700+

satellites.

STSPLUS solves the filetype problem with TRAKSTAR by dynamically

creating a file called STSPLUS.TLE which contains the 2-line elements for

the currently selected satellite. Since STSPLUS can select 2-line elements

from a file of any size, calling TRAKSTAR through STSPLUS also circumvents

a minor problem in the current version of TRAKSTAR which limits the number

of satellites in a 2-line elements file to 250 element sets. As a

convenience, STSPLUS also dynamically creates the files STSPLUS.OBS (which

contains the name, coordinates, and elevation of the current user location)

and TRAKSTAR.CFG (which contains the drive and path information for

TRAKSTAR).

An accurate elevation (above mean sea level) for the user location is

required for accurate calculations in any satellite tracking program,

including STSPLUS, TRAKSTAR, and TRAKSAT. Note that the elevations of most

locations in file STSPLUS.CTY are not readily available and have been set

to zero. The elevation is the last parameter on each line in file

STSPLUS.CTY and is given in integer meters; 1 meter equals 3.28083 feet.

STSPLUS is coded to operate with TRAKSTAR Version 2.15. It may or may

not operate correctly with other versions. To set up program TRAKSTAR

either for independent use or for use with STSPLUS, follow the following

steps:

1.   Copy the TRAKSTAR files to your hard disk. Unpack the files if they

are contained in an archive file such as TRAKSTR2.ZIP. I recommend

using a separate directory called "TRAKSTAR". The complete TRAKSTAR

package includes documentation, Pascal source, example and test files,

and the program itself. The following two files are required for

operation with STSPLUS:

TRAKSTAR.EXE        Main Program

TRAKSTAR.HDR        Header File

Additional files are required for independent operation. Read the

TRAKSTAR documentation for details.

2.   Run STSPLUS and enter the drive and path information for TRAKSTAR by

pressing F7+F5 (Set FILENAMES and PATHS) from the Main Menu. STSPLUS

will automatically default to its own drive and directory if you omit

this step.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 25

3.   Select the desired 2-line elements file and satellite by pressing F2

from the Main Menu.

4.   When the map is on the screen and you have verified that the correct

satellite is being tracked, press ENTER to return to the Main Menu.

5.   Now press F4 from the Main Menu to run TRAKSTAR. You should

immediately see TRAKSTAR's opening screen and the first selection.

Make the various selections by using the up and down arrow keys to

move between selections and press ENTER when you have the correct

selection. Don't forget to press SPACE to select the satellite!

*************

*  CAUTION  *

*************

TRAKSTAR Version 2.15 uses the DOS clock to determine the default

start and stop times for its calculations. Not mentioned in the

TRAKSTAR documentation, however, is the fact that TRAKSTAR assumes

that the computer is set to Coordinated Universal Time (UTC). Be sure

to take that difference, including the date, into account when

entering start and stop times!

6.   TRAKSTAR will now make its calculations and write the results to a

file. The file is written in the TRAKSTAR directory and will overwrite

an existing file of the same name. The time required will be a

function of the time span and time interval requested as well as the

calculation speed of your computer. Read the TRAKSTAR documentation

carefully so that you will recognize the name of the file that

TRAKSTAR writes. For example, requesting visible passes for the Hubble

Space Telescope (NORAD #20830) will result in a filename of "VOB20830"

and a filetype which is the last three digits of the element set

number (such as ".866").

7.   As soon as TRAKSTAR finishes, you will return to STSPLUS's Main Menu

and may continue normal operations.

8.   To examine the tabular data produced by TRAKSTAR, you must either

"shell to DOS" using F9 from the Main Menu or exit STSPLUS by pressing

ESC at the Main Menu. Use an ASCII editor to view the file or send it

to your printer for hard copy.

If STSPLUS cannot find TRAKSTAR.EXE (or TRAKSTAR.BAT, see below) in the

selected directory, an error message will be displayed. Press ENTER to

return to the Main Menu.

An alternative method is to create a file TRAKSTAR.BAT in which you

place all commands required to run TRAKSTAR or the satellite tracking

program of your choice. STSPLUS.TLE and TRAKSTAR.CFG will still be written

to the selected directory but they need not be used. This method is only

recommended for individuals who understand the use and operation of DOS

batch files.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 26

Printing Graphics Screens

-------------------------

Many users have requested that I add a "print" function to STSPLUS.

Given the number and variety of printers available for use with these

systems and the fact that I only have a couple of printers I can use for

testing, this is not practical within STSPLUS. However, DOS includes the

GRAPHICS command which may be used with many computers to enable printing

of graphics images. Check your DOS and printer manuals for details. As an

example, I use the following command on systems equipped with a Hewlett-

Packard LaserJet II or III:

GRAPHICS LASERJETII

There are also quite a number of screen capture and print screen

programs, both commercial and shareware, which can perform this task. Note,

however, that all these programs are TSR's (Terminate and Stay Resident)

and some could interfere with STSPLUS's operation.

Some print screen programs, inculding DOS's built-in GRAPHICS command,

do not render colors very well. Certain colors may not be visible on the

printed copy at all. For such programs, use the "/M" command line option to

force monochrome operation when you wish to print graphics images from the

screen.

Known STSPLUS Problems and Bugs

-------------------------------

STSPLUS is being used on thousands of computers around the world

without any significant problems. However, like almost any computer program

and in spite of my best efforts, there are several known problems or "bugs"

with STSPLUS. Some are the result of slow computers, others are in the

program itself. Hopefully, some or all of these problems, those I can

reproduce at least, will be repaired in due course. Some problems are

caused by other software interfering with the program's operation. Still

other problems are the result of incompatible "IBM-compatible" computers

for which there is no remedy.

One "problem" which I frequently hear about is that the user's CGA

color monitor only displays STSPLUS in monochrome. THIS IS NOT A BUG! CGA

systems display "high resolution" 640 x 200 graphics in monochrome ONLY.

Compared to the EGA or VGA, that resolution is barely acceptable. The color

graphics mode for the CGA is 320 x 200 which is inadequate for STSPLUS.

1.   STSORBIT PLUS has been run extensively on systems using Microsoft DOS

3.3 and 5.0 and there are no known problems with those operating

systems EXCEPT the STACKS problem with DOS 5.0 (see the section

Program Setup and Usage Notes above). Because of the many bugs

reported, I do NOT recommend use of DOS 4.xx under any circumstances;

upgrade ASAP to DOS 5.0! Users report memory allocation problems with

some earlier versions of Digital Research DRDOS 6.0 and certain third

party memory allocation programs. A typical symptom of this kind of

problem is that you cannot return to DOS without either an error

message or the computer freezing.

2.   Note that not all computers (especially older CGA systems) will

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 27

display the extended graphics characters used for the large clock

characters (selected with F2 when the map is displayed). The symptom

of this problem is that the lower left portion of the data block is

mostly blank after pressing F2. If you have this problem and your

computer is running DOS 3.x or DOS 5.0, enter the command "GRAFTABL"

at the DOS prompt before running STSPLUS or include the line

"GRAFTABL" in your AUTOEXEC.BAT file; this sets the "code page" to

enable the computer to display the extended graphics characters. [The

program GRAFTABL.COM is included as part of DOS in most cases.]

3.   Some users report problems with certain Terminate and Stay Resident

(TSR) programs for which the only remedy is to remove the offending

TSR. This usually requires a "trial and error" approach to pinpoint

the TSR causing the problem. The best method is to remove ALL such

programs from your AUTOEXEC.BAT file, including the DOS 5 "DOSSHELL",

to make sure the program will work with your computer. Similarly,

delete all special memory and device drivers from your CONFIG.SYS

file. For DOS 5.0, add the line "STACKS=9,256" to your CONFIG.SYS

file. In some cases it may be necessary to increase the number 256 to

384 or 512. One user reported a problem on an IBM PS/2 when a mouse

driver was used (but I regularly execute STSPLUS on different

computers with my mouse active!).

4.   Several users have reported occasional "BASIC Error 11" problems;

BASIC reports this error as "Division by Zero". The only way I have

been able to reproduce this error is to restart the program using the

command line "STSPLUS /R" after I have deleted the 2-line file used on

the prior run; to avoid this error, do not use the "/R" command line

option and select a new 2-line file using F2. This error may also be

the result of a memory limitation (with multitasking enabled?) or it

may be a BIOS problem associated with a specific computer.

5.   All VGA adapter cards are not equal, in case any of you had some

illusions left. In at least one case, the aspect ratio of the display

is incorrect when the display is operated in the EGA simulation mode.

The vertical scale is compressed by about 20% as compared to either a

true EGA display or other (correct) VGA displays. So far as I know,

there is no remedy. Early VGA cards (the 449 card from Zenith is an

example) are not always recognized as VGA; the card is not register

compatible with the IBM standard and is recognized as EGA instead.

6.   There appears to be a subtle problem when changing to or from Daylight

Savings Time (which recently happened and "announced" the bug). The

display appears to get caught in a loop, endlessly redrawing the

screen or shows the wrong time and/or time zone. To avoid the problem,

change the Daylight Flag then EXIT THE PROGRAM AND RESTART.

7.   Not all computers (especially older CGA systems) will display the

extended graphics characters used for the large clock characters. The

symptom of this problem is that the lower left portion of the data

block is mostly blank after pressing F2. If you have this problem and

your computer is running DOS 3.x or DOS 5.0, enter the command

"GRAFTABL" at the DOS prompt before running STSPLUS or include the

line "GRAFTABL" in your AUTOEXEC.BAT file; this sets the "code page"

to enable the computer to display the extended graphics characters.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 28

[The program GRAFTABL.COM is included as part of DOS in most cases.]

8.   Some math coprocessor chips have caused problems. In at least two

reported cases, early version 386SX math coprocessor chips from IIT

did not execute correctly and generated random errors; ITT replaced

the suspect chips and the problems disappeared. On a 40MHz 386SX

system, a USLI 387SX chip did not operate correctly; a Cyrix 387SX

chip cured the problem. No problems have been reported for Intel or

Cyrix chips.

9.   Finally, as noted elsewhere, all computers are NOT equal. There are

some computers which will not execute STSPLUS under any circumstances.

Tandy is the most common offender followed by Leading Edge. Some

models from these manufacturers have BIOS problems or errors which

prevent programs compiled with the Microsoft BASIC compilers from

operating (sometimes only in graphics modes). There is no remedy.

Other computers, Ergo for example, exhibit "strange" behavior in some

graphics and text modes. There is sometimes an update or workaround

for these problems; check with the manufacturer.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 29

Preparing 2-Line Elements using VEC2TLE by Ken Ernandes

-------------------------------------------------------

Especially for Space Shuttle missions, Earth-centered inertial ("ECI")

cartesian state vectors may be the only orbital information available in

near real time. Such a state vector, consisting of position and velocity

data at a specified time, is sufficient to determine the instantaneous

orbit of a satellite. NASA and other agencies may provide state vectors

referenced to the mean equator and equinox of the Besselian year 1950

("M50", "Mean of 1950", or "B1950") with units of measure in feet and feet

per second. Data may also be available for the true equator and equinox of

date (such as the state vectors generated by STSPLUS), the mean equator and

equinox of the Julian year 2000 ("J2000"), or in the time-independent

Earth-fixed Greenwich ("EFG") coordinate systems. Kilometers or nautical

miles may also be used as the units of measure.

However, STSPLUS and most other satellite tracking programs require

orbital data in the "2-Line Elements" or "TLE" format and state vectors

must be converted to that format before the data may be used. The 2-Line

format originated as 2-Card Elements back in the days of IBM punched cards

at NORAD (North American Aerospace Defense Command, now US Space Command),

and has become the de facto standard format for orbital data used with

satellite tracking software.

Mr. Kenneth J. Ernandes has written program VEC2TLE, Vector to Two

Line Elements, specifically to convert state vectors to the 2-line format.

VEC2TLE is copyrighted software distributed as shareware, and registration

is required prior to regular use. Mr. Ernandes has extensive experience in

orbital mechanics with US Space Command (formerly NORAD) and in industry,

and has used his expertise to create a precision conversion program. For

additional information and registration details, write:

Mr. Kenneth J. Ernandes

16 Freshman Lane

Stony Brook, NY  11790-2712

CompuServe:  70511,3107

Internet:    70511.3107@cis.com

When writing Mr. Ernandes for information, I suggest including a stamped

self-addressed envelope as a courtesy. The current version of VEC2TLE is

usually posted on the RPV ASTRONNOMY BBS and on the CompuServe Astronomy

and Space Forum.

Note that although 2-line elements can be generated using only the

data in an ECI state vector and these elements will yield an accurate

position at the specified time, the "epoch" of the data, additional data

(in particular, Drag and B-Star parameters) are required to generate 2-line

elements which propagate accurately over time. Certain additional

parameters, such as element set number and orbit number, do NOT affect the

accuracy of the propagated position; these data may be obtained from other

sources or default values may be used. US Space Command assigns a Catalog

Number, often referred to as the "NORAD Number", some time after launch;

pre-launch elements for Space Shuttle missions may have a temporary Catalog

Number (corresponding to the mission number) until the actual Catalog

Number is assigned. The IAU Designation is assigned by the International

Astronomical Union at the time a launch is registered by the launching

country and may be blank. Element set ("ElSet") numbers are assigned

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 30

arbitrarily by the originating individual or agency and have no effect on

the orbital data. Orbit (or revolution or "REV") numbers are incremented on

each revolution at the ascending node, the point at which a orbit crosses

the equator heading North. Note that US Space Command does not usually use

the same reference for orbit numbers as does NASA; NASA defines the first

partial orbit as "Rev 1" whereas US Space Command may call that "Rev 0" or

some other arbitrary number. At least for Space Shuttle missions, it is

common practice to adjust USSPACECOM orbit numbers to conform to the NASA

convention.

VEC2TLE accepts all data required to form a complete 2-line orbital

element set, either as manually entered data or from a properly formatted

file, performs limited error checking on these data, then displays and

writes the generated 2-line elements file. The initial public release of

VEC2TLE, Version 9319, supports STSPLUS Data Mode 5 through 7 state vector

formats. The program also supports a variety of coordinate systems, units

of measure, and time formats.

Care must be taken when using VEC2TLE that the proper units of measure

(kilometers, feet, or nautical miles), coordinate system (ECI or EFG), and

epoch (True of Date, Mean of 1950, etc.) are used. See the VEC2TLE

documentation for additional information on the available options. STSPLUS

generates ECI X-Y-Z state vectors for the true equator and equinox of date

and may use any of three units of measure. NASA, on the other hand,

generates their state vectors for the mean equinox and equator of 1950

("M50") and uses feet and feet/second units of measure. Obviously, using

the wrong units of measure or coordinate system will yield invalid results!

VEC2TLE has been validated and tested in conjunction with STSPLUS

using NASA ECI state vectors (provided courtesy Willie Musty, Mission

Support, Rockwell International, Downey, CA) during Space Shuttle missions

STS-56 and STS-55 in early 1993. The resulting 2-line elements yielded

orbiter positions which corresponded exactly with those shown live on NASA

Select TV, and the 2-line elements were in close agreement with 2-line

elements subsequently released by US Space Command for a comparable epoch.

In fact, during mission STS-56, Rockwell used 2-line elements at their

Mission Control Center which I generated using VEC2TLE when USSPACECOM and

NASA 2-line elements were not forthcoming in a timely manner.

In addition to simply converting state vectors to 2-line elements,

the primary purpose of the program, VEC2TLE may be used in conjunction with

STSPLUS or other sources of state vectors to model orbit adjust burns and

similar maneuvers. STSPLUS is used to generate a state vector at the

midpoint of the burn, the appropriate delta velocities (obtained

independently) are added to the state vector quantities Xdot, Ydot, and

Zdot, then new, post-burn 2-line elements are generated with VEC2TLE. More

complex maneuvers may also be calculated or modeled although these more

complicated exercises are not recommended for the novice. VEC2TLE may also

be used to precess a set of 2-line elements to the prior Ascending Node,

required by some software. See the VEC2TLE documentation for additional

discussion and references.

My thanks to Ken Ernandes for writing VEC2TLE, for making it available

to the satellite tracking community, and for his assistance in validating

the precision state vector output data from STSPLUS. Thanks also to Willie

Musty (for providing state vectors) and to Joel Runes (for validation and

testing). As with any complex program, considerable effort has been

expended in writing, testing, and documenting the program. If you use

VEC2TLE, please register your copy so as to encourage Ken and others to

continue writing such useful software.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 31

PROGRAM OPERATION

-----------------

STSPLUS automatically checks for the presence of a VGA or EGA and will

execute in color if one is found UNLESS the /M command line option is used

to force monochrome operation. However, if you wish to operate STSPLUS in

the EGA mode when you have a VGA monitor, you must use the /EGA option. In

cases where a monochrome monitor is connected to an adapter which simulates

color with gray scale, the /M command line option may be omitted but the

various portions of the display may or may not be visible. STSPLUS depends

upon the Microsoft BASIC Compiler to determine whether or not a particular

monitor type is available. Some video adapter boards which claim to be VGA

are not recognized as such by BASIC and therefore cannot be used in the

higher resolution VGA display mode. Similarly, "clone" Hercules Graphics

Cards do not always perform correctly.

The /M option is not required for HGC and CGA graphics operation,

since those adapters always render their "high resolution" graphics in

monochrome. Although color CGA systems do have a 3-color mode, the limited

number of colors and coarse resolution of 320x200 is not suitable for

STSPLUS. Naturally, the appearance of the program is enhanced by the use of

color. The vertical resolution is also adjusted depending upon the type of

adapter which has been detected. Microsoft does not support SVGA adapter

cards, unfortunately. Once STSPLUS has started, the display type may not be

changed without exiting the program at the Main Menu with the ESC key, then

restarting the program with the desired command line options.

The program checks for the presence of a math coprocessor and will use

it if found. Since the calculations required to determine orbital data are

very complex, the use of a math coprocessor will improve the speed of

operation by a very substantial amount.

STSPLUS selects the icon or symbol used to graphically represent the

satellite based upon the satellite name. Names which start with the letters

"STS" will use an icon resembling a plan view of the space shuttle and all

other missions will use an icon resembling the Hubble Space Telescope.

STSPLUS reads the map coordinates from the appropriate map database

files. These coordinates are converted to screen coordinates for the type

of monitor detected, the current projection method, and the current

magnification or zoom factor.

Once the initial "housekeeping" chores have been performed, the Title

Screen is displayed for 15 seconds for normal program operation or for 3

seconds if the /R command line option has been used. The Main Menu,

described in a subsequent section, is then displayed. Press ENTER to

proceed to the Main Menu immediately.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 32

Program STSORBIT PLUS

Space Shuttle and Satellite Orbit Simulation

Version 9338

RighTime Version 2.53 detected!

Current time:  15:02:17 PST   23:02:17 UTC

Current date:    02/28/1993     02/28/1993

Last Mission = Hubble Space Telescope

2-Line Elements File = F:\TLE\TLE147.TXT

CPU Type = 80386DX or 80387SX

NDP Type = 80387DX or 80387SX

Display = VGA Color

(C) Copyright David H. Ransom, Jr., 1989-1993

All rights reserved.

The Title Screen displays the program version, current time and date,

last mission, and 2-line elements path and filename. It also displays the

type of Central Processing Unit (CPU) and Numeric Data Processor (NDP, or

math coprocessor), and the type of display. The program detects 8088,

80286, 80386, and 80486 processors and the associated math coprocessor; if

the math coprocessor is not present or fails a simple test, it will show as

"(not installed)". The program detects CGA, HGC, EGA and VGA display

systems; unless the "/M" command line option is used to force monochrome

operation, EGA and VGA systems will always indicate "Color".

STSPLUS is now "aware" of program RighTime by Tom Becker. If RighTime

is active, the current version number will be displayed; if RighTime is not

active or is not detected, no message will be displayed. STSPLUS is

configured to use RighTime Version 2.5+; performance with prior versions

may be unpredictable and audible alarms should NOT be enabled in that case.

See the section "Accurate Time and the Personal Computer" for a further

discussion of RighTime and other aspects of maintaining accurate DOS time.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 33

STSORBIT PLUS SATELLITE TRACKING FEATURES

-----------------------------------------

The principal objective of STSPLUS is to graphically display the

position of the space shuttle or satellite relative to a map of the world

or some relevant portion of the world along with relevant time and

numerical data. Two map projections and six different map displays are

available: Orthographic, World, Quadrant, Zoom, Location, Tracking Station,

and Satellite Motion. Varying magnifications or zoom factors are available

in most map modes. Each is discussed below.

In addition to the map itself, a number of other items of interest are

displayed. Some features are available only with higher resolution displays

(EGA and VGA) in order to avoid cluttering the display screen. Other

features may be enabled or disabled according to the user's preference. The

sections which follow the map types discuss these various features.

Orthographic Projection Maps

----------------------------

The orthographic projection views the Earth as a sphere as if from a

great distance (perspective is not included) and is the latest addition to

the map projections available in STSPLUS. This projection has the advantage

that the map may be centered at any point on the Earth and may include a

pole, especially helpful for high inclination satellite orbits. Unlike the

world map shown with rectangular projection, only one hemisphere can be

seen at a given time and therefore automatic map redrawing is always

enabled. Because of the more complex calculations required to generate a

map, users with slower computers may find that drawing times in the

orthographic modes are unacceptably long. (A math coprocessor will improve

map drawing times by almost a factor of ten!)

Orbital ground tracks, especially for high inclination orbits, and the

solar terminator are more readily understood using this projection. The

orthographic projection displays circles of visibility as true circles near

the center of the map.

The default magnification for orthographic maps is 100% which displays

the entire globe as a hemisphere. Using the PgUp and PgDn keys, the

magnification may be selected from 100% to 4000% (2000% if Level 1 maps are

not present). Each time the map is drawn, the center of the map is selected

so that the satellite will remain on the map for the longest time

practical. When high magnification factors are selected and the computer is

not equipped with a math coprocessor, it is possible that the map drawing

time will exceed the time the satellite is in view; this will cause the map

to be immediately redrawn. The current map database file and map drawing

time are shown near the bottom of the data block (e.g. "EARTH4   10.91").

Because only one hemisphere is shown (or a portion of a hemisphere

when magnification factors greater than 100% are used), automatic map

generation is ALWAYS enabled in orthographic modes. In addition, even if

the satellite never leaves the current map (as is the case with

geosynchronous satellites), the map will be redrawn every 2.5 hours. Users

without a math coprocessor may find that map drawing times in orthographic

modes are painfully slow. The only remedy is to purchase a new, more

powerful computer or to add a math coprocessor chip; this will improve

performance by about a factor of ten and the math coprocessor chips are now

relatively inexpensive, often under $100.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 34

The orthographic version of the Satellite Motion Map, enabled with the

"M" key when the map is displayed on the screen (EGA and VGA systems ONLY)

centers the satellite on the map and "moves" the map beneath the satellite

using EGA graphics. The next map is started in offscreen memory as soon as

a map is completed and displayed, then that map is displayed when completed

and the process is repeated. Especially for slower computers, this map mode

may be preferred since a map is always on the screen regardless of the

drawing time required.

As a footnote, the orthographic version of the Satellite Motion Map

can duplicate many of the views presented on NASA Select TV during a

mission. NASA frequently uses the equivalent of MAG=150 or MAG=200 for

their display. STSPLUS, however, can display far greater map detail than

can the NASA program, especially when the Level 3 and Level 2 map database

files are present.

Rectangular Projection World Maps

---------------------------------

The STSPLUS rectangular projection (similar to Mercator projection)

ground track display defaults to a map of the world centered on the Prime

Meridian (0 degrees) and extending from approximately +85 degrees North

latitude to -85 degrees South latitude using a linear cylindrical

projection. Omitting the two 5 degree bands at the poles permits better

detail in the mid latitudes where all space shuttle orbits and many other

satellite orbits are concentrated and avoids the extreme distortion

inherent in the rectangular projection near the poles. Ground track details

very near the poles are therefore sacrificed for a better display in the

main portion of typical orbits. The vertical resolution of the display is

automatically adjusted for the type of display system in use from 200 lines

(CGA) to 480 lines (VGA). Two World Maps are available: one centered on the

Prime Meridian at Greenwich, England (0 degrees longitude); and, one

centered on the International Date Line (180 degrees longitude).

All screen maps drawn by STSPLUS use a vector database derived from

the Micro World Database II. The full map of the world as used here

can include up to approximately 470,000 sets of vector coordinates

describing the world's coastlines, islands, lakes, and major rivers when

used with the highest detail (Level 1) map database.

Pressing the "W" or "0" key will switch the display to the World map.

If automatic map generation is enabled, STSPLUS will select the map which

most nearly centers the satellite on the map. If automatic map generation

is disabled, pressing "W" or "0" will toggle between the two maps. When

automatic map generation is enabled, the letter "A" appears in the upper

right of the screen.

Rectangular Projection Quadrant Maps

------------------------------------

The original STSORBIT program used a digitized pixel map of the world

derived from an EGA display. As a consequence of the EGA source, boundaries

were sometimes discontinuous on VGA displays and the display on a CGA was

sometimes difficult to read. STSPLUS uses a vector map drawing method which

automatically adjusts to the display type and the scale of the map. The full

world map (above) is quite similar in appearance to the original display.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 35

However, some geographic details are still difficult to distinguish,

even on a VGA display. STSPLUS includes twelve quadrant maps, each overing

1/4 of the Earth's surface and using rectangular projection. These are

numbered 1 to 12 and are centered on the world map roughly according to the

following illustrations:

0                                180

+---------------------------+     +---------------------------+

|             |             |     |             |             |

|    1        4        7    |     |    7        10       1    |

|             |             |     |             |             |

|             |             |     |             |             |

|             |             |     |             |             |

|    2        5        8    |     |    8        11       2    |

|             |             |     |             |             |

|             |             |     |             |             |

|             |             |     |             |             |

|    3        6        9    |     |    9        12       3    |

|             |             |     |             |             |

+---------------------------+     +---------------------------+

The center vertical quadrants in the left illustration, 4 through 6, are

centered on the Prime Meridian at zero degrees longitude (Greenwich,

England). The center vertical quadrants in the right illustration, 10

through 12, are centered on the International Date Line at 180/-180

degrees. The center horizontal quadrants, 2, 5, 8, and 11, are centered on

the Equator.

Each individual quadrant map may be selected by pressing the

corresponding number key, "1" through "9"; use keys "!", "@", and "#" to

select quadrants 10, 11, and 12 respectively. Pressing any of these keys

for individual quadrant maps will disable automatic map generation if it is

enabled (indicated by the letter "A" at the upper right of the screen).

Pressing "Q" will allow STSPLUS to select the quadrant most appropriate for

the satellite's current position.

Rectangular Projection Zoom Maps

--------------------------------

Although I was pleased with the enhanced maps using the Quadrant Mode,

the map data base files contain far more information than can be

effectively displayed in that mode. The next obvious step was to add the

ZOOM feature, maps which yielded greater detail and which spanned as little

as 30 degrees across the screen, six times better than the 180 degree

quadrant maps. This approaches the practical limit for the map database

files. Because of the smaller area covered, a different approach was used

for map selection. There would simply be too many different possibilities

for manual selection so a fully automatic Zoom Mode was implemented which

calculates the optimum map center point based upon the current position of

the satellite.

Press the "Z" key to enable Zoom Mode. The initial map width is 75

degrees; use PgUp to widen the map width (up to 180 degrees) or PgDn to

narrow the map width (down to 30 degrees. The Home key will always select

75 degrees width and the End key will return to the prior field of view.

The width of the map is shown at the upper left of the map display. Press

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 36

the TAB key to enable or disable automatic map generation (the map will

always be redrawn). When automatic map generation is enabled, the letter

"A" appears in the upper right corner of the screen next to the map width

or field of view. Zoom field of view selections are 30, 45, 60, 75, 90,

120, and 180 degrees.

Location Maps with Isocontours

------------------------------

By popular request, especially from the amateur radio community, I

have added the Location Map with Isocontours. (Isocontours is a term coined

by Rob Matson for his SkyMap program and for which he generously supplied

sample code which I adapted for STSPLUS.) Press the "L" key when the map

display is present to select this display. The map will be drawn with the

current magnification/zoom factor and centered on the user's location. The

usual circle of visibility will be drawn and within that "circle" are seven

isocontours representing viewing angles of 10 through 70 degrees in ten

degree increments (five degree increments at maximum magnification

factors). The projection, orthographic or rectangular, used for the

Location Map is the projection in use when the "L" key is pressed. The

balance of the Location Map includes the usual features.

If you have entered a second location (using F10+F2 from the Main

Menu), pressing the "L" key when the Location Map is already displayed will

toggle between your primary location and the second location. The data

related to your location (Location, Altitude, Azimuth, etc.) is calculated

with respect to the indicated location. If no second location has been

entered, pressing the "L" key while the Location Map is displayed will have

no effect.

The principal advantage of the Location Map is, of course, the

isocontours -- lines of equal viewing altitude (line of sight not taking

into account any refraction near the horizon) from the user's own location.

The user can immediately tell by inspection whether current or upcoming

passes will be "good" and what approximate maximum satellite viewing

altitude can be expected. Amateur radio buffs who need to know if a pass

will appear above some altitude threshold, say 20 degrees, now have that

information available visually.

Since this map mode uses the ZOOM map algorithms, the usual zoom map

features (PgUp, PgDn, Home, End) are active.

Tracking Station Maps with Isocontours

--------------------------------------

The Tracking Station Maps with Isocontours are similar to the Location

Maps except that they use the current TRACKING STATION file locations

rather than the user's location(s). This feature was implemented at the

request of folks working on the STS-49 Intelsat Reboost Mission. STSPLUS

was used operationally by INTELSAT during this mission at their Launch

Control Center near Washington, DC, and at their five ground tracking

stations around the world. I subsequently received a letter thanking me for

the use of the program and saying that it was "critical to mission

success".

Pressing the "T" key will select this map mode. STSPLUS calculates

which of the available tracking stations is nearest to the current

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 37

satellite position and centers that tracking station on the screen. This is

calculated by determining the angular difference between the sub-satellite

point and each tracking station. However, this means that depending upon

the Zoom factor in effect, the satellite may or may not be visible on the

screen. For example, if the sub-satellite point is in South America and the

only tracking station in the Western Hemisphere is in the United States,

the satellite cannot be seen at narrower fields of view.

The "tracking stations" may be any locations the user chooses and

includes in the current TRACKING STATION file. Several different tracking

station files accompany the normal STSPLUS distribution as described in the

section "NASA Ground Tracking Stations" below. Use Function Key F7 from the

Main Menu to select the desired file. STSPLUS contains an internal list of

NASA tracking stations which will be used if the current tracking station

file cannot be found.

Maps in this mode are displayed using current map projection. Since

this map mode uses the ZOOM map algorithms, the usual zoom map features

(PgUp, PgDn, Home, End) may be used to adjust the field of view (zoom or

magnification).

Location and Features Labels

----------------------------

Beginning with Version 9240, STSPLUS can add labels for locations and

features to all maps. Enable or disable location and features labels using

Function Keys F10+F3+F9 from the Main Menu. By default, STSPLUS expects the

geographic location and features data to be in file STSPLUS.LOC. If you

have created your own .LOC file or the file is not in the current

directory, use F7 from the Main Menu to specify an alternate file and/or

path. The supplied file includes 350+ locations (cities) and features

(oceans and seas), their coordinates, and certain information required by

the program. The file is standard ASCII "comma-delimited" data. The

following is a typical data line in the file:

"London",-.1167,51.5,7,100

---+--  ---+-- --+- + -+-

|       |     |  |  |

|       |     |  |  +---  Minimum magnification to display

|       |     |  |

|       |     |  +------  Color to display label (1 to 15)

|       |     |

|       |     +---------  Latitude in degrees

|       |

|       +---------------  Longitude in degrees

|

+-----------------------  Location or feature name

The label will be displayed if the current magnification is equal to or

greater than the minimum magnification value specified for that label. All

label names are converted to upper case for display; avoid the use of

punctuation other than the period ("."), dash ("-"), or comma (",").

The minimum magnification factors in file STSPLUS.LOC have been

carefully selected so that the display is not too "cluttered" at a given

magnification and so that location or feature names near to each other do

not usually overlap each other. Some care is required when adding new

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 38

locations to avoid this problem! If you wish to add your own locations

and/or features, use the supplied STSPLUS.LOC file as a guide and template

for your changes. Use a standard ASCII editor; word processor users must

use the "non-document" mode.

Magnifications are calculated automatically and ranges from 50% to

4000%, depending upon the map mode in effect. In orthographic map mode the

magnification factor is displayed as "MAG". In rectangular map modes, the

displayed ZOOM factor is the approximate field of view and may be converted

to magnification according to the following table:

ZOOM    MAG

-----------

360     50    World maps

180    100    Quadrant and Zoom maps

120    150    )

90    200    )

75    240    ) Zoom maps

60    300    )

45    400    )

30    600    )

The "color" is a number from 1 to 15 according to the following table:

Black   = 0       Gray      = 8

Blue    = 1       LtBlue    = 9

Green   = 2       LtGreen   = 10

Cyan    = 3       LtCyan    = 11

Red     = 4       LtRed     = 12

Magenta = 5       LtMagenta = 13

Brown   = 6       Yellow    = 14

White   = 7       LtWhite   = 15

The color Black is ignored and is only shown for completeness. If the color

is a NEGATIVE number, the small circle marking the location of the city or

feature will NOT be displayed and the label will be displayed centered on

the coordinates specified. Thus, if the a mountain range, lake, or river is

to be labeled, you may wish to set the color negative.

The latitude and longitude are expressed in degrees and decimal

fractions of a degree; West longitudes and South latitudes must be

negative. Note that many atlases show a number such as "24.45" which is

actually 24 degrees and 45 minutes (NOT 24.45 degrees!) and should be

entered as "24.75" (24 + 45/60). When using multi-line feature labels (see

file STSPLUS.LOC for examples), labels should be spaced approximately one

degree apart in latitude for display at a magnification of 100%, less for

higher magnitudes. All labels are automatically centered with respect to

longitude.

There is no restriction on the length of the location name. However,

avoid long names if possible so as to avoid location names displaying on

top of one another or "cluttering" the display.

IMPORTANT NOTE: The program performs no error checking on the contents of

file STSPLUS.LOC. Avoid the use of punctuation other than the period or

comma. Be sure to use a simple ASCII editor or use your word processor in

the "Non-Document" mode when editing file STSPLUS.LOC. Very strange results

can appear if the wrong data are present or the wrong number of items is in

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 39

a data line!

Big Clock Options

-----------------

In rectangular map projections, STSPLUS defaults to a standard display

with a text block shown on the lower five lines of the display; three

different times are shown at the lower left of this display: Launch/Epoch

date and time, UTC date and time, and local date and time. Pressing

Function Key F2 while the map is displayed switches between this default

mode and three Big Clock modes: UTC date and time, local date and time, and

MET/T+Epoch. Because of display space limitations, big clock options are

NOT available in orthographic projections.

For EGA and VGA users, an additional command line option, "/CLK", is

available which changes the number of lines per screen to 43 and 60 lines

respectively for rectangular projections. The big clocks are then placed

below the standard data instead of replacing a portion of the regular data

area.

In orthographic modes, VGA monitors ONLY, F2 will display the time at

the bottom of the data block at the right of the screen. Note that for all

magnification factors above 100%, the time is shortened to hours and

minutes because of display space limitations.

Satellite Motion Maps

---------------------

It is sometimes instructive and interesting to see the ground track

from the satellite point of view. The Satellite Motion Map, available

ONLY with EGA and VGA displays, centers the satellite in the display and

draws the map accordingly. Unlike all other map displays, this mode takes

advantage of the dual-page capability of the EGA display and the VGA

display (operating in EGA emulation mode); the current map is always

displayed and the new map is drawn "off screen" and updated as frequently

as the capability of the processor will permit. For VGA users, the vertical

map resolution in this mode is reduced from 400 lines to 280 lines

(rectangular projections) and from 480 lines to 350 lines (orthographic

projection).

Users with slow computers may prefer this display because, once the

map has been drawn the first time (however long that may take), a complete

map is always displayed. This is especially true at higher zoom or

magnification factors where the map is redrawn more frequently.

The Satellite Motion Map is enabled by pressing the "M" key when the

map is displayed. The Motion Map will be displayed using the same

projection as is presently in effect, rectangular (automatically switches

to zoom) or orthographic. When switching to this map mode, the message

Switching to EGA Dual-Page Mode ...

is displayed on the screen while the initial map is being drawn offscreen.

Thereafter, the map is drawn off-screen and will require the "usual" time

during which the screen will be unchanged. (My 386DX systems update every

10 seconds but my 286 systems can only manage every 20 or 30 seconds -- and

they all have math coprocessors!) The following keys are active when the

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 40

Satellite Motion Map is displayed:

Home      Zoom=75 (rect) or Mag=100 (ortho)

End       Return to last zoom/mag

PgDn      Decrease field of view (zoom in)

PgUp      Increase field of view (zoom out)

M         Return to normal map mode (rect or ortho)

ENTER     Return to Main Menu

Satellite Position and Orbit Projections

----------------------------------------

The focus of the display, and the reason for program STSPLUS, is to

show the position of the spacecraft or satellite. For the space shuttle

(and provided the mission name begins with the letters "STS"), a symbol has

been chosen which resembles that spacecraft. For all other satellites, a

symbol has been chosen which resembles the Hubble Space Telescope. In

either case, the symbol is shown in the following colors (EGA and VGA

displays only):

Satellite is sunlit           Bright White

Satellite is in penumbra      Yellow

Satellite is in umbra         Dim White

In addition, an asterisk ("*") is shown to the right of "Orbit #" when the

satellite is sunlit or in penumbra; this will assist users of monochrome

monitors where colors cannot be distinguished. For better visibility, the

satellite symbol will normally "blink" on CGA monitors; the symbol may be

made to blink on any system if desired by pressing the letter "B" while the

ground track is displayed. Note however that for very slow computers, the

blink may appear erratic if most of the time is spent performing

calculations.

Just seeing the spacecraft or satellite on the map display yields

information as to its present position. However, for satellite viewing and

planning purposes, STSPLUS calculates the predicted ground track for

approximately three hours in the future and the past ground track for

approximately one and a half hours in the past. The ground track may be

selected to display as a series of light green dots (future track) or light

red dots (past track), as a solid light green line, or as a solid light

green line with yellow dots(future track) and light red dots (past track).

The dots are plotted at one minute intervals. To select the desired ground

track display, press Function Key F10 and then press Function Key F5 until

the desired mode is displayed; the available selections are: OFF, DOTS,

LINE, and BOTH.

Note that for satellites in high Earth orbits, the ground track may

appear as a solid line if the dots are very close together. Unlike prior

versions of STSPLUS, the ground track is NOT updated (except to re-color

dots for past track). With automatic map generation off, the map will be

automatically redrawn every 2.5 hours.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 41

Satellite Visibility

--------------------

Satellite visibility, or the ability to see a satellite with the naked

eye (or binoculars for the truly dedicated), attracts the novice and expert

alike. It can be a great thrill to point out the Space Shuttle or MIR Space

Station to a child or a friend as it streaks across the sky where and when

predicted. Unfortunately, predicting that visibility is more complicated

than "simple" orbital mechanics and trigonometry; spacecraft altitude,

position, physical geometry, reflectivity, and attitude with respect to

both the viewer and the Sun, as well as local atmospheric clarity not to

mention weather, all contribute to whether or not a satellite may actually

be seen. Some of these factors are beyond the capabilities of a program

like STSPLUS. All that can be done is to indicate when conditions are such

that the satellite MIGHT be sighted visually. STSPLUS estimates viewer

visibility by calculating that:

a)   The satellite is at least partially lighted by the Sun;

b)   The Sun is 1.7 degrees or more below the viewer's horizon so that

the viewer is in at least partial darkness; and,

c)   The satellite is within the viewer's local circle of visibility

and is four degrees or more above the horizon.

If all these conditions are met, STSPLUS displays the word "VIS" in the

data block next to the orbit inclination. To these calculations must be

added the uncertainties described above. In my experience, the best

satellite sightings have usually occured when I am between the satellite

and the Sun, enabling flat surfaces such as solar panels to reflect the

sunlight back to me.

Although STSPLUS makes the visibility calculations for any satellite,

as a general rule only satellites in low Earth orbit, say under 1000 km

(600 miles) altitude, are likely to be seen with the naked eye. Binoculars

can extend that range somewhat. Satellites in very high or geosynchronous

orbits can be seen only with precision optical or radar equipment. Last but

not least, current 2-line elements must be used for reliable predictions.

**********

*  NOTE  *

**********

SATELLITE VISIBILITY IS ONLY ACTIVE WHEN THE SOLAR

FEATURES ARE ENABLED WITH F10+F3+F8.

User's Circle of Visibility

---------------------------

Centered around the user's geographic location, and marked with a

small circle on EGA and VGA systems, is a magenta circle of the approximate

line of sight visibility for the mission in progress. For rectangular

projections, the "circle" appears on the display as a circle near the

Equator and as a distorted circle at higher latitudes. In near-polar

regions, the circle takes on a very strange shape. The shape is entirely an

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 42

artifact of the map projection; when displayed using the orthographic

projection, it appear as a true circle near the center of the map. The

radius of this circle of visibility is calculated for each satellite based

upon its altitude at the instant the map is first drawn as well as the

user's elevation above mean sea level and corresponds to "line of sight"

visibility for that satellite. When a second location has been selected

using F10+F2, a second circle of visibility will also be drawn for that

location.

When audible alarms are enabled (F10+F3+F8 from the Main Menu), tones

sound 2 minutes before the satellite enters the circle of visibility and

again 30 seconds before the satellite leaves the circle of visibility. Both

the primary and secondary locations are monitored, with slightly different

tones for each. And "up-down" sequence of tones is used prior to entry into

the circles and a series of tones prior to leaving the circles.

When a satellite is within the circle, direct visual, radio or radar

communications with the satellite should be practical. Actual visibility,

of course, depends upon more than simply whether or not the satellite is

above the viewer's horizon. Most important is the sun to satellite to

viewer geometry; the satellite must be in sunlight and the viewer in

darkness for reasonable visibility. Almost as important is the size and

geometry of the satellite itself; a large, bright-metal satellite with huge

solar arrays reflects far more sunlight than a small dark satellite. In

addition to the satellites themselves, many booster rockets and other

"spare parts" are orbiting the Earth. Since they are not attitude

stabilized, booster rockets often are tumbling and may therefore appear

to flash on and off as they pass over.

For the Space Shuttle as well as most other satellites with near

circular orbits, the circle of visibility calculations are reasonably

accurate; however, the position of the sun and the effect of atmospheric

refraction are not taken into account, only whether or not the satellite is

in line of sight view from the observing location. For highly eliptical

orbits, however, the accuracy is substantially degraded since the radius of

the circle of visibility changes dramatically depending upon whether the

satellite is nearer apogee or perigee at the time the calculation is made,

and the period of the orbit. In the course of a single orbit, the altitude

of such a satellite may change by thousands of miles. Satellite "DE 1",

usually included in the TLEnnn.TXT 2-line elements file, is in a highly

elliptical orbit with long period and illustrates the problem.

Spacecraft Circle of Visibility

-------------------------------

The spacecraft circle of visibility is calculated dynamically using

the same alrorithm as for the user's circle of visibility. Like the user's

circle, the spacecraft circle may appear on rectangular projections as an

odd shape because of the scaling factors used by the map projection. The

difference is that the spacecraft circle moves with the spacecraft and

illustrates the approximate area visible from the spacecraft at any given

moment. The circle is updated every ten seconds (or as frequently as the

processor will permit). Comparisons using a VGA display system during the

STS-35/ASTRO-1 mission in December, 1990 confirmed that the circle shown is

quite close to that shown by one of the special graphics displays

occasionally seen on NASA Select Television as well as the actual horizon

view seen from the payload bay television cameras.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 43

SUN and Solar Features

----------------------

Many types of observations, especially Earth observations, often

require that the target or terrain be in sunlight. The performance of solar

panels and certain other instruments on a spacecraft is dependent upon

whether or not the Sun is in view. Also, it is usually impossible to

visually see a spacecraft which is not in sunlight. The solar terminator is

a series of yellow points on the display which represent the line at which

the center of the Sun is at an observer's horizon for Mean Sea Level.

Although a quick glance at the clock should suffice to determine which side

of the terminator line is in sunlight and which in darkness, EGA and VGA

systems also display the Sun as a small yellow circle.

The terminator as displayed by STSPLUS is sometimes confused with a

line denoting sunrise and sunset. Two factors make the terminator only an

approximation: first, the terminator is based upon the center of the Sun,

while actual sunrise and sunset are calculated using the upper limb of the

Sun; and second, the terminator is calculated for Mean Sea Level rather

than a specific local elevation. These two factors combined can result in a

difference of up to ten minutes when the times are compared against

published values or those calculated by my program ASTROCLK, sunrise being

earlier and sunset being later.

The Sun and solar features are enabled and disabled using F10+F3+F8

from the Main Menu. Since these features -- especially the terminator --

require some calculation and drawing time, users with slower computers may

wish to disable these features for faster screen updates. The following

features are included:

Sun:           A yellow circle is plotted at the sub-solar point, the

geographic coordinates directly beneath the current position

of the Sun. The position of the Sun is recalculated every 10

seconds and the display is updated every 60 seconds. (Not

shown on CGA and HGC displays.)

Terminator:    A dotted yellow line is plotted for the solar terminator,

that point on the Earth at which the center of the Sun is at

the horizon. The terminator is partially updated every 10

seconds and is fully updated every 60 seconds.The terminator

is shown for Mean Sea Level and does NOT take into account

the non-spherical shape of the Earth.

Lighting:      The current spacecraft lighting is shown using color for the

satellite icon, and in the data block (to the right of

"Orbit #:") and displayed using the following symbols and

colors:

*    Bright White   Full sunlight

+    Yellow         Partial sunlight (penumbra)

-    Light Red      Refracted sunlight

White          Full shadow (umbra), no symbol

Note that Yellow and Light Red will display as Bright White

or shades of gray on monochrome monitors.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 44

Solar lighting conditions are updated every second or as

rapidly as the speed of the processor will permit. In order

to minimize calculation delays during ground track plotting,

the event times for orbital sunrise and sunset are

approximated. The dynamic lighting calculations, used to

plot the color of the spacecraft icon, are more precise.

Typical errors due to the simplified algorithm, are on the

order of 10 seconds.

SUN Timer:     When Event Timers are enabled, the orbital sunrise (AOS) and

sunset (LOS) times are shown. A blank AOS or LOS timer

indicates the event will not occur within the next four

hours. An asterisk ("*") to the left of "SUN" indicates

sunlight is on the satellite:

AOS    LOS

*SUN  73:20  37:40

In this example, the satellite is in sunlight. Orbital

sunset will occur in 37:40 and the next orbital sunrise will

occur in 73:20.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 45

TDRS and Real Time Satellite Features

-------------------------------------

TDRS and Real Time Satellite features are available only on EGA and

VGA monitors. These features permit up to thirty two additional satellites

to be tracked in near real time. Satellites to be tracked are designated by

the user as "static" (geosynchronous or geostationary) and "real time"

(satellites whose sub-satellite point changes substantially with time).

Static satellites are only plotted when the map is redrawn; real time

satellites are plotted every 10 seconds (X1 and X10), or every 60 seconds

(X60), or as often as the processor can complete the required calculations.

When TDRS and Real Time Satellite coverage is enabled (F10+F3+F2), all

active TDRS satellites (Tracking and Data Relay Satellites), used for most

communications to and from the Space Shuttle, the Hubble Space Telescope,

and other active spacecraft, are shown as a dot inside a small circle near

the Equator (provided they have been included in the current TDRS and Real

Time Satellite configuration). Use F6 from the Main Menu to display and/or

modify that configuration. See the text section on Function Key F6 for a

complete description of Static and Real Time satellites.

As of August, 1993, there are five TDRS satellites in geosynchronous

orbit. The primary satellites consist of TDRS East ("TDRS 3") at

approximately 41 degrees West longitude and the TDRS West Cluster

consisting of two satellites, TDRS West ("TDRS 4") and TDRS Spare ("TDRS

1") at approximately 174 and 170 degrees West longitude respectively. The

TDRS Spare satellite has partially failed but is used occasionally as a

backup; this satellite is also low on propellant and is allowed to drift

considerably with an inclination of about 7 degrees. "TDRS 2" has also

partially failed and is currently parked at approximately 62 degrees West

longitude and is presently dedicated to downlinking data from GRO, the

Gamma Ray Observatory, whose tape recorders have failed. "TDRS 5" is the

most recent satellite launched, is fully operational, and is parked at

about 138 degrees West longitude as an on-orbit spare. Users should check

the current positions of the TDRS satellites since they are periodically

moved or reassigned.

Each TDRS location provides communications coverage for almost half

of the Earth for low Earth orbits and essentially full time coverage for

higher orbits. However, since the NASA Ground Terminals are located at

White Sands, New Mexico, the coverage has been slightly overlaped to

provide good ground communications at White Sands. This, in turn, means

that there is a narrow band, known by NASA as the Zone of Exclusion (marked

"ZOE" on the screen), off the East coast of Africa which is not covered by

either primary TDRS for low Earth orbits. Two red "circles" on the display

show the limits of coverage for each primary TDRS satellite. Each circle,

whose shape may be quite distorted when using rectangular projection,

encloses an area where the TDRS satellite is out of range of the primary

satellite being tracked and is centered on the opposite side of the Earth

from the TDRS satellite's position.

While the ground track is being calculated, STSPLUS also calculates

the times for acquisition of signal (AOS) and loss of signal (LOS). When

TDRS coverage is enabled, these times are displayed for TDRS East and TDRS

West. The method used for the calculation of the TDRS coverage is usually

accurate to about 10 seconds (assuming accurate 2-line elements for the

satellite and for the TDRS). However, spacecraft attitude can cause loss of

TDRS communications at unexpected times. When Event Timers are enabled,

STSPLUS calculates AOS and LOS for four hours (240 minutes) from the time

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 46

the map is drawn. If a time is beyond that limit (or if the condition does

not occur), the time is left blank.

Each time is presented in the form of a countdown clock, minutes and

seconds, until the next occurence of the condition:

*TDRE AOS/LOS  72:42  50:42     (for rectangular projections)

*TDRW AOS/LOS  45:42  22:42

AOS    LOS

*TDRE:  72:42  50:42     (for orthographic projections)

*TDRW:  45:42  22:42

An asterisk ("*") is shown to the left of the satellite name if AOS is in

effect. For rectangular projections, these data are displayed in the lower

right portion of the map. For EGA and VGA users, however, the data may be

displayed in the lower section of the data block by pressing F2 while the

map is displayed until MET/T+E is displayed. For orthographic projections,

the data are part of the standard data block at the right of the screen.

The clocks for each TDRS are color coded to indicate the current

status: GREEN when the satellite is in communication, and RED when the

satellite is out of range of the TDRS. Two minutes prior to a change in

status, the appropriate clock color changes to YELLOW. Users with

monochrome monitors must observe the presence or absence of the asterisk to

determine the status.

STSPLUS can sound an audible alarm (three beeps) 30 seconds prior to

TDRS AOS or LOS. Use F10+F3+F8 to enable or disable the audible alarms.

Most satellites which utilize the TDRS system for communications are

in low Earth orbits (generally below 1500 km). However, other satellites

may also use the TDRS system for regular or backup communications. The

NAVSTAR Global Positioning Satellites (GPS), with orbital altitudes of

about 11,000 nautical miles (20,000 kilometers), are an example. For such

high orbits, the coverage by each TDRS satellite is nearly continuous.

As noted above, there are currently five TDRS satellites in orbit as

of mid 1993, TDRS 1 through TDRS 5. (These numbers are those currently used

by US Space Command in their 2-line elements. NASA sometimes uses different

numbers corresponding to the original launch letters as shown in the

following chart. TDRS "B", which should have become "TDRS 2", was lost in

the Challenger accident.) As of July, 1993, the TDRS assignments are:

TDRS#  *   NORAD#     Long    Description

-----------------------------------------------------------------

TDRS 1 (A)   13969    -170W    TDRS West Spare, used occasionally

TDRS 2 (C)   19548     -62W    Dedicated to Gamma Ray Observatory

TDRS 3 (D)   19883     -41W    TDRS East (STSPLUS default)

TDRS 4 (E)   21639    -174W    TDRS West (STSPLUS default)

TDRS 5 (F)   22314    -138W    On-orbit spare

* Original NASA letter designation at launch

STSPLUS will use TDRS 3 and TDRS 4 by default and approximate positions as

of July, 1993 are automatically saved in file STSPLUS.INI. Note that some

2-line element files (including TLEnnn.TXT) often refer to the TDRS

satellites using numbers 1 through 5 or letters A through D or E. Use the

NORAD numbers to be certain that you display the correct satellite.

TDRS satellites do change position and/or assignment periodically for

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 47

one reason or another. Users who wish the most accurate TDRS positions and

AOS/LOS data should periodically update the default positions by using the

automatic update feature with F2 from the Main Menu to read and update the

2-line elements.

Ground Tracking Stations and .TRK files

---------------------------------------

NASA maintains a number of ground tracking stations around the world.

Some of these tracking stations are essential for the ascent or landing

phases of a space shuttle flight; others are used for in-flight

communications. File STSPLUS.TRK contains the information for these ground

tracking stations. Other files with filetype .TRK contain the information

for other launch and/or tracking stations.

Each ground tracking station is shown as a small symbol surrounded by

a brown or light yellow "circle of visibility" which gives the approximate

area of antenna coverage and shows how small a proportion of each orbit can

be monitored without the TDRS system. When for some reason the TDRS system

is down (as has occurred during infrequent computer failures at White

Sands, New Mexico, the TDRS Ground Station), these ground tracking stations

become the only means of communication with the Space Shuttle. (Another

unexpected method was demonstrated during a 1992 space shuttle mission when

the SAREX, Shuttle Amateur Radio EXperiment, was used during a complete

loss of normal communications!) Because of budget constraints, many of

these ground tracking stations may be (or have already been) shut down.

Some, such as MIL and BDA (see list below) will be retained because they

are required for the ascent phase of a space shuttle mission. Others, such

as HAW, CTS and GWM, are operated jointly with, or independently by, the

U.S. Air Force. NASA is not always consistent as to the abbreviations used

for these tracking stations; on NASA Select TV, Gwan, Hawaii, and

Vandenbert are usually shown as GTS, HTS, and VTS respectively.

When using rectangular map projections, the shape of the antenna range

"circle of visibility" varies as a function of the latitude and is an

artifact of the map projection; when projected on a sphere, as is the case

with orthographic projection, they are true circles. In order to avoid

cluttering the display with countless meaningless lines, tracking station

circles of visibility are shown only if that circle has an angular diameter

of 90 degrees or less.

The following table lists the internal ground tracking stations as of

early 1989 which are shown along with their abbreviations and approximate

map coordinates (longitude, latitude):

MIL     -81,28         Merritt Island, FL

BDA     -64,32         Bermuda

DKR     -17,14         Dakar, Senegal

ACN     -14,-8         Ascension Island

MAD     -5,41          Madrid, Spain

IOS      56,-5         Indian Ocean

HAW     -156,20        Hawaii

GWM      143.33,14     Guam

VAN     -120.57,34.73  Vandenberg, CA

YAR      115,-29       Yarragadee, Australia

CAN      149,-36       Canberra, Australia

GDS     -116.88,35.93  Goldstone, CA

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 48

CTS     -105,38        Colorado Springs, CO

AGO     -71,-34        Santiago, Chile

NGT     -106,33        White Sands, NM

STSPLUS first checks for the presence of file STSPLUS.TRK for its

ground tracking station information. This is the default TRACKING STATION

filename used if no other selection has been made using F7 from the Main

Menu. If that file is present, its data is used instead of the internal

data above. The supplied STSPLUS.TRK has the following format:

"Maui, Hawaii",-156.7,20.9,0,"HAW"

"Vandenberg, CA",-120.5667,34.7333,112,"VAN"

"White Sands, NM",-106,33,0,"NGT"

"Colorado Springs, CO",-105,38,0,"CTS"

"Merritt Island, FL",-81,28,0,"MIL"

"Santiago, Chile",-71,-34,0,"AGO"

"Bermuda",-64,32,0,"BDA"

"Dakar, Senegal",-17,14,0,"DKR"

"Ascension Island",-14,-8,0,"ACN"

"Madrid, Spain",-5,41,0,"MAD"

"Indian Ocean Stn",56,-5,0,"IOS"

"Yarragadee, Australia",115,-29,0,"YAR"

"Guam",143.3333,14,0,"GWM"

"Canberra, Australia",149,-36,0,"CAN"

These files are ASCII and may be prepared or edited with any standard

ASCII editor; if using a word processor, select the "non-document" mode.

Five items are required for each location; the longitude and latatude are

expressed in degrees and fraction of a degree, elevations above Mean Sea

Level are expressed in meters, and names or abbreviations are included in

double quotation marks. The following example illustrates the .TRK file

format:

"Merritt Island, FL",-81.0,28.0,0,"MIL"

--------+----------- --+-- --+- + --+--

|              |     |  |   |

|              |     |  |   +--- 3-Letter Abbreviation

|              |     |  +------- Elevation (meters)

|              |     +---------- Latitude (degrees)

|              +---------------- Longitude (degrees)

+------------------------------- Location Name

For those interested in the Russian space program, a list of Russian

ground tracking stations is provided in file CIS.TRK (data courtesy

Ellwood Marshall). With the breakup of the Soviet Union, some of these

installations may no longer be active or the name may have changed.

"Tyuratam Cosmodrome",63.3392,45.9235,0,"TYR"

"Kaliningrad Cntrl Ctr",37.816,55.916,0,"KAL"

"Plesetsk Cosmodrome",40.7,62.75,0,"PLS"

"Petropavlovsk Russia",158.933,53.216,0,"PTR"

"Tbilisi Georgia",44.75,41.66,0,"TBL"

"Ulan Ude Russia",107.683,51.983,0,"ULN"

"Ussuriysk Russia",132.15,43.8,0,"USS"

"Yevpatoria Ukraine",33.3666,45.2166,0,"YEV"

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 49

Other nations also have facilities for satellite launches. As of early

1993, file SPACENTR.TRK includes the following locations:

"Alcantara LC Brazil",-44.3999,-2.3999,0,"ALC"

"Esrange,Kiruna Sweden",21.067,67.883,0,"ESR"

"Jiuquan Space LC China",100.033,40.83,0,"JIU"

"Kagoshima Center Japan",131.083,31.25,0,"KAG"

"Kourou Space Ctr Fr.Gu",-52.7669,5.23,0,"KOU"

"San Marco Platform",40.2,-2.9329,0,"SMP"

"Sriharikota Ctr India",80.25,13.78,0,"SRI"

"Tanegashima SC Japan",130.967,30.4,0,"TAN"

"Xichang Space LC China",102.217,27.967,0,"XUC"

The first mission of Endeavour, STS-49, was in May, 1992. This

dramatic and exciting mission captured the INTELSAT VI (F3) satellite,

stranded in a useless orbit by its booster rocket failure since January,

1990, and attached a new booster rocket which placed the satellite in its

proper orbit. STSPLUS was used operationally during the mission by

Intelsat, another "first" for the program. Intelsat used its own ground

tracking stations for communications with INTELSAT VI (F3); the ground

stations which participated in the mission are listed in file INTELSAT.TRK

(information courtesy Dee Smith):

"Paumalu, Hawaii",-158.0342,21.6711,157.86,"PAU"

"Tangua, Brazil",-42.7845,-22.7442,35.38,"TAN"

"Jatiluhur, Indonesia",107,-6.5213,161.49,"JAT"

"Perth, Australia",115.25,-31.8,0,"PER"

"Gandoul, Senegal",-17.4745,14.43,0,"GAN"

These TRK files are standard ASCII files and may be edited with any

editor; word processor users be sure to use the ASCII or non-document mode.

The files use a standard comma-delimited format as shown; positions are

given in longitude (degrees) and latitude (degrees), rounded to the nearest

degree. A maximum of 25 ground stations is permitted. The use of TRK files

is not restricted to tracking stations, of course. So long as the correct

data format is observed, any desired location may be included in the

tracking station file up to the maximum of 25 locations.

Event Timers and Audible Alarms

-------------------------------

STSPLUS is often operated for long periods of time with minimum

operator attention or intervention. Users may perform other tasks while the

satellite display is active and while awaiting some subsequent event of

interest. Event timers are displayed for the selected events if they will

occur within approximately four hours of the time that the map is drawn; if

an event will not occur within that time, the event is blank. Audible

alarms serve to alert the user to upcoming selected events. The event

timers are enabled with F10+F7 from the Main Menu. Audible alarms are

enabled with F10+F8 from the Main Menu and require also that the event

timers be enabled. All events are termed "AOS" (Acquisition of Signal or

Source) or "LOS" (Loss of Signal or Source) and are generally calculated

for line of sight conditions.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 50

Each phenomena which may be timed has an associated AOS and LOS timer

which displays the hours and minutes ("HHH:MM") remaining until the next

event if that event will occur within the next four hours (240 minutes),

one hour past the last time for which the ground track is plotted. The

current status of the signal or source is indicated by the color of the

timer digits and the presence or absence of an asterisk ("*") to the left

of the event name: GREEN indicates signal or source acquisition, and RED

indicates signal or source loss. Two minutes prior to an event, the timer

for that event will change from the signal status color (GREEN or RED) to

YELLOW to visually alert the user. Users with monochrome monitors will be

unable to distinguish these color changes, of course, but can determine the

current signal status using the asterisk indicator.

All calculations for upcoming events are made each time the ground

track is drawn on the display and will affect the time required to prepare

the display, especially on slower computers. In order to minimize these

calculation delays, the event calculations for orbital sunrise and sunset

use a simplified algorithm which does not take into account the non-

spherical shape of the earth (unlike the dynamically calculated spacecraft

lighting features which are more accurate). Orbital sunrise and sunset are

the times that the spacecraft transitions between refracted sunlight

(sunlight refracted through the Earth's atmosphere) to partial sunlight

(illumination from only a portion of the solar disk); this corresponds to

the transitions between RED and YELLOW color on the satellite icon and

illumination symbols respectively. The errors resulting from the simplified

algorithm are usually less than plus or minus 10 seconds; because of the

more oblique angles and geometry involved, higher errors are usually

associated with higher inclination orbits.

When no secondary location is selected, the headings "AOS" and "LOS"

will appear in orthographic modes; the headings do not appear in

rectangular modes or in orthographic modes when a secondary location is

selected in order to make room for the additional line of data. Typical

Event Timers are shown in the following examples.

For orthographic projections:

*STN   95:15   6:21           AOS now in effect

AOS will next occur in 95:15

LOS will occur in 6:21

STN   23:47  45:18           LOS now in effect

AOS will occur in 23:47

LOS will next occur in 45:18

and similarly for rectangular projections:

*TDRW AOS/LOS  85:14  33:43   AOS now in effect

AOS will next occur in 85:14

LOS will occur in 33:43

TDRW AOS/LOS  14:21  57:32   LOS now in effect

AOS will occur in 14:21

LOS will next occur in 57:32

except the primary location AOS/LOS, which is unlabeled in rectangular

projections and appears at the upper right of the data block (immediately

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 51

to the right of MET/T+E):

* 89:39   1:27                AOS now in effect

AOS will next occur in 89:39

LOS will occur in 1:27

70:15  76:38                LOS now in effect

AOS will occur in 70:15

LOS will next occur in 76:38

The following events may be timed and will cause an audible alarm if

audible alarms are enabled and the appropriate events are enabled:

Local Visibility    For the primary location ("STN" or "STN1"): three

sets of up/down tones two minutes prior to AOS and

five tones thirty seconds prior to LOS. For the

secondary location ("STN2" if enabled): two sets

of up/down tones two minutes prior to AOS and four

tones thirty seconds prior to LOS. Refers to the

times the satellite enters or leaves the local

circle of visibility.

TDRS Acquisition    ("TDRE" or "TDRW" if enabled) Three short tones

thirty seconds prior to AOS or LOS. Refers to the

times the satellite acquires or loses the ability

to communicate with either of the programmed

Tracking and Data Relay Satellites.

Orbital Sunrise/set ("SUN" if enabled) Two tones thirty seconds prior

to approximate orbital sunrise or sunset. Refers

to actual line of sight solar contact; refracted

sunlight is not included.

The characteristics of the audible tones have been selected to allow the

user to uniquely identify the AOS or LOS event that is about to happen.

STSPLUS is now "aware" of program RighTime by Tom Becker and its use

is recommended for accurate timekeeping. Audible alarms in prior versions

would perform unpredictably when RighTime was active because they use the

hardware clock's timer functions (which RighTime also uses). STSPLUS now

detects RighTime and temporarily disables RighTime while an audible alarm

is being generated and then re-enables RighTime after the alarm has

completed, restoring precise timekeeping. With RighTime active, alarms are

generated in foreground, which may cause a slight delay in screen updating.

*************

*  CAUTION  *

*************

STSPLUS expects RighTime Version 2.5+; performance with prior

versions of RighTime may yield unpredictable results. If using a

prior version of RighTime, do NOT enable audible alarms!

If RighTime is not present or is not detected, the audible alarms are

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 52

generated in background as in prior versions. This usually causes the loss

of several clock ticks in the DOS software clock for each audible alarm.

Although the time loss per audible alarm is very small, the cumulative

error may become significant over extended time periods.

Pausing the Ground Track Display

--------------------------------

Pressing F6 will cause the ground track display to "freeze" at the

current time. This is called PAUSE mode. This permits closer examination of

the data and/or display at any given time and to "move" the display forward

and backward in time. Note that the pause takes effect AFTER the next

second tick on the system clock; thus, if you wish to pause at 01:00 (one

minute exactly on one of the clocks), press F5 when the display reads

00:59.

After pressing F6, the following message will appear near the lower

right of the screen:

PAUSE...Press ENTER

This reminds the user that PAUSE is in effect and to press ENTER to

resume normal operation. When normal operation is resumed, the time

continues from its present value, as if you had set SIMULATED TIME using

F8+F3 or F8+F4 from the Main Menu. To return to REAL TIME, use F8+F1 from

the Main Menu.

While PAUSE is in effect, the "+" and "-" keys may be used to advance

or retard the current time by the amount of the current time step. You may

also use the "=" key instead of the "+" key to avoid pressing the SHIFT

key. The only other key active in PAUSE mode is F4, which may be used

to adjust the "time step" by pressing the key until the desired time

multiplier is displayed at the upper right of the screen. Time multipliers

of "X1" (no message displayed), "X10", and "X60" are selected in

succession. The default time step is 1 second.

Note that the automatic map generation feature is also used in PAUSE

mode; automatic map generation may be enabled or disabled in the

rectangular projections (use the TAB key) and is ALWAYS enabled in

orthographic projections. Thus, when the satellite is moved near the edge

of the display, the map may be redrawn if the appropriate point is reached.

If you wish to synchronize the time used by STSPLUS to some other

source (such as the slightly delayed orthographic displays presented from

time to time on NASA Select TV), you may also use PAUSE mode for this

purpose. Simply pause the display, use the "+" or "-" keys to adjust the

time slightly ahead of the time to which you wish to synchronize. Then

press ENTER when the times agree. Time can only be synchronized in this

manner to plus or minus one second. If you need higher precision, set

SIMULATED TIME using F8+F3 or F8+F4 from the Main Menu.

Switching between MET and T+Epoch

---------------------------------

STSPLUS by default displays the time elapsed since the epoch date of

the elements in the upper right portion of the data block unless the launch

date and time are included in file STSPLUS.LTD, in which case Mission

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 53

Elapsed Time (MET) is the default. This marked on the display as "T+Epoch"

or "T+E" and "MET" respectively. While T+Epoch is not of particular value

for satellite viewing purposes, it does indicate the relative age of the

orbital data. As a general rule, especially for lower Earth orbits, the

effects of orbit decay make position predictions less accurate as time

passes. Data which are more than 10 or 20 days old may produce less

accurate positions.

For a space shuttle mission, however, all mission events are scheduled

against the mission timeline and are reckoned in Mission Elapsed Time

(MET), the time elapsed since launch. It is therefore useful to be able to

display MET during the course of a mission or to review the flight post-

mission. Unfortunately, the standard NASA/NORAD 2-line element format does

not include the launch time and launch date and therefore this information

must be secured independently and manually entered into STSPLUS. Once

entered, STSPLUS saves the information in file STPLUS.LTD.

Once the ground track map is displayed, the F5 command may be used to

switch the display between Time Since Epoch ("T+Epoch" or "T+E") and

Mission Elapsed Time ("MET").

The F5 command checks that you have already entered the launch time

and launch date or that it has been read from file STSPLUS.LTD. If no

launch time and date are present, the command will have no effect. Press

ENTER to return to the Main Menu and press F5 to enter launch time and

launch date.

Using FAST Time

---------------

Press F4 while the map is displayed to use FAST time. FAST TIME is a

variation of SIMULATED TIME which automatically advances the time displayed

by 10 or 60 seconds, as indicated at the upper left of the screen. Press F4

again to change the time step; when "(X10)" or "(X60)" is NOT displayed,

the time step is one second. The actual time increment is a function of the

computer's speed. For fast computers, the time increment will usually be 10

or 60 seconds but may vary by a second occasionally; for slower computers,

the time increment may be somewhat longer. FAST time is disabled when PAUSE

mode is in effect and for the Satellite Motion Map.

FAST time may be used to advance the display to a future time and for

demonstration purposes. Once the desired time has been reached, simply

press F4 until no time step is displayed at the upper left of the screen

and time will advance normally. Note that FAST time sets the program to

SIMULATED TIME; to restore "real time", press F8+F1 from the Main Menu. The

Main Menu also indicates when SIMULATED TIME is in effect: the words

"Current Time" will appear at the left of the times at the top of the

screen when real time (the time your DOS clock is using) is in effect; the

words "Simulated Time" appear when a simulated time is in effect.

On-line Help

------------

An on-line Help Screen is available during the ground track display

to remind the user of the available functions and which keys to press to

trigger those functions. When the ground track display is active, press

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 54

Function Key F1 to display the Help Screen in the lower portion of the

display. The following help screen will appear in rectangular modes:

F1=Resume Data   F6=Pause (+,-)    TAB=Auto Maps On/Off  L=Location Maps

F2=Select Clocks F7=Circle of Vis    W=World Maps        T=Tracking Maps

F3=Printer Log   F8=SUN and Term     Q=Quadrant Maps     M=Motion Map

F4=Time Step     F9=Units (nm/km)    Z=Zoom Maps:        O=Orthographic

F5=MET/T+Epoch  F10=Sat Coordinates    Home,PgUp,PgDn    STSPLUS Ver 9338

The help screen in orthographic modes is similar but in a vertical format

at the right side of the screen.

The ground track display will continue to be updated in real time

while the help screen is displayed. If only the graphical display of the

ground track is of interest, the help screen may be kept on the display

continuously. Press Function Key F1 again to return to the normal data

display in the lower portion of the screen. Note that the Help Screen is

disabled when the Motion Map is displayed.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 55

ACTIVE KEYS DURING GROUND TRACK DISPLAY

---------------------------------------

The following table lists the various keys which are active when the

ground track display is shown on the screen. Some of these features are

more fully described elsewhere.

ENTER  Return to Main Menu (cancel the simulation).

F1  On-line HELP. Press F1 to display a help screen in the lower

portion of the screen. Press F1 again to resume normal data

display.

F2  Selects the Big Clock mode. These modes are selected in the

following order:

0    No clock displayed

1    UTC date and time

2    Local date and time

3    STN/TDRS AOS/LOS and MET or T+Epoch (select with F5)

Note that not all computers (especially older CGA systems) will

display the extended graphics characters used for the large clock

characters. The symptom of this problem is that the lower left

portion of the data block is mostly blank after pressing F2. If

you have this problem and your computer is running DOS 3.x or DOS

5.0, enter the command "GRAFTABL" at the DOS prompt before

running STSPLUS or include the line "GRAFTABL" in your

AUTOEXEC.BAT file; this sets the "code page" to enable the

computer to display the extended graphics characters. [The

program GRAFTABL.COM is included as part of DOS in most cases.]

F3  Enable or disable printer logging. If logging is enabled, the

word LOG appears in the lower right of the screen. Be sure the

printer is turned on BEFORE using the L command. The "L" command

automatically enables the display of ascending and descending

node information.

F4  Toggle FAST mode from x1 to x10 to x60 to x1, etc. When either of

the fast modes is enabled, "(x10)" or "(x60)" will appear at the

upper right of the data block in red. This feature operates in

both the normal (real or simulated time) and PAUSE modes. When

x10 or x60 fast modes are used, automatic map generation is

disabled in rectangular projections; use the TAB key to restore

automatic map generation.

F5  Switches the elapsed time between "T+Epoch" and "MET". If no

launch time and date have been entered, this command will have no

effect.

F6  Enable PAUSE mode. The plot is frozen at the current position and

the "+" and "-" keys are enabled. (The "=" key may be used

instead of the "+" key to avoid use of the SHIFT KEY.) Press

ENTER to resume normal operation using the current simulated

time.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 56

F7  Enable or disable the spacecraft circle of visibility.

F8  Enable or disable the SUN, solar terminator, and spacecraft

lighting feature. NOTE: The SUN and spacecraft lighting are not

available on CGA systems.

F9  Change units of distance between kilometers (km) and nautical

miles (nm).

F10  Change satellite coordinates between Altitude and Azimuth, Right

Ascension and Declination, and Geocentric Rectangular (XYZ)

systems.

+  During PAUSE mode only, moves the satellite to the NEXT

calculated position based upon the FAST mode then in effect:

simulated time is advanced 1, 10, or 60 seconds. (NOTE: The "="

key may be used instead of "+" to avoid the SHIFT KEY.)

-  During PAUSE mode only, moves the satellite to the PREVIOUS

calculated position based upon the FAST mode then in effect:

simulated time is backed up 1, 10, or 60 seconds.

TAB  Enable or disable automatic map generation. Automatic map

generation is ALWAYS enabled in orthographic modes. When

automatic map generation is enabled in rectangular map modes, the

letter "A" will appear in the upper right of the display screen.

Pressing the TAB key will always cause the map to be redrawn.

PgUp  When in one of the zoom modes, increases the field of view up to

a maximum of 180 degrees. Press rapidly to execute multiple zoom

steps without redrawing the map for each keypress.

PgDn  When in one of the zoom modes, decreases the field of view down

to a minimum of 30 degrees. Press rapidly to execute multiple

zoom steps without redrawing the map for each keypress.

Home  When in one of the zoom modes, returns the field of view to 75

degrees (rectangular projections) or the full globe (orthographic

projections).

End  When in one of the zoom modes, returns the field of view to the

last zoom factor used prior to pressing the HOME key.

B  Toggle the BLINK mode of the satellite symbol between blinking

and steady. NOTE: On most systems, the satellite symbol will

appear to blink very briefly as it is erased and redrawn even

when BLINK is OFF.

O  Select Orthographic Projection (the LETTER "O" not the digit

zero). PgUp, PgDn, Home, and End are active to select the

magnification.

W,0  Select World Map display, showing the full world from +85 degrees

North latitude to -85 degrees South latitude using rectangular

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 57

projection. If automatic map generation is disabled, pressing "W"

or "0" will toggle between the two world map displays.

Q  Select Quadrant Map display, showing 180 degrees field of view

(rectangular projection) and selected so as to approximately

center the satellite.

1-9 Select the indicated Quadrant Map. Automatic map generation is

!@# disabled when a specific quadrant map is selected. See the chart

in the section Quadrant Maps for the map numbers.

Z  Select Zoom Map display, showing from 180 to 30 degrees field of

view (rectangular projection) and selected so as to approximately

center the satellite. The default is 75 degrees.

L  Select Location Map display, showing concentric isocontours for

your location. If a second location has been enabled, press "L"

again for that location.

T  Tracking Station Map display, showing concentric isocontours for

the tracking station closest to the current ground track position

of the satellite using the current projection. Uses the data in

the current TRACKING STATION file to select the tracking

station(s); if the file is not found, STSPLUS defaults to an

internal set of tracking stations. Use F7 from the Main Menu to

select the TRACKING STATION filename.

M  Satellite Motion Map display, available on EGA and VGA systems

only. Displays a map with the satellite centered using the map

projection in effect when the key is pressed. In rectangular

modes, the map is shown in zoom. Maps are drawn "off screen" and

a complete map is always displayed. The map is updated every 10

seconds or as rapidly as the computer processor will permit.

While the Satellite Motion Map is displayed, the following keys

are active: ENTER, "M", "Home", "End", "PgUp", and "PgDn"; these

keys perform the same functions as during the normal display

except that the "M" key cancels the Satellite Motion Map and

returns to normal display.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 58

STSORBIT PLUS MAIN MENU

-----------------------

Once the map coordinates have been stored internally, STSORBIT PLUS

presents its Main Menu:

Program STSORBIT PLUS

Space Shuttle and Satellite Orbit Simulation

Version 9338

Current time:  15:47:28 PST   23:47:28 UTC

Current date:    03/11/1993     03/11/1993

F1     Convert Keplerian Data to 2-Line Format

F2     Read NASA/NORAD 2-Line Elements       (.TXT/.TLE Files)

F3     Pass Predictions and Data Output

F4     Tabular Satellite Positions      (TRAKSTAR by TS Kelso)

F5     Set Launch Time and/or Launch Date

F6     Display MET & Launch Times     (now 2-Line Epoch Times)

F7     Set FILENAMES and Paths

F8     Set program TIME and/or DATE

F9     DOS Shell             (CAUTION: DOS Version 3.x+ ONLY!)

F10    Set STSORBIT PLUS Program Options and Features

ENTER  Resume Mission                      (MIR Space Station)

ESC    Quit STSORBIT PLUS and Save Current Mission

Select desired function:

WHILE MAP IS DISPLAYED:  F1 = HELP  ENTER = Main Menu

During operation of STSPLUS, data are displayed by STSPLUS in several

standard formats:

4/05/1990       Date in month/day/year

14:33:00 UTC   Coordinated Universal Time in hours:mins:secs

06:33:00 PST   Local Time in hours:mins:secs (abbr. may vary)

3/09:23:15       MET in days/hours:minutes:seconds

320.50 nm    Distance in nautical miles

551.37 km    Distance in kilometers

-69.34       Angles in degrees; WEST longitude and SOUTH

latitude are negative

Note that latitudes and longitudes also include "N" and "E" for positive

values respectively, and "S" and "W" for negative values respectively. This

convention, which may seem redundant, has been used to avoid possible

confusion; there are a number of representations for latitude and longitude

in common use which use different sign conventions. Azimuth (heading) is

given in the sense North-East-South-West where North is 0 degrees, East is

90 degrees, and so forth. The degree symbol is shown on the display for

all angles but has been omitted from this documentation because it may not

print correctly on all printers.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 59

F1     Convert Keplerian Data to 2-Line Format

----------------------------------------------

This function provides a means to individuals without modems to

receive the so-called "Keplerian Orbital Elements" by voice or other means

and reliably convert those data to the "2-Line Element" format as required

by STSPLUS and other satellite tracking programs. However, more data is

included in the 2-line orbital element set than is usually distributed as

the Keplerian orbital elements; this means that the missing data must

either be obtained from other sources or be set to a specified value or

zero. These instructions and the example and sample form which follows will

help the unskilled user to do these tasks.

This conversion function has but a single purpose: to prepare 2-line

orbital elements from Keplerian orbital elements by means of user keyboard

input. Since the program is designed only to transcribe valid data from one

format to another, no error checking is performed and the program makes no

tests of the "reasonableness" of the various data and parameters. The user

is therefore cautioned to check his data before using this program, or to

use the resulting data with caution until it has been verified.

The information shown below is usually included in the Keplerian

orbital elements as received via modem or voice (amateur radio or

telephone). This sample is an actual file for Space Shuttle Flight STS-55

launched in early 1993 as received via modem direct from the NASA Johnson

Space Center prior to launch:

Satellite: STS-55

Catalog number: 00055

Epoch time:      93073.67556033 =====> (14 MAR 93   16:12:48.41 UTC)

Element set:     JSC-003

Inclination:       28.4697 deg

RA of node:       228.7025 deg          Space Shuttle Flight STS-55

Eccentricity:     .0003812             Prelaunch Keplerian Elements

Arg of perigee:   314.2100 deg         Launch:  14 MAR 93  15:00 UTC

Mean anomaly:      45.8202 deg

Mean motion:   15.90487610 rev/day              G. L. Carman

Decay rate:     1.2020e-03 rev/day~2     NASA  Johnson Space Center

Epoch rev:               2

If you compare the information required by STSPLUS (as shown in the

example on the following page) with that above, several items are missing

or may be in a slightly different format. Here are some suggestions.

1.   A sample form is included in this documentation which will assist you

when receiving Keplerian orbital information by voice or by amateur

radio link. Note that the form includes lines for "IAU Designation"

and "BSTAR Drag"; these items are frequently omitted and must be

determined independently (see below).

2.   The Catalog Number (often referred to as the NORAD Number) is assigned

by US Space Command at Cheyenne Mountain after a satellite has been

successfully launched. Initial or pre-launch Keplerian elements may

use the mission number or some other convenient number for this item.

Most tracking programs will accept any number here. STSPLUS uses the

Catalog number to keep track of launch date and time; if the Catalog

number used for a mission changes, the launch date and time must be

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 60

entered again.

3.   The Epoch time is the instant for which the Keplerial elements have

been calculated and is NOT the same as the launch date and time.

Launch date and time is shown separately, is NOT part of the 2-line

elements, and must be entered in STSPLUS using F5 from the Main Menu.

4.   The IAU (International Astronomical Union) Designation is often not

assigned until well after a flight is in progress; press ENTER to use

all spaces for this item.

5.   For the 2-line orbital element set, the element set number (shown as

"JSC-003" in the example above) MUST be numeric. For this example,

enter "3" or "003". Most tracking programs will accept any number for

this item, up to a maximum of FOUR DIGITS.

6.   The "Epoch Rev/Orbit" is the orbit number at the epoch (the instant

when the data is taken). Up to FOUR DIGITS may be entered here but

except for the orbit number on displays, this number has no effect on

the propagated orbit. NASA assigns Orbit/Rev 1 to the first partial

orbit following launch; US Space Command usually (but not always)

assigns Orbit/Rev 0 to the first partial orbit and the orbit number

for space shuttle missions must usually be adjusted to conform to NASA

convention.

7.   When the program asks for "NDOT2 Drag/Decay", the information being

requested is the "Decay rate" in the Keplerian elements. It may be

entered in the form shown in the example or as a decimal fraction such

as ".002102" (which is identical to the "2.10200e-03" in scientific

notation as used in the example).

8.   When the program asks for "NDDOT6 Drag", press ENTER to insert a value

of zero. This should be satisfactory for most satellites and most

tracking programs.

9.   When the program asks for "BSTAR", you may press ENTER to insert a

value of zero. This will be satisfactory for times very close to the

Epoch Time for the elements. However, as time progresses the satellite

may lag behind the propagated (projected) ground track if no value is

avaialble for BSTAR. For space shuttle missions, NASA often assigns a

default value of "25599-3" or ".00025599". A value from a prior set of

2-line elements may also be used.

10.  After all data have been entered, STSPLUS will request the filename to

which the 2-line orbital data is to be written. Enter the full

filename and filetype, such as "TESTSAT.TXT". The file will be written

to the drive and directory set using F7+F3 from the Main Menu. DO NOT

INCLUDE A DRIVE OR DIRECTORY; THAT IS ADDED AUTOMATICALLY! An existing

file with the same name will be overwritten.

11.  If you make a mistake during the data entry process, you may use the

BACKSPACE key until ENTER has been pressed. If you wish to cancel the

program without writing the data to a file, press ENTER when asked for

the filename and the data will be discarded.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 61

As noted elsewhere, orbital elements have a limited lifetime. How long

that lifetime may be depends primarily upon the orbital altitude. Low Earth

orbit (LEO) satellites, especially those such as the space shuttle and

Russian space station, are frequently maneuvered to maintain the desired

orbit. Orbital elements for these satellites may be valid for only days or

sometimes even hours. Orbital elements for higher orbital altitudes, say

above 1,500 km, tend to be valid for much longer -- up to weeks. Satellites

in very high or geosynchronous orbits exhibit usually long term orbital

stability. The Bottom Line: The accuracy with which any tracking program

can determine a satellite's position is primarily dependent upon having

accurate orbital elements!

Finally, a word about the NDOT2 and BSTAR parameters. A satellite's

orbit is completely and accurately determined at the epoch time without

NDOT2 and BSTAR. These two parameters determine various adjustments as the

orbit is propagated in time and are a function of such things as orbital

altitude, spacecraft attitude and cross-sectional area, atmospheric density

changes due to sunspot activity, and so on. Both US Space Command and

Johnson Space Center have been known to "tweak" these values for a variety

of reasons, usually because the orbit is in a state of change due to

maneuvers and/or excessive drag.

NOTE: A positive exponent for BSTAR, which indicates high acceleration

and is somewhat unusual (and often an indication of "tweaking"), is NOT

handled correctly and must be manually edited. The resulting elements may

not propagate accurately for more than a few hours.

EXAMPLE DATA INPUT

------------------

Enter 2-LINE ELEMENTS

Enter the required data as prompted. Most essential data is included in the

Kelplerian Orbital Data available to amateur radio enthusiasts. Other data

may be inserted if available or may be omitted if not. Accuracy may be

affected, see documentation.

Enter Satellite Name (15 chars max):      sts-55

Enter NORAD Number (NNNNN):               55

Enter IAU Designation (8 chars max):

Enter Epoch Time (YYDDD.TTTTTTTT):        93073.67556033

Enter Element Set Number (NNN):           3

Enter Inclination (DDD.DDDD):             28.4697

Enter RA of Ascend Node (DDD.DDDD):       228.7025

Enter Eccentricity (.NNNNNNN):            .0003812

Enter Arg of Perigee (DDD.DDDD):          314.21

Enter Mean Anomaly (DDD.DDDD):            45.8202

Enter Mean Motion (NN.NNNNNNNN):          15.9048761

Enter Epoch Rev/Orbit (NNN):              2

Enter NDOT2 Drag/Decay (.NNNNNNNN):       .001202

Enter NDDOT6 Drag (NNNNN-N):              00000-0

Enter BSTAR (NNNNN-N):                    36300-3

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 62

EXAMPLE DATA OUTPUT

-------------------

Satellite Data has been entered as:

Satellite Name (15 chars max):            Sts-55

NORAD Number (NNNNN):                     00055

IAU Designation (8 chars max):

Epoch Time (YYDDD.TTTTTTTT):              93073.67556033

Element Set Number (NNN):                  3

Inclination (DDD.DDDD):                    28.4697

RA of Ascend Node (DDD.DDDD):             228.7025

Eccentricity (.NNNNNNN):                  .0003812

Arg of Perigee (DDD.DDDD):                314.2100

Mean Anomaly (DDD.DDDD):                   45.8202

Mean Motion (NN.NNNNNNNN):                 15.90487610

Epoch Rev/Orbit (NNN):                     2

NDOT2 Drag/Decay (.NNNNNNNN):             .00120200

NDDOT6 Drag (NNNNN-N):                    00000-0

BSTAR (NNNNN-N):                          36300-3

Sts-55

1 00055U          93073.67556033  .00120200  00000-0  36300-3 0    33

2 00055  28.4697 228.7025 0003812 314.2100  45.8202 15.90487610    23

Enter FILENAME for 2-Line Elements:

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 63

RECEIVED KEPLERIAN ORBITAL DATA FORM

------------------------------------

This form is provided as a convenience to users receiving Keplerian

orbital data via voice link (amateur radio or telephone). Each set of

underline characters indicates an expected character; the decimal point is

shown where expected (if appropriate for the item). Make multiple copies of

this form prior to a mission.

Satellite:          __ __ __ __ __ __ __ __ __ __ __ __ __ __ __

Catalog Number:     __ __ __ __ __

IAU Designation:    __ __ __ __ __ __ __ __

Epoch Time:         __ __ __ __ __ . __ __ __ __ __ __ __ __

Element Set:        __ __ __ __

Inclination:        __ __ __ . __ __ __ __

RA of Node:         __ __ __ . __ __ __ __

Eccentricity:       . __ __ __ __ __ __ __

Arg of Perigee:     __ __ __ . __ __ __ __

Mean Anomaly:       __ __ __ . __ __ __ __

Mean Motion:        __ __ . __ __ __ __ __ __ __ __

Decay Rate:         . __ __ __ __ __ __ __ __

BSTAR Drag:         __ __ __ __ __ - __

Epoch Rev:          __ __ __ __

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 64

F2     Read/Update NASA/NORAD 2-Line Elements from *.TXT/*.TLE Files

--------------------------------------------------------------------

In order to read or update the NASA/NORAD 2-line elements, you must

have a file with data for the appropriate satellites. A current file is

included in the standard distribution of STSPLUS. These files have names

such as "TLE239.TXT" where the "239" corresponds to the particular US Space

Command Prediction Bulletin number from T. S. Kelso's Celestial BBS and may

change several times per week. Other TLE files are available from my BBS.

Update Current TDRS and Real Time Satellites

--------------------------------------------

Once TDRS and Real Time Satellites have been selected (using F6 from

the Main Menu), it is necessary to periodically update the 2-line elements

("TLEs") for these satellites so that the calculated positions are

accurate. Naturally, this is also true for the primary satellite being

tracked. TLEs have a limited lifetime. For higher altitude satellites such

as geosynchronous satellites, 4 to 8 weeks is probably sufficient unless

the satellite is being maneuvered (although I usually do this task at least

weekly since I have the data). For low Earth orbit satellites, even when

the satellite is not being maneuvered, I recommend a maximum interval

between updates of 1 to 2 weeks; if the satellite is being maneuvered, such

as is usually the case with the Space Shuttle, daily updates may be

required.

To update 2-line elements for the primary satellite and all TDRS and

Real Time satellites, press F2 and select the desired file as described in

the following section. Then enter the satellite name as "&" to request TLE

update mode. The TLE file will be scanned and all satellite TLEs will be

updated if their Epoch Time is later than those presently stored. The

entire process takes only a few seconds. The following prompt illustrates

the "Update" selection:

Select NASA/NORAD 2-Line Elements File

Enter 2-Line Filename  [GSFC267.TXT]:     GSFC267.TXT

Enter Satellite Name/#nnnnn  [#20638]:    &

(Enter '*' to match any satellite name, '&' for AUTO UPDATE)

As STSPLUS updates TLEs, a list of the satellites for which new TLEs have

been loaded is displayed:

Automatic TLE updates for:

Satellite 3: Norad# 22314 @ 93217.59157166

Satellite 2: Norad# 21639 @ 93219.36124578

Satellite 8: Norad# 20580 @ 93218.03265951

Satellite 1: Norad# 19883 @ 93220.90943044

Satellite 4: Norad# 19548 @ 93220.44861278

Satellite 6: Norad# 16609 @ 93220.81848976

Satellite 5: Norad# 13969 @ 93220.48367176

Press any key to continue ...

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 65

The satellite number indicates the "slot" in the TDRS and Real Time

Satellite configuration, or "P" for the Primary Satellite. In the example

shown, the TLEs for seven satellites were updated to the Epoch Times

indicated. The updated TLEs will be saved in file STSPLUS.INI. Press ENTER

to return to the Main Menu.

Read NASA/NORAD 2-Line Elements from a File

-------------------------------------------

For both "Read" and "Update" functions, pressing F2 will display a

list of all available files with default filetypes ".TXT" and ".TLE". The

following example has been edited to show only 8 files:

Select NASA/NORAD 2-Line Elements File

Enter 2-Line Filename  [TLE147.TXT]:

Use ARROW KEYS, press ENTER to use the current default file

shown in square brackets [...], or press ESC to CANCEL.

8 matching files in directory F:\TLE

GPS.TXT

GROUP000.TLE

GSFC198.TXT

GSFC199.TXT

MIR.TLE

TLE141.TXT

TLE146.TXT

TLE147.TXT

The list of files is sorted in alphabetical order by filename then

displayed using up to five columns. The display mode is adjusted for the

maximum number of lines permissible for the active monitor type: 25 lines

for CGA and HGC, 43 lines for EGA, and 50 lines for VGA. The maximum number

of files which may be displayed for each monitor type is shown in the

following table:

Screen     File      Max

Monitor    Lines    Lines    Files

----------------------------------

VGA           50       42      210

EGA           43       35      175

CGA/HGC       25       17       85

To accept the default file shown in the square brackets, TLE147.TXT in

the example, press ENTER. To manually enter a filename, type the name (the

filetype .TXT will be appended if no filetype is typed) and press ENTER. To

select one of the displayed files, use the ARROW KEYS (UP, DOWN, LEFT,

RIGHT), Home, End, PgUp, or PgDn to move through the list until the desired

file is highlighted and shown in the square brackets, then press ENTER. To

cancel the function and return to the Main Menu, press ESC.

STSPLUS defaults the drive and directory to the current directory, the

one from which STSPLUS is being executed. However, some users prefer to use

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 66

a separate directory for 2-line elements files. To specify a different

drive and/or directory, enter the drive (followed by a colon) and the

desired directory (followed by a trailing backslash, "\"). The specified

drive, directory, and filename are saved in file STSPLUS.INI and will be

used the next time STSPLUS is executed. The following examples illustrate

this method:

D:\                 Use the root directory on drive D:

\ELEMENTS\          Use the current drive and directory

\ELEMENTS\

C:\TLE\             Use drive C: and directory \TLE\

Failing to include the trailing backslash will cause STSPLUS to interpret

what you intended as a directory to be a filename! The complete path with

filename and filetype mask may also be entered:

C:\TLE\*.TXT        Use .TXT files on Drive C: and directory TLE

D:\TLE\*.*          Display all files on drive D: and directory

\TLE\

F:\TLE\TLE*.TXT     Use drive F:, directory \TLE\ and all files

matching "TLE*.TXT"

Although STSPLUS defaults to filetypes "*.TXT" and "*.TLE", you may use

this command to temporarily specify a different filename and filetype mask

if desired.

If no files with filetype .TXT or .TLE (or files corresponding to the

current filename and filetype mask) are found in the specified directory,

the following error message will be displayed:

No matching files found in specified drive/directory: E:\JUNK

Press any key to continue ...

To specify NO fileltype, enter the filename followed by a period, i.e.

"ELEMENTS.". Any desired filetype may be used, but the program will always

default to ".TXT" and ".TLE" each time F2 is used.

If you include a drive (such as "D:") and/or directory (the directory

MUST be followed by a trailing backslash, "\"), and the drive or directory

cannot be found, the following error message will be displayed:

Drive or path error: E:\JUNK

Press any key to continue ...

Once the file has been selected, a default satellite name will appear

in the next prompt:

Enter 2-Line Filename  [STS50N38.TXT]:    STS50N38.TXT

Enter Satellite Name/#nnnnn  [STS...]:

(Enter '*' to match any satellite name, '&' for AUTO UPDATE)

STSPLUS will normally display the first three characters of a

satellite name or the full NORAD number, enclosed in square brackets, as

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 67

the default choice. If no prior satellite has been selected, the satellite

name will default to "STS..." for space shuttle missions (provided the

filename begins with "STS") and to "HST..." for all other satellites;

otherwise, it will be the first three letters or full five digit NORAD

number of the currently selected satellite. NORAD numbers are always

prefixed with the "#" character. If you wish to change the information (or

if no default is shown), enter the required information followed by ENTER.

For the satellite name, only sufficient letters to unambiguously identify

the desired satellite, upper or lower case, are required. For example,

"Alou" would select "Alouette 1". However, note that entering "MIR" could

select "MIRANDA" before it finds "MIR" depending upon the ordering of the

2-line elements within the file. Alternatively, you may enter the NORAD

number for the desired satellite by entering "#" followed by the number;

leading zeroes may be omitted.

Once the information has been entered, STSPLUS will attempt to locate

the data for the requested satellite. If a satellite matching the requested

name is found, the data for that satellite are displayed. Certain non-

essential data are not always included in the 2-line elements and may be

replaced by spaces, indicated by "(n/a)".

Satellite Name:              Mir

Satellite NORAD Number:      #16609

Elements File:               TLE233.TXT

Elements File Record#:       315

(*)

Element Set Number:          201

Elements Epoch:              93206.84472440

25 JUL 1993 @ 20:16:24.188 UTC

Orbit # at Epoch:            42516

Launch Year:                 1986

Launch Number:               17

Launch Piece:                A

Inclination:                 51.6213

RA of Ascend Node:           25.384

Eccentricity:                .0004375

Arg of Perigee:              244.4461

Mean Anomaly:                115.6366

Mean Motion:                 15.58931226

Acceleration/Drag:           .00007676

BSTAR Drag:                  .000099999

Press ENTER to ACCEPT this satellite, OR

Press any other key to REJECT and continue searching:

(*) This line is normally blank. However, one of the following

messages will appear here if a checksum error is detected in the

element set:

BAD CHECKSUM in line 1 ignored!

BAD CHECKSUM in line 2 ignored!

BAD CHECKSUM in both lines ignored!

In all three cases, STSPLUS will accept the data and attempt to use

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 68

it. Be advised, however, that the checksums are included to help

detect data errors that might otherwise yield an incorrect position!

Serious errors may even cause STSPLUS to abort with an error message.

For convenience, the Elements Epoch (the instant at which these

orbital elements were calculated) is shown in two formats: the first format

is that used in the 2-line elements, YYDDD.DDDDDDDD; and the second format

is the same time converted into conventional date and time notation. You

may thus see immediately how old the elements are and take this into

account when evaluating the satellite's projected position.

If this is the satellite you wish, press ENTER and the data will be

entered into STSPLUS. If, on the other hand, a different satellite is

desired, press any other key (such as the SPACE BAR) and STSPLUS will

search for another name matching the requested satellite. For example,

there are a number of NAVSTAR Global Positioning Satellites usually

included in the file with official names such as "GPS-0001", "GPS-0002",

"GPS BII-01" and so forth; requesting "GPS" will allow you to cycle through

all the available choices. The file TLEnnn.TXT is an ASCII file; it may be

helpful to view or print the file to see the available satellite names.

Once the satellite has been selected, STSPLUS will require a brief

time to calculate certain required orbital parameters, then will proceed

directly to the display of the ground track. However, if the current

calculated altitude of the satellite is less than 75 nautical miles, the

satellite has probably decayed. STSPLUS will display the following message

before returning to the Main Menu:

Satellite MIR DEB (#22209) indicates a current altitude less than

75 nautical miles and has probably decayed. STSORBIT PLUS can NOT

process the orbital data for this satellite!

Use Function Key F2 from the Main Menu to select another

satellite and verify the satellite NAME and NORAD NUMBER.

Press any key to return to the Main Menu ...

As a point of interest, the 2-line elements for the Space Shuttle

Mission STS-50 used in the example above are as follows:

STS-50

1 22000U 92 34  A 92187.57342677 -.00032668  00000-0 -97874-4 0   380

2 22000  28.4670 275.0700 0007237 340.7929  19.1530 15.91359642  1596

F3     Data Output and Pass Prediction Selections

-------------------------------------------------

By popular request, STSPLUS has been enhanced to send selected data

for the current satellite to other equipment via a serial port (COM1 or

COM2), to a file (STSPLUS.LOG), or to the printer (LPT1). Validation of the

serial port output has been accomplished using two computers and a "Null

Modem" cable. Three classes of data may be selected for output: current

position data in three formats, precision Earth-centered inertial ("ECI")

state vectors in four formats, and tabular Line-of-Sight pass predictions

(which are also displayed on the screen).

The precision state vector outputs have been carefully coordinated

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 69

with Ken Ernandes so that they may be used as input to his program VEC2TLE,

Version 9331 or later. Using these state vectors and VEC2TLE, the user may

generate 2-line elements at any desired time (including just after the

ascending node) for use with STSPLUS or other satellite tracking programs.

With these programs, the user has a very powerful set of tools which can

be used for a variety of analytical and display purposes. Cross validation

of the two programs during Space Shuttle missions STS-56 and STS-55, as

well as comparison with US Space Command data of comparable epoch,

demonstrated high accuracy and excellent correlation.

Each Data Output function is assigned a "Data Mode" number:

1 = Azimuth, Elevation, Range

2 = Latitude, Longitude, Orbit Altitude

3 = Right Ascension, Declination

4 = Ascending Node Data with State Vector

5 = Precision X-Y-Z State Vector (2-Line Data)

6 = Precision X-Y-Z State Vector (Comma Delimited)

7 = Precision X-Y-X State Vector (Labeled Data)

9 = Tabular Line-of-Sight Predictions

The current position data and precision state vectors are generated while

the ground track map is displayed; for all Data Modes EXCEPT #4, data

output may be logged continuously, for a specified time (UTC/GMT or local

time), or for a specified time span (UTC/GMT or local time). Data Mode 4

records data ONLY at the Ascending Node, e.g. when the Northbound equator

crossing is detected. The predicted pass data is calculated "off-line"

using UTC/GMT or local time, and is displayed on the screen as well as

being sent to the selected output destination.

Current position data include the UTC date/time and are generated for

local horizon coordinates (altitude and azimuth), geographic coordinates

(geocentric latitude, longitude, and orbital altitude), and topocentric

equatorial coordinates (right ascension and declination calculated for the

user's location). Precision X-Y-Z Earth-centered inertial state vectors

(ECI position and velocity components) are generated as two numeric data

lines, single line comma delimited, and multi-line labeled data. The

details for each data output format are given in following sections.

The precision ECI X-Y-Z state vectors, generated by STSPLUS for the

true equator and equinox of date, have been extensively tested and

validated in conjunction with Ken Ernandes' program VEC2TLE during Space

Shuttle missions STS-56 and STS-55 in early 1993. For example, the

combination of the two programs, STSPLUS and VEC2TLE, may be used to

convert data between 2-line and ECI formats with very high accuracy. State

vectors from STSPLUS may be read by VEC2TLE and converted into 2-line

elements, then in turn read again by STSPLUS with essentially exact

conversions. VEC2TLE has also been used during STS-56 and STS-55 to convert

real time state vectors ("M50" or Mean of 1950) supplied courtesy Willie

Musty of Mission Support, Rockwell International, into 2-line elements

equal in accuracy to those generated by US Space Command (and made

available four to eight hours sooner!). See the separate text section

describing VEC2TLE.

Note that the timing accuracy for Data Mode 4 is a fixed at 0.01

seconds, regardless of the time step (X1, X10, or X60) then in effect.

STSPLUS detects the Ascending Node data when the latitude switches from

negative to positive on the Northbound crossing of the Equator. An

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 70

iterative process is then used to refine the time to the nearest 0.01

seconds and the data at that time are recorded.

Potential applications for the position data include automatic antenna

pointing systems, off-line high precision plotting, and widespread

distribution of the data within a large facility or via modem. ECI state

vectors may be used in real time to create 2-line elements for a specified

epoch to full precision. Users who develop applications to utilize these

data are invited to contribute their programs and documentation for general

use. Since these are new features for STSPLUS, comments and suggestions are

welcome.

Although STSPLUS retains the capability of performing off-line pass

predictions with TS Kelso's TRAKSTAR or other satellite tracking software,

many users have requested that a similar feature be incorporated directly

into STSPLUS. Pass predictions may only be calculated for satellites having

a mean motion greater than 1.5; this eliminates satellites in near

geosynchronous or higher orbits, but since such satellites don't move much

that does not represent a significant constraint. The satellite's orbit is

examined for 48 hours, starting at the current real or simulated time, with

a sampling interval which ranges from 10 to 60 seconds depending upon the

orbit. Because of this "granularity" in the search algorithm, it is

possible to skip passes whose duration is less than the sampling interval.

Since those brief passes would barely peek above the user's horizon, they

are thus not significant. Pass predictions may be continued in 48 hour

segments until 99 passes have been displayed. Since a typical satellite may

have from about two to seven passes in a 24 hour period, the passes may be

examined for a considerable time into the future.

Predicted pass data are calculated using the current satellite for

Line-of-Sight visibility; that is, when the satellite is in line of sight

to the user's location and without consideration of lighting effects. Since

STSPLUS users are about equally divided between those who track satellites

visually and those who use electronic equipment such as amateur radio, this

method provides data for all users. Passes which occur near local sunrise

or sunset may be tested for lighting constraints and/or ground visibility

by displaying the ground track for the pass or other means. Dates and times

may be displayed in Coordinated Universal Time (UTC/GMT) or in local time.

Note that the date for each pass is given only for AOS (Acquisition of

Signal); it is possible for the pass to span 00:00:00 hours (midnight) for

the time scale in use with a consequent date change during the pass for MAX

VISIBILITY and/or LOS (Loss of Signal).

Prediction calculations may require some time; calculation delays are

noted with the message "calculating ...". Using a processor equipped with a

math coprocessor chip, each 48 hour block may require from less than 10

seconds to a minute or more. However, users without math coprocessor chips

will experience significant delays -- minutes or even tens of minutes! The

following table lists typical calculation times for various processors (all

with math coprocessors!) using the Russian MIR Space Station:

286/287        386SX/387SX         486DX

8 MHz          20 MHz              33 MHz

---------------------------------------------------

MIR       60 sec         30 sec              6 sec

The data output feature MUST be enabled with F3 each time STSPLUS is

run; it is NOT automatically restarted when the "/R" (RESUME) command line

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 71

option is used.

**********

*  NOTE  *

**********

Users are reminded that when data output is sent to the file

STSPLUS.LOG, a considerable volume of data may be accumulated

over long periods of time. It is possible to completely fill a

disk with this data! The file should be periodically copied to

other media or deleted to avoid this problem.

Setting Up Position and State Vector Data Output

------------------------------------------------

Data output of position and state vectors ONLY occurs while the

ground track is displayed; no data are generated until the ground track is

actually in progress! The appropriate data are sent to the destination

device at the selected data interval (continuous), at a specified time, or

at the selected data interval over a specified time span covering no more

than 24 hours.

************************

*  IMPORTANT REMINDER  *

************************

STSPLUS generates Earth-Centered Inertial ("ECI") state vectors

for the true equator and equinox of date. Other software and

various agencies may use different coordinate systems. In

particular, NASA uses the mean equator and equinox of the

Besselian year 1950 ("Mean of 1950", "M50" or "B1950").

Astronomers and other agencies may use the mean equator and

equinox of the Julian year 2000 ("Mean of 2000" or "J2000").

Other agencies, such as the DOD C-Band Radar Network, use a time-

independent coordinate system ("Earth-fixed Greenwich" or "EFG")

for predicted state vectors prior to a launch. Users must take

care that the appropriate coordinate system is used for each

application and/or perform the required conversions.

STSPLUS sets up certain initial default parameters for data output and

displays the current parameters each time F3 is pressed, as shown in the

following example:

STSORBIT PLUS Data Output Parameters:

Data Output:                  STSPLUS.LOG

Data Format:                  7 = Precision X-Y-Z State Vector

Data Interval:                60 seconds (continuous)

Data Units:                   Kilometers, Multi-Line Labeled

Accept Parameters [Y,n,x]:

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 72

To cancel data output and return to the Main Menu, press "X". If the

current parameters are correct, press "Y" (or ENTER) to accept them. If the

parameters are to be changed or if a specified time or time span is

desired, press "N" to be prompted for new parameters. In each case, the

default value which will be used if ENTER is pressed will be shown in

square brackets; if more than one choice is shown, separated by commas, the

first choice will be used if ENTER is pressed. The user must first select

the data output device or destination by pressing the indicated key:

Select Output [F,p,1,2]:

F = File STSPLUS.LOG

P = Printer LPT1:

1 = Serial Port COM1:

2 = Serial Port COM2:

Pressing ENTER or the letter "F" (upper or lower case) will select the FILE

output and the data will be sent to the file STSPLUS.LOG. If the file does

not exist, it will be created; if the file already exists, the data will be

appended to the existing data. Press the letter "P" to direct the data to

the printer on LPT1. Press "1" or "2" to direct the data to one of the two

serial ports.

When a serial port (COM1: or COM2:) is selected, the user next selects

the data rate to be used for communications with the external equipment.

Only the four data rates shown below the prompt are supported. Use the

first character of the desired rate to select it, or press ENTER to use the

data rate shown in the square brackets:

Select DATA RATE [9600]:

(300, 1200, 2400, 9600)

STSPLUS automatically sets the communications parameters to "8,N,1"; these

are fixed and may not be altered. These communications parameters select 8

data bits per transmitted byte, NO parity, and 1 stop bit. Most external

equipment will operate satisfactorily with these parameters.

STSPLUS requires several additional items of information before it can

send data to the external equipment, file or printer. The first is the data

format to be used. Eight different data formats are available. The next

prompt lists the formats and shows the current default in square brackets:

Select Data to Output [7]:

1 = Azimuth, Elevation, Range

2 = Latitude, Longitude, Orbit Altitude

3 = Right Ascension, Declination

4 = Ascending Node X-Y-Z State Vector

5 = Precision X-Y-Z State Vector (2-Line Data)

6 = Precision X-Y-Z State Vector (Comma Delimited)

7 = Precision X-Y-X State Vector (Labeled Data)

9 = Tabular Line-of-Sight Predictions

Press the number key corresponding to the desired Data Mode or press ENTER

to select the choice shown in square brackets. See the Data Mode Formats in

the following sections for specific details on the data included in each

data mode.

For current position and state vector formats except Data Mode 4, the

desired time interval between successive sets of data must be also

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 73

selected. (Data Mode 4 records data immediately after the Ascending Node

and does not use the time interval parameter.) Any interval between 1 and

900 seconds may be selected (that is, up to 15 minutes maximum). Add the

letter "T" or "t" if you wish the data to be logged for a specific time or

time span. Note also that this is the DESIRED time interval; if your

computer is too slow to complete its calculations in that time, the

interval will be longer. In other words, STSPLUS will generate the

requested data no more frequently than the interval requested but may take

longer, depending upon what has to be done each time. In response to the

prompt, press ENTER to accept the default value shown in square brackets or

type the desired numerical value (in seconds) followed by ENTER:

Data Interval (secs) [60]:

(Min = 1 sec, Max = 900 secs; Add 'T' for timer)

If a value less than 1 second is entered, 1 second will be substituted; if

a value greater than 900 seconds is entered, 900 will be substituted. If

"T" is entered by itself, the default value shown in square brackets will

be used for the Data Interval.

If "T" is appended to the desired interval or is entered by itself,

STSPLUS requests the Start Time for logging:

Start Time (HH:MM:SS):

(Add 'U' or 'G' for UTC/GMT)

Enter the desired LOCAL Start Time or add the letter "U" or "G" for UTC/GMT

time. STSPLUS will reformat the entered time and add the appropriate time

zone designation, then prompt for the Stop Time:

Start Time (HH:MM:SS):        08:45:00 PDT

Stop Time (HH:MM:SS):

(Press ENTER for Stop Time = Start Time)

Enter the desired Stop Time using the SAME time scale used for Start Time,

or press ENTER to use the Start Time. STSPLUS will reformat the entered

time and add the appropriate time zone designation.

Stop Time (HH:MM:SS):         08:50:00 PDT

When Start Time equals Stop Time, only one set of data will be recorded.

Reasonable care is required when setting up these times. If the current

time (real or simulated) is past the Start Time, data will be recorded

immediately. Times may be set to less than 24 hours into the future.

When state vectors are requested (Data Modes 5 through 7), STSPLUS

must also know the desired units of measure, kilometers ("km" or "KM"),

feet ("ft" or "FT"), or nautical miles ("nm" or "NM"):

Data Units [KM,ft,nm]:

(Press 1st letter to select)

Press the first letter of the desired units of measure or press ENTER for

the current default units of measure (shown in capital letters in the

prompt, "KM" in the example above).

STSPLUS now displays the new parameters for approval:

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 74

STSORBIT PLUS Data Output Parameters:

Data Output:                  STSPLUS.LOG

Data Format:                  7 = Precision X-Y-Z State Vector

Data Interval:                60 seconds, 08:45:00-08:50:00 PDT

Data Units:                   Kilometers, Multi-Line Labeled

Accept Parameters [Y,n,x]:

If no Start and Stop Times have been entered, "(continuous)" will appear in

place of the Start and Stop Times.

As before, press "Y" (or ENTER) to accept the parameters and enable

data output, "N" to re-enter the parameters, or "X" to cancel data output

and return to the Main Menu.

Setting Up Tabular Pass Predictions

-----------------------------------

Tabulation of predicted passes is always displayed on the screen and

may optionally be sent to a destination device. The appropriate data are

displayed and sent to the destination device at the selected data interval

(continuous), at a specified time, or over a specified time span covering

no more than 24 hours.

STSPLUS sets up certain initial default parameters for tabular pass

predictions and displays the current parameters each time F3 is pressed, as

shown in the following example:

STSORBIT PLUS Data Output Parameters:

Data Output:                  STSPLUS.LOG

Data Format:                  9 = Tabular Line-of-Sight Predictions

(Using PDT for times)

Accept Parameters [Y,n,x]:

To cancel data output and generate pass predictions on the screen

ONLY, press "X". If the current parameters are correct, press "Y" (or

ENTER) to accept them. If the parameters are to be changed or if a

specified time is desired, press "N" to be prompted for new parameters. In

each case, the default value which will be used if ENTER is pressed will be

shown in square brackets; if more than one choice is shown, separated by

commas, the first choice will be used if ENTER is pressed. The user must

first select the data output device or destination by pressing the

indicated key:

Select Output [F,p,1,2]:

F = File STSPLUS.LOG

P = Printer LPT1:

1 = Serial Port COM1:

2 = Serial Port COM2:

Pressing ENTER or the letter "F" (upper or lower case) will select the FILE

output and the data will be sent to the file STSPLUS.LOG. If the file does

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 75

not exist, it will be created; if the file already exists, the data will be

appended to the existing data. Press the letter "P" to direct the data to

the printer on LPT1. Press "1" or "2" to direct the data to one of the two

serial ports.

When a serial port (COM1: or COM2:) is selected, the user next selects

the data rate to be used for communications with the external equipment.

Only the four data rates shown below the prompt are supported. Use the

first character of the desired rate to select it, or press ENTER to use the

data rate shown in the square brackets:

Select DATA RATE [9600]:

(300, 1200, 2400, 9600)

STSPLUS automatically sets the communications parameters to "8,N,1"; these

are fixed and may not be altered. These communications parameters select 8

data bits per transmitted byte, NO parity, and 1 stop bit. Most external

equipment will operate satisfactorily with these parameters.

STSPLUS requires several additional items of information before it can

send data to the external equipment, file or printer. The first is the data

format to be used. Five different data formats are available. The next

prompt lists the formats and shows the current default in square brackets:

Select Data to Output [7]:

1 = Azimuth, Elevation, Range

2 = Latitude, Longitude, Orbit Altitude

3 = Right Ascension, Declination

4 = Ascending Node X-Y-Z State Vector

5 = Precision X-Y-Z State Vector (2-Line Data)

6 = Precision X-Y-Z State Vector (Comma Delimited)

7 = Precision X-Y-X State Vector (Labeled Data)

9 = Tabular Line-of-Sight Predictions

Press the "9" number key to select tabular pass predictions or press ENTER

to select the choice shown in square brackets. See the Data Mode Formats in

the following sections for specific details on the data included in each

data mode.

For tabular line-of-sight predictions, STSPLUS must know the time zone

for which data is to be displayed. The choices are UTC/GMT or the LOCAL

time zone:

Use UTC or PDT time [PDT]:

(Use LEFT LETTER of abbreviation to select);

Depending upon the user's choice when the UTCOffset was set, either "UTC"

or "GMT" will be displayed along with the abbreviation for the local time

zone. Use the left-most letter of the desired time zone, or press ENTER to

accept the time zone shown in the square brackets ("[PDT]" in the example).

STSPLUS now returns to the initial parameter display and again asks if

the parameters are correct. As before, press "Y" to proceed with pass

predictions AND sending the data to the specified destination device, "N"

to change parameters, or "X" return to the Main Menu and display the pass

predictions on the screen WITHOUT sending the data to a destination device.

For tabular Line-of-Sight predictions, the data are displayed on the

screen as calculated and optionally sent to the selected destination

device. The following is an edited sample of the screen display:

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 76

---------16609 AOS--------  ---MAX VISIBILITY--  ------LOS------

#   UTC Date UTC Time    Azm  UTC Time Alt    Azm  UTC Time    Azm Duration

1 02/23/1993 03:05:56  171.5  03:09:39   7  125.1  03:13:23   78.4  0:07:27

2 02/23/1993 04:40:12  230.2  04:45:23  70  318.8  04:50:32   44.9  0:10:20

3 02/23/1993 06:17:58  281.3  06:22:02   9  333.9  06:26:07   26.7  0:08:09

4 02/23/1993 07:57:27  328.5  07:59:40   2  354.4  08:01:53   20.3  0:04:26

5 02/23/1993 09:34:31  339.1  09:37:34   4   15.9  09:40:38   52.7  0:06:07

6 02/23/1993 11:09:57  325.6  11:14:46  21   34.8  11:19:35  103.4  0:09:38

7 02/23/1993 12:46:00  302.3  12:50:56  29  229.0  12:55:55  155.6  0:09:55

8 02/24/1993 02:12:22  131.7  02:13:17   0  121.3  02:14:12  110.9  0:01:50

9 02/24/1993 03:43:39  212.0  03:48:44  42  132.6  03:53:48   53.7  0:10:09

10 02/24/1993 05:20:25  263.7  05:25:02  16  327.6  05:29:40   31.9  0:09:15

11 02/24/1993 06:59:36  314.0  07:02:22   3  347.0  07:05:09   20.0  0:05:33

12 02/24/1993 08:37:54  340.4  08:40:18   2    8.7  08:42:43   37.0  0:04:49

13 02/24/1993 10:13:30  331.3  10:17:49  12   28.7  10:22:09   85.8  0:08:39

14 02/24/1993 11:49:11  311.8  11:54:20  79  224.7  11:59:31  137.0  0:10:20

15 02/24/1993 13:26:53  273.2  13:29:57   4  236.4  13:33:01  199.8  0:06:08

Elapsed time = 27 seconds

Repeat for NEXT 48 HOURS or DISPLAY PASS [N,y,pass#]:

The column headings indicate the data displayed. "16609" indicates that the

data is for NORAD number 16609, the MIR Space Station. "AOS" is Acquisition

of Signal or when the satellite rises above the user's true horizon. "MAX

VISIBILITY" is the maximum altitude above the user's true horizon that the

satellite reaches during the pass. "LOS" is Loss of Signal or when the

satellite sets below the user's true horizon. "#" is an arbitrary pass

number for this set of calculations. The format for the data sent to the

destination device is slightly different from that displayed; see the Data

Mode 9 format description below.

The user may select either UTC/GMT or LOCAL date and times for pass

predictions. If other than UTC is selected, substitute the appropriate time

zone abbreviation as required. The date is given only for AOS; since passes

may span 00:00:00 hours for the selected time zone, the actual date for MAX

VISIBILITY and/or LOS may have to be incremented.

Times are calculated to the nearest second, altitudes are rounded to

the nearest degree, and azimuths are rounded to the nearest tenth of a

degree. Azimuth is always calculated in the sense NESW where North = 0

degrees, West = 90 degrees, etc. Note that the degree symbol will appear on

the display for all "Azm" and "Alt" data but has been deleted in the sample

above in order to maintain compatibility with various printers; the actual

display is thus four columns wider than the example above.

Press ENTER while the passes are being calculated to stop the

calculations. When all calculations for the current 48 hour block have been

completed, the time elapsed for the calculations is displayed and the user

is asked if another set of calculations is desired of if a particular pass

should be displayed. Press "N" or ENTER to return to the Main Menu, or

press "Y" to perform the next 48 hour block of calculations. The

calculations will be stopped when 99 passes have been listed. Typically,

from four to sixteen passes are listed for each 48 hour block, the number

being related to the characteristics of the current satellite's orbit. If

additional passes beyond that time are desired, set simulated time (F8+F3

from the Main Menu) to the desired start time and repeat the predictions.

To display a particular pass, enter the listed pass number. Passes are

assigned arbitrary numbers from 1 to 99 beginning with the first pass which

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 77

occurs during or subsequent to the current real or simulated time. Any pass

number may be selected, from pass #1 to the last listed pass shown on the

screen. (Attempting to enter a pass number larger than the last one shown

will cause the computer to "beep" and the prompt will be repeated.) STSPLUS

will set simulated time to approximately 30 seconds prior to the mid-point

of the selected pass and prepare the display. The pass may then be examined

for details of lighting, ground track, and so forth. STSPLUS displays "VIS"

next to the orbit inclination if a visual sighting may be possible. While

examining the pass, use F6 to PAUSE the display, then use the "+" or "-"

keys to adjust the simulated time forward or backward. The default time

step is one second; press F4 to select a different time step: 1, 10, or 60

seconds. Press ENTER to return to normal operation from the PAUSE mode.

Each time predicted passes are requested, the list begins with the

first pass which occurs at or subsequent to the current real or simulated

time. Note that STSPLUS automatically sets simulated time to display a

predicted pass, and that new simulated time remains in effect until the

user returns to the Main Menu, at which point the time is restored to the

real or simulated time in effect BEFORE the pass prediction was displayed.

Repeated use of pass predictions will therefore generally produce the same

list of passes; however, if sufficient time elapses between predictions

that a pass "comes and goes", new pass numbers will be displayed.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 78

Data Mode 1: Azimuth/Elevation Data Format

------------------------------------------

1         2         3         4         5

0123456789012345678901234567890123456789012345678901234

-------------------------------------------------------

02/10/1993  13:58:09  20580    -2.472   248.222    1675 [CR/LF]

-----+----  ----+---  --+--  ----+---  ----+--- ---+---

|          |       |        |         |       |

|          |       |        |         |       +--- Range

|          |       |        |         |

|          |       |        |         +----------- Aximuth

|          |       |        |

|          |       |        +--------------------- Elevation

|          |       |

|          |       +------------------------------ NORAD #

|          |

|          +-------------------------------------- UTC Time

|

+------------------------------------------------- UTC Date

UTC Date:      Current date in Universal Coordinated Time, mm/dd/yyyy

UTC Time:      Current time in Universal Coordinated Time, hh:mm:ss

NORAD #:       Satellite NORAD Number

Azimuth:       Azimuth in degrees to satellite in the sense NESW

Elevation:     Elevation to satellite in degrees above true horizon

Range:         Range from User Location to Satellite in km

[CR/LF]:       Each data line is terminated with a CR and LF in addition to

the 54 printing characters shown, for a total of 56

characters per data line.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 79

Data Mode 2: Latitude/Longitude Data Format

-------------------------------------------

1         2         3         4         5

0123456789012345678901234567890123456789012345678901234

-------------------------------------------------------

02/11/1993  13:46:40  20580    -5.182   155.667     593 [CR/LF]

-----+----  ----+---  --+--  ----+---  ----+--- ---+---

|          |       |        |         |       |

|          |       |        |         |       +--- Orbit Altitude

|          |       |        |         |

|          |       |        |         +----------- Longitude

|          |       |        |

|          |       |        +--------------------- Latitude

|          |       |

|          |       +------------------------------ NORAD #

|          |

|          +-------------------------------------- UTC Time

|

+------------------------------------------------- UTC Date

UTC Date:      Current date in Universal Coordinated Time, mm/dd/yyyy

UTC Time:      Current time in Universal Coordinated Time, hh:mm:ss

NORAD #:       Satellite NORAD Number

Latitude:      Geodetic Latitude in degrees of sub-satellite point

(satellite ground track)

Longitude:     Geodetic Longitude in degrees of sub-satellite point

(satellite ground track)

Orbit Alt:     Altitude in kilometers of the satellite above the Earth's

surface

[CR/LF]:       Each data line is terminated with a CR and LF in addition to

the 54 printing characters shown, for a total of 56

characters per data line.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 80

Data Mode 3: Topocentric RA/DEC Data Format

-------------------------------------------

1         2         3         4

01234567890123456789012345678901234567890123456

-----------------------------------------------

02/11/1993  13:47:20  20580     7.111   -25.941 [CR/LF]

-----+----  ----+---  --+--  ----+---  ----+---

|          |       |        |         |

|          |       |        |         +----------- DEC

|          |       |        |

|          |       |        +--------------------- RA

|          |       |

|          |       +------------------------------ NORAD #

|          |

|          +-------------------------------------- UTC Time

|

+------------------------------------------------- UTC Date

UTC Date:      Current date in Universal Coordinated Time, mm/dd/yyyy

UTC Time:      Current time in Universal Coordinated Time, hh:mm:ss

NORAD #:       Satellite NORAD Number

RA:            Topocentric Right Ascension in hours

DEC:           Topocentric Declination in degrees

[CR/LF]:       Each data line is terminated with a CR and LF in addition to

the 46 printing characters shown, for a total of 48

characters per data line.

NOTES:

1.   Topocentric coordinates give the right ascension and declination as

seen from the current user's location on the surface of the Earth.

2.   Coordinates are equator and equinox of date.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 81

Data Mode 4: Ascending Node X-Y-X State Vector

----------------------------------------------

STSORBIT PLUS Data Output to STSPLUS.LOG, Data = 4

Satellite Name:         MIR Space Station

Catalog Number:         16609  86 17  A

Epoch Date/Time:        93206.71622934028

07/25/1993  17:11:22.215 UTC

ECI X:                   6083.74442210995 km

Y:                   2969.71930867257 km

Z:                      0.01043524694 km

Xdot:                  -2.09290827983 km/sec

Ydot:                   4.27922666083 km/sec

Zdot:                   6.01892329735 km/sec

Ndot/2 (Drag):              0.00056174000

Nndot/6:                    0.00000000000

B-Star:                     0.00071196000

ElSet #:                  196.00000000000

Rev @ Epoch:            42514.00433526011

STSORBIT PLUS Data Output to STSPLUS.LOG, Data = 14

Satellite Name:         MIR Space Station

Catalog Number:         16609  86 17  A

Epoch Date/Time:        93206.71622934028

07/25/1993  17:11:22.215 UTC

ECI X:               19959752.12027331000 ft

Y:                9743154.40544174400 ft

Z:                     34.23630116129 ft

Xdot:               -6866.48346437341 ft/sec

Ydot:               14039.42991197058 ft/sec

Zdot:               19747.08480675116 ft/sec

Ndot/2 (Drag):              0.00056174000

Nndot/6:                    0.00000000000

B-Star:                     0.00071196000

ElSet #:                  196.00000000000

Rev @ Epoch:            42514.00523843931

STSORBIT PLUS Data Output to STSPLUS.LOG, Data = 24

Satellite Name:         MIR Space Station

Catalog Number:         16609  86 17  A

Epoch Date/Time:        93206.71622934028

07/25/1993  17:11:22.215 UTC

ECI X:                   3284.95919120368 nm

Y:                   1603.52014507239 nm

Z:                      0.00563458372 nm

Xdot:                  -1.13008006471 nm/sec

Ydot:                   2.31059754904 nm/sec

Zdot:                   3.24995858388 nm/sec

Ndot/2 (Drag):              0.00056174000

Nndot/6:                    0.00000000000

B-Star:                     0.00071196000

ElSet #:                  196.00000000000

Rev @ Epoch:            42514.00325144509

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 82

NOTES:

1.   The X-Y-Z Cartesian State Vector is given as a standard Earth-centered

inertial ("ECI") cartesian 6-dimensional state vector where the X-Axis

is pointing toward the vernal equinox, the Z-Axis is pointing toward

the North Pole, and the Y-Axis is mutually orthogonal to the other

axes in a right-handed axis system. All coordinates are for true

equator and equinox of date.

2.   The units of measure for the state vector may be determined by the

tens digit of the Data Mode in the initial header line as well as

being indicated with the data:

4   Kilometers and kilometers per second

14   Feet and feet per second

24   Nautical miles and nautical miles per second

3.   One data item is given per line, labeled as shown in the examples. The

data in the first four lines (Satellite Name, Catalog Number, and two

lines of Date/Time) begin in column 25. The remaining numeric data

items begin in column 21 and use a FORTRAN-like format statement

F21.11.

4.   The Catalog Number is given first as the NORAD Number ("16609" in the

example) and then as the IAU Designation ("86 17  A"). The IAU

Designation is separated from the NORAD Number by two spaces. Some 2-

line elements omit the IAU Designation, in which case that portion

will be blank.

5.   Note that the Date/Time is presented on two lines in two different

formats. The first format is the NASA Day-of-Year ("DOY") format,

YYDDD.DDDDDD, since that is the format used by NASA/JSC for X-Y-Z

state vectors for the Space Shuttle, in 2-line elements, and in

program VEC2TLE. In the DOY format, time is counted from midnight

(00:00 UTC) each day. Some calculations may require instead the Julian

Date format which counts time from noon (12:00 UTC) each day. The

Date/Time is also "decoded" and given in the more conventional

"MM/DD/YYYY  HH/MM/SS.SSS" format for clarity using Coordinated

Universal Time (UTC/GMT).

6.   The ElSet Number is specified in the 2-line elements used to generate

the ground track and is always given as an integer. Note that ElSet

Numbers may not necessarily follow in sequence and that different

sources will use different sequences of ElSet numbers.

7.   The Rev Number at Epoch (the orbit number at the time the data is

sampled) is based upon the Rev Number specified in the 2-line elements

used to generate the ground track. The fractional part is calculated

by STSPLUS geometrically from the ascending node. Note that US Space

Command uses a different orbit numbering convention than does NASA for

space shuttle missions; US Space Command usually specifies the first

(partial) orbit number as Rev 0, while NASA specifies that orbit

number as Rev 1. For satellites which have been in orbit for long

periods of time, the Rev Number may be arbitrary.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 83

Data Mode 5: Precision X-Y-Z Cartesian State Vector, 2 Data Lines

-----------------------------------------------------------------

STSORBIT PLUS Data Output to STSPLUS.LOG, Data = 5

20580  93110.043125      4920.98348         4440.02814        -2158.84295

-4.02147461570      5.78870948196      2.74131815428

20580  93110.043171      4904.85124         4463.14112        -2147.85724

-4.04461763461      5.76773962933      2.75148946765

STSORBIT PLUS Data Output to STSPLUS.LOG, Data = 15

20580  93110.045081  13656864.66720     17514322.54968     -5452252.42794

-16168.27686974290  15789.75251859515  10248.33566657315

20580  93110.045139  13575822.39276     17593013.13238     -5400930.14914

-16248.48336702945  15686.35334359047  10280.38786725583

STSORBIT PLUS Data Output to STSPLUS.LOG, Data = 25

20580  93110.046829      1817.50246         3234.50460         -631.72242

-3.02340746009      2.05326951871      1.82336359537

20580  93110.046991      1774.96638         3262.87575         -606.12299

-3.05304428709      1.99966800523      1.83360328215

NOTES:

1.   The X-Y-Z Cartesian State Vector is given as a standard Earth-centered

inertial ("ECI") cartesian 6-dimensional state vector where the X-Axis

is pointing toward the vernal equinox, the Z-Axis is pointing toward

the North Pole, and the Y-Axis is mutually orthogonal to the other

axes in a right-handed axis system. All coordinates are for true

equator and equinox of date.

2.   The units of measure for the state vector may be determined by the

tens digit of the Data Mode in the initial header line:

5   Kilometers and kilometers per second

15   Feet and feet per second

25   Nautical miles and nautical miles per second

3.   The NASA Day-of-Year format is used here for date and time since that

is the format used by JSC for X-Y-Z state vectors and also in 2-line

elements. In the DOY format, time is counted from midnight (00:00 UTC)

each day. Some calculations may require instead the Julian Date format

which counts time from noon (12:00 UTC) each day.

4.   Two successive data samples are shown for each data mode.

5.   The following FORTRAN-like format statements may be used to read the

two lines of data in this mode for all units of measure:

First Line:

-----------

Catalog #:     I5

2X

Date/Time:     F15.9

X:             F15.5

4X

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 84

Y:             F15.5

4X

Z:             F15.5

CR/LF

Second Line:

------------

23X

Xdot:          F18.11

1X

Ydot:          F18.11

1X

Zdot:          F18.11

CR/LF

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 85

Data Mode 6: Precision X-Y-Z Cartesian State Vector, Comma Delimited

--------------------------------------------------------------------

STSORBIT PLUS Data Output to STSPLUS.LOG, Data = 6

0,0,20580,93110.0476041667,2982.28779295502,6229.01725815628,-940.796339818487,

-5.85010701911522,3.3199940892324,3.46047048985284

0,0,20580,93110.0476851852,2941.25120957693,6252.07713790904,-916.54588610151,

-5.87454395677527,3.2685046500327,3.46815363454982

STSORBIT PLUS Data Output to STSPLUS.LOG, Data = 16

0,1,20580,93110.0482060185,8771281.06140276,20969911.6226162,-2491608.1429704,

-19762.0576284838,9623.02409449012,11524.6952898439

0,1,20580,93110.0482638889,8672341.7437806,21017717.5456987,-2433947.77946384,

-19813.4647395816,9499.28476766938,11539.2506632381

STSORBIT PLUS Data Output to STSPLUS.LOG, Data = 26

0,2,20580,93110.0493865741,1103.65585656622,3591.279279101,-214.476932551084,

-3.4057330597861,1.1598790254898,1.9344163566285

0,2,20580,93110.049537037,1059.27271053518,3605.99939807939,-189.308779132495,

-3.42231018081901,1.10471766926502,1.93751948609809

NOTES:

1.   The X-Y-Z Cartesian State Vector is given as a standard Earth-centered

inertial ("ECI") cartesian 6-dimensional state vector where the X-Axis

is pointing toward the vernal equinox, the Z-Axis is pointing toward

the North Pole, and the Y-Axis is mutually orthogonal to the other

axes in a right-handed axis system. All coordinates are for true

equator and equinox of date.

2.   The units of measure for the state vector may be determined by the

tens digit of the Data Mode in the initial header line as well as the

second parameter in the comma delimited data string:

Data Param

Mode    #2     Units

-----------------------------------------------------------

6     0     Kilometers and kilometers per second

16     1     Feet and feet per second

26     2     Nautical miles and nautical miles per second

3.   The comma delimited data are generated as a single line terminated by

CR/LF. The examples above have been split into two lines for printing

purposes.

4.   The data are written in a single data line in the following order,

separated by a comma between items:

Epoch Flag, always zero signifying equator and equinox of date.

Units Flag (see Note 1 above)

Catalog/NORAD number

Date (YYDDD.DDDDDDDD... format)

ECI X

ECI Y

ECI Z

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 86

ECI Xdot

ECI Ydot

ECI Zdot

5.   Line length will vary as a function of the data.

6.   Two successive data samples are shown for each data mode.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 87

Data Mode 7: Precision X-Y-Z Cartesian State Vector, Labeled Data

-----------------------------------------------------------------

STSORBIT PLUS Data Output to STSPLUS.LOG, Data = 7

Satellite Name:         MIR Space Station

Catalog Number:         16609  86 17  A

Epoch Date/Time:        93192.11956018518

07/11/1993  02:52:10.000 UTC

ECI X:                  -3441.20195444797 km

Y:                  -3110.29870646026 km

Z:                   4920.32069520120 km

Xdot:                   2.90216455238 km/sec

Ydot:                  -6.74909064951 km/sec

Zdot:                  -2.23710677970 km/sec

Ndot/2 (Drag):              0.00008567000

Nndot/6:                    0.00000000000

B-Star:                     0.00011546000

ElSet #:                  167.00000000000

Rev @ Epoch:            42286.31052536559

STSORBIT PLUS Data Output to STSPLUS.LOG, Data = 17

Satellite Name:         MIR Space Station

Catalog Number:         16609  86 17  A

Epoch Date/Time:        93192.12233796297

07/11/1993  02:56:10.000 UTC

ECI X:               -8617248.92526347200 ft

Y:              -15077459.08108566000 ft

Z:               13806887.09734187000 ft

Xdot:               12614.26230523560 ft/sec

Ydot:              -18214.71804776612 ft/sec

Zdot:              -12006.04438377176 ft/sec

Ndot/2 (Drag):              0.00008567000

Nndot/6:                    0.00000000000

B-Star:                     0.00011546000

ElSet #:                  167.00000000000

Rev @ Epoch:            42286.35385448637

STSORBIT PLUS Data Output to STSPLUS.LOG, Data = 27

Satellite Name:         MIR Space Station

Catalog Number:         16609  86 17  A

Epoch Date/Time:        93192.12280092592

07/11/1993  02:56:50.000 UTC

ECI X:                  -1333.74601094830 nm

Y:                  -2598.74977950943 nm

Z:                   2190.96899536823 nm

Xdot:                   2.14691236531 nm/sec

Ydot:                  -2.86693936350 nm/sec

Zdot:                  -2.09121400862 nm/sec

Ndot/2 (Drag):              0.00008567000

Nndot/6:                    0.00000000000

B-Star:                     0.00011546000

ElSet #:                  167.00000000000

Rev @ Epoch:            42286.36107600650

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 88

NOTES:

1.   The X-Y-Z Cartesian State Vector is given as a standard Earth-centered

inertial ("ECI") cartesian 6-dimensional state vector where the X-Axis

is pointing toward the vernal equinox, the Z-Axis is pointing toward

the North Pole, and the Y-Axis is mutually orthogonal to the other

axes in a right-handed axis system. All coordinates are for true

equator and equinox of date.

2.   The units of measure for the state vector may be determined by the

tens digit of the Data Mode in the initial header line as well as

being indicated with the data:

4   Kilometers and kilometers per second

14   Feet and feet per second

24   Nautical miles and nautical miles per second

3.   One data item is given per line, labeled as shown in the examples. The

data in the first four lines (Satellite Name, Catalog Number, and two

lines of Date/Time) begin in column 25. The remaining numeric data

items begin in column 21 and use a FORTRAN-like format statement

F21.11.

4.   The Catalog Number is given first as the NORAD Number ("16609" in the

example) and then as the IAU Designation ("86 17  A"). The IAU

Designation is separated from the NORAD Number by two spaces. Some 2-

line elements omit the IAU Designation, in which case that portion

will be blank.

5.   Note that the Date/Time is presented on two lines in two different

formats. The first format is the NASA Day-of-Year ("DOY") format,

YYDDD.DDDDDD, since that is the format used by NASA/JSC for X-Y-Z

state vectors for the Space Shuttle, in 2-line elements, and in

program VEC2TLE. In the DOY format, time is counted from midnight

(00:00 UTC) each day. Some calculations may require instead the Julian

Date format which counts time from noon (12:00 UTC) each day. The

Date/Time is also "decoded" and given in the more conventional

"MM/DD/YYYY  HH/MM/SS.SSS" format for clarity using Coordinated

Universal Time (UTC/GMT).

6.   The ElSet Number is specified in the 2-line elements used to generate

the ground track and is always given as an integer. Note that ElSet

Numbers may not necessarily follow in sequence and that different

sources will use different sequences of ElSet numbers.

7.   The Rev Number at Epoch (the orbit number at the time the data is

sampled) is based upon the Rev Number specified in the 2-line elements

used to generate the ground track. The fractional part is calculated

by STSPLUS geometrically from the ascending node. Note that US Space

Command uses a different orbit numbering convention than does NASA for

space shuttle missions; US Space Command usually specifies the first

(partial) orbit number as Rev 0, while NASA specifies that orbit

number as Rev 1. For satellites which have been in orbit for long

periods of time, the Rev Number may be arbitrary.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 89

Data Mode 9: Pass Predictions

-----------------------------

--------#20580 AOS--------  --MAX VISIBILITY--  ------LOS------

#   UTC Date UTC Time    Azm  UTC Time Alt   Azm  UTC Time    Azm Duration

1 02/27/1993 03:58:09  205.1  04:03:00   9 157.9  04:07:52  110.8  0:09:43

-+ ---------+---------  --+--  ----+---  -+ --+--  ----+---  --+-- ----+---

|          |             |        |      |   |        |       |       |

Pass #     |             |        |      |   |        |       |       |

|             |        |      |   |        |       |       |

+----------+             |        |      |   |        |       |       |

|                        |        |      |   |        |       |       |

|  +---------------------+        |      |   |        |       |       |

|  |                              |      |   |        |       |       |

|  +----- AOS Azimuth (degrees)   |      |   |        |       |       |

|                                 |      |   |        |       |       |

+-------- AOS Date/Time           |      |   |        |       |       |

(mm/dd/yyyy hh:mm:ss)   |      |   |        |       |       |

|      |   |        |       |       |

+---------------------------------+      |   |        |       |       |

|                                        |   |        |       |       |

|  +-------------------------------------+   |        |       |       |

|  |                                         |        |       |       |

|  |  +--------------------------------------+        |       |       |

|  |  |                                               |       |       |

|  |  +-- MAX Azimuth (degrees)                       |       |       |

|  |                                                  |       |       |

|  +----- MAX Altitude (degrees)                      |       |       |

|                                                     |       |       |

+-------- MAX Time (hh:mm:ss)                         |       |       |

|       |       |

+-----------------------------------------------------+       |       |

|                                                             |       |

|  +----------------------------------------------------------+       |

|  |                                                                  |

|  +----- LOS Azimuth (degrees)                                       |

|                                                                     |

+-------- LOS Time (hh:mm:ss)                                         |

|

Total Pass Duration (hh:mm:ss)  --+

NOTES:

1.   When the Data Output mode is set up for pass predictions, dates and

times for pass predictions may be selected for UTC/GMT or local time

and the selected time zone abbreviation and time scale will be used.

Substitute the appropriate abbreviation for "UTC" as required if other

than UTC has been selected. When UTC or GMT is selected, the Data Mode

will be given as "9"; when local time is selected, the Data Mode will

be given as "19".

2.   The Pass # is an arbitrary number assigned by STSPLUS during the pass

calculations and is a function of the real or simulated time at which

the calculations are performed. If the real or simulated time is

changed, the pass numbers may change and different passes may be

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 90

shown.

3.   The satellite NORAD number is included in the heading, "20580" in the

sample above.

4.   The Date (UTC/GMT or local) is given only for AOS. Since a pass may

span 00:00:00 hours, the date for MAX VISIBILITY and/or LOS may have

to be incremented from that shown for AOS.

5.   All azimuths ("Azm") have been rounded to the nearest 0.1 degree; the

MAX VISIBILITY altitude ("Alt") has been rounded to the nearest

degree.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 91

F4     Calculate Satellite Positions with TRAKSTAR

--------------------------------------------------

Tabular predictions for the currently selected satellite may be made

by using TRAKSTAR by Dr. TS Kelso. STSPLUS has been arranged to operate

seamlessly with TRAKSTAR by simply pressing Function Key F4 from the Main

Menu. All data required by TRAKSTAR is automatically supplied by STSPLUS.

See the section above "Predicting Satellite Passes with TRAKSTAR" and the

TRAKSTAR documentation for full information.

F5     Set Launch Time and Date

-------------------------------

Orbital data for the satellite must be loaded using the F2 command on

the Main Menu before the launch date and time may be set or changed. Once

saved in file STSPLUS.LTD, the launch date and time will be automatically

read from that file each time the satellite is selected (see below).

********************

*  IMPORTANT NOTE  *

********************

LAUNCH TIME AND DATE MUST EITHER BE INCLUDED IN FILE STSPLUS.LTD

OR BE MANUALLY ENTERED FOR EACH SATELLITE OR MISSION SINCE THAT

INFORMATION IS NOT INCLUDED IN 2-LINE ELEMENTS.

Launch date and time are most important for manned missions such as

the Space Shuttle since the mission timeline is reckoned using Mission

Elapsed Time. However, MET may be used whenever actual launch date and time

are known. The only requirement is that 2-line orbital elements must be

available for the satellite. Since launch date and time are NOT included in

the 2-line orbital elements, this means that you obtain the launch date and

launch time independently and manually enter that data.

Pressing F5 to enter launch date and time begins with the prompt:

Enter Launch Time (HH:MM:SS):

[Add 'U'or 'G' for UTC/GMT]

Enter the time in the format shown using 24-hour notation. Add the letter

"U" to signify UTC (Coordinated Universal Time) or the letter "G" to

signify GMT (Greenwich Mean Time, essentially identical to UTC for the

purposes of this program). Use no suffix for local time; it will be

internally converted to UTC/GMT. If you enter "U" or "G", the abbreviation

used in the menus will be set to "UTC" or "GMT" respectively. One or two

digit numbers may be used as required. [The comma is also acceptable as a

separator in place of the colon.] You may omit seconds [or minutes and

seconds] if desired. For example, an entry of "16" will be entered as

16:00:00 or 4:00 PM. Press ENTER to leave the entry unchanged.

Enter Launch Date (MM/DD/YYYY):

[Enter '*' to clear LAUNCH DATE]

Enter the date in the format shown. Note that if you requested UTC or GMT

when entering the time, the date is interpreted as the UTC/GMT date. The

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 92

full four digit year may be used OR two digits as in "92". Be sure to use

the SLASH "/" rather than the MINUS "-" as the separator; STSPLUS's

internal date algorithms will interpret the minus sign as just that and

some rather strange dates can result! You may also use relative dates: -1

will use the prior day, +2 will use two days hence, and so forth. Press

ENTER to leave the date unchanged. Press "*" (followed by ENTER) to clear

the launch date and time; this does NOT remove it from file STSPLUS.LTD.

Press ENTER to accept, SPACE BAR to repeat:

When all data have been entered, the program pauses for your approval. If

all data are correct, press ENTER. Press the SPACE BAR to start over. You

are next asked if you wish to save this data:

Add/Update this data in file STSPLUS.LTD [Y,n]:

Press "Y", "y" or ENTER to add or update the data in file STSPLUS.LTD (see

below). Press "N" or "n" to use the data but not add or update it in file

STSPLUS.LTD. Adding or updating the data to file STSPLUS.LTD makes sure

that the launch date and time data will be available the next time this

particular satellite is selected.

Using File STSPLUS.LTD for Launch Date & Time

---------------------------------------------

An alternative and automatic method to set launch date and time is to

use file STSPLUS.LTD. This file contains the NORAD number and launch date

and time (UTC Julian date) for selected satellites. A sample entry appears

as:

22194,2448918.21503472,0

--+-- -------+-------- +

|          |         |

|          |         +--- (Reserved, must be present)

|          |

|          +------------- Launch Date (UTC Julian date)

|

+------------------------ NORAD Number

The Sample above shows the data for Space Shuttle mission STS-52 (NORAD

#22194) and corresponds to a launch date and time of 22 OCT 1992 @ 17:09:39

UTC. The file is in standard ASCII format and may be edited with any

standard editor; word processor users use the "non-document" mode. Use care

when manually editing the file as STSPLUS performs NO ERROR CHECKING!

Estimated 2-line orbital elements are usually available prior to a

Space Shuttle launch, and actual 2-line orbital elements within about 8 to

12 hours after a launch. Note, however, that the NORAD number is not

assigned until actual launch and a "dummy" NORAD number is used for

estimated pre-launch elements; for example, "00052" for STS-52. Once the

launch has taken place, the permanent NORAD number is assigned. This change

in NORAD number will require either that file STSPLUS.LTD be edited OR that

a new entry be made. Note that there are still a few satellites around with

low NORAD numbers!

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 93

If file STSPLUS.LTD is present and if the selected satellite is found,

the launch time and date will be set and Mission Elapsed Time (MET) will be

used automatically; otherwise, T+Epoch (T+E) will be used. When MET is

displayed, it may be changed to T+E by pressing F6 from the Main Menu

(below) or F5 while the map is displayed.

File STSPLUS.LTD is read each time a new satellite is selected using

F2 from the Main Menu. If file STSPLUS.LTD is NOT present, the launch date

and time will be saved in file STSPLUS.INI and must be MANUALLY MAINTAINED

as in versions prior to 9245! If you wish to use the old method, rename or

delete file STSPLUS.LTD.

F6     Set/Read/Save TDRS and Real Time Satellites

--------------------------------------------------

STSPLUS is able to display up to 32 additional TDRS (or other

geosynchronous/geostationary) or Real Time satellites. The first menu

selection allows the user to select the desired satellite for each of the

sixteen available "slots" as well as abbreviation, mode, and icon/label

color. Note that these features are enabled and disabled using F10+F3+F2

from the Main Menu. The following menu is displayed:

TDRS and Real Time Satellite Maintenance Menu

F1  Display/Modify satellite assignments

F2  Save SCF Satellite Configuration File

F3  Read SCF Satellite Configuration File

F4  Clear Static and Real Time Satellites

Press function key for desired menu or ENTER for Main Menu:

Press ENTER to return to the Main Menu, or press the desired function key

to perform the indicated item.

Before the current position of each satellite can be calculated, 2-

line elements must be read or updated using F2 from the Main Menu. After

this has been performed once, the position will be calculated based upon

those 2-line elements. The user is reminded that the 2-line elements should

be updated periodically so that the calculated position accurately reflects

the actual position.

NOTE:     Users of prior versions of STSPLUS will note that Function Key F6

has been reassigned for its present function. Switching between

MET and T+Epoch may now be accomplished ONLY by pressing F5 while

the ground track is displayed.

F1  Display/Modify Satellite Assignments

----------------------------------------

This menu selection displays the current TDRS and Real Time Satellite

assignments and option selections as a table:

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 94

--------------------------------------------------------------------------

TDRS and Real Time Satellite Setup

Sat#  NORAD#   Abbr        Mode  Color Size  Label  Vis        Color Chart

1   19883   TDRE      Static   10 o    3     ON

2   21639   TDRW      Static   10 o    3     ON               1 = XXXXX

3   22314   TDR5      Static    2 o    3     ON  OFF          2 = XXXXX

4   19548   TDR2      Static    2 o    3     ON  OFF          3 = XXXXX

5   13969   TDR1      Static    2 o    3     ON  OFF          4 = XXXXX

6   16609   MIR    Real Time   12 o          ON   ON          5 = XXXXX

7   21701   UARS   Real Time   13 o          ON  OFF          6 = XXXXX

8   20580   HST    Real Time   14 o          ON  OFF          7 = XXXXX

9   20638   ROSAT  Real Time   13 o          ON  OFF          8 = XXXXX

10   22076   TOPEX  Real Time   13 o          ON  OFF          9 = XXXXX

11       0         (not used)                                 10 = XXXXX

12       0         (not used)                                 11 = XXXXX

13       0         (not used)                                 12 = XXXXX

14       0         (not used)                                 13 = XXXXX

15       0         (not used)                                 14 = XXXXX

16       0         (not used)                                 15 = XXXXX

Enter Satellite # to edit or press ENTER when done:

--------------------------------------------------------------------------

NOTE:     The first two satellite slots, #1 and #2, are reserved for the

standard TDRS East and TDRS West satellites, currently NORAD

Numbers 19833 and 21639. Instead of conventional circles of

visibility, these two satellites generate commumications coverage

circles. Assigning different satellites, especially satellites

which are not geostationary, may produce unpredictable results.

The Satellite Setup table headings indicate:

Sat#      Satellite Assignment Number, 1 to 16.

NORAD#    The NORAD Number of the assigned satellite, or "0" if no

satellite is assigned to this slot.

Abbr      The five character abbreviation assigned to this satellite.

Use only letters and numbers; the common satellite name or

NORAD Number are the recommended choices. This abbreviation

will be used as a label on the map if the Label is ON.

Mode      Three classifications are available: "Static" for

geosynchronous or geostationary satellites; "Real Time" for

satellites to be tracked dynamically; and, "(not used)" for

unassigned slots.

Color     A number selected from the Color Chart at the right which

indicates the color to be used to display the satellite icon

and label (if enabled). Each "XXXXX" in the Color Chart

appears on color monitors as a bar of the corresponding

color. The "o" character to the right of the number is

displayed as a small diamond in the selected color.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 95

Size      For Static satellites only, selects the relative size of the

icon used to represent the satellite. Values may range from

3 to 6. This column is blank for Real Time satellites since

the icon size is fixed for these satellites.

Label     May be set to "ON" or "OFF" to indicate if the satellite

abbreviation is to be displayed. The label color will be the

same as that selected for the satellite.

Vis       May be set to "ON" or "OFF" to indicate if the satellite

circle of visibility is to be drawn. The circle of

visibility is drawn only for Static satellite on normal maps

but is drawn for all satellites on Motion maps.

Color     The color chart displays the 15 available icon and label

Chart     colors (BLACK is omitted). When a monochrone display is used

and the "/M" command line option is specified, two levels of

gray will normally be displayed instead of colors. Each

color is assigned a standard color number which is used for

all data entries.

As indicated by the prompt, simply press ENTER if all data in the

Satellite Setup is correct. You will return to the F6 menu above. If you

wish to add a satellite or change the selections for an existing satellite,

enter the satellite slot number (Sat# in the left column of the table). The

following prompts will be displayed in turn:

Enter NORAD Number:

(Enter 0 to clear the satellite entry)

STSPLUS expects the NORAD number for the desired satellite. This number

will be used to "capture" the 2-line elements when this satellite is

subsequently displayed.

Enter Satellite Abbreviation:

Enter any desired abbreviation, 5 characters or less, or the NORAD number.

Use only letters, numbers, and the period ("."), dash ("-"), and comma

(",") with no leading spaces.

Enter Mode (0=STATIC, 1=REAL TIME):

Enter "0" or "1". STATIC mode satellites are drawn each time the map is

redrawn on the screen. REAL TIME satellites are updated every 10 seconds or

as often as the processor can complete the required calculations.

Enter satellite color (1 to 15):

Enter the number for the desired color, as shown on the Color Chart at the

right of the screen. An illegal color number will default to YELLOW.

Display satellite Label (0=NO, 1=YES):

Enter "0" or "1". The label will be displayed directly below the satellite

icon in the same color as that selected for the icon in the preceeding

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 96

step. For Real Time satellites, the label is only drawn when the map is

redrawn and remains stationary EXCEPT when using the Motion Map (in which

case it follows with the icon).

Display Cir of Visibility (0=NO, 1=YES):

Enter "0" or "1". The circle of visibility is drawn (when enabled) for all

Static satellites, and also for all Real Time satellites when using the

Motion Map.

When a new satellite is entered, a double asterisk ("**") will appear

at the right for that satellite slot. This is to remind the user to display

the ground track for that satellite so that 2-line elements may be stored

in the TLE memory associated with that slot. The following message will

also appear below the chart:

**  Display ground track for this satellite!

To display the ground track, press F2 from the Main Menu, select the file

of 2-line elements which includes the satellite in question, and enter the

NORAD Number ("#nnnnn", where "nnnnn" is the NORAD Number). Once the

satellite has been displayed on the map, the 2-line elements will be saved

for future use.

Although 2-line elements are saved for all TDRS and Real Time

satellites, the user is reminded that these elements have a limited

lifetime. That lifetime varies considerably from satellite to satellite.

For geosynchronous satellites, 4 to 8 weeks is probably reasonable; for

other active satellites, 1 to 2 weeks should be an upper limit before new

elements are used. Use F2 from the Main Menu and enter the satellite name

as "&" to scan through the 2-line elements file and update all elements

which are more recent than those now saved.

F2  Save SCF Satellite Configuration File

-----------------------------------------

Users may wish to switch among several sets of TDRS and Real Time

satellites from time to time. Since it can be tedious to re-enter all the

information repeatedly, STSPLUS can save and read Satellite Configuration

Files, identified by the filetype .SCF. All parameters are saved and

restored, exactly as if entered at the time. The current satellite

configuration is automatically saved in file STSPLUS.INI each time the

program is exited and is restored when the program is restarted. The

following prompt is displayed:

Save SCF Satellite Configuration File

Enter FILENAME (without .SCF filetype), ENTER to quit:

Enter any desired filename WITHOUT the .SCF filetype; STSPLUS automatically

appends the filetype and will reject any filename which includes a

filetype. Filenames may be no longer than 8 characters.

If the file already exists, STSPLUS will display a warning message:

File STANDARD.SCF already exists! Overwrite [y/N]:

where "STANDARD.SCF" will be the requested file including the .SCF

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 97

filetype. Press "Y" or "y" to allow the existing file to be overwritten;

press any other key to cancel the write operation, and then press ENTER to

resume program operation. If the file does not exist, it will be created.

Do not use filename STSPLUS since a sample file by that name is included

with the distribution package (unless you wish to overwrite the supplied

file).

F3  Read SCF Satellite Configuration File

-----------------------------------------

See the comments above for saving SCF files. The following prompt is

displayed:

Read SCF Satellite Configuration File

Enter FILENAME (without .SCF filetype), ENTER to quit:

Available .SCF files are:

STANDARD.SCF

STSPLUS.SCF

STSPLUS displays a list of available SCF files, up to a maximum of 90

files. Press ENTER to cancel the operation. Type the desired filename

WITHOUT the .SCF filetype; STSPLUS automatically appends the filetype and

will reject any filename which includes a filetype. Filenames may be no

longer than 8 characters. If the file does not exist, an error message will

be displayed.

F4  Clear Static and Real Time Satellites

-----------------------------------------

Use this function to clear all static and real time satellites,

numbers 3 through 32. STSPLUS asks the user to confirm the operation to

avoid accidental clearing.

NOTE: Slots 1 and 2 are reserved for TDRS satellites and should ALWAYS

contain TDRS or similar geosynchronous satellites. The program may perform

unpredictably if these slots are cleared or have other types of satellites.

F7     Set FILENAMES and PATHS

------------------------------

Function Key F7 allows the user to select the paths and/or filenames

for the various files that STSPLUS uses to select satellites, tracking

stations, and other features:

Select path or filename to set, press ENTER when done:

F1  Set 2-LINE ELEMENTS path:      [I:\TLE\]

F2  Set TRACKING STATION filename: [STSPLUS.TRK]

F3  Set MAP DATABASE FILES path:   [D:\MAPDATA\]

F4  Set FEATURES LABEL filename:   [STSPLUS.LOC]

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 98

F5  Set TRAKSTAR path:             [D:\STSPLUS\]

F6  Set CITYFILE filename:         [STSPLUS.CTY]

Enter selection or ENTER:

Press the indicated function key for the item you wish to change. The

current path or filename is shown in square brackets for each selection.

Press ENTER to leave a path or filename unchanged. The following is a

typical prompt for filename:

Enter TRACKING STATION filename: _

(Press ENTER to leave unchanged)

For filenames, enter the full filename including filetype. A drive and

directory may also be included if desired. If no filetype is entered,

STSPLUS will automatically supply ".TRK" for tracking station files, and

".LOC" for features label files. If the desired file has no filetype,

include the period in the filename entered (e.g. "STATION.") to prevent the

automatic addition of a filetype.

For the path selections, enter the desired drive and subdirectory. The

trailing backslash will automatically be added if it is omitted. If the

path cannot be found, an error message will be displayed and the path will

default to the current drive and directory. For best performance, use a RAM

disk for Map Database Files; see the section "Using a RAM Disk" for further

information.

After each entry, the Path and Filenames Menu is again displayed with

the current selections. Press ENTER when done to return to the STSPLUS Main

Menu.

F8     Set Program TIME and DATE

--------------------------------

This menu provides a number of time and date functions for use

with STSPLUS. The program clock may be set to real or simulated time

using several methods, current clock corrections applied by program

RIGHTIME may be displayed, and the UTC OFFSET and DAYLIGHT Flag may be

adjusted.

It is often convenient to set the TIME and DATE within STSPLUS to

something other than the current system time and date, or to return to the

current system time and date if the program time and date have been

changed. Press F8 to go to the TIME and DATE Menu:

Program STSORBIT PLUS

Space Shuttle and Satellite Orbit Simulation

Version 9338

Current time:  14:52:24 PST   22:52:24 UTC

Current date:    03/10/1993     03/10/1993

ACTUAL SYSTEM DATE AND TIME SHOWN ABOVE

(Assisted by RIGHTIME)

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 99

F1     Restore SYSTEM date and time (use "real time")

F2     Set DOS SYSTEM CLOCK using calendar date and time

F3     Set SIMULATED date and time using calendar date and time

F4     Set SIMULATED date and time using Mission Elapsed Time

F9     Display Current RIGHTIME Corrections

F10    Set UTC OFFSET and DAYLIGHT Flag

ENTER  Return to MAIN MENU

Select desired function:

The Date and Time Menu, shown above, displays the available time

setting functions along with the actual system date and time as determined

by the DOS software clock in your computer (even if simulated time is in

effect). If program RIGHTIME Version 2.5+ is currently enabled, the message

"(Assisted by RIGHTIME)" will also appear. Both your local date and time,

"PDT" or Pacific Daylight Time in the example, and "UTC" (Coordinated

Universal Time) date and time are displayed. If times have been set using

the letter "G", the abbreviation at the right will be "GMT" (Greenwich Mean

Time).

Press ENTER to return to the Main Menu with the date and time as

displayed on the screen (Current or Simulated).

If you wish to execute STSPLUS in "real time", cancelling any

simulated time that may be in effect, use the F1 command. This will restore

the time and date used for the tracking display to that shown at the top of

the menu. If the actual system date or time displayed is incorrect, use

program TIMESET (if available) or the F2 command to correctly set your

system clock.

Some organizations, NASA for example, continue to use the wording

"Greenwich Mean Time" or "GMT" for what is now usually referred to as

"Coordinated Universal Time" or "UTC" (and sometimes, depending upon the

application, as "UT", "UT1" or "UT2"). STSPLUS uses Coordinated Universal

Time or "UTC", the time used for civil timekeeping and broadcast by radio

stations such as WWV and the BBC. Although technically these different time

standards are not exactly the same, the difference is only a maximum of 0.9

seconds and the program treats them all as identical. STSPLUS defaults to

the abbreviation "UTC" but if you prefer to use "GMT", enter any simulated

time using F3 and include the letter "G" (upper or lower case) at the end.

The time abbreviation at the top of the screen will change from "UTC" to

"GMT" and will continue using that abbreviation until a time is entered

suffixed with "U".

Times are always entered as "HH:MM:SS" where HH is HOURS, MM is

MINUTES, and SS is SECONDS. The time entry format is very flexible. Leading

zeroes are not required. The comma (",") may be used in place of the colon

(":") as a separator if desired. SECONDS or MINUTES and SECONDS may be

omitted if desired. Time entries are assumed to be local time; to enter UTC

or GMT times, add the letter "U" or "G" (upper or lower case) respectively

following the entry. For example, the following are valid time entries:

Entry          Interpreted as

------         --------------------------

12             12:00:00 (LOCAL TIME ZONE)

13,1           13:01:00 (LOCAL TIME ZONE)

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 100

4:1:15         04:01:15 (LOCAL TIME ZONE)

1,1,1          01:01:01 (LOCAL TIME ZONE)

13,45U         13:45:00 UTC

1:20g          01:20:00 GMT

Dates are entered as "MM/DD/YYYY" or "MM/DD/YY" where MM is MONTHS (as

a number from 1 to 12), DD is DAYS, and YYYY is the full four-digit YEAR or

YY is the last two digits of the YEAR. Except for the two digit year

option, the full date must always be entered; leading zeroes are not

required. The date entered is assumed to be for the same time zone as the

time entered. If local time is entered, the date will be treated as the

local date; if UTC (or GMT) time is entered, the date will be treated as

the UTC/GMT date.

After a time or date entry has been accepted (after you press the

ENTER key), STSPLUS reformats the entry to its standard format, clears the

characters you entered, and replaces them by the standard format in both

local and UTC/GMT time zones. This provides a double check that the program

has interpreted your entry as you wished.

F1     Restore System Date and Time

-----------------------------------

Press F1 to restore the program date and time to the system date and

time. This command reads the DOS clock and restores the program to "real

time" operation. If the program date and time have not been changed with

the F3 or F4 commands, this command will have no effect.

F2     Set DOS System Clock

---------------------------

Press F2 to set the DOS system clock. Use this command if you wish to

change the actual date and time on your system. Note that on many systems

using DOS 3.3 or higher, this command will set BOTH the software clock AND

the hardware clock.

*************

*  CAUTION  *

*************

This function should NOT be used when program RIGHTIME is

regulating the DOS clocks UNLESS no other method is available.

Use program TIMESET to set the DOS clocks accurately instead!

Program STSORBIT PLUS

Space Shuttle and Satellite Orbit Simulation

Version 9338

Current time:  19:59:10 PST   03:59:10 UTC

Current date:    02/24/1993     02/25/1993

CAUTION: This function will change the computer's SYSTEM CLOCK!

Press ENTER to leave an item unchanged

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 101

Enter TIME (HH:MM:SS):                16:34:24 PST

Enter DATE (MM/DD/YYYY):                 4/08/1992

Press ENTER to accept, SPACE BAR to repeat: _

The sample above shows the screen after the time and date entries have been

completed. The current ACTUAL system date and time are displayed for

approval. Press ENTER to accept the time and date displayed, or press the

SPACE BAR to repeat the entries.

F3     Set Simulated Date and Time using Calendar Method

--------------------------------------------------------

Press F3 to set a simulated date and time. The date and time may be

either in the past or in the future. This command does NOT affect the DOS

clock in your system! Use the F1 command above to restore the date and time

to "real time".

Program STSORBIT PLUS

Space Shuttle and Satellite Orbit Simulation

Version 9338

Current time:  19:59:10 PST   03:59:10 UTC

Current date:    02/24/1993     02/25/1993

Press ENTER to leave an item unchanged

Enter SIMULATED TIME  [12:05:06]:     13:00:00 PST   21:00:00 UTC

Enter SIMULATED DATE  [11/11/1991]:     11/09/1991     11/09/1991

Press ENTER to accept, SPACE BAR to repeat: _

The sample above shows the screen after the time and date entries have been

completed. The new SIMULATED date and time are displayed (and counting) for

approval. Press ENTER to accept the time and date displayed, or press the

SPACE BAR to repeat the entries.

F4     Set Simulated Date and Time using MET

--------------------------------------------

Press F4 to set a simulated date and time using MET (Mission Elapsed

Time). Note that this command will appear ONLY if the mission name begins

with the letters "STS", signifying a Space Transportation System (Space

Shuttle) mission AND if a launch time and date have previously been

entered.

Program STSORBIT PLUS

Space Shuttle and Satellite Orbit Simulation

Version 9338

Simulated time:  00:17:18 PST   08:17:18 UTC

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 102

Simulated date:    10/09/1992     10/09/1992

Enter desired Mission Elapsed Time (MET)

Enter MET DAY (NN):             3 day(s)

Enter MET TIME (HH:MM:SS):      04:30:00 MET

Press ENTER to accept, SPACE BAR to repeat: _

The sample above shows the screen after the day and time entries have been

completed. The Mission Elapsed Time is immediately converted to actual date

and time and the current SIMULATED date and time, based upon the MET just

entered, are then displayed (and counting) for approval. Press ENTER to

accept the time and date displayed, or press the SPACE BAR to repeat the

entries.

F9     Display Current RIGHTIME Corrections

-------------------------------------------

If program RIGHTIME has been detected, the "F9" menu item will be

displayed and you may press F9 to display the time since the last TIMESET,

the current WARM correction, and the current COOL correction:

RighTime Version 2.53 detected!

Time Since Last TIMESET:   0 days 08:58:20

Current WARM Correction:   -0.01 seconds

Current COOL Correction:   -0.35 seconds

Press any key to continue ... _

The version of RIGHTIME is displayed. The time since the last TIMESET is

saved by program RIGHTIME to the nearest 200 seconds and will therefore not

change until that increment is reached. The time is shown as days followed

by hours:minutes:seconds. If more than 7 days has elapsed since the last

TIMESET, an additional message "(TIMESET suggested!)" will also appear. The

WARM and COOL corrections are shown and are updated when the system time is

set using Function Key F1 from this menu, by program TIMESET, or by other

means. Press any key, such as ENTER, to return to the Time and Date Menu.

F10    Set UTC OFFSET and DAYLIGHT Flag

---------------------------------------

You may change the UTC Offset and/or the setting of the Daylight Flag

using F10. The prompts are self-explanatory; see the section "Program

Options and Features, Set UTC Offset and Daylight Flag" for a detailed

description of the UTC Offset and Daylight flag.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 103

F9     DOS Shell (CAUTION: DOS Version 3.x+ ONLY!)

--------------------------------------------------

If a system function is desired at the Main Menu, press F9 to execute

BASIC's DOS SHELL function. This will return you to a DOS prompt and most

DOS commands may be executed immediately. When the Shell is executed,

STSPLUS remains in memory and the map data will not be re-read when you

return. All files used by STSPLUS are closed. However, this means that a

substantial amount of memory is in use and not available to DOS during the

shell operations. Enter "EXIT" (without the quotation marks and followed by

ENTER) at the DOS prompt when you wish to return to STSORBIT.

CAUTION: The BASIC SHELL function is only reliable for versions of DOS

of 3.0 or higher! Systems with less than 640K memory may fail to

execute the shell and applications requiring large amounts of memory

may also fail.

F10    Set STSORBIT PLUS Program Options and Features

-----------------------------------------------------

A number of program features and display options are set using the F10

Program Options Menu. These selections are further described in the section

"Program Options Menu" below.

ENTER  Resume Mission

---------------------

Pressing ENTER resumes the current mission shown in parentheses to the

right of the command on the Main Menu. The 2-line elements file from which

the data was read is shown in square brackets.

ENTER  Resume Mission                  (STS-41 [STS41F])

Any manually entered data is retained. "ENTER" means the key marked ENTER,

RETURN, or with a left pointing arrow -- but not the backspace or cursor

position keys which may also be marked with arrows! (I am afflicted with

too long a memory; once upon a time this function was known as Carriage

Return and was often shortened to RETURN or even CR. With the advent of

electronic typewriters, video terminals, dot matrix printers and all the

rest, "carriages" have long since disappeared but old habits die hard! Most

PC keyboards are now marked with "ENTER".)

Use ENTER to resume plotting a mission in progress after returning to

the Main Menu to perform some change (such as enabling the node display,

enabling the NASA tracking stations, or adjusting the time or date).

ESC    Quit STSORBIT PLUS and Save Current Mission

--------------------------------------------------

Press ESC (the key marked "ESC" or "Esc", not the letters E+S+C) to

quit program STSORBIT PLUS. If you press ESC to quit the program and have

manually entered orbital data, STSPLUS will save all required mission data

in file STSPLUS.INI prior to terminating. This will be the data available

with the ENTER key the next time you execute the program. The demonstration

data will not be saved, preserving any previously saved mission data.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 104

When you have finished with STSPLUS, press ESC at the Main Menu to

return to DOS. The data (and any adjustments you have made) for the current

mission are saved in file STSPLUS.INI, but the map data is lost and will be

re-read when you next use program STSPLUS.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 105

PROGRAM OPTIONS AND FEATURES MENU

---------------------------------

A number of program features and display options are set using the F10

Program Options Menu. When used with CGA displays, the features shown below

as selected by function keys F3 and F4 are not available because of the low

resolution of the CGA display. The following Options Menu is displayed when

the F10 command is entered from the Main Menu:

Program STSORBIT PLUS

Space Shuttle and Satellite Orbit Simulation

Version 9338

Current time:  19:59:10 PST   03:59:10 UTC

Current date:    02/24/1993     02/25/1993

F1             Program STSORBIT PLUS Information

F2             Set New Local Coordinates (Rancho Palos Verdes, CA)

F3             Select Display Features

F4     A/A     Select Satellite Coordinates: RA/DEC, Alt/Az or XYZ

F5     OFF     Show Ascending & Descending Node Data

F6     ORTHO   Set Map Projection and Size

F7     OFF     Enable/Disable EVENT TIMERS

F8     ON      Enable/Disable Audible ALARMS

F9    -7.00    Set UTC Time Offset and Daylight Flag

F10    OFF     Enable/Disable Printer Logging

ENTER          Return to MAIN MENU

Select desired function:

F1     Program STSORBIT PLUS Information

----------------------------------------

Function Key F1 displays information about program STSORBIT PLUS

including the copyright notice, version number, my name and address, and

the telephone number of my RPV ASTRONOMY BBS (Bulletin Board System). The

current version of STSORBIT PLUS is always posted on the BBS. The BBS has a

power controller; if it hasn't answered after the THIRD RING, hang up and

call back in two minutes. The BBS is available 24 hours per day at 9600,

2400 and 1200 baud.

F2     Set New Local Coordinates

--------------------------------

In order to perform the calculations related to satellite visibility

and altitude/azimuth, STSPLUS must know the geographic coordinates for the

user's location. The name of the current location is shown in parentheses.

When STSPLUS is first started, the default coordinates are set to Rancho

Palos Verdes, California, near Los Angeles. The current location is

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 106

indicated by the name in parentheses on the Options Menu. The program

provides two methods for setting your own coordinates: reading a file of

city names and coordinates (STSPLUS.CTY); or manually entering the

information.

Pressing F2 will display the following reminder:

STSPLUS searchs STSPLUS.CTY for the CITYNAME you enter anywhere in the

city names. 'SAN' appears in 'SAN DIEGO', 'SANTA ANA', 'THOUSAND OAKS.

Press ENTER to delete SECOND LOCATION, or enter '*' to manually enter

a city name and coordinates (and optionally add it to STSPLUS.CTY).

In other words, when you enter a name or partial name, STSPLUS will

attempt to match that group of characters anywhere in the names which

appear in the city file. For example, 'SAN' matches 'SAN diego' as well as

'SAN jose' and 'thouSANd oaks'. To get 'SAN FRANCISCO' on the first try,

enter 'SAN F' with a space between the 'N' and 'F'. Case is not

significant; upper and lower case letters are treated identically. If you

change your mind and wish to cancel the operation, simply press ENTER by

itself. Use BACKSPACE to make corrections.

To begin the search, enter the desired name after the prompt. In the

example which follows, the name 'ran' was entered for the search.

Enter CITYNAME to match:    ran

Processing record  1

City Name:  Rancho Palos Verdes CA

Latitude:     33.7675

Longitude:  -118.4033

Elevation:   186 meters

Press ENTER to ACCEPT this city as your location, OR

Press TAB to also display this city's location, OR

Press SPACE to search for next city:  _

If the city displayed is the one you wish to use as your local

coordinates, press ENTER. If you wish to display this city as a second

location on the display, press TAB. The data will be entered into STSPLUS

and subsequently saved in file STSPLUS.INI. If you wish to search further

in the file, press the SPACE BAR.

NOTE: The elevations contained in file STSPLUS.CTY are almost all zero

except for Rancho Palos Verdes, CA for which I have accurate elevation

above mean sea level. If you know the correct elevation for your

location, edit the file using any ASCII text editor and change the

last number on the line. STSPLUS.CTY contains over 800 cities. If

users send me their correct elevations (or additional cities they wish

added), I will incorporate that data into subsequent versions of file

STSPLUS.CTY.

To enter location data manually, press "*" (without the quotation

marks) followed by ENTER. You will be prompted for the city name, latitude,

longitude, and elevation. Latitude and longitude may be entered using three

different formats for convenience (note the use of comma and decimal

point):

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 107

DDD.DDDDD           Degrees and decimal fraction

DD,MM.MMM           Degrees, minutes and decimal fraction

DD,MM,SS.SS         Degrees, minutes, seconds and fraction

Note that SOUTH latitude and WEST longitude must be entered as NEGATIVE

numbers as measured south of the Equator or west of the Prime Meridian at

Greenwich, respectively. The default unit for elevations is meters above

mean sea level; add "F" (upper or lower case without the quotation marks)

if you wish to use feet.

After the elevation has been entered, the data will be displayed for

approval. All data are converted to degrees or meters as appropriate,

regardless of the units used on input.

City Name:  Rancho Palos Verdes CA

Latitude:     33.7675

Longitude:  -118.4033

Elevation:   186 meters

Press ENTER to ACCEPT this city, OR

Press SPACE to cancel this data:  _

Press ENTER to accept the data as shown or SPACE to cancel the data and

return to the Options Menu. If the data is accepted, STSPLUS will ask if

you wish to append (add) this city/location to the existing file

STSORBIT.CTY so that it will be automatically available thereafter.

Do you with to append this data to file STSPLUS.CTY (Y/n):  _

Press "Y" or ENTER to append the data to the file, or press "N" to not

modify the STSPLUS.CTY file.

F3     Set Display Features

---------------------------

A number of display features may be enabled or disabled using a

separate sub-menu. See the section SET DISPLAY FEATURES below for a full

description.

F4     Select Satellite Coordinates

-----------------------------------

The F4 command may be used to select the units used to display the

current coordinates for the satellite. The choices are:

RA/DEC    Right Ascension and Declination (Equator and Equinox of

Date).

Alt/Az    Altitude and Azimuth. Altitude is the elevation above the

horizon (assuming mean sea level), and azimuth is the

direction in the sense NESW (North to East to South to

West).

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 108

XYZ       Geocentric Cartesion Coordinates. The X-Axis and Y-Axis are

aligned with the Equator with the X-Axis pointing in the

direction of the Vernal Equinox. The Z-Axis points toward

the North Pole.

F5     Show Ascending & Descending Node Data

--------------------------------------------

The nodes of an Earth orbit are the points on the ground track where

the path crosses the Equator. The ascending node crosses from South to

North and the descending node crosses from North to South. Orbit numbers

normally increment at the ascending node. This command adds two additional

lines of data at the lower left of the screen giving the time (MET or time

since epoch) and longitude of the most recent ascending and descending

nodes. This information is useful when comparing STSPLUS's data against

other sources such as the wall map in Mission Control.

F6     Set Map Projection and Size

----------------------------------

The F6 command selects the size and field of view of the displayed

map. By default, the map is displayed using the orthographic projection,

"ORTHO", shows one complete hemisphere, and is centered so that the

selected satellite is visible. This corresponds to a magnification factor

of 100%.

Pressing F6 will select between WORLD, QUAD, ZOOM, and ORTHO maps.

Selecting WORLD will display the full world using rectangular projection

centered on the Prime Meridian at Greenwich, England at 0 degrees

longitude or at the International Date Line at 180 degrees longitude.

Selecting QUAD will select one of twelve Quadrant Maps showing a field of

view (horizontal size) of 180 degrees using rectangular projection.

Selecting ZOOM will select a Zoom Map with field of view adjustable from 30

degrees to 180 degrees; the default field of view is 75 degrees; the Zoom

Map is approximately centered on the current ground track position of the

satellite. Selecting ORTHO will select the orthographic projection. See the

sections ORTHOGRAPHIC MAPS, QUADRANT MAPS, ZOOM MAPS, and AUTOMATIC MAP

GENERATION for additional information.

F7     Enable/Disable EVENT TIMERS

----------------------------------

Press F7 to enable or disable all event timers. Event timers are

enabled by default if file STSPLUS.INI is present. Especially while the map

is being drawn, the calculations associated with the event times require

appreciable time. If the event timers are disabled, the audible alarms will

also be disabled. See the section "Event Timers and Audible Alarms" for a

full discussion of the event timers.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 109

F8     Enable/Disable Audible ALARMS

------------------------------------

Provided event timers are enabled (above), you may press F8 to enable

or disable audible alarms. Many users allow their computer to run STSPLUS

while performing other tasks and the audible alarm will alert them to an

imminent AOS (Acquisition of Signal) or LOS (Loss of Signal) event

associated with either their local circle of visibility or the TDRS

communications satellites.

For the local circle of visibility, an "up/down" tone sounds six times

two minutes prior to AOS and five tones sound thirty seconds prior to LOS.

Provided TDRS coverage is enabled (F10+F3+F2 from the Main Menu), three

brief tones sound thirty seconds prior to AOS or LOS. Provided Sun features

are enabled (F10+F3+F8), two tones will sound approximately thirty seconds

before orbital sunrise and sunset. The characteristics of the audible tones

will thus allow the user to identify what kind of AOS or LOS event is about

to happen.

Depending upon the computer and the version of DOS being used, "music"

such as these audible alarms may cause the DOS clock to lose a small amount

of time each time an alarm sounds. The amount of time loss is quite small

but may accumulate over long periods of time. (The DOS clock may also run

either fast or slow and effectively mask the time loss due to sound

effects.)

F9     Set UTC Time Offset and Daylight Flag

--------------------------------------------

STSPLUS uses UTC or Coordinated Universal Time, an adjusted version

of Universal Time (which STSPLUS considers the same as GMT or Greenwich

Mean Time), for certain functions such as launch time. The difference

between UT, UT1, UT2 and UTC is never more than 0.9 seconds. UTC is used

because it is the standard for civil timekeeping and agrees with standard

atomic time, TDB or Terrestrial Barycentric Time, used by astronomers.

However, NASA continues to use the GMT designation, a holdover from earlier

days before the introduction of UTC. Using UTC permits critical data to be

used across many time zones without conversion. However, it also means that

STSORBIT must know what number of hours to add to UTC in order to obtain

your local time, and whether or not you are currently using daylight

savings time (summer time in the UK).

When prompted, enter the time offset in hours from your local time to

Coordinated Universal Time. Examples are shown for most time zones in North

America. STSPLUS then asks if you are using daylight savings time; enter

"0" if not, and "1" if so. The sum of these two values is shown on the Main

Menu; for example, if the computer is set to Pacific Daylight Time (UTC

offset is -8.00 hours and Daylight Flag = 1), the sum will be -7.00. For

most time zones in North America, the correct zone abbreviation will be

shown on the ground track display for Local date and time. When you change

your computer from/to daylight savings time, use this command to update

STSPLUS. The following shows the display when using the F9 command:

Set UTC TIME ZONE OFFSET and DAYLIGHT FLAG

STSPLUS must know the difference between your local time zone and

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 110

Coordinated Universal Time (UTC), also sometimes known as Greenwich Mean

Time (GMT). With this information, STSPLUS can automatically adjust launch

or Epoch times and dates for your local time zone. In addition, STSPLUS

must know if your computer is now set to STANDARD or DAYLIGHT time.

First, enter the difference between your STANDARD time zone and UTC in

hours. Do NOT include the hour for daylight time if you are now on DAYLIGHT

time; it will be entered separately. For most time zones in the United

States and Canada, the entries required are:

Eastern Standard Time     EST    -5.0

Central Standard Time     CST    -6.0

Mountain Standard Time    MST    -7.0

Pacific Standard Time     PST    -8.0

Enter UTC Offset (hours):          -8

Enter DAYLIGHT Flag (0=OFF, 1=ON): 1

Once this information has been entered, it will be saved in file

STSPLUS.INI and will not be requested again. If you change from Standard to

Daylight Time or vice versa, use the F10+F9 command to update the Daylight

Flag.

If you change the setting of the Daylight Flag, STSPLUS will ask if

you wish to adjust your DOS software clock:

You have changed the setting of the Daylight Flag.

Do you wish to adjust your DOS clock to reflect the

change [y/N]:

If you have already made the change at the DOS prompt (or using some other

software) or do not wish to change the DOS clock, press ENTER (or type "N"

followed by ENTER). If you wish to adjust the DOS clock to correspond to

the new setting of the Daylight Flag, press "Y" followed by ENTER. When

STSPLUS changes the DOS clock, it synchronizes the time change to the

nearest second but there may be a small error introduced; only if your

computer is precisely set would the error be detectable.

*************

*  CAUTION  *

*************

For computers equipped with 80286 or higher processors AND using DOS 3.2 or

higher, changing the DOS clock will ALSO change the hardware clock. 8088-

based computers may or may not have a hardware clock installed and, even if

a hardware clock is present, it may or may not be compatible with the DOS

time setting commands.

F10    Enable/Disable Printer Logging

-------------------------------------

I have found it interesting to log the orbital data and the ascending

and descending node information on my printer when analyzing the mission

data over long periods of time. The F10 command toggles the printer logging

function on and off. The first page of the log includes the current orbital

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 111

data and subsequent pages contain only node information. In addition to the

information presented on the display, the printer log also calculates the

current orbital time, the time from one ascending (descending) node to the

next, for the third and subsequent nodes. A typical log is shown below.

IMPORTANT:     BE SURE THE PRINTER IS TURNED ON PRIOR TO ENTERING THE

F10 COMMAND.

STSORBIT: Space Shuttle Tracking Program, Version 9338          Page 1

ORBITAL DATA for STS-31 Discovery/HST

NORAD Number:            20580

Launch Date:             04/24/1990

Launch Time:             05:33:52

Orbit Inclination:       28.4695

Orbit Altitude:         329.50 nm

UT DATE   UT TIME ORBIT    LONG           MET     TIME

Ascend Node: 04/28/1990  20:32:52    70  -69.95    4/14:58:07

Dscend Node: 04/28/1990  21:20:52    70   97.64    4/15:46:35

Ascend Node: 04/28/1990  22:09:52    71  -94.77    4/16:35:02  1:36:55

When printer logging is enabled and the ground track is displayed, the

word "LOG" will appear in red at the right of the text area. Enabling

printer logging also automatically enagles the display of ascending and

descending node information.

The Launch Date and Launch Time entries are given if that information

has been entered indepentently. The Epoch Date and Epoch Time are always

shown. Note also that the orbit altitude shown is the altitude at the time

the log was started and will not be correct for subsequent entries,

especially if the satellite has an elliptical orbit (high eccentricity).

A printer log may be prepared in advance of a mission by enabling

printer logging from the Set Options Menu (with the F10+F10 command),

setting the desired simulation time (F8+F3 command), then starting the

ground track display with ENTER; once the ground track has appeared on the

screen, pressing the F key twice to set STSPLUS in the X60 fast time mode

will generate the date relatively quickly (although the UT TIME printed may

be off by as much as one minute in the X60 mode). Allow the simulation to

run for the desired length of time, then press ENTER to return to the Main

Menu. While the ground track is active Function Key F3 performs the same

function as the F10+F10 command to enable or disable printer logging.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 112

SET DISPLAY FEATURES

--------------------

Depending upon the satellite and personal preferences, a variety of

display features may be enabled or disabled. Not all features are available

with monochrome or CGA monitors. Pressing F3 on the Set Program Options and

Features Menu will display the following menu:

Program STSORBIT PLUS

Space Shuttle and Satellite Orbit Simulation

Version 9338

Current time:  19:59:10 PST   03:59:10 UTC

Current date:    02/24/1993     02/25/1993

F1     ON      Display LOCAL Circles of Visibility

F2     ON      Display TDRS and Real Time Satellites

F3     ON      Display Additional Map Grid Lines

F4     OFF     Display Tracking Stations

F5     BOTH    Display Ground Track: DOTS/LINE

F6     ON      Display Spacecraft Circle of Visibility

F7     OFF     Display South Atlantic Anomaly Zone

F8     ON      Display Terminator, SUN, and Spacecraft Lighting

F9     ON      Display Map Locations and Features

F10    ON      Display Lakes and Rivers

ENTER          Return to MAIN MENU

Select desired function:

F1     Display LOCAL Circles of Visibility

------------------------------------------

Function Key F1 enables and disables the local circles of visibility,

centered on your location and a second location (if enabled), and shows the

approximate area within which direct line of sight communication with the

satellite is possible. The circle is calculated at the instant the map is

drawn and may not be accurate over long periods of time for satellites with

highly eccentric orbits. In some situations (geosynchronous satellites, for

example), these circles of visibility cover so large an area that they

simply confuse and clutter the display. Use this command to disable the

circles.

F2     Display TDRS and Real Time Satellites

--------------------------------------------

This command provides a menu to display, modify, save, or read

selected real time satellites and static satellites. The communications

coverage for the Tracking and Data Relay Satellites (TDRS) or other

geosynchronous satellites is also shown. The TDRS coverage boundaries

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 113

overlap between the East and West TDRS satellites and Mission Control may

select either satellite during the overlap period. STSPLUS displays the

areas covered by each satellite and the times for acquisition and loss of

signal (AOS and LOS). See the section "TDRS and Real Time Satellite

Features" above for a full discussion of the TDRS coverage features.

F3     Display Additional Map Grid Lines

----------------------------------------

This command is not available for CGA systems. The basic world map

includes the Equator and the meridians at 0 degrees (Prime Meridian) and

108 degrees (International Date Line) shown in bright blue on color

monitors. Turning on the map grid adds additional lines of longitude and

latitude. Displaying the additional grid lines on some monochrome monitors,

especially CGA, may make the screen too "busy".

In the Orthographic, Quadrant and Zoom Map modes, the spacing of the

additional grid lines is adjusted for the map field of view. In rectangular

map modes (EGA and VGA systems only), each grid line is labeled at the left

or bottom of the display screen.

F4     Display Tracking Stations

--------------------------------

The F4 command enables/disables the display of the tracking stations

included in file STSPLUS.TRK or the current TRACKING STATION filename as

set by Function Key F7 on the Main Menu. If that file is not found,

internal data are used for NASA's 14 original ground tracking stations plus

the NASA Ground Terminal at White Sands, NM. Each tracking station is

located with a small red circle. The circle of visibility is also shown if

that circle has an angular diameter of 90 degrees or less. The circles of

visibility are calculated at the instant the map is drawn and may not be

accurate over long periods of time for satellites with highly eccentric

orbits. For all map modes EXCEPT the World Maps, each tracking station is

also labeled with its 3-character abbreviation. This command is not

available for CGA monitors.

F5     Display Ground Track: DOTS/LINE

--------------------------------------

STSPLUS calculates the ground track for the satellite for a period

from one and one half hours in the past to three hours in the future. Press

F5 to change from one mode to the next. Depending upon the user's

preferences, this function may be used to set the displayed ground track to

any of the following modes:

NONE      The ground track is not displayed.

DOTS      The ground track is displayed using RED dots for the past

ground track and GREEN dots for the future ground track. The

dots are spaced at one minute intervals. As time passes, the

GREEN dots will change to RED.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 114

LINE      The ground grack is displayed using a GREEN line.

BOTH      The ground track is displayed using a GREEN line with RED

dots for past ground track minute marks and YELLOW dots for

future ground track minute marks. As time passes, the YELLOW

dots will change to RED.

F5     Display Spacecraft Circle of Visibility

----------------------------------------------

STSPLUS can calculate the approximate circle of visibility from the

spacecraft, the area of the Earth's surface which is visible from the

cockpit windows and television cameras or, for unmanned spacecraft, the

direct line of sight visibility from the ground. Note that the shape of the

"circle" varies depending upon the magnification or zoom factor and map

projection being used. With rectangular projection, the shape is

approximately a circle near the Equator and more like a rounded triangle at

higher latitudes; near the poles, the "circle" spreads out across the map.

This is an artifact of the rectangular map projection.

When enabled, the circle of visibility is recalculated every 10

seconds based upon the spacecraft's current altitude. This means that

orbits with a high eccentricity (that is, a highly elliptical orbit whose

apogee and perigee are very different) will exhibit a constantly changing

circle of visibility.

F7     Display South Atlantic Anomoly Zone

------------------------------------------

The South Atlantic Anamoly (SAA) is an area in the southern hemisphere

lying between southern tip of Africa and South America which can cause

severe electromagnetic disturbances on spacecraft. For example, the

semiconductor memory on the Hubble Space Telescope (which regularly passes

through the SAA) was being changed by this phenomenon until a patch was

uplinked to work around the problem. The area is shown on the ground track

as an ellipse for simplicity; its actual outline is more nearly shaped like

a kidney bean. Using NASA Mission Charts for various Space Shuttle

missions as a reference, the SAA is adjusted for spacecraft altitudes from

160 nm to 350 nm (although it extends out to geosynchronous orbits).

SAA coverage is disabled in orthographic modes pending better data and

the development of a mathematical model for use in those modes.

F8     Display Terminator, Sun, and SpaceCraft Lighting

-------------------------------------------------------

This function enables and disables the Sun and related solar features.

See the section "Sun and Solar Features" for a full discussion. STSPLUS

calculates whether the spacecraft is in full sun, penumbra (partial shadow)

or refracted sunlight, or umbra (full shadow) and adjusts the color of the

spacecraft icon accordingly: bright white, yellow, and dim white

respectively. This feature is not available on CGA and HGC monitors. The

current spacecraft solar lighting is indicated in the data block (next to

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 115

"Orbit #:") by the following symbols:

*    Full sunlight

+    Partial sunlight (penumbra)

-    Refracted sunlight

Full shadow (umbra)

F9     Display Map Locations and Features

-----------------------------------------

This feature enables or disables the display of the map locations and

features contained in file STSPLUS.LOC if present. See the section

"Location and Features Labels" above for a full discussion.

F10    Display Lakes and Rivers

-------------------------------

This feature enables or disables the display of lakes and rivers on

the map. Removing the lakes and rivers will lessen the time required to

draw a map and can improve screen legibility especially for CGA systems.

(The lakes and rivers are always disabled on the rectangular world map to

avoid cluttering an already busy display!)

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 116

STSORBIT PLUS's Orbital Model

-----------------------------

The original version of STSORBIT was first prepared without reference

materials of any kind and the simplest possible orbital model was therefore

selected. The primary objective was to duplicate the NASA wall map at

the Mission Control Center in Houston, Texas. This "simple" model assumed

that the orbit was perfectly circular at a specified altitude and

inclination which never degraded due to other factors such as drag or

perturbation. Some simplifying assumptions were incorporated to handle the

initial ascent portion of a mission and the launch site was hard coded to

Cape Canaveral, Florida. With only minor modifications, the program was

essentially unchanged for the next year.

The launch of STS-31 and the Hubble Space Telescope highlighted the

need for improved accuracy because of public interest and the length of the

mission. The orbital calculations were modified (STSORBIT Version 9015) to

include the J2 factor, the perturbation of low Earth orbits due to

variations in the gravitational field related to the non-spherical shape of

the Earth (among other factors); omission of the J2 factor caused errors in

longitude of approximately -5 to -7 degrees per day. That is, the orbital

track drifted Westward from its true position by that amount. More accurate

models of low orbits also include the J3 and J4 perturbation factors,

atmospheric drag, and a host of other less significant items.

Although reasonably accurate for the first day or so of a space

shuttle mission, the "simple" model is by no means ideal. In calculating

the current orbital longitude, for example, the "simple" model assumes a

circular orbit. For orbits with low inclinations, as is typical for

launches from Kennedy Space Center, the errors are not particularly

significant and are probably overshadowed by the fundamental uncertainties

in orbital parameters and by the limitations imposed by display resolution.

Orbits with higher inclinations, as would be the case if near-polar

launches from Vandenburg AFB in California are ever initiated, would have

much larger periodic errors which would be both noticeable and

objectionable. More important for longer missions and for general satellite

tracking is the fact that due to the method used, errors in the orbital

calculations tend to be cumulative. After a day or two, the errors become

unacceptably large.

The real problem with the simple method, of course, is that the Earth

is not a perfect sphere and actual satellite orbits are never perfectly

circular. Satellite orbits are significantly perturbed by the non-spherical

gravitational field of the Earth, by the Sun and Moon, atmospheric drag,

and other factors. Accurate satellite tracking over longer periods of time

therefore demands more accurate data and a more rigorous treatment of

satellite orbits. The only practical alternative is to use the NASA/NORAD

2-line orbital element sets. Not only are these data readily available

publicly, but they are relatively accurate and are updated regularly.

Therefore, STSORBIT PLUS relies on the NORAD SGP4 prediction model and the

2-line orbital element sets for orbit predictions. 2-line element sets for

non-military space shuttle missions are typically available on the same day

as the launch. Amateur astronomers and satellite tracking experts often

generate "unofficial" 2-line element sets even for military missions.

Six quantities are required by classical gravitational theory to

completely characterize the orbit of one body about another in time and

space, the "Two Body Problem". These six quantities, often referred to as

Keplerian orbital elements, are included in the NASA/NORAD 2-line element

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 117

sets along with other numerical and statistical data. The U.S Space

Command, (formerly NORAD, the North American Air Defense Command)

headquartered in Cheyenne Mountain, Colorado, developed the SGP4 and SDP4

orbital models and the 2-line element format many years ago as part of

their satellite tracking efforts and NASA subsequently adopted the same

format -- more or less. NASA and NORAD do not always use the same

definition for revolution (orbit) numbers; NASA frequently gives a number

one (or two) greater than NORAD, calling the first partial orbit number one

while NORAD calls that same partial orbit number zero. Except for short

duration missions, such as the Space Shuttle, revolution numbers are of no

practical importance.

Having timely and accurate orbital data is of little help without a

computer model or program which can use those data. NORAD has rather

arbitrarily divided satellite orbits into two categories: near Earth orbits

and deep space orbits. Near Earth orbits are defined as those with orbital

periods of 225 minutes or less and deep space orbits are all others.

Computer models are described in the literature for each category. STSORBIT

PLUS employs the SGP4 Near Earth Model only, using a composite of code of

my own combined with translated Fortran and Basic source supplied by Paul

Traufler and C source by Paul Hirose. I plan to add the SDP4 deep space

model in due course. Not only are the near Earth orbits generally of more

interest to observers, but the errors associated with deep space orbits

processed with the SGP4 model (rather than the correct SDP4 model) are not

particularly significant for the purposes of a program such as STSORBIT

PLUS. Further, watching a geostationary satellite orbit on the screen is

not unlike watching grass grow and is about as exciting.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 118

Accurate Time and the Personal Computer

---------------------------------------

For a program like STSPLUS, accuracy and precision of the timekeeping

functions are essential. There is a tendency these days to accept whatever

a computer says as the absolute truth without regard for whether or not the

information is even "reasonable". For something as basic as time, even an

experienced computer user may assume that it is correct. This discussion

attempts to compare reality with that expectation.

Given the clock drift and accuracy problems inherent in the design of

the typical IBM-compatible personal computer, frequent time setting and/or

adjustments are required. Accurate time setting would not be practical

for most people without the various time services provided by the National

Institute of Standards and Technology (NIST) and the U.S. Naval Observatory

(USNO). The NIST radio stations WWV and WWVH provide an inexpensive and

convenient means for "ordinary folks" to synchronize clocks and other

equipment. The NIST and USNO Telephone Time Services offer a high precision

standard time calibration source when such accuracy is required. Similar

radio and telephone services are available in Candad and Europe.

Once a computer clock has been set with reasonable accuracy, the

accuracy of the computer's clock will indeed be sufficient for many

applications; if you are using a word processing or spreadsheet program,

knowing the time to within a minute or two is probably adequate. For

programs such as STSPLUS and other time-dependent applications, however,

this level of accuracy simply will not suffice; when used for satellite

tracking, the time should be accurate to within a second. Unless steps are

taken to both set the clock and to maintain its accuracy, this will not be

the case. No matter how accurately the clock on a typical personal computer

is set, it will only be a matter of hours before the time will have drifted

by some seconds. Measured over a number of days, the accumulated errors can

easily amount to a minute or more.

The timekeeping operations of an IBM-compatible computer are actually

performed by two separate and independent functions: a clock-calendar

CMOS integrated circuit and lithium battery combination which maintains the

current time and date in hardware; and, a section of the DOS operating

system software which maintains the current time and date in software. When

computer power is off, the hardware chip continues to operate using its

battery; when the computer is started ("booted"), the operating system

software reads the hardware clock and sets its internal software clock.

Absent special software, the DOS time thereafter relies entirely on the

software clock until the next time the computer is restarted.

Unfortunately, neither of these clocks was designed for accuracy; early

versions of the PC did not even include the hardware/battery arrangement.

Even the typical electric clock, which uses the power line frequency for

its timekeeping reference, is usually far more accurate.

The accuracy of the DOS time at any instant is the result of the

accumulated errors in both clocks. The hardware clock will drift as a

function of time, temperature, voltage, and crystal aging; the software

clock will gain or lose time depending upon the skill with which its

software was written and how well that software "cooperates" with the

balance of the computer's hardware and software. Some software, especially

network and high speed communications software, can prevent the DOS clock

software from incrementing when it should, usually resulting in the DOS

clock losing time. The problem was compounded with the release of DOS

Version 3.3; beginning with that version, the DOS TIME and DATE commands

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 119

adjust BOTH the hardware and software clocks and thereby potentially

eliminate the hardware clock as even a modestly reliable reference.

Methods for Setting DOS Time

----------------------------

Bearing these considerations in mind, there are a number of approaches

to the DOS time question. The most obvious approach, used by the vast

majority of computer users, is to either ignore the computer clock entirely

or to say "It's close enough". Regardless of the application, I strongly

recommend that the DOS clock be REGULARLY set to the correct time if only

to assure that files are more or less correctly date and time stamped. If

the accuracy of DOS time is important, the computer clock may be set or

synchronized in a number of ways, some of which are described below. In

this context, "ACCURACY" means the accuracy of the time setting operation

and NOT the longer term accuracy and stability of the DOS time.

1.   TELEPHONE: Many local telephone companies offer a telephone time

service, usually with a message such as "When you hear the signal the

time will be ... (beep)". I am not aware of any hardware or software

which uses this signal for time setting purposes.

ACCURACY: Generally plus or minus 5 seconds. With the advent of

digital voice response equipment in recent years, the accuracy has

improved to perhaps plus or minus 1 second.

2.   COMMERCIAL RADIO: Hourly time signals broadcast on commercial radio

may be used to manually set the time. My experience suggests that the

CBS network time signal is usually reliable.

ACCURACY: Usually within plus or minus 2 seconds, depending upon the

source. Satellite distribution of network feeds add a time delay of

approximately 0.25 seconds per "hop" but some stations, including some

network stations, generate time signals locally.

3.   SHORTWAVE RADIO: Time signals are broadcast on shortwave radio

stations WWV and WWVH by the National Institute of Standards and

Technology. These time signals may be used to manually set the time.

WWV and WWVH broadcast on several frequencies: 2.5MHz, 5MHz, 10MHz,

15MHz, and 20 MHz (WWV only). Reception will vary according to your

distance from the transmitter, time of day, and atmospheric

conditions. These time signals are very precise; the only major

variable is the propagation delay, the time it takes the radio signal

to travel from the transmitter to your receiver. The typical

propagation delay is approximately 5 microseconds per mile. Outside

North America, other national radio services such as the British

Broadcasting Company's BBC World Service offer accurate hourly

shortwave time signals.

ACCURACY: Time setting using WWV or WWVH can usually be performed to

within about plus or minus 250 milliseconds, of which up to 25

milliseconds is transmission time and the balance is user response

time. With practice, plus or minus about 100 milliseconds is

practical.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 120

4.   HEATH GC-1000 MOST ACCURATE CLOCK: The GC-1000 is a combination

digital clock and scanning shortwave radio receiver which may be

equipped with an RS-232 communications port for use with computers and

other electronic equipment. Operation with DC power is available to

maintain accurate time during periods of AC power loss. This is the

only method which provides more or less continuous accurate time

information without telephone toll charges.

ACCURACY: When properly configured for your location, equipped with an

external antenna, used with appropriate computer software, and when

the receiver is locked to one of the WWV (or WWVH) time signals, the

GC-1000 can provide time information and a standard calibration

frequency to an accuracy of plus or minus 10 milliseconds. When signal

lock is lost, the receiver scans the 5MHz, 10MHZ, and 15MHz broadcasts

to reacquire signal and lock. Even after signal lock is lost, the

receiver maintains an accuracy of plus or minus 100 milliseconds for

some hours.

5.   NIST/USNO TELEPHONE TIME SERVICE: When real precision and accuracy are

required, the computer clock may be set remotely using the telephone

time service of either the National Institute of Standards and

Technology (NIST, formerly the National Bureau of Standards or NBS) in

Boulder, Colorado, or the U.S. Naval Observatory (USNO) in Washington,

D.C. This method requires a modem connected to a telephone line and is

available for systems using DOS version 3.3 or higher AND equipped

with 80286 processor or higher; some 8088-equipped systems may also

use this method depending upon the type of clock hardware installed

and the version of DOS being used. The recommended method uses the

programs TIMESET and RIGHTIME (see below) although other commercial

and shareware programs may be available.

ACCURACY: This is the most accurate method available for setting and

maintaining the DOS clocks. Depending upon which service is used, NIST

or USNO, whether or not line delay compensation ("lag") is employed,

and the frequency of time setting, the DOS time can be set to within

plus or minus 2 milliseconds. However, since the "time ticks" of the

DOS software clock occur every 55 milliseconds, or 18.2 times per

second, this "granularity" may limit the accuracy of reading the DOS

clocks. See the documentation for programs TIMESET and RIGHTIME for

additional discussion.

Choose one of the methods suggested or a suitable alternative based

upon your precision and accuracy requirements. Other methods of maintaining

an accurate time standard such as atomic clocks, Global Positioning

Satellite (GPS) time receivers, and NIST time code equipment, are also

available -- for a price. Those methods are beyond the scope of this

documentation.

Maintaining Accurate DOS Time

-----------------------------

Just in case you missed the point earlier, accurately setting DOS time

is only half the battle. Even if the DOS time is set very precisely as

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 121

discussed above, all that assures is that the time is correct to the

required accuracy at that instant. The problem then becomes one of knowing

how the DOS clocks change or drift with time and how to compensate for

those changes or, alternatively, checking the DOS time frequently enough

that any drift on the part of the DOS clocks is acceptable for the intended

application.

Of the two clocks in a typical personal computer, the hardware clock

is considerably more consistent and reliable. I have checked perhaps a

dozen PC hardware clocks in recent years, and almost all kept reasonably

good time over a period of several days; as expected, none kept "perfect"

time. Typical drift rates ranged from about 3 seconds per day to near zero

seconds per day, with the magnitude and direction of the drift more or less

constant over the period of measurement. The hardware clock is typically

sensitive to both voltage and temperature, both of which undergo

significant change when the computer is turned on or off. Complete

calibration of the hardware clock requires knowledge of its performance

under both circumstances. Once a hardware clock has been calibrated,

its performance may be predicted with reasonable accuracy over periods of

some weeks or more. Crystal aging rates suggest that calibration should be

performed at least monthly.

The hardware clock is normally interrogated only when the computer is

first started or rebooted. The correct time can therefore be predicted at

that moment for a calibrated hardware clock, given the last time that clock

was synchronized with an appropriate time standard. Microsoft provides no

standard software tools for interrogating the hardware clock at other

times except for low level interrupt services. Quite the contrary;

beginning with DOS Version 3.3, using the DOS TIME and DATE commands to set

the DOS software clock will also set the hardware clock and effectively

destroy its usefulness as a calibrated time reference. I am at a complete

loss to understand the reasoning behind this change in DOS; I presume that

users were being "confused" by differences between the hardware and

software clocks; instead of either explaining or fixing the problem,

Microsoft elected to "legislate" the problem away -- a process any

politician would recognize instantly. The only mitigating consideration is

that any really effective solution would probably require hardware as well

as software changes. Blame IBM, I guess.

The software clock provides the only time information readily

accessible to DOS using standard software. Since this clock is maintained

entirely in software, with no reference to the hardware clock except at

bootup, it is at the mercy of other software which may execute from time to

time. The software clock increments its time using "interrupts", a

technique which stops a software process in progress just long enough to do

the required tasks and then resumes the interrupted process. These

interrupts occur every 55 milliseconds. So long as none is missed, the

software clock should keep accurate time -- if the software is written

correctly and if the computer's crystal controlled oscillator is in turn

accurate. It may be that neither of these conditions is true; certainly the

crystal controlled oscillator (quite similar to the one which runs the

hardware clock) was not designed for accuracy or stability. It's original

purpose was solely to generate the necessary timing signals for the

operation of the computer. Cost, not accurate time, was the primary

consideration in its design.

Other software designers have contributed to the problem by writing

software which, deliberately or inadvertently, prevents the software clock

from being updated. Off-brand BIOS firmware can present occasional

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 122

problems. Local Area Network (LAN) and high speed communications software

are also frequent culprits in this respect. For example, a casual check of

the clock while using a high speed computer-to-computer file transfer

program indicated that the clock was effectively suspended when data

transfers were in progress. In one relatively brief test, the DOS clock

lost about 30 seconds.

As a result of all of these factors, the accuracy of the DOS software

clock can vary wildly from one computer to the next and from one situation

to another. One inexpensive "clone" computer that I'd rather forget

couldn't manage to keep time to better than about 30 seconds per HOUR!

Before planning to use a particular computer as a time reference with

programs like STSPLUS, check the computer hardware and software you intend

to use very carefully.

Programs TIMESET and RIGHTIME

-----------------------------

Two fine programs, TIMESET by Peter Petrakis and RIGHTIME by Tom

Becker, provide all the features required to accurately set and maintain

the computer's hardware and software clocks. Development efforts on these

programs have been carefully coordinated so that they cooperate with each

other. Both programs are copyrighted commercial software distributed as

"shareware" and require registration after an initial evaluation period. I

highly recommend these programs and encourage users to support the authors

and their work. So far as I know, there are no other comparable programs

available at any price!

TIMESET, Version 7.10 or higher, uses the telephone time services of NIST,

USNO, and three European services to precisely set the computer clocks. The

standard distribution also includes several additional time-related utility

programs. It is available on many computer bulletin board systems or direct

from:

Peter Petrakis

Life Sciences Software

8925 271st N.W., Suite 112

Box 1560

Stanwood, Washington  98292  USA

Telephone: (206) 387-9788

RIGHTIME, Version 2.5+, is a program to compensate for the various drift

factors in a computer's hardware and software clocks. When used in

conjunction with TIMESET and properly calibrated, RIGHTIME "learns" the

warm and cool drift factors for a specific computer. As a result, the

clock's can be maintained with an accuracy of a fraction of a second over

long periods of time. It is available on many computer bulletin board

systems or direct from:

Tom Becker

Air System Technologies, Inc.

14232 Marsh Lane, Suite 339

Dallas, Texas  75234  USA

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 123

Telephone: (214) 402-9660

Tom Becker and Peter Petrakis may be also contacted directly on the Air

Systems Technologies computer bulletin board system in Dallas, Texas. The

BBS always has the latest versions of TIMESET and RIGHTIME available for

download:

Air Systems Technologies BBS

(214) 869-2780

STSPLUS is now "aware" of program RighTime and its use is recommended

for accurate timekeeping. Audible alarms in prior versions would perform

unpredictably when RighTime was active because they use the hardware

clock's timer functions (which RighTime also uses). STSPLUS now detects

RighTime and temporarily disables RighTime while an audible alarm is being

generated and then re-enables RighTime after the alarm has completed,

restoring precise timekeeping. With RighTime active, alarms are generated

in foreground, which may cause a slight delay in screen updating.

*************

*  CAUTION  *

*************

STSPLUS expects RighTime Version 2.5+; performance with prior

versions of RighTime may yield unpredictable results. If using a

prior version of RighTime, do NOT enable audible alarms!

If RighTime is not present or is not detected, the audible alarms are

generated in background as in prior versions. This usually causes the loss

of several clock ticks in the DOS software clock for each audible alarm.

Although the time loss per audible alarm is very small, the cumulative

error may become significant over extended time periods.

The following descriptive text is extracted with permission from the

documentation for the current versions of TIMESET and RIGHTIME; please

consult the documentation for each program for full details. Although

future versions of both programs are expected to remain compatible with

STSPLUS, they should be tested carefully before regular use.

FEATURES OF TIMESET 7.10

------------------------

TimeSet has been evolving steadily ever since the first version was

released in the summer of 1987.  That version and several subsequent ones

could only set a computer's clock from the U.S. Naval Observatory (USNO) in

Washington, D.C.  Version 6.00, released in 1990, added ability to use

telephone time signals from the National Institute of Standards and

Technology (NIST) in Boulder, Colorado, making it the first program of its

kind able to address more than one atomic time service.  This made it

possible for computer users in the eastern and western United States to

keep down long distance bills by choosing the time service closest to them.

Version 7.10 continues that evolution with a number of new features

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 124

and supporting utilities:

o    TimeSet can now access five atomic clock-based telephone time services

on two continents: the USNO and the NIST in the United States, as

before, and atomic time services in Sweden (National Time and

Frequency Laboratory), Austria (Technical University of Graz), and

Italy (National Electrotechnical Institute).  People in European

countries who want to set their computers to an atomic clock no longer

need to make a trans-Atlantic phone call.

o    TimeSet 7.10 is designed to interact closely with version 2.5+ of

RighTime (tm), the excellent memory-resident regulator for computer

clocks developed by Tom Becker of Air System Technologies, Inc.,

Dallas.  RighTime learns the drift rate in the computer's clock and

continuously applies a correction to compensate for it, and it refines

the correction each time the computer clock is set.  A computer with

RighTime installed and trained can maintain system clock accuracy

within a second for at least a week. Furthermore, version 2.46

provides true 0.01-sec resolution in the DOS clock, in contrast to the

normal 0.055-sec resolution.  This allows greater accuracy in

timesetting than ever before, indeed the maximum accuracy that can be

obtained with a computer clock.  Life Sciences Software and Air System

Technologies cooperated closely during the development of TimeSet 7.10

and RighTime 2.46, with the result that TimeSet can access several

RighTime functions directly.

"TIMESET" is a trademark of Life Sciences Software (TM)

The following is a screen dump of the data displayed by TIMESET:

+------------------- From NIST.  Connect time: 11.97 sec. --------------------+

| DATA FOR TIME CALCULATIONS (all data pertain to Coordinated Universal Time) |

|    Time data string:  49051 93-03-05 14:07:20 81 0 -.1 051.1 UTC(NIST)      |

|                Date:  03-05-1993                                            |

|         Julian date:  2449051                                               |

|         Day of year:  064                                                   |

|                Hour:  14    Minute:  07    Second:  20                      |

|              The United States mainland is on standard time.                |

|        U.S. daylight time begins on 04-04-1993 at 02:00:00 local time.      |

+-----------------------------------------------------------------------------+

System clock set: 06:07:20.01

+-----------------------------------------------------------------------------+

|                 ACTION SUMMARY (at instant of timesetting)                  |

|    Internal delay adjustment:  .01 sec. (added to set time)                 |

|        Line delay adjustment:  .0511 sec. (precompensated by time service)  |

|                                                                             |

|   Universal Time Coordinated:  14:07:20.01           (time at 0x longitude) |

|                     UTC Date:  03-05-1993, Friday    (date at 0x longitude) |

|                                                                             |

|      Local computer time was:  06:07:20.01 (RighTime-assisted)              |

|                       Set to:  06:07:20.01 Pacific Standard                 |

|      Local computer date was:  03-05-1993                                   |

|                       Set to:  03-05-1993, Friday, Day 064 of 1993          |

+-----------------------------------------------------------------------------+

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 125

FEATURES OF RIGHTIME 2.5+

-------------------------

RighTime brings exceptional system time of day clock performance to

the DOS-based AT-class-compatible PC computer with no additional hardware.

With RighTime installed, the standard real time clock system becomes an

Adaptive Mathematically Compensated Crystal-controlled Oscillator based

clock.  Under stable conditions, RighTime can produce a system clock that

keeps time within one half second per week or better (some testers have

reported accuracy of 0.07 second per week); this is about 0.8 parts per

million error, or more than 100 times better than an unconditioned crystal

time base alone, or 30 times better than a moderately conditioned one like

a modern watch of quality.

o    True one hundredth second DOS clock resolution: the 55-millisecond

barrier is broken!  The standard DOS clock resolves to only about 1/18

second; under RighTime v2, the new high resolution DOS clock resolves

to, and increments in, hundredths while the Int 08h and 1Ch tick rate

remains standard.

o    RighTime intrinsically sets the hardware clock and solves the midnight

rollover date bug that exists in some DOS versions; this eliminates

the need for other utility programs or drivers that perform these

functions.  Unlike DOS alone, the hardware clock seconds transition

will be properly set by RighTime and the time will be set to

hundredths of a second resolution, and these qualities will survive

through rebooting.

o    Each time you set the time, RighTime will improve the accuracy of the

clock error corrections and will subsequently improve the accuracy of

the clocks.  It should be easy to achieve a worst-case error of less

than 0.5 second per day and under good conditions, less than 0.5

second per week; typical results are much better.  Command line

options are provided that allow fine tuning the correction process to

your system.  A trimming option provides for offset adjustments in

hundredths of a second.

The following is a screen dump of the data displayed by RIGHTIME:

RighTime: Indicated DOS clock date and time is 1993/03/05 06:04:45.66.

RighTime: Warm correction rate is +2.83 seconds per day.

RighTime: Cool correction rate is +4.27 seconds per day.

RighTime: Current applied DOS-CMOS RTC offset is +0.46 second.

RighTime: Last CMOS RTC adjustment was 0.00 hours ago.

RighTime: Last timeset was 23.33 hours ago.

RighTime: System has been warm 17% of the time since the last timeset.

RighTime: Stack A headroom is 92 bytes; Stack space used is 68 bytes.

Stack D headroom is 100 bytes; Stack space used is 60 bytes.

RighTime: /?=Help; Version 2.53

RighTime: Copyright 1991-93 GTBecker, Dallas 214/402-9660. All Rights

Reserved.

RighTime: Resident and enabled.

RighTime: Selftest passed.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 126

Computer Bulletin Board Systems

-------------------------------

RPV Astronomy BBS        (310) 541-7299

RPV Hotline BBS          (310) 544-8977

CELESTIAL BBS            (513) 427-0764

NASA GSFC OIG RBBS       (301) 262-6784

NASA Spacelink BBS       (205) 895-0028

CSS BBS                  (416) 458-5907

JPL PAO BBS              (818) 354-1333

Timely 2-line orbital elements are essential for accurate satellite

tracking. In addition to my own bulletin board systems, five other bulletin

board systems provide authoritatve data for the general public. Most files

on these systems are compressed to reduce download time and must be

decompressed before use.

I maintain two computer bulletin board systems as a public service.

The main system is the RPV Astronomy BBS at (310) 541-7299, 2400-14400

baud. The backup system is the RPV Hotline BBS at (310) 544-8977, 1200-2400

baud. During Space Shuttle missions, I post current 2-line elements at

least daily (provided I'm in town!). Both systems usually also have current

2-line elements for a large number of satellites in addition to other files

of interest to space and astronomy enthusiasts. The systems are very

popular and, therefore, are frequently busy!

For the past several years, Major T. S. Kelso, USAF, has been making

the US Space Command (formerly NORAD) orbital data available as a public

service on his Celestial BBS at (513) 427-0674, 1 line at 1200 to 14400

baud. The 2-line element sets are prepared by Kelso using data received

directly from U.S. Space Command (formerly NORAD) by special arrangement. I

regularly post a concantenated and sorted version of the current element

sets on my own RPV ASTRONOMY BBS as file TLEnnn.ZIP, where "nnn" is the

current Prediction Bulletin number. Kelso provides data for several

categories of satellites:  Amateur Radio, Earth Resources, Manned

Spacecraft, Navigation, Weather, and NASA's 30 Day Specials (which contain

objects launched within the last 30 days and are often easy to spot

visually). More specifically, these include the following satellites or

satellite series: OSCAR, Radio Sputnik, UOSAT, Cosmos, LandSat, SeaSat 1,

SPOT, Mir, Salyut 7, Soyuz, Space Shuttle, NAVSTAR (GPS), GOES, Meteor, and

NOAA.

The Canadian Space Society BBS, (416) 458-5907, 1 line at 1200 and

2400 baud, also regularly posts NORAD 2-line elements. Much of the orbital

data is obtained from Celestial BBS but additional data is generated by Ted

Molczan and his worldwide team of observers. Note that the CSS format is

slightly non-standard, having additional information on the first (title)

line for each satellite, and may have to be edited for use with some

tracking programs. The CSS files also have considerable additional text

material (including current satellite news) before and after the actual 2-

line elements data.

The U.S. Space Command sends all unclassified 2-line elements to the

Orbital Information Group at Goddard Space Flight Center. These elements

are available on the GSFC OIG RBBS, (301) 262-6784, 4 lines @ 1200 and 2400

baud. (NOTE: The RBBS telephone number was changed effective July 24,

1993!) The OIG database contains elements for some 7000+ satellites and is

updated every weekday morning except holidays. Elements for popular

satellites are posted in seven ".DAT" files (with the NORAD number only in

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 127

the first line of data rather than the more usual satellite name or IAU

identification) and all other satellites are available on a query basis. I

regularly post a combined and sorted set of the .DAT data as file

GSFCnnn.ZIP on my own RPV ASTRONOMY BBS where "nnn" is a number like "269".

Individuals who wish access to the RBBS must write (include full name and

address):

NASA Goddard Space Flight Center

Project Operations Branch/513

Attn: Orbital Information Group

Greenbelt, MD  20771  USA

The OIG RBBS began operation in September, 1991. Until that time, the only

method for obtaining the OIG data was by mail. OIG now plans to discontinue

all mail services in early 1993 and thereafter the only method for

obtaining the OIG data will be via the RBBS.

The NASA SpaceLink BBS in Huntsville, Alabama, (205) 895-0028, 8 lines

@ 300-9600 baud, provides mission information for all space shuttle

missions and (usually) 2-line orbital elements both pre-mission and while a

mission is in progress. 2-line elements for selected satellites are also

available. In addition, SpaceLink has a wealth of other NASA information,

computer programs, teaching materials, and image files.

In January of 1993 the Public Affairs Office at the NASA Jet

Propulsion Laboratory began BBS service related to JPL-supported missions.

Mission status reports and high quality GIF images are the principal files

currently available. The number is (818) 354-1333, two lines at 1200 to

9600 baud.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 128

STSORBIT PLUS Revision History

------------------------------

Each released version of STSPLUS uses a four digit revision code such

as 9338. The first two digits indicate the year and the second two digits

indicate the week of the year. In some cases, an additional letter suffix

is added to distinguish changes occurring within the same week or to

identify special versions. A partial week at the beginning or end of the

year is counted as a full week. Using this method, a year will typically

have 53 weeks although it is possible to have 54 weeks in a leap year (1972

is an example). The current year-week revision code is shown on the Julian

Date display, Display Mode 7, in my program ASTROCLK.

This file records the revision history of program STSPLUS through all

of the minor twists and turns that usually accompany the evolution of such

a complex program. It illustrates the tortuous process of maintaining and

refining a program as ideas and problems are reported from every quarter.

These notes may also be helpful to users who are upgrading from one version

to another to find out what has changed.

David H. Ransom, Jr.

Version 9338 -- September 1993

------------------------------

-Repaired a bug in multi-satellite modes which caused a "trail of dots" to

be left behind if the primary satellite icon fell on top of a secondary

satellite icon (such as during close proximity operations).

-Repaired a bug when reading SCF files. In prior versions, the primary

satellite was changed if the SCF file had a different primary satellite.

The primary satellite in the SCF file is now ignored. The format of the SCF

file is unchanged to maintain compatibility with prior versions.

-Modified the World and Zoom Maps in rectangular projection so that the

PgDn key switches to Zoom Map (180 degrees) from World Map, and the PgUp

key switches to World Map from Zoom Map (180 degrees).

-Added list of available .SCF files when reading, F6+F3 from the Main Menu.

-Added a warning when writing .SCF files if the requested file already

exists. F6+F2 from the Main Menu. User must now press "Y" to overwrite an

existing file, ENTER or any other key to cancel.

-By user request, modified the small character set (used for labels) to

include the DASH ("-").

-After numerous user complaints, I've finally repaired the BLINK mode so

that for most systems the primary satellite icon will blink on and off once

per second. Press "B" while the map is displayed to toggle the BLINK mode.

-Repaired a minor bug which caused the message "Pause ... press ENTER" not

to be displayed if the map was redrawn during Pause Mode.

-Thanks to the many users who took the time to carefully document these

bugs, enabling me to find and fix them!

Version 9334 -- August 1993

---------------------------

-The maximum number of Static and Real Time satellites is increased to 32.

This permits the entire GPS constellation, currently 25 active Block I and

Block II satellites, to be tracked. The satellite setup display, F6+F1 from

the Main Menu, has been modified to two display pages of 16 satellites

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 129

each; the second page of 16 is not displayed if no active satellites are

present.

-Added "Clear Static and Real Time Satellites", F6+F4 from the Main Menu,

to speed reconfiguration of these satellites. It may be used to clear any

block of satellites NOT INCLUDING #1 AND #2 (which are assigned to TDRS

East and TDRS West and must be cleared manually). Users are cautioned that

once cleared, the satellite data for the cleared satellites is lost. If in

doubt, save the current satellite configuration to a SCF file using F2

BEFORE clearing!

-Somehow the "bullseye" was lost in the Tracking Mode for rectangular

projection several releases back. It has now returned. Thanks to Alan Pound

for reporting the problem!

-As an experiment, I changed the color of land boundaries from Light Cyan

to Cyan to make multi-satellites and other display features a little easier

to spot. Feedback welcome!

-When Extra Grid Lines are OFF (F10+F3+F3), changed the color of the grid

lines from Light Blue to Blue and added lines of latitude at +80 and -80

degrees.

-Various repairs to eliminate spurious pixels and other minor problems

associated with multi-satellites.

Version 9333 -- August 1993

---------------------------

-This is a MAJOR UPGRADE, adding multi-satellite capability. This is the

first in a series of upgrades for tracking multiple satellites and

rendezvous missions. Comments, suggestions, and bug reports are welcome!

-Special thanks to Joel Runes for his assistance during beta testing!

-OPERATIONAL HINT: With the addition of multi-satellite tracking, the

Motion Map feature now really comes into its own; press "M" when the normal

map is displayed to switch to the EGA Motion Map, press "M" or ENTER to

return to normal map or to the Main Menu. All satellites are updated as

fast as the computer can draw the map. See the notes on a "vanilla" boot

without memory managers for dedicated STSPLUS uses.

-The format of file STSPLUS.INI has been substantially modified and files

from prior versions of STSPLUS will be ignored. The configuration

information must therefore be re-entered.

-Increased to sixteen the number of additional TDRS and real time satellite

positions in file STSPLUS.INI so that all active satellites may be updated

and displayed. Sample data are shown below. See the section "TDRS and Real

Time Satellite Features" for additional information.

19883,"TDRE ", -0.04218, -41.14169,10014

21639,"TDRW ", -0.03643,-174.14074,10014

22314,"TDR5 ", -0.31155,-138.36021,10004

19548,"TDR2 ",  0.05352, -61.66467,10004

13969,"TDR1 ", -6.59117,-170.55876,10004

16609,"MIR  ",-17.45706,-178.73251,14115

-TLEs for each active satellite are now saved in the .INI file so that

accurate positions may be calculated dynamically. These TLEs should be

updated periodically.

-TLEs are may now also be saved in and read from special .SCF (Satellite

Configuration File) files so that multiple TDRS and Real Time satellite

configurations may be saved and/or selected. A sample file, STSPLUS.SCF is

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 130

included in the standard distribution. Function Key F6 from the Main Menu

is used to display, modify, save, or read these data.

-The 5-character abbreviation of each TDRS satellite may now be used to

label the satellite. The size and color of the icon used to display each

TDRS or real time satellite may now be independently controlled.

-STSPLUS now uses the calculated LATITUDE for the display of all TDRS

satellites. This will cause TDRS1, for example, to appear up to 7 degrees

North or South of the Equator (as of 8/9/93).

-The drive, path, and name of the .CTY file may now be set with F7 from the

Main Menu.

-The Motion Map is now available for the World Map in rectangular

projection. This is helpful with multiple real time satellites.

-Repaired an infrequent bug discovered by Joel Runes which causes the

orthographic map to be redrawn continuously when a) the satellite has a

high eccentricity (.73 in the test case), and b) when the map is being

drawn at approximately the time of perigee. The code has been modified to

take the Eccentricity into account in calculating the map offset time.

-Corrected a minor bug which caused MET in excess of 9999 days to overflow

the assigned format statement for the data block and appear as

"%12038/00:00:00 MET" with the "MET" wrapping around to the left side of

the screen. Obviously, there aren't many satellites with this problem, but

NORAD #00051 is one such. The STSPLUS.LTD entry for this satellite is:

00051,2437158.90208333,0

and the problem was noted because of the pre-launch "temporary" NORAD

number assignment for STS-51.

-A similar problem occured with the large clock displays. In both

projection modes, only the last four days of MET are now displayed (12038

days will display as "2038").

-Added a reminder in the text that when entering filenames using F7 from

the Main Menu, a drive and path may also be included if desired. That

capability has been present for some time but was not stated explicitly.

-Changed the precision that the "plain English" display of the elements

Epoch Time is displayed for approval (F2 from the Main Menu) from [rounded]

integer seconds to 0.001 seconds, coordinated with VEC2TLE V9331.

-Several V9332 Beta Versions were released privately for testing.

Version 9331 -- July 1993

-------------------------

-In coordination with Ken Ernandes' VEC2TLE, the data output for Format 7,

ECI State Vector (Labeled Data), has been modified. The data have been

supplemented by the addition of two lines: Element Set Number; and, Rev

Number at Epoch. The IAU Designation has been added to the Catalog Number

line and the Epoch Time is now given to .001 seconds. The data output for

Format 4 has been changed so that it is identical to Format 7. See text for

details.

-Per Ken Ernandes, the Format 4 & 7 label "Ndot/3" is changed to "Nndot/6".

-I have received NO REPORTS from anyone using Data Formats 5 and 6. Those

formats may not continue to be supported indefinitely UNLESS I am informed

of applications which use them.

CAUTION: STSPLUS Version 9331 Data Formats 4 and 7 are NOT compatible

with VEC2TLE Version 9322 when multiple state vectors are generated!

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 131

Use a release of VEC2TLE with a version number equal to or greater

than 9331. (However, manually editing the data to remove the Elset and

Rev Number lines can restore compatibility with the older version.)

NOTE: Users are urged to update to VEC2TLE Version 9331 or the current

version, which ever is later! Not only does the current version of

VEC2TLE maintain compatibility with STSPLUS but several important new

features have been added and a bug in Version 9322 which affected data

accuracy under certain circumstances has been repaired.

-The Tracking Station Mode is now active in both Orthographic and

Rectangular projections. The projection used will same as the one which is

active when the "T" key is pressed.

-Repaired a bug which caused the NORAD number displayed in the Pass

Prediction Mode to be incorrect under certain circumstances. Thanks to

Grant Pinto!

-Repaired a bug in the pass prediction logic which caused the MAX

VISIBILITY data to be missstated by a significant amount for certain

satellites, especially ones with higher eccentricities. Thanks to Grant

Pinto for spotting and reporting the bug!

-Changed the default time zone abbreviation for unrecognized (foreign) time

zones from all spaces (which confused the Pass Prediction setup!) to "LCL".

-Corrected a bug using F2 from the Main Menu which showed no 2-line

elements files if no .TXT files were found in the current directory but one

or more .TLE files were present. Thanks to Bob Krohn for reporting the bug.

If no files of either filetype are found, an error is displayed and the

user is returned to the Main Menu.

-Corrected a bug in default paths if the root directory was used. (Path

defaulted to "B:\\" instead of "B:\" for drive B:, etc.)

-Corrected a bug in the NORAD number for F2 from the Main Menu. If the

NORAD number was greater than 32767, a negative nunber was shown.

-Disabled RIGHTIME detection for 8086/8088 processors. The detection

process affected the time in certain XT-class machines.

-Corrected a minor bug which caused the "blink" feature for the satellite

icon not to function under certain circumstances.

-Corrected a minor bug which initialized the Location Flag incorrectly for

CGA systems when file STSPLUS.INI was not present. Locations and Features

were displayed even though the menu display indicated they were OFF.

-Various cosmetic changes.

-Most of the changes in this release were included in Beta Version 9329.

Version 9320 -- May 1993

------------------------

-This is a MAJOR UPGRADE, adding new improvements and features along with

high precision state vector data output for use with Ken Ernandes' program

VEC2TLE.

-By popular request, STSPLUS now estimates if a satellite may be visible to

the naked eye or binoculars, and displays "VIS" in bright white next to the

orbit inclination if a visual sighting may be possible. See the section

"Satellite Visibility" for additional discussion.

-Also by popular request, the pass predections (F3, Data Mode 9) have been

enhanced to permit dates and times to be displayed for either UTC/GMT or

LOCAL time. An additional prompt has been added for that selection.

-When displaying predicted passes (F3, Data Mode 9), the satellite is now

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 132

approximately centered in the display (instead of being well to one side).

-After considerable confusion and several user comments, I have reworked

the pass prediction logic so that when the user returns to the Main Menu

after the ground track for a predicted pass is displayed, the time is

automatically restored to the real or simulated time in effect BEFORE the

pass prediction was displayed. This means that repeated use of the pass

prediction feature will generally display the same list of numbered passes

and the user no longer needs to restore real time (or reset simulated time)

after displaying predicted passes.

-Corrected a cosmetic bug on pass predictions when an illegal pass number

(greater than the last pass number displayed) was entered at the prompt.

-Added an asterisk ("*") at the left of each event timer when the satellite

is AOS (signal/Sun is being received). This will particularly benefit users

with monochrome or shades of gray.

-Enhanced the Precision X-Y-Z State Vector Data Modes (F3, Data Modes 4

through 7) to include four output formats: multi-line Ascending Node with

state vector, 2 numeric data lines, comma delimited, and multi-line labeled

data. State vectors may now be logged continuously, for a specific time, or

for a specified time span. See text for details.

-Corrected a problem with state vectors being generated at the wrong time

(Data Modes 5 through 7) when the time was entered in UTC and local time

was a different date.

-Removed the low precision state vector data output (F3, Data Mode 4).

-Changed the angle used to calculate the Earth's partial penumbra from 1.2

degrees to 0.3 degrees to better correspond with observed lighting. The Sun

AOS and LOS penumbral calculations were also slightly adjusted; timings

during STS-56 indicated about a 15 to 20 second error before these changes.

-Updated the TDRS information in the section "TDRS Satellite Features" and

all five TDRS satellites are now displayed on the maps at their approximate

locations as of 05/05/93. Thanks to Jim Walls for reminding me!

-Corrected a bug which caused "BASIC Error = 5" when the selected satellite

had probably decayed. #22209 MIR Debris was an example in file TLE180.TXT.

STSPLUS now displays a warning message if the satellite has a current

altitude less than 75 nautical miles and then returns to the Main Menu.

-Corrected the conversion from kilometers to feet per Ken Ernandes. (My

original conversion factor was taken from a 40+ year old Handbook of

Chemistry and Physics and was very slightly in error!)

-Repaired (I think...) a truncation problem which sometimes caused MET/T+E

to run one second slow.

-For all those who refuse to read documentation, I added a reminder to the

Main Menu: "WHILE MAP IS DISPLAYED: F1 = HELP, ENTER = Main Menu".

-Special thanks to Ken Ernandes, Joes Runes, and Willie Musty for state

vectors, testing, and validation of the new features in recent versions!

-Version 9319 was released privately for beta testing.

Version 9316 -- April 1993

--------------------------

-This is a BETA MAINTENANCE UPDATE not released publicly.

-Added Data Output Function #5, "Precision XYZ State Vector (km)", to

generate full precision state vectors. The position and velocity data are

in km and km/sec. The format is similar to Data Output Function #4 except

that it requires two data lines per state vector.

-Help is now available in all display modes by pressing Function Key F1.

Press F1 again to return to normal data display.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 133

-The default (and minimum) magnification for the End key in orthographic

modes has been changed from 100 to 150. Thus, pressing the End key will

return to the prior magnification or to MAG=150, whichever is greater.

-Added missing minus sign on large character MET during the last 24 hours

prior to launch time. Display was "0/00:01:00" instead of "-0/00:01:00".

-Corrected the latitude for Mauritius in file STSPLUS.LOC. The latitude was

missing a minus sign! Changed the "trigger" magnitude for Athens, Greece to

500 from 2000 following a user complaint. Also corrected the longitude for

Luxor, Egypt which was off just that little bit ...

-Various minor cosmetic changes.

Version 9314 -- March 1993

--------------------------

-This is a MINOR UPDATE to correct several problems. Except for these

notes, the documentation is essentially unchanged.

-Corrected a bug with F6 on the Main Menu. Switching between MET and T+E

using F6 changed the label but NOT the data. (F5 worked correctly when the

ground track is displayed.) Thanks to Paul Ferrante for reporting the bug.

-Corrected (again!) a format error in file STSPLUS.OBS which is created for

use with TRAKSTAR. For names shorter than 20 characters, the numerical data

was shifted left. Thanks to Michael Simmons for helping track down the bug!

-Corrected the CPU identification displayed for 386 processors to "80386DX

or 80386SX". Due to a typo, the SX processor was shown as "80387SX".

-Corrected the spelling of Yarragadee, Australia. Thanks to Elwood Marshall

for the correction.

Version 9311 -- March 1993

--------------------------

-This is a FULL RELEASE with updated documentation, and includes all

changes listed here as well as those made in Preliminary Versions 9307

through 9310. Please see the update notes below for details.

-Added F1 to the Main Menu, Convert Keplerian Elements to 2-Line Format,

which incorporates the features formerly provided separately in my program

MAK2LINE. See new text for discussion and for a form which may be used to

transcribe data received by voice or modem.

-Repaired a bug which caused the Location Map isocontours to be drawn at

the wrong location when a second location was enabled in orthographic mode.

Thanks to Paul Ferrante for reporting the problem!

-Added Function Key F9 to the Time and Date Menu (F8 from the Main Menu) to

show the last system TIMESET and the current RIGHTIME corrections. This

menu item will not appear if program RIGHTIME is not detected.

-Added additional code to maintain the prior state of RIGHTIME per Tom

Becker. Thanks, Tom!

-Various minor cosmetic changes.

-Documentation updated.

Version 9310 -- March 1993

--------------------------

-This is a MAINTENANCE UPDATE and also includes several improvements to

Version 9309. Users updating from Version 9250 should also read the notes

for Version 9309 below.

-In my rush to release Version 9309 of STSPLUS prior to STS-55, I neglected

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 134

to finish updating the code for F2 (Read 2-line elements). Hopefully, that

code has now been repaired correctly ... sorry!

-STSPLUS is now "aware" of program RighTime by Tom Becker. Audible alarms

in prior versions would perform unpredictably when RighTime was active

because MS BASIC uses the hardware clock's timer functions for "music"

(which RighTime also uses). STSPLUS now detects RighTime and temporarily

disables RighTime while an audible alarm is being generated and then re-

enables RighTime after the alarm has completed, restoring precise

timekeeping. With RighTime active, alarms are generated in foreground,

which may cause a slight delay in screen updating.

-CAUTION: STSPLUS expects RighTime Version 2.5+; performance with prior

versions of RighTime may yield unpredictable results. If using a prior

version of RighTime, do NOT enable audible alarms!

-If RighTime is not present or is not detected, the audible alarms are

generated in background as in prior versions. This usually causes the loss

of several clock ticks in the DOS software clock for each audible alarm.

Although the time loss per audible alarm is very small, the cumulative

error may become significant over extended time periods.

-Added the flashing message "calculating ..." during Pass Prediction

calculations so that users with slow computers (especially those without a

math coprocessor) will know that the process is continuing. Users with 286

computers and no math coprocessor report delays up to tens of minutes for

satellites with low mean motions!

Version 9309 -- February 1993

-----------------------------

-This is a PRELIMINARY RELEASE of a major update for STSPLUS and the first

public release since Version 9250. This version is being released as an

UPDATE ONLY for Versions 9250 and 9307; the full version will be released

shortly, complete with full documentation, pending the results of testing

for this preliminary release. See also the update notes for Version 9307

below. Comments and suggestions are invited, particularly with respect to

the new Data Output and Pass Prediction features.

-Function Key F3 on the Main Menu, Pass Predictions and Data Output, has

been rewritten and improved.

-Added Line-of-Sight Pass Predictions for the current satellite. Use F3

from the Main Menu and select Data Format 9. Pass predictions are made for

48-hour blocks and for up to 99 passes. Users may immediately display a

listed pass by entering the pass number. STSPLUS then automatically sets

SIMULATED TIME to the middle of the pass and switches to the ground track

display. See the section "Pass Predictions and Data Output" for additional

information. The capability to use TRAKSTAR or other external tabular pass

prediction software has been retained for those who prefer that technique.

-Improved and refined the Data Output capabilities introduced in Beta

Version 9307. Added FILE Data Output to file STSPLUS.LOG, and PRINTER Data

Output to printer LPT1:.

-Modified F2 on the Main Menu (Read 2-line Elements) to require the "#"

symbol when entering the NORAD number; enter the NORAD number as "#20580".

This now permits satellite names such as "1993 012A" to be found correctly;

prior versions would treat such an entry as a NORAD number and the search

would fail.

-Entering a full drive and path specification in addition to a filename

with F2 on the Main Menu (Read 2-line Elements) will now correctly set the

path without the need to use F7 (Set Paths and Filenames). Root directories

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 135

("C:\") are now processed correctly. Press ESC to cancel.

-Added text section titled "Time and the Personal Computer" which offers

information on setting and maintaining the PC clocks over long periods of

time. Includes information on programs TIMESET and RIGHTIME.

Version 9307 -- February 1993

-----------------------------

-This is a BETA TEST VERSION, not released publicly.

-Added Function Key F3 to the Main Menu. This provides data output on the

selected COM port with specified data items. Three different data formats

have been implemented so far.

-Removed the original Function Key F3 (Read .INI file) from the Main Menu.

Use F2 to read 2-line elements and/or set file names. In Version 9250, the

satellite data was not being read when the .INI file was changed. This was

caused by logic changes to speed up initialization of the program. Also,

reading "foreign" .INI files sometimes caused problems.

-Corrected a bug which caused a new launch time/date NOT to be added to

file STSPLUS.LTD if the curret satellite's NORAD number was less than the

last NORAD number in the file.

-In response to a request by Maj. T.S. Kelso, USAF, who prepares the 2-line

orbital elements which I post on my RPV ASTRONOMY BBS, I have changed the

name of that file from "NASAnnn.TXT" to "TLEnnn.TXT". The default file

which STSPLUS looks for has been similarly changed. Since the orbital data

in the file does NOT come from NASA but from US Space Command, the use of

"NASA" in the filename was confusing and did not give credit where it was

due.

-Corrected a minor bug which caused five pixels to be restored incorrectly

under the starbord (right) wing of the space shuttle icon.

-Modified the operation of Function Key F5 from the Main Menu to ADD or

UPDATE new launch time and date in file STSPLUS.LTD rather than to append

the new data at the end of the file. This assures that the satellite is

included only once in the file and that the new or updated information is

always read correctly. Prior versions required the user to manually edit

the file to remove multiple entries.

-Adjusted the maximum angle displayed for the isocontours (concentric

circles of satellite visibility) on the Location Map in orthographic mode

to take satellite altitude into account.

-Corrected the STSPLUS.OBS file format for use with TRAKSTAR. The data was

correct but the spacing caused TRAKSTAR to sometimes read the data

incorrectly. Thanks to Michael Simmons for reporting the problem!

-Corrected the latitude of Sao Paulo, Brazil in file STSPLUS.LOC. It was

one degree off to the South and wound up in the Atlantic Ocean ...

Version 9250 -- December 1992

-----------------------------

-This is a MAINTENANCE AND FEATURES UPDATE.

-In response to numerous suggestions, I have redesigned and enlarged the

space shuttle icon and switched from a profile to a plan view to make the

icon easier to find on the screen. The new icon has almost three times as

many pixels and takes more time to draw. Comments are invited!

-In response to many user requests, I have improved file selection logic

for 2-line elements using F7 from the Main Menu to "remember" the path and

save it in file STSPLUS.INI. Users may now keep all 2-line elements files

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 136

for STSPLUS and other tracking programs in a separate directory.

-While the default filetype for 2-line elements is still ".TXT", the

program now also searches for filetype ".TLE".

-Event timers (showing AOS and LOS for the local station, TDRS, etc.) are

now enabled by F10+F7 from the Main Menu.

-I have added the Location Map in orthographic mode; press "L" while an

orthographic map is displayed. The projection used for the Location Map is

now based upon the current projection, rectangular or orthographic, when

"L" is pressed.

-Improved resolution of spacecraft lighting from 10 seconds to 1 second and

the Sun's position is now calculated every 10 seconds instead of every 60

seconds.

-Spacecraft lighting icon and symbol (to the right of "Orbit #:" in the

data block) now indicate the full range of solar illumination:

*    Bright White   Full sunlight

+    Yellow         Partial sunlight (penumbra)

-    Light Red      Refracted sunlight

White          Full shadow (umbra)

Note that LIGHT RED and the "+" and "-" symbols have been added to

distinguish refracted sunlight from partial sunlight.

-In response to several user requests, I have added AOS and LOS event

timers for orbital sunrise and sunset, displayed when the solar features

are enabled. In order to avoid excessive delays during the ground track

plotting (when the timers are set up), I have used a simpler algorithm for

orbital sunrise and sunset than that used during program operation. The

simpler algorithm is usually accurate to about plus or minus 15 seconds.

The actual spacecraft lighting is calculated dynamically and is both more

accurate and takes into account the non-spherical shape of the Earth.

-After several user comments about the "big clocks", I have again tweaked

the shape of the large digit "5" (back to what I started with) and modified

the "6" to better distinguish it from the "5".

-In order to free a display line in orthographic mode, the map drawing time

and map database file information have been combined on a single line (e.g.

"EARTH4  10.91") at the bottom of the display block.

-STSPLUS now automatically detects the type of processor and math

coprocessor; if no math coprocessor chip is detected, the program displays

a caution message at startup. The SLOW MODE is now automatically set or

reset depending upon the presence or absence of the math coprocessor and

the menu selection for the SLOW MODE has been removed.

-Performance may be improved if a memory manager such as EMM386, QEMM386 or

386MAX is NOT used! See the section "PROGRAM SETUP AND USAGE NOTES" for

specific information.

-Changed the grid spacing in orthographic mode, MAG=200, to 10 degrees from

5 degrees to make the display a bit less cluttered.

-STSPLUS is now compiled using Microsoft Visual Basic for DOS.

-Updated this documentation. Added the sections "SUN and Solar Features",

"Event Timers and Audible Alarms", and "F5  Display Ground Track:

DOTS/LINE". Moved the section "F5  Show Ascending & Descending Node Data"

to the proper place in the text.

-Corrected a bug which caused a minus sign (if present on Y or Z) to

persist when changing from XYZ to Alt/Az or RA/DEC satellite coordinates.

-Various minor cosmetic changes.

-Version 9249 was a limited release beta version.

Version 9245 -- October, 1992

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 137

-----------------------------

-This is a MAINTENANCE UPDATE, correcting several bugs and adds the launch

date and time capability.

-Added file STSPLUS.LTD which contains the NORAD number and launch date

(Julian date, UTC) for selected satellites. A sample entry appears as:

22194,2448918.21503472,0

This example is the data for Space Shuttle mission STS-52 (NORAD #22194)

and corresponds to a launch date and time of 22 OCT 1992 @ 17:09:39 UTC.

The last parameter is reserved and should be set to zero. Launch date and

time may be appended to file STSPLUS.LTD when entered using F5 from the

Main Menu. See the section "Using File STSPLUS.LTD for Launch Date & Time"

for a complete discussion.

-Corrected a misplaced statement which caused "BASIC PDS Error = 5" when

the Satellite Motion Map was requested from rectangular modes.

-Changed future dots on ground track to LIGHT GREEN when only dots are

shown to avoid confusion with solar terminator. (That's what it was

supposed to be but I messed up ...)

-Changed satellite circle of visibility on the Satellite Motion Map to

solid white (from light white dots) for greater clarity.

-Added a default magnification of 200 for the End key.

-Corrected the coordinates for Shanghi, China, in STSPLUS.LOC.

Version 9244A -- October, 1992

------------------------------

-On rectangular projections, the LAUNCH TIME and LAUNCH DATE were shown

incorrectly. The calculations for MET were correct.

Version 9244 -- October, 1992

-----------------------------

-Version 9244 is a MAINTENANCE UPDATE which adds several new features in

addition to repairing a number of bugs which have been discovered as a

result of the addition of the orthographic map mode and other new features

such as enhanced TDRS coverage. This is still "early in the game" following

the major rework associated with the addition of orthographic projection.

Comments, suggestions, and bug reports will be appreciated.

-Added EGA dual-page Satellite Motion Map to orthographic modes, available

ONLY for EGA and VGA systems. Press "M" while the orthographic map is

displayed. Press "M" or ENTER to return to normal map modes. PgUp, PgDn,

Home, and End are the only other keys which are active with the Satellite

Motion Map.

-In response to MANY requests, I have restored the "red dots" for past

orbital ground track. (My original method no longer worked when I added the

orthographic projection.)

-AOS/LOS for secondary location now available in orthographic mode and in

rectangular mode with "/CLK" command line option AND when large clock is

selected for MET/T+Epoch (using F2 when the map is displayed). Finding

display space is getting very difficult, particularly in normal rectangular

modes! Primary location is labeled "STN1" and secondary location is labeled

"STN2".

-The PAUSE mode has FINALLY been repaired so that it operates correctly.

Press F6 while the map is displayed to enter PAUSE, press ENTER to resume

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 138

normal operation. Use the "+" and "-" keys to advance or retard the time,

and use F4 to set the time step. See the section "Pausing the Ground Track

Display" for a full description.

-The FAST mode has also been repaired so that it works more or less in the

manner intended. Press F4 while the map is displayed to enter FAST mode.

Note that the actual time increment is a function of the computer's speed.

For fast computers, the time increment will usually be 10 or 60 seconds but

may vary by a second occasionally; for slower computers, the time increment

may be somewhat longer. See the section "Using FAST Time" for a full

description.

-Corrected a problem with TDRS coverage when updating from one version to

another. The TDRS AOS/LOS labels read ":" instead of "TDRE:" and "TDRW:"

and the AOS/LOS calculations could be incorrect or blanked out. (The TDRS

data were not being set up correctly.) As a work-around, the problem could

be corrected by deleting file STSPLUS.INI and restarting the program.

-Corrected a crash with "BASIC PDS Error 5" which occurred under certain

circumstances when drawing circles of visibility in orthographic mode. (An

array index was being overrun.)

-Corrected a minor bug in the AOS/LOS logic that limited the time

resolution to 2 seconds instead of 1 second (4 seconds instead of 2 seconds

when the SLOW flag is set). Thanks to Joel Runes for spotting the problem!

-Various minor cosmetic bugs.

NOTE:     Version 9243 was a beta version released on a limited basis to

individuals who had problems and/or registered the program

immediately before and during Mission STS-52. Version 9243

included some of the changes noted above for Version 9244.

Version 9242 -- October, 1992

-----------------------------

-Version 9242 is primarily a Maintenance Update, correcting a number of

minor bugs and documentation errors. However, several "new and improved"

features have also been added as a result of continued testing and user

requests. Thanks to everyone who sent in encouragement and suggestions!

-In response to MANY user requests, I have added an interface (via F4 from

the Main Menu) to TS Kelso's new satellite prediction program TRAKSTAR.

Download TRAKSTR2.ZIP (filename for Version 2.15) from RPV ASTRONOMY BBS or

from Celestial BBS (see text for telephone number). CAUTION: TRAKSTAR uses

UTC time rather than LOCAL time! See text for full description and setup

instructions.

-By popular request, I've added an audible warning for AOS and LOS at the

user's location. The AOS warning occurs 2 minutes prior to AOS, and the LOS

warning occurs 30 seconds prior to LOS. Audible warnings are enabled and

disabled using F10+F8 from the Main Menu. Past versions of Microsoft's

compilers have caused the system clock to lose time when "music" was

played; I would appreciate feedback from users if this feature causes

problems.

-A similar (but shorter) audible warning has also been added for TDRS

coverage. The TDRS warnings occur 30 seconds before AOS or LOS if TDRS

coverage and audible alarms are both enabled.

-AOS and LOS clocks now change to YELLOW if the time is 2 minutes or less,

as stated in the documentation. A minor bug has been repaired that caused

the AOS or LOS clocks to sometimes "freeze" at 00:00 or 00:01 when no

future event was detected.

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 139

-AOS and LOS calculations have been improved in precision by a factor of

two and the average time required for the calculations has decreased by

about 5:1 using a binary search algorithm. This may be just an exercise in

performance improvement since factors such as antenna pointing errors and

atmospheric conditions may introduce errors of up to tens of seconds!

-When launch time and data have been entered and Mission Elapsed Time is

selected for display, the HOURS:MINUTES for MET will appear at the bottom

of the data block in large digits for orthographic modes. (When MAG is

greather than 100, seconds are not displayed because of space limitations.

The full MET, including days, is always displayed near the top of the data

block in normal characters.)

-Corrected error when changing to/from DAYLIGHT TIME using F8+F10 from the

Main Menu. If the Daylight setting is changed, STSPLUS now asks if you wish

to adjust your DOS clock. CAUTION: Setting the DOS clock may NOT execute

correctly for 8088 computers! (Prior versions required exit from the

program and restart before the time was shown correctly.)

-Corrected the satellite coordinates when orthographic mode was selected

with the second city displayed. (V9240 displayed the primary location name

but the data for the secondary location.)

-Corrected a problem (which left the screen black most of the time and then

was redrawn immediately) when Motion Map was requested (by pressing "M")

from other than Zoom Mode.

-Corrected TDRS AOS/LOS title placement error with "/CLK" command line

option.

-Meridian drawing algorithm adjusted to (hopefully) always draw the lines

of equal longitude to the bottom or top of the map in orthographic mode.

-Added a test for field overflow in the first derivative of mean motion

(dx/dt, the parameter following the epoch on Line 1). Field overflow is

indicated by a numeric field of ".********" and it is now set to a value of

1 or -1 depending upon the leading sign. This is often an indication that

the satellite has decayed or soon will decay.

-Corrected the default directory upon return from DOS using F9 on the Main

Menu. (Default directory was left where the user last logged and file

STSPLUS.INI was written there instead of where it belonged.)

-Corrected file list to show CIS.TRK instead of USSR.TRK and updated the

distribution files accordingly.

-Description of TDRS satellite locations in the text corrected.

-Description of MCX, MCP, and XYZ map database files corrected to restore a

line of text that was dropped by my word processor.

Version 9241 was released for beta testing only.

Version 9240 -- October, 1992

-----------------------------

*********************

*  IMPORTANT NOTES  *

*********************

After almost four years, I have at last discontinued support

for the simple orbital model in STSORBIT PLUS. Users who wish

to continue using the simple orbital model must use prior

versions of STSPLUS or use the original STSORBIT program. I

plan to add manual generation of estimated 2-line orbital

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 140

elements for the Space Shuttle in the next release of STSPLUS.

Motion maps are available only with rectangular projection.

The FAST modes are still not really repaired. Maybe I'll get

around to that in the next release ...

The South Atlantic Anomaly (SAA) is disabled for

orthographic modes until I digitize a better model from

NASA's Mission Charts.

-Version 9240 is a MAJOR UPGRADE and is the initial public release with the

orthographic projection techniques used in my program ORTHOGRAPHIC EARTH

VIEW.

-While the principal new feature is the orthographic projection, that

addition affected almost all parts of the program and many different

routines have been updated or modified. I have also rewritten the AOS/LOS

code for TDRS and the user's location. Although considerable beta testing

has been performed, there are surely a few bugs that have not yet been

discovered. Special thanks to Joel Runes for his help with the beta

versions! Comments and bug reports are welcome.

-Added Orthographic Projection maps and new map databases. Orthographic

maps are selected using the letter "O" when the map is displayed or by

selecting "ORTHO" using F10+F6 from the Main Menu. PgUp, PgDn, Home, and

End may be used to select the magnification factor. See the text for a full

description.

-A math coprocessor is REQUIRED for the orthographic maps. Orthographic

maps require MUCH more computation time. Most computers not equipped with a

math coprocessor chip, even 386 systems, will yield poor or unacceptable

performance in orthographic modes. Typical orthographic map drawing times

range from 5 or 10 seconds for a 486DX/33 to 600 seconds or more for an

8088 without a math coprocessor!

-All rectangular projection map routines have been rewritten to use the new

indexed map database files and the maximum zoom factor has been increased

to a field of view of 30 degrees. This yields much improved map detail at

higher zoom factors and decreases the map drawing time in most cases. The

old map database file, STSPLUS.MVF, may be deleted.

-Because of their size, the high detail map database files cannot be

downloaded from the RPV ASTRONOMY BBS except for callers with 9600 or

faster baud modems. See file README for additional information on these

files.

-The TDRS sub-satellite coordinates are now included in STSPLUS.INI and may

be updated by displaying the appropriate satellite. For TDRS East display

NORAD #19883, and for TDRS West display NORAD# 21639. See the section "TDRS

Satellite Features" for additional information. Thanks to Jim Henderson of

NASA White Sands (and others) for the suggestion.

-Changed TDRS coverage algorithm to reflect actual coverage as a function

of satellite altitude above the Earth's surface. Provided current TDRS

position data is used, accuracy should now be on the order of tens of

seconds. Prior versions assumed a "customer" satellite orbital altitude of

approximately 160 miles.

-Added location and features labels to all maps, enabled or disabled by

F10+F3+F9 from the Main Menu. Labels are contained in file STSPLUS.LOC. See

the section "Location and Features Labels" for additional information.

-Rewrote the time base algorithms to better accomodate slower computers and

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 141

the longer calculation times required in orthographic modes. The program

now operates correctly even when the map drawing and updating times are

longer than ten seconds. Updating of the spacecraft circle of visibility,

for example, will occur at ten second intervals if the necessary

calculations and screen operations can be performed in less than one

second; if not, the update interval will be adjusted as necessary.

-Partly because of the new timebase algorithms, the ground track has been

changed. The "minute marker" dots are now yellow when DOTS+LINES are

enabled and the dots no longer change color. (A synchronization problem.)

-Changed the algorithm for the Circle of Visibility to correctly calculate

near the poles. The new algorithm is now used in all map projections. Users

who complained about the circles of visibility for high inclination orbits

will find them now correct -- although somewhat slower. The algorithm was

adapted from FORTRAN code generously supplied by Paul Traufler. Thanks

yet again, Paul!

-Changed most circle of visibility plotting routines from double to single

precision and used in-line code to improve performance.

-Added a black dot on the satellite icons to show the center of the icon.

-Corrected a fatal error if the map data files were missing. This problem

was frequently caused by using an old version of PKUNZIP (earlier than

Version 1.10) which could not unpack the map data file, the only file which

used the most recent compression algorithm. No error message was displayed

and the computer had to be rebooted. (Users who downloaded the program from

NASA SpaceLink BBS often had this problem since NASA had the wrong version

of PKUNZIP posted -- which they corrected at my request!)

-Corrected an error in the internal tracking station data which caused a

fatal Basic PDS Error 3 ("Syntax Error") if file STSPLUS.TRK was missing.

-Changed "current file" display to use inverse video when selecting 2-line

elements files. The color change used in prior versions did not show on

monochrome systems!

-Corrected a bug in the 2-line file selection logic that prevented entry of

a specific filename and improved the error trapping.

-Corrected the longitude of Cape Canaveral in STSPLUS.CTY so that it was

not out in the Atlantic Ocean. (Thanks to all of you who reported it!)

-Increased internal STACK size to 6144 bytes (vs. 3072) in the hope of

eliminating some reported memory problems (which I can't duplicate).

-Corrected a minor bug in the (25 line modes only) large character display

of MET/T+Epoch if the days were four characters; the time was shifted left

one digit. Days may now range from -999 to +9999. Also eliminated a double

minus sign if MET or T+Epoch was negative.

-Corrected a minor bug in satellite coordinates if a distance was greater

than -99999.99 km/nm. The display wrapped because of field overflow. The

distance is now shown as -NNNNNN.N (one less digit to the right of the

decimal point) for large distances. Most folks won't notice this one!

-Corrected a minor bug in large character negative MET/T+Epoch that caused

the time to be green for one second before turning red under certain

circumstances.

-Corrected an occasional bug which caused the colors to go awry or to

monochrome.

-Various minor cosmetic bugs.

(Intermediate update notes deleted to save space, available on request.)

Program STSORBIT PLUS Satellite Orbit Simulation                   Page 142

Beta Version 9137 -- September, 1991

------------------------------------

-Initial public beta version.

##########################################################################################