-------------------------------------------------------------------------

The following are things that are specific to CLOVER

-------------------------------------------------------------------------

In terminal mode there are a small number of commands you can use to control

the operation of the PCI-4000 and it's radio.  These commands are also

available in FWD.MB scripts, and as "Init", "Connect",  and "Discon"

commands in PORTS.MB.

CH n	    -  Use voice subchannel n (1 to 5).

CN <call>   -  Initiate a normal link to <call>.

CR <call>   -  Initiate a robust link to <call>.

CW <on/off> -  Turn CWID on or off.

RC <number> -  Set CCB retry count.

RN <number> -  Set NORMAL retry count.

RR <number> -  Set ROBUST retry count.

BF	    -  Set fast bias.

BN	    -  Set normal bias.

BR	    -  Set robust bias.

S  <nn>     -  Set chat threshhold to nn (0 is normal).

***  Serial port control of the radio:

These commands support COM1 through COM4, at the following addresses:

COM1 - 3F8

COM2 - 2F8

COM3 - 3E8

COM4 - 2E8

Note that the COM port baud rate, etc. must have been previously set

(using for example the DOS MODE command), and that CTS/RTS handshaking

is used.  If CTS/RTS is not required, those pins must be jumpered at

the PC end of the cable.

--------------------------------------------------------------------------

"XI" command:

Used to send initialize the serial port used to control a radio.

XI p baud parity bits stop

Example:

XI 1 4800 N 8 2

--------------------------------------------------------------------------

"X" command:

Used to send commands to a Kenwood radio using the Kenwood interface.

X p cmd cmd ... -  Send commands out serial port p.

Examples:

PX 1 FN2;    ->  Using COM1, tell transceiver to use memory channels.

PX 1 SC1; SC0; MC023;  ->  Start scan, stop scan, QSY to memory channel 23

before making this connect.

QX 1 MC001;  ->  QSY to memory channel 1 after disconnecting.

--------------------------------------------------------------------------

"XY" command:

Used to send commands to a Yaesu radio using the CAT system.

XY p opcode par1 par2 par3 par4

The opcode and par are the numbers per the Yaesu CAT documentation.

Bytes are sent in reverse of the order given, i.e par4 par3 par2 par1 opcode.

If the CAT opcode does not require all parameters, you may leave them off.

Examples:

XY 1 2 12 0 0 0  ->  Set radio to memory channel 12

XY 1 2 12	 ->  Set radio to memory channel 12

--------------------------------------------------------------------------

The following are available only in terminal mode.

T	    -  Send test tones.

I <a> <b>   -  Initialize the board using the PCC.LOD file given as <a>

and the PCC.S28 file given as <b>.  MYCALL is set, and

other parameters required for correct MailBox operation

are also set.

I	    -  With no file names given, defaults to using the files

\PCC\PCC.LOD and \PCC\PCC.S28

< file	    -  Read commands from file.