--------------------------------------------------------------------

IRDQ - INFRA-RED Command Line Control

--------------------------------------------------------------------

Version 2.0

November 1994

Copyright 1993,1994 - D.Huras - All Rights Reserved

------------------------------------------------IRDQ.DOC v2.00------

CONTENTS

--------------------------------------------------------------------

1. DESCRIPTION

2. WARRANTY

3. SETUP INSTRUCTIONS

4. COMMAND LINE ARGUMENTS

5. IRDQ DOS ENVIRONMENT VARIABLE

6. BUTTON NAMES

7. PROGRAM OVERVIEW

8. RELEASE HISTORY

1. DESCRIPTION

--------------------------------------------------------------------

IRDQ is a program that sends commands to a ONE FOR ALL (*1) Remote

Control that instruct the remote control to emit INFRA RED (I/R)

signals.  Essentially, IRDQ emulates a person pressing buttons on

the remote control keypad.  Any button sequence that can be

physically entered on the remote control keypad can be emulated with

the IRDQ program including setting the Device type.

IRDQ is a companion program to IRDC.   The full functioning version

of IRDQ is supplied with the Registered Version of IRDC.  A limited

function (*2) version of IRDQ is supplied with the Shareware Version

of IRDC.

This brief document focuses on the use of IRDQ and is meant to

accompany the IRDC document which goes into much more detail

regarding the hardware interfaces, communication ports, software

applications and problem determination.

----

(*1) ONE FOR ALL, ONE FOR ALL 5, ONE FOR ALL 6, ONE FOR ALL 8,

ONE FOR ALL 12 and The "Universal" Remote are trademarks of

Universal Electronics Inc.

(*2) The limited function version is subject to the following

limitations:

i)   5 Second Pause upon program load.

ii)  Quiet Mode not supported.

iii) Supports only 2 Button names per file with /F: argument.

2. WARRANTY

--------------------------------------------------------------------

IRDQ is supplied AS IS.  The author disclaims all warranties,

expressed or implied, including, without limitation, the warranties

of merchantability and of fitness for any purpose.  The author

assumes no liability for damages, direct or consequential, which may

result from the use of IRDQ.

3. SETUP INSTRUCTIONS

--------------------------------------------------------------------

Setup Instructions differ based on the model of the ONE FOR ALL

remote that you are using.  It is very important that you correctly

identify the model before using the program.  Refer to the IRDC.DOC

file to identify your remote and then follow the appropriate Setup

Instructions below.

SETUP INSTRUCTIONS FOR:

-----------------------

ONE FOR ALL 6  [URC4000]

ONE FOR ALL 12 [URC5000]

1) Plug the Special Serial Cable into the PC Serial Port.

2) Plug the Special Serial Cable into the ONE FOR ALL Remote Control.

3) Ensure that there are no loose connections.

4) Run the IRDQ Program.

5) VERY IMPORTANT: DO NOT TOUCH ANY BUTTONS on the remote control when

when using the software.

NOTES:

------

� The author assumes no liability for damages, direct or

consequential, which may result from the use of IRDQ.

� If you physically press buttons on the keypad while the software

is communicating with the remote control strange things can happen

that could affect the operability of your remote control.  If the

memory contents of your remote control are altered then the only way

to restore it to normal operation is to have it reloaded by

Universal Electronics or a local Service Dealer.

--------------------------------------------------------------------

SETUP INSTRUCTIONS FOR:

-----------------------

ONE FOR ALL 5  [URC2005]

ONE FOR ALL 6A [URC4005]

ONE FOR ALL 6B [URC4040]

ONE FOR ALL 8  [URC6050]

1) Plug the Special Serial Cable into the PC Serial Port.

2) Plug the Special Serial Cable into the ONE FOR ALL Remote Control.

3) Ensure that there are no loose connections.

4) VERY IMPORTANT: You must physically press a button on the remote

control and then run the IRDQ program with a button name WITHIN 5

SECONDS to put the remote online. Once the remote is online it will

remain online (and will not respond to physical key presses) until

you explicitly take it offline by running IRDQ with the "OFFLINE"

button name (or the OFFLINE button within a button file).  The LED

on the remote will flash twice in acknowledgement of the request.

NOTE:

-----

� The author assumes no liability for damages, direct or

consequential, which may result from the use of IRDQ.

4. COMMAND LINE ARGUMENTS

--------------------------------------------------------------------

DEFAULTS:  If the program is run without any command line arguments

it will default to COMMPORT 1 & MODEL 6.

The following command line arguments can be entered in any order:

BUTTON    - Name of Button to be pressed (see BUTTON NAMES).

/Annnn(n) - Base I/O Address applicable to User Defined CommPort

(Must be specified as a decimal number)

If /I argument is present, but /A is not; then Base

I/O Address will default to 1016 (3F8 Hex).

/Cn  - COMMPORT where n = 1,2,3 or 4

/F:[d]:\filespec - File Name containing Button Names of Buttons

to be pressed. (eg. /F:C:\IRDQ\TV.ON)

/In  - IRQ applicable to User Defined CommPort where n = 1 - 15

If /A argument is present, but /I is not; then IRQ

will default to 4.

/Mn  - MODEL where n = 5, 6, 6A, 6B, 8 or 12

/Q   - Quiet Mode; Display no messages during program execution

/?   - Display Command Line Arguments & Environment Variable syntax

NOTE: - COMMAND LINE OPTIONS OVERRIDE ANY SETTINGS STORED WITHIN

DOS ENVIRONMENT VARIABLE, IRDQ.

- MULTIPLE OCCURRENCES OF THE SAME COMMAND LINE ARGUMENT

ARE ACCEPTED; THE LAST INSTANCE WILL BE USED BY IRDQ.

5. IRDQ DOS ENVIRONMENT VARIABLE

--------------------------------------------------------------------

To minimize program size and improve performance, IRDQ does not

support an INI file of program parameters.  However, you can avoid

using command line arguments by setting up a DOS environment

variable that contains program settings.  The DOS environment

variable 'IRDQ' is used to pass settings to IRDQ.EXE.  You use the

DOS 'SET' command to assign a value to an environment variable.

The syntax is:

SET IRDQ=m,c,i,a,q

where: m = Model (5,6,6A,6B,8 or 12)

c = CommPort (1 thru 4)

i = IRQ (1 - 15)

a = Base I/O address (must be > 0)

q = Quiet Mode

You do not need to specify values for parameters that you do

not wish to change.  However, IRDQ environment variable parameters

are positional so you must use comma's as placeholders if you

wish to skip a parameter.  Trailing comma placeholders are not

required.  Examples are shown below:

To turn on Quiet Mode:      SET IRDQ=,,,,Q

To set Model to 12:         SET IRDQ=12

To set CommPort to 2:       SET IRDQ=,2

To set Model 12,CommPort 2

and turn Quiet Mode On:    SET IRDQ=12,2,,,Q

To set IRQ to 5:            SET IRDQ=,,5

To set Model 12,IRQ 5, &

Base I/O Address* 12832:    SET IRDQ=12,,5,12832,Q   *specify in

decimal

The DOS environment variable only needs to be SET once so a logical

place for it is within your AUTOEXEC.BAT file.

6. BUTTON NAMES

--------------------------------------------------------------------

The following Button Names can be passed directly on the Command

Line as an argument OR within the Button File passed to IRDQ with

the /F: command line argument.

+----------------------+-----------------------------------------+

|                      |    ONE FOR ALL MODELS SUPPORTED         |

|                      +------+------+------+------+------+------+

|   BUTTON NAMES       |  5   |  6   |  6A  |  6B  |  8   |  12  |

|   (& Synonyms)       | 2005 | 4000 | 4050 | 4005 | 6050 | 5000 |

+----------------------+------+------+------+------+------+------+

| "<"                  |      |  X   |      |      |      |      |

+----------------------+------+------+------+------+------+------+

| "<<",REW,REWIND      |  X   |  X   |  X   |  X   |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| ">>",FF              |  X   |  X   |  X   |  X   |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| 0                    |  X   |  X   |  X   |  X   |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| 1                    |  X   |  X   |  X   |  X   |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| 2                    |  X   |  X   |  X   |  X   |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| 3                    |  X   |  X   |  X   |  X   |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| 4                    |  X   |  X   |  X   |  X   |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| 5                    |  X   |  X   |  X   |  X   |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| 6                    |  X   |  X   |  X   |  X   |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| 7                    |  X   |  X   |  X   |  X   |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| 8                    |  X   |  X   |  X   |  X   |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| 9                    |  X   |  X   |  X   |  X   |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| A                    |  X   |  X   |  X   |  X   |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| A/B                  |      |      |      |      |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| AMP                  |      |      |      |      |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| AUDIO                |      |      |      |      |  X   |      |

+----------------------+------+------+------+------+------+------+

| AUX                  |  X   |      |      |      |      |  X   |

+----------------------+------+------+------+------+------+------+

| AUX1                 |      |  X   |  X   |  X   |  X   |      |

+----------------------+------+------+------+------+------+------+

| AUX2                 |      |  X   |  X   |  X   |  X   |      |

+----------------------+------+------+------+------+------+------+

| B                    |  X   |  X   |  X   |  X   |  X   |      |

+----------------------+------+------+------+------+------+------+

| B/AUDIO              |      |      |      |      |      |  X   |

+----------------------+------+------+------+------+------+------+

| C                    |  X   |  X   |  X   |  X   |  X   |      |

+----------------------+------+------+------+------+------+------+

| C/VIDEO              |      |      |      |      |      |  X   |

+----------------------+------+------+------+------+------+------+

| CABLE,CBL            |  X   |  X   |  X   |  X   |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| CD                   |  X   |  X   |  X   |  X   |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| CH+                  |  X   |  X   |  X   |  X   |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| CH-                  |  X   |  X   |  X   |  X   |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| DISPLAY,DISP         |  X   |      |  X   |  X   |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| ENTER,ENT            |  X   |  X   |  X   |  X   |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| F1                   |      |      |      |      |      |  X   |

+----------------------+------+------+------+------+------+------+

| F2                   |      |      |      |      |      |  X   |

+----------------------+------+------+------+------+------+------+

| F3                   |      |      |      |      |      |  X   |

+----------------------+------+------+------+------+------+------+

| F4                   |      |      |      |      |      |  X   |

+----------------------+------+------+------+------+------+------+

| MAGIC                |  X   |      |      |  X   |  X   |      |

+----------------------+------+------+------+------+------+------+

| MUTE                 |  X   |  X   |  X   |  X   |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| PAUSE                |  X   |  X   |  X   |  X   |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| PLAY                 |  X   |  X   |  X   |  X   |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| POWER,PWR            |  X   |  X   |  X   |  X   |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| PROGRAM,PGM          |  X   |      |      |  X   |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| RECALL,RCL           |  X   |  X   |  X   |  X   |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| RECORD,REC           |  X   |  X   |  X   |  X   |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| SAT                  |      |      |      |      |      |  X   |

+----------------------+------+------+------+------+------+------+

| SCAN                 |  X   |      |  X   |  X   |  X   |      |

+----------------------+------+------+------+------+------+------+

| SHIFT                |      |      |  X   |  X   |  X   |      |

+----------------------+------+------+------+------+------+------+

| SLEEP                |      |      |      |      |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| STOP                 |  X   |  X   |  X   |  X   |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| TUNER, TUN           |      |      |      |      |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| TV                   |  X   |  X   |  X   |  X   |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| TV/VCR               |      |  X   |      |      |      |      |

+----------------------+------+------+------+------+------+------+

| VCR                  |  X   |  X   |  X   |  X   |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| VIDEO                |      |      |      |      |  X   |      |

+----------------------+------+------+------+------+------+------+

| VOL+                 |  X   |  X   |  X   |  X   |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| VOL-                 |  X   |  X   |  X   |  X   |  X   |  X   |

+----------------------+------+------+------+------+------+------+

| OFFLINE              |  X   |      |  X   |  X   |  X   |      |

+----------------------+------+------+------+------+------+------+

| WAIT                 |  X   |  X   |  X   |  X   |  X   |  X   |

+----------------------+------+------+------+------+------+------+

NOTES: 1) Double Quotes required for <<,<, & >>

-----     You can use REW, FF rather than "<<" and ">>" respectively.

2) The WAIT button does not exist; it simply instructs IRDQ

to pause for � second.  This is useful when you want the

program to wait for the remote control to "catch up",

ie. when running a macro.  (When the remote is running a

macro, it will not act upon additional instructions

until it has completed the macro).  If you are having

problems with a button file then try inserting

additional WAIT commands between the button names.

2) The OFFLINE button does not exist; it instructs specific

ONE FOR ALL Models to go offline (and respond to physical

keypresses).

7. PROGRAM OVERVIEW

--------------------------------------------------------------------

When the program loads, it looks for the IRDQ variable in the DOS

Environment and then proceeds to parse it if found.  If any errors

are detected then the program will terminate with an appropriate

message, otherwise it will parse the command line arguments (which

override the IRDQ environment variable).  Once again, if any errors

are found it will terminate with an appropriate message.

If the /F: argument was included on the command line then IRDQ will

attempt to open the file and process the button names.  Otherwise,

IRDQ will look for a button name on the command line and execute

it.  Note: If both the /F: argument AND a button name are passed

on the command line, IRDQ will ignore the button name and process

the button file.

8. RELEASE HISTORY - IRDQ Commercial Software

--------------------------------------------------------------------

Date      Version     Changes

------   ----------   ----------------------------------------------

NOV'94      2.00      -Added support for 4 additional ONE FOR ALL

remote control models (URC2005,URC4005,

URC4050,URC6050)

-Added Quick Upgrade/Registration Support

-Limited Function version distributed with

Shareware Version of IRDC.

APR'94      1.01      -Corrected Error Message displayed when an

invalid Base I/O Address is detected.

OCT'93      1.00      -First Public Release

-OFA-12 Button Definition Problems Corrected

(Mode Buttons, A,B,C, Power, CH+,CH-)

-European Date Format Support Added

SEP'93      1.00�     Beta Test

------