�������ۻ �����ۻ  �ۻ �����ۻ  �����ۻ  ������ۻ

�������ͼ ������ۻ �ۺ ������ۻ ������ۻ ������ͼ

�ۺ    ������ɼ �ۺ ������ɼ ������ɼ ������ۻ

�ۺ    ������ۻ �ۺ ������ۻ ������ۻ ������ۺ

�ۺ    �ۺ  �ۺ �ۺ ������ɼ ������ɼ ������ۺ

�ͼ    �ͼ  �ͼ �ͼ �����ͼ  �����ͼ  ������ͼ

�������� ������� ������� ������� ��������

�� �� �� ��   �� ��   �� ������  �� �� ��

�� �� �� ��   �� ��   �� ������  �� �� ��

�� �� �� ������� ������� ������� �� �� ��

������� ��   �� ���� ������� �������

��  ��� ��   ��  ��  ��   �� ������

��   �� ��   ��  ��  ��   �� ������

������� ������� ���� ������� �������

Version 4.0  Revised 01/03/94

Brought to you by: WBS & Middle Earth Online

FREE for all sysops of TriBBS or other BBS software!  Please distribute

freely. Revisions will be made as deemed neccassary.

Welcome to the TriBBS init string documentation.

What is this file for?

Well it's to provide everyone who has problems finding the right init

string for their modem with some sample that might just work for them!

NOTE: These work for some people with TRiBBS. They may not work with

your modem. And they may not work with the BBS software!

You may still have to toy with them a little bit.

To update something in this file, add to it, or just to send me your

comments/suggestions please leave a mail to:

Karl Duncan on WME- TriBBS Support Conf., Sysop Issues

TriBBSNet- (any conference)

or call my BBS: Middle Earth Online at 610-921-8836.

Brand New!!!  This file is now in HyperText format!  You don't have to

page through all the modems anymore, select the one you want and you are

there!

__________________________________________________________

2400 baud modem inits           AT&T Paradyne

Boca Modems                     Cardinal Modems

DataTrek Modems                 DallasFax Modems

DigiCom Modems                  Hayes Modems

Intel Modems                    Lightning Communications Modems

MicrComm Mdems                  MultiTech Modems

Patriot Modems                  Practical Peripherals

Supra Modems                    TwinCom Modems

US Robotics                     Wang Modems

Zoom Modems                     Zyxel Modems

Description of Communication Standards

Closing Notes

Reveal Modems

2400 baud modem owners:

You should use the default TriBBS init string as it should work

with all 2400 baud modems.

Standard 2400 bps or 1200 bps Hayes Compatibles (NON-MNP):

If your modem supports the extended "&" command set, then try:

ATX4Q0E0V1&C1&D2S2=251S7=40

X4     Enables extended result codes

Q0     Modem returns Result Codes

E0     Modem does not echo commands back to the computer

V1     Modem returns Verbal Result Codes

&C1    Modem asserts Carrier Detect based on connection status

&D2    On DTR high-to-low transition, Modem Hangs Up.

S2=251 Disable Modem Escape Sequence

S7=40  Time Modem waits for a carrier when initiating a new connection

If your modem does not support the & command set, try using:

ATX4Q0E0V1S2=251S7=40

-------------------

AT&T Paradyne 3820 and 3830

AT H0 Z0

NVRAM

M0      turn off the speaker

X6      extended result codes

&R0     RS-232 control

&S1             "

\D1             "

\Q3             "

%C0     Disable MNP5

"H3     Disable V.42

-------------------

Boca Internal 14.4 V32bis

AT&K3&C1\N3E0H0M0Q0V1X4S2=251S7=60

Boca Research M14401 V.32bis 14.4kb

AT&F S0=0 Q0 E0 M0 V1 H0 &C1 &D2 S95=3 W2 ^M

Boca 14.4k Internal v.32bis

ATZ^M

NVRAM Settings

AT&F^M

ATB1E1F1H0L2M0N1Q0V1W2X4^M

AT&C1&D2&G0&J0&K3&L0&M0^M

AT&P0&Q9&R0&X0S0=0S2=43^M

ATS6=2S7=50S8=2S9=6S10=14^M

ATS11=55S26=1S37=11S48=7^M

ATS82=128\G0\L0\N3%D3%C1^M

AT%M3%S64S95=3^M

AT&W0^M

ATZ^M

-------------------

Cardinal 14.4

ATS0=0Q0E0M0V1H0S2=43X1W2S95=3^M

Cardinal 9600:

(No NRAM settings submitted)

ATW2Q0V1E0&D2&C1S2=251M0&W2S95=1235

- or -

ATQ0V1E0&D2&C1S2=251S7=45M0

-------------------

DataTrek 9600 V.32 (external)

AT S0=0 Q0 E0 MO V1 H0 S2=43 X1 ^M

-------------------

Dallas FAX  Nova Plus 14.4

AT&F &C1 &D2 &S1 \J0 "O125 T E0 S11=50 S7=60 S95=3 S2=251

-------------------

Digicom Scout (9600 V.32)

AT &F *E9 *G0 S0=0 S9=6.

-------------------

Hayes Optima 144 + Fax 144

AT H0 &F &Q9 L3 W2 &D2 ^M

NRAM settings for the Hayes Optima 28.8 V.FC+Fax modem:

ACTIVE PROFILE: B1 B16 B76 E1 L2 M0 N1 P Q0 V1 W2 X4 Y0 &A0 &C1 &D3 &G0 &K3 &Q9

&R0 &S0 &T4 &U0 &X0 &Y0 S00:000 S02:043 S03:013 S04:010 S05:008 S06:002 S07:050

S08:002 S09:006 S10:014 S11:095 S12:050 S25:005 S26:001 S36:007 S37:000 S38:020

S46:002 S48:007 S49:064 S50:192 S108:003 S109:4094 S110:002

STORED PROFILE 0:   B1 B16 B76 E1 L2 M0 N1 P Q0 V1 W2 X4 Y0 &A0 &C1 &D3 &G0 &K3

&Q9 &R0 &S0 &T4 &U0 &X0 S00:000 S02:043 S06:002 S07:050 S08:002 S09:006 S10:014

S11:095 S12:050 S25:005 S26:001 S36:007 S37:000 S38:020 S46:002 S48:007 S49:064

S50:192 S108:003 S109:4094 S110:002

STORED PROFILE 1:   B1 B16 B76 E1 L2 M1 N1 P Q0 V1 W0 X4 Y0 &A0 &C1 &D2 &G0 &K3

&Q5 &R0 &S0 &T4 &U0 &X0 S00:000 S02:043 S06:002 S07:050 S08:002 S09:006 S10:014

S11:095 S12:050 S25:005 S26:001 S36:007 S37:000 S38:020 S46:002 S48:007 S49:064

S50:192 S108:001 S109:4094 S110:002

&Z0=

&Z1=

&Z2=

&Z3=

Here is the initialization string I use for TriBBS:

ATS0=0Q0E0M0V1H0S2=43X1^M

I use a simple ATZ^M with communications programs.

Hayes Accura 14.4

AT&F,E0M0H0X4S37=11S95=3^M

(Note this string should work for most any Hayes 14.4 modem!)

-------------------

Intel 14.4e fax/modem

init

ATZS0=0E0M0H0^M

NRAM settings

ACTIVE PROFILE:

B1 E1 L1 M1 Q0 T V1 X4 Y0 &C1 &D2 &G0 &P0 &S0 &Y0

\A3 %A013 \C0 %C1 %E1 \G0 \J1 \K5 \N3 \Q0 \T000 \V1 \X0 -J1 "H3

S00:000 S01:000 S02:043 S03:013 S04:010 S05:008 S06:002 S07:030

S08:002 S09:006 S10:014 S11:070 S12:050 S18:255 S23:16H S25:005 S26:000

Intel 14.4ex Fax/modem

init

ATzS0=0E0M0H0^M

NRAM settings

B1 E1 L2 M0 T Q0 V1 X4 Y0 &C1 &D2 &G0 &L0 &P0 &Q0 &R0 &S0 &X0 &Y0

\A3 %A013 \C0 %C0 %E1 \G0 \J0 \K5 \N3 \Q3 \T00 \V2 \X0 -J1 "H3 "S1 "O250

S00:000 S01:000 S02:043 S03:013 S04:010 S05:008 S06:002 S07:060

S08:002 S09:006 S10:014 S11:070 S12:050 S18:000 S25:005 S26:001 S31:000

Intel 144i

AT&F\N0\Q0\J1\V0^M

Intel & OS/2

Intel 400i under OS/2

AT S0=99 E0 H0 M0 L0 ^M

Also, you might read the following.... although you might not be

running OS/2, there are some considerations you need to make to get

the Intel to run completely.

Finally, there is one configuration item that TriBBS needs to have

added if you're trying to run both a BBS and a FAX thru the Intel

(Mark Goodwin has graciously agreed to make this change in the next

version of TriBBS):  TriBBS needs to be configured to *NOT* issue the

standard    ATH   when it sees the    RING    response from the data

section of the modem.  The Intel firmware takes care of answering the

phone; if TriBBS sends the   ATH    while the Intel is trying to

discriminate between an incoming data call and an incoming FAX call,

the ATH will cause the Intel firmware to not make that determination

correctly.

Would you please add a note in TRINITS that this modem requires

TriBBS to be set as follows:

Node's Modem Answer Command:

Was:    ATA^M

Change To:               <nothing, null, delete entirely!>

For those reading this, this change now makes it possible to setup a

TriBBS node with an Intel modem which can not only answer the phone

for "data" calls to the BBS, but it can now also answer FAX calls on

that same phone line !!!!!   :)

Works great! here ... am running FaxWorks and TriBBS at the same time

on the same phone line, under OS/2.  The only call arbitration is

done by the Intel modem.

==> Bruce           [Team OS/2]

========================= stored settings ============================

STORED PROFILE 0:

B1 E1 L0 M0 T Q0 V1 X4 Y0 &C1 &D2 &G0 &L0 &P0 &Q0 &R0 &S0 &X0

\A3 %A013 \C0 %C1 %E1 \G0 \J0 \K5 \N3 \Q3 \T20 \V2 \X0 -J1 "H3 "S0

S00:099 S07:030 S11:070 S18:000 S23:030 S25:005 S26:001 S27:064 S31:000

======================================================================

-------------------

Lightning Communications:

AT&FE0&D2&C1&Q9S7=65W2M0

-------------------

MicroComm 28.8 modems

From your terminal program TYPE.

AT&F <ENTER>

AT H0 M0 N1 Q0 T V1 W2 X4 Y0 <ENTER>

AT &C1 &D2 &G0 &K3 &Q5 &S1 <ENTER>

AT S7=60 S11=50 S24=255 S95=3 <ENTER>

AT&W0 <ENTER>

The AT&W0 saves it to the NRAM of the modem, then in TriBBS my init

string is ATZ0^M

-------------------

MULTITech MT932BA 9600 BAUD FAX/MODEM

USES TRIBBS DEFAULT INIT WITH NO PROBLEMS

-------------------

Patriot 96i5 v.32/v.42 MNP5 9600 baud modem --

Profile - Use default, factory profile (&F)

Init String:  ATE0M0X4&D2\N7Q2\V2%A32%C3S2=251S7=30

-------------------

Practical Peripherals PM14400FXSA v.32bis

AT S0=0 Q0 E0 M0 V1 H0 X4 &D2 W2 &Q9 S95=47 ^M

PPI FXMT 14.4 Fax Modem.

ACTIVE PROFILE:            DTE:57600      DCE:14400     PARITY:NONE

B16 B1 E1 L1 M1 N1 T Q0 V1 W2 X4 Y0 &A0 &B1 &C1 &D2 &G0 &J0 &K3 &L0 &Q5

&R0 &S0

&T4 &U0 &X0 &Y0

S00:000 S01:000 S02:043 S03:013 S04:010 S05:008 S06:002 S07:055 S08:002

S09:006

S10:014 S11:065 S12:050 S18:000 S25:005 S26:001 S30:000 S36:007 S37:000

S38:020

S43:011 S46:002 S48:007 S49:064 S50:220 S63:000 S82:128 S86:000 S95:047

S97:030

S108:001 S109:062 S110:002

NVRAM settings for PM14400FXMT

at&v

ACTIVE PROFILE:            DTE:19200      DCE:9600      PARITY:NONE

B16 B1 E1 L2 M0 N1 T Q0 V1 W2 X4 Y0 &A0 &B1 &C1 &D2 &G0 &J0 &K3 &L0 &Q5 &R0 &S0

&T4 &U0 &X0 &Y0

S00:000 S01:000 S02:043 S03:013 S04:010 S05:008 S06:002 S07:050 S08:002 S09:006

S10:014 S11:070 S12:050 S18:000 S25:005 S26:001 S30:000 S36:007 S37:000 S38:020

S43:009 S46:002 S48:007 S49:064 S50:220 S63:000 S82:128 S86:000 S95:047 S97:030

S108:001 S109:062 S110:002

-------------------

Reveal 14.4 Modems

Try this one:  AT &F2 S0-0 S9=6 S10=11 S11=60 &G0

-------------------

SupraFAX V.32bis (internal/external)

AT &F2 M0 W2 S95=3 &K3 X4^M

SupraFAX V.32bis External 14.4

AT&F2 S0=0Q0E0M0V1H0S2=43X1W2S95=3&K3^M

Supra 14.4 internal fax modem, v.32bis/v32

ATS0=0&F2E0H0M0X1W2S95=3^M

Supra/Fax 14.4:

NRAM settings can be run as default factory....

AT&F2  and then AT&W to write the set.

AT&K3\N3\T5E0H0M0Q0V1X4S0=0S2=1W2S95=3

-------------------

TwinCom V.32:

(No NRAM settings submitted)

ATE0H0M0Q0V1X4S0=0S2=1W0S95=145

-------------------

ati5

USRobotics Courier 16800 HST Dual Standard NVRAM Settings...

DIAL=TONE   B0  F1  M3  X7

BAUD=57600  PARITY=N  WORDLEN=8

&A3  &B1  &G0  &H1  &I0  &K3  &L0  &M4

&N0  &P0  &R2  &S1  &T5  &X0  &Y1  %R0

S02=043  S03=013  S04=010  S05=008  S06=002  S07=060

S08=001  S09=006  S10=007  S11=055  S12=050  S13=000

S15=000  S19=000  S21=010  S22=017  S23=019  S24=150

S25=005  S26=001  S27=128  S28=015  S29=020  S32=006

S33=000  S34=000  S35=000  S36=000  S37=000  S38=000

S41=255  S42=255  S43=255  S44=255

STORED PHONE #0:

#1:

#2:

#3:

OK

All the settings are stored in the NVRAM and are loaded when the ATZ^M is sent.

Should you not use a front end, then use ATZ^M as the init string for TriBBS.

Note that this setup forces the v.32bis to answer first, then the HST mode, so

if you dial out to a HST modem and want an HST connection, you must use  ATB1D

as a dial string.  Otherwise you'll get a 2400 connect (which you probably

don't want.  Hope this is what you wanted.

ATI4

USRobotics Courier 16800 HST Dual Standard Fax Settings...

NVRAM settings

B0  C1  E1  F1  M0  Q0  V1  X7

BAUD=38400  PARITY=N  WORDLEN=8

DIAL=HUNT   ON HOOK   TIMER

&A3  &B1  &C1  &D2  &G0  &H1  &I0  &K1  &L0

&M4  &N0  &P0  &R2  &S1  &T5  &X0  &Y1  %R0

S00=000  S01=000  S02=043  S03=013  S04=010  S05=008

S06=002  S07=060  S08=002  S09=006  S10=007  S11=055

S12=050  S13=000  S14=001  S15=000  S16=000  S17=000

S18=000  S19=005  S20=000  S21=010  S22=017  S23=019

S24=150  S25=000  S26=001  S27=000  S28=008  S29=020

S30=000  S31=000  S32=001  S33=000  S34=000  S35=000

S36=000  S37=000  S38=000  S39=000  S40=000  S41=000

S42=126  S43=200  S44=015

USRobotics Sportster V.32bis 14.4/PC FAX

AT&F S0=0 &A3 &B1 &H1 &R2 X4 E0 M0 H0 ^M

USR Sportster V.32bis (external)

AT &F M0 &B1 &H1 &R2 &S1 &C1 &D2 &T4 X4^M

ATI4

USRobotics Sportster 14400 Fax Settings...

init string

ATZ

NVRAM setings

B0  E1  F1  L2  M1  Q0  V1  X4

BAUD=19200  PARITY=N  WORDLEN=8

DIAL=HUNT   ON HOOK

&A1  &B1  &C1  &D2  &G0  &H1  &I0  &K1

&M4  &N0  &P0  &R2  &S0  &T5  &Y1

S00=000  S01=000  S02=043  S03=013  S04=010  S05=008  S06=002

S07=060  S08=002  S09=006  S10=015  S11=070  S12=050  S13=000

S14=000  S15=008  S16=000  S17=000  S18=000  S19=000  S20=000

S21=010  S22=017  S23=019  S24=000  S25=050  S26=000  S27=000

S28=008  S29=020  S30=000  S31=000  S32=000  S33=000  S34=006

S35=000  S36=000  S37=000  S38=000  S39=000  S40=000  S41=000

S42=000  S43=000  S44=015  S45=000  S46=000  S47=000  S48=000

S49=000  S50=000  S51=000

ATI5

USRobotics Sportster 14400 Fax

Init string

ATZ

NVRAM Settings...

DIAL=TONE   B0  F1  L2  M0  X4

BAUD=19200  PARITY=N  WORDLEN=8

&A1  &B1  &G0  &H1  &I0  &K1  &M4

&N0  &P0  &R2  &S0  &T5  &Y1

S02=043  S03=013  S04=010  S05=008  S06=002  S07=060  S08=002

S09=006  S10=015  S11=070  S12=050  S13=000  S14=000  S15=008

S19=000  S21=010  S22=017  S23=019  S24=000  S25=050  S26=000

S27=000  S28=008  S29=020  S31=000  S32=000  S33=000  S34=006

S35=000  S36=000  S37=000  S38=000  S41=000  S42=000  S43=000

S44=015  S51=000

STORED PHONE #0:

#1:

#2:

#3:

USRobotics Sportster 14.4 FAX modem

init:

AT & F S0=0 &A3 &B1 &H1 &R2 X4 E0 M0 H0 ^M

USRobotics Courier HST/PC Dual Standard V.32 V.42 V.42bis Internal.

ATS0=0H0M0^M

US Robotics Courier HST / Courier Dual Standard:

NRAM Settings Recommended:

X7     Return full result codes

&H1    Turns on hardware handshaking for output from DTE (CTS line) *

&R2    Turns on hardware handshaking for input to DTE    (RTS line) *

&B1    Fixes the DTE to DCE rate **

&N0    Negotiates highest link rate with fallback/fallforward enabled

&M4    Enables ARQ if remote modem has ARQ

&C1    Modem asserts carrier detect based on connection status

&D2    On DTR high-to-low transition, modem hangs up

&A3    Give FULL EXTENDED Result Codes

(choose one of these 3 &K commands based on your preference)

&K1    Enables data compression if remote modem requests compression

&K3    Enables V.42 bis data compression but not MNP5

&K0    Disables all compression

ATI4

USRobotics Courier 14400 HST Dual Standard Settings...

B0  C1  E1  F1  M1  Q0  V1  X7

BAUD=38400  PARITY=N  WORDLEN=8

DIAL=HUNT   ON HOOK   TIMER

&A3  &B1  &C1  &D2  &G0  &H1  &I0  &K3  &L0

&M4  &N0  &P0  &R2  &S0  &T5  &X0  &Y1  %R0

S00=000  S01=000  S02=251  S03=013  S04=010

S05=008  S06=002  S07=045  S08=002  S09=006

S10=007  S11=050  S12=050  S13=000  S14=000

S15=008  S16=000  S17=000  S18=000  S19=000

S20=000  S21=010  S22=017  S23=019  S24=150

S25=000  S26=000  S27=000  S28=008  S29=020

S30=000  S31=000  S32=001  S33=000  S34=000

S35=000  S36=000  S37=000  S38=000

- Or use this -

ATI4

USRobotics Courier 14400 HST Dual Standard Settings...

B0  C1  E1  F1  M1  Q0  V1  X7

BAUD=19200  PARITY=N  WORDLEN=8

DIAL=HUNT   ON HOOK   TIMER

&A3  &B0  &C1  &D2  &G0  &H1  &I0  &K3  &L0

&M4  &N0  &P0  &R2  &S0  &T5  &X0  &Y1  %R0

S00=000  S01=000  S02=251  S03=013  S04=010

S05=008  S06=002  S07=045  S08=002  S09=006

S10=007  S11=070  S12=050  S13=000  S14=001

S15=008  S16=000  S17=000  S18=000  S19=000

S20=000  S21=010  S22=017  S23=019  S24=150

S25=000  S26=000  S27=000  S28=008  S29=020

S30=000  S31=000  S32=001  S33=000  S34=000

S35=000  S36=000  S37=000  S38=000

Init Strings:

ATX7Q0E0V1S2=251S7=25&H1&R2&B1&M4&N0&K1&A3

- or, at least -

ATX7Q0E0V1S2=251S7=25&H1&R2&B1&A3

--------------------------------------------------------------------------

USR 9600 HST:

Use the init string above, stated for the USR/DS.

USRobotics Courier 9600 HST Settings....

C=1  E=1  F=1  M=3  Q=0  V=1  X=7  B=1

BAUD=19200  PARITY=N  WORDLEN=8

DIAL=TONE   ON HOOK    TIMER

&A1  &B1  &G0  &H1  &I2  &K0

&M4  &N0  &P0  &R2  &S1  &Y1

S00=000  S01=000  S02=251  S03=013

S04=010  S05=008  S06=002  S07=025

S08=002  S09=006  S10=007  S11=040

S12=050  S13=000  S14=001  S15=000

S16=000  S17=000  S18=000  S19=005

S20=000  S21=010  S22=017  S23=019

NVRAM Settings

USRobotics Courier 9600 HST NRAM Settings....

DIAL=TONE   M=3  X=7  F=1  B=1

BAUD=19200  PARITY=N  WORDLEN=8

&A1  &B1  &G0  &H1  &I2  &K0

&M4  &N0  &P0  &R2  &S1  &Y1

S02=251  S03=013  S04=010  S05=008

S06=002  S07=025  S08=002  S09=006

S10=007  S11=040  S12=050  S13=000

S15=000  S19=005  S21=010  S22=017

S23=019

-------------------

WANG 14/14i

ATE1Q0V1M1 W2 S95=3 S7=50 S11=55 S0=0^M

-------------------

ZOOM 14.4 s and 1 ZOOM V.32 12000Data Pum, all use the same inits.

ATZ~^MATS0=0HOM0&C1&D2%E0W2S95=3^M

Zoom/Fax 14.4  (also works for Zoom/Fax 9600)

ATS0=0Q0E0M0V1H0&C1S2=43S36=7S95=3X1^M

AT&FH0L1E0S95=3M0&C1&D2^M

AT&F~S0=0Q0E0H0M0V1&C1&D2S95=3W2^M

ZOOM FaxModem VFP V.32bis 14.4kb

AT&F S0=0 Q0 E0 M0 V1 H0 &C1 &D2 S95=3 W2 ^M

Zoom FaxModem VFX v.32bis

ATS0=0S2=43S95=3E0H0M0Q0V1W2X1&C1&K3^M

Zoom 14.4 v32bis Fax Modem..

ATS0=0Q0E0M0V1H0&C1S2=43S36=7S95=3X1W2^M

Zoom v.32bis VFP internal

NVRAM

STORED PROFILE 0:

B1 E1 L0 M0 N1 Q0 T V1 W0 X4 Y0 &C1 &D2 &G0 &J0 &K3 &Q5 &R0 &S0 &T4

&X0 S00:000 S02:043 S06:002 S07:050 S08:002 S09:006 S10:014 S11:095

S12:050 S18:000 S25:005 S26:001 S36:007 S37:000 S38:020 S44:003

S46:138 S48:007 S49:008 S50:255

NRAM settings for the Zoom 28.8 VFP modem+Fax:

ACTIVE PROFILE:

B1 E1 L1 M1 N1 Q0 T V1 W0 X4 Y0 &C0 &D0 &G0 &J0 &K3 &Q5 &R1 &S0 &T4 &X0 &Y0

S00:000 S01:000 S02:043 S03:013 S04:010 S05:008 S06:002 S07:050 S08:002 S09:006

S10:014 S11:095 S12:050 S18:000 S25:005 S26:001 S36:007 S37:000 S38:020 S44:020

S46:138 S48:007 S95:000

STORED PROFILE 0:

B1 E1 L1 M1 N1 Q0 T V1 W0 X4 Y0 &C0 &D0 &G0 &J0 &K3 &Q5 &R1 &S0 &T4 &X0

S00:000 S02:043 S06:002 S07:050 S08:002 S09:006 S10:014 S11:095 S12:050 S18:000

S36:007 S37:000 S40:105 S41:131 S46:138 S95:000

STORED PROFILE 1:

B1 E1 L1 M1 N1 Q0 T V1 W0 X4 Y0 &C0 &D0 &G0 &J0 &K3 &Q5 &R1 &S0 &T4 &X0

S00:000 S02:043 S06:002 S07:050 S08:002 S09:006 S10:014 S11:095 S12:050 S18:000

S36:007 S37:000 S40:105 S41:131 S46:138 S95:000

TELEPHONE NUMBERS:

0=                                     1=

2=                                     3=

Here is the initialization string I use for TriBBS:

AT&F~~~~ATW2S95=3S0=0Q0E0M0V1H0S2=43X4^M

-------------------

ZyXel High Speed:

this is saved as Profile 0.  Init string is just ATZ0.

Be aware that register S20 sets the DTE speed, set to 19200, so you

may want to change this to S20=2 or S20=1.

Current   Setting.............

B0   E0   L4   M0   N5   Q0   V1   X6

&B1  &C1  &D2  &G0  &H3  &J0  &K4  &L0  &M0  &N0  &P0  &R1  &S0  &X0  &Y0

*B0  *C0  *D0  *E0  *I0  *M0  *P9  *Q2  *S0

S00=000   S01=000   S02=251   S03=013   S04=010

S05=008   S06=003   S07=060   S08=002   S09=006

S10=007   S11=070   S12=000   S13=000   S14=002

S15=002   S16=000   S17=018   S18=000   S19=000

S20=003   S21=176   S22=000   S23=112   S24=133

S25=000   S26=000   S27=156   S28=064   S29=000

S30=000   S31=017   S32=019   S33=000   S34=030

S35=000   S36=000   S37=000   S38=016   S39=000

ZyXEL 14400 V.32 V.32bis V.42 V.42bis Send/Recieve Fax/Modem:

AT&FX6E0&C1&D2&G2H0M0|

-------------------

---------------------------------------------------------------------------

DESCRIPTION OF COMMUNICATION STANDARDS.  INCLUDES MODEM & FAX.

---------------------------------------------------------------------------

STANDARD NAME    DESCRIPTION

---------------------------------------------------------------------------

CCITT v.21       Standard for 300 bps.  (FSK).

Bell  103J

Bell  202        Old standard for 1200 bps, half duplex. Leased lines.

CCITT v.22       Standard for 1200 bps. (DPSK).

Bell  212A

CCITT v.22bis    Standard for 2400 bps. (DPSK).

CCITT v.26ter    Standard for 2400 bps with echo cancellation.

Bell  208        Standard for 4800 bps. Used on leased lines.

CCITT v.27ter    Standard for G3 FAX at 4800 bps with fall back to 2400 bps.

CCITT v.29       Standard for G3 FAX at 9600 bps with fall back to 7200 and

4800 bps.  Also v.27ter/v.29 is used by CompuCom modems.

Half duplex.  Reverse direction bps (slow?).

CCITT v.32       Standard for 9600 bps with echo cancellation.  Fall back

to 4800 bps.  Both direction simultaneously.  (QAM).

CCITT v.32bis    Standard for 14400 bps with echo cancellation.  Fall back

to 12000, 9600, and 7200 bps.  (TCM).  Both directions

simultaneously.

CCITT v.33       14400 with fall back to 12000 bps.  Used for what?

USR HST14400     US Robotics HST modulation.  14400 bps in one direction

and 450 bps in reverse direction.  Fall back to 12000,

9600, and 4800 bps.

Trailblazer      Proprietary and fast!  Up to 18000 bps.  ???

Hays V-Series    Proprietary full-duplex 9600 baud.  Hays its own CCITT LAP-B

error correction and its own scheme for data compression.

EIA Class2 FAX   Fax standard called G2.  ???

CCITT v.FAST     Standard for 19200 bps or slightly higher.  Uses echo

CCITT v.32bisbis canellation and is not FINALIZED.  The first modems will be

available in late 1993.  The name of the standard has not

been determined yet.  It will have a much greater capability

to adapt modem speeds to meet existing conditions (more than

any existing modem).  Theory sets a limit in the low 20000

bps range.  This will probably be the LAST analog modem

standard.

ISDN             Integrated Services Digital Network.  This uses fiber optic

cable.  It is currently available in larger cities within

3 miles (has been lengthen) of the central office. Available

in two flavors: Primary Rate Interface service and Basic

Rate Interface service.  Basic, also known as 2B+D provides

users with two 64Kbit/sec channels for voice and data and

one 16Kbit/sec channel used by the carrier for call setup

and network management.  The primary, also known as 23B+D

provides 23 64Kbit/sec channels and one 16Kbit/sec channel.

Some recent west coast subscribers will be paying a one

time $75 fee for a network connection device and $97 per

month for service, which includes two voice lines.

SMDS             Switched Multi-Megabit Data Service.  Uses fiber optics.

???

HDSL             High-rate Digital Subscriber Link.  Bell Labs and Bellcore

have recently developed this technology.  It is capable of

carrying 1.5 megabits/sec over copper phone lines!  Limited

to about 3 miles from the central office.  Benificial since

it provides high speed data rate without running new fiber

optic cable to the customer.  This is still in the labs.

---------------------------------------------------------------------------

MODE          BPS       BAUD      MODULATION      NOTES

---------------------------------------------------------------------------

v.33         14400      2400       128-TCM

v.33         12000      2400        64-TCM

v.32bisbis   19200      2400       512-TCM        TBD ???

v.32bis      14400      2400       128-TCM

v.32bis      12000      2400        64-TCM

v.32bis       9600      2400        32-TCM

v.32bis       7200      2400        16-TCM

v.32          9600      2400        32-TCM

v.32          9600      2400        16-QAM        Uncoded

v.32          4800      2400         4-DPSK

v.29          9600      2400        16-QAM

v.29          7200      2400         8-QAM

v.29          4800      2400         4-DPSK

v.27ter       4800      1600         8-PSK

v.27ter       2400      1200         4-DPSK

G3 FAX         ->        ->           ->          According to v.29/v.27ter

v.22bis       2400       600        16-QAM

v.22          1200       600         4-DPSK       Same as BELL 212A

ISDN         65536     65536       FiberOptic     Digital

The baud rate over the phone line means that more than two tones are used at

the baud frequency to achieve the bps rate.  4-DPSK means four tones...

---------------------------------------------------------------------------

NAME      DESCRIPTION OF ERROR CORRECTION AND DATA COMPRESSION

---------------------------------------------------------------------------

MNP1      Hardware error correction.

MNP2      Hardware error correction.

MNP3      Hardware error correction.  Strips off start and stop bits during

transmission over phone line.

MNP4      Hardware error correction.  Fastest way to send compressed files.

Strips off start and stop bits during transmission.  Also adapts

to line conditions.

MNP5      Hardware error correction and data compression (up to 2 time

compression on TEXT).  Do NOT use on compressed files!

MNP6      Additional Universal Link Negotiation and Statistical Duplexing

to MNP5 service

MNP7      Additional Enhanced Data Compression with MNP4.

MNP8      Nothing available ???

MNP9      Additional Enhanced Data Compression with v.32 modems  Also adds

Enhanced Universal Link Negotiation, which allows connection to

both MNP and non-MNP modems at the highest performance level.

MNP10     Hardware error correction and data compression.  Special!

v.42      CCITT error correction. LAPM.  Fastest way to send compressed

files.  Strips off start and stop bits during transmission.

v.42bis   CCITT data compression (up to 4 time compression on TEXT).

Uses a version of the Lempel-Ziv data compression algorithm.

---------------------------------------------------------------------------

baud  means Actual rate of symbols trasmitted per second.

bps   means Bits Per Second.

MNP   means Microcom Networking Protocol.

CCITT means International Telephone and Telegraph Consultative Committee.

FSK   means Frequency Shift Keying.

DPSK  means Phase Shift Keying.

QAM   means Quadrature Amplitude Modulation.  Combination of PSK and AM.

TCM   means Trellis Coded Modulation.

LAPM  means Link Access Procedure for Modems.

--------------------------------------------------------------------------

This file is FREE to all sysops.

I hope it helps you trouble shoot before you have to ask.

It is to help save time and get you running as fast as

possible in the very limited amount of time.

Special thanks to the following:

John Betmanis           Michael Disabato        Jim Ryan

Rudy Hilsmann           Brad Reddekopp          Klaas Tuinman

Dean D'agostino         Richard Yates           Larry Kilgore

James Marshall          Joe Klemmer             Koby Fuessler

Ben Askew               Leo Pereira             Paul Hirsch

Don Brockman            Mike Jones              Rick Young

Jason Malvuccio         Hal Holland             Mark Goodwin