EPCOM Version 1

__________

Copyright 01/26/95

Eagle Professional Software

P.O. Box 876

Marion, Illinois 62959

Email: reid.smith@poohs.com

BBS:   618-997-8009

FAX:   618-997-8009

---- DESCRIPTION  ----------------------------------------------------------

I searched for years trying to find a Async unit for Trubo Pascal

that worked like it was supposed to and had fossil calls. I never

found one so I realized that I'd have to write my own to get what I

needed. Some of the units worked but still had problems. The problems

were caused by many different things such as Fossil drivers loaded or

just unloaded, Qemm installed or Desqview. Some wouldn't even reconize

the modem with the program installed. Some had probems when the fossil

driver was uninstalled and some worked but had random lock ups.

After finding the problems I searched for one Unit that worked

the best then built EPCOM on that unit. I want to thank the writers

of the SWAG librarys and the people listed below for some of there

PD sorces that I used for examples that makes up EPCOM.

Basic Async unit By Joel Bergen and Gary Gordon

Fossil Wizard Unit v1.0 by Steve Connet

Ansi screen emulation routines By Scott Baker and Derrick Parkhurst

EPCOM will handle all of your Terminal or BBS Async programing

needs. If a fossil driver is loaded it's autodetected and used. There

are many other features that should be added to the fossil side of the

unit. If no fossil is loaded the library uses it's own internal Async

features. These features could use some more work and additions but

are for the most part functional. If you wish to see future additions

REGISTER !!!!!

Twenty dollars is a very very small price to pay for all the

features in EPCOM and no way pays for the hours spent on writing and

debuging. If you register EPCOM I'll keep adding functions and

procedures until it's the most complete package you'll ever need.

There is a small Terminal program added (TERM.EXE - .PAS) as an

example of how to use EPCOM. It's basic and simple and gets the

job done.

---- FEATURES  ------------------------------------------------------

#  Handles up to 4 ports simultaneously.

#  Interrupt driven serial Reads. Interrupt Writes might be

added later.

#  Created as a TP Unit.

#  Supports both RTS/CTS and XON/XOFF handshaking,

#  Includes all Turbo Pascal code.

===========================================================================

PROCEDURE  ComInit(portnum:byte;baud:longint;parms:byte);

FUNCTION   ComOpen : boolean;

These two features INIT the serial port depending on what the

portnum. The Baud if your not using a fossil driver should

be set to just above the modem's max speed. If you have a 14.4

modem then set the speed to 19200, 28.8 to 38400. If your using a

fossil driver then the speed can be set at any rate. Look at TERM.PAS

===========================================================================

PROCEDURE  ComClose;

This procedure MUST be called when exiting your term. program.

If not called it will leave interrupt vectors hooked and a possable

system lock up. Look at TERM.PAS

===========================================================================

FUNCTION   ComRead : CHAR;

FUNCTION   ComBuffer_Check : BOOLEAN;

Simple read from the serial port. You must check the ComBuffer first

to see if there are characers available. Look at TERM.PAS

===========================================================================

PROCEDURE  FlushRead;

PROCEDURE  FlushWrite;                        {just fossil}

Flush the interrupt driven read buffer. If there are characters

available then they will be lost.

===========================================================================

PROCEDURE  ComWrite(instring : string);

PROCEDURE  ComWriteLN(instring : string);

Works much the same as Write and Writeln but the information is

directed to the serial port. ComWrite will handle either one character

or a string. This procedure also contains the raiseDTR procedure

because the DTR line must be high to send information to the modem.

===========================================================================

FUNCTION   InitModem(instring : string) : Boolean;

This function will take an INIT string and send it to the modem.

after the string is sent the modem should report OK. If the modem

accepts the string and OK was returned then it will return true.

If your using numeric codes then the 'OK' should be changed to

the numeric responce. Look at TERM.PAS

===========================================================================

FUNCTION   AnswerModem(AnswerString : string) : string;

This function will answer the modem and return the CONNECT string.

This is what most BBS's use to find the connect rate. If your using

numeric codes then the 'OK' should be changed to the numeric responce.

===========================================================================

PROCEDURE  RaiseDTR;

PROCEDURE  LowerDTR;

Raise and Lower Data Transmit Ready register.

===========================================================================

FUNCTION   CarrierDetect : BOOLEAN;

Returns true if there is a carrier present.

===========================================================================

FUNCTION   RingDetect : BOOLEAN;

Returns true if the phone is ringing.

===========================================================================

PROCEDURE  WAIT(secs : Word);

Much Like the TPascal Delay command but uses seconds instead of

milliseconds.

===========================================================================

FUNCTION   DetectLocalAnsi : Boolean;

This function doesn't work with DVANSI  but will work with ANSI.SYS.

It returns true if ANSI.SYS is loaded.

===========================================================================

PROCEDURE  AnsiWrite(ch: char);

This procedure will write ansi characters to the screen as they

come in over the serial port even if ANSI.SYS isn't loaded.

===========================================================================

Global Vars.....

CONST

N81=$03;   E81 =$1b;   O81 =$0b;  IER = 1;  LSR = 5;

N82=$07;   E82 =$1f;   O82 =$0f;  IIR = 2;  MSR = 6;

N71=$02;   E71 =$1a;   O71 =$0a;  LCR = 3;  I8088_IMR  = $21;

N72=$06;   E72 =$1e;   O72 =$0e;  MCR = 4;

{ The next two vars can be changed inside your program.

If you have a nonstandard setting they can be changed as needed }

Combase      : array[1..4] of word = ($03F8,$02F8,$03E8,$02E8);

ComIRQ       : ARRAY[1..4] OF WORD = (4,3,4,3);

{  Preset to COM1, 2400 baud, and N81 }

ComPort           : Integer = 1;

Baudrate          : Longint = 2400;

Params            : byte = N81;

{ Maximum read buffer size }

ComBuffer_Max = 4096;

------- DISCLAIMER ----------------------------------------------------

EaglePro will not be held responsible for any damage WHAT SO EVER this

program might do to your computer including hardware damage, software damage,

or data damage. This program has had many hours of testing before it was

released. To the best of my knowledge it could not possibly do any damage to

your system. This disclaimer was placed here just for legal protection from

those who think that it is possible that this program could do damage to a

system.

This program may be uploaded to and downloaded from commercial systems

such as CompuServe, the Source, BIX and any BBS, as long as there is no

charge for the product itself.  Those copying, sharing, and electronically

transmitting this product are required not to delete or modify the

copyright notice and restrictive notices from the program or documentation.

Any distribution of must include all files contained in the packed format.

---- FUTURE ADDITIONS ------------------------------------------------------

1)  All or any future additions depend of you !!!! This program is not

Freeware and if it doesn't get the support from you then there

won't be any additions or upgrades in the future.

2)  A write buffer isn't nessary but will be added.

3)  Better Fossil FIFO handling. At the present time if there isn't a

Fossil driver loaded then EPCOM turns the FIFO buffer off. This

slows down serial reads but prevents lockups. The locks are caused

by more than one character in the FIFO buffer. Epcom can only read

one character per interrupt at the present causing the FIFO buffer

to not get emptied and lock.

4)  Function to return the fossil name and version.

5)  Procedure to Lock and unlock Fossil drivers.

6)  Better TimeOut functions incase there's a problem with reading

or writing to the modem.

7)  Better ComReadLn command with timeouts if no info is recieved.

8)  Desqview, OS/2, and Windows support.

9)  X-Y-Z modem protocals. Wouldn't it be nice to have Zmodem built

into a term unit for fast easy use?

10)  Much nicer and better documentation.

--- REGISTRATION -----------------------------------------------------------

EPCOM is shareware and you are granted a license to use this

program for a period of 30 days. At the end of the 30 day period

you must either send $20.00 to the address listed below or remove

the program. Usage of the EPCOM for longer than a 30 day evaluation

period requires that you register the product with the author. Failure

to do so violates Federal Copyright Laws and places you in a position

to which the author can pursue monetary damages in a court of law.

If you feel that this program is a good addition to your library of

programs and would like to see future additions added to this program,

then PLEASE send $20.00 to:

EaglePro

P.O. Box 876

Marion, Illinois

You can download the newest version of EPCOM or contact

me by leaving a message to Reid Smith on these

FINE BBS's:

EaglePro (SmithCom)                          (618) 997-8009

Pooh's Corner                                (618) 985-4747

If you have access to the internet you can also post a message to

reid.smith@poohs.com.

Please make all checks payable to EaglePro. All proceeds from

this program will be used in the treatment for Lymes Disease

and helping those who have it.