GSCN2APR   Version 1.0

This converter will convert from a GSI scanner .APR aperture file to

GraphiCode's .APR aperture file (although the files have the same extension,

they have very different formats).  The converter can be used to convert

single files or groups of files.

usage: GSCN2APR <aperture file> [<outfile>]

Example:

To convert "ALIST1.APR" to "NEWALIST.AP_" type:

GSCN2APR ALIST1 NEWALIST

To convert several files at once type:

GSCN2APR *.APR

GraphiCode aperture files will then be produced with the extension

.AP_  to distinguish them from the original GSI .APR files.

Special apertures, those other than ROUND, DRAW, SQUARE, OVAL or

RECT are declared as custom apertures using "TRIANGLE.CAP".  If you

are using one of these apertures, you will need to use APER to

create the desired custom aperture, or choose from the other

aperture types available in our system.

This is a sample aperture list for this converter:

Drawing Limits:  14.12  by  10.21

(not output to the translated .ger file)

Aperture table definitions:

Ap #    Type   Shape      X       Y      Tol    Symb    Tool

010    Draw   Round   0.0050  0.0050  0.0000     0      D10

034    Flash  Round   0.0390  0.0390  0.0040     0      D34

076    Flash  Square  0.0600  0.0600  0.0040     0      D76

079    Flash  Oval    0.0750  0.0900  0.0100     0      D79

093    Flash  Rect    0.0350  0.1500  0.0040     0      D93

Any comments about this converter may be directed to THE SYSOP of the

GraphiCode BBS (206)771-6980.

Release 8-18-92 LAFM

Corrected Problem with overrunning aperture list