CADM2APR   Version 1.2

Copyright (C) GraphiCode Inc 1993  All Rights Reserved.

This converter will convert from a CADAM .APT aperture file to a

GraphiCode .APR aperture file.  The converter can be used to convert single

files or groups of files.

usage: CADM2APR <aperture file> [<outfile>]

CADAM .APT files may contain more than one aperture list.  In this case,

CADM2APR produces multiple output files, using the extension ".AP2",

".AP3" etc for the second, third, etc output files.

Example:

To convert the aperture file "ALIST1.APT" for use with GraphiCode

products, type:

CADM2APR ALIST1

This creates the GraphiCode aperture wheel file "ALIST1.APR".

To convert "ALIST1.APT" to "NEWALIST.APR" type:

CADM2APR ALIST1 NEWALIST

To convert several aperture files type:

CADM2APR *.APT

Here is a sample of the CADAM format:

APERTURE SETUP NO.1 ---

D#   STATION               SIZE            SHAPE      FLASH/LINE

--------------------------------------------------------------------------------

D10     1                  .020             RD             L

--------------------------------------------------------------------------------

D18     9               .040 X .010         REC            F

--------------------------------------------------------------------------------

D36    31           .120 OD X .040 ID     DONUT            F

--------------------------------------------------------------------------------

D86    77        .058 OD X .038 ID THERMAL w/ .010 X-SHAPE CONNECTIONS

Version 1.2 ignores empty lines, and apertures described as "BLANK" or "   ".

Version 1.1 handles multiple lists within a single file.

Any comments about this converter may be directed to THE SYSOP of the

GraphiCode BBS (206)771-6980.