Documentation for VIEW.EXE, version 1.54

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

VIEW [filename|filespec] [options]

VIEW can read faxes stored in ZFAX version 2 (1D-MH only), QuickLink II

QFX format, or TIFF-Class-F format (Faxworks).  It can print these faxes

also.

The default filespec is "*.??X" which nicely handles *.FAX, *.QFX, *.PCX,

and *.DCX.  If you specify a filename rather than a filespec, VIEW works

in batch mode rather than menu mode.  (See notes on BATCH mode at the end

of this document.)

To use VIEW, change into the directory where all your faxes are located,

and then start it, by typing "VIEW".  If if finds any faxes, it will bring

up a list of them on the screen, and you can use the up and down arrows to

select which fax you wish to view or print.

The available commands are listed on the bottom of the screen...

<Enter>=view, <Ctrl-Enter>=print, <F2>=convert to PCX, <F3>=convert to DCX,

and <Delete>=delete the fax

If you start VIEW without any command line parameters, it will automatically

detect either a CGA, EGA, or VGA display.  However, it will not detect the

printer type, so you will have to specify that manually.

To force a specific video mode when starting VIEW...

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Mode  Resolution

----  ----------

VIEW /CGA                    $6    640x200

VIEW /EGA                    $10   640x350

VIEW /VGA                    $11   640x480

VIEW /800 (or VIEW /SVGA)    $102  800x600

VIEW /1024                   $104  1024x768

VIEW /1280                   $106  1280x1024

Your video card and/or monitor may not be able to support all of the modes

available.

Commands available while viewing the fax itself

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

When the fax image is on the screen, you have a few choices...

Alt-I  invert the fax colors (black becomes white, white becomes black)

Alt-F  flip the fax for thoese people that send you upside down faxes

Alt-S  shrink the fax so that the image isn't quite so big on the screen

You can pan around by using the up, down, left, and right arrow keys.

To go to the next page in a fax, <PgDn>, the previous page <PgUp>

To use a specific printer driver when starting VIEW...

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Switch   DPI    Printer

------ -------  -------------

VIEW /P24   180x180  Epson compatible 24-pin dot matrix

VIEW /P25   360x180  Epson compatible 24-pin dot matrix

VIEW /P26   180x180  Canon BJ-200 compatible bubble-jet printer

VIEW /P27   360x360  Canon BJ-200 compatible bubble-jet printer

VIEW /P37   150x150  PCL level 3 compatible laser printer

VIEW /P38   150x150  PCL level 4 compatible laser printer (faster than PCL3)

VIEW /P39   150x150  PCL level 5 compatible laser printer (faster than PCL4)

VIEW /P40   300x300  PCL level 3 compatible laser printer

VIEW /P41   300x300  PCL level 4 compatible laser printer (faster than PCL3)

VIEW /P42   300x300  PCL level 5 compatible laser printer (faster than PCL4)

VIEW /P24 is recommened for most dot matrix printers, and /P40 is recommened

for most lasers if you do not know what PCL level it supports.

*** What about printing to ports other than LPT1 ???

VIEW /P40:LPT2

Will print to a laser printer on LPT2 as a software interrupt 17 device.

(Should use this.)

VIEW /P40:+LPT2

Will print to a laser printer on LPT2 as a "file" rather than a device.

(Sometimes this prints a little faster, but usually it prints much slower.)

VIEW /P40:+TEST.PRN

then from a DOS prompt, "copy test.prn lpt2" as an alternate way of printing

*** What about A4 or US-Legal paper sizes?

VIEW /P40 /A4     for metric-A4 (rather than US-letter)

VIEW /P40 /LP     for US-legal (rather than US-letter)

Miscellaneous switches

~~~~~~~~~~~~~~~~~~~~~~

VIEW /DB    Try to display bad scan lines (normally, VIEW just puts a dotted

line across the screen if it detects a bad scan line.)

VIEW /BO    Display fax using a backwards bit order.  This may be necessary

if you have received a fax on a USR modem with Binkley's internal

fax routines, or if you have started BGFAX with the incorrect

command line switch.

Multiple command lines parameters

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

VIEW /p40 /1024

For example would make VIEW use 1024x768 resolution when viewing faxes and

print faxes (when the user hits Ctrl-Enter) to a laser printer.  It is

suggested that you make a batch file to call VIEW with these parameters so

that you don't have to always type them in.

BATCH MODE

~~~~~~~~~~

You can also start VIEW in a "batch" mode, which requires no user

intervention.

"VIEW FAX0001.FAX /P40" would print the fax0001.fax file to a laser printer.

"VIEW fax0001.fax /dcx" would convert that fax to a DCX file

"VIEW fax0001.fax /pcx" would convert that fax to PCX files

(Note that a DCX file is basically a multipage PCX file, and many fax

applications support the DCX format.  PCX format gets messy because each

fax page has to be saved into a seperate file, but PCX format is nearly

universal and just about every graphics program on the planet will let

you load a PCX image.)

Note about 2D-MR compressed faxes

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

VIEW does not currently support 2D-MR faxes properly.  A 2D-MR fax is

basically a 1D-MH normal fax with every other line having only "difference"

information rather than the whole scan line encoded.  VIEW will correctly

view the 1D-MH lines, but will show the 2D-MR lines as improperly decoded

scan lines.

Regards,

B.J. Guillot