STARTPRG (Start Program) Utility

This utility should be used in conjuction with StartGem or TOS 1.4 for

autoloading NeoDesk (or any other program) when you desire to have a pause

before having that program executed.

Some accessories, which work best when installed from the built-in GEM

desktop, require this pause so that they can fully install themselves and

load any files that they require before any autoloading program is executed.

It works by waiting for all disk access to terminate and then waiting "x"

number of seconds before executing the program specified.  It will remain

in memory (taking up less than 2K) to prevent any problems from accessories

that might change its vectors.

The program reads the same STARTGEM.INF file that StartGem uses.  The format

of the STARTGEM.INF file should be as follows:

Line 1> The name and path of the program to be run by StartGem.  If

using STARTPRG.PRG then this line should contain the name and

path of STARTPRG.PRG.  Example: "C:\STARTPRG.PRG".  If you are not

using StartGem, you can then leave this line blank.  Even if you

are not using StartGem, this line must still be present.

Line 2> The name and path of the program to be run by STARTPRG.PRG.  If

you are not using STARTPRG.PRG, then this line and the next can

be left blank.  Example: "C:\NEODESK\NEOMASTR.PRG"

Line 3> The number of seconds that STARTPRG will wait after all disk access

has stopped before running the program in line 2.  If there is no

number on line 3, then the default is 1 second.

If you are running TOS 1.4 or greater, you should set STARTPRG.PRG to

autoboot from the GEM desktop via Install Application; do not use StartGem

in the AUTO folder.  In this case, line 1 does not matter.

Line 2 and 3 are only needed if line 1 points to STARTPRG.PRG or if you have

installed STARTPRG to autoboot under TOS 1.4.

Sample STARTGEM.INF:

--------cut here---------

C:\STARTPRG.PRG           <- Program executed by StartGem

C:\NEODESK\NEOMASTR.PRG   <- Program executed by STARTPRG.PRG

5                         <- Amount of time after disk access to wait.

--------cut here---------

Examples:

If you had a 1040 with TOS 1.0 (or a Mega with TOS 1.2), you would probably

place STARTGEM.PRG in your AUTO folder and then place STARTGEM.INF (as shown

as above) in the root directory of your boot drive along with STARTPRG.PRG.

StartGem would execute StartPrg, which would then wait for "x" number of

seconds (after the end of any disk activity) before autoloading the

application you wanted to run.

If you had a Mega with TOS 1.4, you would then NOT use StartGem.  Instead

you would place StartPrg and STARTGEM.INF in the same directory and then

install StartPrg as an "auto" application using the Install Application

command.  StartPrg would execute and read from STARTGEM.INF the name and

path of the program you wanted to execute; and then wait "x" number of

seconds (after the end of any disk activity) before autoloading the

application you wanted to run.

Note about MultiDesk & HeadStart:

--------------------------------

This is similar to the effect produced when HeadStart is used in conjunction

with MultiDesk.  It will wait until MultiDesk is finished and then autoload

whatever application is chosen.  The different is that StartPrg will work

with the autoloading feature of TOS 1.4 (and greater).  If you are alredy

running HeadStart with MultiDesk, then you have no need for StartPrg.

Users of MultiDesk who wish to autoload any program (and not use HeadStart)

should find this utility useful.  Any previous problems that you were

experiencing when autoloading a program (using StartGem or TOS 1.4) should

be remedied by this utility (within the specified limits).  Initial testing

seems to show that it is best to specify a wait time of about 5 seconds or

so.  MultiDesk will load into memory and then proceed to load all of its

accessories.  Once it has finished loading all of its accessories (and thus

disk access stops) then STARTPRG will wait for the amount of time specified

before executing the program you wanted to autoload.

The second thing to keep in mind is that you should NEVER change the buffer

size (or select "clear all accessories") unless you are at the built-in

desktop.  Attempting to do this from the inside of any program will result

in the vectors of that program being changed.  MultiDesk sometimes warns you,

though it will sometimes get confused and "think" that it is at the built-in

desktop when it is not.  What does that mean?  It means that the system will

problem crash as soon as you exit the program you were running when you

changed the buffer (or shortly thereafter).  If you DO change the buffer size

(or clear all accessories) then you must make sure that you do NOT exit the

program that you were in when you changed the buffer.